3GPP TSG RAN WG4 Meeting #90BIS					R4-1905213

Xian, China, 8th – 12th April, 2019

Title:	Draft reply LS on NR V2X UE RF parameters for NR V2X service

Response to:	R1-1810006

Release:	Release 16

Work Item:	5G_V2X_NRSL-Core

Source:	3GPP RAN WG4

To:	3GPP RAN WG1

Cc:

Contact Person:

Name:	Suhwan Lim

E-mail Address:	suhwan.lim@lge.com

Attachments:	None.

1. Overall Description:

RAN WG4 already sent the reply LS on the NR V2X RF parameters in R4-1816666. However, there are some FFS for the detail RF parameters such as AGC enhancement, Timing error and Frequency error. Hence RAN4 further discussed on the NR V2X detail RF/RRM parameters for NR V2X physical layer design aspect. According to the RAN4 discussion, RAN4 provide following answer to RAN1.

- Question 1: RAN1 would like to ask RAN4 to confirm the validity of the evaluation assumptions or provide guidance on appropriate values for:

- AGC settling time

- TX/RX switching time (already completed in R4-1816666)

- Timing error between a UE and its synchronization reference

- Frequency error between a UE and its synchronization reference

Answer 1: The below RF/RRM parameters are recommended for NR V2X evaluation in RAN1

- Timing error between a UE and its synchronization reference.

  - FR1:

    - gNB synchronization reference source: reuse NR UE UL transmission timing error defined in Table 7.1.2-1 in TS38.133

    - GNSS synchronization reference source: Time error is 12*64Tc for 15kHz SCS. It is starting point, RAN4 will further analyse the timing error by GNSS synchronization source and subject to change during NR V2X WI phase. Other SCSs are FFS

    - UE synchronization reference source: FFS

  - FR2:

    - gNB synchronization reference source: reuse NR UE UL transmission timing error defined in Table 7.1.2-1 in TS38.133. It is starting point, RAN4 will further analyse the timing error by gNB synchronization source and subject to change during NR V2X WI phase.

    - GNSS synchronization reference source: FFS

    - UE synchronization reference source: FFS

- Frequency error between a UE and its synchronization reference

  - FR1: ±0.1 ppm for gNB/GNSS/NR UE synchronization reference source

  - FR2: ±0.1 ppm for gNB/GNSS/NR UE synchronization reference source.

2. Actions:

To RAN WG1:

RAN WG4 respectfully asks RAN WG1 to evaluate the NR V2X sidelink operation taking into account the above recommendation. RAN4 will further study the remaining open issues in the upcoming meetings.

3. Date of Next TSG-RAN Meetings:

3GPP RAN WG4 #91	   13th – 17th May 2019				Reno, US

3GPP RAN WG4 #92	    26th – 30th August 2019				Ljubljana, Slovenia

Reference

[1] R1-1810006 “LS on IBE model for V2X” RAN WG1

[2] R4-1816666, “Reply LS on NR V2X RF parameters for RAN1 physical layer design aspects” RAN WG4