3GPP TSG-RAN WG4 Meeting #91	R4-1905383

Reno, USA, 13th – 17th May 2019

Title: 	Discussion on SAR requirement for EN-DC PC2 UE

Source: 	CATT

Agenda item:	9.20.3

Document for:	Discussion

- Introduction

In the last RAN4 meeting, a way forward on PC2 EN-DC was approved for SAR RF requirements [1] with the following agreement:

Agreements for PC2 EN-DC Combination (LTE TDD PC3+NR TDD PC3)

- As network based solution, at least the following is proposed.

  - Introducing a new signaling which indicates that the NR maxUplinkDutyCycle capability corresponds to LTE UL/DL configuration and it is a per band combination capability

  - The NR maxUplinkDutyCycle capability for PC2 inter-band EN-DC is optional.

    - The UE behaviour is FFS if NR maxUplinkDutyCycle capability for PC2 inter-band EN-DC is absent

  - It is assumed that the UE follows the UL grant regardless of duty-cycle indication (but can account for the indication)

  - The values range of NR maxUplinkDutyCycle for PC2 inter-band EN-DC are from [20]% to 100% and the default value for NR uplink duty cycle can be specified according to different TDD configurations as following

| LTE UL/DL Configuration (Uplink duty cycle) | 0 (60%) | 1 (40%) | 2 (20%) | 3 (30%) | 4 (20%) | 5 (10%) | 6 (50%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Default value for NR maxUplinkDutyCycle (10% as granularity) | [40]% | [60]% | [80]% | [70]% | [80]% | [90]% | [50]% |

- UE based solution is FFS. Also solution to combine network and UE based solution will be FFS

This document will discuss further how to address SAR requirement for PC2 inter-band EN-DC within FR1 and give our proposals.

- Discussion

From the approved way forward, it is decided to introduce a new signalling maxUplinkDutyCycle for NR. UE will report it to network to indicate its supported maximum uplink duty cycle capability on each inter band EN-DC combination.

To our understanding, based on UE maxUplinkDutyCycle capability reporting for NR, the gNB can reference this capability but doesn’t have to be limited by this capability. The gNB can schedule uplink duty cycle large than the value of UE reported capability assuming that the UE will not transmit with maximum output power . UE will transmit according to gNB uplink scheduling, but the maximum output power will reduced when scheduled uplink duty cycle is large than the UE reported one. In this case, the maximum allowable output power will be specified in specification, i.e. the NR maximum output power will be reduced 3dB.

In the way forward, there is an open issue on the UE behaviour. It is FFS if NR maxUplinkDutyCycle capability for PC2 inter-band EN-DC is absent. To our understanding, the UE will use the default values as the duty cycle limit to decide whether to do power back off when maxUplinkDutyCycle is absent.

In summary, the UE behaviour is proposed as following,

- If UE reports NR maxUplinkDutyCycle capability, and is scheduled uplink duty cycle large than the reported value, the NR maximum output power will be reduced 3dB, i.e. 20dBm. Otherwise, the NR maximum output power will be 23dBm.

- If UE does not report NR maxUplinkDutyCycle capability, the limitations of scheduled uplink duty cycle are the default value based on LTE UL/DL configuration. When scheduled uplink duty cycle large than the default value of maxUplinkDutyCycle, the NR maximum output power will be reduced 3dB, i.e. 20dBm. Otherwise, the NR maximum output power will be 23dBm.

Based on above scheme, the clarification for meeting SAR requirement for inter band EN-DC can be defined as following:

==========

If a UE supports a different power class than the default UE power class for an EN-DC band combination and the supported power class enables the higher maximum output power than that of the default power class:

–	if the field of NR capability maxUplinkDutyCycle is absent and the percentage of NR uplink symbols transmitted plus the percentage of E-UTRA uplink configuration in a certain evaluation period is larger than 100% (The exact evaluation period is no less than one radio frame); or

–	if the field of NR capability maxUplinkDutyCycle is not absent and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle as defined in TS38.331 (The exact evaluation period is no less than one radio frame); or

–	if the IE P-Max as defined in TS 38.331 is provided and set to the maximum output power of the default power class or lower;

–	shall apply all requirements for the default power class and set the configured transmitted power as specified in TS36.101 for E-UTRA carrier, and as specified in TS38.101-1 for NR carrier, and ΔPPowerClass = 3dB is applied on E-UTRA carrier and NR carrier respectively;

–	Else

–	shall apply all requirements for the default power class and set the configured transmitted power as specified in TS36.101 for E-UTRA carrier, and as specified in TS38.101-1 for NR carrier.

==========

- Conclusion

This document discussed how to meet SAR requirement for PC2 EN-DC UE. It is proposed to capture the following requirement for PC2 inter-band EN-DC into the UE spec.

==========

If a UE supports a different power class than the default UE power class for an EN-DC band combination and the supported power class enables the higher maximum output power than that of the default power class:

–	if the field of NR capability maxUplinkDutyCycle is absent and the percentage of NR uplink symbols transmitted plus the percentage of E-UTRA uplink configuration in a certain evaluation period is larger than 100% (The exact evaluation period is no less than one radio frame); or

–	if the field of NR capability maxUplinkDutyCycle is not absent and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle as defined in TS38.331 (The exact evaluation period is no less than one radio frame); or

–	if the IE P-Max as defined in TS 38.331 is provided and set to the maximum output power of the default power class or lower;

–	shall apply all requirements for the default power class and set the configured transmitted power as specified in TS36.101 for E-UTRA carrier, and as specified in TS38.101-1 for NR carrier, and ΔPPowerClass = 3dB is applied on E-UTRA carrier and NR carrier respectively;

–	Else

–	shall apply all requirements for the default power class and set the configured transmitted power as specified in TS36.101 for E-UTRA carrier, and as specified in TS38.101-1 for NR carrier.

==========

- Reference

[1]	R4-1905001, WF on PC2 EN-DC (1 LTE TDD band + 1 NR TDD band), CMCC, vivo, OPPO, CATT

[2]	R4-1903245, Further discussion on SAR requirement for EN-DC PC2 UE, CATT