3GPP TSG-RAN WG4 #91	R4- 1906184

Reno, Nevada, US, 13th – 17th May 2019

Source: 	Ericsson

Title:  	Elaboration on 7-24 GHz PA power scaling and AAS dependency

Agenda Item:	10.4.7

Document for:	Discussion

# Introduction

In RAN4#90 and RAN4#90bis, the discussion around technical aspects and considerations for 7-24 GHz was initiated. In [1] and [2], the technology trends for power amplifiers (PA) considering different semiconductor technologies as well as power, PAE and ACLR dependencies based on two different approaches for 22 GHz example frequency was described in detail.

In this paper, we discuss scaling principles for PA RF power vs. frequency when considering equipment based on an antenna array to further support the discussion on the scaling of PA performance metrics vs. frequency.

# Discussion

As discussed in [1] the power capability  PC (the maximum level of signal power that can be generated) of a PA is fundamentally governed by the Johnsons’ figure of merit. Once the semiconductor technology has been chosen (CMOS, SiGe, GaN, etc.) for implementing a power amplifier, the power capability  PC is bounded by  PC∝1/f2 (-20dB per decade).

Over the frequency range (7-24GHz) considered, however, it is not enough to consider the power capability of single PA. Rather, it may be justified to assume some sort of multi-antenna arrangement or more specifically an antenna array, at least for the upper part of the frequency range. Figure 1 illustrates a conceptual unit area of an antenna array with N elements and will be used to support the discussion below. Here, the array is a uniform array (equidistant distance between elements) and without lack of generality one PA is associated with respective element.

One may rightfully say that the antenna element separation d is constant over frequency when measured in wavelengths λ, e.g. 0.5λ. Consequently, the antenna separation measured in metric meters will scale as 1/f where f is the target RF frequency for which the antenna array has been designed, and thus the number of elements per unit area N will be ∝f2 (+20dB per decade).

The DC power provided to drive one power amplifier is PDC and the DC power provided to the unit area equals PDC,UA=N PDC. Correspondingly, the RF power generated by one PA is PRF=PAE PDC where PAE is the efficiency of the PA, and the RF power generated by the unit area is PRF,UA=N PRF=N PAE PDC (the exact definition of PAE also includes the PA input signal power but this is left out here for simplicity).

Unit area of antenna array with N elements

PDC

PRF=PAE PDC

•   •   •

•   •   •

•   •   •

d

d

PDC,UA

PRF,UA

Figure 1 Relation between power, number of elements and frequency of operation.

If  PAE and ACLR had been constant over frequency, a constant DC power per unit area (irrespective of frequency) would obviously imply that the RF power per unit area PRF,UA would also be constant over frequency, as the power capability per PA scales -20dB/decade while the number of elements scales as +20dB/decade.

For FR2 ACLR is 28dB but it is expected that it should improve towards lower frequencies, given the substantially higher ACLR for FR1. The aim of this discussion is to present one such scaling mechanism.

In [2] a trend of -5dB/decade (1/f) was identified for PAE based on empirical data for peak power. It is expected that this scaling also applies when operating in back-off. With a cap on DC power per unit area, such a trend would allow for more RF power per unit area with a preserved ACLR for lower frequencies, i.e. increasing by 5dB when lowering the frequency by one decade. This exemplified in Figure 2 (constant ACLR) for power levels referring to one PA.

Alternatively, one can assume a constant DC as well as constant RF power per unit area and let the PAs towards lower frequencies operate at a lower PAE, to instead allow a higher ACLR, see Figure 2 (improved ACLR). Exploiting this to its full extent means that all the PAE improvement for lower frequencies is translated to an equivalent reduction of RF power per PA compared to the RF power per PA for a given ACLR. In [2] basic analysis, backed up by PA simulations, showed that ACLR improves roughly by 2dB for every dB of RF power reduction for a given PA in the ACLR performance regime of interest.

Example: An array operating at 30GHz provides ACLR of 28dB. An array operating at 15GHz with the same DC and RF power per unit area will have an ACLR of 28+2·5·log10(30/15) =31dB.

log⁡(P)

log⁡(f)

PRF

PDC

log⁡(PAE)

constant ACLR

improved ACLR

-20dB/dec

-20dB/dec

Figure 2 Illustration of PA power metrics vs frequency for constant ACLR vs improved ACLR (towards lower frequencies).

The discussion in this paper is then used when output power, ACLR and PAE dependencies for 7-24 GHz example frequencies is elaborated in detail in [3].

# Summary and conclusion

In this paper, scaling principles for PA RF power over frequency when considering BS based on an antenna array instead of a single was discussed. The intention is to further support the discussion on the scaling of PA performance metrics such as output power, ACLR and PAE characteristics over frequency

Different approaches based on keeping certain metrics constant which result in different trade-offs were further elaborated.

# References

[1]	R4-1900832, “7-24 GHz technical aspects and considerations”, Ericsson

[2]	R4-1903464, “7-24 GHz PA considerations and dependencies”, Ericsson

[3]	R4-1906180, “Further elaboration on 7-24 GHz PA dependencies for different example frequencies”, Ericsson