3GPP TSG-RAN WG4 #91	R4-1906661

Reno, Nevada, USA, 13th – 17th May, 2019

Title:	LS on switching and interruption time for UE dynamic adaptation to the maximum number of MIMO layers

Release:	Rel-16

Work Item:	NR_UE_pow_sav-Core

Reply To:

Source:	RAN WG4

To:	RAN WG1

CC:

Contact Person:

#### Name:	Din-hwa Huang

E-mail Address:	Althea.huang@mediatek.com

1. Overall Description:

RAN4 would like to inform RAN1 about the RAN4 conclusion on required switching and interruption times for Rel-16 UE dynamic adaptation to the maximum number of MIMO layers. Note that new maximum number of MIMO layers should not be larger than the per-cell parameter maxMIMO-Layers under PDSCH-ServingCellConfig.

The following switching time is supported:

- For DCI-triggered switch, after UE receives a request at slot n, UE shall be ready to receive PDSCH with new settings at slot n+ TMIMOswitchDelay. The values for TMIMOswitchDelay are provided in the following table for different SCS.

| μ (SCS = 15 kHz⋅2μ) | NR Slot length (ms) | TMIMOswitchDelay (slots)* |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 2 |
| 2 | 0.25 | 3 |
| 3 | 0.125 | 4 |

The interruption to other serving CCs is not required for this dynamic adaptation to the maximum number of MIMO layers.

2. Actions:

To RAN WG1

ACTION: RAN4 respectfully asks RAN1 to consider the above conclusions in the further work.

3. Date of Next RAN4 Meetings:

RAN4#92	14th – 18th August 2019 	Ljubljana, Slovenia

RAN4#92Bis	26th – 30th October 2019	China