3GPP TSG-RAN WG4 Meeting #91	R4-1906725

# Reno, US, May 13 – 17, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.817-02 | CR | 0041 | rev |  | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TR 38.817-02 removal of Tx Diversity for TAE testing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-05-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The description for time alignment error (TAE) testing states that TX diversity can be used. However, the current physical layer standards do not have an explicit TX diversity mode. The change will remove the text for “TX diversity” |  |  |  |  |  |  |  |  |
| Summary of change: |  | There are several changes implemented: - The text for “TX diversity” is removed for NR - The reason for not supporting TX diversity for NR for TAE testing is provided - Removal of requirement related items. The principle is not to include the requirement in the TR, as it is included already in the TS. - Formatting |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This ensures consistency in the specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.3.1, 9.6.3, 9.6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

************************* BEGIN CHANGE 1 ********************************

### 6.5.3	Time alignment error

#### 6.5.3.1	General

Frames of NR signals present at antenna connectors (for 1-C type NR BS), or at TAB connectors (for 1-H type NR BS) are not perfectly aligned in time. In relation to each other, the RF signals present at the BS transmitter antenna port(s) or at the transceiver array boundary may experience certain timing differences.

For a specific set of NR signals/transmitter configurations/transmission modes, the conducted Time Alignment Error (TAE) is defined as the largest allowed timing difference (i.e. error) between two different reference signals belonging to different antenna connectors (for 1-C type NR BS), or TAB connectors (for 1-H type NR BS).

TAE is only applicable for NR BS transmitting from multiple antennas via MIMO, CA, or some combinations of these two.

In UTRA and E-UTRA, TAE requirement was defined for TX diversity, MIMO transmission, carrier aggregation and their combinations. However in NR, there is no TX diversity transmission mode defined. As a result, no TAE tests need to be developed for TX diversity for NR operation.

In case of 1-H type NR BS the number of TAB connectors could be large. Therefore, a concept for reducing number of test combinations was introduced in 3GPP TS 37.105 [7]. In TAE requirement for AAS BS is designed as:

The TAE between any two TAB connectors from different transmitter groups shall not exceed the specified minimum requirements above, where transmitter groups are associated with the TAB connectors in the transceiver unit array corresponding to MIMO transmission, CA, etc.

************************* END CHANGE 1 ********************************

************************* BEGIN CHANGE 2 ********************************

### 9.6.3	OTA time alignment error for FR1

Similar to the conducted TAE requirement motivation captured in sub-clause 6.5.3, the OTA TAE requirement is defined as the largest timing difference between any two different NR signals belonging to different reference symbols in the radiated domain. The OTA TAE requirement is defined as a directional requirement at the RIB and shall be met within the OTA coverage range.

This requirement applies to frame timing in MIMO transmission, carrier aggregation and their combinations in FR1.

The same general comments apply for carrier aggregation, as already stated in 6.5.3.1 carrier aggregation.

### 9.6.4	OTA time alignment error for FR2

The same general comments apply for carrier aggregation, as already stated in 6.5.3.1 carrier aggregation.

************************* END CHANGE 2 ********************************