3GPP TSG-RAN WG4 Meeting #91	R4-1906998

Reno, US, 12th – 17th May 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR | <CR#> | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TS 38.141-2: Introduction of radiated performance requirements for PUSCH for HST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-5-3 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In order to introduce BS performance requirements for HST to BS specification |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduction of conducted PUSCH performance requirements for HST |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR does not support HST |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

--------------Start of text proposal-------------

8.2.3	Performance requirements for high speed train condition with transform precoding disabled

8.2.3.1	Definition and applicability

The performance requirement of PUSCH is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in annex A. The performance requirements assume HARQ re-transmissions. The performance requirements for High Speed Train conditions are optional.

Which specific test(s) are applicable to BS is based on the test applicability rules defined in subclause 	8.1.2.1.

8.2.3.2	Minimum Requirement

For BS type 1-O, the minimum requirement is in TS 38.104 [2], subclause 11.2.1.3.

8.2.3.3	Test purpose

The test shall verify the receiver’s ability to achieve throughput under multipath fading propagation conditions for a given SNR.

8.2.3.4	Method of test

8.2.3.4.1	Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested: M, see subclause 4.9.1.

Direction to be tested:

-	For BS type 1-O, receiver target reference direction (see D.31 in table 4.6-1).

8.2.3.4.2	Procedure

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.3.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex E.3. Each of the demodulation branch signals should be transmitted on each polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A, and according to additional test parameters listed in table 8.2.3.4.2-1.

Table 8.2.3.4.2-1: Test parameters for testing PUSCH

| Parameter |  | BS type 1-O |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Uplink-downlink allocation for TDD |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | Maximum number of OFDM symbols for front loaded DM-RS | 1 |
|  | Number of additional DM-RS symbols | 1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | EPRE ratio of PUSCH to DM-RS | -3 dB |
|  | DM-RS port | {0}, {0,1} |
|  | DM-RS sequence generation | NID=0, nSCID=0 |
| Time domain resource | PUSCH mapping type | A |
|  | PUSCH starting symbol index | 0 |
|  | PUSCH symbol length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| TPMI index for 2Tx two layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| PTRS configuration | Frequency density (KPT-RS) | N.A. |
|  | Time density (LPT-RS) | N.A. |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex J.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the BS receiver is as specified in subclause 8.2.3.5.1 for BS type 1-O, and that the SNR at the BS receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 8.2.3.4.2-2.

Table 8.2.3.4.2-2: AWGN power level at the BS input

| BS type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 15 | 5 | [-83.5] - ΔOTAREFSENS dBm / 4.5MHz |
|  |  | 10 | [-80.3] - ΔOTAREFSENS dBm / 9.36MHz |
|  |  | 20 | [-77.2] - ΔOTAREFSENS dBm / 19.08MHz |
|  | 30 | 10 | [-80.7] - ΔOTAREFSENS dBm / 8.64MHz |
|  |  | 20 | [-77.4] - ΔOTAREFSENS dBm / 18.36MHz |
|  |  | 40 | [-74.2] - ΔOTAREFSENS dBm / 38.16MHz |
|  |  | 100 | [-70.1] - ΔOTAREFSENS dBm / 98.28MHz |

8)	For reference channels applicable to the BS, measure the throughput.

8.2.3.5	Test Requirement

8.2.3.5.1	Test requirement for BS type 1-O

The throughput measured according to subclause 8.2.3.4.2 shall not be below the limits for the SNR levels specified in table 8.2.3.5.1-1 to table 8.2.3.5.1-7 for 1Tx.

Table 8.2.3.5.1-1: Test requirements for PUSCH, Type A, 5 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex J) | Fraction of maximum throughput | FRC (Annex A) | DM-RS configuration | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | HST Scenario 3 | 30 % | TBD | 1+1 | TBD |
|  |  | Normal | HST Scenario 3 | 70 % | TBD | 1+1 | TBD |

Table 8.2.3.5.1-2: Test requirements for PUSCH, Type A, 10 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex J) | Fraction of maximum throughput | FRC (Annex A) | DM-RS configuration | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | HST Scenario 3 | 30 % | TBD | 1+1 | TBD |
|  |  | Normal | HST Scenario 3 | 70 % | TBD | 1+1 | TBD |

Table 8.2.3.5.1-3: Test requirements for PUSCH, Type A, 20 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex J) | Fraction of maximum throughput | FRC (Annex A) | DM-RS configuration | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | HST Scenario 3 | 30 % | TBD | 1+1 | TBD |
|  |  | Normal | HST Scenario 3 | 70 % | TBD | 1+1 | TBD |

Table 8.2.3.5.1-4: Test requirements for PUSCH, Type A, 10 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex J) | Fraction of maximum throughput | FRC (Annex A) | DM-RS configuration | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | HST Scenario 3 | 30 % | TBD | 1+1 | TBD |
|  |  | Normal | HST Scenario 3 | 70 % | TBD | 1+1 | TBD |

Table 8.2.3.5.1-5: Test requirements for PUSCH, Type A, 20 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex J) | Fraction of maximum throughput | FRC (Annex A) | DM-RS configuration | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | HST Scenario 3 | 30 % | TBD | 1+1 | TBD |
|  |  | Normal | HST Scenario 3 | 70 % | TBD | 1+1 | TBD |

Table 8.2.3.5.1-6: Test requirements for PUSCH, Type A, 40 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex J) | Fraction of maximum throughput | FRC (Annex A) | DM-RS configuration | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | HST Scenario 3 | 30 % | TBD | 1+1 | TBD |
|  |  | Normal | HST Scenario 3 | 70 % | TBD | 1+1 | TBD |

Table 8.2.3.5.1-7: Test requirements for PUSCH, Type A, 100 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex J) | Fraction of maximum throughput | FRC (Annex A) | DM-RS configuration | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | HST Scenario 3 | 30 % | TBD | 1+1 | TBD |
|  |  | Normal | HST Scenario 3 | 70 % | TBD | 1+1 | TBD |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

8.2.4	Performance requirements for high speed train condition with transform precoding enabled

8.2.4.1	Definition and applicability

The performance requirement of PUSCH is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in annex A. The performance requirements assume HARQ re-transmissions. The performance requirements for High Speed Train conditions are optional.

Which specific test(s) are applicable to BS is based on the test applicability rules defined in subclause 8.1.2.

8.2.4.2	Minimum Requirement

For BS type 1-O, the minimum requirement is in TS 38.104 [2], subclause 11.2.1.4.

8.2.4.3	Test Purpose

The test shall verify the receiver’s ability to achieve throughput under multipath fading propagation conditions for a given SNR.

8.2.4.4	Method of test

8.2.4.4.1	Initial Conditions

Test environment:	Normal, see subclause B.2.

RF channels to be tested:	M, see subclause 4.9.1.

Direction to be tested:

-	For BS type 1-O, receiver target reference direction (see D.31 in table 4.6-1).

8.2.4.4.2	Procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2.4.

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.3.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex E.3. Each of the demodulation branch signals should be transmitted on each polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A, and according to additional test parameters listed in table 8.2.4.4.2-1.

Table 8.2.4.4.2-1: Test parameters for testing PUSCH

| Parameter |  | BS type 1-O |
| --- | --- | --- |
| Transform precoding |  | Enabled |
| Uplink-downlink allocation for TDD |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | Maximum number of OFDM symbols for front loaded DM-RS | 1 |
|  | Number of additional DM-RS symbols | 1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | EPRE ratio of PUSCH to DM-RS | -3 dB |
|  | DM-RS port | 0 |
|  | DM-RS sequence generation | NID=0, group hopping and sequence hopping are disabled |
| Time domain resource | PUSCH mapping type | A |
|  | PUSCH starting symbol index | 0 |
|  | PUSCH symbol length | 14 |
| Frequency domain resource | RB assignment | 15 kHz SCS: 25 PRBs in the middle of the test bandwidth 30 kHz SCS: 24 PRBs in the middle of the test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS |  | Not configured |

6)	The channel simulators shall be configured according to the corresponding channel model defined in Annex J.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the BS receiver is as specified in subclause 8.2.4.5.1 for BS type 1-O, and that the SNR at the BS receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 8.2.4.4.2-2.

Table 8.2.4.4.2-2: AWGN power level at the BS input

| BS type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 15 kHz | 5 | [-83.5] - ΔOTAREFSENS dBm / 4.5MHz |
|  | 30 kHz | 10 | [-80.7] - ΔOTAREFSENS dBm / 8.64MHz |

8)	For reference channels applicable to the BS, measure the throughput.

8.2.4.5	Test Requirement

8.2.4.5.1	Test requirement for BS type 1-O

The throughput measured according to subclause 8.2.4.4.2 shall not be below the limits for the SNR levels specified in table 8.2.4.5.1-1 to table 8.2.4.5.1-2.

Table 8.2.4.5.1-1: Test requirements for PUSCH, Type A, 5 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex J) | Fraction of maximum throughput | FRC (Annex A) | DM-RS configuration | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | HST Scenario 3 | 30 % | TBD | 1+1 | TBD |
|  |  | Normal | HST Scenario 3 | 70 % | TBD | 1+1 | TBD |

Table 8.2.4.5.1-2: Test requirements for PUSCH, Type A, 10 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex J) | Fraction of maximum throughput | FRC (Annex A) | DM-RS configuration | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | HST Scenario 3 | 30 % | TBD | 1+1 | TBD |
|  |  | Normal | HST Scenario 3 | 70 % | TBD | 1+1 | TBD |

--------------Omitted unchanged sections-------------

J.3	High speed train condition

High speed train condition is as follow:

Scenario 3: Tunnel for multi-antennas

The high speed train condition for the test of the baseband performance is two non-fading propagation channels. For BS with Rx diversity defined in scenario 3, the Doppler shift variation is the same between antennas.

Doppler shift for both scenarios is given by:

(J.3.1)

where  is the Doppler shift and  is the maximum Doppler frequency. The cosine of angle is given by:

, 	(J.3.2)

,                            (J.3.3)

,                            (J.3.4)

where  is the initial distance of the train from BS, and  is BS-Railway track distance, both in meters;  is the velocity of the train in m/s,  is time in seconds.

Doppler shift and cosine angle is given by equation J.3.1 and J.3.2-J.3.4 respectively, where the required input parameters listed in table J.3-1. and the resulting Doppler shift shown in Figure J.3-1 and J.3-2 are applied for all frequency bands.

Table J.3-1: Parameters for high speed train conditions

| Parameter | Value |
| --- | --- |
|  | Scenario 3 |
|  | 300 m |
|  | 2 m |
|  | 300 km/h |
|  | 1150 Hz for 15kHz SCS 2000 Hz for 30kHz SCS |

NOTE1:	Parameters for HST conditions in table J.3-1 including  and Doppler shift trajectories presented on figures J.3-1 and J.3-2 were derived from Band n1 and Band n77, respectively, and are applied for performance verification in all frequency bands.

Figure J.3-1: Doppler shift trajectory for scenario 3 (15kHz SCS)

Figure J.3-2: Doppler shift trajectory for scenario 3 (30kHz SCS)

--------------End of text proposal-------------