3GPP TSG RAN WG4 Meeting #91	R4-1907106

Reno, US, May 13-17, 2019

Agenda Item:	6.8.3.3

Source:	Huawei, HiSilicon

Title:	Aligning RBG mapping for resource allocation type 0

Document for:	Discussion

# Introduction

TM1.2, TM3.2, and TM3.3 use a boosting pattern that is based on resource block groups (RBGs). With the introduction of the filler bits in the frequency domain, the boosting pattern was shifted by 3 RBs. However, direct application of resource allocation type 0 for the boosting pattern may be difficult with the 3 RB shift but resource allocation type 1 may be used. After some discussions with Keysight, examination of the impact to the boosting pattern was performed if resource allocation type 0 were used directly for the boosting pattern. This contribution evaluates the type of changes needed to facilitate discussions.

# Discussion

## Background

The boosting pattern for several test models is provided using tables in 38.141-1 such as below.

Table 4.9.2.2.2-1: Specific physical channel parameters of NR-FR1-TM1.2

| Parameter | Value |
| --- | --- |
| Percent of QPSK PDSCH PRBs boosted (target) | x=40% |
| # of QPSK PDSCH RBGs which are boosted | NRBG=minxNRB-3P,12NRB-3P-NRB-3Pmod2+1, where P is determined from table 5.1.2.2.1-1 from TS 38.214 [18], configuration 1 column using NRB-3 as the size of the bandwidth part |
| Level of boosting (dB) | 3 |
| Locations of PDSCH PRBs which are boosted | Pk+3, Pk+4, …, P(k+1)+2 where k=0,2,4…,2NRBG-2 and NRB-3P-1 |
| # of QPSK PDSCH PRBs which are deboosted | NRB-3-PNRBG |
| Level of deboosting (dB) | 10log10NRB-3-103/10PNRBGNRB-3-PNRBG |
| Modulation of PDSCH PRBs with nRNTI=2 | QPSK |

The procedure for mapping the boosting pattern (as stated in the table) is

- Group RBs into sets of P RBs

- Determine the number of RBGs available

- Determine the number of boosted RBGs among the set of RBGs

- Place a boosted set at the first RBG (RBG#0)

- Place a boosted set at the last complete RBG

- Place a boosted RBG at every other RBG location (2, 4, …) until all boosted RBGs are mapped

- The number of boosted RBGs is used to determine the deboosting level.

One of the benefits of using RBGs is to minimize the specification work / reduce maintenance efforts

## Filler PDSCH

With the introduction of the filler PDSCH, the design of the boosting pattern was modified so that the boosting pattern begins after the filler PDSCH in the frequency domain. The modification was to shift the RB space by 3 RBs and to reduce the RB space by 3 RBs. In addition, since the RBGs no longer start at RB#0, a statement about the RBG concept could be used to identify the RBGs was effectively introduced.

Resource allocation type 1 was used to locate all the RBs in the test model. Type 1 allocation specifies a starting RB and a length.

## Actual RBG groups

Keysight is proposing to use RBG equations in TS 38.214 that include an offset and BWP size. Logically, the boosting pattern is contained within a BWP, but the BWP for the boosting pattern spans the RBs from the end of the filler PDSCH to NRB in the Keysight proposal.

BWP0

BWP1

RAN#90b

proposal

NRB-3

Figure 1. Comparison of the boosting pattern in RAN4#90bis and Keysight proposal.

The equations in TS 38.214 subclause 5.1.2.2.1 are

NRBG=NBWPsize+NBWPstartmodP/P

- the size of the first RBG is RBG0size=P-NBWPstartmodP,

- the size of last RBG is

RBGlastsize=NBWPsize+NBWPstartmodP,  &NBWPsize+NBWPstartmodP>0P,  &otherwise

- the size of all other RBGs is P.

Where (alternative to Table 5.1.2.2.1-1)

P=21+maxlog2NBWPsize/36,0

Using NBWPstart=3 and NBWPsize=NRB-3, the following mapping of RBGs is shown for P=2 and P=4 in Figure 2.

Start=3

P=2

P=4

RBG0

RBG1

RBG8

Start=3

RBG0

RBG1

RBG3

RBG2

RBG4

RBG9

RBG5

Figure 2. RBG numbering for 21 RBs

For the P=2 mapping in Figure 2, there are NRB=21 RBs. From the equations

NRBG=NBWPsize+NBWPstartmodP/P=18+3mod2/2=10

- the size of the first RBG is RBG0size=P-NBWPstartmodP=2-3mod2=1,

- the size of last RBG is RBGlastsize=1

For the P=4 mapping, there are NRB=21 RBs (for illustrative purposes only. P=4 is used when the number of RBs is between 37 and 72). From the equations

NRBG=NBWPsize+NBWPstartmodP/P=18+3mod4/4=6

- the size of the first RBG is RBG0size=P-NBWPstartmodP=4-3mod4=1,

- the size of last RBG is RBGlastsize=1

## Mapping

Use NBWPstart=3 and NBWPsize=NRB-NBWPstart where possible

### Last RBG index

Among the NRBG resource block groups, the first RBG does not have P RBs. The last RBG may not have P RBs. As a result, the number of RBGs with P RBs is either NRBG-1 or NRBG-2. The first RBG will always be skipped because it has P-3 RB, but using the last RBG is complicated.

- P=2, NRB=21. NRB=10 (see example). RBG#9 does not have P RBs.

- P=2, NRB=22. NRB=10. RBG#9 has P RBs.

- P=4, NRB=21. NRB=6 (see example). RBG#5 does not have P RBs. (also true for NRB=22, NRB=23)

- P=4, NRB=24. NRB=6 (see example). RBG#5 has P RBs.

The following formula, which is a modification of the formula above, gives the index of the last RBG with P RBs. The indexing is 0 based.

RBGlast=NBWPsize+NBWPstartmodPP-1

### Number of possible boosted RBGs

The number of RBGs that can be boosted is determined by the load factor x and is xNBWPsizeP. No change in value.

### Maximum number of boosted RBGs

Assume there are Y RBGs with P RBs, where Y=RBGlast. If Y is even, then RBGs #1, 3, 5, …, Y-1, Y can hold boosted RBs for a total of Y/2+1 RBGs that can be boosted. If Y is odd, then RBGs #1, 3, 5, …,Y can hold boosted RBs for a total of (Y+1)/2 RBGs that can be boosted. Mathematically, the formula is 12Y'-Y'mod2+1. This is same as the current equation but Y is slightly different.

The first W-1 RBGs are located at indices 1, 3, 5, … An exception is needed when W equals 1.

## Possible modification

This is just a possible change to the mapping formulas.

Table 4.9.2.2.2-1: Specific physical channel parameters of NR-FR1-TM1.2

| Parameter | Valu |
| --- | --- |
| Percent of QPSK PDSCH PRBs boosted (target) | x=40% |
| # of QPSK PDSCH RBGs which are boosted | NRBG=minxNRB-3P,12NRB-3+3modP-PP-NRB-3+3modP-PPmod2+1, where P is determined from table 5.1.2.2.1-1 from TS 38.214 [18], configuration 1 column using NBWPsize=NRB-3 as the size of the bandwidth part and NBWPstart=3 |
| Level of boosting (dB) | 3 |
| Locations of PDSCH RBGs which are boosted | k=0,2,4…,2NRBG-2 NRB-3+3modPP-1 and if NRBG>1, 1, 3, 2NRBG-2+1. |
| # of QPSK PDSCH PRBs which are deboosted | NRB-3-PNRBG |
| Level of deboosting (dB) | 10log10NRB-3-103/10PNRBGNRB-3-PNRBG |
| Modulation of PDSCH PRBs with nRNTI=2 | QPSK |

The statements for resource allocation 1 / 0 may be clarified. Example

- TM1.1, TM3.1: type 1 for nRNTI=0, nRNTI=2

- TM1.2, TM3.2, TM3.3: type 0 for nRNTI=0, type 0 for nRNTI=1, type 1 for nRNTI=2

- TM2: type 1 for nRNTI=2

## Analysis

The formulas above were checked with Table 1 and the figures in the appendix. Initial analysis shows the results are consistent. Another factor is the boosting / deboosting amounts to ensure there are valid.

Table 1. Computation of values for boosting and RBGs as a function of bandwidth

|  |  |  | 0 based | 40% | 3 dB | 60% | -3 dB | 50% | -6 dB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NRB | NBWPsize | P | RBGlast | NRBG | Deboost | NRBG | Boost | NRBG | Boost |
| 11 | 8 | 2 | 3 | 1 | -1.75 | 2 | 1.76 | 2 | 2.43 |
| 18 | 15 | 2 | 7 | 3 | -4.73 | 4 | 1.96 | 3 | 1.76 |
| 24 | 21 | 2 | 10 | 4 | -4.12 | 6 | 2.21 | 5 | 2.25 |
| 25 | 22 | 2 | 10 | 4 | -3.65 | 6 | 2.04 | 5 | 2.11 |
| 31 | 28 | 2 | 13 | 5 | -3.50 | 7 | 1.76 | 7 | 2.43 |
| 38 | 35 | 2 | 17 | 7 | -4.73 | 9 | 1.84 | 8 | 2.12 |
| 51 | 48 | 4 | 11 | 4 | -2.99 | 6 | 1.76 | 6 | 2.43 |
| 52 | 49 | 4 | 12 | 4 | -2.86 | 7 | 2.21 | 6 | 2.35 |
| 65 | 62 | 4 | 15 | 6 | -4.30 | 8 | 1.85 | 7 | 2.09 |
| 78 | 75 | 8 | 8 | 3 | -2.74 | 5 | 1.96 | 4 | 1.92 |
| 79 | 76 | 8 | 8 | 3 | -2.67 | 5 | 1.92 | 4 | 1.89 |
| 93 | 90 | 8 | 10 | 4 | -3.46 | 6 | 1.96 | 5 | 2.04 |
| 106 | 103 | 8 | 12 | 5 | -4.34 | 7 | 2.03 | 6 | 2.18 |
| 107 | 104 | 8 | 12 | 5 | -4.23 | 7 | 1.99 | 6 | 2.15 |
| 121 | 118 | 8 | 14 | 5 | -3.10 | 8 | 2.02 | 7 | 2.24 |
| 132 | 129 | 8 | 15 | 6 | -3.87 | 8 | 1.74 | 8 | 2.40 |
| 133 | 130 | 8 | 15 | 6 | -3.79 | 8 | 1.71 | 8 | 2.37 |
| 135 | 132 | 8 | 15 | 6 | -3.65 | 8 | 1.67 | 8 | 2.32 |
| 160 | 157 | 16 | 9 | 3 | -2.50 | 5 | 1.81 | 4 | 1.81 |
| 162 | 159 | 16 | 9 | 3 | -2.44 | 5 | 1.78 | 4 | 1.77 |
| 189 | 186 | 16 | 10 | 4 | -3.21 | 6 | 1.85 | 5 | 1.95 |
| 216 | 213 | 16 | 12 | 5 | -3.96 | 7 | 1.91 | 6 | 2.08 |
| 217 | 214 | 16 | 12 | 5 | -3.92 | 7 | 1.90 | 6 | 2.07 |
| 245 | 242 | 16 | 14 | 6 | -4.61 | 8 | 1.93 | 7 | 2.16 |
| 264 | 261 | 16 | 15 | 6 | -3.76 | 8 | 1.70 | 8 | 2.36 |
| 270 | 267 | 16 | 15 | 6 | -3.55 | 8 | 1.64 | 8 | 2.28 |
| 273 | 270 | 16 | 16 | 6 | -3.46 | 9 | 1.96 | 8 | 2.24 |

# Conclusion

The contribution examines using resource allocation type 0 for the RBG mapping as proposed by Keysight. The contribution shows that boosting patterns are achievable for all RB sizes and provides possible formulas.

# References

R4-1901500, “Discussion of clarifying EVM measurements for test models”, Huawei, HiSilicon, RAN4#90, Feb. 25 – Mar. 1, 2019

# Appendix

RBG mapping for all RB sizes.

| 11 RB | 18 RB |
| --- | --- |

| 24 RB | 25 RB |
| --- | --- |

| 31 RB | 38 RB |
| --- | --- |

| 51 RB | 52 RB |
| --- | --- |

| 78 RB | 79 RB |
| --- | --- |

| 132 RB | 133 RB |
| --- | --- |

| 160 RB | 162 RB |
| --- | --- |

| 270 RB | 273 RB |
| --- | --- |