3GPP TSG-RAN WG4 Meeting #91	R4-1907277

# Reno, Nevada, US, 13th – 17th May 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.104 | CR | DraftCR | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR for 38.104 on PUSCH requirements with CP-OFDM and FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  | Date: |  |  | 2019-05-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Demodulation requirements for PUSCH FR1 in section 8: SNR value updates and parameter naming modernization. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This draftCR introduces several editorial changes to align parameter naming with RAN1 and RAN2 specifications. Tables with test requirements are not yet finished and will be updated during the meeting, when agreements about SNR values and other test settings are available. The tables also contain some formatting corrections. This draftCR makes changes on top of “R4-1904056 DraftCR 38104-f50 Combined updates after Xian.docx”. New changes are additionally marked with yellow highlight. They can also be distinguished via the author tag: “Mueller, Axel (Nokia - FR/Paris-Saclay)”. Comments to the editor from the previous draftCR, which remain valid due to the intermediate base document: Tables 8.2.1.2-1 to 8.2.1.2-7 have been deleted, but are re-added just after with modified content. This is done since deletions of rows are not shown in marks and row deletion messes up layout beyond comprehension. Agenda Item: 6.12.2.2.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements remain incomplete and parameter naming remains outdated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.141-1, 38.141-2 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<<Start of first change>>

### 8.2	Requirements for PUSCH with transform precoding disabled

#### 8.2.1.1	General

The performance requirement of PUSCH is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in annex A. The performance requirements assume HARQ retransmissions.

Table: 8.2.1.1-1 Test parameters for testing PUSCH

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Uplink-downlink allocation for TDD |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0}, {0, 1} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |

#### 8.2.1.2	Minimum requirements

The throughput shall be equal to or larger than the fraction of maximum throughput for the FRCs stated in tables 8.2.1.2-1 to 8.2.1.2-14 at the given SNR for 1Tx or for 2Tx two-layer spatial multiplexing transmission. FRCs are defined in annex A.

Table 8.2.1.2-1: Minimum requirements for PUSCH, Type A, 5 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas |  | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-8 | pos1 | [-2.5] |
|  |  | Normal |  | TDLC300-100 Low | 70 % | G-FR1-A4-8 | pos1 | [10.2] |
|  |  | Normal |  | TDLA30-10 Low | 70 % | G-FR1-A5-8 | pos1 | [12.6] |
|  | 4 | Normal |  | TDLB100-400 Low | 70 % | G-FR1-A3-8 | pos1 | [-5.8] |
|  |  | Normal |  | TDLC300-100 Low | 70 % | G-FR1-A4-8 | pos1 | [6.4] |
|  |  | Normal |  | TDLA30-10 Low | 70 % | G-FR1-A5-8 | pos1 | [9.0] |
|  | 8 | Normal |  | TDLB100-400 Low | 70 % | G-FR1-A3-8 | pos1 | [-8.6] |
|  |  | Normal |  | TDLC300-100 Low | 70 % | G-FR1-A4-8 | pos1 | [3.1] |
|  |  | Normal |  | TDLA30-10 Low | 70 % | G-FR1-A5-8 | pos1 | [5.9] |
| 2 | 2 | Normal |  | TDLB100-400 Low | 70 % | G-FR1-A3-22 | pos1 | [0.3] |
|  |  | Normal |  | TDLC300-100 Low | 70 % | G-FR1-A4-22 | pos1 | [18.2] |
|  | 4 | Normal |  | TDLB100-400 Low | 70 % | G-FR1-A3-22 | pos1 | [-3.1] |
|  |  | Normal |  | TDLC300-100 Low | 70 % | G-FR1-A4-22 | pos1 | [10.6] |
|  | 8 | Normal |  | TDLB100-400 Low | 70 % | G-FR1-A3-22 | pos1 | [-6.2] |
|  |  | Normal |  | TDLC300-100 Low | 70 % | G-FR1-A4-22 | pos1 | [6.4] |

Table 8.2.1.2-2: Minimum requirements for PUSCH, Type A, 10 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-9 | pos1 | [-2.6] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-9 | pos1 | [10.4] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-9 | pos1 | [12.6] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-9 | pos1 | [-5.9] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-9 | pos1 | [6.5] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-9 | pos1 | [8.8] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-9 | pos1 | [-8.8] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-9 | pos1 | [3.2] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-9 | pos1 | [5.7] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-23 | pos1 | [1.0] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-23 | pos1 | [18.5] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-23 | pos1 | [-2.9] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-23 | pos1 | [10.8] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-23 | pos1 | [-5.8] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-23 | pos1 | [6.5] |

Table 8.2.1.2-3: Minimum requirements for PUSCH, Type A, 20 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-10 | pos1 | [-2.0] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-10 | pos1 | [10.3] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-10 | pos1 | [12.7] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-10 | pos1 | [-5.5] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-10 | pos1 | [6.5] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-10 | pos1 | [8.9] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-10 | pos1 | [-8.7] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-10 | pos1 | [3.1] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-10 | pos1 | [5.7] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-24 | pos1 | [1.7] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-24 | pos1 | [18.4] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-24 | pos1 | [-2.2] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-24 | pos1 | [10.7] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-24 | pos1 | [-5.7] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-24 | pos1 | [6.5] |

Table 8.2.1.2-4: Minimum requirements for PUSCH, Type A, 10 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-11 | pos1 | [-2.6] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-11 | pos1 | [10.2] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-11 | pos1 | [12.1] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-11 | pos1 | [-6.0] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-11 | pos1 | [6.1] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-11 | pos1 | [8.3] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-11 | pos1 | [-8.9] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-11 | pos1 | [3.0] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-11 | pos1 | [5.2] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-25 | pos1 | [0.9] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-25 | pos1 | [18.0] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-25 | pos1 | [-2.9] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-25 | pos1 | [10.8] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-25 | pos1 | [-5.9] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-25 | pos1 | [6.6] |

Table 8.2.1.2-5: Minimum requirements for PUSCH, Type A, 20 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas |  | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-12 | pos1 | [-3.0] |
|  |  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-12 | pos1 | [10.1] |
|  |  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-12 | pos1 | [12.2] |
|  | 4 |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-12 | pos1 | [-6.3] |
|  |  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-12 | pos1 | [6.0] |
|  |  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-12 | pos1 | [8.2] |
|  | 8 |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-12 | pos1 | [-9.1] |
|  |  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-12 | pos1 | [2.9] |
|  |  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-12 | pos1 | [5.2] |
| 2 | 2 |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-26 | pos1 | [0.9] |
|  |  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-26 | pos1 | [18.1] |
|  | 4 |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-26 | pos1 | [-2.6] |
|  |  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-26 | pos1 | [10.9] |
|  | 8 |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-26 | pos1 | [-6.0] |
|  |  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-26 | pos1 | [6.5] |

Table 8.2.1.2-6: Minimum requirements for PUSCH, Type A, 40 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 |  | Normal |  | TDLB100-400 Low | 70 % |  | G-FR1-A3-13 |  | pos1 | [-2.8] |
|  |  |  | Normal |  | TDLC300-100 Low | 70 % |  | G-FR1-A4-13 |  | pos1 | [TBD] |
|  |  |  | Normal |  | TDLA30-10 Low | 70 % |  | G-FR1-A5-13 |  | pos1 | [11.7] |
|  | 4 |  | Normal |  | TDLB100-400 Low | 70 % |  | G-FR1-A3-13 |  | pos1 | [-6.1] |
|  |  |  | Normal |  | TDLC300-100 Low | 70 % |  | G-FR1-A4-13 |  | pos1 | [6.0] |
|  |  |  | Normal |  | TDLA30-10 Low | 70 % |  | G-FR1-A5-13 |  | pos1 | [8.3] |
|  | 8 |  | Normal |  | TDLB100-400 Low | 70 % |  | G-FR1-A3-13 |  | pos1 | [-9.0] |
|  |  |  | Normal |  | TDLC300-100 Low | 70 % |  | G-FR1-A4-13 |  | pos1 | [2.8] |
|  |  |  | Normal |  | TDLA30-10 Low | 70 % |  | G-FR1-A5-13 |  | pos1 | [5.1] |
| 2 | 2 |  | Normal |  | TDLB100-400 Low | 70 % |  | G-FR1-A3-27 |  | pos1 | [1.0] |
|  |  |  | Normal |  | TDLC300-100 Low | 70 % |  | G-FR1-A4-27 |  | pos1 | [18.1] |
|  | 4 |  | Normal |  | TDLB100-400 Low | 70 % |  | G-FR1-A3-27 |  | pos1 | [-2.6] |
|  |  |  | Normal |  | TDLC300-100 Low | 70 % |  | G-FR1-A4-27 |  | pos1 | [10.9] |
|  | 8 |  | Normal |  | TDLB100-400 Low | 70 % |  | G-FR1-A3-27 |  | pos1 | [-5.6] |
|  |  |  | Normal |  | TDLC300-100 Low | 70 % |  | G-FR1-A4-27 |  | pos1 | [6.6] |

Table 8.2.1.2-7: Minimum requirements for PUSCH, Type A, 100 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas |  | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-14 | pos1 | [-2.9] |
|  |  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-14 | pos1 | [10.2] |
|  |  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-14 | pos1 | [12.7] |
|  | 4 |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-14 | pos1 | [-6.0] |
|  |  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-14 | pos1 | [6.2] |
|  |  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-14 | pos1 | [8.7] |
|  | 8 |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-14 | pos1 | [-9.0] |
|  |  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-14 | pos1 | [2.9] |
|  |  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-14 | pos1 | [5.5] |
| 2 | 2 |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-28 | pos1 | [1.1] |
|  |  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-28 | pos1 | [18.6] |
|  | 4 |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-28 | pos1 | [-2.6] |
|  |  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-28 | pos1 | [11.3] |
|  | 8 |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-28 | pos1 | [-5.7] |
|  |  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-28 | pos1 | [6.7] |

Table 8.2.1.2-8: Minimum requirements for PUSCH, Type B, 5 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas |  | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-8 | pos1 | [-3.0] |
|  |  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-8 | pos1 | [9.9] |
|  |  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-8 | pos1 | [12.1] |
|  | 4 |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-8 | pos1 | [-6.3] |
|  |  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-8 | pos1 | [6.0] |
|  |  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-8 | pos1 | [8.5] |
|  | 8 |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-8 | pos1 | [-9.3] |
|  |  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-8 | pos1 | [2.6] |
|  |  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-8 | pos1 | [5.4] |
| 2 | 2 |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-22 | pos1 | [0.7] |
|  |  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-22 | pos1 | [17.9] |
|  | 4 |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-22 | pos1 | [-3.0] |
|  |  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-22 | pos1 | [10.7] |
|  | 8 |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-22 | pos1 | [-6.1] |
|  |  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-22 | pos1 | [6.4] |

Table 8.2.1.2-9: Minimum requirements for PUSCH, Type B, 10 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-9 | pos1 | [-2.6] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-9 | pos1 | [10.1] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-9 | pos1 | [12.5] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-9 | pos1 | [-6.4] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-9 | pos1 | [6.1] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-9 | pos1 | [8.5] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-9 | pos1 | [-9.4] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-9 | pos1 | [2.9] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-9 | pos1 | [5.5] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-23 | pos1 | [1.1] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-23 | pos1 | [18.7] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-23 | pos1 | [-2.7] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-23 | pos1 | [10.9] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-23 | pos1 | [-6.1] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-23 | pos1 | [6.6] |

Table 8.2.1.2-10: Minimum requirements for PUSCH, Type B, 20 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-10 | pos1 | [-2.7] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-10 | pos1 | [10.1] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-10 | pos1 | [12.2] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-10 | pos1 | [-6.1] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-10 | pos1 | [6.0] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-10 | pos1 | [8.5] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-10 | pos1 | [-9.1] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-10 | pos1 | [2.7] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-10 | pos1 | [5.3] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-24 | pos1 | [1.1] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-24 | pos1 | [18.2] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-24 | pos1 | [-2.4] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-24 | pos1 | [10.8] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-24 | pos1 | [-5.7] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-24 | pos1 | [6.5] |

Table 8.2.1.2-11: Minimum requirements for PUSCH, Type B, 10 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas |  | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-11 | pos1 | [-2.8] |
|  |  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-11 | pos1 | [10.1] |
|  |  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-11 | pos1 | [12.1] |
|  | 4 |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-11 | pos1 | [-6.1] |
|  |  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-11 | pos1 | [6.0] |
|  |  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-11 | pos1 | [8.3] |
|  | 8 |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-11 | pos1 | [-9.1] |
|  |  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-11 | pos1 | [2.9] |
|  |  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-11 | pos1 | [5.3] |
| 2 | 2 |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-25 | pos1 | [0.7] |
|  |  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-25 | pos1 | [18.1] |
|  | 4 |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-25 | pos1 | [-2.9] |
|  |  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-25 | pos1 | [10.9] |
|  | 8 |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-25 | pos1 | [-6.0] |
|  |  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-25 | pos1 | [6.6] |

Table 8.2.1.2-12: Minimum requirements for PUSCH, Type B, 20 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas |  | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-12 | pos1 | [-3.0] |
|  |  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-12 | pos1 | [10.1] |
|  |  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-12 | pos1 | [12.1] |
|  | 4 |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-12 | pos1 | [-6.3] |
|  |  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-12 | pos1 | [6.0] |
|  |  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-12 | pos1 | [8.3] |
|  | 8 |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-12 | pos1 | [-9.3] |
|  |  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-12 | pos1 | [2.8] |
|  |  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-12 | pos1 | [5.3] |
| 2 | 2 |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-26 | pos1 | [0.9] |
|  |  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-26 | pos1 | [18.2] |
|  | 4 |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-26 | pos1 | [-2.7] |
|  |  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-26 | pos1 | [10.8] |
|  | 8 |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-26 | pos1 | [-5.8] |
|  |  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-26 | pos1 | [6.6] |

Table 8.2.1.2-13: Minimum requirements for PUSCH, Type B, 40 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-13 | pos1 | [-2.8] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-13 | pos1 | [10.0] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-13 | pos1 | [12.2] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-13 | pos1 | [-6.1] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-13 | pos1 | [5.9] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-13 | pos1 | [8.3] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-13 | pos1 | [-9.1] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-13 | pos1 | [2.7] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-13 | pos1 | [5.2] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-27 | pos1 | [1.3] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-27 | pos1 | [18.3] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-27 | pos1 | [-2.5] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-27 | pos1 | [10.8] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-27 | pos1 | [-5.6] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-27 | pos1 | [6.5] |

Table 8.2.1.2-14: Minimum requirements for PUSCH, Type B, 100 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-14 | pos1 | [-2.8] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-14 | pos1 | [10.1] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-14 | pos1 | [12.8] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-14 | pos1 | [-6.1] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-14 | pos1 | [6.0] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-14 | pos1 | [8.6] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-14 | pos1 | [-9.1] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-14 | pos1 | [2.8] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-14 | pos1 | [5.5] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-28 | pos1 | [1.2] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-28 | pos1 | [18.8] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-28 | pos1 | [-2.6] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-28 | pos1 | [11.2] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-28 | pos1 | [-5.7] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-28 | pos1 | [6.7] |

## <<End of first change>>