3GPP TSG- RAN4 Meeting #92	R4-1907972

# Ljubljana, Slovenia, 26th - 30th August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0004 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Antenna configuration for LTE cell in EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-08-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For EN-DC demodulation test cases the LTE PCell setup currently specifies an antenna configuration of 2x2, regardless of the frequency range of the NR cell. However, the demodulation requirements differ depending on whether the NR cell is in FR1 or in FR2, taking SDR as an example: - In 9.4B.1.1, EN-DC within FR1, “During the test, the PDSCH performance on both the NR cell(s) and LTE cell(s) shall be verified” - In 9.4B.1.2, EN-DC including FR2 NR carrier, “During the test, only the PDSCH performance on the NR cell(s) on FR2 carriers is verified” EN-DC within FR1 will be a conducted test, and the LTE Reference channels include 2 layer MIMO, so 2x2 is reasonable and its SDR performance is part of the requirement. However EN-DC including FR2 NR carrier will be a radiated test, with the LTE cell providing a link only, and its performance is not part of the requirement so there is no need for more than 1 LTE layer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For EN-DC including FR2 NR carrier, specify LTE Pcell setup with antenna configuration 1 Tx and TM1, which provides 1 LTE layer. The , values are adjusted accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecesary requirements, not needed for the test purpose, would be placed on the test system. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Table 9.1.2.1-1, Table 9.1.2.2-1. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | Y |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

### 9.1.2	LTE Pcell setup

This sub-clause provides the parameters for LTE Pcell during the demodulation performance test for EN-DC unless otherwise stated. For EN-DC with multiple LTE carriers or bands, randomly selected one carrier or band that can be used for Pcell as LTE Pcell for the connection setup.

#### 9.1.2.1	FDD

The parameters specified in Table 9.1.2.1-1 and Table 9.1.2.1-2 are used to setup an LTE Pcell. One of test setup in Table 9.1.2.1-2 will be selected for the LTE Pcell depending on the maximum bandwidth of an LTE carrier for all the EN-DC band combinations supported by the UE.

The measurement channels in Table 9.1.2.1-2 and OCNG pattern OP.1 FDD are specified in TS 36.101 [4]. The physical channel setup with downlink power allocation is according to Annex C.3.2 of TS 36.101 [4].

Table 9.1.2.1-1: Common Test Parameters (FDD)

| Parameter | Unit | Value |
| --- | --- | --- |
| Cyclic prefix |  | Normal |
| Physical Cell ID |  | 0 |
| Number of PDCCH symbols | symbols | 1 |
| PHICH Ng (Note 1) |  | 1 |
| PHICH duration |  | Normal |
| Number of HARQ processes per component carrier | Processes | 8 |
| Maximum number of HARQ transmission |  | 4 |
| Redundancy version coding sequence |  | {0,0,1,2} for 64QAM |
| Propagation condition |  | Static propagation condition No external noise sources are applied |
| Transmission mode |  | All NR cells are in FR1: 3 |
|  |  | Any NR cells is in FR2: 1 |
| Transmission time difference between LTE cell and NR cell(s) | μs | 0 |
| Antenna configuration |  | All NR cells are in FR1: 2x2 |
|  |  | Any NR cells is in FR2: 1 TxNote 1 |
| Codebook subset restriction |  | 10 |
| Symbols for all unused Res |  | OCNG in Annex A.5 |
| Note 1: As the link can be provided over the air, the UE Rx antenna configuration is not relevant for the test configuration and has no impact on the test implementation. |  |  |

Table 9.1.2.1-2: Specific Test Parameters (FDD [64QAM])

|  | Test setup | Bandwidth (MHz) | Downlink power allocation (dB) |  |  |
| --- | --- | --- | --- | --- | --- |
| All NR cells are in FR1 | 1 | 5 | -3 | -3 | 0 |
|  | 2 | 10 | -3 | -3 | 0 |
|  | 3 | 15 | -3 | -3 | 0 |
|  | 4 | 20 | -3 | -3 | 0 |
| Any NR cells is in FR2 | 1 | 5 | 0 | 0 | 0 |
|  | 2 | 10 | 0 | 0 | 0 |
|  | 3 | 15 | 0 | 0 | 0 |
|  | 4 | 20 | 0 | 0 | 0 |

#### 9.1.2.2	TDD

The parameters specified in Table 9.1.2.2-1 and Table 9.1.2.2-2 are used to setup an LTE Pcell. One of test setup in Table 9.1.2.2-2 will be selected for the LTE Pcell depending on the maximum bandwidth of an LTE carrier for all the EN-DC band combinations supported by the UE.

The measurement channels in Table 9.1.2.2-2 and OCNG pattern OP.1 TDD are specified in TS 36.101 [4]. The physical channel setup with downlink power allocation is according to Annex C.3.2 of TS 36.101 [4].

Table 9.1.2.2-1: Common Test Parameters (TDD)

| Parameter | Unit | Value |
| --- | --- | --- |
| UL DL configuration |  | 2 (Note1) |
| Special subframe configuration |  | 7 |
| Number of PDCCH symbols | symbols | 1 |
| PHICH Ng (Note 3) |  | 1 |
| PHICH duration |  | Normal |
| Cyclic prefix |  | Normal |
| Cell ID |  | 0 |
| Maximum number of HARQ transmission |  | 4 |
| Redundancy version coding sequence |  | {0,0,1,2} for 64QAM |
| Propagation condition |  | Static propagation condition No external noise sources are applied |
| Transmission mode |  | All NR cells are in FR1: 3 |
|  |  | Any NR cells is in FR2: 1 |
| Transmission time difference between LTE cell and NR cell(s) | μs | 0 |
| Antenna configuration |  | All NR cells are in FR1: 2x2 |
|  |  | Any NR cells is in FR2: 1 TxNote 2 |
| Codebook subset restriction |  | 10 |
| Symbols for all unused Res |  | OCNG in Annex A.5 |
| NOTE 1: The start of transmission of LTE frame is delayed by 2 LTE subframes with respect to the start of transmission of NR frame when TDD-TDD EN-DC configuration is configured during the test. NOTE 2: As the link can be provided over the air, the UE Rx antenna configuration is not relevant for the test configuration and has no impact on the test implementation. |  |  |

Table 9.1.2.2-2: Specific Test Parameters (FDD 64QAM)

|  | Test setup | Bandwidth (MHz) | Downlink power allocation (dB) |  |  |
| --- | --- | --- | --- | --- | --- |
| All NR cells are in FR1 | 1 | 10 | -3 | -3 | 0 |
|  | 2 | 15 | -3 | -3 | 0 |
|  | 3 | 20 | -3 | -3 | 0 |
| Any NR cells is in FR2 | 1 | 10 | 0 | 0 | 0 |
|  | 2 | 15 | 0 | 0 | 0 |
|  | 3 | 20 | 0 | 0 | 0 |

<< End of changes >>