3GPP TSG-RAN WG4 Meeting #92                                         R4-1908076

Ljubljana,SI, 26th – 30th May, 2019

Agenda Item:   9.5.2

Source:        Samsung

Title:           Simulation result for layout 2

Document for:  Discussion

# Introduction

In RAN4#91 meeting simulation assumption was discussed in AH meeting [1] and agreed for IAB co-existence study in [2]. This contribution provides assessment results and observation based on those results for layout 2.

# Simulation assumptions

Same as layout 1 simulation, we use the ACLR and ACS assumption as table2 in [3] for scenario 1 of layout2. For scenario 2 of layout2 ACLR and ACS of IAB are assumed as 28B and 23 dB respectively. And the similar concern on distance between Micro and UE is applicable to this layout as well. Further detail can be found in companion contribution for layout 1 in [3].

According to the agreement in WF, layout 2 is homogenous layout. And only FR2 is assumed for layout2 for both scenario 1 and 2. To simplify the simulator, in our simulation up to 2hops from IAB to the only donor BS in the center is considered. For 6 sites in the cycle around center donor BS they can be assumed to communicate with donor directly. For the remaining 12 sites they have to access to central unit through IAB allocated in aforementioned

Since there are 3pannels assumed for simulation, only one panel is for purpose of IAB-MT. And for certain panel it is either operating as IAB-MT or IAB-DU but will not be configured for both purposes simultaneously for both scenarios. Hence for certain IAB site with 3panels, according to system topology there are up to 3 IAB-DU active or 2 IAB-DU+ 1IAB-MT active in 3cells in its coverage. Based on this assumption it is further divided the simulation to several mode with different active IAB and UE for TRX.

Table 1 active# for IAB and UE for scenario 1

| Mode | Active IAB number in 1st hop | Total IAB#(Rx in DL or Tx in UL ) | Total UE#(Rx in DL or Tx in UL) (up to 1 UE active in one cell ) | Total# active IAB and UE |
| --- | --- | --- | --- | --- |
| 0 | 0 | 12 | 9 | 21 |
| 1 | 1 | 11 | 13 | 24 |
| 2 | 2 | 10 | 17 | 27 |
| 3 | 3 | 9 | 21 | 30 |

Regarding the simulation case for scenario 1 in layout 2, there is slight difference compared with cases for layout1. Due to multiple hops considered, in UL transmission both IAB-MT and UE may be active. In addition, since there is strict constraint of half-duplexer in scenario 1, no simultaneous transmission

- Case 1: IAB-MT TX for ACLR

  - Baseline interference for victim system: co-channel interference with the same system, adjacent channel interference without IAB

  - New interference case: co-channel interference with the same system, IAB-MT TX interference &UE TX interference on adjacent channel.

- Case 2: IAB-MT RX for ACS

  - Baseline interference: co-channel interference from gNB TX, co-channel interference from IAB-DU TX

  - New interference case: co-channel interference from gNB TX, co-channel interference from IAB-DU TX, adjacent channel interference from aggressor system

Figure 1: case1 for scenario 1                                    Figure 2: case2 for scenario 1

For scenario2 of layout 2, the only donor gNB in the center is possible in either in RX or TX mode in DL time slot according to assumption. But if it is in TX (or RX) mode, it would be TX(or RX) in all 3sectors, and the same to IAB site on IAB-MT and IAB-DU. It should be emphasized that since the IAB-MT is assumed in DL time slot for both TX and RX, in the simulation UE will always work on RX mode if active. Especially for MT TX in DL time slot to derive the MT ACLR, the baseline interference for victim system would be co-channel interference with the same NR system and adjacent channel NR system interference without IAB. For the aggressor system there are totally 19 sites with 57 gNB nodes active. However, if 18 sites are replaced as IAB as DL time slot, there would be less than 57 cell active in TX mode. It can be estimated that the interference from adjacent channel to victim system would be less than baseline level. And for IAB MT as victim system since the adjacent channel is just normal NR operation with 57 gNB active in DL transmission, it is not expected significant performance degradation if ACS of IAB-MT on the same level of NR gNB and/or UE. Hence we just take one example case to check the performance. Here mode 3 in table 1 is taken for simulation. What’s needed to be clarified is since only DL considered here, three IAB MT panels are in RX mode in 1st hop or they are in TX mode. Then they are named as mode 3_1 and mode 3_2.

- Case 1: IAB as aggressor

  - Baseline interference for victim system: co-channel interference with the same system, adjacent channel interference without IAB

  - New interference case: co-channel interference with the same system, IAB-MT TX interference, IAB-DU TX interference and/or gNB TX interference on adjacent channel.

- Case 2: IAB system as victim

  - Baseline interference: co-channel interference from gNB TX(Not considered for mode 3_2), co-channel interference from IAB-MT TX &IAB-DU TX

  - New interference case: co-channel interference from gNB TX(Not considered for mode 3_2), co-channel interference from IAB-MT TX &IAB-DU TX, adjacent channel gNB interference from aggressor system

Figure 3: case1 for scenario 2                                                   Figure 4: case2 for scenario 2

# Simulation results

Table 1: Simulation results for scenario 1 case 1(UL)

| MT UL Power control | mode | ACLR and ACS (dB) | Average user throughput (Kbps) | Performance gain(%) | 5-tile edge user throughput (Kbps) | Performance gain(%) |
| --- | --- | --- | --- | --- | --- | --- |
| n/a | Baseline | UE-ACLR:17 NB-ACS:23 | 370796.945 | NA | 242803.110 | NA |
| PC ON | Mode 0 | IAB-ACLR:28 | 372185.417 | 0.37 | 248732.410 | 2.44 |
|  |  | IAB-ACLR:24 | 371692.049 | 0.24 | 248087.740 | 2.18 |
|  |  | IAB-ACLR:20 | 370600.468 | -0.05 | 245214.150 | 0.99 |
|  |  | IAB-ACLR:17 | 369181.624 | -0.44 | 234151.490 | -3.56 |
|  | Mode 1 | IAB-ACLR:28 | 371793.466 | 0.27 | 246985.390 | 1.72 |
|  |  | IAB-ACLR:24 | 371725.993 | 0.25 | 247958.150 | 2.12 |
|  |  | IAB-ACLR:20 | 370033.370 | -0.21 | 238520.440 | -1.76 |
|  |  | IAB-ACLR:17 | 369267.687 | -0.41 | 233478.990 | -3.84 |
|  | Mode 2 | IAB-ACLR:28 | 371851.175 | 0.28 | 248945.410 | 2.53 |
|  |  | IAB-ACLR:24 | 371714.491 | 0.25 | 250016.350 | 2.97 |
|  |  | IAB-ACLR:20 | 370254.190 | -0.15 | 239913.240 | -1.19 |
|  |  | IAB-ACLR:17 | 369123.841 | -0.45 | 235751.050 | -2.90 |
|  | Mode 3 | IAB-ACLR:28 | 371292.432 | 0.13 | 246555.740 | 1.55 |
|  |  | IAB-ACLR:24 | 371184.586 | 0.10 | 247481.500 | 1.93 |
|  |  | IAB-ACLR:20 | 370380.225 | -0.11 | 239546.580 | -1.34 |
|  |  | IAB-ACLR:17 | 369510.210 | -0.35 | 236196.990 | -2.72 |
| PC off | Mode 0 | IAB-ACLR:28 | 370093.249 | -0.19 | 240325.800 | -1.02 |
|  |  | IAB-ACLR:24 | 369169.111 | -0.44 | 235921.450 | -2.83 |
|  |  | IAB-ACLR:20 | 367001.875 | -1.02 | 227550.020 | -6.28 |
|  |  | IAB-ACLR:17 | 364566.022 | -1.68 | 213523.980 | -12.06 |
|  | Mode 1 | IAB-ACLR:28 | 369926.111 | -0.23 | 239251.030 | -1.46 |
|  |  | IAB-ACLR:24 | 369345.520 | -0.39 | 236338.320 | -2.66 |
|  |  | IAB-ACLR:20 | 366730.129 | -1.10 | 223311.910 | -8.03 |
|  |  | IAB-ACLR:17 | 365042.959 | -1.55 | 217130.500 | -10.57 |
|  | Mode 2 | IAB-ACLR:28 | 370036.845 | -0.20 | 241391.510 | -0.58 |
|  |  | IAB-ACLR:24 | 369580.162 | -0.33 | 239486.270 | -1.37 |
|  |  | IAB-ACLR:20 | 367288.379 | -0.95 | 226345.390 | -6.78 |
|  |  | IAB-ACLR:17 | 365144.005 | -1.52 | 216776.520 | -10.72 |
|  | Mode 3 | IAB-ACLR:28 | 369720.382 | -0.29 | 239842.230 | -1.22 |
|  |  | IAB-ACLR:24 | 369222.266 | -0.42 | 239545.890 | -1.34 |
|  |  | IAB-ACLR:20 | 367708.897 | -0.83 | 227946.650 | -6.12 |
|  |  | IAB-ACLR:17 | 366167.094 | -1.25 | 224111.080 | -7.70 |

Table 2: Simulation results for scenario 1 case 2(DL)

| NB-ACLR:28 IAB-ACS: 23 |  | Total users (IAB+UE) |  |  |  | IAB users |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Average user throughput (Kbps) | Performance gain(%) | 5-tile edge user throughput (Kbps) | Performance gain(%) | Average user throughput (Kbps) | Performance gain(%) | 5-tile edge user throughput (Kbps) | Performance gain(%) |
| Mode 0 | baseline | 730140.068 |  | 274514.930 |  | 490306.814 |  | 184134.070 |  |
|  | With aggressor | 724427.105 | -0.78 | 262787.030 | -4.27 | 488297.958 | -0.41 | 175666.510 | -4.60 |
| Mode 1 | baseline | 793722.937 |  | 305180.800 |  | 511791.237 |  | 181146.130 |  |
|  | With aggressor | 786793.460 | -0.87 | 293210.640 | -3.92 | 509330.917 | -0.48 | 175821.220 | -2.94 |
| Mode 2 | baseline | 837878.301 |  | 347828.790 |  | 525918.240 |  | 201799.140 |  |
|  | With aggressor | 830593.372 | -0.87 | 334271.660 | -3.90 | 523153.064 | -0.53 | 195222.550 | -3.26 |
| Mode 3 | baseline | 864946.918 |  | 375586.050 |  | 529128.485 |  | 227378.860 |  |
|  | With aggressor | 857227.260 | -0.89 | 363865.940 | -3.12 | 526209.980 | -0.55 | 219889.910 | -3.29 |

Table 3: Simulation results for scenario 2 case 1

|  | Average user throughput (Kbps) | Performance gain(%) | 5-tile edge user throughput (Kbps) | Performance gain(%) |
| --- | --- | --- | --- | --- |
| Baseline | 978350.450 |  | 562286.180 |  |
| Mode 3_1 | 982821.034 | 0.46 | 565975.510 | 0.66 |
| Mode 3_2 | 986222.322 | 0.80 | 575221.680 | 2.30 |

Table 4: Simulation results for scenario 2 case 2

|  | Total users (IAB+UE) |  |  |  | IAB users |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Average user throughput (Kbps) | Performance gain(%) | 5-tile edge user throughput (Kbps) | Performance gain(%) | Average user throughput (Kbps) | Performance gain(%) | 5-tile edge user throughput (Kbps) | Performance gain(%) |
| Mode 3_1 baseline | 803815.012 |  | 96486.580 |  | 471277.243 |  | 23295.190 |  |
| Mode 3_1 | 797039.425 | -0.8 | 95493.840 | -1.03 | 469196.625 | -0.44 | 23279.100 | -0.07 |
| Mode 3_2 baseline | 587517.901 |  | 146371.260 |  | 488359.412 |  | 120481.320 |  |
| Mode 3_2 | 584046.314 | -0.5 | 143635.640 | -1.87 | 486234.026 | -0.44 | 119523.160 | -0.80 |

# Summary

For layout2, in case of scenario 1 deployment, with power control the performance degradation is negligible even if IAB MT reuses the UE ACLR and ACS requirement.

For layout 2, in case of scenario 2 deployment, it is not expected co-existence issue.

# Reference

[1] R4-1907824 AH meeting minutes for IAB ad-hoc

[2] R4-1907825 WF on simulation assumption for IAB co-existence study

[3] R4-1908075  Simulation results for layout 1