3GPP TSG-RAN WG4 Meeting #92	R4-1908414

Ljubljana, Slovenia, 26th – 30th Aug. 2019

Source: 	CATT

Title: 	TP for TR 38.716-03-01: interference analysis for CA_n39A-n41A-n79A with 1 uplink

Agenda Item:	10.10.2

Document for:	Approval

Introduction

This contribution presents a Text proposal on interference analysis for CA_n39A-n41A-n79A with 1 uplink.

Discussion

It is proposed to approve the text proposal in the annex.

- References

[1]	RP-1908373, TR 38.716-03-01 v 0.2.0, CATT

- Text Proposal

<TP for TR 38.716-02-00>

- 6.x	CA_n39A-n41A-n79A

6.x.1	Operating bands for CA

Table 6.x.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n39-n41-n79 | n39 | 1880MHz | – | 1920MHz | 1880MHz | – | 1920MHz | TDD |
|  | n41 | 2496MHz | – | 2690MHz | 2496MHz | – | 2690MHz | TDD |
|  | n79 | 4400MHz | – | 5000MHz | 4400MHz | – | 5000MHz | TDD |

6.1.2	Channel bandwidths per operating band for CA

Table 6.x.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA Configuration | UL Config | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n39A-n41A-n79A | - | n39 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
| CA_n3A-n41A-n79A | - | n39 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  | 1 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in Table 5.3.5-1 of  TS 38.101-1 [3] and in Table 5.3.5-1 of TS 38.101-2 when operating in single carrier mode.

6.x.3	Co-existence studies

Table 6.x.3-1 summarizes frequency ranges where harmonics occur due to 3DL bands CA with 1 UL. It can be seen that the 2nd order harmonic of Band n41 will fall into a small portion of Band n79. However it is not a problem for this combination since the lower part of Band n41 will not be used in the region for this combination.

Table 6.x.3-1: Harmonic Interference for 3DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band |  | UL Low Band Edge |  | UL High Band Edge |  | DL Low Band Edge |  | DL High Band Edge |  | UL Low Band Edge |  | UL High Band Edge | UL Low Band Edge |  | UL High Band Edge | UL Low Band Edge |  | UL High Band Edge |
| n39 |  | 1880 |  | 1920 |  | 1880 |  | 1920 |  | 3760 |  | 3840 | 5640 |  | 5760 |  |  |  |
| n41 |  | 2496 |  | 2690 |  | 2496 |  | 2690 |  | 4992 |  | 5380 | 7488 |  | 8070 |  |  |  |
| n79 |  | 4400 |  | 5000 |  | 4400 |  | 5000 |  | 8800 |  | 10000 | 1320 |  | 15000 |  |  |  |

Table 6.x.3-2 gives harmonic mixing issue for CA with Band n39, n41 and n79. It is seen that a small part of Band n79 Tx frequency coincides with Band n41x2.

Table 6.x.3-2 Harmonic mixing for 3DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n39 | 1880 | 1920 | 1880 | 1920 | 3760 | 3840 | 5640 | 5760 |  |  |
| n41 | 2496 | 2690 | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 |  |  |
| n79 | 4400 | 5000 | 4400 | 5000 | 8800 | 10000 | 1320 | 15000 |  |  |

For single uplink, the UE coexistence requirement is already considered for these bands in TS 38.101-1 [3].

6.x.4	∆TIB,c and ∆RIB,c values

For three simultaneous DLs and one UL of Band n39, n41 and n79, the TIB,c and RIB,c  values are shown in table 6.x.4-1 and  table 6.x.4-2, respectively.

Table 6.x.4-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_39-41-79 | X |  |
|  | Y |  |
|  | Z |  |

Table 6.x.4-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_39-41-79 | X |  |
|  | Y |  |
|  | Z |  |

6.x.5	REFSENS requirements

<Text will be added>

…..

<End of TP >