3GPP TSG-RAN WG4 Meeting #92	R4-190xxxx

Ljubljana, Slovenia, 26th– 30th August, 2019

Source: 	Huawei, HiSilicon

Title:	Applicability of requirements and test configurations

Agenda Item:	12.1.2

Document for:	Approval

1	Introduction

In last RAN plenary meeting #83, a new WID [1] was approved to introduce to the MSR specifications support for [GSM+E-UTRA+NR] and [UTRA+E-UTRA+NR] and [GSM+UTRA+E-UTRA+NR]. For conformance part, the following objectives are identified.

- Update the conformance specification 37.141 to incorporate new CS(s) and associated test combinations

- Update the conformance specification 37.141 test requirements as appropriate

In this contribution we provide some consideration on applicability of requirements and test configurations.

2	Discussion

2 CS and associated supported configurations are proposed in [2], as shown below.

| Capability Set supported by the BS | CS18 | CS19 |
| --- | --- | --- |
| Supported RATs | GSM, E-UTRA3, NR | UTRA, E-UTRA3, NR |
| Supported configurations | SR E-UTRA3 (SC, MC, CA) SR NR (SC, MC, CA) MR GSM + E-UTRA3 MR GSM + NR MR E-UTRA3 + NR MR GSM+ E-UTRA3 + NR | SR UTRA (SC, MC) SR E-UTRA3 (SC, MC, CA) SR NR (SC, MC, CA) MR UTRA + E-UTRA3 MR UTRA + NR MR E-UTRA3 + NR MR UTRA + E-UTRA3 + NR |
| Applicable BC | BC2 | BC1, BC2 |
| NOTE 1: MC denotes multi-carrier in single RAT; SC denotes single carrier; MR denotes multi-RAT; SR denotes single-RAT. NOTE 2: For this configuration related to BC2 bands, the support of UTRA in band 3 is declared by the manufacturer. NOTE 3: Includes optional (declared by the manufacturer) support of NB-IoT in-band and/or NB-IoT guard band operation. |  |  |

For the test of those new CSs, new test configurations need to be considered. In order to keep the number of test configurations as small as possible, the test configuration should be generated with the max capability on the supported RATs. According to per antenna connector, the max capability on supported RATs for CS18 is MR UTRA + E-UTRA + NR, and for CS19 it is MR GSM + E-UTRA + NR. To minimize number of new TCs, as we did for CS 16, TC21 for contiguous operation and NTC21 for non-contiguous operation are constructed for conformance testing, the same approach can be reused. Hence it is proposed,

Proposal 1: for CS 18 MR operation, TC23 for contiguous operation and NTC23 for non-contiguous operation are constructed for conformance testing

Proposal 2: for CS 19 MR operation, TC24 for contiguous operation and NTC24 for non-contiguous operation are constructed for conformance testing

The test configurations for CS 18 and 19 can be found in Table 2-1. It is noted that some single RAT GSM/EDGE tests are not necessary since single RAT GSM/EDGE is not included in the CS.

Table 2-1: Test configurations for capability sets (CS18-19) for Multi-RAT capable BS

| Capability Set | GSM + E-UTRA3 + NR (CS 18) | UTRA + E-UTRA3 + NR (CS 19) |
| --- | --- | --- |
| BS test case | BC2 | BC1 and BC2 |
| 6.2 Base Station output power | - | - |
| Base Station maximum output power | C, NI, NG: TC23 CNC, NCNI, NCNG: NTC23 C/NC, C/NCNI, C/NCNG: NTC23, TC23 | C, NI, NG: TC24 CNC, NCNI, NCNG: NTC24 C/NC, C/NCNI, C/NCNG: NTC24, TC24 |
| E-UTRA for DL RS power | (TS 36.141) | (TS 36.141) |
| NB-IoT for DL RS power | (TS 36.141) | (TS 36.141) |
| UTRA FDD primary CPICH power | N/A | (TS 25.141) |
| UTRA FDD secondary CPICH power | N/A | (TS 25.141) |
| 6.3 Output power dynamics | - | - |
| E-UTRA | (TS 36.141) | (TS 36.141) |
| NB-IoT | (TS 36.141) | (TS 36.141) |
| NR | (TS 38.141-1) | (TS 38.141-1) |
| UTRA FDD | N/A | (TS 25.141) |
| 6.4 Transmit ON/OFF power | - | - |
| Transmitter OFF power | N/A | N/A |
| Transmitter transient period | N/A | N/A |
| 6.5 Transmitted signal quality | - | - |
| 6.5.1 Modulation quality | - | - |
| E-UTRA | C: TC23 NI, NG: (Note 4) CNC: TC23 NCNI, NCNG: (Note 4) C/NC: NTC23, TC23 C/NCNI, C/NCNG: (Note 4) | C: TC24 NI, NG: (Note 4) CNC: TC24 NCNI, NCNG: (Note 4) C/NC: NTC24, TC24 C/NCNI, C/NCNG: (Note 4) |
| NB-IoT | N/A (Note 4) | N/A (Note 4) |
| NR | C: TC23 CNC: TC23 C/NC: NTC23, TC23 | C: TC24 CNC: TC24 C/NC: NTC24, TC24 |
| UTRA FDD | N/A | C: TC24 CNC: TC24 C/NC: NTC24, TC24 |
| GSM/EDGE | C: TC23 CNC: TC23 C/NC: NTC23, TC23 | N/A |
| 6.5.2 Frequency error | - | - |
| E-UTRA | Same TC as 6.5.1 | Same TC as 6.5.1 |
| NB-IoT | N/A (Note 4) | N/A (Note 4) |
| NR | Same TC as 6.5.1 | Same TC as 6.5.1 |
| UTRA FDD | N/A | Same TC as 6.5.1 |
| GSM/EDGE | Same TC as 6.5.1 | N/A |
| 6.5.3 Time alignment error | - | - |
| E-UTRA | (TS 36.141) NI, NG: (Note 4) NCNI, NCNG: (Note 4) C/NCNI, C/NCNG: (Note 4) | (TS 36.141) NI, NG: (Note 4) NCNI, NCNG: (Note 4) C/NCNI, C/NCNG: (Note 4) |
| NB-IoT | N/A (Note 4) | N/A (Note 4) |
| NR | (TS 38.141-1) | (TS 38.141-1) |
| UTRA FDD | N/A | (TS 25.141) |
| 6.6 Unwanted emissions | - | - |
| 6.6.1 Transmitter spurious emissions | - | - |
| (Category A) | C, NI, NG: TC23 CNC, NCNI, NCNG: NTC23 C/NC, C/NCNI, C/NCNG: NTC23, TC23 | C, NI, NG: TC24 CNC, NCNI, NCNG: NTC24 C/NC, C/NCNI, C/NCNG: NTC24, TC24 |
| (Category B) | C, NI, NG: TC23 CNC, NCNI, NCNG: NTC23 C/NC, C/NCNI, C/NCNG: NTC23, TC23 | C, NI, NG: TC24 CNC, NCNI, NCNG: NTC24 C/NC, C/NCNI, C/NCNG: NTC24, TC24 |
| Protection of the BS receiver of own or different BS | C, NI, NG: TC23 CNC, NCNI, NCNG: NTC23 C/NC, C/NCNI, C/NCNG: NTC23, TC23 | C, NI, NG: TC24 CNC, NCNI, NCNG: NTC24 C/NC, C/NCNI, C/NCNG: NTC24, TC24 |
| Additional spurious emissions requirements | C, NI, NG: TC23 CNC, NCNI, NCNG: NTC23 C/NC, C/NCNI, C/NCNG: NTC23, TC23 | C, NI, NG: TC24 CNC, NCNI, NCNG: NTC24 C/NC, C/NCNI, C/NCNG: NTC24, TC24 |
| Co-location with other Base Stations | C, NI, NG: TC23 CNC, NCNI, NCNG: NTC23 C/NC, C/NCNI, C/NCNG: NTC23, TC23 | C, NI, NG: TC24 CNC, NCNI, NCNG: NTC24 C/NC, C/NCNI, C/NCNG: NTC24, TC24 |
| 6.6.2 Operating band unwanted emissions | - | - |
| General requirement for Band Categories 1 and 3 | N/A | C, NI, NG: TC24 CNC, NCNI, NCNG: NTC24 C/NC, C/NCNI, C/NCNG: NTC24, TC24 SC: (Note 3) |
| General requirement for Band Category 2 | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 SC: (Note 3) | N/A |
| Additional requirements | Compliance stated by manufacturer declaration | Compliance stated by manufacturer declaration |
| 6.6.3 Occupied bandwidth | - | - |
| Minimum requirement | (TS 36.141) (TS 38.141-1) | (TS 25.141) (TS 36.141) (TS 38.141-1) |
| 6.6.4 Adjacent Channel Leakage power Ratio (ACLR) | - | - |
| E- UTRA | C: TC23 CNC: NTC23 C/NC: NTC23, TC23 | C: TC24 CNC: NTC24 C/NC: NTC24, TC24 |
| NB-IoT | NI: TC23 NG: TC23 NCNI: NTC23 NCNG: NTC23 C/NCNI, C/NCNG: NTC23, TC23 | NI: TC24 NG: TC24 NCNI: NTC24 NCNG: NTC24 C/NCNI, C/NCNG: NTC24, TC24 |
| NR | C: TC23 CNC: NTC23 C/NC: NTC23, TC23 | C: TC24 CNC: NTC24 C/NC: NTC24, TC24 |
| UTRA FDD | N/A | C: TC24 CNC: NTC24 C/NC: NTC24, TC24 |
| Cumulative ACLR | CNC: NTC23 C/NC: NTC23 | CNC: NTC24 C/NC: NTC24 |
| 6.7 Transmitter intermodulation | - | - |
| General requirement | Same TC as used in 6.6 | Same TC as used in 6.6 |
| Additional requirement (BC1 and BC2) | CNC: NTC23 C/NC: NTC23 | CNC: NTC24 C/NC: NTC24 |
| Additional requirement (BC3) | N/A | N/A |
| 7.2 Reference sensitivity level | - | - |
| E-UTRA | (TS 36.141) | (TS 36.141) |
| NB-IoT | (TS 36.141) | (TS 36.141) |
| NR | (TS 38.141-1) | (TS 38.141-1) |
| UTRA FDD | N/A | (TS 25.141) |
| GSM/EDGE | (TS 51.021) | N/A |
| 7.3 Dynamic range |  |  |
| E-UTRA | (TS 36.141) | (TS 36.141) |
| NB-IoT | (TS 36.141) | (TS 36.141) |
| NR | (TS 38.141-1) | (TS 38.141-1) |
| UTRA FDD | N/A | (TS 25.141) |
| 7.4 In- band selectivity and blocking | - | - |
| General blocking requirement | C, NI, NG: TC23 CNC, NCNI, NCNG: NTC23 C/NC, C/NCNI, C/NCNG: NTC23, TC23 | C, NI, NG: TC24 CNC, NCNI, NCNG: NTC24 C/NC, C/NCNI, C/NCNG: NTC24, TC24 |
| General narrowband blocking requirement | C, NI, NG: TC23 CNC, NCNI, NCNG: NTC23 C/NC, C/NCNI, C/NCNG: NTC23, TC23 | C, NI, NG: TC24 CNC, NCNI, NCNG: NTC24 C/NC, C/NCNI, C/NCNG: NTC24, TC24 |
| Additional BC3 blocking requirement | N/A | C, NI, NG: TC24 CNC, NCNI, NCNG: NTC24 C/NC, C/NCNI, C/NCNG: NTC24, TC24 |
| 7.5 Out-of-band blocking | - | - |
| General requirement | C, NI, NG: TC23 CNC, NCNI, NCNG: NTC23 C/NC, C/NCNI, C/NCNG: NTC23, TC23 | C, NI, NG: TC24 CNC, NCNI, NCNG: NTC24 C/NC, C/NCNI, C/NCNG: NTC24, TC24 |
| Co-location requirement | C, NI, NG: TC23 CNC, NCNI, NCNG: NTC23 C/NC, C/NCNI, C/NCNG: NTC23, TC23 | C, NI, NG: TC24 CNC, NCNI, NCNG: NTC24 C/NC, C/NCNI, C/NCNG: NTC24, TC24 |
| 7.6 Receiver spurious emissions | - | - |
| General requirement | C, NI, NG: TC23 CNC, NCNI, NCNG: NTC23 C/NC, C/NCNI, C/NCNG: NTC23, TC23 | C, NI, NG: TC24 CNC, NCNI, NCNG: NTC24 C/NC, C/NCNI, C/NCNG: NTC24, TC24 |
| Additional requirement for BC2 (Category B) | C, NI, NG: TC23 CNC, NCNI, NCNG: NTC23 C/NC, C/NCNI, C/NCNG: NTC23, TC23 | C, NI, NG: TC24 CNC, NCNI, NCNG: NTC24 C/NC, C/NCNI, C/NCNG: NTC24, TC24 |
| 7.7 Receiver intermodulation | - | - |
| General intermodulation requirement | C, NI, NG: TC23 CNC, NCNI, NCNG: NTC23 C/NC, C/NCNI, C/NCNG: NTC23, TC23 | C, NI, NG: TC24 CNC, NCNI, NCNG: NTC24 C/NC, C/NCNI, C/NCNG: NTC24, TC24 |
| General narrowband intermodulation requirement | C, NI, NG: TC23 CNC, NCNI, NCNG: NTC23 C/NC, C/NCNI, C/NCNG: NTC23, TC23 | C, NI, NG: TC24 CNC, NCNI, NCNG: NTC24 C/NC, C/NCNI, C/NCNG: NTC24, TC24 |
| 7.8 In-channel selectivity | - | - |
| E-UTRA requirement | (TS 36.141) | (TS 36.141) |
| NB-IoT | (TS 36.141) | (TS 36.141) |
| NR | (TS 38.141-1) | (TS 38.141-1) |

Proposal 3: It is proposed that the test configurations for each RF requirement for CS 18 and 19 is defined in Table 2-1.

3	Conclusions

In this contribution we provide discussion on test aspects on MSR new CS.

Proposal 1: for CS 18 MR operation, TC23 for contiguous operation and NTC23 for non-contiguous operation are constructed for conformance testing

Proposal 2: for CS 19 MR operation, TC24 for contiguous operation and NTC24 for non-contiguous operation are constructed for conformance testing

Proposal 3: It is proposed that the test configurations for each RF requirement for CS 18 and 19 is defined in Table 2-1.

4	References

[1] RP-190642, New WID on introduction of GSM, UTRA, E-UTRA and NR capability set(s) (CS(s)) to the multi-standard radio (MSR) specifications

[2] R4-1905571, Options on MSR new capability sets, Huawei