3GPP TSG-RAN WG4 Meeting #92	R4-1909605

# Ljubljana, Slovenia, August 26 – 30, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR 38.133 (8.10.2) TCI state known condition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-core |  |  |  |  |  | Date: |  |  | 2019-08-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Condition for known TCI state is incomplete as the time between last provided resources for measurements of the target TCI state and the command that orders switching to the target TCI state, is not defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Defining a maximum time between resources for measurements of, and command for switching to, the target TCI state, where the time depends on whether the target TCI state is in the list of active TCI states for PDSCH. The rationale is that when the TCI state is not in the active TCI state list for PDSCH, additional time for updating control loops is allowed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Condition for known TCI state will remain undefined and performance for TCI state switching cannot be guaranteed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.10.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Unchanged Sections Omitted

First Modification

### 8.10.2	Known conditions for TCI state

The TCI state is known if it has been meeting the following conditions:

-	TCI state switch is within: 
-	for TCI state in active TCI state list for PDSCH: [160] ms
-	for TCI state not in the active TCI state list for PDSCH: 
	-	[4]s for devices supporting power class 1
	- 	[3]s for devices supporting power class 2/3/4
of last transmission of the resource for beam reporting/measurement for the target TCI state

-	The UE has sent at least 1 measurement report for the target TCI state

-	The TCI state shall remain detectable during the TCI state switching period

-	SNR of the reference signal(s) associated with the TCI state is > -3dB

Otherwise, the TCI state is unknown.

End of First Modification

Unchanged Sections Omitted