3GPP TSG-RAN WG4 Meeting #92	R4-1909805

Ljubljana, Slovenia, 26 August – 30 August 2019

Source:	Ericsson, T-Mobile US

Title:	TP to TR 37.716-21-21: Addition of EN-DC configuration DC_66A_n41A-n260A, DC_66A_n41A-n260(2A), DC_66A_n41A-n260(3A) and DC_66A_n41A-n260(4A)

Agenda item:	10.8.3

Document for:	Approval

# Background

This contribution provides text proposal on DC_66A_n41A-n260A, DC_66A_n41A-n260(2A), DC_66A_n41A-n260(3A) and DC_66A_n41A-n260(4A) as defined in the revised WID [1].

# Text Proposal

##### ---Start of changes---

## 6.x	DC_66_n41-n260

### 6.x.1	Operating bands for DC

Table 6.x.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n41-n260 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

### 6.x.2	Channel bandwidths per operating band for DC

Table 6.x.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_66A_n41A-n260A | DC_66A_n41A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 520 |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n260 | 60 |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |
| DC_66A_n41A-n260(66A) | DC_66A_n41A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 920 |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n260 | See CA_n260(2A) in Table 5.5A.2-1 of TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_66A_n41A-n260(3A) | DC_66A_n41A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 1320 |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n260 | See CA_n260(3A) in Table 5.5A.2-1 of TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_66A_n41A- n260(4A) | DC_66A_n41A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 1720 |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n260 | See CA_n260(4A) in Table 5.5A.2-1 of TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.x.3	Co-existence studies

Co-existence analysis for DC_66_n41 shows that there is no impact from DC_66_n41 UL to Band n260 DL.

Co-existence analysis for DC_66_n260 shows that there is no impact from DC_66_n260 UL to Band n41 DL.

### 6.x.4	∆TIB and ∆RIB values

For DC_66_n41-n260 the TIB,c and RIB,c values are given in the tables below.

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n41-n260 | 66 | 0.5 |
|  | n41 | 0.81 |
|  |  | 1.32 |
|  | n260 | 0 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545 MHz. |  |  |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n41-n260 | 66 | 0.5 |
|  | n41 | 0.51 |
|  |  | 12 |
|  | n260 | 0 |

### 6.x.5	MSD

No further MSD is needed.

##### ---End of changes---

# Reference

[1] 		RP-190949, “Revised WID on EN-DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL)”, LG Electronics