3GPP TSG-RAN WG4> Meeting # 92	R4-1909893

# LjubljanaNevada, Slovenia,  26 - 30 August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.843 | CR | 0030 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TR 37.843: adding background information on in-band unwanted emissions using beam-based directions in subclause 10.8.3.1.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-perf |  |  |  |  |  | Date: |  |  | 26-08-2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The applicability of the beam-based directions procedure to compute a TRP estimate of unwanted emisisons is still an open issue as outlined in the way forward on TRP measurements (R4-1902272). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding background information on the use of beam-based procedure for in-band unwanted emissions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Background information is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.8.3.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

##### 10.8.3.1.4	Beam-based directions

This method is only applicable if directivity of EUT antenna is known for the operating frequency. For the Rel-13 AAS BS EIRP accuracy requirement, the peak EIRP of a beam is measured at the beam peak direction within the beam direction pair. Following the same approach, the peak EIRP of a beam can be obtained, which is used to derive TRP estimate using directivity of EUT antenna as follows:

TRPEstimate=EIRPpeakDEUT

, where DEUT is the directivity of EUT.

For the case of OTA BS output power, the directivity (DEUT,BSoutputpower ) shall be defined as:

DEUT,BSOutputpower= Prated,c,EIRPPrated,c,TRP

Unlike radiation patterns of the operating frequency (or wanted signal), radiation patterns of in-band unwanted emissions (which include ACLR and OBUE) are generally unknown for AAS BS. This is because the unwanted emissions radiation pattern may be design dependent and varied from one implementation to another. The beam-based directions procedure assumes the radiation pattern of in-band unwanted emissions is correlated with that of the operating frequency. Thus, the directivity of antenna in an in-band neighbouring frequency is considered to be equivalent to that of the operating frequency. Consequently, the beam-based directions procedure will yield a correct TRP estimate if the radiation pattern of the in-band neighbouring frequency is correlated with that of the operating frequency, otherwise it gives a wrong output. An example of two different radiation patterns for the same in-band neighbouring frequency (fn= 37 GHz) is shown in figure 10.8.3.1.4-1. The two radiation patterns were generated using a 16×16 uniform rectangular antenna array. The directivty on the left plot is 29.7 dB while the one on the right is -7.1 dB. In this example, the radiation pattern on the left plot is assumed to be correlated with that of the operating frequency. Hence, the beam-based directions procedure would yield a correct TRP estimate but not for the plot on the right due to dissimilar directivity values. The difference in directivity between the two radiation patterns is 36.8 dB.

Figure 10.8.3.1.4-1: Examples of different power radiation patterns

Based on the discussion above, a test methodology is needed to demonstrate whether the radiation pattern of in-band neighbouring frequency is correlated with that of the frequency in operation prior to using the procedure. The grid-based TRP methods specified in subclause 10.8.2 can be used for such a purpose but this would nullify the benefits of the procedure. Therefore, the test methodology should only need the minimum number of measurements to be carried out for demonstrating correlation in the radiation pattern between an in-band neighbouring frequency and the operating frequency. In addition, the test methodology should also deal with the directivity in different in-band neighbouring frequencies. Even though, correlation is successfully demonstrated, the directivity of antenna is frequency dependent. This means, there could be a difference in the directivity between the frequency in the centre and those near the band edge for a wide operating band such as FR2.

Correlation is a measure of how much the radiation pattern in an in-band neighbouring frequency resembles that of the frequency in operation. If correlation exists, it can be assumed that the maximum radiation (peak EIRP) of the in-band neighbouring frequency occurs in the same direction as that of the operating frequency. Furthermore, it is assumed that side lobes of the in-band unwanted emissions (neighbouring frequencies) are relatively low compared with the main lobe; therefore, they can be neglected.

Based on the above assumptions, it is sufficient to demonstrate if correlation exists by correlating the main lobe of an in-band neighbouring frequency with that of the operating frequency instead of complete radiation patterns. Hence, the following criteria can be established for the test methodology:

- The main lobe of the operating and in-band neighbouring frequencies with respect to the axis of maximum radiation (peak EIRP) should have similar symmetry.

- Measured azimuth and elevation HPBWs for an in-band neighbouring frequency should match the ones derived from the HPBWs of the operating frequency.

- The directivity-beamwidth product for an in-band neighbouring frequency should correspond to the directivity-beamwidth product for the operating frequency.

To determine if two main lobes are symmetrical, azimuth (ϕr) and elevation (θr) half-power angular ratios are defined as

ϕr= ϕAϕB  and θr= θAθB

where ϕA=ϕmax-ϕ1, ϕB=ϕmax-ϕ2, θA=θmax-θ1	and θB=θmax-θ2.

Refer to figures 10.8.3.1.4-2 and 10.8.3.1.4-2 for more details.

If the azimuth and elevation half-power angular ratios for an in-band neighbouring frequency match those for the operating frequency, then the main lobe of the operating and in-band neighbouring frequencies has similar symmetry.

The azimuth (ϕHPBW) and elevation (θHPBW) HPBWs for in-band neighbouring frequency fn can be expressed in terms of the corresponding HPBWs at the operating frequency fo as

ϕHPBWfn=λnλoϕHPBWfo=fofnϕHPBWfo and θHPBWfn=λnλoθHPBWfo=fofnθHPBWfo

The measured azimuth and elevation HPBWs and those derived above should correspond.

Similar to the HPBW, the directivity for an in-band neighbouring frequency fn can be expressed in terms of the directivity of the operating frequency fo as

DEUTfn=λoλn2DEUTfo=fnfo2DEUTfo

where DEUT is in linear units.

The directivity-beamwidth product for an in-band neightbouring frequency fn is calculated as the directivity derived above and the measured azimuth and elevation HPBWs, which should correspond to that for the operating frequency

If any of the above is not met, it can be concluded that the radiation pattern for an in-band neighbouring frequency is not correlated with that of the operating frequency.

Figure 10.8.3.1.4-2: Two-dimensional radiation pattern on the spherical azimuth plane

Figure 10.8.3.1.4-3: Two-dimensional radiation pattern on the spherical elevation plane

<End of modified section>