3GPP TSG-RAN WG4 Meeting #92	R4-1910393

Ljubljana, Slovenia, 26th – 30th Aug. 2019

Title: 	TP for band and bandwidth for NR V2X

Source: 	CATT

Agenda item:	9.4.1

Document for:	Approval

- Introduction

In the last RAN4 meeting, the operating band for NR V2X has been approved [1]. It should be captured in TR38.886.

The RF requirements for NR V2X are discussed in [2], and bandwidth of 10MHz, 20MHz and 40MHz are applied for NR V2X. The bandwidths should be captured in TR38.886.

The text proposals are attached in section 3 in this document.

- Reference

[1]	R4-1906943, On Band Definition of NR V2X, Qualcomm

[2]	R4-1908377, Discussion on NR V2X UE RF requirements, CATT

- Annex: TP for TR38.886 for band and bandwidth for NR V2X

7.1	Operating bands

7.1.1	Operating bands in FR1

NR V2X communication is designed to operate in the operating bands in FR1 defined in Table 7.1.1-1.

Table 7.1.1-1 NR V2X operating bands in FR1

| NR V2X Operating Band | Sidelink (SL) Transmission operating band |  |  | Sidelink (SL) Reception operating band |  |  | Duplex Mode | Interface |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |  |
| n47 | 5855 MHz | - | 5925 MHz | 5855 MHz | - | 5925 MHz | TDD | PC5 |

7.1.2	Operating bands in FR2

7.2	Channel bandwidth

7.2.1	Channel bandwidth in FR1

The NR V2X Communication channel bandwidths and operating bands are shown in Table 7.2.1-1. The same (symmetrical) channel bandwidth is specified for both the TX and RX path.

Table 7.2.1-1 NR V2X Communication channel bandwidth

|  | NR V2X band / V2X channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR V2X Operating Band | 10 MHz | 20 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| 47 | Yes | Yes |  | Yes |  |  |  |  |  |

7.2.2	Channel bandwidth in FR2