3GPP TSG RAN WG4 Meeting RAN4 #92-Bis                                                       R4-1910916

Chongqing, China, October 14th – 20th, 2019

Agenda item: 8.7.3

Source: MediaTek Inc.

Title: Discussion on RRM measurement relaxation for RRC IDLE/INACTIVE

Document for: Discussion and Decision

# 1	Introduction

This tdoc focus on the discussions for RRM measurement relaxation for RRC IDLE/INACTIVE based on power saving gain analysis.

# 2	NR UE Idle Mode Power Analysis

To determine the power saving benefit for RRM measurement relaxation for RRC IDLE, we first analyze the power consumption for NR UE in idle mode based on the power value given in [1]. Without loss of generality, the following UE behavior in on IDRX cycle and NW setting is assumed:

- Before paging occasion, two SSB bursts are used for intra-frequency measurement and time/frequency sync for paging reception. After paging occasion, one SMTC window is used for inter-frequency measurement.

- For inter-frequency measurement, RAN4 does not have any requirement on the timing alignment, so UE needs to receive the whole 5ms SMTC window.

- Only one SSB is transmitted in each SSB burst for a one-beam FR1 deployment scenario

- Different timing offsets are considered between the TDM-ed paging occasion (PO) and SSB

The R15 baseline UE behavior is shown below in Figure 1. The normalized UE power consumption in one I-DRX cycle PNR_IDLE can be calculate based on UE processing timeline show in Figure 1 and the power model given in [1]. The detailed calculation formula is put in the Appendix (4.4).

Figure 1: NR Idle mode UE processing timeline

One extremely-relaxed case is that UE never performs neighboring cell measurement, e.g., in each IDRX cycle, the intra-frequency neighbor cell measurement (intraF-RRM) and inter-frequency neighbor cell measurement (interF-RRM) are skipped. Note that UE still has to do the serving cell measurement (Serv-RRM). An illustration to show the power consumption in this extremely-relaxed case is shown below in Figure 2 for two different paging occasions.

Figure 2: NR Idle mode normal RRM measurement v.s. extremely-relaxed RRM measurement (for two different paging occasions)

Based on Figure 2, it can be seen that the extremely-relaxed RRM measurement saves the following power components:

- Inter-frequency neighbor cell measurement

- Intra-frequency neighbor cell measurement (minus serving cell measurement)

- RRM measurement induced sleep power

The detailed calculation formula can be found in the Appendix (4.5).

With the analysis shown above, the power saving gain of the extremely-relaxed RRM measurement for different paging occasions could be calculated as in Figure 3:

Figure 2: Power saving gain of extremely-relaxed RRM measurement for different paging occasions

With above analysis, we are ready to evaluate the power saving gain for different measurement relaxation factors through interpolation. The averaged power saving gain equals 20.43% if we always perform extremely-relaxed RRM measurement. In contrast, if normal RRM measurement (as shown in Figure 1) is always performed, the power saving gain equals 0. If IDLE mode measurement period is relaxed by a factor of 2, meaning normal RRM measurement and relaxed RRM measurement are performed alternatively, the power saving gain equals 10.22%. Table 1 shows the power saving gain for different measurement period extensions.

Table I: Power saving gain

|  | IDLE measurement relaxation factors |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 |
| Power saving gain | 0 | 10.22% | 15.32% | 17.88% |

Observation 1: Extending the measurement period provides more than 10% power saving gain.

Proposal 1: The measurement period can be extended to 2/4/8 times when the measurement relaxation condition (define by RAN2) is satisfied to achieve UE power saving.

# 3	Summary

This tdoc focus on the discussions for RRM measurement relaxation for RRC IDLE/INACTIVE based on power saving gain analysis and have the following observations and proposals:

Observation 1: Extending the measurement period provides more than 10% power saving gain.

Proposal 1: The measurement period can be extended to 2/4/8 times when the measurement relaxation condition (define by RAN2) is satisfied to achieve UE power saving.

# 4	Appendix

Assumptions on the time durations, power, and energy values derived based on [1] are summarized as follows:

4.1 Assumptions on time durations (with 30KHz subcarrier spacing)

| Parameters | Time duration (ms) |
| --- | --- |
| NR paging occasion reception duration | TPOR = 0.5 |
| NR SSB burst duration in SMTC | TSSB = 0.5 |
| NR SMTC period | TSP = 20 |
| IDRX cycle length | TICYC = 1280 |

4.2 Assumptions on power consumption

(Normalized w.r.t. deep sleep power)

AGC processing power = 100

Paging processing = PDCCH-only processing = 100

Power scaling with bandwidth in idle mode = 0.5

| Deep sleep (PDS) | Light sleep (PLS) | SSB processing for RRM (PSSB) | CDRX_ON/IDRX_ON processing (PCDON/PIDON) |
| --- | --- | --- | --- |
| 1 | 20 | 150 | 100/50 |

Note: PCDON is assumed to operate in 100MHz BW while PIDON is assumed to operate in 20MHz BW

4.3 Assumptions on wake up cost

| Wakeup stage | Wake up energy (power * duration) |
| --- | --- |
| Deep sleep | EDSWU = 450 |
| Light sleep | ELSWU = 100 |

4.4 UE Power consumption calculation in idle mode

Normalized UE power consumption in one I-DRX cycle

PNR_IDLE=[PAGC*TSSB+PIntraF*TIntraF+PPOR *TPOR+PInterF *TInterF+PLS*TLS+PMS*TMS+PDS *TIDRX-TSSB-TIntraF-TPOR-TInterF-TLS-TMS+EDSWU +ELSWU*(1 or 2)]/TIDRX

Where

4.5 Relaxed RRM measurement power saving gain in idle mode

Power saving amount =

PInterF *TInterF+(PIntraF-PServ)*TIntraF+RRM_induced_sleep_power

Power saving gain = Power saving amountTotal UE power consumption*100%,

where Total UE power consumption equals PNR_IDLE in 4.4.

# Reference

3GPP TR 38.840 V16.0.0, “Study on User Equipment (UE) power saving in NR”