3GPP TSG-RAN WG4 Meeting #92bis	R4-1911463

Chongqing, China, 14th – 18th October, 2019

Source:	Huawei, HiSilicon

Title:	TP for TR 36.716-02-01 for CA_7A-13A, CA_7C-13A , CA_7A-7A-13A

Agenda item:	7.2.3

Document for:	Approval

Background

This contribution provides text proposal on the LTE CA configuration CA_7A-13A, CA_7C-13A , CA_7A-7A-13A as defined in the revised WID [1].

Reference

[1]	RP-191733, “Rel-16 LTE inter-band CA for 2 bands DL with 1 band UL”, Qualcomm Incorporated

Text Proposal

---Start of changes---

5.X	CA_7-13

5.X.1	Channel bandwidths per operating band for CA

Table 5.x.1-1: Supported E-UTRA bandwidths per CA configuration for 2DL inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_7A-13A | - | 7 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  |  | 13 |  |  | Yes | Yes |  |  |  |  |
| CA_7C-13A | - | 7 | See CA_7C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  | 50 | 0 |
|  |  | 13 |  |  | Yes | Yes |  |  |  |  |
| CA_7A-7A-13A | - | 7 | See CA_7A-7A Bandwidth combination set 1 in Table 5.6A.1-3 |  |  |  |  |  | 50 | 0 |
|  |  | 13 |  |  | Yes | Yes |  |  |  |  |

5.X.2 	Co-existence studies

Table 5.x.2-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 7 | 2500 | 2570 | 2620 | 2690 | 5000 | 5140 | 7500 | 7710 | 10000 | 10280 |
| 13 | 777 | 787 | 746 | 756 | 1554 | 1574 | 2331 | 2361 | 3180 | 3148 |

There is no harmonic interference for CA_7-13.

Table 5.x.2-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 7 | 2500 | 2570 | 2620 | 2690 | 5240 | 5380 | 7860 | 8070 | 10480 | 10760 |
| 13 | 777 | 787 | 746 | 756 | 1492 | 1512 | 2238 | 2268 | 2983 | 3024 |

There is no harmonic mixing for CA_7-13.

5.X.3		ΔTIB,c and ΔRIB,c values

ΔRIB,c values for CA_7-13 are derived from CA_7-28.  For the ΔTIB,c the  values from CA_7-28 are reused. Values will be defined in the following tables in TS 36.101.

Table 5.x.3-1: ΔTIB,c (two bands)

| E-UTRA operating band combination | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_7-13, CA_7-7-13 | 7 | 0.3 |
|  | 13 | 0.3 |

Table 5.x.3-2: ΔRIB,c (two bands)

| E-UTRA operating band combination | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_7-13, CA_7-7-13 | 7 | 0 |
|  | 13 | 0 |

5.X.4	REFSENS requirements

There are no reference sensitivity requirements needed.

---End of changes---