3GPP TSG-RAN WG4 Meeting #92bis	R4-1912483

Chongqing, China, 14 – 18 October 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	Further discussion on beam-based directions for TRP estimates of in-band unwanted emissions

Agenda item:	6.8.5.1

Document for:	Discussion

# 1	Introduction

At the RAN4#92 meeting, [1] discussed the open issue in [2] concerning the applicability of the beam-based directions procedure to compute a TRP estimate for in-band unwanted emissions. The discussion attempted to answer the question whether it is possible to determine if an unwanted emissions radiation pattern is correlated with that of the wanted signal without performing full spherical measurements. The outcomes of the discussion were presented.

Based on the feedback received following online and offline discussion, the outcomes have been revised and summarized in this document. The document also addresses the minimum number of measurement points needed to demonstrate correlation and what these points are, and how to measure those points for in-band unwanted emissions.

# 2	Discussion

## 2.1	Summary

From [2], the open issue is as follows:

Proposal 2: Methodologies for demonstrating the equivalence of DEUT between the desired signals and in-band unwanted emissions (ACLR and OBUE) is FFS.

Companies are encouraged to submit their views for Proposal 2.

In [1], we discussed the above open issue and raised the following question:

Question 1: Is it possible to demonstrate if the radiation pattern of unwanted emissions is correlated with that of the operating frequency without full spherical measurements?

Here, the outcomes of the discussion in [1] are summarized below:

- Maximum radiation of unwanted emissions in an in-band neighbouring frequency fn occurs in the same direction as that of the wanted signal at an operating frequency fo.

- The main lobe of the wanted signal at an operating frequency fo and the unwanted emissions in an in-band neighbouring frequency fn with respect to the axis of maximum radiation should have a similar symmetry. In certain cases, the point of maximum EIRP may not lie in the geometric centre of the main lobe.

- Measured HPBWs in the ϕ and θ directions for unwanted emissions in an in-band neighbouring frequency fn should correspond to those derived from the HPBWs for the wanted signal at the operating frequency fo.

- The directivity-beamwidth for the unwanted emissions in an in-band neighbouring frequency fn should correspond to that for the wanted signal at the operating frequency fo, i.e.,

DEUTfnϕHPBW'fnθHPBW'fndirectivity-beamwidth product(fn)=DEUTfoϕHPBWfoθHPBWfodirectivity-beamwidth product(fo)

where ϕHPBW' and θHPBW' denote the measured HPBW, and ϕHPBW and θHPBW denote HPBW derived from the HPBW of the operating frequency.

What is the minimum number of measurement points needed to demonstrate correlation?

From (a) and (c) above, only 5 measurement points are needed, which include:

- Maximum EIRP value (i.e., EIRPpeak).

- A pair of half-power (EIRP3dB,ϕ) values in the ϕ direction.

- A pair of half-power (EIRP3dB,θ) values in the θ direction.

## 2.2	How to measure the minimum number of points

In this subsection, we outline how to measure those 5 points in the radiation pattern of unwanted emissions as stated in subsection 2.1. If correlation exists, then the criteria (a) to (d) are met and the direction of maximum EIRP and half-power EIRP values can be determined from the parameters of the wanted signal:

- Maximum EIRP for in-band unwanted emissions occurs in the same direction as the maximum EIRP for the wanted signal.

- Angular positions of half-power values (EIRP3dB,ϕ and EIRP3dB,θ) can be computed using the HPBW and beam symmetry of the wanted signal.

- The measured EIRP3dB,ϕ and EIRP3dB,θ at the angular positions computed in Step 2 should be within (EIRPpeak – 3dB + Measurement error) to (EIRPpeak – 3dB – Measurement error) if correlation exists.

Based on the above, the complete measurement procedure is described:

Step 1: Measure maximum EIRP (EIRPpeak) of the unwanted emissions in an in-band neighbouring frequency fn in the same angular position as the maximum EIRP of the wanted signal at an operating frequency fo. This assumes the angular position of the wanted signal is known, denoted as (ϕmax, θmax).

Step 2: Measure half-power values (EIRP3dB,ϕ) at angular positions in the ϕ direction for the unwanted emissions in an in-band neighbouring frequency fn, where the angular positions are computed as follows (see Appendix A for the derivation of the equation):

ϕ1fn=ϕmaxfo+ϕHPBW(fo)fofnϕr(fo)1+ϕr(fo)    and     ϕ2fn=ϕmaxfo-fofnϕHPBW(fo)1+ϕr(fo)

Step 3: Measure half-power values (EIRP3dB,θ) at angular positions in the θ direction for the unwanted emissions in an in-band neighbouring frequency fn, where the angular positions are computed as follows (see Appendix A for the derivation of the equations):

θ1fo=θmaxfo+θHPBW(fo)fofnθr(fo)1+θr(fo)      and    θ2fn=θmaxfo-fofnθHPBW(fo)1+θr(fo)

Step 4: The measured half-power values in Steps 2 and 3 should be within (EIRPpeak – 3dB + MU) to (EIRPpeak – 3dB – MU) if the criteria (b) to (d) in subsection 2.1 are met. This implies, correlation exists, and the beam-based directions procedure can be applied to compute TRP estimate. Thus, the directivity (in linear units) at the in-band neighbouring fn as follows:

Dfn=fnfo2D(fo)

In Step 4, the MU derived for the Radiated Transmit Power requirement is adopted since the measurement in Steps 2 and 3 are basically EIRP measurements. From Tables 4.1.2.2-1 and 4.1.2.2-2 in TS 38.141-2, the MU for radiated transmit power under normal condition is as follows:

- MU = 1.1 dB, f ≤ 3 GHz

- MU = 1.3 dB, 3 GHz < f ≤ 6 GHz

- MU = 1.7 dB, 24.25 < f ≤ 29.5 GHz

- MU = 2.0 dB, 37 < f ≤ 40 GHz

# 3	Conclusions

The document has revised and summarized the main outcomes based on the discussion in an earlier document. The outcomes allow us to determine what to measure and how many measurement points are needed for in-band unwanted emissions. Further, a simple measurement procedure is outlined, where angular positions of half-power values (EIRP3dB,ϕ and EIRP3dB,θ) for in-band unwanted emissions can be computed using the HPBW and beam symmetry of the wanted signal. If the measured half-power values are within (EIRPpeak – 3dB + MU) to (EIRPpeak – 3dB – MU), it can be concluded that correlation exists and the beam-based directions procedure can be used to compute in-band TRP estimates. The MU derived for the Radiated Transmit Power requirement is adopted.

# References

- R4-1909891, On beam-based directions for TRP estimates of in-band unwanted emissions, Nokia, Nokia Shanghai Bell

- R4-1902272, Way forward on TRP measurements, NTT DoCoMo, Nokia, Nokia Shanghai Bell, ZTE

# Appendix A

Symmetry of main lobe with respect to the axis of maximum radiation

The definition is given below for the main lobe in the ϕ and θ planes.

Azimuth Angle ϕ (degrees)

ϕHPBW

ϕA

ϕB

ϕ1

ϕ2

ϕmax

Figure 1: Measurement points in the ϕ plane

In the ϕ plane as shown Figure 1, the beam symmetry ϕr is defined as

ϕr=ϕmax-ϕ1ϕ2-ϕmax 																						(1)

The HPBW is

ϕHPBW= ϕ1-ϕ2 																					(2)

Solving Equations (1) and (2) for ϕ1 and ϕ2, we get

ϕ1=ϕmax+ϕHPBWϕr1+ϕr       																	(3a)

ϕ2=ϕmax-ϕHPBW1+ϕr              																		(3b)

According to (b) in subsection 2.1, the main lobe of the wanted signal at an operating frequency fo and the unwanted emissions in an in-band neighbouring frequency fn with respect to the axis of maximum radiation should have a similar symmetry, i.e.,

ϕrfn=ϕr(fo)																						(4)

The symmetry of the wanted signal is assumed to be known in Equation (4).

HPBW of unwanted emissions with respect to the HPBW of the operating frequency

According to (c) in subsection 2.1, the measured HPBWs in Equations (3a) and (3b) for unwanted emissions in an in-band neighbouring frequency fn should correspond to those derived from the HPBWs for the wanted signal at the operating frequency fo, i.e.,

ϕHPBWfn=fofnϕHPBW(fo) 																		(5)

The HPBW of the wanted signal in Equation (5) is assumed to be known.

Maximum radiation direction

According to (a) in subsection 2.1, the maximum radiation of unwanted emissions at an in-band frequency fn occurs at the same angular position as the wanted signal at the operating frequency fo, i.e.,

ϕmaxfn=ϕmax(fo) 																				(6a)

θmaxfn=θmax(fo) 																				(6b)

Substituting Equations (4), (5) and (6) into (3a) and (3b), we can express ϕ1(fn) and ϕ2(fn) for an in-band neighbouring frequency fn as a function of the wanted signal operating frequency fo, i.e.,

ϕ1fn=ϕmaxfo+ϕHPBW(fo)fofnϕr(fo)1+ϕr(fo) 													(7a)

ϕ2fn=ϕmaxfo-fofnϕHPBW(fo)1+ϕr(fo)              														(7b)

Elevation Angle θ (degrees)

θHPBW

θA

θB

θ1

θ2

θmax

Figure 2: Measurement points in the θ plane

In the θ plane as shown in Figure 2, the same approach as above is used to derive θ1(fn) and θ2(fn) given in Equations (8a) and (8b).

θ1fo=θmaxfo+θHPBW(fo)fofnθr(fo)1+θr(fo)      											(8a)

θ2fn=θmaxfo-fofnθHPBW(fo)1+θr(fo)															(8b)