| 3GPP TR 38.XXX V0.1.0(2019-11) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Background for Integrated access and backhaul radio transmission and reception (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	7

4	General aspect	8

4.1	Specification organization	8

4.2	RF Requirements reference points	8

4.3	IAB classification	8

4.4	Regulatory aspects	8

4.5	IAB architecture	8

5  Operating band and channel arrangement	10

5.1	General	10

5.2	Operating bands	10

5.3	Channel bandwidth	10

5.4	Channel arrangement	10

6  Coexistence study	10

6.1	System layout and scenario	10

6.1.x	Co-location	10

6.2	Simulation assumption	11

6.3	Simulation result	11

6.4	Summary on co-existence study	12

7	Conducted transmitter characteristics	12

7.1	General	12

7.2	IAB output power	12

7.3	Output power dynamics	12

7.4	Transmit ON/OFF power	12

7.5	Transmitted signal quality	12

7.6	Unwanted emissions	12

7.7	Transmitter intermodulation	12

8	Conducted receiver characteristics	12

8.1	General	12

8.2	Reference sensitivity level	12

8.3	Dynamic range	13

8.4	In-band selectivity and blocking	13

8.5	Out-of-band blocking	13

8.6	Receiver spurious emissions	13

8.7	Receiver intermodulation	13

8.8	In-channel selectivity	13

9	Radiated transmitter characteristics	13

9.1	General	13

9.2	Radiated transmit power	13

9.3	IAB OTA output power	13

9.4	OTA output power dynamics	13

9.5	OTA transmit ON/OFF power	14

9.6	OTA transmitted signal quality	14

9.7	OTA unwanted emissions	14

9.8	OTA transmitter intermodulation	14

9.8	Beam correspondence for IAB-MT	14

10	Radiated receiver characteristics	14

10.1	General	14

10.2	OTA sensitivity	14

10.3	OTA reference sensitivity level	14

10.4	OTA Dynamic range	14

10.5	OTA in-band selectivity and blocking	14

10.6	OTA out-of-band blocking	14

10.7	OTA receiver spurious emissions	15

10.8	OTA receiver intermodulation	15

10.9	OTA in-channel selectivity	15

11	IAB RRM requirements	15

12	IAB EMC requirements	15

13	IAB performance requirements	15

14	IAB conformance testing aspect	15

Annex <A> (normative): <Normative annex for a Technical Report>	16

Annex <X> (informative): Change history	17

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document is the Technical Report for the Work Item on Integrated Access and Backhaul for NR. The present document captures the background and the decisions on IAB requirements.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

Example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

<ABBREVIATION>	<Expansion>

# 4	General aspect

## 4.1	Specification organization

<Text will be added>

## 4.2	RF Requirements reference points

<Text will be added>

## 4.3	IAB classification

<Text will be added>

## 4.4	Regulatory aspects

<Text will be added>

## 4.5	IAB architecture

The logical architecture (considering the RF interfaces) of the IAB is as follows:

Figure 4.5-1: Logical IAB functions showing RF interfaces

In the RF specification these logical functions have been separated into the HW entities the IAB-DU and the IAB-MT IAB-DU. The HW entities may be implemented in the same radio hardware as shown in figure 4.2-2 or separate radio hardware as shown in figure 4.5-3, the diagrams show the OTA architecture but the same applies for the hybrid architecture.

Figure 4.5-2: Shared IAB hardware

Figure 4.5-3: Separate IAB hardware

The figures show the hardware as either completely shared or completely separate, there are of course many levels of possible integration in between these 2 extremes.

When considering the OTA architecture and specifications the nature of HW is not relevant as the node can be treated as a black box, however in deriving the requirements for the IAB nodes it is important to consider both implementations so that both can be implemented if required.

# 5   Operating band and channel arrangement

## 5.1	General

The Frequency range definitions for IAB will be the same as for the NR BS in TS 38.104 [x].

## 5.2	Operating bands

Operating bands will be added to the IAB specification as they are approved, no bands are precluded at this stage and can be added based on consensus. Currently the FR1 bands n41 and n79 from TS 38.104 [x] and all Rel-15 FR2 bands shown in table 5.2-1 are to be included in the IAB specification.

Table 5.2-1: NR operating bands in FR2

| NR operating band | Uplink (UL) and Downlink (DL) operating band BS transmit/receive UE transmit/receive FUL,low – FUL,high FDL,low – FDL,high | Duplex mode |
| --- | --- | --- |
| n257 | 26500 MHz – 29500 MHz | TDD |
| n258 | 24250 MHz – 27500 MHz | TDD |
| n260 | 37000 MHz – 40000 MHz | TDD |
| n261 | 27500 MHz – 28350 MHz | TDD |

## 5.3	Channel bandwidth

<Text will be added>

## 5.4	Channel arrangement

<Text will be added>

# 6   Coexistence study

## 6.1	System layout and scenario

<Text will be added>

### 6.1.x	Co-location

An IAB node is capable of transmitting in the DL (IAB-DU) or the UL (IAB-MT). When acting as an IAB-MT there are 2 possible co-location interference scenarios between the IAB-MT and a BS.

- Aggressor IAB-MT transmitting in UL, victim BS receiving in UL

- Aggressor BS transmitting in DL, victim IAB-MT receiving in DL

For co-location, the interference is given by:

Pinterference =10*log1010PACLR10+10PACS10

Where:

PACLR = Ptx_aggressor – ACLRaggressor – coupling

PACS = Ptx_aggressor – ACSvictim – coupling

A conservative estimate for the coupling between two co-located systems is; 30dB for FR1 and 45dB for FR2.

Note: this figure is used only for this analysis. It is not an agreed FR2 isolation figure.

For a micro BS scenario:

Table 6.1.x-1: Co-location interference between BS and IAB-MT for FR1 and FR2

|  |  | IAB |  | BS |  |
| --- | --- | --- | --- | --- | --- |
|  | unit | FR1 | FR2 | FR1 | FR2 |
| Ptx | dBm | 30 | 30 | 33 | 33 |
| ACLR | dB | 45 (Note1) | 28 (Note1) | 45 | 28 |
| Sensitivity (FR2 approx. equivalent conducted sensitivity) | dBm | -96.5 (4.5MHz) (Note2) | approx. -85 (50MHz) (Note2) | -96.5 (4.5MHz) | approx. -85 (50MHz) |
| ACS | dB | 45 | 24 | 45 | 24 |
| Coupling | dB | 30 | 45 (Note3) | 30 | 45 (Note3) |
| IAB to BS interference (UL) | dBm |  |  | -42.0 | -37.5 |
| BS to IAB interference (DL) | dBm | -41.9 | -34.5 |  |  |
| Note1: the ACLR figures used are BS values, it has not been agreed to use BS figure for IAB, however UE figures will result in worse interference. Note 2: sensitivity values based on NF assumption in co-location simulation see sub-clause x.x.x Note 3: coupling figures for FR2 are not formally agreed, assumption used only for this example |  |  |  |  |  |

Note for FR2 there are no conducted requirements so the coupling and the sensitivity are estimated to a virtual conducted point for the purposes of comparison.

It can be seen that for both FR1 and FR2 significant additional isolation (50 to 60dB) is required if the systems are to be co-located.

The issue exists for both scenario 1 and scenario 2 (see sub-clause 6.1.y) as it occurs in both the UL and the DL.

## 6.2	Simulation assumption

Detailed structure of the subclause is TBD.

## 6.3	Simulation result

<Text will be added>

## 6.4	Summary on co-existence study

<Text will be added>

# 7	Conducted transmitter characteristics

## 7.1	General

<Text will be added>

## 7.2	IAB output power

Detailed structure of the subclause is TBD.

## 7.3	Output power dynamics

Detailed structure of the subclause is TBD.

## 7.4	Transmit ON/OFF power

Detailed structure of the subclause is TBD.

## 7.5	Transmitted signal quality

Detailed structure of the subclause is TBD.

## 7.6	Unwanted emissions

Detailed structure of the subclause is TBD.

## 7.7	Transmitter intermodulation

Detailed structure of the subclause is TBD.

# 8	Conducted receiver characteristics

## 8.1	General

<Text will be added>

## 8.2	Reference sensitivity level

Detailed structure of the subclause is TBD.

## 8.3	Dynamic range

Detailed structure of the subclause is TBD.

## 8.4	In-band selectivity and blocking

Detailed structure of the subclause is TBD.

## 8.5	Out-of-band blocking

Detailed structure of the subclause is TBD.

## 8.6	Receiver spurious emissions

Detailed structure of the subclause is TBD.

## 8.7	Receiver intermodulation

Detailed structure of the subclause is TBD.

## 8.8	In-channel selectivity

Detailed structure of the subclause is TBD.

# 9	Radiated transmitter characteristics

## 9.1	General

## 9.2	Radiated transmit power

Detailed structure of the subclause is TBD.

## 9.3	IAB OTA output power

Detailed structure of the subclause is TBD.

## 9.4	OTA output power dynamics

Detailed structure of the subclause is TBD.

## 9.5	OTA transmit ON/OFF power

Detailed structure of the subclause is TBD.

## 9.6	OTA transmitted signal quality

Detailed structure of the subclause is TBD.

## 9.7	OTA unwanted emissions

Detailed structure of the subclause is TBD.

## 9.8	OTA transmitter intermodulation

Detailed structure of the subclause is TBD.

## 9.8	Beam correspondence for IAB-MT

Detailed structure of the subclause is TBD.

# 10	Radiated receiver characteristics

## 10.1	General

## 10.2	OTA sensitivity

Detailed structure of the subclause is TBD.

## 10.3	OTA reference sensitivity level

Detailed structure of the subclause is TBD.

## 10.4	OTA Dynamic range

Detailed structure of the subclause is TBD.

## 10.5	OTA in-band selectivity and blocking

Detailed structure of the subclause is TBD.

## 10.6	OTA out-of-band blocking

Detailed structure of the subclause is TBD.

## 10.7	OTA receiver spurious emissions

Detailed structure of the subclause is TBD.

## 10.8	OTA receiver intermodulation

Detailed structure of the subclause is TBD.

## 10.9	OTA in-channel selectivity

Detailed structure of the subclause is TBD.

# 11	IAB RRM requirements

Detailed structure of the subclause is TBD.

# 12	IAB EMC requirements

Detailed structure of the subclause is TBD.

# 13	IAB performance requirements

Detailed structure of the subclause is TBD.

# 14	IAB conformance testing aspect

Detailed structure of the subclause is TBD.

Annex <A> (normative):
<Normative annex for a Technical Report>

<Text will be added>

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | Tdoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-10 | RAN4#92bis | R4-1912889 |  |  |  | Initial TR skeleton | 0.0.1 |
| 2019-11 | RAN4#93 | R4-1913332 |  |  |  | Updated TR to incorporate below TPs endorsed during meeting: R4-1916003 [IAB] TP to TR 38.xxx Co-location scenarios R4-1916166 [IAB] TP to TR 38.xxx System parameter R4-1914761 [IAB] TP to TR 38.xxx Architecture | 0.1.0 |