3GPP TSG-RAN WG4 Meeting #93              	R4-1913370

Reno, USA, 18th– 22th Nov, 2019

Agenda item:	9.9.2.3

Source:	Samsung

Title:	View on URLLC performance requirements in NR Rel-16

Document for:	Discussion

# 1	Introduction

During the agreed work plan for URLLC demodulation performance requirements in [1], from the RAN4#92bis meeting, it is agreed to identify and prioritize the features that related demodulation performance requirement for high reliability from the following features based on Rel-15 URLLC functionalities:

- PDSCH repetitions over multiple slots

- PUSCH repetitions over multiple slots

- 4-bit CQI Table 3

- MCS index table 3

- Other features are not precluded

And to identify and prioritize the features that related demodulation performance requirement for low latency from the following features based on Rel-15 URLLC functionalities:

- PDSCH processing capability 2

- Self-contained slot and or not-slot for DL

- PDSCH and PUSCH mapping type A/B

- Pre-emption indication for DL

- Other features are not precluded

In this contribution, the view on URLLC performance requirements is presented.

# 2	Discussion

## 2.1	Requirements for high reliability

PUSCH requirement

In order to improve reliability, PUSCH based on slot repetitions is the important feature for URLLC. Based on RAN1 design, the number of repetitions can be supported if UE is configured with pusch-AggregationFactor as follows

pusch-AggregationFactor                 ENUMERATED { n2, n4, n8 }                                           OPTIONAL,   -- Need S

mcs-Table                               ENUMERATED {qam256, qam64LowSE}                                     OPTIONAL,   -- Need S

mcs-TableTransformPrecoder              ENUMERATED {qam256, qam64LowSE}                                     OPTIONAL,   -- Need S

Meanwhile, as indicated, the PUSCH is limited to a single transmission layer. The UE shall repeat the TB across the pusch-AggregationFactor consecutive slots applying the same symbol allocation in each slot, which means there is no inter-slot frequency hopping.

Proposal 1: The PUSCH requirement with repetition should be considered for URLLC in RAN4. The requirement is available for only single transmission layer.

Excepting for repetition, the normal HARQ combination with different RV can be regarded as an efficient way to improve the reliability. The following are different RV configuration for repetitions is presented

| rvid indicated by the DCI scheduling the PUSCH | rvid to be applied to nth transmission occasion |  |  |  |
| --- | --- | --- | --- | --- |
|  | n mod 4 = 0 | n mod 4 = 1 | n mod 4 = 2 | n mod 4 = 3 |
| 0 | 0 | 2 | 3 | 1 |
| 2 | 2 | 3 | 1 | 0 |
| 3 | 3 | 1 | 0 | 2 |
| 1 | 1 | 0 | 2 | 3 |

MCS table

New MCS table with low coding rate is supported for PUSCH to improve reliability. The same coding rate is available for both CP-OFDM and DFT-s-OFDM. It is straight forwards that the requirement should be considered in RAN4.

Proposal 2 The PUSCH requirement with new MCS table for URLLC with reliability should be considered in RAN4.

PUCCH requirement

In eMBB, multi-slot is supported for PUCCH format 1/3/4 to improve the reliability of UCI detection performance.  Meanwhile, the intra-slot frequency and inter-slot frequency hopping are also supported to improve the detection performance with additional frequency diversity gain. NR Rel-15 has already defined the requirement with frequency hopping and multi-slot transmission. From the demodulation performance point of view, there is not different with multi-slot or repetition transmission.

Proposal 3: There is no PUCCH performance with repetition for URLLC

## 2.2	Requirements for low latency

For latency requirements, it should be related with UE/BS processing timing, response time, timing adjustment, UL schedule (both SR and SPL).

In [2], the timing assumption with different SCS and symbol lengths is given by

| Timing | 15/30kHz SCS |  |  | 120kHz SCS |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| #TTI | 7os TTI | 4os TTI | 2os TTI | 7os TTI | 4os TTI | 2os TTI |
| gNB processing | 1 | 1 | 2 | 2 | 3 | 5 |
| UL timing | 1 | 1 | 2 | 2 | 3 | 5 |

The latency for DL and UL data can be illustrated as

Figure 1, latency for data transmission with DL and UL

Based on this procedure, the latency for FDD based on HARQ retransmission can be analyzed simply as [1]

Table 1, latency for data transmission with HARQ-based retransmission

| Latency (ms) | HARQ | 15kHz SCS |  |  |  | 30kHz SCS |  |  |  | 120kHz SCS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14-os TTI | 7-os TTI | 4-os TTI | 2-os TTI | 14-os TTI | 7-os TTI | 4-os TTI | 2-os TTI | 14-os TTI | 7-os TTI | 4-os TTI | 2-os TTI |
| DL data | 1st transmission | 2.4 | 1.4 | 1.0 | 0.71 | 1.2 | 0.71 | 0.50 | 0.36 | 0.41 | 0.29 | 0.23 | 0.20 |
|  | 1 retx | 5.4 | 2.9 | 1.9 | 1.4 | 2.7 | 1.5 | 0.93 | 0.71 | 0.79 | 0.60 | 0.48 | 0.39 |
|  | 2 retx | 8.4 | 4.4 | 2.7 | 2.1 | 4.2 | 2.2 | 1.4 | 1.1 | 1.2 | 0.91 | 0.73 | 0.59 |
|  | 3 retx | 11 | 5.9 | 3.6 | 2.9 | 5.7 | 3.0 | 1.8 | 1.4 | 1.5 | 1.2 | 0.98 | 0.79 |
| UL data (SR) | 1st transmission | 4.5 | 2.5 | 1.6 | 1.4 | 2.3 | 1.3 | 0.82 | 0.68 | 0.67 | 0.54 | 0.46 | 0.38 |
|  | 1 retx | 8.4 | 4.4 | 2.7 | 2.1 | 4.2 | 2.2 | 1.4 | 1.1 | 1.2 | 0.91 | 0.73 | 0.59 |
|  | 2 retx | 11 | 5.9 | 3.6 | 2.9 | 5.7 | 3.0 | 1.8 | 1.4 | 1.5 | 1.2 | 0.98 | 0.79 |
|  | 3 retx | 14 | 7.4 | 4.4 | 3.6 | 7.2 | 3.7 | 2.2 | 1.8 | 1.9 | 1.5 | 1.2 | 0.98 |
| UL data (SPS) | 1st transmission | 2.4 | 1.4 | 1.0 | 0.71 | 1.2 | 0.71 | 0.50 | 0.36 | 0.41 | 0.29 | 0.23 | 0.20 |
|  | 1 retx | 5.4 | 2.9 | 1.9 | 1.4 | 2.7 | 1.5 | 0.93 | 0.71 | 0.79 | 0.60 | 0.48 | 0.39 |
|  | 2 retx | 8.4 | 4.4 | 2.7 | 2.1 | 4.2 | 2.2 | 1.4 | 1.1 | 1.2 | 0.91 | 0.73 | 0.59 |
|  | 3 retx | 11 | 5.9 | 3.6 | 2.9 | 5.7 | 3.0 | 1.8 | 1.4 | 1.5 | 1.2 | 0.98 | 0.79 |

As indicated in this table, slot based transmission with 120 KHz can meet 1ms target. With 15KHz SCS and higher, mini-slot based transmission can fulfill latency target. With 30KHz SCS and higher, mini-slot HARQ-based retransmission can fulfill latency target.

For Rel-15 NR, The performance requirement for both slot based and not slot based PUSCH with type A and type B have been defined. To verify the latency feature, mini slot based transmission can be regarded to one solution to meeting latency targeting, such as 2, 4 or 7 symbols.

It is agreed that no combined performance requirements will be defined by meeting both reliability of 99.999% and low latency requirements. That means some features with slot repetitions to improve high reliability should not be combined.

Proposal 4: The mini-slot based transmission could be considered to define requirements for meeting latency targeting.

PUSCH mapping type A/B

NR defines two kinds of time domain resource allocation, type A and type B. The start symbol and symbol length can be configured in time domain as follows:

Table 3: time domain resource allocation

|  | Start symbol | length |
| --- | --- | --- |
| Type A | 0 | {4,…,14} |
| Type B | {0,13} | {1,….14} |

For mapping type B, the front loaded DMRS is located with the first allocated data symbol. From the implementation point view, the process delay is less for type B, which can help decoding latency by obtaining a fast channel estimate and performing sequential decoding of frequency-first mapped code blocks. In NR Rel-15, the requirement for type B is defined. So, there is no need to verify the requirement of type B individually. To reduce the number of test cases, the type B requirement should be combined with other URLLC feature.

Proposal 5: PUSCH type B requirement should be considered for URLLC combined with other URLLC feature. There is no individual requirement for mapping type B.

# Conclusion

In this contribution, the initial results are provided to analyze the URLLC with high reliability requirement.

Proposal 1: The PUSCH requirement with repetition should be considered for URLLC in RAN4. The requirement is available for only single transmission layer.

Proposal 2 The PUSCH requirement with new MCS table for URLLC with reliability should be considered in RAN4.

Proposal 3: There is no PUCCH performance with repetition for URLLC

Proposal 4: The mini-slot based transmission could be considered to define requirements for meeting latency targeting.

Proposal 5: PUSCH type B requirement should be considered for URLLC combined with other URLLC feature. There is no individual requirement for mapping type B.

# References

R4-1910044, WF on URLLC demodulation performance requirements, Huawei

R1-1806016, Evaluation of UP latency in NR	Ericsson