3GPP TSG-RAN WG4 Meeting #93																		 R4-1913461

Reno, USA, 18th – 22nd Nov, 2019

Agenda Item:	9.8.2.1.2

Source:	Intel Corporation

Title:	Link-level simulation Assumptions for RSTD measurement

Document for:		Discussion

# Introduction

In the previous RAN4 meeting it was agreed that both system-level and link-level simulation studies were necessary for NR positioning requirements [1]. In this contribution, we propose scenarios and a set of initial link-level simulation assumptions for PRS RSTD measurements.

# Simulation Scenarios

For DL OTDoA positioning, the measurement of both PRS RSTD and PRS RSRP are needed. In order to conduct the realistic performance requirements for DL OTDoA, both PRS RSTD and RSRP link level simulation works shall be studied in RAN4.

In principle, UE RSTD measurements for OTDoA positioning are related to the geometric distance between UE and gNB as shown in the figure below. Therefore, at least the measurement from two neighbor cells are necessary.

Figure 1. RSTD measurement in NR

Observation 1: As a baseline scenario, we propose to model three explicit cells in the link level simulations. The interference from the rest of the network, which is not explicitly simulated, can be modeled as AWGN noise.

As a start point, the simulation scenario was assumed to be synchronous and one set of timing offsets below between cells was evaluated.

- Synchronous case: relative delay1=0, 2=0, 3=CP/2

It should be further discussed what is reasonable assumption for the maximum offset between the transmission timings of PRS subframes in different cells.

The other simulation parameters can be initially leveraged from LTE PRS simulation [2].

Table 1. General parameters

| Parameter | Value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Carrier frequency, GHz | 2 GHz, 4 GHz | 30 GHz |
| Bandwidth, MHz | 5 MHz, 50 MHz for 2 GHz 100 MHz for 4 GHz | 100 MHz, 400 MHz |
| Subcarrier spacing, kHz | 15 kHz for 5 MHz and 50 MHz 30 kHz for 100 MHz | 120 kHz |
| Cell ID scenarios | (0, 1, 2), (0, 6, 12) | (0, 1, 2), (0, 6, 12) |
| Network synchronization | Synchronous | Synchronous |
| Data and control load | 100% | 100% |
| Cyclic prefix | Normal | Normal |
| DRX | OFF | OFF |
| UE speed | - 3 km/h - 30 km/h | - 3 km/h |
| Channel model | AWGN, TDL-A/B/C | AWGN, TDL-A/B/C |
| Noc (does not include received powers of the three simulated cells), [dBm/15kHz] | AWGN, values are TBD | AWGN, values are TBD |
| Ês/Noc for three cells, [dB] | [TBD] | [TBD] |
| Number of transmit antennas | 1 | 1 |
| Number of receive antennas | 2 equal-gain uncorrelated antennas | 2 equal-gain uncorrelated antennas |
| PRS muting | No | No |
| Number of PRS resource per TRP | 1 | 1 |
| PRS Measurement bandwidth | Full carrier BW | Full carrier BW |
| L1 measurement period | 40ms, 160ms | 160ms,640ms |
| Note: It is the side condition determined by system simulation results. |  |  |

Table 2. PRS configuration parameters

| Parameter | Value |  |
| --- | --- | --- |
|  | FR1 | FR2 (can be low priority) |
| Number of consecutive positioning slots | 2, 4, 6 non-coherent accumulation | 2, 4, 6 non-coherent accumulation |
| PRS transmission bandwidth | 24 PRBs, 96 PRBs; optional: 48 PRBs, 192 PRBs (not for 60 kHz), 264 PRBs (not for 60 kHz) | 24 PRBs, 96 PRBs; optional: 48 PRBs, 192 PRBs, and 264 PRBs |
| PRS period（slots） | 5, 160, 1280 | 5120, 20480 |
| Repetition factor | 1 | 1 |
| Comb size | 2,4,6 | 2,4,6 |

# Performance Metric

The intention of these simulation works is to provide a basis for PRS RSTD accuracy requirements

Thus

Proposal 1: The following performance indicators are proposed for link-level studies:

- CDF of RSTD estimation error

Proposal 2:  RSTD estimation error can be defined as:

- “An RSTD error is the difference between the RSTD estimated by the UE and the ideal value with noise free.”

Other factors affecting the RSTD measurement error (e.g. the quantization error, RF impairments etc.) can be included after the initial results calibration.

# Conclusion

In this contribution, the view on the link level simulation assumption for PRS RSTD is provided.

Observation 1: As a baseline scenario, we propose to model three explicit cells in the link level simulations. The interference from the rest of the network, which is not explicitly simulated, can be modeled as AWGN noise.

Proposal 1: The following performance indicators are proposed for link-level studies:

- CDF of RSTD estimation error

Proposal 2:  RSTD estimation error can be defined as:

- “An RSTD error is the difference between the RSTD estimated by the UE and the ideal value with noise free.”

# References

- R4-1912793, “ Ad-hoc minutes for NR Positioning RRM”

- R4-094533, “Proposed link-level simulation assumptions for defining OTDOA RSTD requirements”

- 3GPP TS 38.133 v15.7.0: "NR: Requirements for support of radio resource management "

- 3GPP TS 36.133 v15.7.0: "E-UTRA: Requirements for support of radio resource management "