3GPP TSG-RAN4 Meeting #93	R4-1913504

# Reno, NV, USA, 18th – 22nd November, 2019

Title:	Reply LS on applicable timing for pathloss RS activated/updated by MAC-CE

Reply To:	R1-1911616

Release:	Rel-16

Work Item:	NR_eMIMO-Core

Source:	RAN WG 4

To:	RAN WG 1

Contact Person:

Name:	Jie Cui

E-mail Address:	jie_cui@apple.com

Attachments:	N.A.

1. Overall Description:

RAN4 would like to thank RAN1 for the LS on applicable timing for pathloss RS activated/updated by MAC-CE (R1-1911616). After discussion in RAN4, the following conclusions are achieved,

-	RAN4 will define the delay requirement for pathloss RS activation/updating in Rel-16 eMIMO scope. It could be decided in next RANP meeting if the eMIMO WID needs to be updated accordingly or not

-	It’s up to RAN1 to capture the description of pathloss RS activation/updating procedure in RAN1 spec, and if captured in RAN1 spec, the expected activation/updating delay shall still be referred to RAN4 spec

-	RAN4 may revise the sample number for pathloss RS tracking during the pathloss RS activation/updating and that would be reflected in pathloss RS activation/updating delay requirement.

-	The filtered RSRP value for previous pathloss RS will be used within the application time, which is 2ms after the latest pathloss RS measurement sample.

2. Actions:

To RAN WG1 group.

ACTION: 	RAN4 kindly asks RAN1 to take the above information into account in their future work.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN4 #94			24th – 28th Feb 2020 		Athens, Greece

TSG-RAN4 #94bis			20th – 24th Apr 2020 		China