3GPP RAN WG4 Meeting #93		R4-1913507

Reno, USA, 18 – 22th November 2019

Agenda item:	9.16.1

Source:	Apple Inc.

Title:	On definition of CSI-RS based intra-frequency and inter-frequency layers

WI:	NR_CSIRS_L3meas-Core

Release:	Rel-16

Document for:	Discussion

# 1	Introduction

In RAN4#93, different options of the definition of CSI-RS intra- and inter-frequency MO are summarized in [1]. In general, the following metric compared between serving and target frequency layers are used to classify a target frequency layer as intra-frequency or inter-frequency.

- SCS of CSI-RS

- Center frequency of CSI-RS

- Bandwidth of CSI-RS

On top of that, whether or not CSI-RS resource are contained within UE active BWP is another consideration. In this paper, further analysis is provided on the potential way to specify the definition of CSI-RS based measurements.

# 3	Definition of CSI-RS based Intra- and inter-frequency Measurements

As illustrated in Figure 1 [1], the commonality between Option 1 and 2 is CSI-RS resources in both target and serving cells are within the active BWP and have the same SCS. The only difference is about the center frequency.

| Option 1: same center frequency, SCS and within active BWP | Option 2: different center frequency, SCS and within active BWP |
| --- | --- |

The question then becomes if the center frequency of Cell2 CSI-RS resource will result in the different L3 measurement requirements. Based on our analysis, Cell2 CSI-RS L3 measurement requirements should be the same in both Option 1 and 2. Therefore, there is no obvious motivation to differentiate inter- and intra-frequency based on center frequency for CSI-RS L3 measurement.  With this, it is proposed that

Proposal 1:

- When the SCS of CSI-RS on serving cell and target cell is the same and the CSI-RS resource of the target cell is fully contained within the active BWP of the serving cell, the corresponding CSI-RS L3 measurement is defined as intra-frequency measurement without gap.

- When the SCS of CSI-RS on serving cell and target cell is NOT the same or the CSI-RS resource of the target cell is fully contained within the active BWP of the serving cell, the corresponding CSI-RS L3 measurement is defined as inter-frequency measurement.

In TS38.331, it is specified that

For CSI-RS resources, the network applies the procedure as follows:

-	to ensure that all CSI-RS resources configured in each measurement object have the same center frequency, (startPRB+floor(nrofPRBs/2))

That indicates MO is specified based on center frequency of CSI-RS resource. If CSI-RS based intra-frequency measurement is defined based on Proposal 1, there can be multiple MO for intra-frequency.

Proposal 2: Multiple intra-frequency measurement objects can be configured for intra-frequency measurements.

Meanwhile, it is specified in TS38.331, within the same MO, CSI-RS resources of the target cells can be different in terms of density, bandwidth and periodicity. From this point of view, CSI-RS resource bandwidth, density and periodity should not be used to distinguish between CSI-RS based intra- and inter-frequency measurement.

Proposal 3: CSI-RS resource bandwidth, density and periodicity should not be used to distinguish between CSI-RS based intra- and inter-frequency measurement.

# 5 Conclusion

In this contribution, the definition of CSI-RS based intra- and inter-frequency measurement are analysed. It is proposed that

Proposal 1:

- When the SCS of CSI-RS on serving cell and target cell is the same and the CSI-RS resource of the target cell is fully contained within the active BWP of the serving cell, the corresponding CSI-RS L3 measurement is defined as intra-frequency measurement without gap.

- When the SCS of CSI-RS on serving cell and target cell is NOT the same or the CSI-RS resource of the target cell is fully contained within the active BWP of the serving cell, the corresponding CSI-RS L3 measurement is defined as inter-frequency measurement.

Proposal 2: Multiple intra-frequency measurement objects can be configured for intra-frequency measurements.

Proposal 3: CSI-RS resource bandwidth, density and periodicity should not be used to distinguish between CSI-RS based intra- and inter-frequency measurement.

# References

[1] R4-1910956 WF on CSI-RS based RRM measurements requirements, CATT