3GPP RAN WG4 Meeting #93		R4-1913536

Reno, USA, 18 – 21 November 2019

Agenda item:	10.2.3

Source:	Apple Inc.

Title:	Remaining issues with the FR2 UL inter-band CA feature

WI/SI:	NR_CADC_R16_2BDL_xBUL

Release:	Rel-16

Document for:	Discussion

# 1	Introduction

The Rel-16 FR2 RF work item includes scope to define requirements for UL inter-band CA in FR2 [1]:

- FR2 UE requirements for inter-band UL CA [RAN4] - Phase 1: Study if both simultaneous UE transmission on aggregated UL carriers and non-simultaneous transmission on aggregated UL carriers with UE switching between two carriers could and should be specified. Study potential impacts of non-simultaneous transmission on RAN1 and/or RAN2 specifications - Phase 2: Define FR2 UE requirements for inter-band UL CA based on the outcome of the Phase 1 study

Since RAN4 meeting #92, which was the first meeting of the work item, RAN4 has reached zero agreements on FR2 UL inter-band CA.  This contribution examines the current status of the feature and provides recommendations for a way forward in light of carrier requests for FR2 UL inter-band CA configurations.

# 2	Discussion

The only technical proposal to enable the entire FR2 UL inter-band CA feature via the non-simultaneous Tx mechanism, has been provided in [2].  One company suggested the following handling of the feature in [3]:

RAN4 will postpone work for inter-band UL CA until understanding on how to set the requirements for DL spherical coverage are understood.

During the RAN4 #92bis meeting progress was achieved on the non-simultaneous UL proposal with further clarifications submitted in [4] and agreements on the mechanisms to consider captured in a way forward in [5].

According to our understanding, non-simultaneous UL is the only mechanism available to RAN4 to enable UL inter-band CA in FR2 in Rel-16.

Proposal 1:	Non-simultaneous UL is the only mechanism available to RAN4 to enable UL inter-band CA in FR2 in Rel-16.

If there is a strong need in the industry to also enable simultaneous Tx for inter-band UL CA, then the feature can be postponed to Rel-17.

Proposal 2:	The UL inter-band CA in FR2 feature can be postponed to Rel-17 due to lack of progress on aspects related to simultaneous UL.

# 3	Conclusions

This contribution provided a status of the UL inter-band CA feature in FR2 and made the following proposals:

Proposal 1:	Non-simultaneous UL is the only mechanism available to RAN4 to enable UL inter-band CA in FR2 in Rel-16.

Proposal 2:	The UL inter-band CA in FR2 feature can be postponed to Rel-17 due to lack of progress on aspects related to simultaneous UL.

# 4	References

- RP-190761, “NR RF requirements for FR2”

- R4-1908014, “Non-simultaneous transmission for inter-band UL CA in FR2,” Apple Inc., 3GPP RAN4 #92, August 2019

- R4-1907984, “Work plan and general issues with inter-band CA for FR2,” Qualcomm Incorporated, 3GPP RAN4 #92, August 2019

- R4-1911512, “Carrier switching to enable non-simultaneous transmission for intra-band non-contiguous UL CA in FR2,” Apple Inc., 3GPP RAN4 #92bis, October 2019

- R4-1913055, “WF on non-contiguous UL CA in FR2,” Apple Inc., 3GPP RAN4 #92bis, October 2019

- R4-1913543, “Non-simultaneous Tx for FR2 inter-band UL CA,” Apple Inc., 3GPP RAN4 #93, November 2019