3GPP TSG-RAN WG4 Meeting #93	R4-1913571

Reno, USA, 18th - 22nd Nov, 2019

Agenda item:	9.16.1.4

Source: 		NTT DOCOMO, INC.

Title: 	QCL assumptions for CSI-RS based L3 measurement

Document for:	Discussion

1. Introduction

In this contribution, we review RAN1 and RAN2 specifications [1, 2] on CSI-RS-based L3 measurement and propose our views on measurement requirements considering QCL assumptions, i.e., time and beam synchronization.

2. Discussion

Figures 1(a) and 1(b) show beam and timing synchronization assumption, which is captured in RAN1 and RAN2 specifications [1, 2].

(a) Timing synchronization

As presented in Fig.1 (a), UE determines the timing of CSI-RS reception depending on RRC parameters, i.e., associatedSSB and refServCellIndex. Depending on the RRC parameters, UE can rely CSI-RS reception timing on either of (1) cell indicated by the cellID, (2) serving cell indicated by refServCellIndex or (3) PCell.

(b) Beam synchronization

According to TS38.331 Sec. 6.3.2, UE can know whether configured CSI-RS is QCL-typeD with associatedSSB or not based on isQuasiColocated as presented in Fig.1 (b). If associatedSSB is configured and isQuasiColocated is ON, UE will deal with CSI-RS assuming QCL-typeD with associatedSSB. However, when isQuasiColocated is OFF or not configured, there is no SSB that is QCLed to the CSI-RS. In this sense, we need to discuss how to deal with the different QCL assumption in terms of measurement requirement.

Observation: When isQuasiColocated is OFF or not configured, there is no SSB that is QCLed to the CSI-RS.

Proposal: Study how to design RRM specifications when isQuasiColocated is OFF or not configured, e.g., larger scaling factor for Rx beam sweeping, no support, etc.

(a) Timing synchronization aspect

(b) Beam synchronization aspect

Fig.1 Sequence diagram on associatedSSB and isQuasiColocated

3. Conclusion

In this contribution, we reviewed RAN1 and RAN2 specifications on CSI-RS-based L3 measurement and proposed our views on measurement requirements considering QCL assumptions, i.e., time and beam synchronization.

Observation: When isQuasiColocated is OFF or not configured, there is no SSB that is QCLed to the CSI-RS.

Proposal: Study how to design RRM specifications when isQuasiColocated is OFF or not configured, e.g., larger scaling factor for Rx beam sweeping, no support, etc.

Reference

[1] TS38.214 (v.15.7.0).

[2] TS38.331 (v.15.7.0).