3GPP TSG-RAN WG4 Meeting #93	R4-1913702

Reno, USA, 18th – 22th Nov. 2019

Title: 	Discuss on the channel arrangement design for NR V2X UE

Source: 	CATT

Agenda item:	9.4.3.2

Document for:	Discussion

- Introduction

In last RAN4#92bis meeting, a way forward was approved [1]. Some requirements were reached as following:

Channel raster for band n47:

- Channel raster for n47 should be based on general channel raster definition in NR which is SCS based channel raster.

  - The co-existence of LTE V2X when the channel is shared should be considered and frequency shift is FFS.

Sync raster for band n47:

- Do not define sync raster for NR V2X band n47.

- Send LS to RAN1 and RAN2 to inform:

  - RAN4 does not define sync raster for NR V2X unlicensed bands.

Channel raster for licensed bands:

- Reuse NR Uu channel raster for sidelink in the NR V2X licensed bands.

  - The co-existence of LTE Uu when the channel is shared should be considered and frequency shift is FFS.

Sync raster for licensed bands:

- Do not define sync raster for SL interface in NR V2X licensed bands.

- Send LS to RAN1 and RAN2 to inform:

  - RAN4 does not define sync raster for NR V2X licensed bands.

From the way forward [1] some issues should be discussed and decided including how to set NR V2X UE operating frequency on band 47 following management requirements for channel plan, and how to set NR V2X UE operating frequency on band 47 for co-existence of LTE V2X when the channel is shared. This document will further discuss the channel arrangement design for NR V2X UE.

- Discussion

It is agreed that channel raster for n47 should be based on general channel raster definition in NR which is SCS based channel raster. The channel raster for NR is designed as following:

For above 3GHz, the frequency based on NR channel raster is defined as:

FREF = FREF-Offs + ΔFGlobal (NREF – NREF-Offs)	                                                                      (1)

Table 5.4.2.1-1 (TS 38.101-1): NR-ARFCN parameters for the global frequency raster

| Frequency range (MHz) | ΔFGlobal (kHz) | FREF-Offs (MHz) | NREF-Offs | Range of NREF |
| --- | --- | --- | --- | --- |
| 0 – 3000 | 5 | 0 | 0 | 0 – 599999 |
| 3000 – 24250 | 15 | 3000 | 600000 | 600000 – 2016666 |

It is discussed that the channel frequencies of ECC management is planned as 10MHz bandwidth and the operating frequencies is designed at 5860MHz, 5870MHz, 5880MHz, 5890MHz, 5900MHz, 5910MHz, and 5920MHz on band n47. The channel raster of LTE V2X is 100 kHz, and the operating frequencies may be set on these frequencies.

From above definitions, the ECC, LTE V2X, and NR V2X operating frequency is calculated as Table 1. The NR V2X channel frequency can’t be always set at ECC channel frequency. The frequency offset is listed in Table 1.

On the other hand, the mapping of NR channel raster to resource element is defined in Table 5.4.2.2-1 in TS 38.101-1. The channel frequency set by channel raster is not the centre frequency (fc) of the maximum transmission bandwidth configuration. The centre frequencies (fc) are always SCS/2 below the NR channel frequency. The fc and frequency offset of fc to ECC/LTE V2X channel frequency are also listed in Table 1. For channel 2 and channel 5, the NR V2X channel raster can be adjusted, i.e. the NREF plus 1, to set the frequency offset of fc to ECC/LTE V2X channel frequency less than 7.5kHz.

Table 1.  ECC channel, LTE V2X, and NR V2X channel frequency

|  | Channel 1 | Channel 2 | Channel 3 | Channel 4 | Channel 5 | Channel 6 | Channel 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ECC Frequency (MHz) | 5860 | 5870 | 5880 | 5890 | 5900 | 5910 | 5920 |
| LTE V2X operating frequency (MHz) | 5860 | 5870 | 5880 | 5890 | 5900 | 5910 | 5920 |
| NR V2X channel raster (NREF) | 790667 | 791333 | 792000 | 792667 | 793333 | 794000 | 794667 |
| NR V2X operating frequency | 5860.005 | 5869.995 | 5880 | 5890.005 | 5899.995 | 5910 | 5920.005 |
| Frequency offset (kHz) | +5 | -5 | 0 | +5 | -5 | 0 | +5 |
| fc for 15kHz SCS (MHz) | 5859.9975 | 5869.9875 | 5879.9925 | 5889.9975 | 5899.9875 | 5909.9925 | 5919.9975 |
| Frequency offset of fc to LTE V2X (kHz) | -2.5 | -12.5 | -7.5 | -2.5 | -12.5 | -7.5 | -2.5 |
| Adjusted channel raster | - | 791334 | - | - | 793334 | - | - |
| Adjusted fc (MHz) | - | 5870.0025 | - | - | 5900.0025 | - | - |
| Adjusted frequency offset (kHz) | -2.5 | 2.5 | -7.5 | -2.5 | 2.5 | -7.5 | -2.5 |

Table 5.4.2.2-1 (TS 38.101-1): Channel raster to resource element mapping

|  | NRBmod2 = 0 | NRBmod2 = 1 |
| --- | --- | --- |
| Resource element index | 0 | 6 |
| Physical resource block number |  |  |

From the calculation in Table 1, the frequency offset always exists, even using 7.5 kHz offset as for SUL. So the operating frequency of NR V2X can’t be set as the same frequency with ECC/LTE V2X channel, and the SCS frequency of NR V2X can't align with LTE V2X if only reusing NR Uu channel raster.

Observation: The operating frequency of NR V2X can’t be set as the same frequency with ECC/LTE V2X channel, and the SCS frequency of NR V2X can't be aligned with LTE V2X if only reusing NR Uu channel raster.

To solve this issue, some offsets for NR V2X operating frequency should be used as for SUL. It is proposed that the NR V2X operating frequency is defined as:

FREF = FREF-Offs + ΔFGlobal (NREF – NREF-Offs) + N∆f * 0.0025MHz

Here, N∆f is signalled by network configuration or pre-configuration. It can be set as one of following values：-2, -1, 0, 1, 2, 3.

By using this definition, the NR V2X channel frequency or centre frequency of the maximum transmission bandwidth configuration can align with ECC channel schedule and LTE V2X operating frequency. The SCS of NR V2X and LTE V2X can be aligned for co-existence.

Proposal: The NR V2X operating frequency is defined as:

FREF = FREF-Offs + ΔFGlobal (NREF – NREF-Offs) + N∆f * 0.0025MHz

Here, the N∆f is signalled by network configuration or pre-configuration. It can be set as one of following values：-2, -1, 0, 1, 2, 3.

For this definition, an LS needs to be sent to RAN2 for related signalling design to configure parameter N∆f.

- Conclusion

This paper gives our analysis for channel arrangement for NR V2X. Following observation and proposal are presented:

Observation: The operating frequency of NR V2X can’t be set as the same frequency with ECC/LTE V2X channel, and the SCS frequency of NR V2X can't be aligned with LTE V2X if only reusing NR Uu channel raster.

Proposal: The NR V2X operating frequency is defined as:

FREF = FREF-Offs + ΔFGlobal (NREF – NREF-Offs) + N∆f * 0.0025MHz

Here, the N∆f is signalled by network configuration or pre-configuration. It can be set as one of following values：-2, -1, 0, 1, 2, 3.

For this definition, an LS needs to be sent to RAN2 for related signal design to configure parameter N∆f.

Based on above proposals, a TP document [3] is presented in other paper.

- Reference

[1]	R4-1912880, WF for channel raster and sync raster for NR V2X for band n47 and licensed bands, vivo, CATT, LGE