3GPP TSG-RAN WG4 Meeting #93	R4-1913730

# Reno, USA, 18th - 22nd Nov., 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR | <CR#> | rev | <Rev#> | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS38.141-2: Introduce PUSCH performance requirements at 30% throughput testing point |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_perf_enh-Perf |  |  |  |  |  | Date: |  |  | 2019-11-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | PUSCH performance requirements at 30% throughput testing point should be added to the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add PUSCH performance requirements at 30% throughput testing point. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PUSCH performance requirements are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.5.1, 8.2.1.5.2, 8.2.2.5.1, 8.2.2.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

8.2	OTA performance requirements for PUSCH

8.2.1	Performance requirements for PUSCH with transform precoding disabled

8.2.1.1	Definition and applicability

The performance requirement of PUSCH is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in annex A. The performance requirements assume HARQ re-transmissions.

Which specific test(s) are applicable to BS is based on the test applicability rules defined in subclause 8.1.2.1.

8.2.1.2	Minimum Requirement

For BS type 1-O, the minimum requirement is in TS 38.104 [2], subclause 11.2.1.1.

For BS type 2-O, the minimum requirement is in TS 38.104 [2], subclause 11.2.2.1.

8.2.1.3	Test purpose

The test shall verify the receiver's ability to achieve throughput under multipath fading propagation conditions for a given SNR.

8.2.1.4	Method of test

8.2.1.4.1	Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested: M, see subclause 4.9.1.

Direction to be tested: OTA REFSENS receiver target reference direction (see D.54 in table 4.6-1).

8.2.1.4.2	Procedure

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.3.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex E.3. Each of the demodulation branch signals should be transmitted on each polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A, and according to additional test parameters listed in table 8.2.1.4.2-1.

Table 8.2.1.4.2-1: Test parameters for testing PUSCH

| Parameter |  | BS type 1-O | BS type 2-O |
| --- | --- | --- | --- |
| Transform precoding |  | Disabled |  |
| Uplink-downlink allocation for TDD |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U | 60 kHz and 120kHz SCS: 3D1S1U, S=10D:2G:2U |
| HARQ | Maximum number of HARQ transmissions | 4 |  |
|  | RV sequence | 0, 2, 3, 1 |  |
| DM-RS | DM-RS configuration type | 1 |  |
|  | DM-RS duration | single-symbol DM-RS |  |
|  | Additional DM-RS position | pos1 | {pos0, pos1} |
|  | Number of DM-RS CDM group(s) without data | 2 |  |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |  |
|  | DM-RS port(s) | {0}, {0,1} |  |
|  | DM-RS sequence generation | NID0=0, nSCID=0 |  |
| Time domain resource assignment | PUSCH mapping type | A, B | B |
|  | Start symbol | 0 | 0 |
|  | Allocation length | 14 | 10 |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |  |
|  | Frequency hopping | Disabled |  |
| TPMI index for 2Tx two layer spatial multiplexing transmission |  | 0 |  |
| Code block group based PUSCH transmission |  | Disabled |  |
| PTRS configuration | Frequency density (KPT-RS) | N.A. | 2 |
|  | Time density (LPT-RS) | N.A. | 1 |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex J.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the BS receiver is as specified in subclause 8.2.1.5.1 and 8.2.1.5.2 for BS type 1-O and BS type 2-O respectively, and that the SNR at the BS receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 8.2.1.4.2-2.

Table 8.2.1.4.2-2: AWGN power level at the BS input

| BS type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 15 | 5 | -86.5 - ΔOTAREFSENS dBm / 4.5 MHz |
|  |  | 10 | -83.3 - ΔOTAREFSENS dBm / 9.36 MHz |
|  |  | 20 | -80.2 - ΔOTAREFSENS dBm / 19.08 MHz |
|  | 30 | 10 | -83.6 - ΔOTAREFSENS dBm / 8.64 MHz |
|  |  | 20 | -80.4 - ΔOTAREFSENS dBm / 18.36 MHz |
|  |  | 40 | -77.2 - ΔOTAREFSENS dBm / 38.16 MHz |
|  |  | 100 | -73.1 - ΔOTAREFSENS dBm / 98.28 MHz |
| BS type 2-O | 60 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 47.52 MHz |
|  |  | 100 | EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz |
|  | 120 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz |
|  |  | 100 | EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz |
|  |  | 200 | EISREFSENS_50M + ΔFR2_REFSENS + 21 dBm / 190.08 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.53 in table 4.6-1 and subclause 7.1. NOTE 2: ΔFR2_REFSENS = -3 dB as described in subclause 7.1, since the OTA REFSENS reference direction (as declared in D.54 in table 4.6-1) is used for testing. NOTE 3: EISREFSENS_50M as declared in D.28 in table 4.6-1. |  |  |  |

8)	For reference channels applicable to the BS, measure the throughput.

8.2.1.5	Test Requirement

8.2.1.5.1	Test requirement for BS type 1-O

The throughput measured according to subclause 8.2.1.4.2 shall not be below the limits for the SNR levels specified in table 8.2.1.5.1-1 to table 8.2.1.5.1-14 for 1Tx and for 2Tx two layer spatial multiplexing transmission.

Table 8.2.1.5.1-1: Test requirements for PUSCH, Type A, 5 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-8 | pos1 | [-1.6] |
|  |  |  |  | 30 % | G-FR1-A3-8 | pos1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-8 | pos1 | [10.8] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-8 | pos1 | [13.2] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-22 | pos1 | [1.4] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-22 | pos1 | [19.0] |

Table 8.2.1.5.1-2: Test requirements for PUSCH, Type A, 10 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-9 | pos1 | [-1.8] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-9 | pos1 | [11.0] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-9 | pos1 | [13.2] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-23 | pos1 | [2.1] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-23 | pos1 | [19.3] |

Table 8.2.1.5.1-3: Test requirements for PUSCH, Type A, 20 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-10 | pos1 | [-1.4] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-10 | pos1 | [10.9] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-10 | pos1 | [13.3] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-24 | pos1 | [2.5] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-24 | pos1 | [19.2] |

Table 8.2.1.5.1-4: Test requirements for PUSCH, Type A, 10 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-11 | pos1 | [-2.1] |
|  |  |  |  | 30 % | G-FR1-A3-11 | pos1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-11 | pos1 | [10.8] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-11 | pos1 | [12.7] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-25 | pos1 | [1.7] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-25 | pos1 | [18.8] |

Table 8.2.1.5.1-5: Test requirements for PUSCH, Type A, 20 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-12 | pos1 | [-2.4] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-12 | pos1 | [10.7] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-12 | pos1 | [12.8] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-26 | pos1 | [1.7] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-26 | pos1 | [18.9] |

Table 8.2.1.5.1-6: Test requirements for PUSCH, Type A, 40 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-13 | pos1 | [-2.2] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-13 | pos1 | [TBD] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-13 | pos1 | [12.3] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-27 | pos1 | [1.4] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-27 | pos1 | [18.9] |

Table 8.2.1.5.1-7: Test requirements for PUSCH, Type A, 100 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-14 | pos1 | [-2.3] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-14 | pos1 | [10.8] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-14 | pos1 | [13.3] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-28 | pos1 | [1.9] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-28 | pos1 | [19.4] |

Table 8.2.1.5.1-8: Test requirements for PUSCH, Type B, 5 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-8 | pos1 | [-2.0] |
|  |  |  |  | 30 % | G-FR1-A3-8 | pos1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-8 | pos1 | [10.5] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-8 | pos1 | [12.7] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-22 | pos1 | [1.9] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-22 | pos1 | [18.7] |

Table 8.2.1.5.1-9: Test requirements for PUSCH, Type B, 10 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-9 | pos1 | [-2.0] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-9 | pos1 | [10.7] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-9 | pos1 | [13.1] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-23 | pos1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-23 | pos1 | [19.5] |

Table 8.2.1.5.1-10: Test requirements for PUSCH, Type B, 20 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-10 | pos1 | [-1.8] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-10 | pos1 | [10.7] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-10 | pos1 | [12.8] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-24 | pos1 | [1.9] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-24 | pos1 | [19.0] |

Table 8.2.1.5.1-11: Test requirements for PUSCH, Type B, 10 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-11 | pos1 | [-2.2] |
|  |  |  |  | 30 % | G-FR1-A3-11 | pos1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-11 | pos1 | [10.7] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-11 | pos1 | [12.7] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-25 | pos1 | [1.5] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-25 | pos1 | [18.9] |

Table 8.2.1.5.1-12: Test requirements for PUSCH, Type B, 20 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-12 | pos1 | [-2.4] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-12 | pos1 | [10.7] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-12 | pos1 | [12.7] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-26 | pos1 | [1.7] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-26 | pos1 | [19.0] |

Table 8.2.1.5.1-13: Test requirements for PUSCH, Type B, 40 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-13 | pos1 | [-2.2] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-13 | pos1 | [10.6] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-13 | pos1 | [12.8] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-27 | pos1 | [2.1] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-27 | pos1 | [19.1] |

Table 8.2.1.5.1-14: Test requirements for PUSCH, Type B, 100 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-14 | pos1 | [-2.2] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-14 | pos1 | [10.7] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-14 | pos1 | [13.4] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-28 | pos1 | [2.0] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-28 | pos1 | [19.6] |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

8.2.1.5.2	Test requirement for BS type 2-O

The throughput measured according to subclause 8.2.1.4.2 shall not be below the limits for the SNR levels specified in table 8.2.1.5.2-1 to 8.2.1.5.2-5.

Table 8.2.1.5.2-1: Test requirements for PUSCH, 50 MHz Channel Bandwidth, 60 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | PT-RS | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-1 | pos0 | No | [-1.4] |
|  |  |  |  |  | G-FR2-A3-13 | pos1 | No | [-1.6] |
|  |  |  |  | 30 % | G-FR2-A3-1 | pos0 | No | [TBD] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-1 | pos0 | Yes | [12.6] |
|  |  |  |  |  |  |  | No | [12.1] |
|  |  |  |  |  | G-FR2-A4-11 | pos1 | Yes | [11.3] |
|  |  |  |  |  |  |  | No | [11.3] |
|  |  | Normal | TDLA30-75 Low | 70 % | G-FR2-A5-1 | pos0 | Yes | [14.3] |
|  |  |  |  |  |  |  | No | [13.7] |
|  |  |  |  |  | G-FR2-A5-6 | pos1 | Yes | [14.0] |
|  |  |  |  |  |  |  | No | [13.5] |
| 2 |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-6 | pos0 | No | [2.3] |
|  |  |  |  |  | G-FR2-A3-18 | pos1 | No | [2.0] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-6 | pos0 | Yes | [TBD] |
|  |  |  |  |  |  |  | No | [TBD] |
|  |  |  |  |  | G-FR2-A4-16 | pos1 | Yes | [20.4] |
|  |  |  |  |  |  |  | No | [18.9] |

Table 8.2.1.5.2-2: Test requirements for PUSCH, 100 MHz Channel Bandwidth, 60 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | PT-RS | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-2 | pos0 | No | [-1.5] |
|  |  |  |  |  | G-FR2-A3-14 | pos1 | No | [-1.8] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-2 | pos0 | Yes | [12.8] |
|  |  |  |  |  |  |  | No | [11.8] |
|  |  |  |  |  | G-FR2-A4-12 | pos1 | Yes | [11.8] |
|  |  |  |  |  |  |  | No | [11.2] |
|  |  | Normal | TDLA30-75 Low | 70 % | G-FR2-A5-2 | pos0 | Yes | [14.8] |
|  |  |  |  |  |  |  | No | [13.9] |
|  |  |  |  |  | G-FR2-A5-7 | pos1 | Yes | [14.3] |
|  |  |  |  |  |  |  | No | [13.7] |
| 2 |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-7 | pos0 | No | [2.3] |
|  |  |  |  |  | G-FR2-A3-19 | pos1 | No | [2.0] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-7 | pos0 | Yes | [TBD] |
|  |  |  |  |  |  |  | No | [TBD] |
|  |  |  |  |  | G-FR2-A4-17 | pos1 | Yes | [19.6] |
|  |  |  |  |  |  |  | No | [19.1] |

Table 8.2.1.5.2-3: Test requirements for PUSCH, 50 MHz Channel Bandwidth, 120 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | PT-RS | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-3 | pos0 | No | [-1.2] |
|  |  |  |  |  | G-FR2-A3-15 | pos1 | No | [-1.5] |
|  |  |  |  | 30 % | G-FR2-A3-3 | pos0 | No | [TBD] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-3 | pos0 | Yes | [12.2] |
|  |  |  |  |  |  |  | No | [11.5] |
|  |  |  |  |  | G-FR2-A4-13 | pos1 | Yes | [11.5] |
|  |  |  |  |  |  |  | No | [11.1] |
|  |  | Normal | TDLA30-75 Low | 70 % | G-FR2-A5-3 | pos0 | Yes | [14.3] |
|  |  |  |  |  |  |  | No | [13.7] |
|  |  |  |  |  | G-FR2-A5-8 | pos1 | Yes | [13.8] |
|  |  |  |  |  |  |  | No | [13.6] |
| 2 |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-8 | pos0 | No | [2.2] |
|  |  |  |  |  | G-FR2-A3-20 | pos1 | No | [2.1] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-8 | pos0 | Yes | [21.9] |
|  |  |  |  |  |  |  | No | [19.4] |
|  |  |  |  |  | G-FR2-A4-18 | Pos1 | Yes | [20.4] |
|  |  |  |  |  |  |  | No | [18.4] |

Table 8.2.1.5.2-4: Test requirements for PUSCH, 100 MHz Channel Bandwidth, 120 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | PT-RS | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-4 | pos0 | No | [-1.8] |
|  |  |  |  |  | G-FR2-A3-16 | pos1 | No | [-1.9] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-4 | pos0 | Yes | [12.5] |
|  |  |  |  |  |  |  | No | [11.1] |
|  |  |  |  |  | G-FR2-A4-14 | pos1 | Yes | [11.7] |
|  |  |  |  |  |  |  | No | [11.1] |
|  |  | Normal | TDLA30-75 Low | 70 % | G-FR2-A5-4 | pos0 | Yes | [14.1] |
|  |  |  |  |  |  |  | No | [13.5] |
|  |  |  |  |  | G-FR2-A5-9 | pos1 | Yes | [14.0] |
|  |  |  |  |  |  |  | No | [13.4] |
| 2 |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-9 | pos0 | No | [2.2] |
|  |  |  |  |  | G-FR2-A3-21 | pos1 | No | [2.0] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-9 | pos0 | Yes | [21.6] |
|  |  |  |  |  |  |  | No | [20.2] |
|  |  |  |  |  | G-FR2-A4-19 | pos1 | Yes | [19.3] |
|  |  |  |  |  |  |  | No | [18.8] |

Table 8.2.1.5.2-5: Test requirements for PUSCH, 200 MHz Channel Bandwidth, 120 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | PT-RS | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-5 | pos0 | No | [-1.2] |
|  |  |  |  |  | G-FR2-A3-17 | pos1 | No | [-1.5] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-5 | pos0 | Yes | [11.9] |
|  |  |  |  |  |  |  | No | [11.5] |
|  |  |  |  |  | G-FR2-A4-15 | pos1 | Yes | [11.8] |
|  |  |  |  |  |  |  | No | [11.1] |
|  |  | Normal | TDLA30-75 Low | 70 % | G-FR2-A5-5 | pos0 | Yes | [14.7] |
|  |  |  |  |  |  |  | No | [13.7] |
|  |  |  |  |  | G-FR2-A5-10 | pos1 | Yes | [14.3] |
|  |  |  |  |  |  |  | No | [13.6] |
| 2 |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-10 | pos0 | No | [2.2] |
|  |  |  |  |  | G-FR2-A3-22 | pos1 | No | [1.9] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-10 | pos0 | Yes | [22.3] |
|  |  |  |  |  |  |  | No | [21.0] |
|  |  |  |  |  | G-FR2-A4-20 | pos1 | Yes | [19.8] |
|  |  |  |  |  |  |  | No | [19.0] |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

8.2.2	Performance requirements for PUSCH with transform precoding enabled

8.2.2.1	Definition and applicability

The performance requirement of PUSCH is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in annex A. The performance requirements assume HARQ re-transmissions.

Which specific test(s) are applicable to BS is based on the test applicability rules defined in subclause 8.1.2.

8.2.2.2	Minimum Requirement

For BS type 1-O, the minimum requirement is in TS 38.104 [2], subclause 11.2.1.2.

For BS type 2-O, the minimum requirement is in TS 38.104 [2], subclause 11.2.2.2.

8.2.2.3	Test Purpose

The test shall verify the receiver's ability to achieve throughput under multipath fading propagation conditions for a given SNR.

8.2.2.4	Method of test

8.2.2.4.1	Initial Conditions

Test environment:	Normal, see subclause B.2.

RF channels to be tested:	M, see subclause 4.9.1.

Direction to be tested:

-	For BS type 1-O, OTA REFSENS receiver target reference direction (see D.54 in table 4.6-1).

-	For BS type 2-O, OTA REFSENS receiver target reference direction (see D.54 in table 4.6-1).

8.2.2.4.2	Procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2.4.

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.3.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex E.3. Each of the demodulation branch signals should be transmitted on each polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A, and according to additional test parameters listed in table 8.2.2.4.2-1.

Table 8.2.2.4.2-1: Test parameters for testing PUSCH

| Parameter |  | BS type 1-O | BS type 2-O |
| --- | --- | --- | --- |
| Transform precoding |  | Enabled |  |
| Uplink-downlink allocation for TDD |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U | 60 kHz and 120kHz SCS: 3D1S1U, S=10D:2G:2U |
| HARQ | Maximum number of HARQ transmissions | 4 |  |
|  | RV sequence | 0, 2, 3, 1 |  |
| DM-RS | DM-RS configuration type | 1 |  |
|  | DM-RS duration | single-symbol DM-RS |  |
|  | Additional DM-RS position | pos1 | pos0, pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |  |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |  |
|  | DM-RS port(s) | 0 |  |
|  | DM-RS sequence generation | NID0=0, group hopping and sequence hopping are disabled |  |
| Time domain resource assignment | PUSCH mapping type | A, B | B |
|  | Start symbol | 0 | 0 |
|  | Allocation length | 14 | 10 |
| Frequency domain resource assignment | RB assignment | 15 kHz SCS: 25 PRBs in the middle of the test bandwidth 30 kHz SCS: 24 PRBs in the middle of the test bandwidth | 30 PRBs in the middle of the test bandwidth |
|  | Frequency hopping | Disabled |  |
| Code block group based PUSCH transmission |  | Disabled |  |
| PT-RS |  | Not configured |  |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex J.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the BS receiver is as specified in subclause 8.2.2.5.1 and 8.2.2.5.2 for BS type 1-O and BS type 2-O respectively, and that the SNR at the BS receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 8.2.2.4.2-2.

Table 8.2.2.4.2-2: AWGN power level at the BS input

| BS type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 15 | 5 | -86.5 - ΔOTAREFSENS dBm / 4.5 MHz |
|  | 30 | 10 | -83.6 - ΔOTAREFSENS dBm / 8.64 MHz |
| BS type 2-O | 60 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 47.52 MHz |
|  | 120 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.53 in table 4.6-1 and section 7.1. NOTE 2: ΔFR2_REFSENS = -3 dB as described in section 7.1, since the OTA REFSENS reference direction (as declared in D.54 in table 4.6-1) is used for testing. NOTE 3: EISREFSENS_50M as declared in D.28 in table 4.6-1. |  |  |  |

8)	For reference channels applicable to the BS, measure the throughput.

8.2.2.5	Test Requirement

8.2.2.5.1	Test requirement for BS type 1-O

The throughput measured according to subclause 8.2.2.4.2 shall not be below the limits for the SNR levels specified in table 8.2.2.5.1-1 to table 8.2.2.5.1-4.

Table 8.2.2.5.1-1: Test requirements for PUSCH, Type A, 5 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-31 | pos1 | [-2.1] |
|  |  |  |  | 30 % | G-FR1-A3-31 | pos1 | [TBD] |

Table 8.2.2.5.1-2: Test requirements for PUSCH, Type A, 10 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-32 | pos1 | [-2.2] |
|  |  |  |  | 30 % | G-FR1-A3-32 | pos1 | [TBD] |

Table 8.2.2.5.1-3: Test requirements for PUSCH, Type B, 5 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-31 | pos1 | [-2.0] |
|  |  |  |  | 30 % | G-FR1-A3-31 | pos1 | [TBD] |

Table 8.2.2.5.1-4: Test requirements for PUSCH, Type B, 10 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-32 | pos1 | [-2.4] |
|  |  |  |  | 30 % | G-FR1-A3-32 | pos1 | [TBD] |

8.2.2.5.2	Test requirement for BS type 2-O

The throughput measured according to subclause 8.2.2.4.2 shall not be below the limits for the SNR levels specified in table 8.2.2.5.2-1 to table 8.2.2.5.2-2.

Table 8.2.2.5.2-1: Test requirements for PUSCH, Type B, 50 MHz channel bandwidth, 60 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-11 | pos0 | [-1.2] |
|  |  |  |  |  | G-FR2-A3-23 | pos1 | [-1.3] |
|  |  |  |  | 30 % | G-FR2-A3-11 | pos0 | [TBD] |

Table 8.2.2.5.2-2: Test requirements for PUSCH, Type B, 50 MHz channel bandwidth, 120 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-12 | pos0 | [-1.2] |
|  |  |  |  |  | G-FR2-A3-24 | pos1 | [-1.3] |
|  |  |  |  | 30 % | G-FR2-A3-12 | pos0 | [TBD] |

<End of Change>