3GPP TSG-RAN WG4 Meeting #93                                                                 R4-1913734

# Reno, USA, 18th – 22nd Nov., 2019

Source: 	CATT

Title: 	TP for TR 38.883 Updated simulation results for 256QAM for FR2

Agenda item:	9.12.1

Document for:	Approval

- Introduction

This contribution provides a TP to update the link level simulation results for 256QAM for FR2.

- Text Proposal

<Start of TP 1>

5.2.1.7	Results from CATT [10]

From the Figure 5.2.1.7-1(a) and 5.2.1.7-1(b), the achievable throughput for 256QAM is worse than 64QAM at 3.5% TX EVM and 3.5% RX EVM at SNR 40dB under TDL-A and TDL-D fading channel. The results for TDL-A and TDL-D fading channel indicate that the FR2 256QAM is hard to be deployed in scenarios include Homes, Roof-above or indoor, Commercial centre or official building. From the Figure 5.2.1.7-1(c), the achievable throughput for 256QAM is better than 64QAM at 3.5% TX EVM and 3.5% RX EVM in the SNR range from 30dB to 40dB in static channel. However there may be very limited scenarios with static channel condition in practice.

| a) TDL-A | b) TDL-D |
| --- | --- |
| c) Static channel |  |

Figure 5.2.1.7-1: performance comparison of 64QAM and 256QAM in TDL-A in option b phase noise model

The performance improvement of 256QAM realative to 64QAM  at 30 and 35 dB SNR point for all Tx EVM + Rx EVM cases are summaried and shown in Table 5.2.1.7-1.

Table 5.2.1.7-1 Performance improvement of 256QAM relative to 64QAM at 29 GHz

| Channel model | Tx EVM+Rx EVM | Total EVM | SNR required to achieve gains for 256QAM [dB] | Performance improvement 256QAM relative to 64QAM at same Tx EVM + Rx EVM |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | @30dB | @35dB |
| TDL-A | 1.5%+1.5% | 2.1% | 26 | 9% | 14% |
|  | 2.5%+2.5% | 3.5% | 32 | x[Note1] | 4% |
|  | 3.5%+3.5% | 5% | NO | x | x |
|  | 4.5%+4.5% | 6.4% | NO | x | x |
| TDL-D | 1.5%+1.5% | 2.1% | 25 | 15% | 20% |
|  | 2.5%+2.5% | 3.5% | 27 | 6% | 11% |
|  | 3.5%+3.5% | 5% | NO | x | x |
|  | 4.5%+4.5% | 6.4% | NO | x | x |
| Static channel | 1.5%+1.5% | 2.1% | 20 | 33% | 33% |
|  | 2.5%+2.5% | 3.5% | 21 | 33% | 33% |
|  | 3.5%+3.5% | 5% | 22 | 30% | 33% |
|  | 4.5%+4.5% | 6.4% | 25 | 27% | 30% |
| Note 1: x denote no performance improvement. |  |  |  |  |  |

<End of TP 1>

<Start of TP 2>

5.2.1.10	Conclusion

Editor Note: this conclusion is an initial conclusion based on the initial results provided in RAN4#91meeting.Further update will be made based on the results updated in the next meeting.

Based on the simulation results and observations provided above, the following table summarizes the SNR ranges in which 256QAM shows benefit by comparing to 64QAM below in table 5.2.1.10-1.

Table 5.2.1.10-1: SNR required to achieve gains for 256QAM

| Contributor | AWGN SNR (dB) | TDL-D SNR (dB) | TDL-A SNR (dB) |
| --- | --- | --- | --- |
| China Telecom |  |  | > 25dB |
| Nokia |  | > 28dB |  |
| DoCoMo | > 21dB |  |  |
| Huawei |  | > 24dB |  |
| Ericsson |  |  | No benefit |
| CATT | > 21dB | > 27dB | > 32dB |
| Intel | > 25dB | > 30dB | > 35dB |
| Qualcomm | > 21dB | > 23dB |  |
| Average | > 21dB | > 26.4dB |  |

<End of TP 2>