3GPP TSG-RAN WG4 #93	R4-1913828

November 18th ‒ 22nd, 2019

Reno, NV, US

Agenda item:	9.1.2.1

Source: 	Qualcomm Incorporated

Title: 	NR-U single carrier interlaced waveform and emissions

Document for:	Approval

# Introduction

This contribution is a follow-on to [1].  One of the outcomes from RAN4 #92bis was to establish an emission mask from [5] as a starting point for further evaluation.  This contribution provides simulation results to characterize the achievable output power when an NR-U waveform is applied to a PA designed for WiFi at 5 GHz.  Not only is the mask from [5] studied, but also the NR general SEM as well as other masks, ACLR, and spurious emissions.  Building upon the results in [1], this contribution presents results for all possible NR-U waveforms including fully allocated, single interlace, and multiple interlace.

# Discussion

The transmit waveform, PA and front-end loss calibration, and emission requirement assumptions are listed below.  Simulation results are then presented.

## Interlaced waveform

A single 20, 40, 60, or 80 MHz channel was simulated.  For uplink PUSCH and PUCCH with uniform spacing, the following PRB-based interlace design has been simulated for the case of 20 MHz carrier bandwidth

15 kHz SCS: M=10 interlaces with N=10 or 11 PRB’s per interlace

30 kHz SCS: M=5 interlaces with N=10 or 11 PRB’s per interlace

In addition, all other possible waveforms consisting of fully allocated and partially allocated by 10-bit bitmap (i.e., 1023 waveforms exempting the empty waveform with no allocation) have also been simulated.  At this time, RAN1 has not yet agreed on a resource allocation scheme so eventually, not all of these waveforms may be allowed.

These definitions are extended for the wider channels.  Other options for non-uniform spacing have been considered in RAN1 but are outside of the scope of this contribution.  Sub-carrier spacing values of 15 kHz and 30 kHz are evaluated as are CP-OFDM and DFT-S-OFDM waveform types.  QPSK, 16QAM, 64QAM, and 256QAM modulation are also included.  Baseband WOLA shaping is modeled with 25% CP overlap, LO and IQ imbalance of -28 dBc, and CIM3 of -60 dBc are included, but no other baseband or transceiver artifacts such as CFR or DPD at the moment.

## PA and FE loss calibration

Since this study is intended for PC5 and the possibilities to reuse PA’s designed for WiFi, the PA in the simulation was calibrated against WiFi requirements according to [2] with an assumption of 3.5 dB additional front-end loss to reach the antenna port from the transmit module output.  For this particular PA, the calibration offset against a WiFi reference is 2 dB to enable deriving worst case specifications extrapolated from a typical PA.

## Emission requirements

Emission requirements have not yet been agreed for NR-U.  However, the following were checked by simulation.    These simulations only include the PA but in some instances for the higher order modulations, also model effects of the transceiver.  For emission requirements dominated by non-linear response of the transceiver and PA such as ACLR and SEM, both the transceiver and PA are modeled by a single lumped model.  Specifically, the PA is used to model both transceiver and PA nonlinearities rather than treating them independently, but with the addition of spurious terms such as LO, IQ image, and counter-IM.  However, for EVM which is especially important for higher order modulation such as 64QAM and 256QAM where the power backoff (MPR) is larger, other impairments in the transmitter chain including phase noise and transmitter nonlinearity become significant so that modeling the entire chain with a PA is no longer appropriate.  For these cases, the EVM distortion is budgeted between the PA and the transceiver with improved performance relative to QPSK required.  The additional impairments at the levels shown below in the budget have also been included in the simulation.  Alternatively, the PA could have been simulated against its distortion budget to determine its required power backoff but in this case, all identified aspects in the budget were directly included in the simulation and backoff determined to meet the entire system EVM rather than only the portion allocated to the PA.

While agreement on these has not yet been achieved, the general requirements that were simulated against include the following

- ACLR:  25.5, 27, and 30 dB

- SEM:  NR general SEM according to Table 6.5.2.2-1 of TS 38.101-1, IEEE 802.11ax Tx mask (for 60 MHz, the 40 MHz mask was scaled while maintaining a 1 MHz transition region from passband to first attenuation region), ETSI EN 301 893 Tx mask, and the figure 1 mask of [5] which is similar to the 802.11ax and ETSI masks except with a 1 MHz transition from 0 dB to -20 dB over the frequency range from N/2 to N/2 +  1 MHz where N is the nominal bandwidth.

- General spurious emissions:  -30 dBm/MHz at offsets greater than channel bandwidth + 5 MHz

- EVM:  According to Table 6.4.2.1-1 of TS 38.101

  - QPSK = 17.5%, 16QAM = 12.5%, 64QAM = 8 %, 256QAM = 3.5%

- EVM distortion budget [3],[4] for 64QAM and 256QAM

| Non-linearity source | 256QAM |  | 64QAM |  |
| --- | --- | --- | --- | --- |
|  | % | C/N | % | C/N |
| PA | 1.50% | 36.5 | 4.00% | 28.0 |
| Transmitter nonlinearity | 1.34% | 37.4 | 3.37% | 29.5 |
| LO Phase noise | 1.78% | 35.0 | 2.24% | 33.0 |
| IQ Image | 2.24% | 33.0 | 5.62% | 25.0 |
| Transmitter total | 3.50% | 29.1 | 8.00% | 21.9 |

## Simulation results

### General emission mask

The first evaluation is to compare the SEM general emission requirement.  In [6] it was proposed to apply the NR general SEM mask, but the agreed starting point from RAN4 #92bis was the proposal in [5].  For a fully allocated waveform when evaluating only these two criteria themselves to remove the influence of any other requirement, the results are plotted below as other waveform parameters are varied (modulation from QPSK, 16QAM, 64QAM, 256QAM, SCS of 15 kHz, CP-OFDM and DFT-S precoding, 20 MHz channel).  From the figure below, it can be seen that mask from [5] imposes a reduction in maximum output power of on average 1.5 dB and as much as 2.7 dB compared to the NR general SEM.

Observation:  The proposed general emission mask requirement from [5] degrades the maximum output power by as much as 2.7 dB compared to the general NR SEM mask.

Proposal:  Apply the NR general SEM mask as the general requirement.  There is no justification to degrade the uplink link budget by as much as 2.7 dB if an ETSI-type mask is not required where the UE is being operated.

### ACLR

The next evaluation is a comparison of ACLR among the options 25.5, 27, and 30 dB.  For this, all modulations, waveforms, CP-OFDM and DFT-S, 15 kHz and 30 kHz SCS were simulated.  An obvious trend observed is that ACLR of 27 dB and 30 dB degrades output power by approximately 0.5 to 1.5 dB compared to 25.5 dB.  The other observation is that the single interlace waveforms, though not readily identifiable in the diagram, show degraded output power for the same ACLR.  The amount of degradation is larger for more stringent values of ACLR.  It is anticipated that the sparse interlaced waveforms will require higher MPR than more dense waveforms.

Observation:  ACLR of 27 dB requires approximately 0.5 dB more MPR than ACLR of 25.5 dB.  ACLR of 30 dB requires approximately 1.5 dB more MPR than ACLR of 25.5 dB.  Sparse interlaced waveforms will require more MPR.

### Maximum output power

Under the most benign set of conditions, the achievable maximum output power is plotted below.  The ACLR is 25.5 dB, the general NR SEM applies as the general emission requirements, NR general spurious emissions of -30 dBm/MHz is required.  Since EVM is not checked, only QPSK modulation is included but both DFT-S and CP-OFDM are represented.  15 kHz and 30 kHz SCS are included with both full allocation and single interlace allocation.  In the lower figure, all waveforms simulated are included, distinguished by DFT-S or CP-OFDM.  It can be seen that no waveform is able to meet 20 dBm MOP and that the impact of CP-OFDM is MOP loss of approximately 0.8 dB.

Observation:  For the waveforms observed on the PA modeled, it is not possible to reach PC5 maximum output power level of 20 dBm for the most relaxed requirements.  More stringent requirements will only degrade this achievable MOP.

# Conclusion

Additional simulation results are provided to illustrate the output power achievable with single carrier NR-U waveforms to meet emission requirements reusing a PA originally designed for WiFi applications.  The results show the impact of various emission requirements such as ACLR, SEM, and EVM on the achievable maximum output power to be possibly included in an MPR specification for a PC5 20 dBm power class.

The following observations are made

Observation:  The proposed general emission mask requirement from [1] degrades the maximum output power by as much as 2.7 dB compared to the general NR SEM mask.

Observation:  ACLR of 27 dB requires approximately 0.5 dB more MPR than ACLR of 25.5 dB.  ACLR of 30 dB requires approximately 1.5 dB more MPR than ACLR of 25.5 dB.  Sparse interlaced waveforms will require more MPR.

Observation:  For the waveforms observed on the PA modeled, it is not possible to reach PC5 maximum output power level of 20 dBm for the most relaxed requirements.  More stringent requirements will only degrade this achievable MOP.

In general, it is found that more stringent emission requirements result is reduced maximum output power and therefore reduced uplink link budget.  In some cases, this may be justified for example to comply with regional regulatory requirements.  However, for general requirements it is proposed that the most relaxed requirements be defined to enable the most robust NR-U uplink operation for deployments where tighter emission requirements are not needed.  Where they are needed, it has already been agreed that an NS signaling approach can be adopted and necessarily the uplink power will be reduced accordingly.

The impact to maximum output power is even more profound since for the PA modeled in this contribution, it appears that 20 dBm can not be achieved for any waveform.  Therefore, even before the imposition of additional emission requirements, the maximum output power will already be challenged.

Proposal:  Apply the NR general SEM mask as the general requirement.  There is no justification to degrade the uplink link budget by as much as 2.7 dB if an ETSI-type mask is not required where the UE is being operated.

# Reference

- R4-1912492, “NR-U single carrier interlaced waveform and emissions,” Qualcomm Incorporated

- R4-1907961, “NR-U PA calibration,” Qualcomm Incorporated

- R4-150975, “EVM and MPR requirements in support of 64QAM UL,” Qualcomm Incorporated

- R4-163852, “TX Challenges for 256 QAM,” Qualcomm Incorporated

- R4-1911609, “NR-U – Baseline Spectral Emission Mask,” Nokia, Nokia Shanghai Bell

- R4-1912487, “NR-U general emission requirements,” Qualcomm Incorporated