3GPP TSG RAN WG4 Meeting #93	R4-1913945

Reno, Nevada, U.S., 18th-22nd Nov. 2019

Agenda item:	10.4.2

Source:	LG Electronics, SoftBank Corp.

Title:	TP for TR 37.716-21-11: MSD for EN-DC_1-8_n28

Document for:	Approval

1	Introduction

Then new EN-DC of 2B LTE and 1B NR of DC_1-8_n28 was approved in RAN#85[1]. The corresponding TP was approved in RAN4 #92BIS meeting. In this paper we propose the MSD test configuration and required MSD levels for the EN-DC_1-8_n28.

2.   Proposed text for TR37.716-21-11:

[Unchanged Parts Skipped]

5.1.73	DC_1-8_n28

### 5.1.73.1	Operating bands for DC_1-8_n28

Table 5.1.73.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-8_n28 | CA_1-8 | n28 | No |

### 5.1.73.2	Configurations for DC_1-8_n28

Table 5.1.73.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-8A_n28A | DC_1A_n28A DC_8A_n28A | CA_1A-8A | n28A |

### 5.1.73.3	Co-existence studies

### When Uplink EN-DC configuration is DC_1_n28, 5th IMD will fall into Rx band of Band 8. When Uplink EN-DC configuration is DC_8_n28, 5th IMD will fall into Rx band of Band 1.

### However, when consider the 5MHz UL channel BW for DC_8_n28, the IMD5 was not fall into the B1 own Rx band.

### 5.1.73.4	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.1.73.4-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-8_n28 | 1 | 0.3 |
|  | 8 | 0.6 |
|  | n28 | 0.6 |

Table 5.1.73.4-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-8_n28 | 1 | 0 |
|  | 8 | 0.2 |
|  | n28 | 0.2 |

### 5.1.73.5	REFSENS requirements

As mentioned in 5.1.73.3, IMD5 of B1 and n28 to Band 8 Rx need to be addressed for REFSENS relaxation. The following values are proposed:

Table 5.1.73.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order | Single UL allowed |
| DC_1A-8A_n28A | 1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |  |
|  | n28 | 730 | 5 | 25 | 785 | N/A | FDD | N/A |  |
|  | 8 | 905 | 5 | 25 | 950 | 3.3 | FDD | IMD5 |  |
| DC_1A-8A_n28A | 8 | 882.5 | 5 | 25 | 927.5 | N/A | FDD | N/A |  |
|  | n28 | 745.5 | 5 | 25 | 800.5 | N/A | FDD | N/A |  |
|  | 1 | 1922.5 | 5 | 25 | 2112.5 | 0 | FDD | IMD5 |  |

[End of changes]