3GPP TSG RAN WG4 #93 meeting                   	R4-1913952

Reno, Nevada, U.S., 18th – 22nd Nov., 2019

Agenda Item:	9.4.1

Source:	LG Electronics

Title:	Consideration on candidate RF architecture for 5G V2X UE

Document for:	Approval

- Introduction

In the last RAN4 #92 meetings, RAN4 agreed how to specify the V2X UE RF requirements [1] according to NR V2X UE at 5.9GHz or licensed bands which was dependant NR V2X coexistence evaluation results [2]. Currently RAN4 only agreed that NR V2X UE can coexist with DSRC and LTE V2X at 5.9GHz.

In this paper, we provide our views what is baseline RF architecture for NR V2X UE at both unlicensed and licensed operating bands to specify the RF requirements and V2X operating scenarios.

- Basic RF architecture for NR V2X UE

For LTE-based V2X service in Rel-14, RAN4 can consider example RF architecture as shown in Figure 1.

- Example RF architecture for NR Uu(Band X) + NR SL (n47) operation

(b) Example RF architecture for NR Uu(Band X) + NR SL (Band X) operation

Figure 1. Example V2X UE RF architectures for con-current operation

In the RAN4 meeting, the following priority are discussed for V2X operating scenarios as following

- 1st priority of NR V2X operation:

- Specify RF core requirements in the ITS spectrum,

- Specify RF core requirements for NR SL (at n47) and LTE SL (at B47) as TDM for Tx transmission and simultaneous receptions,

- Specify RF core requirements for licensed bands in which the entire band is allocated for SL operation in a region.

    - Sidelink operation in partial bandwidth in licensed band is not precluded in WI

- 2nd priority in con-current operation (with ITS spectrum):

- NR V2X PC5 (at n47) + NR/LTE Uu (at licensed bands) on inter-band con-current operation

  - NR V2X PC5 (at n47) + NR Uu(at licensed bands)

  - NR V2X PC5 (at n47) + LTE Uu(at licensed bands)

- NR V2X SL (at licensed band Y) + LTE/NR Uu (at licensed band X) on inter-band con-current operation

  - NR V2X PC5 (at licensed band Y) + NR Uu(at licensed band X)

  - NR V2X PC5 (at licensed band Y) + LTE Uu(at licensed band X)

- 3rd Priority for con-current operation (without ITS spectrum):

- LTE Band 47 PC5 + NR Uu (at licensed bands) : need to study on RAN4 impact when NR Uu control the LTE sidelink operation since the agreed WID did not reflect RAN4 impact

- NR V2X PC5 (at licensed band) + LTE/NR Uu (at licensed bands) on inter-band or intra-band combination

  - NR V2X PC5 (at licensed band X) + LTE Uu (at licensed band X)

  - NR V2X PC5 (at licensed band X) + NR Uu (at licensed band X)

  - NR V2X con-current operation in licensed bands

Based on the above RF architecture and operating scenarios, the intra-band WAN-V2X operation in same carrier was not allowed the simultaneous transmission since Tx chain between NR Uu and V2X PC5 is shared. So only TDM operation is available. Also the adjacent carrier V2X con-current operation in Figure 2-(a) is expected the self-interference problem between V2X SL and Uu UL transmission when the SL operating band is close to the Uu UL frequency band.

Therefore, RAN4 only allow TDM operation in same carrier at licensed band.  But the V2X con-current operation for inter-band WAN-V2X UE is possible if the Uu UL frequency band X is far away from the V2X PC5 operating band Y as following in Figure 2-(b).

However, in coexistence evaluation results in previous RAN4 meeting, RAN4 do not allow the SL operating in licensed bands when SL carrier and NR/LTE Uu carrier are located in adjacent frequency.

So the RAN4 just allow the inter-band V2X con-current operating bands based on these frequency separation between Uu band X and SL band Y. The frequency separation is depend on the attenuation level of filter performance of Band X and Band Y.

- Problem of V2X SL reception by UL Tx leakage of NR V2X UE at adjacent carrier

- Feasibility of V2X con-current operation at inter band V2X operation

Figure 2. Consideration on intra-band con-current operation for NR V2X UE at licensed band

For the TDD case, additional Tx/Rx chain is not necessary in same carrier. TDM operation with shared RF chain will be consider in intra-band V2X con-current operation. But if RAN4 consider different carrier in intra-band, then RAN4 need to consider power imbalance, different numerology issue and synch source problem between Uu UL transmission and V2X SL transmission even if RAN4 consider separate RF chain. Therefore, we recommend that RAN4 only consider TDM operation between Uu UL transmission and V2X SL transmission in intra-band con-current V2X operation.

However, the same carrier or adjacent carrier operation between NR SL operation and NR Uu operation did not allow based on coexistence evaluation results.

For inter-band V2X con-current operation in TDD case, RAN4 consider different RF chain. Hence same principle (frequency separation between Uu UL transmission and V2X SL transmission) in FDD case will be considered when RAN4 allow the inter-band con-current V2X operation.

Based on above analysis, we propose as follow

Proposal 1: For both FDD/ TDD intra-band con-current operation, SL operation did not allow in licensed bands based on coexistence evaluation results when SL carrier and NR/LTE Uu carrier are located in adjacent frequency.

Proposal 2: For both FDD/ TDD inter-band con-current operation, RAN4 carefully check the frequency separation between V2X SL operation and Uu uplink transmission. Then, the inter-band con-current V2X operating bands will be allowed for both cases.

- NR V2X SL (at n47) + NR/LTE Uu (at licensed bands) on inter-band con-current operation

- NR V2X SL (at licensed band Y) + LTE/NR Uu (at licensed band X) on inter-band con-current operation. The Band Y should be used for SL operation only.

  - Clarification of general NR V2X UE RF parameters

In [3], some questions are raised in previous RAN4 meeting. So RAN4 need to make some common understanding for the general parameters of NR V2X UE.

Based on the above RF architecture and question in [3], we provide common RF parameters for NR V2X UE in rel-16.

    - Requirements

It is commonly understood that LTE and NR SL are to be received simultaneously. It is also understood that LTE and NR SL Tx are to operate in TDM manner.

Question 1: What is the assumption on the number of antennas for each of LTE and NR Rx and Tx?

Answer 1: Basically, RAN4 assume same as LTE V2X UE with 1Tx and 2Rx RF architecture for NR V2X UE at n47. Also RAN4 can consider 2Tx/2Rx RF architecture in some specific licensed band. It is depend on band specific issue and RAN4 will decide the RF architecture for each licensed band if needed.

Question 2: What is the assumption of the total number of antennas available for V2X operation on the cars?

Answer 2: Currently, RAN4 assume same as LTE V2X UE with 1Tx and 2Rx RF architecture for NR V2X operation on the car at n47. We do not decide total number of antenna for V2X operation on the cars. It is depend on Vehicular UE implementation issues.

Question 3: Is the assumption that multiple antenna transmission needs to be supported on both LTE and NR?

Answer 3: In rel-16, RAN4 only consider single antenna transmission for NR V2X service.  RAN4 will assume the multiple antenna transmission in enhanced NR V2X WI as phased approach.

    - Unlicensed band operation for LTE and NR SL

If both LTE and NR SL operate in 5.9 GHz unlicensed band, then:

Question 4: Is the assumption that same Tx chain (modem and RF) will be shared across LTE and NR SL?

Answer 4: It is up to UE implementation that NR V2X RF chain and LTE V2X RF chain can be shared or independently according to supporting both NR/LTE V2X chipset or not. The NR/LTE V2X UE just follow the individual RF requirements according to RAT.

Question 5: Is the assumption that a different RF and modem Tx chain will be used for LTE and NR SL?

Answer 5: Similar view as answer 4. It is depend on V2X UE implementation

Question 6: Do we need to create capability to signal this information (answers to questions 4 and 5) to gNB?

Answer 6: RAN4 would not create UE capability on the same/different RF and modem Tx chain. In ITS spectrum, The NR/LTE V2X UE just follow the individual RF requirements according to RAT. It is just one candidate scenarios for NR V2X operation by gNB based control.

    - CA operation with LTE or NR Uu

In case of a CA operation between LTE or NR Uu:

Question 7: Is the assumption that same Tx chain (modem and RF) will be shared across NR SL and LTE / NR Uu?

Answer 7: In Intra-band con-current operation, RAN4 assume share Tx chain as TDM operation. However, it is not allowed based on co-existence evaluation results. In inter-band con-current operation, RAN4 assume different Tx chain with FDM manner. But these combination can be allowed based on analysis results of self desense problem.

Question 8: Is the assumption that a different RF and modem Tx chain will be used for NR SL and LTE / NR Uu?

Answer 8: Same view as answer 7.

Question 9: Do we need to create capability to signal this information (answers to questions 7 and 8) to gNB?

Answer 9: RAN4 only consider the inter-band con-current operation. The baseline RF architecture can assume the different RF Tx chain. So do not need to define UE capability signaling.

Above clarification points, RAN4 make has common understanding on the UE RF parameters in rel-16 as following proposals.

Proposal 3: The baseline antenna configuration is 1Tx/2Rx in both licensed band and n47. RAN4 would decide the specific antenna configuration for each licensed band if needed.

Proposal 4: RAN4 will assume the multiple antenna transmission in enhanced NR V2X WI as phased approach.

Proposal 5: RAN4 would not create UE capability on the same/different RF and modem Tx chain. In ITS spectrum, the NR/LTE V2X UE just follow the individual RF requirements according to RAT.

Proposal 6: For inter-band con-current operation, the baseline RF architecture assume the different RF Tx chain between NR SL operation and NR Uu operation. So do not need to define UE capability signalling.

- Conclusions

In this contribution, we provide our views on which basic RF architecture will be consider the V2X con-current operation both licensed bands and unlicensed band.

Above analysis in session 2, we propose as following

Proposal 1: For both FDD/ TDD intra-band con-current operation, SL operation did not allow in licensed bands based on coexistence evaluation results when SL carrier and NR/LTE Uu carrier are located in adjacent frequency.

Proposal 2: For both FDD/ TDD inter-band con-current operation, RAN4 carefully check the frequency separation between V2X SL operation and Uu uplink transmission. Then, the inter-band con-current V2X operating bands will be allowed for both cases.

- NR V2X SL (at n47) + NR/LTE Uu (at licensed bands) on inter-band con-current operation

- NR V2X SL (at licensed band Y) + LTE/NR Uu (at licensed band X) on inter-band con-current operation. The Band Y should be used for SL operation only.

Proposal 3: The baseline antenna configuration is 1Tx/2Rx in both licensed band and n47. RAN4 would decide the specific antenna configuration for each licensed band if needed.

Proposal 4: RAN4 will assume the multiple antenna transmission in enhanced NR V2X WI as phased approach.

Proposal 5: RAN4 would not create UE capability on the same/different RF and modem Tx chain. In ITS spectrum, the NR/LTE V2X UE just follow the individual RF requirements according to RAT.

Proposal 6: For inter-band con-current operation, the baseline RF architecture assume the different RF Tx chain between NR SL operation and NR Uu operation. So do not need to define UE capability signalling.

Reference

[1] R4-1913031, “NR V2X adhoc meeting minutes,” LG Electronics

[2] R4-1913066, “TP to TR 38.886 for coexistence simulation results,” LG Electronics

[3] R4-1912477, “RF Requirements for V2X,” Qualcomm