3GPP TR 37.716-21-21 V0.8.0 (2019-11)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Networks;

E-UTRA-NR Dual Connectivity (EN-DC) of x bands (x=1, 2, 3, 4)  LTE inter-band CA (x DL/1 UL) and 2 bands NR Inter-band CA  (2 DL/1 UL) band combinations

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<keyword[, keyword]>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2018, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	17

1	Scope	18

2	References	24

3	Definitions, symbols and abbreviations	25

3.1	Definitions	25

3.2	Symbols	25

3.3	Abbreviations	25

4	Background	25

4.1	TR Maintenance	25

5	EN-DC band combinations of LTE x bands DL/1UL(x=1,2,3,4) + NR 2 bands DL/1UL: General Part	26

5.1	UE RF architectures	26

5.2 	General treatment of ∆TIB and ∆RIB values	26

5.3	summary of self-interference analysis	26

6	EN-DC band combinations of LTE 1 band DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part	89

6.1	DC_3A_n1A-n77A	89

6.1.1	Operating bands for DC	89

6.1.2	Channel bandwidths per operating band for DC	90

6.1.3	Co-existence studies	90

6.1.4	∆TIB and ∆RIB values	91

6.1.5	MSD	92

6.2	DC_1A-n78A-n257	92

6.2.1	Operating bands for DC	92

6.2.2	Channel bandwidths per operating band for DC	93

6.2.3	Co-existence studies	93

6.2.4	∆TIB and ∆RIB values	94

6.2.5	MSD	94

6.3	DC_3-n78-n257	94

6.3.1	Operating bands for DC	94

6.3.2	Channel bandwidths per operating band for DC	94

6.3.3	Co-existence studies	97

6.3.4	∆TIB and ∆RIB values	97

6.3.5	MSD	97

6.4	DC_5A-n78A-n257	97

6.4.1	Operating bands for DC	97

6.4.2	Channel bandwidths per operating band for DC	97

6.4.3	Co-existence studies	98

6.4.4	∆TIB and ∆RIB values	99

6.4.5	MSD	99

6.5	DC_7A-n78A-n257	99

6.5.1	Operating bands for DC	99

6.5.2	Channel bandwidths per operating band for DC	99

6.5.3	Co-existence studies	100

6.5.4	∆TIB and ∆RIB values	101

6.5.5	MSD	101

6.6	DC_7A-7A_n78A-n257	101

6.6.1	Operating bands for DC	101

6.6.2	Channel bandwidths per operating band for DC	101

6.6.3	Co-existence studies	102

6.6.4	∆TIB and ∆RIB values	103

6.6.5	MSD	103

6.7	DC_1A_n3A-n78A	103

6.7.1	Operating bands for DC	103

6.7.2	Channel bandwidths per operating band for DC	104

6.7.3	Co-existence studies	104

6.7.4	∆TIB and ∆RIB values	104

6.7.5	MSD	104

6.8	DC_3_n1-n78	105

6.8.1	Operating bands for DC	105

6.8.2	Channel bandwidths per operating band for DC	105

6.8.3	Co-existence studies	106

6.8.4	∆TIB and ∆RIB values	107

6.8.5	MSD	107

6.9	DC_3A_n7A-n78A	108

6.9.1	Operating bands for DC	108

6.9.2	Channel bandwidths per operating band for DC	108

6.9.3	Co-existence studies	108

6.9.4	∆TIB and ∆RIB values	109

6.9.5	MSD	109

6.10	DC_3C_n7A-n78A	110

6.10.1	Operating bands for DC	110

6.10.2	Channel bandwidths per operating band for DC	110

6.10.3	Co-existence studies	110

6.10.4	∆TIB and ∆RIB values	110

6.10.5	MSD	110

6.11	DC_13A_n5A-n260A	110

6.11.1	Operating bands for DC	110

6.11.2	Channel bandwidths per operating band for DC	111

6.11.3	Co-existence studies	111

6.11.4	∆TIB and ∆RIB values	111

6.11.5	MSD	111

6.12	DC_66A_n5A-n260A	112

6.12.1	Operating bands for DC	112

6.12.2	Channel bandwidths per operating band for DC	112

6.12.3	Co-existence studies	112

6.12.4	∆TIB and ∆RIB values	112

6.12.5	MSD	112

6.13	DC_3A_n20A-n78A	113

6.13.1	Operating bands for DC	113

6.13.2	Channel bandwidths per operating band for DC	113

6.13.3	Co-existence studies	113

6.13.4	∆TIB and ∆RIB values	113

6.13.5	MSD	114

6.14	DC_28A_n8A-n78A	114

6.14.1	Operating bands for DC	114

6.14.2	Channel bandwidths per operating band for DC	114

6.14.3	Co-existence studies	114

6.14.4	∆TIB and ∆RIB values	114

6.14.5	MSD	115

6.15	DC_28A_n8A-n258A	115

6.15.1	Operating bands for DC	115

6.15.2	Channel bandwidths per operating band for DC	115

6.15.3	Co-existence studies	116

6.15.4	∆TIB and ∆RIB values	116

6.15.5	MSD	116

6.16	DC_3C_n28A-n78A	116

6.16.1	Operating bands for DC	116

6.16.2	Channel bandwidths per operating band for DC	117

6.16.3	Co-existence studies	117

6.16.4	∆TIB and ∆RIB values	117

6.16.5	MSD	118

6.17	DC_7C_n28A-n78A	118

6.17.1	Operating bands for DC	118

6.17.2	Channel bandwidths per operating band for DC	119

6.17.3	Co-existence studies	119

6.17.4	∆TIB and ∆RIB values	119

6.17.5	MSD	120

6.18	DC_66_n71-n260	120

6.18.1	Operating bands for DC	120

6.18.2	Channel bandwidths per operating band for DC	120

6.18.3	Co-existence studies	121

6.18.4	∆TIB and ∆RIB values	121

6.18.5	MSD	121

6.19	DC_66A-n71A-n261	122

6.19.1	Operating bands for DC	122

6.19.2	Channel bandwidths per operating band for DC	122

6.19.3	Co-existence studies	122

6.19.4	∆TIB and ∆RIB values	122

6.19.5	MSD	123

6.20	Void	123

6.21	DC_1A_n7A-n78A	123

6.21.1	Operating bands for DC	123

6.21.2	Channel bandwidths per operating band for DC	123

6.21.3	Co-existence studies	124

6.21.4	∆TIB and ∆RIB values	124

6.21.5	MSD	124

6.22	DC_8_n77-n257	125

6.22.1	Operating bands for DC	125

6.22.2	Channel bandwidths per operating band for DC	125

6.22.3	Co-existence studies	125

6.22.4	∆TIB and ∆RIB values	125

6.22.5	MSD	126

6.23	DC_1A_n40A-n78A	126

6.23.1	Operating bands for DC	126

6.23.2	Channel bandwidths per operating band for DC	126

6.23.3	Co-existence studies	127

6.23.4	∆TIB and ∆RIB values	127

6.24	DC_3_n1-n79	127

6.24.1	Operating bands for DC	127

6.24.2	Channel bandwidths per operating band for DC	128

6.24.3	Co-existence studies	128

6.24.4	∆TIB and ∆RIB values	131

6.25	DC_3_n40-n78	132

6.25.1	Operating bands for DC	132

6.25.2	Channel bandwidths per operating band for DC	132

6.25.3	Co-existence studies	133

6.25.4	∆TIB and ∆RIB values	133

6.26	DC_66_n25-n41	134

6.26.1	Operating bands for DC	134

6.26.2	Channel bandwidths per operating band for DC	134

6.26.3	Co-existence studies	134

6.26.4	∆TIB and ∆RIB values	136

6.27	DC_3A_n1A-n257A	138

6.27.1	Operating bands for DC	138

6.27.2	Channel bandwidths per operating band for DC	138

6.27.3	Co-existence studies	138

6.27.4	∆TIB and ∆RIB values	141

6.27.5	MSD	142

6.28	DC_1_n77-n258	142

6.28.1	Operating bands for DC	142

6.28.2	Channel bandwidths per operating band for DC	142

6.28.3	Co-existence studies	142

6.28.4	∆TIB and ∆RIB values	142

6.28.5	MSD	143

6.29	DC_1_n78-n258	143

6.29.1	Operating bands for DC	143

6.29.2	Channel bandwidths per operating band for DC	143

6.29.3	Co-existence studies	143

6.29.4	∆TIB and ∆RIB values	143

6.29.5	MSD	144

6.30	DC_1_n79-n258	144

6.30.1	Operating bands for DC	144

6.30.2	Channel bandwidths per operating band for DC	144

6.30.3	Co-existence studies	145

6.30.4	∆TIB and ∆RIB values	145

6.30.5	MSD	145

6.31	DC_3_n77-n258	145

6.31.1	Operating bands for DC	145

6.31.2	Channel bandwidths per operating band for DC	146

6.31.3	Co-existence studies	146

6.31.4	∆TIB and ∆RIB values	146

6.31.5	MSD	146

6.32	DC_3_n78-n258	147

6.32.1	Operating bands for DC	147

6.32.2	Channel bandwidths per operating band for DC	147

6.32.3	Co-existence studies	147

6.32.4	∆TIB and ∆RIB values	147

6.32.5	MSD	148

6.33	DC_3_n79-n258	148

6.33.1	Operating bands for DC	148

6.33.2	Channel bandwidths per operating band for DC	148

6.33.3	Co-existence studies	148

6.33.4	∆TIB and ∆RIB values	148

6.33.5	MSD	149

6.34	DC_3-3_n78_n257	149

6.34.1	Operating bands for DC	149

6.34.2	Channel bandwidths per operating band for DC	149

6.34.3	Co-existence studies	151

6.34.4	∆TIB and ∆RIB values	151

6.34.5	MSD	151

6.35	DC_20_n3-n78	151

6.35.1	Operating bands for DC	151

6.35.2	Channel bandwidths per operating band for DC	152

6.35.3	Co-existence studies	152

6.35.4	∆TIB and ∆RIB values	152

6.35.5	MSD	152

6.36	DC_7_n1-n78	153

6.36.1	Operating bands for DC	153

6.36.2	Channel bandwidths per operating band for DC	153

6.36.3	Co-existence studies	154

6.36.4	∆TIB and ∆RIB values	154

6.36.5	MSD	154

6.37	DC_20_n1-n78	155

6.37.1	Operating bands for DC	155

6.37.2	Channel bandwidths per operating band for DC	155

6.37.3	Co-existence studies	155

6.37.4	∆TIB and ∆RIB values	155

6.37.5	MSD	156

6.38	DC_7_n3-n78	156

6.38.1	Operating bands for DC	156

6.38.2	Channel bandwidths per operating band for DC	156

6.38.3	Co-existence studies	156

6.38.4	∆TIB and ∆RIB values	157

6.38.5	MSD	157

6.39	   DC_40A_n41A-n79A	158

6.39.1	 Operating bands for DC	158

6.39.2	 Channel bandwidths per operating band for DC	158

6.39.3	 Co-existence studies	158

6.39.4	 ∆TIB and ∆RIB values	158

6.39.5	 MSD	159

6.40	DC_3-3_n1-n78	159

6.40.1	Operating bands for DC	159

6.40.2	Channel bandwidths per operating band for DC	159

6.40.3	Co-existence studies	160

6.40.4	∆TIB and ∆RIB values	160

6.40.5	MSD	160

6.41	DC_7-7_n1-n78	161

6.41.1	Operating bands for DC	161

6.41.2	Channel bandwidths per operating band for DC	161

6.41.3	Co-existence studies	161

6.41.4	∆TIB and ∆RIB values	161

6.41.5	MSD	162

6.42	DC_1_n5-n78	162

6.42.1	Operating bands for DC	162

6.42.2	Channel bandwidths per operating band for DC	162

6.42.3	Co-existence studies	162

6.42.4	∆TIB and ∆RIB values	162

6.42.5	MSD	163

6.43	DC_3_n5-n78	163

6.43.1	Operating bands for DC	163

6.43.2	Channel bandwidths per operating band for DC	163

6.43.3	Co-existence studies	164

6.43.4	∆TIB and ∆RIB values	164

6.43.5	MSD	164

6.44	DC_7_n5-n78	165

6.44.1	Operating bands for DC	165

6.44.2	Channel bandwidths per operating band for DC	165

6.44.3	Co-existence studies	165

6.44.4	∆TIB and ∆RIB values	165

6.44.5	MSD	166

6.45	DC_28_n5-n78	166

6.45.1	Operating bands for DC	166

6.45.2	Channel bandwidths per operating band for DC	166

6.45.3	Co-existence studies	167

6.45.4	∆TIB and ∆RIB values	167

6.45.5	MSD	167

6.46	DC_2A_n41A-n261A	168

6.46.1	Operating bands for DC	168

6.46.2	Channel bandwidths per operating band for DC	168

6.46.3	Co-existence studies	168

6.46.4	∆TIB and ∆RIB values	168

6.46.5	MSD	169

6.47	DC_66A_n41A-n261A	169

6.47.1	Operating bands for DC	169

6.47.2	Channel bandwidths per operating band for DC	169

6.47.3	Co-existence studies	169

6.47.4	∆TIB and ∆RIB values	169

6.47.5	MSD	170

6.48	DC_2_n41-n260	170

6.48.1	Operating bands for DC	170

6.48.2	Channel bandwidths per operating band for DC	170

6.48.3	Co-existence studies	171

6.48.4	∆TIB and ∆RIB values	171

6.48.5	MSD	171

6.49	DC_66_n41-n260	172

6.49.2	Channel bandwidths per operating band for DC	172

6.49.3	Co-existence studies	172

6.49.4	∆TIB and ∆RIB values	172

6.49.5	MSD	173

6.50	DC_3_n77-n257	173

6.50.1	Operating bands for DC	173

6.50.2	Channel bandwidths per operating band for DC	174

6.50.3	Co-existence studies	174

6.50.4	∆TIB and ∆RIB values	175

6.50.5	MSD	175

6.51	DC_8_n3-n28	175

6.51.1	Operating bands for DC	175

6.51.2	Channel bandwidths per operating band for DC	176

6.51.3	Co-existence studies	176

6.51.4	∆TIB and ∆RIB values	176

6.52	DC_11_n77-n257	177

6.52.1	Operating bands for DC	177

6.52.2	Channel bandwidths per operating band for DC	178

6.52.3	Co-existence studies	179

6.52.4	∆TIB and ∆RIB values	179

6.52.5	MSD	179

6.53	 DC_3A_n40A-n41A	179

6.53.1	Operating bands for DC	179

6.53.2	Channel bandwidths per operating band for DC	180

6.53.3	Co-existence studies	180

6.53.4	∆TIB and ∆RIB values	180

6.53.5	MSD	180

6.54	DC_8A_n1A-n78A	181

6.54.1	Operating bands for DC	181

6.54.2	Channel bandwidths per operating band for DC	181

6.54.3	Co-existence studies	181

6.54.4	∆TIB and ∆RIB values	183

6.54.5	MSD	183

6.55	DC_66_n41-n71	183

6.55.1	Operating bands for DC	183

6.55.2	Channel bandwidths per operating band for DC	183

6.55.3	Co-existence studies	184

6.55.4	∆TIB and ∆RIB values	184

6.55.5	MSD	184

6.56	DC_8_n78-n257	185

6.56.1	Operating bands for DC	185

6.56.2	Channel bandwidths per operating band for DC	185

6.56.3	Co-existence studies	187

6.56.4	∆TIB and ∆RIB values	187

6.56.5	MSD	187

6.57	DC_2_n41-n71	187

6.57.1	Operating bands for DC	187

6.57.2	Channel bandwidths per operating band for DC	187

6.57.3	Co-existence studies	188

6.57.4	∆TIB and ∆RIB values	188

6.57.5	MSD	188

6.58	DC_1_n3-n28	189

6.58.1	Operating bands for DC	189

6.58.2	Channel bandwidths per operating band for DC	189

6.58.3	Co-existence studies	189

6.58.4	∆TIB and ∆RIB values	189

6.58.5	MSD	190

6.59 DC_39_n41-n79	190

6.60	DC_12_n7-n78	191

6.61	DC_2_n66-n78	191

6.62	DC_12_n66-n78	192

6.63	DC_7_n66-n78	192

6.64	DC_20_n1-n28	193

6.65	DC_20_n7-n28	193

6.66	DC_20_n7-n78	194

6.67	DC_3_n7-n28	194

6.68	DC_18_n3-n78	195

6.69	DC_3_n40-n79	195

6.70	DC_3_n41-n79	195

6.71	DC_8_n40-n79	196

6.72	DC_8_n41-n79	196

6.73	DC_39_n40-n79	197

6.74	DC_7-7_n66-n78	197

7	EN-DC band combinations of LTE 2 bands DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part	199

7.1	DC_1A-3A_n78A-n257	199

7.1.1	Operating bands for DC	199

7.1.2	Channel bandwidths per operating band for DC	199

7.1.3	Co-existence studies	201

7.1.4	∆TIB and ∆RIB values	201

7.1.5	MSD	201

7.2	DC_1A-5A_n78A-n257	202

7.2.1	Operating bands for DC	202

7.2.2	Channel bandwidths per operating band for DC	202

7.2.3	Co-existence studies	204

7.2.4	∆TIB and ∆RIB values	204

7.2.5	MSD	204

7.3	DC_1A-7A_n78A-n257	205

7.3.1	Operating bands for DC	205

7.3.2	Channel bandwidths per operating band for DC	205

7.3.3	Co-existence studies	207

7.3.4	∆TIB and ∆RIB values	207

7.3.5	MSD	207

7.4	DC_1A-7A-7A_n78A-n257	207

7.4.1	Operating bands for DC	207

7.4.2	Channel bandwidths per operating band for DC	208

7.4.3	Co-existence studies	210

7.4.4	∆TIB and ∆RIB values	210

7.4.5	MSD	210

7.5	DC_3A-5A_n78A-n257	210

7.5.1	Operating bands for DC	210

7.5.2	Channel bandwidths per operating band for DC	211

7.5.3	Co-existence studies	213

7.5.4	∆TIB and ∆RIB values	213

7.5.5	MSD	213

7.6	DC_3A-7A_n78A-n257	214

7.6.1	Operating bands for DC	214

7.6.2	Channel bandwidths per operating band for DC	214

7.6.3	Co-existence studies	216

7.6.4	∆TIB and ∆RIB values	216

7.6.5	MSD	216

7.7	DC_3A-7A-7A_n78A-n257	216

7.7.1	Operating bands for DC	216

7.7.2	Channel bandwidths per operating band for DC	217

7.7.3	Co-existence studies	219

7.7.4	∆TIB and ∆RIB values	219

7.7.5	MSD	219

7.8	DC_5A-7A_n78A-n257	219

7.8.1	Operating bands for DC	219

7.8.2	Channel bandwidths per operating band for DC	220

7.8.3	Co-existence studies	222

7.8.4	∆TIB and ∆RIB values	222

7.8.5	MSD	222

7.9	DC_5A-7A-7A_n78A-n257	222

7.9.1	Operating bands for DC	222

7.9.2	Channel bandwidths per operating band for DC	223

7.9.3	Co-existence studies	225

7.9.4	∆TIB and ∆RIB values	225

7.9.5	MSD	225

7.10	DC_1A-3C_n28A-n78A	226

7.10.1	Operating bands for DC	226

7.10.2	Channel bandwidths per operating band for DC	226

7.10.3	Co-existence studies	226

7.10.4	∆TIB and ∆RIB values	226

7.10.5	MSD	227

7.11	DC_1A-7C_n28A-n78A	227

7.11.1	Operating bands for DC	227

7.11.2	Channel bandwidths per operating band for DC	227

7.11.3	Co-existence studies	227

7.11.4	∆TIB and ∆RIB values	228

7.11.5	MSD	228

7.12	DC_1-3_n7-n78	228

7.12.1	Operating bands for DC	228

7.12.2	Channel bandwidths per operating band for DC	229

7.12.3	Co-existence studies	229

7.12.4	∆TIB and ∆RIB values	229

7.12.5	MSD	230

7.13	DC_3-7_n28-n78	230

7.13.1	Operating bands for DC	230

7.13.2	Channel bandwidths per operating band for DC	231

7.13.3	Co-existence studies	231

7.13.4	∆TIB and ∆RIB values	231

7.13.5	MSD	232

7.14	DC_1-8_n77-n257	232

7.14.1	Operating bands for DC	232

7.14.2	Channel bandwidths per operating band for DC	233

7.14.3	Co-existence studies	235

7.14.4	∆TIB and ∆RIB values	235

7.14.5	MSD	235

7.15	DC_1-19_n77-n79	235

7.15.1	Operating bands for DC	235

7.15.2	Channel bandwidths per operating band for DC	235

7.15.3	Co-existence studies	236

7.15.4	∆TIB and ∆RIB values	236

7.15.5	MSD	236

7.16	DC_1-19_n78-n79	236

7.16.1	Operating bands for DC	236

7.16.2	Channel bandwidths per operating band for DC	237

7.16.3	Co-existence studies	237

7.16.4	∆TIB and ∆RIB values	237

7.16.5	MSD	237

7.17	DC_1-21_n77-n79	238

7.17.1	Operating bands for DC	238

7.17.2	Channel bandwidths per operating band for DC	238

7.17.3	Co-existence studies	238

7.17.4	∆TIB and ∆RIB values	238

7.17.5	MSD	239

7.18	DC_1-21_n78-n79	239

7.18.1	Operating bands for DC	239

7.18.2	Channel bandwidths per operating band for DC	239

7.18.3	Co-existence studies	239

7.18.4	∆TIB and ∆RIB values	240

7.18.5	MSD	240

7.19	DC_1-3_n77-n79	240

7.19.1	Operating bands for DC	240

7.19.2	Channel bandwidths per operating band for DC	240

7.19.3	Co-existence studies	241

7.19.4	∆TIB and ∆RIB values	241

7.19.5	MSD	241

7.20	DC_1-3_n78-n79	241

7.20.1	Operating bands for DC	241

7.20.2	Channel bandwidths per operating band for DC	242

7.20.3	Co-existence studies	242

7.20.4	∆TIB and ∆RIB values	242

7.20.5	MSD	242

7.21	DC_1-42_n77-n79	243

7.21.1	Operating bands for DC	243

7.21.2	Channel bandwidths per operating band for DC	243

7.21.3	Co-existence studies	243

7.21.4	∆TIB and ∆RIB values	243

7.21.5	MSD	244

7.22	DC_1-42_n78-n79	244

7.22.1	Operating bands for DC	244

7.22.2	Channel bandwidths per operating band for DC	244

7.22.3	Co-existence studies	244

7.22.4	∆TIB and ∆RIB values	245

7.22.5	MSD	245

7.23	DC_19-21_n77-n79	245

7.23.1	Operating bands for DC	245

7.23.2	Channel bandwidths per operating band for DC	245

7.23.3	Co-existence studies	246

7.23.4	∆TIB and ∆RIB values	246

7.23.5	MSD	246

7.24	DC_19-21_n78-n79	246

7.24.1	Operating bands for DC	246

7.24.2	Channel bandwidths per operating band for DC	247

7.24.3	Co-existence studies	247

7.24.4	∆TIB and ∆RIB values	247

7.24.5	MSD	247

7.25	DC_19-42_n77-n79	248

7.25.1	Operating bands for DC	248

7.25.2	Channel bandwidths per operating band for DC	248

7.25.3	Co-existence studies	248

7.25.4	∆TIB and ∆RIB values	248

7.25.5	MSD	249

7.26	DC_19-42_n78-n79	249

7.26.1	Operating bands for DC	249

7.26.2	Channel bandwidths per operating band for DC	249

7.26.3	Co-existence studies	249

7.26.4	∆TIB and ∆RIB values	250

7.26.5	MSD	250

7.27	DC_21-42_n77-n79	250

7.27.1	Operating bands for DC	250

7.27.2	Channel bandwidths per operating band for DC	250

7.27.3	Co-existence studies	251

7.27.4	∆TIB and ∆RIB values	251

7.27.5	MSD	251

7.28	DC_21-42_n78-n79	251

7.28.1	Operating bands for DC	251

7.28.2	Channel bandwidths per operating band for DC	252

7.28.3	Co-existence studies	252

7.28.4	∆TIB and ∆RIB values	252

7.28.5	MSD	252

7.29	DC_3-19_n77-n79	253

7.29.1	Operating bands for DC	253

7.29.2	Channel bandwidths per operating band for DC	253

7.29.3	Co-existence studies	253

7.29.4	∆TIB and ∆RIB values	253

7.29.5	MSD	254

7.30	DC_3-19_n78-n79	254

7.30.1	Operating bands for DC	254

7.30.2	Channel bandwidths per operating band for DC	254

7.30.3	Co-existence studies	254

7.30.4	∆TIB and ∆RIB values	255

7.30.5	MSD	255

7.31	DC_3-21_n77-n79	255

7.31.1	Operating bands for DC	255

7.31.2	Channel bandwidths per operating band for DC	255

7.31.3	Co-existence studies	256

7.31.4	∆TIB and ∆RIB values	256

7.31.5	MSD	256

7.32	DC_3-21_n78-n79	256

7.32.1	Operating bands for DC	256

7.32.2	Channel bandwidths per operating band for DC	257

7.32.3	Co-existence studies	257

7.32.4	∆TIB and ∆RIB values	257

7.32.5	MSD	257

7.33	DC_3-42_n77-n79	258

7.33.1	Operating bands for DC	258

7.33.2	Channel bandwidths per operating band for DC	258

7.33.3	Co-existence studies	258

7.33.4	∆TIB and ∆RIB values	258

7.33.5	MSD	259

7.34	DC_3-42_n78-n79	259

7.34.1	Operating bands for DC	259

7.34.2	Channel bandwidths per operating band for DC	259

7.34.3	Co-existence studies	259

7.34.4	∆TIB and ∆RIB values	260

7.34.5	MSD	260

7.35	DC_3-3-7_n78_n257, DC_3-3-7-7_n78_n257,	260

7.35.1	Operating bands for DC	260

7.35.2	Channel bandwidths per operating band for DC	260

7.35.3	Co-existence studies	263

7.35.4	∆TIB and ∆RIB values	263

7.35.5	MSD	263

7.36	DC_3-7_n1-n78	264

7.36.1	Operating bands for DC	264

7.36.2	Channel bandwidths per operating band for DC	264

7.36.3	Co-existence studies	265

7.36.4	∆TIB and ∆RIB values	265

7.36.5	MSD	265

7.37	DC_3-3-7_n1-n78, DC_3-7-7_n1-n78, DC_3-3-7-7_n1-n78	265

7.37.1	Operating bands for DC	265

7.37.2	Channel bandwidths per operating band for DC	265

7.37.3	Co-existence studies	266

7.37.4	∆TIB and ∆RIB values	266

7.37.5	MSD	266

7.38	DC_1-3_n5-n78	267

7.38.1	Operating bands for DC	267

7.38.2	Channel bandwidths per operating band for DC	267

7.38.3	Co-existence studies	267

7.38.4	∆TIB and ∆RIB values	267

7.38.5	MSD	268

7.39	DC_1-7_n5-n78	268

7.39.1	Operating bands for DC	268

7.39.2	Channel bandwidths per operating band for DC	268

7.39.3	Co-existence studies	269

7.39.4	∆TIB and ∆RIB values	269

7.39.5	MSD	269

7.40	DC_1-28_n5-n78	269

7.40.1	Operating bands for DC	269

7.40.2	Channel bandwidths per operating band for DC	270

7.40.3	Co-existence studies	270

7.40.4	∆TIB and ∆RIB values	270

7.40.5	MSD	270

7.41	DC_3-7_n5-n78	271

7.41.1	Operating bands for DC	271

7.41.2	Channel bandwidths per operating band for DC	271

7.41.3	Co-existence studies	272

7.41.4	∆TIB and ∆RIB values	272

7.41.5	MSD	272

7.42	DC_3-28_n5-n78	272

7.42.1	Operating bands for DC	272

7.42.2	Channel bandwidths per operating band for DC	272

7.42.3	Co-existence studies	273

7.42.4	∆TIB and ∆RIB values	273

7.42.5	MSD	273

7.43	DC_7-28_n5-n78	274

7.43.1	Operating bands for DC	274

7.43.2	Channel bandwidths per operating band for DC	274

7.43.3	Co-existence studies	274

7.43.4	∆TIB and ∆RIB values	274

7.43.5	MSD	275

7.44	DC_1A-3A_n77A-n257	275

7.44.1	Operating bands for DC	275

7.44.2	Channel bandwidths per operating band for DC	275

7.44.3	Co-existence studies	276

7.44.4	∆TIB and ∆RIB values	277

7.44.5	MSD	277

7.45	DC_2-66_n41-n71	277

7.45.1	Operating bands for DC	277

7.45.2	Channel bandwidths per operating band for DC	278

7.45.3	Co-existence studies	278

7.45.4	∆TIB and ∆RIB values	278

7.45.5	MSD	279

7.46	DC_2-66_n41-n260	279

7.46.1	Operating bands for DC	279

7.46.2	Channel bandwidths per operating band for DC	279

7.46.3	Co-existence studies	280

7.46.4	∆TIB and ∆RIB values	280

7.46.5	MSD	281

7.47	DC_2A-66A_n41A-n261	281

7.47.1	Operating bands for DC	281

7.47.3	Co-existence studies	282

7.47.4	∆TIB and ∆RIB values	282

7.47.5	MSD	282

7.48	DC_3-8_n78-n257	282

7.48.1	Operating bands for DC	282

7.48.2	Channel bandwidths per operating band for DC	283

7.48.3	Co-existence studies	285

7.48.4	∆TIB and ∆RIB values	285

7.48.5	MSD	285

7.49	DC_1-8_n78-n257	285

7.49.1	Operating bands for DC	285

7.49.2	Channel bandwidths per operating band for DC	286

7.49.3	Co-existence studies	288

7.49.4	∆TIB and ∆RIB values	288

7.49.5	MSD	288

7.50	DC_2A-7A-7A_n38A-n78A	288

7.50.1	Operating bands for DC	288

7.50.2	Channel bandwidths per operating band for DC	289

7.50.3	Co-existence studies	289

7.50.4	∆TIB and ∆RIB values	289

7.50.5	MSD	289

7.51	DC_7A-7A-66A_n38A-n78A	290

7.51.1	Operating bands for DC	290

7.51.2	Channel bandwidths per operating band for DC	290

7.51.3	Co-existence studies	290

7.51.4	∆TIB and ∆RIB values	290

7.51.5	MSD	291

8	EN-DC band combinations of LTE 3 bands DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part	292

8.1	DC_1A-3A-5A_n78A-n257	292

8.1.1	Operating bands for DC	292

8.1.2	Channel bandwidths per operating band for DC	292

8.1.3	Co-existence studies	294

8.1.4	∆TIB and ∆RIB values	294

8.1.5	MSD	294

8.2	DC_1A-3A-7A_n78A-n257	295

8.2.1	Operating bands for DC	295

8.2.2	Channel bandwidths per operating band for DC	295

8.2.3	Co-existence studies	297

8.2.4	∆TIB and ∆RIB values	297

8.2.5	MSD	297

8.3	DC_1A-3A-7A-7A_n78A-n257	297

8.3.1	Operating bands for DC	297

8.3.2	Channel bandwidths per operating band for DC	298

8.3.3	Co-existence studies	300

8.3.4	∆TIB and ∆RIB values	300

8.3.5	MSD	300

8.4	DC_1A-5A-7A_n78A-n257	300

8.4.1	Operating bands for DC	300

8.4.2	Channel bandwidths per operating band for DC	301

8.4.3	Co-existence studies	303

8.4.4	∆TIB and ∆RIB values	303

8.4.5	MSD	303

8.5	DC_1A-5A-7A-7A_n78A-n257	303

8.5.1	Operating bands for DC	303

8.5.2	Channel bandwidths per operating band for DC	304

8.5.3	Co-existence studies	306

8.5.4	∆TIB and ∆RIB values	306

8.5.5	MSD	306

8.6	DC_3A-5A-7A_n78A-n257	306

8.6.1	Operating bands for DC	306

8.6.2	Channel bandwidths per operating band for DC	307

8.6.3	Co-existence studies	309

8.6.4	∆TIB and ∆RIB values	309

8.6.5	MSD	309

8.7	DC_3A-5A-7A-7A_n78A-n257	309

8.7.1	Operating bands for DC	309

8.7.2	Channel bandwidths per operating band for DC	310

8.7.3	Co-existence studies	312

8.7.4	∆TIB and ∆RIB values	312

8.7.5	MSD	312

8.8	DC_1-3-7_n28-n78	313

8.8.1	Operating bands for DC	313

8.8.2	Channel bandwidths per operating band for DC	313

8.8.3	Co-existence studies	313

8.8.4	∆TIB and ∆RIB values	314

8.8.5	MSD	314

8.9	DC_1-19-42_n77-n79	314

8.9.1	Operating bands for DC	314

8.9.2	Channel bandwidths per operating band for DC	314

8.9.3	Co-existence studies	315

8.9.4	∆TIB and ∆RIB values	315

8.9.5	MSD	315

8.10	DC_1-19-42_n78-n79	315

8.10.1	Operating bands for DC	315

8.10.2	Channel bandwidths per operating band for DC	316

8.10.3	Co-existence studies	316

8.10.4	∆TIB and ∆RIB values	316

8.10.5	MSD	317

8.11	DC_1-21-42_n77-n79	317

8.11.1	Operating bands for DC	317

8.11.2	Channel bandwidths per operating band for DC	317

8.11.3	Co-existence studies	317

8.11.4	∆TIB and ∆RIB values	317

8.11.5	MSD	318

8.12	DC_1-21-42_n78-n79	318

8.12.1	Operating bands for DC	318

8.12.2	Channel bandwidths per operating band for DC	318

8.12.3	Co-existence studies	319

8.12.4	∆TIB and ∆RIB values	319

8.12.5	MSD	319

8.13	DC_1-3-21_n77-n79	319

8.13.1	Operating bands for DC	319

8.13.2	Channel bandwidths per operating band for DC	320

8.13.3	Co-existence studies	320

8.13.4	∆TIB and ∆RIB values	320

8.13.5	MSD	320

8.14	DC_1-3-21_n78-n79	321

8.14.1	Operating bands for DC	321

8.14.2	Channel bandwidths per operating band for DC	321

8.14.3	Co-existence studies	321

8.14.4	∆TIB and ∆RIB values	321

8.14.5	MSD	322

8.15	DC_19-21-42_n77-n79	322

8.15.1	Operating bands for DC	322

8.15.2	Channel bandwidths per operating band for DC	322

8.15.3	Co-existence studies	323

8.15.4	∆TIB and ∆RIB values	323

8.15.5	MSD	323

8.16	DC_19-21-42_n78-n79	323

8.16.1	Operating bands for DC	323

8.16.2	Channel bandwidths per operating band for DC	324

8.16.3	Co-existence studies	324

8.16.4	∆TIB and ∆RIB values	324

8.16.5	MSD	324

8.17	DC_1-7-28_n5-n78	325

8.17.1	Operating bands for DC	325

8.17.2	Channel bandwidths per operating band for DC	325

8.17.3	Co-existence studies	325

8.17.4	∆TIB and ∆RIB values	325

8.17.5	MSD	326

8.18	DC_3-7-28_n5-n78	326

8.18.1	Operating bands for DC	326

8.18.2	Channel bandwidths per operating band for DC	326

8.18.3	Co-existence studies	327

8.18.4	∆TIB and ∆RIB values	327

8.18.5	MSD	328

8.19	DC_1-3-7_n5-n78	328

8.19.1	Operating bands for DC	328

8.19.2	Channel bandwidths per operating band for DC	328

8.19.3	Co-existence studies	329

8.19.4	∆TIB and ∆RIB values	329

8.19.5	MSD	330

8.20	DC_1-3-28_n5-n78	330

8.20.1	Operating bands for DC	330

8.20.2	Channel bandwidths per operating band for DC	330

8.20.3	Co-existence studies	330

8.20.4	∆TIB and ∆RIB values	331

8.20.5	MSD	331

8.21	DC_1-3-8-n78-n257	331

8.21.1	Operating bands for DC	331

8.21.2	Channel bandwidths per operating band for DC	331

8.21.3	Co-existence studies	334

8.21.4	∆TIB and ∆RIB values	334

8.21.5	MSD	335

9	EN-DC band combinations of LTE 4 band DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part	336

9.1	DC_1A-3A-5A-7A_n78A-n257	336

9.1.1	Operating bands for DC	336

9.1.2	Channel bandwidths per operating band for DC	336

9.1.3	Co-existence studies	338

9.1.4	∆TIB and ∆RIB values	338

9.1.5	MSD	339

9.2	DC_1A-3A-5A-7A-7A_n78A-n257	339

9.2.1	Operating bands for DC	339

9.2.2	Channel bandwidths per operating band for DC	339

9.2.3	Co-existence studies	341

9.2.4	∆TIB and ∆RIB values	341

9.2.5	MSD	342

9.3	DC_1-3-7-28_n5-n78	342

9.3.1	Operating bands for DC	342

9.3.2	Channel bandwidths per operating band for DC	342

9.3.3	Co-existence studies	343

9.3.4	∆TIB and ∆RIB values	343

9.3.5	MSD	343

Annex A: Change history	344

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

1	Scope

The present document is a technical report for E-UTRA-NR Dual connectivity (DC) band combinations of LTE xDL/1UL (x=1,2,3,4) + inter-band NR 2DL/1UL under Rel-16 time frame. The purpose is to gather the relevant background information and studies in order to address EN-DC band combinations of LTE xDL/1UL (x=1,2,3,4) + inter-band NR 2DL/1UL in Rel-16 as shown in Table 1-1.

Table 1-1: Release 16 EN-DC band combination of LTE xDL/1UL (x=1,2,3,4) + inter-band NR 2DL/1UL

| Combination |  |  |
| --- | --- | --- |
| LTE X bands (X=1,2,3,4) |  | NR band combinations (2 Bands) |
| LTE 1 Band | 1 | CA_n1-n78 |
|  | 1 | CA_n3-n28 |
|  | 1 | CA_n3-n78 |
|  | 1 | CA_n5-n78 |
|  | 1 | CA_n7-n78 |
|  | 1 | CA_n40-n78 |
|  | 1 | CA_n77-n78 |
|  | 1 | CA_n3-n257 |
|  | 1 | CA_ n77-n257 |
|  | 1 | CA_ n77-n258 |
|  | 1 | CA_ n78-n257 |
|  | 1 | CA_ n78-n258 |
|  | 1 | CA_ n79-n258 |
|  | 2 | CA_n7-n66 |
|  | 2 | CA_n41-n71 |
|  | 2 | CA_n66-n78 |
|  | 2 | CA_n5-n258 |
|  | 2 | CA_n5-n260 |
|  | 2 | CA_n5-n261 |
|  | 2 | CA_n12-n258 |
|  | 2 | CA_n12-n260 |
|  | 2 | CA_n12-n261 |
|  | 2 | CA_n41-n260 |
|  | 2 | CA_n41-n261 |
|  | 2 | CA_n71-n258 |
|  | 2 | CA_n71-n260 |
|  | 2 | CA_n71-n261 |
|  | 3 | CA_n1-n7 |
|  | 3 | CA_n1-n28 |
|  | 3 | CA_n1-n77 |
|  | 3 | CA_n1-n78 |
|  | 3 | CA_n1-n79 |
|  | 3 | CA_n3-n77 |
|  | 3 | CA_n3-n78 |
|  | 3 | CA_n5-n78 |
|  | 3 | CA_n7-n28 |
|  | 3 | CA_n7-n78 |
|  | 3 | CA_n20-n78 |
|  | 3 | CA_n28-n78 |
|  | 3 | CA_n40-n41 |
|  | 3 | CA_n40-n78 |
|  | 3 | CA_n40-n79 |
|  | 3 | CA_n41-n79 |
|  | 3 | CA_n77-n78 |
|  | 3 | CA_n1-n257 |
|  | 3 | CA_ n77-n257 |
|  | 3 | CA_ n77-n258 |
|  | 3 | CA_ n78-n257 |
|  | 3-3 | CA_n1-n78 |
|  | 3-3 | CA_ n78-n257 |
|  | 3 | CA_ n78-n258 |
|  | 3 | CA_ n79-n258 |
|  | 5 | CA_n7-n78 |
|  | 5 | CA_ n78-n257 |
|  | 7 | CA_n1-n3 |
|  | 7 | CA_n1-n78 |
|  | 7 | CA_n3-n78 |
|  | 7 | CA_n5-n78 |
|  | 7 | CA_n7-n78 |
|  | 7 | CA_n28-n78 |
|  | 7 | CA_n66-n78 |
|  | 7 | CA_ n78-n257 |
|  | 7-7 | CA_n1-n78 |
|  | 7-7 | CA_n66-n78 |
|  | 7-7 | CA_n78-n257 |
|  | 8 | CA_n1-n78 |
|  | 8 | CA_n3-n28 |
|  | 8 | CA_n8-n78 |
|  | 8 | CA_n40-n41 |
|  | 8 | CA_n40-n79 |
|  | 8 | CA_n41-n79 |
|  | 8 | CA_ n77-n257 |
|  | 8 | CA_ n78-n257 |
|  | 11 | CA_ n77-n257 |
|  | 12 | CA_n7-n66 |
|  | 12 | CA_n7-n78 |
|  | 12 | CA_n66-n78 |
|  | 13 | CA_n5-n260 |
|  | 18 | CA_n3-n78 |
|  | 18 | CA_n3-n257 |
|  | 18 | CA_n78-n257 |
|  | 19 | CA_n77-n78 |
|  | 20 | CA_n1-n3 |
|  | 20 | CA_n1-n7 |
|  | 20 | CA_n1-n28 |
|  | 20 | CA_n1-n78 |
|  | 20 | CA_n3-n78 |
|  | 20 | CA_n7-n28 |
|  | 20 | CA_n7-n78 |
|  | 20 | CA_n20-n78 |
|  | 21 | CA_n77-n78 |
|  | 28 | CA_n3-n78 |
|  | 28 | CA_n5-n78 |
|  | 28 | CA_n8-n78 |
|  | 28 | CA_n28-n78 |
|  | 28 | CA_n3-n257 |
|  | 28 | CA_n8-n258 |
|  | 28 | CA_n78-n257 |
|  | 39 | CA_n40-n41 |
|  | 39 | CA_n40-n79 |
|  | 39 | CA_n41-n79 |
|  | 40 | CA_n41-n79 |
|  | 41 | CA_n78-n257 |
|  | 42 | CA_n78-n257 |
|  | 66 | CA_n7-n78 |
|  | 66 | CA_n25-n41 |
|  | 66 | CA_n41-n71 |
|  | 66 | CA_n66-n78 |
|  | 66 | CA_n5-n258 |
|  | 66 | CA_n5-n260 |
|  | 66 | CA_n5-n261 |
|  | 66 | CA_n12-n258 |
|  | 66 | CA_n12-n260 |
|  | 66 | CA_n12-n261 |
|  | 66 | CA_n41-n260 |
|  | 66 | CA_n41-n261 |
|  | 66 | CA_n71-n258 |
|  | 66 | CA_n71-n260 |
|  | 66 | CA_n71-n261 |
|  | 66-66 | CA_n7-n78 |
| LTE 2 Bands | CA_1A-3A CA_1A-3C | CA_n5-n78 |
|  | CA_1A-3A CA_1A-3C | CA_n7-n78 |
|  | CA_1A-3A | CA_n28-n78 |
|  | CA_1A-3A | CA_n38-n78 |
|  | CA_1A-3A | CA_n77-n78 |
|  | CA_1A-3A | CA_n77-n79 |
|  | CA_1A-3A | CA_n78-n79 |
|  | CA_1A-3A | CA_n77-n257 |
|  | CA_1A-3A | CA_n78-n257 |
|  | CA_1A-5A | CA_n78-n257 |
|  | CA_1A-7A | CA_n3-n78 |
|  | CA_1A-7A CA_1A-7C | CA_n5-n78 |
|  | CA_1A-7A CA_1A-7C | CA_n28-n78 |
|  | CA_1A-7A | CA_n78-n257 |
|  | CA_1A-7A-7A | CA_n78-n257 |
|  | CA_1A-8A | CA_n3-n28 |
|  | CA_1A-8A | CA_n77-n257 |
|  | CA_1A-8A | CA_n78-n257 |
|  | CA_1A-11A | CA_n77-n257 |
|  | CA_1A-18A | CA_n3-n78 |
|  | CA_1A-18A | CA_n3-n257 |
|  | CA_1A-18A | CA_n78-n257 |
|  | CA_1A-19A | CA_n77-n79 |
|  | CA_1A-19A | CA_n78-n79 |
|  | CA_1A-20A | CA_n3-n38 |
|  | CA_1A-20A | CA_n3-n78 |
|  | CA_1A-21A | CA_n77-n78 |
|  | CA_1A-21A | CA_n77-n79 |
|  | CA_1A-21A | CA_n78-n79 |
|  | CA_1A-28A | CA_n3-n78 |
|  | CA_1A-28A | CA_n3-n257 |
|  | CA_1A-28A | CA_n5-n78 |
|  | CA_1A-28A | CA_n28-n78 |
|  | CA_1A-28A | CA_n78-n257 |
|  | CA_1A-41A CA_1A-41C | CA_n78-n257 |
|  | CA_1A-42A | CA_n77-n78 |
|  | CA_1A-42A | CA_n77-n79 |
|  | CA_1A-42A | CA_n78-n79 |
|  | CA_1A-42A CA_1A-42C | CA_n78-n257 |
|  | CA_2A-7A CA_2A-7A-7A CA_2A-7C | CA_n38-n66 |
|  | CA_2A-7A CA_2A-7A-7A CA_2A-7C | CA_n38-n78 |
|  | CA_2A-7A | CA_n66-n78 |
|  | CA_2A-7A-7A | CA_n66-n78 |
|  | CA_2A-66A | CA_n41-n71 |
|  | CA_2A-66A | CA_n66-n78 |
|  | CA_2A-66A | CA_n41-n260A CA_n41-n260(2A) CA_n41-n260(3A) CA_n41-n260(4A) |
|  | CA_2A-66A | CA_n41-n261 |
|  | CA_3A-3A-7A | CA_n78-n257 |
|  | CA_3A-3A-7A-7A | CA_n78-n257 |
|  | CA_3A-5A | CA_n78-n257 |
|  | CA_3A-7A CA_3A-7C CA_3C-7C | CA_n28-n78 |
|  | CA_3A-7A CA_3A-7C CA_3C-7A CA_3C-7C CA_3A-3A-7A CA_3A-7A-7A CA_3A-3A-7A-7A | CA_n1-n78 |
|  | CA_3A-7A CA_3C-7A CA_3A-7C CA_3C-7C | CA_n5-n78 |
|  | CA_3A-7A | CA_n78-n257 |
|  | CA_3A-7A-7A | CA_n78-n257 |
|  | CA_3A-8A | CA_n1-n78 |
|  | CA_3A-8A | CA_n78-n257 |
|  | CA_3A-18A | CA_n78-n257 |
|  | CA_3A-19A | CA_n77-n78 |
|  | CA_3A-19A | CA_n77-n79 |
|  | CA_3A-19A | CA_n78-n79 |
|  | CA_3A-20A CA_3C-20A | CA_n1-n7 |
|  | CA_3A-20A CA_3C-20A | CA_n1-n28 |
|  | CA_3C-20A | CA_n1-n78 |
|  | CA_3A-20A CA_3C-20A | CA_n7-n28 |
|  | CA_3A-20A CA_3C-20A | CA_n7-n78 |
|  | CA_3C-20A | CA_n28-n78 |
|  | CA_3A-20A | CA_n38-n78 |
|  | CA_3A-28A | CA_n28-n78 |
|  | CA_3A-21A | CA_n77-n78 |
|  | CA_3A-21A | CA_n77-n79 |
|  | CA_3A-21A | CA_n78-n79 |
|  | CA_3A-28A CA_3C-28A | CA_n5-n78 |
|  | CA_3A-28A | CA_n78-n257 |
|  | CA_3A-41A CA_3A-41C | CA_n78-n257 |
|  | CA_3A-42A | CA_n77-n78 |
|  | CA_3A-42A | CA_n77-n79 |
|  | CA_3A-42A | CA_n78-n79 |
|  | CA_3A-42A CA_3A-42C | CA_n78-n257 |
|  | CA_5A-7A | CA_n78-n257 |
|  | CA_5A-7A-7A | CA_n78-n257 |
|  | CA_7A-8A | CA_n1-n78 |
|  | CA_7A-20A | CA_n3-n78 |
|  | CA_7A-28A CA_7C-28A | CA_n5-n78 |
|  | CA_7A-28A | CA_n28-n78 |
|  | CA_7A-66A CA_7A-7A-66A CA_7C-66A | CA_n38-n78 |
|  | CA_7A-66A | CA_n66-n78 |
|  | CA_7A-7A-66A | CA_n66-n78 |
|  | CA_8A-11A | CA_n77-n257 |
|  | CA_18A-42A CA_18A-42C | CA_n78-n257 |
|  | CA_19A-21A | CA_n77-n78 |
|  | CA_19A-21A | CA_n77-n79 |
|  | CA_19A-21A | CA_n78-n79 |
|  | CA_19A-42A | CA_n77-n78 |
|  | CA_19A-42A | CA_n77-n79 |
|  | CA_19A-42A | CA_n78-n79 |
|  | CA_21A-42A | CA_n77-n78 |
|  | CA_21A-42A | CA_n77-n79 |
|  | CA_21A-42A | CA_n78-n79 |
|  | CA_28A-41A CA_28A-41C | CA_n78-n257 |
|  | CA_28A-42A CA_28A-42C | CA_n78-n257 |
|  | CA_41A-42A CA_41A-42C CA_41C-42A CA_41C-42C | CA_n78-n257 |
| LTE 3 Bands | CA_1A-3A-5A | CA_n78-n257 |
|  | CA_1A-3A-7A CA_1A-3C-7A CA_1A-3A-7C CA_1A-3C-7C | CA_n5-n78 |
|  | CA_1A-3A-7A CA_1A-3A-7C | CA_n28-n78 |
|  | CA_1A-3A-7A | CA_n78-n257 |
|  | CA_1A-3A-7A-7A | CA_n78-n257 |
|  | CA_1A-3A-8A CA_1A-3C-8A | CA_n78-n257 |
|  | CA_1A-3A-18A | CA_n78-n257 |
|  | CA_1A-3A-20A | CA_n38-n78 |
|  | CA_1A-3A-21A | CA_n77-n78 |
|  | CA_1A-3A-21A | CA_n77-n79 |
|  | CA_1A-3A-21A | CA_n78-n79 |
|  | CA_1A-3A-28A, CA_1A-3C-28A | CA_n5-n78 |
|  | CA_1A-3A-28A, CA_1A-3C-28A | CA_n28-n78 |
|  | CA_1A-3A-28A | CA_n78-n257 |
|  | CA_1A-3A-41A CA_1A-3A-41C | CA_n78-n257 |
|  | CA_1A-3A-42A CA_1A-3A-42C | CA_n78-n257 |
|  | CA_1A-5A-7A | CA_n78-n257 |
|  | CA_1A-5A-7A-7A | CA_n78-n257 |
|  | CA_1A-7A-20A | CA_n3-n78 |
|  | CA_1A-7A-28A, CA_1A-7C-28A | CA_n5-n78 |
|  | CA_1A-7A-28A, CA_1A-7C-28A | CA_n28-n78 |
|  | CA_1A-8A-11A | CA_n77-n257 |
|  | CA_1A-18A-42A CA_1A-18A-42C | CA_n78-n257 |
|  | CA_1A-19A-42A CA_1A-19A-42C | CA_n77-n79 |
|  | CA_1A-19A-42A CA_1A-19A-42C | CA_n78-n79 |
|  | CA_1A-20A-38A | CA_n3-n78 |
|  | CA_1A-21A-42A | CA_n77-n78 |
|  | CA_1A-21A-42A CA_1A-21A-42C | CA_n77-n79 |
|  | CA_1A-21A-42A CA_1A-21A-42C | CA_n78-n79 |
|  | CA_1A-28A-42A CA_1A-28A-42C | CA_n78-n257 |
|  | CA_1A-41A-42A CA_1A-41A-42C CA_1A-41C-42A CA_1A-41C-42C | CA_n78-n257 |
|  | CA_2A-7A-66A | CA_n66-n78 |
|  | CA_2A-7A-7A-66A | CA_n66-n78 |
|  | CA_3A-5A-7A | CA_n78-n257 |
|  | CA_3A-5A-7A-7A | CA_n78-n257 |
|  | CA_3A-7A-8A | CA_n1-n78 |
|  | CA_3A-7A-28A, CA_3A-7C-28A, CA_3C-7A-28A, CA_3C-7C-28A, | CA_n5-n78 |
|  | CA_3A-7A-28A, CA_3A-7C-28A, CA_3C-7A-28A, CA_3C-7C-28A, | CA_n28-n78 |
|  | CA_3A-18A-42A CA_3A-18A-42C | CA_n78-n257 |
|  | CA_3A-28A-41A CA_3A-28A-41C | CA_n78-n257 |
|  | CA_3A-28A-42A CA_3A-28A-42C | CA_n78-n257 |
|  | CA_3A-41A-42A CA_3A-41A-42C CA_3A-41C-42A CA_3A-41C-42C | CA_n78-n257 |
|  | CA_19A-21A-42A | CA_n77-n78 |
|  | CA_19A-21A-42A CA_19A-21A-42C | CA_n77-n79 |
|  | CA_19A-21A-42A CA_19A-21A-42C | CA_n78-n79 |
|  | CA_28A-41A-42A CA_28A-41A-42C CA_28A-41C-42A CA_28A-41C-42C | CA_n78-n257 |
| LTE 4 Bands | CA_1A-3A-5A-7A | CA_n78-n257 |
|  | CA_1A-3A-5A-7A-7A | CA_n78-n257 |
|  | CA_1A-3A-7A-28A, CA_1A-3C-7A-28A, CA_1A-3A-7C-28A, CA_1A-3C-7C-28A | CA_n5-n78 |
|  | CA_1A-3A-7A-28A, CA_1A-3C-7A-28A, CA_1A-3A-7C-28A, CA_1A-3C-7C-28A | CA_n28-n78 |
|  | CA_1A-3A-18A-42A CA_1A-3A-18A-42C | CA_n78-n257 |
|  | CA_1A-3A-28A-42A CA_1A-3A-28A-42C | CA_n78-n257 |
|  | CA_1A-3A-41A-42A CA_1A-3A-41A-42C CA_1A-3A-41C-42A CA_1A-3A-41C-42C | CA_n78-n257 |
|  | CA_3A-28A-41A-42A CA_3A-28A-41A-42C CA_3A-28A-41C-42A CA_3A-28A-41C-42C | CA_n78-n257 |

This TR contains a general part and band specific combination part. The actual requirements are added to the corresponding technical specifications.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]		3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 30.007: “Guideline on WI/SI for new Operating Bands”

[3]	3GPP TS 38.101-1: “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

[4]	3GPP TS 38.101-2: “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”

[5]	3GPP TS 38.101-3: “NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

4	Background

The present document is a technical report for EN-DC band combinations of LTE x bands (x=1,2,3,4) inter-band CA (x DL/1UL) + inter-band NR 2 bands (2DL/1UL) under Rel-16 time frame. It covers both the UE and BS side. The document is divided in two different parts:

- 	General part: this part covers BS and UE specific which is band combination independent.

- 	Specific band combination part:  this part covers each band combination and its specific issues independently from each other (i.e. one subclause is defined per band combination)

The specific band combination parts are independent and therefore, the working speed also differs.

4.1	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band combination to ensure that the TPs related to the band combination have been implemented.

5	EN-DC band combinations of LTE x bands DL/1UL(x=1,2,3,4) + NR 2 bands DL/1UL: General Part

5.1	UE RF architectures

RAN4 consider same example RF architecture which are described in section 5.1 in TR37.863-01-01.  One different point is that LTE and NR modemchip can be used one chip solutions from rel-16.

To support multiple component carriers in LTE and NR, RAN4 consider specific RF components such as Diplexer, Triplexer, Quadplexer and Hexaplexer to derive self-interference problems according to the LTE x bands DL/1UL(x=1,2,3,4) + NR 2 band DL/1UL EN-DC band combinations.

5.2 	General treatment of ∆TIB and ∆RIB values

For the LTE x bands DL/1UL(x=1,2,3,4) + NR 2 band DL/1UL EN-DC band combinations, RAN4 should consider to reuse agreed additional insertion losses for all EN-DC band combinations in rel-15 when new RF components are not introduced to support this basket WI. If the new RF components are introduced, then more detail decription will be captured in some specific EN-DC band combinations.

5.3	summary of self-interference analysis

Table 5.3-1 summarizes the EN- DC band combinations with self-interference problems for 3DL/2UL EN-DC operation.

Table 5.3-1: Summary of Self-interference analysis for LTE 1 band & NR 2 bands DL and 2 bands UL EN-DC operation

| Downlink band configuration |  | Uplink DC Configuration |  | Harmonic relation issues |  | intermodulation to own rx band |  | interference due to small frequency separation |  | MSD |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1_n1-n78 |  | DC_1A_n78A |  |  |  | 4th IMD into n1 |  |  |  | - The 4th IMD already covered in 2DL/2UL DC_1A_n78A |  |
| DC_1_n3-n78 |  | DC_1A_n3A |  | 2nd harmonic from n3 into n78 |  | 2nd & 4th IMDs into n78 |  | Yes |  | - Harmonic problems already covered in DC_n3A-n78A. - 28.4 dB by 2nd IMD in rel-15 TR - 11.2 dB by 4th IMD in Rel-15 TR. - Small freq. gap was covered in Table 7.3.1A-0bA in TS36.101 |  |
|  |  | DC_1A_n78A |  | - |  | 2nd IMD into n3 |  | - |  | - 27.9 dB by 2nd IMD in rel-15 TR. |  |
| DC_1_n7-n78 |  | DC_1A_n7A |  | - |  | 4th & 5th IMDs into n78 |  | - |  | - 10.1 dB by 4th IMD problem. - 3.8 dB by 5th IMD problem That were already covered in DC_7_n1-n78 in rel-15 TR |  |
|  |  | DC_1A_n78A, DC_1A_n78(2A) |  | - |  | 4th IMD into B7 |  | - |  | -9.1dB by 4th IMD problem that was already covered in DC_1-7_n78 in rel-15 TR |  |
| DC_1_n78-n257 |  | DC_1A_n78A |  | 7th & 8th Harmonics from n78 into n257 |  | - |  | - |  | - No harmonic problems by 7th & 8th order between FR1 and FR2 |  |
|  |  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I |  | 2nd harmonic from B1 into n78 |  | - |  | - |  | - Harmonic problem already discussed and will be solved in DC_1A_n78A |  |
| DC_3_n1-n77 |  | DC_3A_n1A |  | 2nd harmonic from B3 into n77 |  | 2nd & 4th IMDs into n77 |  | Yes |  | - Harmonic problems will be covered in DC_3A_n77A. - 2nd & 4th IMDs problems was same as DC_3A_n1A-n78A - Small freq. gap was covered in Table 7.3.1A-0bA in TS36.101 |  |
|  |  | DC_3A_n77A |  | - |  | 2nd & 5th IMD into n1 |  | - |  | - 31.0 dB by 2nd IMD that follow DC_1A-3A_n77A in TR37.863-02-01 in rel-15. - Follow DC_3A_n1A-n78A for 5th IMD problem. |  |
| DC_3_n1-n78 |  | DC_3A_n1A |  | 2nd harmonic from 3 into n78 |  | 2nd & 4th IMDs into n78 |  | Yes |  | - Harmonic problems was already covered in DC_3A_n78A. - 2nd &4th IMDs problems same as DC_1A_n3A-n78A. - Small freq. gap was covered in Table 7.3.1A-0bA in TS36.101 |  |
|  |  | DC_3A_n78A |  | - |  | 5th IMD into n1 |  | - |  | - 3.5 dB by 5th IMD in rel-15 TR |  |
| DC_3_n3-n77 |  | DC_3A_n3A |  | 2nd harmonic into n77 |  | - |  | - |  | - This harmonic impact already covered in 2DL/2UL DC_3A-n77A |  |
|  |  | DC_3A_n77A |  | 2nd harmonic into n77 |  | - |  | - |  | - This harmonic impact already covered in 2DL/2UL DC_3A-n77A In rel-15 |  |
| DC_3_n3-n78 |  | DC_3A_n3A |  | 2nd harmonic into n78 |  | 2nd , 3rd ,4th & 5th IMDs into n3 |  | - |  | - This harmonic & IMD problems are already covered in 2DL/2UL DC_3A-n78A |  |
|  |  | DC_3A_n78A |  | 2nd harmonic into n78 |  | - |  | - |  | - This harmonic impact already covered in 2DL/2UL DC_3A-n78A In rel-15. |  |
| DC_3_n7-n78 |  | DC_3A_n7A |  | 2nd harmonic from 3 into n78 |  | 3rd IMD into n78 |  | - |  | - Harmonic problem already solved in DC_3A_n78A - 16.1dB by 3rd IMD problem that already covered in DC_7A_n3A-n78A in rel-15 TR. |  |
|  |  | DC_3A_n78A, DC_3A_n78(2A) |  | - |  | - |  | - |  | No issues |  |
| DC_3_n20-n78 |  | DC_3A_n20A |  | 2nd & 4th harmonics into n78 |  | 3rd & 5th IMDs into n78 |  | - |  | These harmonic issues are already covered in DC_3A_n78A and DC_20A_n78A - 16.1 dB by 3rd IMD in rel-15 TR - 4.5 dB by 5th IMD in rel-15 TR |  |
|  |  | DC_3A_n78A |  | - |  | - |  | - |  | No issues |  |
| DC_3_n28-n78 |  | DC_3A_n28A |  | 2nd harmonic into n78 5th harmonic into n78 |  | 5th IMD into n78 |  | - |  | - Harmonic problem already discussed and will be solved in DC_3A_n78A or DC_28A_n78A - 4.5dB by 5th IMD problem in rel-15 TR |  |
|  |  | DC_3A_n78A |  | 2nd harmonic into n78 |  | 2nd,4th & 5th IMDs into B3 4th & 5th IMDs into n78 |  | - |  | - 2nd harmonic problem already discussed and will be solved in DC_3A_n78A - 2nd,4th & 5th IMDs problems already discussed and will be solved in DC_3A_n78A - 4th & 5th IMDs problems are no issue for TDD band |  |
| DC_3_n78-n257 DC_3-3_n78-n257 |  | DC_3A_n78A |  | 7th & 8th harmonics from n78 into n257 |  | - |  | - |  | - No harmonic problems by 7th & 8th order between FR1 and FR2 |  |
|  |  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I |  | 2nd harmonic from B3 into n78 |  | - |  | - |  | - Harmonic problem will be solved in DC_3A_n78A |  |
| DC_5_n78-n257 |  | DC_5A_n78A |  | 7th & 8th harmonics from n78 into n257 |  | - |  | - |  | - No harmonic problems by 7th & 8th order between FR1 and FR2 |  |
|  |  | DC_5A_n257A |  | 4th harmonic from B5 into n78 |  | - |  | - |  | - Harmonic problem will be solved in DC_5A_n78A |  |
| DC_7_n1-n3 |  | DC_7A_n1A |  |  |  |  |  | Yes |  | No issues |  |
|  |  | DC_7A_n3A |  |  |  |  |  | - |  | No issues |  |
| DC_7_n1-n78 |  | DC_7A_n1A, DC_7C_n1A |  |  |  | 4th & 5th IMDs into n78 |  | - |  | - 10.1 dB by 4th IMD in rel-15 TR. - 3.8 dB by 5th IMD in rel-15 TR. |  |
|  |  | DC_7A_n78A, DC_7C_n78A |  |  |  | 4th IMD into n1 |  | - |  | - 9.0 dB by 4th IMD in rel-15 TR. |  |
| DC_7_n3-n78 |  | DC_7A_n3A |  | 2nd harmonic from n3 into n78 |  | 3rd IMD into n78 |  | - |  | - Harmonic problem already solved in CA_n3A-n78A - 16.1 dB by 3rd IMD in rel-15 TR |  |
|  |  | DC_7A_n78A |  | - |  | 3rd & 4th IMDs into n3 |  | - |  | - 15.6 dB by 3rd IMD in rel-15 TR - 9.2 dB by 4th IMD in rel-15 TR. |  |
| DC_7_n28-n78 |  | DC_7A_n28A |  | 5th harmonic into n78 |  | 2nd,4th IMDs into n78 |  | - |  | - 5th harmonic problem will be solved in DC_28A_n78A - 29.7 dB by 2nd IMD in rel-15 TR - 10.3 dB by 4th IMD problem in rel-15 TR |  |
|  |  | DC_7A_n78A |  | - |  | 2nd & 5th IMDs into B28 4th IMD into n78 |  | - |  | - 28.8 dB by 2nd IMD in rel-15 TR - 4.0 dB by 5th IMD problems in rel-15 TR - 4th IMD problem is no issue for TDD band |  |
| DC_7-7_n78-n257 |  | DC_7A_n78A |  | 7th harmonic from B7 into n257 |  |  |  | - |  | - Up to 7th harmonic order will be not any impact to n257 |  |
|  |  | DC_7A_n257A |  | - |  | - |  | - |  | No issues |  |
| DC_7_n78-n257 |  | DC_7A_n78A |  | 7th & 8th harmonics from n78 into n257 |  | - |  | - |  | - No harmonic problems by 7th & 8th order between FR1 and FR2 |  |
|  |  | DC_7A_n257A |  | - |  | - |  | - |  | No issues |  |
| DC_8_n8-n78 |  | DC_8A_n78A |  | - |  | 4th IMD into n8 |  | - |  | The 4th IMD issue already covered in DC_8A-n78A |  |
| DC_8_n78-n257 |  | DC_8A_n78A |  | 7th & 8th harmonics from n78 into n257 |  | - |  | - |  | - No harmonic problems by 7th & 8th order between FR1 and FR2 |  |
|  |  | DC_8A_n257A |  | 4th harmonic from B8 into n78 |  | - |  | - |  | - Harmonic problem will be solved in DC_5A_n78A |  |
| DC_13_n5-n260 |  | DC_13A_n5A |  | - |  | - |  | - |  | No issue |  |
|  |  | DC_13A_n260A |  | - |  | - |  | - |  | No issue |  |
| DC_20_n1-n3 |  | DC_20A_n1A |  |  |  |  |  | - |  | No issues |  |
|  |  | DC_20A_n3A |  |  |  |  |  | - |  | No issues |  |
| DC_20_n1-n78 |  | DC_20A_n1A |  | 4th harmonic from B20 into n78 |  | 3rd IMD into n78 |  | - |  | - Harmonic problem already solved in DC_20A_n78A - 16.0 dB by 3rd IMD in rel-15 TR |  |
|  |  | DC_20A_n78A |  | - |  | 3rd IMD into n1 |  | - |  | - 15.3 dB by 3rd IMD in rel-15 TR |  |
| DC_20_n3-n78 |  | DC_20A_n3A |  | - 2nd harmonic from n3 into n78 - 4th harmonic from B20 into n78 |  | 3rd & 5th IMDs into n78 |  | - |  | - 16.1 dB by 3rd IMD in rel-15 TR - 4.5 dB by 5th IMD in rel-15 TR |  |
|  |  | DC_20A_n78A |  | - |  | 3rd IMD into n3 |  | - |  | - 15.7 dB by 3rd IMD in rel-15 TR |  |
| DC_20_n20-n78 |  | DC_20A_n78A |  | - |  | 4th IMD into n20 |  | - |  | This IMD impact already covered in 2DL/2UL DC_20A-n78A |  |
| DC_28_n8-n78 |  | DC_28A_n8A |  | 4th harmonic into n78 5th harmonic into n78 |  | 4th & 5th IMDs into n78 |  | - |  | - 4th harmonic problem will be solved in DC_8A-n78A - 5th harmonic problem will be solved in DC_28A-n78A - 9.1 dB by 4th IMD in rel-15 TR - 3.7 dB by 5th IMD in rel-15 TR |  |
|  |  | DC_28A_n78A |  | 5th harmonic into n78 |  | 5th IMD into B28 5th IMD into n8 |  | - |  | - 5th harmonic problem will be solved in DC_28A-n78A - 5th IMD problem already discussed and will be solved in DC_28A-n78A - 4.3 dB by 5th IMD in rel-15 TR |  |
| DC_28_n8-n258 |  | DC_28A_n8A |  | 33th harmonics into n258 27th harmonics into n258 |  | - |  | - |  | - No harmonic problems by 27th & 33th |  |
|  |  | DC_28A_n258A |  | - |  | - |  | - |  | - No issue |  |
| DC_28_n28-n78 |  | DC_28A_n78A |  | - |  | 5th IMD into n28 |  | - |  | This IMD impact already covered in 2DL/2UL DC_28A-n78A |  |
| DC_66_n5-n260 |  | DC_66A_n5A |  | 22th harmonics into n260 |  | - |  | - |  | Up to 7th harmonic order will be not any impact to n260. |  |
|  |  | DC_66A_n260A |  | - |  | - |  | - |  | No issues |  |
| DC_66_n71-n260 DC_66_n71-n260(2A) |  |  | DC_66A_n71A | 22th harmonics into n260 |  | - |  | - |  | Up to 7th harmonic order will be not any impact to n257. |  |
|  |  |  | DC_66A_n260A | - |  | - |  | - |  | No issues |  |
| DC_66_n71-n261 DC_66_n71-n261(2A) |  |  | DC_66A_n71A | 16th harmonics into n261 |  | - |  | - |  | Up to 7th harmonic order will be not any impact to n261. |  |
|  |  |  | DC_66A_n261A | - |  | - |  | - |  | No issues |  |
| DC_1_n77-n78 | DC_1A_n77A |  |  |  |  |  | 4th IMD & 5th IMD |  | - |  | These MSD issues are same in DC_1A_n77A. So the MSD level by 4th IMD can follow 2DL/2UL_DC_1A_n77A |
|  | DC_1A_n78A |  |  |  | 2nd harmonic |  | 4th IMD & 5th IMD |  |  |  | 2nd harmonic issue already specified in TS38.101-1 for DC_1A_n77A These MSD issues are same in DC_1A_n78A. So the MSD level by 4th IMD can follow 2DL/2UL_DC_1A_n78A |
| DC_3_n77-n78 | DC_3A_n77A |  |  |  | 2nd harmonic |  | 4th IMD & 5th IMD |  |  |  | 2nd harmonic issue already specified in TS38.101-1 for DC_3A_n78A These MSD issues are same in DC_3A_n77A. So the MSD level by 4th IMD can follow 2DL/2UL_DC_3A_n77A |
|  | DC_3A_n78A |  |  |  | 2nd harmonic |  | 4th IMD & 5th IMD |  |  |  | 2nd harmonic issue already specified in TS38.101-1 for DC_3A_n77A These MSD issues are same in DC_3A_n77A. So the MSD level by 4th IMD can follow 2DL/2UL_DC_3A_n78A |
| DC_19_n77-n78 | DC_19A_n77A |  |  |  | 4th harmonic |  | 2nd IMD |  |  |  | 4th harmonic & 2nd IMD will be impact to n78 band. However, consider specific band for n77 in Japan, there was no IMD problem |
|  | DC_19A_n78A |  |  |  | 4th & 5th harmonic |  | 2nd IMD |  |  |  | 5th IMD already specified for DC_19A_n77A in TS38.101-3 Consider specific band for n78 in Japan, there was no IMD problem |
| DC_21_n77-n78 | DC_21A_n77A |  |  |  |  |  | 4th IMD & 5th IMD |  | - |  | These MSD issues are same in DC_21A_n77A. However, RAN4 consider the specific band for n77 in Japan, there was no IMD problem |
|  | DC_21A_n78A |  |  |  |  |  | 4th IMD |  |  |  | These MSD issue is same in DC_21A_n78A. However, RAN4 consider the specific band for n78 in Japan, there was no IMD problem |
| DC_1_n40-n78 | DC_1A_n40A |  |  |  |  |  | 4th IMD & 5th IMD |  | - |  | - 9.8dB by 4th IMD - 4.2dB by 5th IMD |
|  | DC_1A_n78A |  |  |  |  |  | 4th IMD |  | - |  | - 10.6dB by 4th IMD |
| DC_3_n1-n79 | DC_3A_n1A |  |  |  |  |  | 5th IMD |  | Yes |  | - 4.7dB by 5th IMD |
|  | DC_3A_n79A |  |  |  |  |  | 5th IMD |  | - |  | - 3.6dB by 5th IMD |
| DC_3_n40-n78 | DC_3A_n40A |  |  |  | 2nd harmonic |  | 5th IMD |  | - |  | Harmonic issue already covered in DC_3A_n78A - 4.8dB by 5th IMD |
|  | DC_3A_n78A |  |  |  |  |  | 5th IMD |  | - |  | - 4.4dB by 5th IMD |
| DC_66_n25-n41 | DC_66A_n25A |  |  |  | - |  | - |  | - |  | No issue |
|  | DC_66A_n41A |  |  |  |  |  | 4th IMD |  | - |  | - 11.0dB by 4th IMD |
| DC_1_n77-n257 | DC_1A_n77A |  |  |  | 7th & 8th Harmonics from n77 into n257 |  | - |  | - |  | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  | DC_1A_n257A |  |  |  | 2nd harmonic from B1 into n77 |  | - |  | - |  | - Harmonic problem already covered in DC_1A_n77A |
| DC_1_n77-n258 | DC_1A_n77A |  |  |  | 6th, 7th & 8th Harmonics from n77 into n258 |  | - |  | - |  | - No harmonic problems by 6th, 7th & 8th order between FR1 and FR2 |
|  | DC_1A_n258A |  |  |  | 2nd harmonic from B1 into n77 |  | - |  | - |  | - Harmonic problem already covered in DC_1A_n77A |
| DC_1_n78-n258 | DC_1A_n78A |  |  |  | 7th & 8th Harmonics from n78 into n258 |  | - |  | - |  | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  | DC_1A_n258A |  |  |  | - |  | - |  | - |  | No issue |
| DC_1_n79-n258 | DC_1A_n79A |  |  |  | 5th & 6th Harmonics from n79 into n258 |  | - |  | - |  | - Harmonic problem will be solved in DC_n79A-n258A (5th order) |
|  | DC_1A_n258A |  |  |  | - |  | - |  | - |  | No issue |
| DC_3_n1-n257 | DC_3A_n1A |  |  |  | - |  | - |  | Yes |  | No issue |
|  | DC_3A_n257A |  |  |  | - |  | - |  | - |  | No issue |
| DC_3_n77-n258 | DC_3A_n77A |  |  |  | 6th, 7th & 8th harmonics from n77 into n258 |  | - |  | - |  | - No harmonic problems by 6th ~ 8th order between FR1 and FR2 |
|  | DC_3A_n258A |  |  |  | 2nd harmonic from B3 into B77 |  | - |  | - |  | - Harmonic problem already covered in DC_3A_n77A |
| DC_3_n78-n258 | DC_3A_n78A |  |  |  | 7th & 8th harmonics from n78 into n258 |  | - |  | - |  | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  | DC_3A_n258A |  |  |  | 2nd harmonic from B3 into B78 |  | - |  | - |  | - Harmonic problem already covered in DC_3A_n78A |
| DC_3_n79-n258 | DC_3A_n79A |  |  |  | 5th & 6th Harmonics from n79 into n258 |  | - |  | - |  | - Harmonic problem will be solved in DC_n79A-n258A (5th order) |
|  | DC_3A_n258A |  |  |  | - |  | - |  | - |  | No issue |
| DC_8_n77-n257 | DC_8A_n77A |  |  |  | 7th & 8th Harmonics from n77 into n257 |  | - |  | - |  | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  | DC_8A_n257A |  |  |  | 4th harmonic from B8 into n77 |  | - |  | - |  | - Harmonic problem already covered in DC_8A_n77A |
| DC_1_n5-n78 | DC_1A_n5A |  |  |  | - |  | 3rd IMD |  | - |  | - 18.1 |
|  | DC_1A_n78A |  |  |  | - |  | 5th IMD |  | - |  | Already specified in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
| DC_3_n5-n78 | DC_3A_n5A, DC_3C_n5A |  |  |  | 2nd harmonic from B3 into n78 |  | 3rd & 5th IMDs |  | - |  | - 2nd harmonic problem already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - 16.1 - FFS |
|  | DC_3A_n78A, DC_3C_n78A |  |  |  | - |  | - |  | - |  | No issue |
| DC_7_n5-n78 | DC_7A_n5A, DC_7C_n5A |  |  |  | - |  | 2nd & 4th IMDs |  | - |  | - 29.7 - FFS |
|  | DC_7A_n78A, DC_7C_n78A |  |  |  | - |  | 2nd & 5th IMDs |  | - |  | Already specified these IMDs problems for DC_5-7_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
| DC_28_n5-n78 | DC_28A_n5A |  |  |  | 4th harmonic from n5 into n78 5th harmonic from B28 into n78 |  |  |  |  |  | - These harmonic problems already specified in Table 7.3B.2.3.1-1 in TS38.101-3 |
|  | DC_28A_n78A |  |  |  | - |  | 5th IMD |  | - |  | - 3.8 |
| DC_39_n41-n79 | DC_39A_n41A |  |  |  | 2nd harmonic from n41 into n79 |  | 2nd I& 5th MDs |  | - |  | - Over 4992MHz frequency in n79 was impacted by 2nd harmonic, but not used - FFS - FFS |
|  | DC_39A_n79A |  |  |  | - |  | 2nd & 5th IMDs |  | - |  | - FFS - FFS |
| DC_40_n41-n79 | DC_40A_n41A |  |  |  | 2nd harmonic from n40 into n79 2nd harmonic from n41 into n79 |  | 2nd & 4th IMDs |  | - |  | - FFS by 2nd harmonic from B40 - Over 4992MHz frequency in n79 was impacted by 2nd harmonic, but not used - FFS - No impact to china band since over 4800MHz is considered in NR |
|  | DC_40A_n79A |  |  |  | - |  | 2nd , 4th & 5th IMDs |  | - |  | Band 40 and n41 should be operate the TDD synchronous operation No MSD into n41 |
| DC_2_n7-n66 | DC_2A_n7A |  |  |  | - |  | - |  | - |  | No issue |
|  | DC_2A_n66A |  |  |  | - |  | - |  | - |  | No issue |
| DC_12_n7-n66 | DC_12A_n7A |  |  |  | 3rd harmonic from B12 into n66 |  | - |  | - |  | - The harmonic problem already specified in Table 7.3B.2.3.1-1 in TS38.101-3 |
|  | DC_12A_n66A |  |  |  | - |  | - |  | - |  | No issue |
| DC_5_n7-n78 | DC_5A_n7A |  |  |  | - |  | 2nd & 4th IMDs |  | - |  | - These MSD issue follow DC_7_n5-n78 |
|  | DC_5A_n78A |  |  |  | - |  | 2nd IMD |  | Yes |  | - 2nd IMD problems was covered in DC_5A-7A_n78A. - The cross band isolation issue already covered in Table 7.3B.2.3.4-1 |
| DC_7_n7-n78 | DC_7A_n7A |  |  |  | - |  | - |  | - |  | No issue |
|  | DC_7A_n78A |  |  |  | - |  | - |  | Yes |  | - The cross band isolation issue already covered in Table 7.3B.2.3.4-1 |
| DC_12_n7-n78 | DC_12A_n7A |  |  |  | 5th harmonic from B12 into n78 |  | 4th IMD |  |  |  | - 5th harmonic issue will be covered in DC_12A_n78A - FFS |
|  | DC_12A_n78A |  |  |  | - |  | 2nd IMD |  | Yes |  | - FFS - The cross band isolation issue already covered in Table 7.3B.2.3.4-1 |
| DC_66_n7-n78 | DC_66A_n7A |  |  |  | 2nd harmonic from B66 into n78 |  | 3rd IMD |  | - |  | - The harmonic problem already specified in Table 7.3B.2.3.1-1 in TS38.101-3 |
|  | DC_66A_n78A |  |  |  | - |  | - |  | Yes |  | - The cross band isolation issue already covered in Table 7.3B.2.3.4-1 |
| DC_2_n66-n78 | DC_2A_n66A |  |  |  | 2nd harmonic from B2 into n78 2nd harmonic from n66 inton78 |  | 2nd & 4th IMDs |  | - |  | - These harmonic problems already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - FFS - FFS |
|  | DC_2A_n78A |  |  |  | - |  | 4th IMD |  | - |  | 4th IMD problem same as DC_2A-66A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
| DC_12_n66-n78 | DC_12A_n66A |  |  |  | 2nd harmonic from n66 inton78 5th harmonic from B12 into n78 |  | 5th IMD |  |  |  | - 2nd harmonic is same as DC_66A_n78A and 5th harmonic will be covered in DC_12A_n78A - FFS |
|  | DC_12A_n78A |  |  |  | 3rd harmonic from B12 into n66 |  | 3rd IMD |  |  |  | - The harmonic problem already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - FFS |
| DC_3-3_n1_n78 | DC_3A_n1A |  |  |  | 2nd harmonic from 3 into n78 |  | 2nd & 4th IMDs |  | - |  | - Harmonic problems was already covered in DC_3A_n78A. - 2nd &4th IMDs problems same as DC_1A_n3A-n78A. |
|  | DC_3A_n78A |  |  |  | - |  | 5th IMD |  | Yes |  | - 3.5 dB by 5th IMD in rel-15 TR - Small freq. gap was covered in Table 7.3.1A-0bA in TS36.101 |
| DC_7-7_n1_n78 | DC_7A_n1A |  |  |  |  |  | 4th & 5th IMDs |  | Yes |  | - 10.1 dB by 4th IMD in rel-15 TR. - 3.8 dB by 5th IMD in rel-15 TR. - The cross band isolation issue already covered in Table 7.3B.2.3.4-1 |
|  | DC_7A_n78A |  |  |  |  |  | 4th IMD |  | - |  | - 9.0 dB by 4th IMD in rel-15 TR. |
| DC_2_n41-n260 | DC_2A_n41A |  |  |  | High order harmonic into n260 |  | - |  | - |  | - No harmonic problem |
| DC_2_n41-n261 | DC_2A_n41A |  |  |  | High order harmonic into n261 |  | - |  | - |  | - No harmonic problem |
| DC_66_n41_n260 | DC_66A_n41A |  |  |  | High order harmonic into n260 |  | - |  | - |  | - No harmonic problem |
| DC_66_n41_n261 | DC_66A_n41A |  |  |  | High order harmonic into n261 |  | - |  | - |  | - No harmonic problem |
| DC_2_n5-n258 | DC_2A_n5A |  |  |  | High order harmonic into n258 |  | - |  | - |  | - No harmonic problem |
|  | DC_2A_n258A |  |  |  | - |  | - |  | - |  | No issue |
| DC_2_n5-n260 | DC_2A_n5A |  |  |  | High order harmonic into n260 |  | - |  | - |  | - No harmonic problem |
|  | DC_2A_n260A |  |  |  | - |  | - |  | - |  | No issue |
| DC_2_n5-n261 | DC_2A_n5A |  |  |  | High order harmonic into n261 |  | - |  | - |  | - No harmonic problem |
|  | DC_2A_n261A |  |  |  | - |  | - |  | - |  | No issue |
| DC_2_n71-n258 | DC_2A_n71A |  |  |  | High order harmonic into n258 |  | - |  | - |  | - No harmonic problem |
|  | DC_2A_n258A |  |  |  | - |  | - |  | - |  | No issue |
| DC_2_n71-n260 | DC_2A_n71A |  |  |  | High order harmonic into n260 |  | - |  | - |  | - No harmonic problem |
|  | DC_2A_n260A |  |  |  | - |  | - |  | - |  | No issue |
| DC_2_n71-n261 | DC_2A_n71A |  |  |  | High order harmonic into n261 |  | - |  | - |  | - No harmonic problem |
|  | DC_2A_n261A |  |  |  | - |  | - |  | - |  | No issue |
| DC_66_n5-n258 | DC_66A_n5A |  |  |  | High order harmonic into n258 |  | - |  | - |  | - No harmonic problem |
|  | DC_66A_n258A |  |  |  | - |  | - |  | - |  | No issue |
| DC_66_n5-n261 | DC_66A_n5A |  |  |  | High order harmonic into n261 |  | - |  | - |  | - No harmonic problem |
|  | DC_66A_n261A |  |  |  | - |  | - |  | - |  | No issue |
| DC_66_n71-n258 | DC_66A_n71A |  |  |  | High order harmonic into n258 |  | - |  | - |  | - No harmonic problem |
|  | DC_66A_n258A |  |  |  | - |  | - |  | - |  | No issue |
| DC_3_n77-n257 | DC_3A_n77A |  |  |  | 7th & 8th harmonics from n77 into n257 |  | - |  | - |  | - No harmonic problems by 6th ~ 8th order between FR1 and FR2 |
|  | DC_3A_n257A, DC_3A_n257D, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I |  |  |  | 2nd harmonic from B3 into B77 |  | - |  | - |  | - Harmonic problem already covered in DC_3A_n77A |
| DC_7_n66-n78 | DC_7A_n66A |  |  |  | 2nd harmonic from n66 into n78 |  | 3rd IMD |  |  |  | - Harmonic problem already covered in DC_66A_n78A - FFS |
|  | DC_7A_n78A |  |  |  | - |  | 4th IMD |  | Yes |  | - 4th IMD problem same as DC_7A-66A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
| DC_66_n66-n78 | DC_66A_n66A |  |  |  | 2nd harmonic from B66 into n78 |  | 2nd & 4th IMDs |  |  |  | - Harmonic problem already covered in DC_66A_n78A - FFS - FFS |
|  | DC_66A_n78A |  |  |  | - |  | 5th IMD |  |  |  | - Follow MSD result for DC_66A_n78A |
| DC_20_n1-n7 | DC_20A_n1A |  |  |  | - |  | - |  | - |  | No issue |
|  | DC_20A_n7A |  |  |  | - |  | - |  | - |  | No issue |
| DC_3_n1-n7 | DC_3A_n1A, DC_3C_n1A |  |  |  | - |  | - |  | - |  | No issue |
|  | DC_3A_n7A, DC_3C_n7A |  |  |  | - |  | - |  | - |  | No issue |
| DC_20_n1-n28 | DC_20A_n1A |  |  |  | - |  | - |  | - |  | No issue |
|  | DC_20A_n28A |  |  |  | 3rd harmonic from n28 into n1 |  | 5th IMD |  | - |  | - Harmonic problem already covered in DC_1A_n28A - FFS |
| DC_3_n1-n28 | DC_3A_n1A, DC_3C_n1A |  |  |  | - |  | - |  | - |  | No issue |
|  | DC_3A_n28A, DC_3C_n28A |  |  |  | 3rd harmonic from n28 into n1 |  | 4th IMD |  | - |  | - Harmonic problem already covered in DC_1A_n28A - 4th IMD problem same as DC_1A-3A_28A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
| DC_20_n7-n28 | DC_20A_n7A |  |  |  | - |  | 3rd & 5th IMDs |  | - |  | - FFS - FFS |
|  | DC_20A_n28A |  |  |  | - |  | 5th IMD |  | - |  | - 5th IMD problem same as DC_7A-20A_n28A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
| DC_20_n7-n78 | DC_20A_n7A |  |  |  | 4th harmonic from B20 into n78 |  | 2nd & 4th IMDs |  | - |  | - The harmonic problem already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - FFS - FFS |
|  | DC_20A_n78A |  |  |  | - |  | 2nd IMD |  | Yes |  | - 2nd IMD problem same as DC_7A-20A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 - The cross band isolation issue already covered in Table 7.3B.2.3.4-1 |
| DC_3_n7-n28 | DC_3A_n7A, DC_3C_n7A |  |  |  |  |  | 2nd IMD |  | - |  | - FFS |
|  | DC_3A_n28A, DC_3C_n28A |  |  |  | - |  | 3rd IMD |  | - |  | - 2nd IMD problem same as DC_3A-7A_n28A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
| DC_2_n41-n71 | DC_2A_n41A |  |  |  | - |  | 2nd & 5th IMDs |  | - |  | - FFS - FFS |
|  | DC_2A_n71A |  |  |  | 4th harmonic from n71 into n41 |  | 2nd IMD |  | - |  | The harmonic problem will be treated CA_n41A_n71A - FFS |
| DC_66_n41-n71 | DC_66A_n41A |  |  |  | - |  | - |  | - |  | No issue |
|  | DC_66A_n71A |  |  |  | 4th harmonic from n71 into n41 |  | - |  | - |  | The harmonic problem will be treated CA_n41A_n71A |
| DC_18_n3-n78 | DC_18A_n3A |  |  |  | 2nd harmonic from n3 into n78 4th harmonic from B18 into n78 |  | 3rd & 5th IMDs |  | - |  | These harmonic problem already specified in in Table 7.3B.2.3.1-1 in TS38.101-3 - FFS - FFS |
|  | DC_18A_n78A |  |  |  | - |  | 3rd IMD |  |  |  | - Only one Japan operation use B18 and the n78 freq. range is allocated 3600 to 3800 MHz. So No IMD problem |
| DC_28_n3-n78 | DC_28A_n3A |  |  |  | 2nd harmonic from n3 into n78 5th harmonic from B28 into n78 |  | 5th IMD |  | - |  | - 5th IMD problem same as DC_3A_n28A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
|  | DC_28A_n78A |  |  |  | 5th harmonic from B28 into n78 |  | 3rd IMD |  | - |  | - The harmonic problem already specified in in Table 7.3B.2.3.1-1 in TS38.101-3 - 3rd IMD already covered in DC_3A-28A_n78A |
| DC_8_n1-n78 | DC_8A_n1A |  |  |  | 4th harmonic from B8 into n78 |  | 3rd IMD |  | - |  | - The harmonic problem already specified in in Table 7.3B.2.3.1-1 in TS38.101-3 - FFS |
|  | DC_8A_n78A |  |  |  | - |  | - |  | - |  | No issue |
| DC_3_n40-n41 | DC_3A_n40A |  |  |  | 3rd harmonic from B3 into n41 |  | - |  | - |  | CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band |
|  | DC_3A_n41A |  |  |  | - |  | - |  | Yes |  | CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band - The cross band isolation issue already covered in Table 7.3B.2.3.4-1 |
| DC_3_n40-n79 | DC_3A_n40A |  |  |  | 2nd harmonic from n40 into n79 |  | 5th IMD |  | - |  | - The 2nd harmonic problem will be studied in CA_n40-n79. - FFS |
|  | DC_3A_n79A |  |  |  | - |  | 5th IMD |  | - |  | - FFS |
| DC_3_n41-n79 | DC_3A_n41A |  |  |  | 2nd harmonic from n41 into n79 |  | 2nd & 5th IMDs |  | Yes |  | - The 2nd harmonic problem will be studied in CA_n41-n79. - FFS - FFS - The cross band isolation issue already covered in Table 7.3B.2.3.4-1 |
|  | DC_3A_n79A |  |  |  | - |  | 2nd & 5th IMDs |  | - |  | - Already MSD analysis are covered in DC_3A-41A_n79A |
| DC_8_n40-n41 | DC_8A_n40A |  |  |  | 3rd harmonic from B8 into n41 |  | - |  | - |  | - Harmonic problem will be studied in DC_8A_n41A. - CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band |
|  | DC_8A_n41A |  |  |  | - |  | 5th IMD |  | - |  | - CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band |
| DC_8_n40-n79 | DC_8A_n40A |  |  |  | 2nd harmonic from n40 into n79 5th harmonic from B8 into n79 |  | 4th IMD |  | - |  | - These harmonic problem will be treated lower order DC or CA band combos. - FFS |
|  | DC_8A_n79A |  |  |  | - |  | 4th IMD |  | - |  | - FFS |
| DC_8_n41-n79 | DC_8A_n41A |  |  |  | 2nd harmonic from n41 into n79 5th harmonic from B8 into n79 |  | 3rd IMD |  | - |  | - These harmonic problem will be treated lower order DC or CA band combos. - FFS |
|  | DC_8A_n79A |  |  |  | 3rd harmonic from B8 into n41 |  | 3rd IMD |  | - |  | - FFS |
| DC_39_n40-n41 | DC_39A_n40A |  |  |  | - |  | 3rd IMD |  | - |  | - CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band |
|  | DC_39A_n41A |  |  |  | - |  | - |  | - |  | - CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band |
| DC_39_n40-n79 | DC_39A_n40A |  |  |  | 2nd harmonic from n40 into n79 |  | 4th IMD |  | - |  | - The harmonic problem will be treated CA_n40-n79 band combos. - FFS |
|  | DC_39A_n79A |  |  |  | - |  | - |  | - |  | No issue |
| DC_41_n78-n257 | DC_41A_n78A |  |  |  | 7th & 8th harmonics from n78 into n257 |  | - |  | - |  | No harmonic impact to n257 |
|  | DC_41A_n257A, DC_41A_n257G, DC_41A_n257H, DC_41A_n257I |  |  |  | - |  | - |  | - |  | No issue |
| DC_42_n78-n257 | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I |  |  |  | - |  | - |  | - |  | No issue |
| DC_18_n78-n257 | DC_18A_n78A |  |  |  | 7th & 8th harmonics from n78 into n257 |  | - |  | - |  | No harmonic impact to n257 |
|  | DC_18A_n257A, DC_18A_n257G, DC_18A_n257H, DC_18A_n257I |  |  |  | - |  | - |  | - |  | No issue |
| DC_28_n78-n257 | DC_28A_n78A |  |  |  | 7th & 8th harmonics from n78 into n257 |  | - |  | - |  | No harmonic impact to n257 |
|  | DC_28A_n257A, DC_28A_n257G, DC_28A_n257H, DC_28A_n257I |  |  |  | - |  | - |  | - |  | No issue |
| DC_1_n3-n257 | DC_1A_n3A |  |  |  | High order harmonic into n257 |  | - |  | -- |  | No harmonic impact to n257 |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I |  |  |  | - |  | - |  | Yes |  | Cross isolation issue already covered in Table 7.3.1A-0bA in TS36.101 |
| DC_18_n3-n257 | DC_18A_n3A |  |  |  | High order harmonics into n257 |  | - |  | - |  | No harmonic problem to n257 |
|  | DC_18A_n257A, DC_18A_n257G, DC_18A_n257H, DC_18A_n257I |  |  |  | - |  | - |  | - |  | No issue |
| DC_28_n3-n257 | DC_28A_n3A |  |  |  | High order harmonics into n257 |  | - |  | - |  | No harmonic problem to n257 |
|  | DC_28A_n257A, DC_28A_n257G, DC_28A_n257H, DC_28A_n257I |  |  |  | - |  | - |  | - |  | No issue |
| DC_1_n3-n28 | DC_1A_n3A |  |  |  | - |  | - |  | - |  | No issue |
|  | DC_1A_n28A |  |  |  | - |  | 5th IMD |  | Yes |  | - The 5th IMD problem same as DC_1A-3A_n28A in Table 7.3B.2.3.5.2-1 in TS38.101-3 - The cross band isolation already treated in TS36.101 |
| DC_8_n3-n28 | DC_8A_n3A |  |  |  | - |  | 2nd & 5th IMDs |  | - |  | - FFS - FFS |
|  | DC_8A_n28A |  |  |  | 2nd harmonic from B8 into n3 |  | - |  | - |  | The 2nd harmonic problem will be covered in DC_8A_n3A |
| DC_11_n77-n257 | DC_11A_n77A |  |  |  | 7th & 8th harmonics from n77 into n257 |  | - |  | - |  | No harmonic problem to n257 |
|  | DC_11A_n257A |  |  |  | - |  | - |  | - |  | No issue |
| DC_2_n12-n258 | DC_2A_n12A |  |  |  | High order harmonic into n258 |  | - |  | - |  | No harmonic problem to n258 |
|  | DC_2A_n258A |  |  |  | - |  | - |  | - |  | No issue |
| DC_2_n12-n260 | DC_2A_n12A |  |  |  | High order harmonic into n260 |  | - |  | - |  | No harmonic problem to n260 |
|  | DC_2A_n260A |  |  |  | - |  | - |  | - |  | No issue |
| DC_2_n12-n261 | DC_2A_n12A |  |  |  | High order harmonic into n261 |  | - |  | - |  | No harmonic problem to n261 |
|  | DC_2A_n261A |  |  |  | - |  | - |  | - |  | No issue |
| DC_66_n12-n258 | DC_66A_n12A |  |  |  | High order harmonic into n258 |  | - |  | - |  | No harmonic problem to n258 |
|  | DC_66A_n258A |  |  |  | - |  | - |  | - |  | No issue |
| DC_66_n12-n260 | DC_66A_n12A |  |  |  | High order harmonic into n260 |  | - |  | - |  | No harmonic problem to n260 |
|  | DC_66A_n260A |  |  |  | - |  | - |  | - |  | No issue |
| DC_66_n12-n261 | DC_66A_n12A |  |  |  | High order harmonic into n261 |  | - |  | - |  | No harmonic problem to n261 |
|  | DC_66A_n261A |  |  |  | - |  | - |  | - |  | No issue |
| DC_7-7_n66-n78 | DC_7A_n66A |  |  |  | 2nd harmonic from n66 into n78 |  | 3rd IMD |  | - |  | The harmonic problem already covered in Table 7.3B.2.3.1-1 in TS38.101-3 - FFS |
|  | DC_7A_n78A |  |  |  | - |  | 4th IMD |  | Yes |  | - The 4th IMD problem same as DC_7A-66A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 - The cross band isolation issue already covered in Table 7.3B.2.3.4-1 |

Table 5.3-2 summarizes the EN-DC band combinations with self-interference problems for 4DL/2UL DC operation.

Table 5.3-2: Summary of Self-interference analysis for LTE 2 bands & NR 2 bands DL and 2 bands UL EN-DC operation

| Downlink band configuration | Uplink DC Configuration |  | Harmonic relation issues |  | intermodulation to own rx band |  | interference due to small frequency separation | MSD |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1-3_n7-n78 | DC_1A_n7A |  | - |  | 4th & 5th IMDs into n78 |  | Yes | These IMD problems were already covered in DC_7_n1-n78 with 2UL_DC_7A_n1A |  |
|  | DC_1A_n78A, DC_1A_n78(2A) |  | - |  | 2nd IMD into n3 4th IMD into n7 |  | Yes | These IMD issues were already covered in DC_1A-3A_n78A and DC_1A_n7A-n78A. |  |
|  | DC_3A_n7A |  | 2nd harmonic from 3 into n78 |  | 3rd IMD into n78 |  | - | - Harmonic problem was solved in DC_3A_n78A - 3rd IMD problem already covered in DC_3A_n7A-n78A. |  |
|  | DC_3A_n78A, DC_3A_n78(2A) |  |  |  | 5th IMD into B1 |  |  | -5th IMD problem was solved in 3DL_DC_1A-3A_n78A |  |
| DC_1-3_n28-n78 | DC_1A_n28A |  | 3rd harmonic into B1 5th harmonic into n78 |  | 3rd IMD into n78 5th IMD into B3 |  | Yes | - Harmonic problems already solved in DC_1A_n28A or DC_28A_n78A - 3rd IMD problem was solved in 3DL_DC_1A_n28A-n78A - 5th IMD problem was solved in 3DL_DC_1A-3A_n28A |  |
|  | DC_1A_n78A |  | - |  | 2nd IMD into B3 4th IMD into B1 4th IMD into n78 5th IMD into n28 |  | Yes | - 2nd IMD problem was solved in 3DL_DC_1A-3A_n78A - 4th IMD problem already covered in DC_1A_n78A - 4th IMD problem is no issue for TDD band - 5th IMD problem was solved in 3DL_DC_1A_n28A-n78A |  |
|  | DC_3A_n28A |  | 2nd harmonic into n78 3rd harmonic into B1 5th harmonic into n78 |  | 4th IMD into B1 5th IMD into n78 |  |  | - Harmonic problems already solved in DC_3A_n78A or DC_1A_n28A or DC_28A_n78A - 4th IMD problem was solved in 3DL_DC_1A-3A_n28A - 5th IMD problem was solved in 3DL_DC_3A_n28A-n78A |  |
|  | DC_3A_n78A |  | 2nd harmonic into n78 |  | 2nd,4th & 5th IMDs into B3 4th & 5th IMDs into n78 5th IMD into B1 |  |  | - 2nd harmonic problem already solved in DC_3A_n78A - 2nd,4th & 5th IMDs problems already solved in DC_3A_n78A - 4th & 5th IMDs problems are no issue for TDD band -5th IMD problem was solved in 3DL_DC_1A-3A_n78A |  |
| DC_1-3_n78-n257 | DC_1A_n78A |  | 7th & 8th Harmonics from n78 into n257 |  | 2nd IMD into B3 |  | Yes | - Up to 7th harmonic order will be not any impact to n257 - This IMD problem will be covered in DC_1A-3A_n78A - Proximity also follow LTE_CA_1A-3A |  |
|  | DC_1A_n257A/D/G/H/I |  | 2nd harmonic from B1 into n78 |  | - |  | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |  |
|  | DC_3A_n78A |  | 7th & 8th harmonics from n78 into n257 |  | 5th IMD into B1 |  | - | - Up to 7th harmonic order will be not any impact to n257 - 5th IMD problems will be covered in DC_1A-3A_n78A. |  |
|  | DC_3A_n257A/D/G/H/I |  | 2nd harmonic from B3 into n78 |  | - |  | - | - Harmonic problem will be solved in DC_3A_n78A |  |
| DC_1-5_n78-n257 | DC_1A_n78A |  | 7th & 8th Harmonics from n78 into n257 |  | 5th IMD into B5 |  | - | - Up to 7th harmonic order will be not any impact to n257 - 5th IMD problems will be covered in DC_1A-5A_n78A. |  |
|  | DC_1A_n257A |  | 2nd harmonic from B1 into n78 |  | - |  | - | - Harmonic problem already discussed and will be solved in DC_1A_n78A |  |
|  | DC_5A_n78A |  | 7th & 8th harmonics from n78 into n257 |  | 3rd IMD into B1 |  | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 3rd IMD problems will be covered in DC_1A-5A_n78A. |  |
|  | DC_5A_n257A |  | 4th harmonic from B5 into n78 |  | - |  | - | - Harmonic problem will be solved in DC_5A_n78A |  |
| DC_1-7_n28-n78 | DC_1A_n28A |  | 3rd harmonic into B1 5th harmonic into n78 |  | 2nd IMD into B7 3rd IMD into n78 |  |  | - Harmonic problems already solved in DC_1A_n78A or DC_28A_n78A - 2nd IMD problem was solved in 3DL_DC_1A-7A_n28A - 3rd IMD problem was solved in 3DL_DC_1A_n28A-n78A |  |
|  | DC_1A_n78A |  | - |  | 4th IMD into B1 4th IMD into n7 5th IMD into n28 |  |  | - 4th IMD problem already solved in DC_1A_n78A - 4th IMD problem is no issue for TDD band - 5th IMD problem was solved in 3DL_DC_1A_n28A-n78A |  |
|  | DC_7A_n28A |  | 3rd harmonic into B1 5th harmonic into n78 |  | 2nd,4th IMDs into n78 |  | - | - Harmonic problem was solved in DC_1A_n28A or DC_28A_n78A - 2nd & 4th IMDs problems were solved in 3DL_DC_7A_n28A-n78A |  |
|  | DC_7A_n78A |  | - |  | 2nd & 5th IMDs into B28 4th IMD into n78 4th IMD into B1 |  | - | - 2nd & 5th IMDs problems were solved in 3DL_DC_7A-28A_n78A - 4th IMD problem is no issue for TDD band - 4th IMD problem was solved in 3DL_DC_1A-7A_n78A |  |
| DC_1-7-7_n78-n257 | DC_1A_n78A |  | 7th & 8th Harmonics from n78 into n257 |  | 4th IMD into B7 |  | - | - Up to 7th harmonic order will be not any impact to n257 - 4th IMD problems will be covered in DC_1A-7A_n78A. |  |
|  | DC_1A_n257A |  | 2nd harmonic from B1 into n78 |  | - |  | - | - Harmonic problem already discussed and will be solved in DC_1A_n78A |  |
|  | DC_7A_n78A |  | 7th harmonic from B7 into n257 |  | 4th IMD into B1 |  | - | - Up to 7th harmonic order will be not any impact to n257 - The IMD problem will be covered in DC_1A_7A-n78A |  |
|  | DC_7A_n257A |  | - |  | - |  | - | No issues |  |
| DC_1-7_n78-n257 | DC_1A_n78A |  | 7th & 8th Harmonics from n78 into n257 |  | 4th IMD into B7 |  | - | - Up to 7th harmonic order will be not any impact to n257 - 4th IMD problems will be covered in DC_1A-7A_n78A. |  |
|  | DC_1A_n257A |  | 2nd harmonic from B1 into n78 |  | - |  | - | - Harmonic problem already discussed and will be solved in DC_1A_n78A |  |
|  | DC_7A_n78A |  | 7th harmonic from B7 into n257 |  | 4th IMD into B1 |  | - | - Up to 7th harmonic order will be not any impact to n257 - The IMD problem will be covered in DC_1A_7A-n78A |  |
|  | DC_7A_n257A |  | - |  | - |  | - | No issues |  |
| DC_1-8_n78-n257 | DC_1A_n78A |  | 7th & 8th Harmonics from n78 into n257 |  | 5th IMD into B8 |  | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 5th IMD already covered in DC_1A-8A_n78A |  |
|  | DC_1A_n257A |  | 2nd harmonic from B1 into n78 |  | - |  | - | - Harmonic problem already discussed and will be solved in DC_1A_n78A |  |
|  | DC_8A_n78A |  | 7th & 8th Harmonics from n78 into n257 |  | - |  | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |  |
|  | DC_8A_n257A |  | 4th harmonic from B8 into n78 |  | - |  | - | - Harmonic problem will be solved in DC_8A_n78A |  |
| DC_3-5_n78-n257 | DC_3A_n78A |  | 7th & 8th harmonics from n78 into n257 |  | - |  | - | - Up to 7th harmonic order will be not any impact to n257 |  |
|  | DC_3A_n257A |  | 2nd harmonic from B3 into n78 |  | - |  | - | - Harmonic problem will be solved in DC_3A_n78A |  |
|  | DC_5A_n78A |  | 7th & 8th harmonics from n78 into n257 |  | 3rd IMD into B3 |  | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 3rd IMD problems will be covered in DC_3A-5A_n78A. |  |
|  | DC_5A_n257A |  | 4th harmonic from B5 into n78 |  | - |  | - | - Harmonic problem will be solved in DC_5A_n78A |  |
| DC_3-7_n28-n78 | DC_3A_n28A |  | 2nd harmonic into n78 5th harmonic into n78 |  | 3rd IMD into B7 5th IMD into n78 |  | - | - Harmonic problems already solved in DC_3A_n78A or DC_28A_n78A - 3rd IMD problem was solved in 3DL_DC_3A-7A_n28A - 5th IMD problem was solved in 3DL_DC_3A_n28A-n78A |  |
|  | DC_3A_n78A |  | 2nd harmonic into n78 |  | 2nd,4th & 5th IMDs into B3 4th & 5th IMDs into n78 |  |  | - 2nd harmonic problem already solved in DC_3A_n78A - 2nd,4th & 5th IMDs problems already solved in DC_3A_n78A - 4th & 5th IMDs problems are no issue for TDD band |  |
|  | DC_7A_n28A |  | 5th harmonic into n78 |  | 2nd IMD into B3 2nd,4th IMDs into n78 |  | - | - Harmonic problem was solved in DC_28A_n78A - 2nd IMD problems was solved in 3DL_DC_3A-7A_n28A - 2nd & 4th IMDs problems were solved in 3DL_DC_7A_n28A-n78A |  |
|  | DC_7A_n78A |  | - |  | 2nd & 5th IMDs into B28 3rd & 4th IMDs into B3 4th IMD into n78 |  | - | - 2nd & 5th IMDs problems were solved in 3DL_DC_7A_n28A-n78A - 3rd & 4th IMDs problems were solved in 3DL_DC_3A-7A_n78A - 4th IMD problem is no issue for TDD band |  |
| DC_3-7-7_n78-n257 DC_3-3-7_n78-n257 DC_3-3-7-7_n78-n257 | DC_3A_n78A |  | 7th & 8th harmonics from n78 into n257 |  | - |  | - | - Up to 7th harmonic order will be not any impact to n257 |  |
|  | DC_3A_n257A |  | 2nd harmonic from B3 into n78 |  | - |  | - | - Harmonic problem will be solved in DC_3A_n78A |  |
|  | DC_7A_n78A |  | 7th harmonic from B7 into n257 |  | 3rd & 4th IMDs into B3 |  | - | - Up to 7th harmonic order will be not any impact to n257 - The IMD problem will be covered in DC_3A-7A_n78A |  |
|  | DC_7A_n257A |  | - |  | - |  | - | No issues |  |
| DC_3-7_n78-n257 | DC_3A_n78A |  | 7th & 8th harmonics from n78 into n257 |  | - |  | - | - Up to 7th harmonic order will be not any impact to n257 |  |
|  | DC_3A_n257A |  | 2nd harmonic from B3 into n78 |  | - |  | - | - Harmonic problem will be solved in DC_3A_n78A |  |
|  | DC_7A_n78A |  | 7th harmonic from B7 into n257 |  | 3rd & 4th IMDs into B3 |  | - | - Up to 7th harmonic order will be not any impact to n257 - The IMD problem will be covered in DC_3A-7A_n78A |  |
|  | DC_7A_n257A |  | - |  | - |  | - | No issues |  |
| DC_3-8_n78-n257 | DC_3A_n78A |  | 7th & 8th harmonics from n78 into n257 |  | - |  | - | - Up to 7th harmonic order will be not any impact to n257 |  |
|  | DC_3A_n257A |  | 2nd harmonic from B3 into n78 |  | - |  | - | - Harmonic problem will be solved in DC_3A_n78A |  |
|  | DC_8A_n78A |  | 2nd harmonic from B8 into B3 |  | 3rd IMD into B3 |  | - | - Harmonic problem already be solved in CA_3A_8A - 3rd IMD already covered in DC_3A-8A_n78A |  |
|  | DC_8A_n257A |  | 4th harmonic from B8 into n78 |  | - |  | - | - Harmonic problem will be solved in DC_8A_n78A |  |
| DC_5-7-7_n78-n257 | DC_5A_n78A |  | 7th & 8th harmonics from n78 into n257 |  | 2nd IMD into B7 |  | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 2nd IMD problems will be covered in DC_5A-7A_n78A. |  |
|  | DC_5A_n257A |  | 4th harmonic from B5 into n78 |  | - |  | - | - Harmonic problem will be solved in DC_5A_n78A |  |
|  | DC_7A_n78A |  | 7th harmonic from B7 into n257 |  | 2nd & 5th IMDs into B5 |  | - | - Up to 7th harmonic order will be not any impact to n257 - The IMD problems will be covered in DC_5A-7A_n78A |  |
|  | DC_7A_n257A |  | - |  | - |  | - | No issues |  |
| DC_5-7_n78-n257 | DC_5A_n78A |  | 7th & 8th harmonics from n78 into n257 |  | 2nd IMD into B7 |  | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 2nd IMD problems will be covered in DC_5A-7A_n78A. |  |
|  | DC_5A_n257A |  | 4th harmonic from B5 into n78 |  | - |  | - | - Harmonic problem will be solved in DC_5A_n78A |  |
|  | DC_7A_n78A |  | 7th harmonic from B7 into n257 |  | 2nd & 5th IMDs into B5 |  | - | - Up to 7th harmonic order will be not any impact to n257 - The IMD problems will be covered in DC_5A-7A_n78A |  |
|  | DC_7A_n257A |  | - |  | - |  | - | No issues |  |
| DC_1-3_n77-n78 | DC_1A_n77A |  | - |  | 2nd, 4th & 5th IMDs into B3 4th & 5th IMD into n78 |  | Yes | These MSD issues are already covered in 3DL_DC_1A-3A_n77A with 2UL_DC_1A_n77A. These MSD issues are already covered in 3DL_DC_1A_n77A-n78A with 2UL_DC_1A_n77A. |  |
|  | DC_1A_n78A |  | 2nd harmonic from B1 into n77 |  | 2nd IMD into B3 4th & 5th IMDs into n77 |  | Yes | Harmonic problem already covered in DC_1A_n77A 2nd IMD problem already covered in 3DL_DC_1A-3A_n78A with 2UL_DC_1A_n78A 4th & 5th IMDs problem already covered in 3DL_DC_1A_n77A-n78A with 2UL_DC_1A_n78A |  |
|  | DC_3A_n77A |  | 2nd harmonic from B3 into n78 |  | 2nd & 5th IMD into B1 4th IMD & 5th IMD into n78 |  | - | 2nd harmonic issue already covered in DC_3A_n78A These MSD issues are already covered in 3DL_DC_1A-3A_n77A with 2UL_DC_3A_n77A and 3DL_3A_n77A-n78A with 2UL_DC_3A_n77A |  |
|  | DC_3A_n78A |  | 2nd harmonic from B3 into n77 |  | 4th & 5th IMDs into n77 5th IMD into B1 |  | - | 2nd harmonic issue already covered in DC_3A_n77A These MSD issues are already covered in 3DL_DC_1A-3A_n78A with 2UL_DC_3A_n78A and 3DL_3A_n77A-n78A with 2UL_DC_3A_n78A |  |
| DC_1-3_n77-n79 | DC_1A_n77A |  | - |  | 2nd, 4th & 5th IMDs into B3 3rd, 4th & 5th IMDs into n79 |  | Yes | These MSD issues are already covered in 3DL_DC_1A-3A_n77A with 2UL_DC_1A_n77A. These MSD issues will be covered in 3DL_DC_1A_n77A-n79A with 2UL_DC_1A_n77A. |  |
|  | DC_1A_n79A |  | 2nd harmonic from B1 into n77 |  | 5th IMD into n77 |  | Yes | 2nd harmonic issue already covered in DC_1A_n77A The MSD issue is already covered in 3DL_DC_1A_n77A-n79A with 2UL_DC_1A_n79A |  |
|  | DC_3A_n77A |  | - |  | 2nd & 5th IMD into B1 3rd & 4th IMD into n79 |  | - | These MSD issues are already covered in 3DL_DC_1A-3A_n77A with 2UL_DC_3A_n77A and 3DL_3A_n77A-n79A with 2UL_DC_3A_n79A |  |
|  | DC_3A_n79A |  | 2nd harmonic from B3 into n77 |  | 5th IMD into B1 5th IMD into n77 |  | - | 2nd harmonic issue already covered in DC_3A_n77A The MSD issue will be covered in 3DL_DC_1A-3A_n79A with 2UL_DC_3A_n79A and 3DL_3A_n77A-n79A with 2UL_DC_3A_n79A |  |
| DC_1-3_n78-n79 | DC_1A_n78A |  | - |  | 2nd IMD into B3 3rd & 5th IMDs into n79 |  | Yes | 2nd IMD problem already covered in 3DL_DC_1A-3A_n78A with 2UL_DC_1A_n78A 3rd & 5th IMDs problem will be covered in 3DL_DC_1A_n78A-n79A with 2UL_DC_1A_n78A |  |
|  | DC_1A_n79A |  | - |  | 5th IMD into n78 |  | Yes | The MSD issue is already covered in 3DL_DC_1A_n78A-n79A with 2UL_DC_1A_n79A |  |
|  | DC_3A_n78A |  | - |  | 5th IMD into B1 3rd IMD into n79 |  | - | These MSD issues are already covered in 3DL_DC_1A-3A_n78A with 2UL_DC_3A_n78A and 3DL_3A_n78A-n79A with 2UL_DC_3A_n78A |  |
|  | DC_3A_n79A |  | 2nd harmonic from B3 into n78 |  | 5th IMD into B1 5th IMD into n78 |  | - | 2nd harmonic issue already covered in DC_3A_n78A The MSD issue will be covered in 3DL_DC_1A-3A_n79A with 2UL_DC_3A_n79A and 3DL_3A_n78A-n79A with 2UL_DC_3A_n79A |  |
| DC_1-19_n77-n79 | DC_19A_n77A |  | 6th harmonic from B19 into n79 |  | 3rd IMD into B1 2nd, 3rd, 4th & 5th IMDs into n79 |  | - | 6th harmonic will not any impact to n79 The MSD issue will be covered in 3DL_DC_1A-19A_n77A with 2UL_DC_19A_n77A and 3DL_19A_n77A-n79A with 2UL_DC_19A_n77A |  |
|  | DC_19A_n79A |  | 4th harmonic from B19 into n77 |  | 4th IMD into B1 2nd & 3rd IMDs into n77 |  | - | 4th harmonic will be covered in DC_19A_n77A The MSD issue will be covered in 3DL_DC_1A-19A_n79A with 2UL_DC_19A_n79A and 3DL_19A_n77A-n79A with 2UL_DC_19A_n79A |  |
| DC_1-19_n78-n79 | DC_19A_n78A |  | 6th harmonic from B19 into n79 |  | 3rd IMD into B1 2nd, 3rd, 4th & 5th IMDs into n79 |  |  | 6th harmonic will not any impact to n79 The MSD issue will be covered in 3DL_DC_1A-19A_n78A with 2UL_DC_19A_n78A and 3DL_19A_n78A-n79A with 2UL_DC_19A_n78A |  |
|  | DC_19A_n79A |  | 4th harmonic from B19 into n78 |  | 4th IMD into B1 2nd & 3rd IMDs into n78 |  | - | 4th harmonic will be covered in DC_19A_n78A The MSD issue will be covered in 3DL_DC_1A-19A_n79A with 2UL_DC_19A_n79A and 3DL_19A_n78A-n79A with 2UL_DC_19A_n79A |  |
| DC_1-21_n77-n78 | DC_1A_n77A |  | - |  | 2nd & 5th IMDs into B21 4th & 5th IMD into n78 |  | - | These MSD issues are already covered in 3DL_DC_1A-21A_n77A with 2UL_DC_1A_n77A. These MSD issues are already covered in 3DL_DC_1A_n77A-n78A with 2UL_DC_1A_n77A. |  |
|  | DC_1A_n78A |  | 2nd harmonic from B1 into n77 |  | 2nd & 5th IMDs into B21 4th & 5th IMD into n77 |  | - | These MSD issues are already covered in 3DL_DC_1A-21A_n78A with 2UL_DC_1A_n78A. These MSD issues are already covered in 3DL_DC_1A_n77A-n78A with 2UL_DC_1A_n78A. |  |
| DC_1-21_n77-n79 | DC_1A_n77A |  | - |  | 2nd & 5th IMDs into B21 3rd, 4th & 5th IMD into n79 |  | - | These MSD issues are already covered in 3DL_DC_1A-21A_n77A with 2UL_DC_1A_n77A. These MSD issues are already covered in 3DL_DC_1A_n77A-n79A with 2UL_DC_1A_n77A. |  |
|  | DC_1A_n79A |  | 2nd harmonic from B1 into n77 |  | 4th IMD into B21 5th IMD into n77 |  | - | 2nd harmonic issue already covered in DC_1A_n77A The MSD issues are already covered in 3DL_DC_1A-21A_n79A with 2UL_DC_1A_n79A and 3DL_DC_1A_n77A-n79A with 2UL_DC_1A_n79A |  |
| DC_1-21_n78-n79 | DC_1A_n78A |  | - |  | 2nd & 5th IMDs into B21 3rd & 5th IMD into n79 |  | - | These MSD issues are already covered in 3DL_DC_1A-21A_n78A with 2UL_DC_1A_n78A. These MSD issues are already covered in 3DL_DC_1A_n78A-n79A with 2UL_DC_1A_n78A. |  |
|  | DC_1A_n79A |  | - |  | 4th IMD into B21 5th IMD into n78 |  | - | The MSD issues are already covered in 3DL_DC_1A-21A_n79A with 2UL_DC_1A_n79A and 3DL_DC_1A_n78A-n79A with 2UL_DC_1A_n79A |  |
| DC_1-42_n77-n78 | DC_1A_n77A |  | - |  | 4th & 5th IMD into B42 4th & 5th IMD into n78 |  | - | These MSD issues are already covered in 3DL_DC_1A-42A_n77A with 2UL_DC_1A_n77A. These MSD issues are already covered in 3DL_DC_1A_n77A-n78A with 2UL_DC_1A_n77A. |  |
|  | DC_1A_n78A |  | 2nd harmonic from B1 into n77 |  | 4th IMD into B42 4th & 5th IMD into n77 |  | - | These MSD issues are already covered in 3DL_DC_1A-42A_n78A with 2UL_DC_1A_n78A. These MSD issues are already covered in 3DL_DC_1A_n77A-n78A with 2UL_DC_1A_n78A. |  |
| DC_1-42_n77-n79 | DC_1A_n77A |  | - |  | 4th & 5th IMD into B42 3rd, 4th & 5th IMD into n79 |  | - | These MSD issues are already covered in 3DL_DC_1A-42A_n77A with 2UL_DC_1A_n77A. These MSD issues are already covered in 3DL_DC_1A_n77A-n79A with 2UL_DC_1A_n77A. |  |
|  | DC_1A_n79A |  | 2nd harmonic from B1 into n77 |  | 5th IMD into B42 5th IMD into n77 |  | - | 2nd harmonic issue already covered in DC_1A_n77A The MSD issue are already covered in 3DL_DC_1A-42A_n79A with 2UL_DC_1A_n79A and 3DL_DC_1A_n77A-n79A with 2UL_DC_1A_n79A |  |
| DC_1-42_n78-n79 | DC_1A_n78A |  | - |  | 4th IMD into B42 3rd & 5th IMD into n79 |  | - | These MSD issues are already covered in 3DL_DC_1A-42A_n78A with 2UL_DC_1A_n78A. These MSD issues are already covered in 3DL_DC_1A_n78A-n79A with 2UL_DC_1A_n78A. |  |
|  | DC_1A_n79A |  | - |  | 5th IMD into B42 5th IMD into n78 |  | - | The MSD issue are already covered in 3DL_DC_1A-42A_n79A with 2UL_DC_1A_n79A and 3DL_DC_1A_n78A-n79A with 2UL_DC_1A_n79A |  |
| DC_3-19_n77-n78 | DC_19A_n77A |  | 4th harmonic from B19 into n78 |  | 3rd IMD into B3 2nd IMD into n78 |  | - | 4th harmonic problem already covered in DC_19A_n78A These MSD issues are already covered in 3DL_DC_3A-19A_n77A with 2UL_DC_19A_n77A and 3DL_DC_19A_n77A-n78A with 2UL_DC_19A_n77A. |  |
|  | DC_19A_n78A |  | 4th harmonic from B19 into n77 |  | 3rd IMD into B3 2nd & 5th IMDs into n77 |  | - | 4th harmonic problem already covered in DC_19A_n77A These MSD issues are already covered in 3DL_DC_3A-19A_n78A with 2UL_DC_19A_n78A and 3DL_DC_19A_n77A-n78A with 2UL_DC_19A_n78A. |  |
| DC_3-19_n77-n79 | DC_19A_n77A |  | 6th harmonic from B19 into n79 |  | 3rd IMD into B3 2nd, 3rd, 4th & 5th IMD into n79 |  | - | 6th harmonic problem already covered in DC_19A_n78A These MSD issues are already covered in 3DL_DC_3A-19A_n77A with 2UL_DC_19A_n77A and 3DL_DC_19A_n77A-n78A with 2UL_DC_19A_n77A. |  |
|  | DC_19A_n79A |  | 4th harmonic from B19 into n77 |  | 4th IMD into B3 2nd & 3rd IMDs into n77 |  | - | 4th harmonic will be covered in DC_19A_n77A The MSD issue will be covered in 3DL_DC_3A-19A_n79A with 2UL_DC_19A_n79A and 3DL_19A_n77A-n79A with 2UL_DC_19A_n79A |  |
| DC_3-19_n78-n79 | DC_19A_n78A |  | 6th harmonic from B19 into n79 |  | 3rd IMD into B3 2nd, 3rd, 4th & 5th IMD into n79 |  | - | 6th harmonic problem already covered in DC_19A_n79A These MSD issues are already covered in 3DL_DC_3A-19A_n78A with 2UL_DC_19A_n78A and 3DL_DC_19A_n78A-n79A with 2UL_DC_19A_n78A. |  |
|  | DC_19A_n79A |  | 4th harmonic from B19 into n78 |  | 4th IMD into B3 2nd & 3rd IMDs into n78 |  | - | 4th harmonic will be covered in DC_19A_n78A The MSD issue will be covered in 3DL_DC_3A-19A_n79A with 2UL_DC_19A_n79A and 3DL_19A_n78A-n79A with 2UL_DC_19A_n79A |  |
| DC_3-21_n77-n78 | DC_3A_n77A |  | 2nd harmonic from B3 into n78 |  | 4th & 5th IMDs into B21 4th & 5th IMDs into n78 |  | - | 2nd harmonic issue already covered in DC_3A_n78A These MSD issues are already covered in 3DL_DC_3A-21A_n77A with 2UL_DC_3A_n77A and 3DL_3A_n77A-n78A with 2UL_DC_3A_n77A |  |
|  | DC_3A_n78A |  | 2nd harmonic from B3 into n77 |  | 4th & 5th IMDs into B21 4th & 5th IMDs into n77 |  | - | 2nd harmonic issue already covered in DC_3A_n77A These MSD issues are already covered in 3DL_DC_3A-21A_n78A with 2UL_DC_3A_n78A and 3DL_3A_n77A-n78A with 2UL_DC_3A_n78A |  |
|  | DC_21A_n77A |  | - |  | 2nd & 5th IMDs into B3 4th & 5th IMDs into n78 |  | - | These MSD issues are already covered in 3DL_DC_3A-21A_n77A with 2UL_DC_21A_n77A and 3DL_21A_n77A-n78A with 2UL_DC_21A_n77A |  |
|  | DC_21A_n78A |  | - |  | 2nd IMDs into B3 4th IMD into n77 |  | - | These MSD issue is already covered in 3DL_DC_3A-21A_n78A with 2UL_DC_21A_n78A and 3DL_21A_n77A-n78A with 2UL_DC_21A_n78A |  |
| DC_3-21_n77-n79 | DC_3A_n77A |  | - |  | 4th & 5th IMDs into B21 3rd & 4th IMD into n79 |  | - | These MSD issues are already covered in 3DL_DC_3A-21A_n77A with 2UL_DC_3A_n77A and 3DL_3A_n77A-n79A with 2UL_DC_3A_n77A |  |
|  | DC_3A_n79A |  | 2nd harmonic from B3 into n77 |  | 3rd IMD into B21 5th IMD into n77 |  | - | 2nd harmonic issue already covered in DC_3A_n77A The MSD issue will be covered in 3DL_DC_3A-21A_n79A with 2UL_DC_3A_n79A and 3DL_3A_n77A-n79A with 2UL_DC_3A_n79A |  |
|  | DC_21A_n77A |  |  |  | 2nd & 5th IMDs into B3 2nd & 4th IMDs into n79 |  | - | These MSD issues are already covered in 3DL_DC_3A-21A_n77A with 2UL_DC_21A_n77A and 3DL_21A_n77A-n79A with 2UL_DC_21A_n77A |  |
|  | DC_21A_n79A |  | - |  | 3rd IMD into B3 2nd IMD into n77 |  |  | These MSD issues are already covered in 3DL_DC_3A-21A_n79A with 2UL_DC_21A_n79A and 3DL_21A_n77A-n79A with 2UL_DC_21A_n79A |  |
| DC_3-21_n78-n79 | DC_3A_n78A |  | - |  | 4th & 5th IMDs into B21 3rd IMD into n79 |  | - | These MSD issues are already covered in 3DL_DC_3A-21A_n78A with 2UL_DC_3A_n78A and 3DL_3A_n78A-n79A with 2UL_DC_3A_n78A |  |
|  | DC_3A_n79A |  | 2nd harmonic from B3 into n78 |  | 3rd IMD into B21 5th IMD into n78 |  | - | 2nd harmonic issue already covered in DC_3A_n78A The MSD issue will be covered in 3DL_DC_3A-21A_n79A with 2UL_DC_3A_n79A and 3DL_3A_n78A-n79A with 2UL_DC_3A_n79A |  |
|  | DC_21A_n78A |  | - |  | 2nd IMDs into B3 2nd & 4th IMDs into n79 |  | - | These MSD issue is already covered in 3DL_DC_3A-21A_n78A with 2UL_DC_21A_n78A and 3DL_21A_n77A-n78A with 2UL_DC_21A_n78A |  |
|  | DC_21A_n79A |  | - |  | 3rd IMD into B3 2nd IMD into n78 |  | - | These MSD issues are already covered in 3DL_DC_3A-21A_n79A with 2UL_DC_21A_n79A and 3DL_21A_n78A-n79A with 2UL_DC_21A_n79A |  |
| DC_3-42_n77-n78 | DC_3A_n77A |  | 2nd harmonic from B3 into B42 & n78 |  | 4th & 5th IMDs into B42 4th & 5th IMDs into n78 |  | - | 2nd harmonic issue already covered in DC_3A-42A and DC_3A_n78A These MSD issues are already covered in 3DL_DC_3A-42A_n77A with 2UL_DC_3A_n77A and 3DL_3A_n77A-n78A with 2UL_DC_3A_n77A |  |
|  | DC_3A_n78A |  | 2nd harmonic from B3 into B42 & n77 |  | 4th & 5th IMDs into B42 4th & 5th IMDs into n77 |  | - | 2nd harmonic issue already covered in DC_3A-42A and DC_3A_n77A These MSD issues are already covered in 3DL_DC_3A-42A_n78A with 2UL_DC_3A_n78A and 3DL_3A_n77A-n78A with 2UL_DC_3A_n78A |  |
| DC_3-42_n77-n79 | DC_3A_n77A |  | 2nd harmonic from B3 into B42 |  | 4th & 5th IMDs into B42 3rd & 4th IMD into n79 |  | - | 2nd harmonic issue already covered in DC_3A-42A These MSD issues are already covered in 3DL_DC_3A-42A_n77A with 2UL_DC_3A_n77A and 3DL_3A_n77A-n79A with 2UL_DC_3A_n77A |  |
|  | DC_3A_n79A |  | 2nd harmonic from B3 into B42 & n77 |  | 5th IMD into B42 5th IMD into n77 |  | - | 2nd harmonic issue already covered in DC_3A-42A and DC_3A_n77A The MSD issue will be covered in 3DL_DC_3A-42A_n79A with 2UL_DC_3A_n79A and 3DL_3A_n77A-n79A with 2UL_DC_3A_n79A |  |
| DC_3-42_n78-n79 | DC_3A_n78A |  | 2nd harmonic from B3 into B42 |  | 4th & 5th IMDs into B42 3rd IMD into n79 |  | - | 2nd harmonic issue already covered in DC_3A-42A These MSD issues are already covered in 3DL_DC_3A-42A_n78A with 2UL_DC_3A_n78A and 3DL_3A_n78A-n79A with 2UL_DC_3A_n78A |  |
|  | DC_3A_n79A |  | 2nd harmonic from B3 into B42 & n78 |  | 5th IMD into B42 5th IMD into n78 |  | - | 2nd harmonic issue already covered in DC_3A-42A and DC_3A_n78A The MSD issue will be covered in 3DL_DC_3A-42A_n79A with 2UL_DC_3A_n79A and 3DL_3A_n78A-n79A with 2UL_DC_3A_n79A |  |
| DC_19-21_n77-n78 | DC_19A_n77A |  | 4th harmonic from B19 into n78 |  | 4th IMD into B21 2nd IMD into n78 |  | - | 4th harmonic problem already covered in DC_19A_n78A These MSD issues are already covered in 3DL_DC_19A-21A_n77A with 2UL_DC_19A_n77A and 3DL_DC_19A_n77A-n78A with 2UL_DC_19A_n77A. |  |
|  | DC_19A_n78A |  | 4th harmonic from B19 into n77 |  | 2nd & 5th IMDs into n77 |  | - | 4th harmonic problem already covered in DC_19A_n77A These MSD issues are already covered in 3DL_DC_19A_n77A-n78A with 2UL_DC_19A_n78A. |  |
| DC_19-21_n77-n79 | DC_19A_n77A |  | 6th harmonic from B19 into n79 |  | 4th IMD into B21 2nd, 3rd , 4th & 5th IMDs into n79 |  | - | 6th harmonic problem already covered in DC_19A_n79A These MSD issues are already covered in 3DL_DC_19A-21A_n77A with 2UL_DC_19A_n77A and 3DL_DC_19A_n77A-n79A with 2UL_DC_19A_n77A. |  |
|  | DC_19A_n79A |  | 4th harmonic from B19 into n77 |  | 5th IMD into B21 2nd & 3rd IMDs into n77 |  | - | 4th harmonic will be covered in DC_19A_n77A The MSD issue will be covered in 3DL_DC_19A-21A_n79A with 2UL_DC_19A_n79A and 3DL_19A_n77A-n79A with 2UL_DC_19A_n79A |  |
| DC_19-21_n78-n79 | DC_19A_n78A |  | 6th harmonic from B19 into n79 |  | 2nd, 3rd, 4th & 5th IMD into n79 |  | - | 6th harmonic problem already covered in DC_19A_n79A These MSD issues are already covered in 3DL_DC_19A_n78A-n79A with 2UL_DC_19A_n78A. |  |
|  | DC_19A_n79A |  | 4th harmonic from B19 into n78 |  | 5th IMD into B21 2nd & 3rd IMDs into n78 |  | - | 4th harmonic will be covered in DC_19A_n78A The MSD issue will be covered in 3DL_DC_19A-21A_n79A with 2UL_DC_19A_n79A and 3DL_19A_n78A-n79A with 2UL_DC_19A_n79A |  |
| DC_19-42_n77-n78 | DC_19A_n77A |  | 4th harmonic from B19 into n78 |  | 2nd IMD into n78 |  | - | 4th harmonic problem already covered in DC_19A_n78A These MSD issues are already covered in 3DL_DC_19A_n77A-n78A with 2UL_DC_19A_n77A. |  |
|  | DC_19A_n78A |  | 4th harmonic from B19 into n77 |  | 2nd & 5th IMDs into n77 |  | - | 4th harmonic problem already covered in DC_19A_n77A These MSD issues are already covered in 3DL_DC_19A_n77A-n78A with 2UL_DC_19A_n78A. |  |
| DC_19-42_n77-n79 | DC_19A_n77A |  | 6th harmonic from B19 into n79 |  | 2nd, 3rd , 4th & 5th IMDs into n79 |  | - | 6th harmonic problem already covered in DC_19A_n79A These MSD issues are already covered in 3DL_DC_19A_n77A-n79A with 2UL_DC_19A_n77A. |  |
|  | DC_19A_n79A |  | 4th harmonic from B19 into n77 |  | 2nd IMD into B42 2nd & 3rd IMDs into n77 |  | - | 4th harmonic will be covered in DC_19A_n77A The MSD issue will be covered in 3DL_DC_19A-42A_n79A with 2UL_DC_19A_n79A and 3DL_19A_n77A-n79A with 2UL_DC_19A_n79A |  |
| DC_19-42_n78-n79 | DC_19A_n78A |  | 6th harmonic from B19 into n79 |  | 2nd, 3rd, 4th & 5th IMD into n79 |  | - | 6th harmonic problem already covered in DC_19A_n79A These MSD issues are already covered in 3DL_DC_19A_n78A-n79A with 2UL_DC_19A_n78A. |  |
|  | DC_19A_n79A |  | 4th harmonic from B19 into n78 |  | 2nd IMD into B42 2nd & 3rd IMDs into n78 |  | - | 4th harmonic will be covered in DC_19A_n78A The MSD issue will be covered in 3DL_DC_19A-42A_n79A with 2UL_DC_19A_n79A and 3DL_19A_n78A-n79A with 2UL_DC_19A_n79A |  |
| DC_21-42_n77-n78 | DC_21A_n77A |  | - |  | 5th IMD into B42 4th & 5th IMDs into n78 |  | - | These MSD issues are already covered in 3DL_DC_21A-42A_n77A with 2UL_DC_21A_n77A and 3DL_21A_n77A-n78A with 2UL_DC_21A_n77A |  |
|  | DC_21A_n78A |  | - |  | 4th IMD into n77 |  | - | These MSD issue is already covered in 3DL_21A_n77A-n78A with 2UL_DC_21A_n78A |  |
| DC_21-42_n77-n79 | DC_21A_n77A |  | - |  | 5th IMD into B42 2nd & 4th IMDs into n79 |  | - | These MSD issues are already covered in 3DL_DC_21A-42A_n77A with 2UL_DC_21A_n77A and 3DL_21A_n77A-n79A with 2UL_DC_21A_n77A |  |
|  | DC_21A_n79A |  | - |  | 2nd IMD into B42 2nd IMD into n77 |  |  | These MSD issues are already covered in 3DL_DC_21A-42A_n79A with 2UL_DC_21A_n79A and 3DL_21A_n77A-n79A with 2UL_DC_21A_n79A |  |
| DC_21-42_n78-n79 | DC_21A_n78A |  | - |  | 2nd & 4th IMDs into n79 |  | - | These MSD issue is already covered in 3DL_21A_n78A-n79A with 2UL_DC_21A_n78A |  |
|  | DC_21A_n79A |  | - |  | 2nd IMD into B42 2nd IMD into n78 |  | - | These MSD issues are already covered in 3DL_DC_21A-42A_n79A with 2UL_DC_21A_n79A and 3DL_21A_n78A-n79A with 2UL_DC_21A_n79A |  |
| DC_1-28_n28-n78 | DC_1A_n28A |  | 5th harmonic from n28 into n78 |  | 3rd IMD into n78 |  | - | - Harmonic problems already solved in CA_n28A-n78A - 3rd IMD problem was solved in 3DL_DC_1A_n28A-n78A |  |
|  | DC_1A_n78A |  | - |  | 5th IMD into B28 & n28 |  | - | - 5th IMD problem was solved in 3DL_DC_1A_n28A-n78A or 3DL_DC_1A-28A_n78A |  |
|  | DC_28A_n28A |  | 3rd harmonic into B1 |  | 3rd & 5th IMDs into B1 5th IMD into n78 |  | - | 4th harmonic will be covered in DC_28A_n1A The MSD issue will be covered in 3DL_DC_1A-28A_n28A with 2UL_DC_28A_n28A and 3DL_28A_n28A-n78A with 2UL_DC_28A_n28A |  |
|  | DC_28A_n78A |  | 3rd harmonic from B28 into B1 |  | 3rd IMD into B1 5th IMD into n28 |  | - | 3rd harmonic will be covered in CA_1A-28A The MSD issue will be covered in 3DL_DC_1A-28A_n78A with 2UL_DC_28A_n78A and 3DL_28A_n28A-n78A with 2UL_DC_28A_n78A |  |
| DC_3-7_n1-n78 | DC_3A_n1A |  | 2nd harmonic from 3 into n78 |  | 2nd & 4th IMDs into n78 |  | Yes | 2nd harmonic will be covered in DC_3A_n78A The MSD issue will be covered in 3DL_3A_n1A-n78A with 2UL_DC_3A_n1A |  |
|  | DC_3A_n78A |  | - |  | 5th IMD into n1 |  | - | These MSD issues are already covered in 3DL_DC_3A_n1A-n78A with 2UL_DC_3A_n78A |  |
|  | DC_7A_n1A, DC_7C_n1A |  | - |  | 4th & 5th IMDs into n78 |  | Yes | These IMD problems were already covered in DC_7A_n1A-n78A with 2UL_DC_7A_n1A |  |
|  | DC_7A_n78A, DC_7C_n78A |  | - |  | 3rd & 4th IMDs into B3 4th IMD into n1 |  | - | These IMD problems were already covered in 3DL_DC_3A-7A_n78A with 2UL_DC_7A_n78A and 3DL_DC_7A_n1A-n78A with 2UL_DC_7A_n78A |  |
| DC_3-28_n28-n78 | DC_3A_n28A DC_3C_n28A |  | 2nd harmonic into n78 5th harmonic into n78 |  | 5th IMD into n78 |  | - | - Harmonic problems already solved in DC_3A_n78A or CA_n28A-n78A - 5th IMD problem was solved in 3DL_DC_3A_n28A-n78A with 2UL_DC_3A_n28A |  |
|  | DC_3A_n78A DC_3C_n78A |  | - |  | - |  | - | No issue |  |
|  | DC_28A_n28A |  | - |  | 5th IMD into n78 |  | - | The MSD issue will be covered in 3DL_28A_n28A-n78A with 2UL_DC_28A_n28A |  |
|  | DC_28A_n78A |  | - |  | 3rd IMD into B3 5th IMD into n28 |  | - | The MSD issue will be covered in 3DL_DC_3A-28A_n78A with 2UL_DC_28A_n78A and 3DL_28A_n28A-n78A with 2UL_DC_28A_n78A |  |
| DC_7-28_n28-n78 | DC_7A_n28A DC_7C_n28A |  | 5th harmonic into n78 |  | 2nd,4th IMDs into n78 |  | - | - Harmonic problem was solved in DC_28A_n78A - 2nd & 4th IMDs problems were solved in 3DL_DC_7A_n28A-n78A with 2UL_DC_7A_n28A |  |
|  | DC_7A_n78A DC_7C_n78A |  | - |  | 2nd & 5th IMDs into B28 & n28 |  | - | - 2nd & 5th IMDs problems were solved in 3DL_DC_7A_n28A-n78A or 3DL_DC_7A-28A_n78A |  |
|  | DC_28A_n28A |  | - |  | 5th IMD into n78 |  | - | The MSD issue will be covered in 3DL_28A_n28A-n78A with 2UL_DC_28A_n28A |  |
|  | DC_28A_n78A |  | - |  | 2nd IMD into B7 5th IMD into n28 |  | - | The MSD issue will be covered in 3DL_DC_7A-28A_n78A with 2UL_DC_28A_n78A and 3DL_28A_n28A-n78A with 2UL_DC_28A_n78A |  |
| DC_1-8_n77-n257 | DC_1A_n77A |  | 7th & 8th Harmonics from n77 into n257 |  | 5th IMD into B8 |  | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 5th IMD already covered in DC_1A-8A_n77A |  |
|  | DC_1A_n257A |  | 2nd harmonic from B1 into n77 |  | - |  | - | - Harmonic problem already discussed and will be solved in DC_1A_n77A |  |
|  | DC_8A_n77A |  | 7th & 8th Harmonics from n77 into n257 |  | 3rd IMD into B1 |  | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 The MSD issue will be covered in 3DL_DC_1A-8A_n77A with 2UL_DC_8A_n77A |  |
|  | DC_8A_n257A |  | 4th harmonic from B8 into n77 |  | - |  | - | - Harmonic problem will be solved in DC_8A_n77A |  |
| DC_1-3_n5-n78 | DC_1A_n5A | 4th harmonic from n5 into n78 |  | 3rd IMD into n78 |  | Yes |  |  | - 4th harmonic already covered in Table 7.3B.2.3.1-1 in TS38.101-3 - The 3rd IMD issue will be covered in DC_1_n5A-n78A |
|  | DC_3A_n5A, DC_3C_n5A | 2nd harmonic from B3 into n78 |  | 3rd & 5th IMDs into n78 |  | - |  |  | - 2nd harmonic problem already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - These IMD issue will be covered in DC_3_n5A-n78A |
|  | DC_1A_n78A | - |  | 2nd IMD into B3 5th IMD into n5 |  | Yes |  |  | - The 2nd IMD issue will be covered in DC_1A-3A_n78A - The 5th IMD issue will be covered in DC_1A_n5A-n78A |
|  | DC_3A_n78A, DC_3C_n78A | - |  | 5th IMD into B1 |  | - |  |  | - The 5th IMD issue will be covered in DC_1A-3A_n78A |
| DC_1-7_n5-n78 | DC_1A_n5A | 4th harmonic from n5 into n78 |  | 3rd IMD into n78 |  | - |  |  | - 4th harmonic already covered in Table 7.3B.2.3.1-1 in TS38.101-3 - The 3rd IMD issue will be covered in DC_1_n5A-n78A |
|  | DC_7A_n5A, DC_7C_n5A | - |  | 2nd & 4th IMDs into n78 |  | - |  |  | These IMDs problem will be covered in DC_7A_n5A-n78A |
|  | DC_1A_n78A | - |  | 5th IMD into n5 4th IMD into B7 |  | Yes |  |  | The 5th IMD will be covered in DC_1A_n5A-n78A The 4th IMD will be covered in DC_1A-7A_n78A |
|  | DC_7A_n78A, DC_7C_n78A | - |  | 4th IMD into B1 2nd & 5th IMDs into n5 |  | Yes |  |  | The 4th IMD will be covered in DC_1A-7A_n78A The 2nd & 5th IMDs will be covered in DC_7A_n5A-n78A |
| DC_1-28_n5-n78 | DC_1A_n5A | 4th harmonic from n5 into n78 |  | 3rd IMD into n78 |  | - |  |  | - 4th harmonic already covered in Table 7.3B.2.3.1-1 in TS38.101-3 - The 3rd IMD issue will be covered in DC_1_n5A-n78A |
|  | DC_28A_n5A | 3rd harmonic from B28 into B1 4th harmonic from n5 into n78 5th harmonic from B28 into n78 |  | 5th IMD into B1 |  |  |  |  | - The 3rd harmonic already covered in CA_1A-28A. - The other harmonic problems already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - The 5th IMD problem will be covered in DC_1A-28A_n5A |
|  | DC_1A_n78A | - |  | 5th IMD into n5 5th IMD into B28 |  | - |  |  | - The 5th IMD problem will be covered in DC_1A_n5A-n78A - The 5th IMD problem will be covered in DC_1A-28A_n78A |
|  | DC_28A_n78A | 3rd harmonic from B28 into B1 |  | 3rd IMD into B1 5th IMD into n5 |  | - |  |  | 3rd harmonic will be covered in CA_1A-28A The 3rd IMD issue will be covered in DC_1A-28A_n78A The 5th IMD issue will be covered in DC_28A_n5A-n78A |
| DC_3-7_n5-n78 | DC_3A_n5A, DC_3C_n5A | 2nd harmonic from B3 into n78 |  | 2nd & 3rd IMDs into B7 3rd & 5th IMDs into n78 |  | - |  |  | - 2nd harmonic problem already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - These IMD issue will be covered in DC_3A-7A_n5A - These IMD issue will be covered in DC_3A_n5A-n78A |
|  | DC_7A_n5A, DC_7C_n5A | - |  | 2nd & 4th IMDs into n78 |  | - |  |  | These IMDs problem will be covered in DC_7A_n5A-n78A |
|  | DC_3A_n78A, DC_3C_n78A | - |  | - |  | - |  |  | No issue |
|  | DC_7A_n78A, DC_7C_n78A | - |  | 2nd & 5th IMDs into n5 3rd & 4th IMDs into B3 |  | Yes |  |  | The 2nd & 5th IMDs will be covered in DC_7A_n5A-n78A The 3rd & 4th IMDs will be covered in DC_3A-7A_n78A |
| DC_3-28_n5-n78 | DC_3A_n5A, DC_3C_n5A | 2nd harmonic from B3 into n78 |  | 3rd & 5th IMDs into n78 4th IMD into B28 |  | - |  |  | - 2nd harmonic problem already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - These IMD issue will be covered in DC_3A_n5A-n78A The 4th IMD issue will be covered in DC_3A-28A_n5A |
|  | DC_28A_n5A | 4th harmonic from n5 into n78 5th harmonic from B28 into n78 |  | 4th IMD into B3 |  |  |  |  | - These harmonic problems already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - The 4th IMD will be covered in DC_3A-28A_n5A |
|  | DC_3A_n78A, DC_3C_n78A | - |  | - |  | - |  |  | No issue |
|  | DC_28A_n78A | - |  | 3rd IMD into B3 5th IMD into n5 |  | - |  |  | - The 3rd IMD will be covered in DC_3A-28A_n78A - The 5th IMD will be covered in DC_28A_n5A-n78A |
| DC_7-28_n5-n78 | DC_7A_n5A, DC_7C_n5A | - |  | 2nd & 4th IMDs into n78 3rd & 5th IMDs into B28 |  | - |  |  | - These IMDs problem will be covered in DC_7A_n5A-n78A – These IMDs problems will be covered in DC_7A-28A_n5A |
|  | DC_28A_n5A | 4th harmonic from n5 into n78 5th harmonic from B28 into n78 |  | 5th IMD into B7 |  | - |  |  | - These harmonic problems already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - The 5th IMD issue will be covered in 7A-28A_n5A |
|  | DC_7A_n78A, DC_7C_n78A | - |  | 2nd & 5th IMDs into n5 2nd & 5th IMDs into B28 |  | Yes |  |  | 2nd & 5th IMDs will be covered in DC_7A_n5A-n78A Other 2nd & 5th IMDs will be covered in DC_7A-28A_n78A |
|  | DC_28A_n78A | - |  | 5th IMD into n5 2nd IMD into B7 |  | Yes |  |  | - The 5th IMD will be covered in DC_28A_n5A-n78A - 2nd IMD will be covered in DC_7A-28A_n78A |
| DC_1-7_n3-n78 | DC_1A_n3A | 2nd harmonic from 3 into n78 |  | 2nd & 4th IMDs into n78 |  | Yes |  |  | - Harmonic problems was already covered in DC_3A_n78A. - 2nd &4th IMDs problems same as DC_1A_n3A-n78A. |
|  | DC_1A_n78A | - |  | 4th IMD into B7 2nd IMD into n3 |  | Yes |  |  | The 4th IMD will be covered in DC_1A-7A_n78A The 2nd IMD will be covered in DC_1A_n3A-n78A |
|  | DC_7A_n3A | 2nd harmonic from n3 into n78 |  | 3rd IMD into n78 |  | - |  |  | - Harmonic problem already solved in CA_n3A-n78A - The IMD problem already covered in DC_7A_n3A-n78A |
|  | DC_7A_n78A |  |  | 3rd & 4th IMDs into n3 4th IMD into B1 |  | Yes |  |  | - The 3rd & 4th IMDs will be covered in DC_3A-7A_n78A - The 4th IMD will be covered in DC_1A-7A_n78A |
| DC_2-7_n38-n66 | DC_2A_n66A | - |  | - |  | - |  |  | No issue |
| DC_2-7_n38-n78 | DC_2A_n78A | - |  | - |  | - |  |  | No issue |
| DC_2-66_n41-n260 | DC_2A_n41A | High order harmonic into n260 |  | - |  | - |  |  | - No harmonic problem |
|  | DC_66A_n41A | High order harmonic into n260 |  | 4th IMD into B2 |  | - |  |  | - No harmonic problem - The 4th IMD issue will be covered in DC_2A-66A_n41A |
| DC_2-66_n41-n261 | DC_2A_n41A | High order harmonic into n261 |  | - |  | - |  |  | - No harmonic problem |
|  | DC_66A_n41A | High order harmonic into n261 |  | 4th IMD into B2 |  | - |  |  | - No harmonic problem - The 4th IMD issue will be covered in DC_2A-66A_n41A |
| DC_7-66_n38-n78 | DC_66A_n78A | - |  | - |  | - |  |  | No issue |
| DC_1-3_n77-n257 | DC_1A_n77A | 7th & 8th Harmonics from n77 into n257 |  | 2nd, 4th & 5th IMDs into B3 |  | Yes |  |  | - No harmonic problems by 7th & 8th order between FR1 and FR2 - These IMDs issue will be covered in DC_1A-3A_n77A |
|  | DC_3A_n77A | 7th & 8th harmonics from n77 into n257 |  | 2nd & 5th IMD into B1 |  | - |  |  | - No harmonic problems - These IMD problem will be covered in DC_1A-3A_n77A_ |
|  | DC_1A_n257A/D/G/H/I | 2nd harmonic from B1 into n77 |  | - |  | Yes |  |  | - Harmonic problem already discussed and will be solved in DC_1A_n77A |
|  | DC_3A_n257A//D/G/H/I | 2nd harmonic from B3 into B77 |  | - |  | - |  |  | - Harmonic problem already covered in DC_3A_n77A |
| DC_3-20_n1-n7 | DC_3A_n1A, DC_3C_n1A | - |  | - |  | - |  |  | No issue |
|  | DC_3A_n7A, DC_3C_n7A | - |  | 2nd IMD into B20 |  | - |  |  | - 2nd IMD issue will be covered in DC_3A-20A_n7A |
|  | DC_20A_n1A | - |  | - |  | Yes |  |  | No issue |
|  | DC_20A_n7A | - |  | - |  | - |  |  | No issue |
| DC_3-20_n1-n28 | DC_3A_n1A, DC_3C_n1A | - |  | - |  | - |  |  | No issue |
|  | DC_3A_n28A, DC_3C_n28A | 3rd harmonic from n28 into n1 |  | 4th IMD into n1 |  |  |  |  | - Harmonic problems already solved in DC_1A_n28A - 4th IMD problem was solved in 3DL_DC_1A-3A_n28A |
|  | DC_20A_n1A | - |  | - |  | Yes |  |  | No issue |
|  | DC_20A_n28A | 3rd harmonic from n28 into n1 |  | 5th IMD into n1 |  | - |  |  | - Harmonic problem already covered in DC_1A_n28A - 5th IMD issue also covered in DC_20A_n1A-n28A |
| DC_3-20_n7-n28 | DC_3A_n7A, DC_3C_n7A | - |  | 2nd IMD into B20 2nd IMD into n28 |  | - |  |  | - 2nd IMD issue will be covered in DC_3A-20A_n7A - 2nd IMD issue will be covered in DC_3A_n7A-n28A |
|  | DC_3A_n28A, DC_3C_n28A | - |  | 3rd IMD into n7 |  | - |  |  | - 3rd IMD problem was solved in 3DL_DC_3A_n7A-n28A |
|  | DC_20A_n7A | - |  | 3rd & 5th IMDs into n28 |  | - |  |  | - These IMDs problem were solved in DC_20A_n7A-n28A |
|  | DC_20A_n28A | - |  | 5th IMD into n7 |  | - |  |  | - 5th IMD problem same as DC_7A-20A_n28A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
| DC_3-20_n7-n78 | DC_3A_n7A | 2nd harmonic from 3 into n78 |  | 2nd IMD into B20 3rd IMD into n78 |  | - |  |  | - Harmonic problem was solved in DC_3A_n78A - 2nd IMD issue will be covered in DC_3A-20A_n7A - 3rd IMD problem already covered in DC_3A_n7A-n78A. |
|  | DC_3A_n78A | - |  | - |  | Yes |  |  | No issue |
|  | DC_20A_n7A | 4th harmonic from B20 into n78 |  | 2nd & 4th IMDs into n78 |  | - |  |  | - The harmonic problem already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - These IMD issue will be covered in DC_20A_n7A-n78A |
|  | DC_20A_n78A |  |  | 2nd IMD into n7 3rd IMD into B3 |  | Yes |  |  | - 2nd IMD problem same as DC_7A-20A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 - 3rd IMD issue will be covered in DC_3A-20A_n78A |
| DC_3-20_n1-n78 | DC_3A_n1A | 2nd harmonic from 3 into n78 |  | 2nd & 4th IMDs into n78 |  | - |  |  | - 2nd harmonic will be covered in DC_3A_n78A - The MSD issue will be covered in 3DL_3A_n1A-n78A |
|  | DC_3A_n78A | - |  | 5th IMD into n1 |  | - |  |  | - The 5th IMD issue will be covered in DC_3A_n1A-n78A |
|  | DC_20A_n1A | 4th harmonic from B20 into n78 |  | 3rd IMD into n78 |  | - |  |  | - Harmonic problem already solved in DC_20A_n78A - 3rd IMD will be covered in DC_20A_n1A-n78A |
|  | DC_20A_n78A | - |  | 3rd IMD into B3 3rd IMD into n1 |  | Yes |  |  | - 3rd IMD issue will be covered in DC_3A-20A_n78A - 3rd IMD issue will be covered in DC_20A_n1A-n78A |
| DC_3-20_n28-n78 | DC_3A_n28A | 2nd harmonic into n78 5th harmonic into n78 |  | 5th IMD into n78 |  | - |  |  | - Harmonic problems already solved in DC_3A_n78A or CA_n28A-n78A - 5th IMD problem was solved in 3DL_DC_3A_n28A-n78A with 2UL_DC_3A_n28A |
|  | DC_3A_n78A | - |  | - |  | - |  |  | No issue |
|  | DC_20A_n28A | 4th harmonic from B20 into n78 5th harmonic from n28 into n78 |  | 4th & 5th IMDs into n78 |  |  |  |  | - Harmonic problems already solved in DC_20A_n78A or CA_n28A-n78A - These IMD issue will be covered in DC_20A_n28A-n78A |
|  | DC_20A_n78A | - |  | 3rd IMD into B3 4th IMD into n28 |  | - |  |  | - 3rd IMD issue will be covered in DC_3A-20A_n78A - 4th IMD issue will be covered in DC_20A_n28A-n78A |
| DC_2-66_n41-n71 | DC_2A_n41A | - |  | 2nd & 5th IMDs into n71 |  | - |  |  | - These IMD issue will be covered in DC_2A_n41A-n71A |
|  | DC_2A_n71A | 4th harmonic from n71 into n41 |  | 2nd IMD into n41 |  | - |  |  | - The harmonic problem will be treated CA_n41A_n71A - 2nd IMD issue will be covered in DC_2A_n41A-n71A |
|  | DC_66A_n41A |  |  | 4th IMD into B2 |  | - |  |  | - 4th IMD issue will be covered in DC_2A-66A_n41A |
|  | DC_66A_n71A | 4th harmonic from n71 into n41 |  | - |  | - |  |  | The harmonic problem will be treated CA_n41A_n71A |
| DC_1-18_n3-n78 | DC_1A_n3A | 2nd harmonic from 3 into n78 |  | 2nd & 4th IMDs into n78 |  | Yes |  |  | - Harmonic problems was already covered in DC_3A_n78A. - 2nd &4th IMDs problems will be covered in DC_1A_n3A-n78A. |
|  | DC_1A_n78A |  |  | 2nd IMD into n3 5th IMD into B18 |  | Yes |  |  | - The 2nd IMD will be covered in DC_1A_n3A-n78A - The 5th IMD will be covered in DC_1A-18A_n78A |
|  | DC_18A_n3A | 2nd harmonic from n3 into n78 4th harmonic from B18 into n78 |  | 3rd & 5th IMDs into n78 |  | - |  |  | - These harmonic problem already specified in in Table 7.3B.2.3.1-1 in TS38.101-3 - These IMD issues will be covered in DC_18A_n3A-n78A |
|  | DC_18A_n78A | - |  | 3rd IMD into n3 3rd IMD into B1 |  | - |  |  | - 3rd IMD issue will be covered in DC_18A_n3A-n78A - 3rd IMD issue will be covered in DC_1A-18A_n78A |
| DC_1-28_n3-n78 | DC_1A_n3A | 2nd harmonic from 3 into n78 |  | 2nd & 4th IMDs into n78 |  | Yes |  |  | - Harmonic problems was already covered in DC_3A_n78A. - 2nd &4th IMDs problems will be covered in DC_1A_n3A-n78A. |
|  | DC_1A_n78A | - |  | 2nd IMD into n3 5th IMD into B28 |  | Yes |  |  | - The 2nd IMD will be covered in DC_1A_n3A-n78A - The 5th IMD will be covered in DC_1A-28A_n78A |
|  | DC_28A_n3A | 2nd harmonic from n3 into n78 5th harmonic from B28 into n78 3rd harmonic from B28 into B1 |  | 4th IMD into B1 5th IMD into n78 |  | - |  |  | - These harmonic issue already covered in CA_n3A-n78A or DC_28A_n78A. or CA_1A-28A - 4th IMD problem will be covered in DC_1A-28A_n3A - 5th IMD problem same as DC_3A_n28A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
|  | DC_28A_n78A | 3rd harmonic from B28 into B1 |  | 3rd IMD into B1 3rd IMD into n3 |  | - |  |  | - 3rd harmonic will be covered in CA_1A-28A - 3rd IMD issue will be covered in DC_1A-28A_n78A - 3rd IMD will be covered in DC_28A_n3A-n78A |
| DC_1-8_n3-n28 | DC_1A_n3A | - |  | - |  | - |  |  | No issue |
|  | DC_1A_n28A | - |  | 5th IMD into n3 5th IMD into B8 |  | Yes |  |  | - 5th IMD problem same as DC_1A-3A_n28A in Table 7.3B.2.3.5.2-1 in TS38.101-3 - 5th IMD issue will be covered in DC_1A-8A_n28A |
|  | DC_8A_n3A | - |  | 2nd & 5th IMDs into n28 |  | - |  |  | - These IMD issues will be covered in DC_8A_n3A-n28A |
|  | DC_8A_n28A | 2nd harmonic from B8 into n3 3rd harmonic from n28 into B1 |  | 4th IMD into B1 |  | - |  |  | These harmonic problem will be covered in DC_8A_n3A or DC_1A_n28A 4th IMD issue will be covered in DC_1A-8A_n28A |
| DC_3-8_n1-n78 | DC_3A_n1A | 2nd harmonic from 3 into n78 |  | 2nd & 4th IMDs into n78 |  | - |  |  | - 2nd harmonic will be covered in DC_3A_n78A The MSD issue will be covered in 3DL_3A_n1A-n78A |
|  | DC_3A_n78A | - |  | 5th IMD into n1 |  | - |  |  | - The 5th IMD issue will be covered in DC_3A_n1A-n78A |
|  | DC_8A_n1A | 2nd harmonic from B8 into B3 4th harmonic from B8 into n78 |  | 3rd IMD into n78 |  | - |  |  | - The harmonic problem already covered in CA_3A-8A or DC_8A_n78A - 3rd IMD issue will be covered in DC_8A_n1A-n78A |
|  | DC_8A_n78A | 2nd harmonic from B8 into B3 |  | 3rd IMD into B3 |  | - |  |  | - Harmonic problem already be solved in CA_3A_8A - 3rd IMD already covered in DC_3A-8A_n78A |
| DC_7-8_n1-n78 | DC_7A_n1A | - |  | 4th & 5th IMDs into n78 5th IMD into B8 |  | Yes |  |  | - These IMD problems were already covered in DC_7A_n1A-n78A with 2UL_DC_7A_n1A - 5th IMD issue will be covered in DC_7A-8A_n1A |
|  | DC_7A_n78A |  |  | 2nd & 5th IMDs into B8 4th IMD into n1 |  | Yes |  |  | - 2nd & 5th IMDs will be covered in DC_7A-8A_n78A - 4th IMD will be covered in DC_7A_n1A-n78A |
|  | DC_8A_n1A | 3rd harmonic from B8 into B7 4th harmonic from B8 into n78 |  | 3rd IMD into n78 |  | - |  |  | - The harmonic problem already covered in CA_7A-8A or DC_8A_n78A - 3rd IMD issue will be covered in DC_8A_n1A-n78A |
|  | DC_8A_n78A | 3rd harmonic from B8 into B7 |  | 2nd IMD into B7 |  | Yes |  |  | - The harmonic problem already covered in CA_7A-8A - 2nd IMD issue will be covered in DC_7A-8A_n78A |
| DC_1-3_n38-n78 | DC_3A_n38A | 2nd harmonic from B3 into n78 |  | 3rd IMD into n78 |  | - |  |  | - The harmonic problem already covered in DC_3A_n78A - 3rd IMD issue will be covered in DC_3A_n38A-n78A |
|  | DC_3A_n78A | - |  | 5th IMD into B1 |  | - |  |  | - The 5th IMD issue will be covered in DC_1A-3A_n78A |
| DC_3-20_n38-n78 | DC_3A_n38A | 2nd harmonic from B3 into n78 |  | 2nd & 3rd IMDs into B20 3rd IMD into n78 |  | - |  |  | - The harmonic problem already covered in DC_3A_n78A - 2nd & 3rd IMDs issue will be covered in DC_3A-20A_38A - 3rd IMD issue will be covered in DC_3A_n38A-n78A |
|  | DC_3A_n78A | - |  | - |  | - |  |  | No issue |
| DC_1-20_n3-n78 | DC_1A_n3A | 2nd harmonic from 3 into n78 |  | 2nd & 4th IMDs into n78 |  | Yes |  |  | - Harmonic problems was already covered in DC_3A_n78A. - 2nd &4th IMDs problems will be covered in DC_1A_n3A-n78A. |
|  | DC_20A_n3A | - 2nd harmonic from n3 into n78 - 4th harmonic from B20 into n78 |  | 3rd & 5th IMDs into n78 |  | - |  |  | - Harmonic problems was already covered in DC_3A_n78A or DC_20A_n78A. - 3rd &5th IMDs problems will be covered in DC_20A_n3A-n78A. |
| DC_1-20_n3-n38 | DC_1A_n3A | - |  | - |  | - |  |  | No issue |
|  | DC_20A_n3A | 3rd harmonic from B20 into n38 |  | 2nd & 3rd IMDs into n38 |  | - |  |  | - Harmonic problems was already covered in DC_20A_n38A. - These IMDs problems will be covered in DC_20A_n3A-n38A. |
| DC_7-20_n3-n78 | DC_7A_n3A | 2nd harmonic from n3 into n78 |  | 2nd IMD into B20 3rd IMD into n78 |  | - |  |  | - Harmonic problem already solved in CA_n3A-n78A - 2nd IMD problem will be covered in DC_7A-20A_n3A - 3rd IMD problem already covered in DC_7A_n3A-n78A |
|  | DC_20A_n3A | - 2nd harmonic from n3 into n78 - 4th harmonic from B20 into n78 |  | 2nd & 3rd IMDs into nB7 3rd & 5th IMDs into n78 |  | - |  |  | - Harmonic problems was already covered in DC_3A_n78A or DC_20A_n78A. - 2nd & 3rd IMDs problems will be covered in DC_7A-20A_n3A. - 3rd &5th IMDs problems will be covered in DC_20A_n3A-n78A. |
| DC_2-66_n66-n78 | DC_2A_n66A | 2nd harmonic from B2 into n78 2nd harmonic from n66 inton78 |  | 2nd & 4th IMDs into n78 5th IMD into B66 |  | - |  |  | - These harmonic problems already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - 2nd & 4th IMDs will be covered in DC_2A_n66A-n78A - 5th IMD will be covered in DC_2A-66A_n66A |
|  | DC_2A_n78A | - |  | 4th IMD into B66/n66 |  | - |  |  | 4th IMD problem same as DC_2A-66A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
|  | DC_66A_n66A | 2nd harmonic from B66 into n78 |  | 2nd & 4th IMDs into n78 |  |  |  |  | - Harmonic problem already covered in DC_66A_n78A - 2nd & 4th IMDs issues will be covered in DC_66A_n66A-n78A |
|  | DC_66A_n78A | - |  | 2nd , 4th & 5th IMDs into B2 5th IMD into n66 |  |  |  |  | - 2nd , 4th & 5th IMDs issue will be covered in DC_2A-66A_n78A - 5th IMD issue will be covered in DC_66A_n66A-n78A |
| DC_7-66_n66-n78 | DC_7A_n66A | 2nd harmonic from n66 into n78 |  | 3rd IMD into n78 |  | - |  |  | The harmonic problem already covered in Table 7.3B.2.3.1-1 in TS38.101-3 3rd IMD issue will be covered in DC_7A_n66A-n78A |
|  | DC_7A_n78A | - |  | 4th IMD into B66/n66 |  | Yes |  |  | - The 4th IMD problem same as DC_7A-66A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
|  | DC_66A_n66A | 2nd harmonic from B66 into n78 |  | 2nd & 4th IMDs into n78 |  |  |  |  | - Harmonic problem already covered in DC_66A_n78A - 2nd & 4th IMDs issues will be covered in DC_66A_n66A-n78A |
|  | DC_66A_n78A |  |  | 5th IMD into n66 |  |  |  |  | - 5th IMD issue will be covered in DC_66A_n66A-n78A |
| DC_2-7_n66-n78 DC_2-7-7_n66-n78 | DC_2A_n66A | 2nd harmonic from B2 into n78 2nd harmonic from n66 inton78 |  | 2nd & 4th IMDs into n78 |  | - |  |  | - These harmonic problems already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - 2nd & 4th IMDs will be covered in DC_2A_n66A-n78A |
|  | DC_2A_n78A | - |  | 4th IMD into n66 |  | - |  |  | 4th IMD problem same as DC_2A-66A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
|  | DC_7A_n66A | 2nd harmonic from n66 into n78 |  | 3rd IMD into n78 |  | - |  |  | The harmonic problem already covered in Table 7.3B.2.3.1-1 in TS38.101-3 3rd IMD issue will be covered in DC_7A_n66A-n78A |
|  | DC_7A_n78A | - |  | 4th IMD into n66 4th IMD into B2 |  | Yes |  |  | The 4th IMD problem same as DC_7A-66A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 4th IMD issue will be covered in DC_2A-7A_n78A |
| DC_7-7-66-n66-n78 | DC_7A_n66A | 2nd harmonic from n66 into n78 |  | 3rd IMD into n78 4th IMD into B7 |  | - |  |  | The harmonic problem already covered in Table 7.3B.2.3.1-1 in TS38.101-3 3rd IMD issue will be covered in DC_7A_n66A-n78A 4th IMD issue will be covered in DC_7A-7A_n66A |
|  | DC_7A_n78A | - |  | 4th IMD into B66/n66 |  | Yes |  |  | The 4th IMD problem same as DC_7A-66A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 4th IMD issue will be covered in DC_2A-7A_n78A |
|  | DC_66A_n66A | 2nd harmonic from B66 into n78 |  | 2nd & 4th IMDs into n78 |  |  |  |  | - Harmonic problem already covered in DC_66A_n78A - 2nd & 4th IMDs issues will be covered in DC_66A_n66A-n78A |
|  | DC_66A_n78A |  |  | 5th IMD into n66 |  |  |  |  | - 5th IMD issue will be covered in DC_66A_n66A-n78A |
| DC_1-18_n3-n257 | DC_1A_n3A | High order harmonic into n257 |  | - |  | -- |  |  | No harmonic impact to n257 |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | - |  | - |  | - |  |  | No issue |
|  | DC_18A_n3A | High order harmonics into n257 |  | - |  | - |  |  | No harmonic problem to n257 |
|  | DC_18A_n257A, DC_18A_n257G, DC_18A_n257H, DC_18A_n257I | - |  | - |  | - |  |  | No issue |
| DC_1-28_n3-n257 | DC_1A_n3A | High order harmonic into n257 |  | - |  | -- |  |  | No harmonic impact to n257 |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | - |  | - |  | - |  |  | No issue |
|  | DC_28A_n3A | 3rd harmonic from B28 into B1 |  | 4th IMD into B1 |  | - |  |  | - These harmonic issue already covered in CA_n3A-n78A or DC_28A_n78A. or CA_1A-28A - 4th IMD problem will be covered in DC_1A-28A_n3A |
|  | DC_28A_n257A, DC_28A_n257G, DC_28A_n257H, DC_28A_n257I | - |  | - |  | - |  |  | No issue |
| DC_1-41_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 |  | 4th IMD into B41 |  | - |  |  | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 4th IMD issue will be covered in DC_1A-41A_n78A |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | 2nd harmonic from B1 into n78 |  | - |  | - |  |  | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_41A_n78A | 7th & 8th harmonics from n78 into n257 |  | 4th IMD into B1 |  | - |  |  | - No harmonic impact to n257 - 4th IMD issue will be covered in DC_1A-41A_n78A |
|  | DC_41A_n257A, DC_41A_n257G, DC_41A_n257H, DC_41A_n257I | - |  | - |  | - |  |  | No issue |
| DC_1-42_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 |  | 4th IMD into B42 |  | - |  |  | - No harmonic problems by 7th & 8th order between FR1 and FR2 - B42 and n78 will be synchronous operation. |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | - |  | - |  | - |  |  | No issue |
|  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - |  | - |  | - |  |  | No issue |
| DC_3-41_n78-n257 | DC_3A_n78A | 7th & 8th harmonics from n78 into n257 |  | - |  | - |  |  | - Up to 7th harmonic order will be not any impact to n257 |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 |  | - |  | - |  |  | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_41A_n78A | 7th & 8th harmonics from n78 into n257 |  | 3rd & 4th IMDs into B3 |  | - |  |  | - No harmonic impact to n257 - 3rd & 4th IMDs issue will be covered in DC_3A-41A_n78A |
|  | DC_41A_n257A, DC_41A_n257G, DC_41A_n257H, DC_41A_n257I | - |  | - |  | - |  |  | No issue |
| DC_3-42_n78-n257 | DC_3A_n78A | 2nd harmonic from B3 into B42 7th & 8th harmonics from n78 into n257 |  | 4th & 5th IMDs into B42 - |  | - |  |  | - 2nd harmonic already covered in CA_3A-42A and Up to 7th harmonic order will be not any impact to n257 - These MSD issues are already covered in 3DL_DC_3A-42A_n78A with 2UL_DC_3A_n78A |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 |  | - |  | - |  |  | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - |  | - |  | - |  |  | No issue |
| DC_41-42_n78-n257 | DC_41A_n78A | 7th & 8th harmonics from n78 into n257 |  | - |  | - |  |  | - No harmonic impact to n257 |
|  | DC_41A_n257A, DC_41A_n257G, DC_41A_n257H, DC_41A_n257I | - |  | - |  | - |  |  | No issue |
|  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - |  | - |  | - |  |  | No issue |
| DC_1-18_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 |  | 5th IMD into B18 |  | - |  |  | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 5th IMD issue will be covered in DC_1A-18A_n78A |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | - |  | - |  | - |  |  | No issue |
|  | DC_18A_n78A | 7th & 8th harmonics from n78 into n257 |  | 3rd IMD into B1 |  | - |  |  | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 3rd IMD issue will be covered in DC_1A-18A_n78A |
|  | DC_18A_n257A, DC_18A_n257G, DC_18A_n257H, DC_18A_n257I | - |  | - |  | - |  |  | No issue |
| DC_3-18_n78-n257 | DC_3A_n78A | 7th & 8th harmonics from n78 into n257 |  |  |  |  |  |  | Up to 7th harmonic order will be not any impact to n257 |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 |  | - |  | - |  |  | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_18A_n78A | 7th & 8th harmonics from n78 into n257 |  | 3rd IMD into B3 |  | - |  |  | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 3rd IMD issue will be covered in DC_3A-18A_n78A |
|  | DC_18A_n257A, DC_18A_n257G, DC_18A_n257H, DC_18A_n257I | - |  | - |  | - |  |  | No issue |
| DC_18-42_n78-n257 | DC_18A_n78A | 7th & 8th harmonics from n78 into n257 |  |  |  | - |  |  | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  | DC_18A_n257A, DC_18A_n257G, DC_18A_n257H, DC_18A_n257I | - |  | - |  | - |  |  | No issue |
|  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - |  | - |  | - |  |  | No issue |
| DC_3-28_n78-n257 | DC_3A_n78A | 7th & 8th harmonics from n78 into n257 |  |  |  |  |  |  | Up to 7th harmonic order will be not any impact to n257 |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 |  | - |  | - |  |  | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_28A_n78A | 7th & 8th harmonics from n78 into n257 |  | 3rd IMD into n3 |  | - |  |  | Up to 7th harmonic order will be not any impact to n257 - 3rd IMD will be covered in DC_28A_n3A-n78A |
|  | DC_28A_n257A, DC_28A_n257G, DC_28A_n257H, DC_28A_n257I | - |  | - |  | - |  |  | No issue |
| DC_28-41_n78-n257 | DC_28A_n78A | 7th & 8th harmonics from n78 into n257 |  | 2nd IMD into B41 |  | - |  |  | Up to 7th harmonic order will be not any impact to n257 - 2nd IMD issue will be covered in DC_28A-41A_n78A |
|  | DC_28A_n257A, DC_28A_n257G, DC_28A_n257H, DC_28A_n257I | - |  | - |  | - |  |  | No issue |
|  | DC_41A_n78A | 7th & 8th harmonics from n78 into n257 |  | 2nd & 5th IMDs into B28 |  | - |  |  | - No harmonic impact to n257 - 2nd & 5th IMDs will be covered in DC_28A-41A_n78A |
|  | DC_41A_n257A, DC_41A_n257G, DC_41A_n257H, DC_41A_n257I | - |  | - |  | - |  |  | No issue |
| DC_28-42_n78-n257 | DC_28A_n78A | 7th & 8th harmonics from n78 into n257 |  | - |  | - |  |  | Up to 7th harmonic order will be not any impact to n257 |
|  | DC_28A_n257A, DC_28A_n257G, DC_28A_n257H, DC_28A_n257I | - |  | - |  | - |  |  | No issue |
|  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - |  | - |  | - |  |  | No issue |
| DC_1-28_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 |  | 5th IMD into B28 |  | - |  |  | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 5th IMD issue will be covered in DC_1A-28A_n78A |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | - |  | - |  | - |  |  | No issue |
|  | DC_28A_n78A | 3rd harmonic from B28 into B1 7th & 8th harmonics from n78 into n257 |  | -3rd IMD into B1 |  | - |  |  | - 3rd harmonic already covered in CA_1A-28A Up to 7th harmonic order will be not any impact to n257 - 3rd IMD issue will be covered in DC_1A-28A_n78A |
|  | DC_28A_n257A, DC_28A_n257G, DC_28A_n257H, DC_28A_n257I | - |  | - |  | - |  |  | No issue |
| DC_1-11_n77-n257 | DC_1A_n77A | 7th & 8th Harmonics from n77 into n257 |  | 2nd & 5th IMDs into B11 |  | Yes |  |  | - No harmonic problems by 7th & 8th order between FR1 and FR2 - These IMDs issue will be covered in DC_1A-11A_n77A |
|  | DC_1A_n257A | 2nd harmonic from B1 into n77 |  | - |  | - |  |  | - Harmonic problem already discussed and will be solved in DC_1A_n77A |
|  | DC_11A_n77A | 7th & 8th harmonics from n77 into n257 |  | 2nd & 5th IMDs into B1 |  | - |  |  | No harmonic problem to n257 - These IMDs issue will be covered in DC_1A-11A_n77A |
|  | DC_11A_n257A | - |  | - |  | - |  |  | No issue |
| DC_8-11_n77-n257 | DC_8A_n77A | 7th & 8th Harmonics from n77 into n257 |  | 3rd & 4th IMDs into B11 |  | - |  |  | - No harmonic problems by 7th & 8th order between FR1 and FR2 The MSD issue will be covered in 3DL_DC_8A-11A_n77A |
|  | DC_8A_n257A | 4th harmonic from B8 into n77 |  | - |  | - |  |  | - Harmonic problem will be solved in DC_8A_n77A |
|  | DC_11A_n77A | 7th & 8th harmonics from n77 into n257 |  | 3rd & 4th IMDs into B8 |  | - |  |  | No harmonic problem to n257 - These IMDs issue will be covered in DC_8A-11A_n77A |
|  | DC_11A_n257A | - |  | - |  | - |  |  | No issue |

Table 5.3-3 summarizes the EN-DC band combinations with self-interference problems for 5DL/2UL DC operation.

Table 5.3-3: Summary of Self-interference analysis for LTE 3 bands & NR 2 bands DL and 2 bands UL EN-DC operation

| Downlink band configuration | Uplink DC Configuration | Harmonic relation issues | intermodulation to own rx band | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- |
| DC_1-3-5_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 2nd IMD into B3 5th IMD into B5 | Yes | - Up to 7th harmonic order will be not any impact to n257 - These IMDs problem will be covered in DC_1A-3A_n78A and DC_1A-5A_n78A - Proximity also follow LTE_CA_1A-3A |
|  | DC_1A_n257A | 2nd harmonic from B1 into n78 | - | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_3A_n78A | 7th & 8th harmonics from n78 into n257 | 5th IMD into B1 | - | - Up to 7th harmonic order will be not any impact to n257 - 5th IMD problems will be covered in DC_1A-3A_n78A. |
|  | DC_3A_n257A | 2nd harmonic from B3 into n78 | - | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_5A_n78A | 7th & 8th harmonics from n78 into n257 | 3rd IMD into B1 3rd IMD into B3 | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 3rd IMD problems will be covered in DC_1A-5A_n78A and DC_3A-5A_n78A. |
|  | DC_5A_n257A | 4th harmonic from B5 into n78 | - | - | - Harmonic problem will be solved in DC_5A_n78A |
| DC_1-3-7_n28-n78 | DC_1A_n28A | 3rd harmonic into B1 5th harmonic into n78 | 2nd IMD into B7 3rd IMD into n78 | Yes | - Harmonic problems already solved in DC_1A_n78A or DC_n28A-n78A - 2nd IMD problem was solved in 3DL_DC_1A-7A_n28A - 3rd IMD problem was solved in 3DL_DC_1A_n28A-n78A |
|  | DC_1A_n78A | - | 2nd IMD into B3 4th IMD into B1 4th IMD into n78 5th IMD into n28 | Yes | - 2nd IMD problem was solved in 3DL_DC_1A-3A_n78A - 4th IMD problem was solved in DC_1A-n78A - 4th IMD problem is no issue for TDD band - 5th IMD problem was solved in 3DL_DC_1A_n28A-n78A |
|  | DC_3A_n28A | 2nd harmonic into n78 3rd harmonic into B1 5th harmonic into n78 | 3rd IMD into B7 4th IMD into B1 5th IMD into n78 |  | - Harmonic problems already solved in DC_3A_n78A or DC_1A_n28A or DC_28A-n78A - 3rd IMD problem was solved in 3DL_DC_3A-7A_n28A - 4th IMD problem was solved in 3DL_DC_1A-3A_n28A - 5th IMD problem was solved in 3DL_DC_3A_n28A-n78A |
|  | DC_3A_n78A | 2nd harmonic into n78 | 2nd,4th & 5th IMDs into B3 4th & 5th IMDs into n78 5th IMD into B1 |  | - 2nd harmonic problem already solved in DC_3A-n78A - 2nd,4th & 5th IMDs problems already solved in DC_3A-n78A - 4th & 5th IMDs problems are no issue for TDD band -5th IMD problem was solved in 3DL_DC_1A-3A_n78A |
|  | DC_7A_n28A | 3rd harmonic into B1 5th harmonic into n78 | 2nd IMD into B3 2nd,4th IMDs into n78 |  | - Harmonic problem was solved in DC_1A_n28A or DC_n28A-n78A - 2nd IMD problem was solved in 3DL_DC_3A-7A_n28A - 2nd & 4th IMDs problems were solved in 3DL_DC_7A_n28A-n78A |
|  | DC_7A_n78A | - | 2nd & 5th IMDs into n28 3rd & 4th IMDs into B3 4th IMD into n78 4th IMD into B1 |  | - 2nd & 5th IMDs problems were solved in 3DL_DC_7A-28A_n78A - 3rd & 4th IMDs problem were solved in 3DL_DC_3A-7A_n78A - 4th IMD problem is no issue for TDD band - 4th IMD problem was solved in 3DL_DC_1A-7A_n78A |
| DC_1-3-7-7_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 2nd IMD into B3 4th IMD into B7 | Yes | - Up to 7th harmonic order will be not any impact to n257 - These IMDs problem will be covered in DC_1A-3A_n78A and DC_1A-7A_n78A - Proximity also follow LTE_CA_1A-3A |
|  | DC_1A_n257A | 2nd harmonic from B1 into n78 | - | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_3A_n78A | 7th & 8th harmonics from n78 into n257 | 5th IMD into B1 | - | - Up to 7th harmonic order will be not any impact to n257 - 5th IMD problems will be covered in DC_1A-3A_n78A. |
|  | DC_3A_n257A | 2nd harmonic from B3 into n78 | - | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_7A_n78A | 7th harmonic from B7 into n257 | 3rd & 4th IMDs into B3 4th IMD into B1 | - | - Up to 7th harmonic order will be not any impact to n257 - The IMD problem will be covered in DC_3A-7A_n78A and DC_1A-7A_n78A |
|  | DC_7A_n257A | - | - | - | No issues |
| DC_1-3-7_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 2nd IMD into B3 4th IMD into B7 | Yes | - Up to 7th harmonic order will be not any impact to n257 - These IMDs problem will be covered in DC_1A-3A_n78A and DC_1A-7A_n78A - Proximity also follow LTE_CA_1A-3A |
|  | DC_1A_n257A | 2nd harmonic from B1 into n78 | - | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_3A_n78A | 7th & 8th harmonics from n78 into n257 | 5th IMD into B1 | - | - Up to 7th harmonic order will be not any impact to n257 - 5th IMD problems will be covered in DC_1A-3A_n78A. |
|  | DC_3A_n257A | 2nd harmonic from B3 into n78 | - | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_7A_n78A | 7th harmonic from B7 into n257 | 3rd & 4th IMDs into B3 4th IMD into B1 | - | - Up to 7th harmonic order will be not any impact to n257 - The IMD problem will be covered in DC_3A-7A_n78A and DC_1A-7A_n78A |
|  | DC_7A_n257A | - | - | - | No issues |
| DC_1-3-8_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 2nd IMD into B3 5th IMD into B8 | Yes | - Up to 7th harmonic order will be not any impact to n257 - These IMDs problem will be covered in DC_1A-3A_n78A and DC_1A-8A_n78A - Proximity also follow LTE_CA_1A-3A |
|  | DC_1A_n257A | 2nd harmonic from B1 into n78 | - | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_3A_n78A | 7th & 8th harmonics from n78 into n257 | 5th IMD into B1 | - | - Up to 7th harmonic order will be not any impact to n257 - 5th IMD problems will be covered in DC_1A-3A_n78A. |
|  | DC_3A_n257A | 2nd harmonic from B3 into n78 | - | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_8A_n78A | 2nd harmonic from B8 into B3 | 3rd IMD into B3 | - | - Harmonic problem already be solved in CA_3A_8A - 3rd IMD already covered in DC_3A-8A_n78A |
|  | DC_8A_n257A | 4th harmonic from B8 into n78 | - | - | - Harmonic problem will be solved in DC_8A_n78A |
| DC_1-5-7-7_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 4th IMD into B7 5th IMD into B5 | Yes | - Up to 7th harmonic order will be not any impact to n257 - These IMDs problem will be covered in DC_1A-7A_n78A and DC_1A-5A_n78A - Proximity also follow LTE_CA_1A-3A |
|  | DC_1A_n257A | 2nd harmonic from B1 into n78 | - | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_5A_n78A | 7th & 8th harmonics from n78 into n257 | 2nd IMD into B7 3rd IMD into B1 | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - These IMDs problems will be covered in DC_5A-7A_n78A and DC_1A-5A_n78A. |
|  | DC_5A_n257A | 4th harmonic from B5 into n78 | - | - | - Harmonic problem will be solved in DC_5A_n78A |
|  | DC_7A_n78A | 7th harmonic from B7 into n257 | 2nd & 5th IMDs into B5 4th IMD into B1 | - | - Up to 7th harmonic order will be not any impact to n257 - These IMDs problems will be covered in DC_5A-7A_n78A and DC_1A-7A_n78A |
|  | DC_7A_n257A | - | - | - | No issues |
| DC_1-5-7_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 4th IMD into B7 5th IMD into B5 | Yes | - Up to 7th harmonic order will be not any impact to n257 - These IMDs problem will be covered in DC_1A-7A_n78A and DC_1A-5A_n78A - Proximity also follow LTE_CA_1A-3A |
|  | DC_1A_n257A | 2nd harmonic from B1 into n78 | - | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_5A_n78A | 7th & 8th harmonics from n78 into n257 | 2nd IMD into B7 3rd IMD into B1 | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - These IMDs problems will be covered in DC_5A-7A_n78A and DC_1A-5A_n78A. |
|  | DC_5A_n257A | 4th harmonic from B5 into n78 | - | - | - Harmonic problem will be solved in DC_5A_n78A |
|  | DC_7A_n78A | 7th harmonic from B7 into n257 | 2nd & 5th IMDs into B5 4th IMD into B1 | - | - Up to 7th harmonic order will be not any impact to n257 - These IMDs problems will be covered in DC_5A-7A_n78A and DC_1A-7A_n78A |
|  | DC_7A_n257A | - | - | - | No issues |
| DC_3-5-7-7_n78-n257 | DC_3A_n78A | 7th & 8th harmonics from n78 into n257 | - | - | - Up to 7th harmonic order will be not any impact to n257 |
|  | DC_3A_n257A | 2nd harmonic from B3 into n78 | - | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_5A_n78A | 7th & 8th harmonics from n78 into n257 | 3rd IMD into B3 2nd IMD into B7 | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 3rd IMD problems will be covered in DC_3A-5A_n78A and DC_5A-7A_n78A. |
|  | DC_5A_n257A | 4th harmonic from B5 into n78 | - | - | - Harmonic problem will be solved in DC_5A_n78A |
|  | DC_7A_n78A | 7th harmonic from B7 into n257 | 2nd & 5th IMDs into B5 3rd & 4th IMDs into B3 | - | - Up to 7th harmonic order will be not any impact to n257 - These IMDs problems will be covered in DC_5A-7A_n78A and DC_3A-7A_n78A |
|  | DC_7A_n257A | - | - | - | No issues |
| DC_3-5-7_n78-n257 | DC_3A_n78A | 7th & 8th harmonics from n78 into n257 | - | - | - Up to 7th harmonic order will be not any impact to n257 |
|  | DC_3A_n257A | 2nd harmonic from B3 into n78 | - | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_5A_n78A | 7th & 8th harmonics from n78 into n257 | 3rd IMD into B3 2nd IMD into B7 | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 3rd IMD problems will be covered in DC_3A-5A_n78A and DC_5A-7A_n78A. |
|  | DC_5A_n257A | 4th harmonic from B5 into n78 | - | - | - Harmonic problem will be solved in DC_5A_n78A |
|  | DC_7A_n78A | 7th harmonic from B7 into n257 | 2nd & 5th IMDs into B5 3rd & 4th IMDs into B3 | - | - Up to 7th harmonic order will be not any impact to n257 - These IMDs problems will be covered in DC_5A-7A_n78A and DC_3A-7A_n78A |
|  | DC_7A_n257A | - | - | - | No issues |
| DC_1-3-21_n77-n78 | DC_3A_n77A | 2nd harmonic from B3 into n78 | 2nd & 5th IMDs into B1 4th & 5th IMDs into B21 4th & 5th IMDs into n78 | - | 2nd harmonic issue already covered in DC_3A_n78A These MSD issues are already covered in 3DL_DC_1A-3A_n77A and 3DL_DC_3A-21A_n77A and 3DL_DC_3A_n77A-n78A with 2UL_DC_3A_n77A |
|  | DC_3A_n78A | 2nd harmonic from B3 into n77 | 5th IMD into B1 4th & 5th IMDs into B21 4th & 5th IMDs into n77 | - | 2nd harmonic issue already covered in DC_3A_n77A These MSD issues are already covered in 3DL_DC_1A-3A_n78A and 3DL_DC_3A-21A_n78A and 3DL_DC_3A_n77A-n78A with 2UL_DC_3A_n78A |
| DC_1-3-21_n77-n79 | DC_3A_n77A | - | 2nd & 5th IMDs into B1 4th & 5th IMDs into B21 3rd & 4th IMDs into n79 | - | These MSD issues are already covered in 3DL_DC_1A-3A_n77A and 3DL_DC_3A-21A_n77A and 3DL_DC_3A_n77A-n79A with 2UL_DC_3A_n77A |
|  | DC_3A_n79A | 2nd harmonic from B3 into n77 | 5th IMD into B1 3rd IMD into B21 5th IMD into n77 | - | 2nd harmonic issue already covered in DC_3A_n77A These MSD issues are already covered in 3DL_DC_1A-3A_n79A and 3DL_DC_3A-21A_n79A and 3DL_DC_3A_n77A-n79A with 2UL_DC_3A_n79A |
|  | DC_1A_n77A | - | 2nd, 4th & 5th IMDs into B3 2nd & 5th IMDs into B21 3rd, 4th & 5th IMD into n79 | Yes | - These IMDs issue will be covered in DC_1A-3A_n77A - 2nd & 5th IMDs will be covered in DC_1A-21A_n77A - 3rd, 4th & 5th IMDs will be covered in DC_1A_n77A-n79A |
|  | DC_1A_n79A | 2nd harmonic from B1 into n77 | 4th IMD into B21 5th IMD into n77 | - | 2nd harmonic issue already covered in DC_1A_n77A The MSD issue are already covered in 3DL_DC_1A-21A_n79A The MSD issue are already covered in 3DL_DC_1A_n77A-n79A |
|  | DC_21A_n77A |  | 2nd & 5th IMDs into B1 2nd & 5th IMDs into B3 2nd & 4th IMDs into n79 | - | These MSD issues are already covered in 3DL_DC_1A-21A_n77A These MSD issues are already covered in 3DL_DC_3A-21A_n77A These MSD issues are already covered in 3DL_21A_n77A-n79A |
|  | DC_21A_n79A |  | 3rd IMD into B3 2nd IMD into n77 |  | These MSD issues are already covered in DC_3A-21A_n79A These MSD issues are already covered in DC_21A_n77A-n79A |
| DC_1-3-21_n78-n79 | DC_3A_n78A | - | 5th IMD into B1 4th & 5th IMDs into B21 3rd IMD into n79 | - | These MSD issues are already covered in 3DL_DC_1A-3A_n78A and 3DL_DC_3A-21A_n78A and 3DL_DC_3A_n78A-n79A with 2UL_DC_3A_n78A |
|  | DC_3A_n79A | 2nd harmonic from B3 into n78 | 5th IMD into B1 3rd IMD into B21 5th IMD into n78 | - | 2nd harmonic issue already covered in DC_3A_n78A These MSD issues are already covered in 3DL_DC_1A-3A_n79A and 3DL_DC_3A-21A_n79A and 3DL_DC_3A_n78A-n79A with 2UL_DC_3A_n79A |
|  | DC_1A_n78A | - | 2nd IMD into B3 2nd & 5th IMDs into B21 3rd & 5th IMD into n79 | Yes | These MSD issues are already covered in 3DL_DC_1A-3A_n78A. These MSD issues are already covered in 3DL_DC_1A-21A_n78A. These MSD issues are already covered in 3DL_DC_1A_n78A-n79A |
|  | DC_1A_n79A | - | 4th IMD into B21 5th IMD into n78 | - | The MSD issue are already covered in 3DL_DC_1A-21A_n79A The MSD issue are already covered in 3DL_DC_1A_n78A-n79A |
|  | DC_21A_n78A | - | 2nd & 5th IMDs into B1 2nd IMDs into B3 2nd & 4th IMDs into n79 | - | These MSD issue is already covered in 3DL_DC_1A-21A_n78A These MSD issue is already covered in 3DL_DC_3A-21A_n78A. These MSD issue is already covered in 3DL_DC_21A_n78A-n79A |
|  | DC_21A_n79A | - | 3rd IMD into B3 2nd IMD into n78 | - | These MSD issues are already covered in DC_3A-21A_n79A These MSD issues are already covered in DC_21A_n78A-n79A |
| DC_1-3-28_n28-n78 | DC_1A_n28A | 5th harmonic from n28 into n78 | 3rd IMD into n78 | Yes | Harmonic issue already covered in CA_n28A-n78A These MSD issues are already covered in 3DL_DC_1A_n28A-n78A with 2UL_DC_1A_n28A |
|  | DC_3A_n28A DC_3C_n28A | 2nd harmonic from B3 into n78 3rd harmonic from n28 into B1 5th harmonic from n28 into n78 | 4th IMD into B1 5th IMD into n78 |  | Harmonic issues are already covered in DC_3A_n78A, DC_1A_n28A and CA_n28A-n78A. These MSD issues are already covered in 3DL_DC_1A-3A_n28A with 2UL_DC_3A_n28A and 3DL_DC_3A_n28A-n78A with 2UL_DC_3A_n28A |
|  | DC_28A_n28A | 3rd harmonic from B28 into B1 | 3rd & 5th IMDs into B1 5th IMD into n78 | - | Harmonic issue already covered in DC_1A_n28A The MSD issue will be covered in 3DL_1A-28A_n28A with 2UL_DC_28A_n28A and 3DL_28A_n28A-n78A with 2UL_DC_28A_n28A |
|  | DC_1A_n78A | - | 2nd IMD into B3 5th IMD into n28 | Yes | The MSD issue will be covered in 3DL_1A-3A_n78A with 2UL_DC_1A_n78A and 3DL_1A_n28A-n78A with 2UL_DC_1A_n78A |
|  | DC_3A_n78A DC_3C_n78A | - | 5th IMD into B1 | - | The MSD issue will be covered in 3DL_1A-3A_n78A with 2UL_DC_3A_n78A |
|  | DC_28A_n78A | 3rd harmonic from B28 into B1 | 3rd IMD into B1 3rd IMD into B3 5th IMD into n28 | - | Harmonic issue already covered in DC_1A_n28A The MSD issue will be covered in 3DL_DC_1A-28A_n78A and 3DL_DC_3A-28A_n28A-n78A and 3DL_DC_28A_n28A-n78A with 2UL_DC_28A_n78A |
| DC_1-7-28_n28-n78 | DC_1A_n28A | 5th harmonic from n28 into n78 | 2nd IMD into B7 3rd IMD into n78 | - | Harmonic issue already covered in CA_n28A-n78A These MSD issues are already covered in 3DL_DC_1A-7A_n28A and 3DL_DC_1A_n28A-n78A with 2UL_DC_1A_n28A |
|  | DC_7A_n28A DC_7C_n28A | 3rd harmonic into B1 5th harmonic into n78 | 2nd,4th IMDs into n78 | - | Harmonic issue already covered in DC_1A_n28A or CA_n28A-n78A These MSD issues are already covered in 3DL_DC_7A_n28A-n78A with 2UL_DC_7A_n28A |
|  | DC_28A_n28A | 3rd harmonic from B28 into B1 | 3rd & 5th IMDs into B1 5th IMD into n78 | - | Harmonic issue already covered in DC_1A_n28A The MSD issue will be covered in 3DL_1A-28A_n28A with 2UL_DC_28A_n28A and 3DL_28A_n28A-n78A with 2UL_DC_28A_n28A |
|  | DC_1A_n78A | - | 4th IMD into B7 5th IMD into B28 or n28 | - | The MSD issue will be covered in 3DL_DC_1A-7A_n78A with 2UL_DC_1A_n78A and 3DL_DC_1A_n28A-n78A with 2UL_DC_1A_n78A |
|  | DC_7A_n78A DC_7C_n78A | - | 4th IMD into B1 2nd & 5th IMDs into B28 or n28 | - | The MSD issue will be covered in 3DL_DC_1A-7A_n78A with 2UL_DC_7A_n78A and 3DL_DC_7A_n28A-n78A with 2UL_DC_7A_n78A |
|  | DC_28A_n78A | 3rd harmonic from B28 into B1 | 3rd IMD into B1 2nd IMD into B7 5th IMD into n28 |  | Harmonic issue already covered in DC_1A_n28A The MSD issue will be covered in 3DL_DC_1A-28A_n78A and 3DL_DC_7A-28A_n78A and 3DL_DC_28A_n28A-n78A with 2UL_DC_28A_n78A |
| DC_1-19-42_n77-n79 | DC_19A_n77A | 6th harmonic from B19 into n79 | 3rd IMD into B1 2nd, 3rd , 4th & 5th IMDs into n79 | - | 6th harmonic problem already covered in DC_19A_n79A These MSD issues are already covered in 3DL_DC_1A-19A_n77A and 3DL_DC_19A_n77A-n79A with 2UL_DC_19A_n77A. |
|  | DC_19A_n79A | 4th harmonic from B19 into n77 | 4th IMD into B1 2nd IMD into B42 2nd & 3rd IMDs into n77 | - | 4th harmonic will be covered in DC_19A_n77A The MSD issue will be covered in 3DL_DC_1A-19A_n79A and 3DL_DC_19A-42A_n79A and 3DL_DC_19A_n77A-n79A with 2UL_DC_19A_n79A |
|  | DC_1A_n77A |  | 5th IMD into B19 4th & 5th IMD into B42 3rd, 4th & 5th IMD into n79 | - | These MSD issues are already covered in 3DL_DC_1A-19A_n77A. These MSD issues are already covered in 3DL_DC_1A-42A_n77A. These MSD issues are already covered in 3DL_DC_1A_n77A-n79A. |
|  | DC_1A_n79A | 2nd harmonic from B1 into n77 | 3rd & 4th IMDs into B19 5th IMD into B42 5th IMD into n77 | - | 2nd harmonic issue already covered in DC_1A_n77A The MSD issue are already covered in 3DL_DC_1A-19A_n79A The MSD issue are already covered in 3DL_DC_1A-42A_n79A The MSD issue are already covered in 3DL_DC_1A_n77A-n79A |
| DC_1-19-42_n78-n79 | DC_19A_n78A | 6th harmonic from B19 into n79 | 3rd IMD into B1 2nd, 3rd, 4th & 5th IMD into n79 | - | 6th harmonic problem already covered in DC_19A_n79A These MSD issues are already covered in 3DL_DC_1A-19A_n78A and 3DL_DC_19A_n78A-n79A with 2UL_DC_19A_n78A. |
|  | DC_19A_n79A | 4th harmonic from B19 into n78 | 4th IMD into B1 2nd IMD into B42 2nd & 3rd IMDs into n78 | - | 4th harmonic will be covered in DC_19A_n78A The MSD issue will be covered in 3DL_DC_1A-19A_n79A and 3DL_DC_19A-42A_n79A and 3DL_DC_19A_n78A-n79A with 2UL_DC_19A_n79A |
|  | DC_1A_n78A | - | 5th IMD into B19 4th IMD into B42 3rd & 5th IMD into n79 | - | The MSD issue will be covered in 3DL_DC_1A-19A_n78A. The MSD issue will be covered in 3DL_DC_1A-42A_n78A. These MSD issue will be covered in 3DL_DC_1A_n78A-n79A. |
|  | DC_1A_n79A | - | 3rd & 4th IMDs into B19 5th IMD into B42 5th IMD into n78 | - | 2nd harmonic issue already covered in DC_1A_n77A The MSD issue are already covered in 3DL_DC_1A-19A_n79A The MSD issue are already covered in 3DL_DC_1A-42A_n79A The MSD issue are already covered in 3DL_DC_1A_n78A-n79A |
| DC_1-21-42_n77-n78 | DC_1A_n77A | - | 2nd & 5th IMDs into B21 4th & 5th IMD into B42 4th & 5th IMD into n78 | - | These MSD issues are already covered in 3DL_DC_1A-21A_n77A and 3DL_DC_1A-42A_n77A and 3DL_DC_1A_n77A-n78A with 2UL_DC_1A_n77A |
|  | DC_1A_n78A | 2nd harmonic from B1 into n77 | 2nd & 5th IMDs into B21 4th IMD into B42 4th & 5th IMD into n77 | - | The harmonic already covered in DC_1A_n77A These MSD issues are already covered in 3DL_DC_1A-21A_n78A and 3DL_DC_1A-42A_n78A and 3DL_DC_1A_n77A-n78A with 2UL_DC_1A_n78A. |
| DC_1-21-42_n77-n79 | DC_1A_n77A | - | 2nd & 5th IMDs into B21 4th & 5th IMD into B42 3rd, 4th & 5th IMD into n79 | - | These MSD issues are already covered in 3DL_DC_1A-21A_n77A and 3DL_DC_1A-42A_n77A and 3DL_DC_1A_n77A-n79A with 2UL_DC_1A_n77A |
|  | DC_1A_n79A | 2nd harmonic from B1 into n77 | 4th IMD into B21 5th IMD into B42 5th IMD into n77 | - | 2nd harmonic issue already covered in DC_1A_n77A The MSD issue are already covered in 3DL_DC_1A-21A_n79A and 3DL_DC_1A-42A_n79A and 3DL_DC_1A_n77A-n79A with 2UL_DC_1A_n79A |
|  | DC_21A_n77A | - | 2nd & 5th IMDs into B1 5th IMD into B42 2nd & 4th IMDs into n79 | - | These MSD issues are already covered in 3DL_DC_1A-21A_n77A These MSD issues are already covered in 3DL_DC_21A-42A_n77A These MSD issues are already covered in 3DL_21A_n77A-n79A |
|  | DC_21A_n79A | - | 2nd IMD into B42 2nd IMD into n77 | - | These MSD issues are already covered in DC_21A-42A_n79A These MSD issues are already covered in DC_21A_n77A-n79A |
| DC_1-21-42_n78-n79 | DC_1A_n78A | - | 2nd & 5th IMDs into B21 4th IMD into B42 3rd & 5th IMD into n79 | - | These MSD issues are already covered in 3DL_DC_1A-21A_n78A and 3DL_DC_1A-42A_n78A and 3DL_DC_1A_n78A-n79A with 2UL_DC_1A_n78A. |
|  | DC_1A_n79A | - | 4th IMD into B21 5th IMD into B42 5th IMD into n78 | - | The MSD issue are already covered in 3DL_DC_1A-21A_n79A and 3DL_DC_1A-42A_n79A and 3DL_DC_1A_n78A-n79A with 2UL_DC_1A_n79A |
|  | DC_21A_n78A | - | 2nd & 5th IMDs into B1 2nd & 4th IMDs into n79 | - | These MSD issue is already covered in 3DL_DC_1A-21A_n78A. These MSD issue is already covered in 3DL_DC_21A_n78A-n79A |
|  | DC_21A_n79A | - | 2nd IMD into B42 2nd IMD into n78 | - | These MSD issues are already covered in DC_21A-42A_n79A These MSD issues are already covered in DC_21A_n78A-n79A |
| DC_3-7-28_n28-n78 | DC_3A_n28A DC_3C_n28A | 2nd harmonic from B3 into n78 5th harmonic from n28 into n78 | 3rd IMD into B7 5th IMD into n78 |  | Harmonic issues are already covered in DC_3A_n78A or CA_n28A-n78A. These MSD issues are already covered in 3DL_DC_3A-7A_n28A with 2UL_DC_3A_n28A and 3DL_DC_3A_n28A-n78A with 2UL_DC_3A_n28A |
|  | DC_7A_n28A DC_7C_n28A | 5th harmonic from n28 into n78 | 2nd IMD into B3 2nd,4th IMDs into n78 | - | Harmonic issue already covered in CA_n28A-n78A These MSD issues are already covered in 3DL_DC_3A-7A_n28A with 2UL_DC_7A_n28A and 3DL_DC_7A_n28A-n78A with 2UL_DC_7A_n28A |
|  | DC_28A_n28A | - | 5th IMD into n78 | - | The MSD issue will be covered in 3DL_DC_28A_n28A-n78A with 2UL_DC_28A_n28A |
|  | DC_3A_n78A DC_3C_n78A | - | - | - | No issue |
|  | DC_7A_n78A DC_7C_n78A | - | 3rd & 4th IMDs into B3 2nd & 5th IMDs into B28 or n28 | - | The MSD issue will be covered in 3DL_DC_3A-7A_n78A with 2UL_DC_7A_n78A and 3DL_DC_7A_n28A-n78A with 2UL_DC_7A_n78A |
|  | DC_28A_n78A | - | 3rd IMD into B3 2nd IMD into B7 5th IMD into n28 |  | The MSD issue will be covered in 3DL_DC_3A-28A_n78A and 3DL_DC_7A-28A_n78A and 3DL_DC_28A_n28A-n78A with 2UL_DC_28A_n78A |
| DC_19-21-42_n77-n78 | DC_19A_n77A | 4th harmonic from B19 into n78 | 4th IMD into B21 2nd IMD into n78 | - | 4th harmonic problem already covered in DC_19A_n78A These MSD issues are already covered in 3DL_DC_19A-21A_n77A and 3DL_DC_19A_n77A-n78A with 2UL_DC_19A_n77A. |
|  | DC_19A_n78A | 4th harmonic from B19 into n77 | 2nd & 5th IMDs into n77 | - | 4th harmonic problem already covered in DC_19A_n77A These MSD issues are already covered in 3DL_DC_19A_n77A-n78A with 2UL_DC_19A_n78A. |
| DC_19-21-42_n77-n79 | DC_19A_n77A | 6th harmonic from B19 into n79 | 4th IMD into B21 2nd, 3rd , 4th & 5th IMDs into n79 | - | 6th harmonic problem already covered in DC_19A_n79A These MSD issues are already covered in 3DL_DC_19A-21A_n77A and 3DL_DC_19A_n77A-n79A with 2UL_DC_19A_n77A. |
|  | DC_19A_n79A | 4th harmonic from B19 into n77 | 5th IMD into B21 2nd IMD into B42 2nd & 3rd IMDs into n77 | - | 4th harmonic will be covered in DC_19A_n77A The MSD issue will be covered in 3DL_DC_19A-21A_n79A and 3DL_DC_19A-42A_n79A and 3DL_DC_19A_n77A-n79A with 2UL_DC_19A_n79A |
|  | DC_21A_n77A | - | 3rd & 4th IMDs into B19 5th IMD into B42 2nd & 4th IMDs into n79 | - | These MSD issues are already covered in 3DL_DC_19A-21A_n77A These MSD issues are already covered in 3DL_DC_21A-42A_n77A These MSD issues are already covered in 3DL_21A_n77A-n79A |
|  | DC_21A_n79A | - | 5th IMD into B19 2nd IMD into B42 2nd IMD into n77 | - | These MSD issues are already covered in DC_19A-21A_n79A These MSD issues are already covered in DC_21A-42A_n79A These MSD issues are already covered in DC_21A_n77A-n79A |
| DC_19-21-42_n78-n79 | DC_19A_n78A | 6th harmonic from B19 into n79 | 2nd, 3rd, 4th & 5th IMD into n79 | - | 6th harmonic problem already covered in DC_19A_n79A These MSD issues are already covered in 3DL_DC_19A_n78A-n79A with 2UL_DC_19A_n78A. |
|  | DC_19A_n79A | 4th harmonic from B19 into n78 | 5th IMD into B21 2nd IMD into B42 2nd & 3rd IMDs into n78 | - | 4th harmonic will be covered in DC_19A_n78A The MSD issue will be covered in 3DL_DC_19A-21A_n79A and 3DL_DC_19A-42A_n79A and 3DL_DC_19A_n78A-n79A with 2UL_DC_19A_n79A |
|  | DC_21A_n78A | - | 3rd & 4th IMDs into B19 2nd & 4th IMDs into n79 | - | These MSD issue is already covered in DC_19A-21A_n78A. These MSD issue is already covered in DC_21A_n78A-n79A |
|  | DC_21A_n79A | - | 5th IMD into B19 2nd IMD into B42 2nd IMD into n78 | - | These MSD issues are already covered in DC_19A-21A_n79A These MSD issues are already covered in DC_21A-42A_n79A These MSD issues are already covered in DC_21A_n78A-n79A |
| DC_1-3-7_n5-n78 | DC_1A_n5A | 4th harmonic from n5 into n78 | 3rd IMD into n78 | - | - 4th harmonic already covered in Table 7.3B.2.3.1-1 in TS38.101-3 - The 3rd IMD issue will be covered in DC_1_n5A-n78A |
|  | DC_3A_n5A, DC_3C_n5A | 2nd harmonic from B3 into n78 | 2nd & 3rd IMDs into B7 3rd & 5th IMDs into n78 | - | - 2nd harmonic problem already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - These IMD issue will be covered in DC_3A-7A_n5A These IMD issue will be covered in DC_3A_n5A-n78A |
|  | DC_7A_n5A, DC_7C_n5A | - | 2nd & 4th IMDs into n78 | - | These IMDs problem will be covered in DC_7A_n5A-n78A |
|  | DC_1A_n78A | - | 2nd IMD into B3 5th IMD into n5 4th IMD into B7 | - | The MSD issue will be covered in 3DL_DC_1A-3A_n78A. The MSD issue will be covered in 3DL_DC_1A_n5A-n78A. The MSD issue will be covered in 3DL_DC_1A-7A_n78A. |
|  | DC_3A_n78A, DC_3C_n78A | - | 5th IMD into B1 | - | The MSD issue will be covered in 3DL_1A-3A_n78A. |
|  | DC_7A_n78A, DC_7C_n78A |  | 4th IMD into B1 3rd & 4th IMDs into B3 2nd & 5th IMDs into n5 | - | The MSD issue will be covered in DC_1A-7A_n78A. The MSD issue will be covered in DC_3A-7A_n78A. The MSD issue will be covered in DC_7A_n5A-n78A. |
| DC_1-3-28_n5-n78 | DC_1A_n5A | 4th harmonic from n5 into n78 | 3rd IMD into n78 | - | - 4th harmonic already covered in Table 7.3B.2.3.1-1 in TS38.101-3 - The 3rd IMD issue will be covered in DC_1_n5A-n78A |
|  | DC_3A_n5A, DC_3C_n5A | 2nd harmonic from B3 into n78 | 4th IMD into B28 3rd & 5th IMDs into n78 | - | - 2nd harmonic problem already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - The IMD issue will be covered in DC_3A-28A_n5A These IMD issue will be covered in DC_3A_n5A-n78A |
|  | DC_28A_n5A | 3rd harmonic from B28 into B1 4th harmonic from n5 into n78 5th harmonic from B28 into n78 | 5th IMD into B1 4th IMD into B3 |  | - These harmonic problems already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - The 5th IMD will be covered in DC_1A-28A_n5A - The 4th IMD will be covered in DC_3A-28A_n5A |
|  | DC_1A_n78A | - | 2nd IMD into B3 5th IMD into n5 5th IMD into B28 | - | The MSD issue will be covered in 3DL_DC_1A-3A_n78A. The MSD issue will be covered in 3DL_DC_1A_n5A-n78A. The MSD issue will be covered in 3DL_DC_1A-28A_n78A. |
|  | DC_3A_n78A, DC_3C_n78A | - | 5th IMD into B1 | - | The MSD issue will be covered in 3DL_1A-3A_n78A. |
|  | DC_28A_n78A | 3rd harmonic from B28 into B1 | 3rd IMD into B1 3rd IMD into B3 5th IMD into n5 | - | Harmonic issue already covered in DC_1A_n28A The MSD issue will be covered in DC_1A-28A_n78A. The MSD issue will be covered in DC_3A-28A_n78A. The MSD issue will be covered in DC_28A_n5A-n78A. |
| DC_1-7-28_n5-n78 | DC_1A_n5A | 4th harmonic from n5 into n78 | 3rd IMD into n78 | - | - 4th harmonic already covered in Table 7.3B.2.3.1-1 in TS38.101-3 - The 3rd IMD issue will be covered in DC_1_n5A-n78A |
|  | DC_7A_n5A, DC_7C_n5A | - | 3rd & 5th IMDs into B28 2nd & 4th IMDs into n78 | - | These IMDs problem will be covered in DC_7A-28A_n5A These IMDs problem will be covered in DC_7A_n5A-n78A |
|  | DC_28A_n5A | 3rd harmonic from B28 into B1 4th harmonic from n5 into n78 5th harmonic from B28 into n78 | 5th IMD into B1 5th IMD into B7 |  | - These harmonic problems already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - The 5th IMD will be covered in DC_1A-28A_n5A - The 5th IMD will be covered in DC_7A-28A_n5A |
|  | DC_1A_n78A | - | 5th IMD into n5 4th IMD into B7 5th IMD into B28 | - | The MSD issue will be covered in DC_1A_n5A-n78A. The MSD issue will be covered in DC_1A-7A_n78A The MSD issue will be covered in DC_1A-28A_n78A.. |
|  | DC_7A_n78A, DC_7C_n78A |  | 4th IMD into B1 2nd & 5th IMDs into n5 2nd & 5th IMDs into B28 | - | The MSD issue will be covered in DC_1A-7A_n78A. The MSD issue will be covered in DC_7A_n5A-n78A. The MSD issue will be covered in DC_7A-28A_n78A. |
|  | DC_28A_n78A | 3rd harmonic from B28 into B1 | 3rd IMD into B1 5th IMD into n5 2nd IMD into B7 | - | Harmonic issue already covered in DC_1A_n28A The MSD issue will be covered in DC_1A-28A_n78A. The MSD issue will be covered in DC_28A_n5A-n78A. The MSD issue will be covered in DC_7A-28A_n78A. |
| DC_3-7-28_n5-n78 | DC_3A_n5A, DC_3C_n5A | 2nd harmonic from B3 into n78 | 2nd & 3rd IMDs into B7 4th IMD into B28 3rd & 5th IMDs into n78 | - | - 2nd harmonic problem already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - The IMD issue will be covered in DC_3A-7A_n5A - The IMD issue will be covered in DC_3A-28A_n5A These IMD issue will be covered in DC_3A_n5A-n78A |
|  | DC_7A_n5A | - | 3rd & 5th IMDs into B28 2nd & 4th IMDs into n78 | - | These IMDs problem will be covered in DC_7A-28A_n5A These IMDs problem will be covered in DC_7A_n5A-n78A |
|  | DC_28A_n5A | 4th harmonic from n5 into n78 5th harmonic from B28 into n78 | 4th IMD into B3 5th IMD into B7 |  | - These harmonic problems already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - The 4th IMD will be covered in DC_3A-28A_n5A - The 5th IMD will be covered in DC_7A-28A_n5A |
|  | DC_3A_n78A, DC_3C_n78A | - | - | - | No issue |
|  | DC_7A_n78A |  | 3rd & 4th IMDs into B3 2nd & 5th IMDs into n5 2nd & 5th IMDs into B28 | - | The MSD issue will be covered in DC_3A-7A_n78A. The MSD issue will be covered in DC_7A_n5A-n78A. The MSD issue will be covered in DC_7A-28A_n78A. |
|  | DC_28A_n78A | - | 3rd IMD into B3 5th IMD into n5 2nd IMD into B7 | - | The MSD issue will be covered in DC_3A-28A_n78A. The MSD issue will be covered in DC_28A_n5A-n78A. The MSD issue will be covered in DC_7A-28A_n78A. |
| DC_2-7-66-n66-n78 DC_2-7-7-66-n66-n78 | DC_2A_n66A | 2nd harmonic from B2 into n78 2nd harmonic from n66 inton78 | 5th IMD into B66 2nd & 4th IMDs into n78 | - | - These harmonic problems already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - 5th IMD will be covered in DC_2A-66A_n66A - 2nd & 4th IMDs will be covered in DC_2A_n66A-n78A |
|  | DC_2A_n78A | - | 4th IMD into B66/n66 | - | 4th IMD problem same as DC_2A-66A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
|  | DC_7A_n66A | 2nd harmonic from n66 into n78 | 3rd IMD into n78 | - | The harmonic problem already covered in Table 7.3B.2.3.1-1 in TS38.101-3 3rd IMD issue will be covered in DC_7A_n66A-n78A |
|  | DC_7A_n78A | - | 4th IMD into n66 4th IMD into B2 | Yes | The 4th IMD problem same as DC_7A-66A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 4th IMD issue will be covered in DC_2A-7A_n78A |
|  | DC_66A_n66A | 2nd harmonic from B66 into n78 | 2nd & 4th IMDs into n78 |  | - Harmonic problem already covered in DC_66A_n78A - 2nd & 4th IMDs issues will be covered in DC_66A_n66A-n78A |
|  | DC_66A_n78A | - | 2nd , 4th & 5th IMDs into B2 5th IMD into n66 |  | - 2nd , 4th & 5th IMDs issue will be covered in DC_2A-66A_n78A - 5th IMD issue will be covered in DC_66A_n66A-n78A |
| DC_1-3-20_n38-n78 | DC_3A_n78A | - | 5th IMD into B1 | - | The MSD issue will be covered in 3DL_1A-3A_n78A. |
|  | DC_20A_n78A | 3rd harmonic from B20 into n38 | 3rd IMD into B1 3rd IMD into B3 2nd IMD into n38 | - | The 3rd harmonic issue will be covered in DC_20A_n38A - 3rd IMD issue will be covered in DC_1A-20A_n78A - 3rd IMD issue will be covered in DC_3A-20A_n78A - 2nd IMD issue will be covered in DC_20A_n38A-n78A |
|  | DC_3A_n38A | 2nd harmonic from B3 into n78 | 2nd & 3rd IMDs into B20 3rd IMD into n78 | - | - The harmonic problem already covered in DC_3A_n78A - 2nd & 3rd IMDs issue will be covered in DC_3A-20A_38A - 3rd IMD issue will be covered in DC_3A_n38A-n78A |
|  | DC_20A_n38A | 4th harmonic from B20 into n78 | 2nd & 4th IMDs into n78 |  | - The harmonic problem already covered in DC_20A_n78A - 2nd & 4th IMDs issue will be covered in DC_20A_n38A-n78A |
| DC_1-20-38_n3-n78 | DC_1A_n3A | 2nd harmonic from 3 into n78 | 2nd & 4th IMDs into n78 | Yes | - Harmonic problems was already covered in DC_3A_n78A. - 2nd &4th IMDs problems will be covered in DC_1A_n3A-n78A. |
|  | DC_20A_n3A | 3rd harmonic from B20 into n38 - 2nd harmonic from n3 into n78 - 4th harmonic from B20 into n78 | 2nd & 3rd IMDs into n38 3rd & 5th IMDs into n78 | - | - Harmonic problems was already covered in DC_20A_n38A or DC_20A_n78A or CA_n3A-n78A. - These IMDs problems will be covered in DC_20A_n3A-n38A - These IMDs problems will be covered in DC_20A_n3A-n78A. |
|  | DC_1A_n78A | - | 2nd IMD into n3 5th IMD into B20 4th IMD into B38 | Yes | The MSD issue will be covered in DC_1A_n3A-n78A. The MSD issue will be covered in DC_1A-20A_n78A. The MSD issue will be covered in DC_1A-38A_n78A |
|  | DC_20A_n78A | 3rd harmonic from B20 into B38 | 3rd IMD into B1 3rd IMD into n3 2nd IMD into B38 | - | The 3rd harmonic issue will be covered in CA_20A-38A - 3rd IMD issue will be covered in DC_1A-20A_n78A - 3rd IMD issue will be covered in DC_20A_n3A-n78A - 2nd IMD issue will be covered in DC_20A-38A_n78A |
| DC_1-7-20_n3-n78 | DC_1A_n3A | 2nd harmonic from 3 into n78 | 2nd & 4th IMDs into n78 | Yes | - Harmonic problems was already covered in DC_3A_n78A. - 2nd &4th IMDs problems will be covered in DC_1A_n3A-n78A. |
| DC_3-7-8_n1-n78 | DC_3A_n1A | 2nd harmonic from B3 into n78 | 2nd & 4th IMDs into n78 | Yes | - Harmonic problems was already covered in DC_3A_n78A. - 2nd &4th IMDs problems will be covered in DC_3A_n1A-n78A. |
|  | DC_3A_n78A | - | 5th IMD into n1 | - | The MSD issue will be covered in 3DL_3A_n1A-78A. |
|  | DC_7A_n1A | - | 4th & 5th IMDs into n78 5th IMD into B8 | Yes | - These IMD problems were already covered in DC_7A_n1A-n78A with 2UL_DC_7A_n1A - 5th IMD issue will be covered in DC_7A-8A_n1A |
|  | DC_7A_n78A | - | 4th IMD into n1 3rd & 4th IMDs into B3 2nd & 5th IMDs into B8 | Yes | - Up to 7th harmonic order will be not any impact to n257 - 4th IMD problem will be covered in DC_7A_n1A-n78A 3rd & 4th IMDs issue will be covered in DC_3A-7A_n78A 2nd & 5th IMDs issue will be covered in DC_7A-8A_n78A |
|  | DC_8A_n1A | 2nd harmonic from B8 into B3 3rd harmonic from B8 into B7 4th harmonic from B8 into n78 | 3rd IMD into n78 | - | - The harmonic problem already covered in CA_3A-8A or CA_7A-8A or DC_8A_n78A - 3rd IMD issue will be covered in DC_8A_n1A-n78A |
|  | DC_8A_n78A | 2nd harmonic from B8 into B3 3rd harmonic from B8 into B7 | 3rd IMD into B3 2nd IMD into B7 | -Yes | - Harmonic problem already be solved in CA_3A_8A or CA_7A-8A - 3rd IMD already covered in DC_3A-8A_n78A - 2nd IMD issue will be covered in DC_7A-8A_n78A |
| DC_1-3-41_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 2nd IMD into B3 4th IMD into B41 | Yes | - Up to 7th harmonic order will be not any impact to n257 - The IMD problem will be covered in DC_1A-3A_n78A - The IMD problem will be covered in DC_1A-41A_n78A |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | 2nd harmonic from B1 into n78 | - | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_3A_n78A | 7th & 8th harmonics from n78 into n257 | 5th IMD into B1 | - | - Up to 7th harmonic order will be not any impact to n257 - 5th IMD problems will be covered in DC_1A-3A_n78A. |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 | - | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_41A_n78A | 7th & 8th harmonics from n78 into n257 | 4th IMD into B1 3rd & 4th IMDs into B3 | - | - No harmonic impact to n257 - 4th IMD issue will be covered in DC_1A-41A_n78A - 3rd & 4th IMDs issue will be covered in DC_3A-41A_n78A |
|  | DC_41A_n257A, DC_41A_n257G, DC_41A_n257H, DC_41A_n257I | - | - | - | No issue |
| DC_1-3-42_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 2nd IMD into B3 4th IMD into B42 | Yes | - Up to 7th harmonic order will be not any impact to n257 - The IMD problem will be covered in DC_1A-3A_n78A - The IMD problem will be covered in DC_1A-42A_n78A |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | 2nd harmonic from B1 into n78 | - | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_3A_n78A | 2nd harmonic from B3 into B42 7th & 8th harmonics from n78 into n257 | 5th IMD into B1 4th & 5th IMDs into B42 | - | - 2nd harmonic already covered in CA_3A-42A and Up to 7th harmonic order will be not any impact to n257 - 5th IMD issue will be covered in DC_1A-3A_n78A - These MSD issues are already covered in 3DL_DC_3A-42A_n78A |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 | - | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - | - | - | No issue |
| DC_1-41-42_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 4th IMD into B41 4th IMD into B42 | Yes | - Up to 7th harmonic order will be not any impact to n257 - The IMD problem will be covered in DC_1A-41A_n78A - The IMD problem will be covered in DC_1A-42A_n78A |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | 2nd harmonic from B1 into n78 | - | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_41A_n78A | 7th & 8th harmonics from n78 into n257 | 4th IMD into B1 | - | - No harmonic impact to n257 - 4th IMD issue will be covered in DC_1A-41A_n78A |
|  | DC_41A_n257A, DC_41A_n257G, DC_41A_n257H, DC_41A_n257I | - | - | - | No issue |
|  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - | - | - | No issue |
| DC_3-41-42_n78-n257 | DC_3A_n78A | 2nd harmonic from B3 into B42 7th & 8th harmonics from n78 into n257 | 4th & 5th IMDs into B42 | - | - 2nd harmonic already covered in CA_3A-42A and Up to 7th harmonic order will be not any impact to n257 - These MSD issues are already covered in 3DL_DC_3A-42A_n78A |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 | - | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_41A_n78A | 7th & 8th harmonics from n78 into n257 | 3rd & 4th IMDs into B3 | - | - No harmonic impact to n257 - 3rd & 4th IMDs issue will be covered in DC_3A-41A_n78A |
|  | DC_41A_n257A, DC_41A_n257G, DC_41A_n257H, DC_41A_n257I | - | - | - | No issue |
|  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - | - | - | No issue |
| DC_1-3-18_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 2nd IMD into B3 5th IMD into B18 | Yes | - Up to 7th harmonic order will be not any impact to n257 - The IMD problem will be covered in DC_1A-3A_n78A - The IMD problem will be covered in DC_1A-18A_n78A |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | 2nd harmonic from B1 into n78 | - | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_3A_n78A | 7th & 8th harmonics from n78 into n257 | 5th IMD into B1 | - | - Up to 7th harmonic order will be not any impact to n257 - 5th IMD issue will be covered in DC_1A-3A_n78A |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 | - | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_18A_n78A | 7th & 8th harmonics from n78 into n257 | 3rd IMD into B1 3rd IMD into B3 | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 3rd IMD issue will be covered in DC_1A-18A_n78A - 3rd IMD issue will be covered in DC_3A-18A_n78A |
|  | DC_18A_n257A, DC_18A_n257G, DC_18A_n257H, DC_18A_n257I | - | - | - | No issue |
| DC_1-18-42_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 5th IMD into B18 4th IMD into B42 | Yes | - Up to 7th harmonic order will be not any impact to n257 - The IMD problem will be covered in DC_1A-18A_n78A - The IMD problem will be covered in DC_1A-42A_n78A |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | 2nd harmonic from B1 into n78 | - | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_18A_n78A | 7th & 8th harmonics from n78 into n257 | 3rd IMD into B1 | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 3rd IMD issue will be covered in DC_1A-18A_n78A |
|  | DC_18A_n257A, DC_18A_n257G, DC_18A_n257H, DC_18A_n257I | - | - | - | No issue |
|  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - | - | - | No issue |
| DC_3-18-42_n78-n257 | DC_3A_n78A | 2nd harmonic from B3 into B42 7th & 8th harmonics from n78 into n257 | 4th & 5th IMDs into B42 | - | - 2nd harmonic already covered in CA_3A-42A and Up to 7th harmonic order will be not any impact to n257 - These IMDs issue will be covered in DC_3A-42A_n78A |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 | - | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_18A_n78A | 7th & 8th harmonics from n78 into n257 | 3rd IMD into B3 | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 3rd IMD issue will be covered in DC_3A-18A_n78A |
|  | DC_18A_n257A, DC_18A_n257G, DC_18A_n257H, DC_18A_n257I | - | - | - | No issue |
|  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - | - | - | No issue |
| DC_3-28-41_n78-n257 | DC_3A_n78A | 7th & 8th harmonics from n78 into n257 | - | - | - Up to 7th harmonic order will be not any impact to n257 |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 | - | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_28A_n78A | 7th & 8th harmonics from n78 into n257 | 3rd IMD into B3 2nd IMD into B41 | - | Up to 7th harmonic order will be not any impact to n257 - 3rd IMD issue will be covered in DC_3A-28A_n78A - 2nd IMD issue will be covered in DC_28A-41A_n78A |
|  | DC_28A_n257A, DC_28A_n257G, DC_28A_n257H, DC_28A_n257I | - | - | - | No issue |
|  | DC_41A_n78A | 7th & 8th harmonics from n78 into n257 | 3rd & 4th IMDs into B3 2nd & 5th IMDs into B28 | - | - No harmonic impact to n257 - 3rd & 4th IMDs issue will be covered in DC_3A-41A_n78A - 2nd & 5th IMDs issue will be covered in DC_28A-41A_n78A |
|  | DC_41A_n257A, DC_41A_n257G, DC_41A_n257H, DC_41A_n257I | - | - | - | No issue |
| DC_3-28-42_n78-n257 | DC_3A_n78A | 2nd harmonic from B3 into B42 7th & 8th harmonics from n78 into n257 | 4th & 5th IMDs into B42 | - | - 2nd harmonic already covered in CA_3A-42A and Up to 7th harmonic order will be not any impact to n257 - These IMDs issue will be covered in DC_3A-42A_n78A |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 | - | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_28A_n78A | 7th & 8th harmonics from n78 into n257 | 3rd IMD into B3 | - | Up to 7th harmonic order will be not any impact to n257 - 3rd IMD issue will be covered in DC_3A-28A_n78A |
|  | DC_28A_n257A, DC_28A_n257G, DC_28A_n257H, DC_28A_n257I | - | - | - | No issue |
|  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - | - | - | No issue |
| DC_28-41-42_n78-n257 | DC_28A_n78A | 7th & 8th harmonics from n78 into n257 | 2nd IMD into B41 | - | Up to 7th harmonic order will be not any impact to n257 - 3rd IMD issue will be covered in DC_28A-41A_n78A |
|  | DC_28A_n257A, DC_28A_n257G, DC_28A_n257H, DC_28A_n257I | - | - | - | No issue |
|  | DC_41A_n78A | 7th & 8th harmonics from n78 into n257 | 2nd & 5th IMDs into B28 | - | - No harmonic impact to n257 - 2nd & 5th IMDs issue will be covered in DC_28A-41A_n78A |
|  | DC_41A_n257A, DC_41A_n257G, DC_41A_n257H, DC_41A_n257I | - | - | - | No issue |
|  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - | - | - | No issue |
| DC_1-3-28_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 2nd IMD into B3 5th IMD into B28 | Yes | - Up to 7th harmonic order will be not any impact to n257 - The IMD problem will be covered in DC_1A-3A_n78A - The IMD problem will be covered in DC_1A-28A_n78A |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | 2nd harmonic from B1 into n78 | - | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_3A_n78A | 7th & 8th harmonics from n78 into n257 | 5th IMD into B1 | - | - Up to 7th harmonic order will be not any impact to n257 - The IMD issue will be covered in DC_1A-3A_n78A |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 | - | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_28A_n78A | 3rd harmonic from B28 into B1 7th & 8th harmonics from n78 into n257 | 3rd IMD into B1 3rd IMD into B3 | - | - 3rd harmonic already covered in CA_1A-28A and Up to 7th harmonic order will be not any impact to n257 - 3rd IMD issue will be covered in DC_1A-28A_n78A - 3rd IMD issue will be covered in DC_3A-28A_n78A |
|  | DC_28A_n257A, DC_28A_n257G, DC_28A_n257H, DC_28A_n257I | - | - | - | No issue |
| DC_1-28-42_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 5th IMD into B28 4th IMD into B42 | - | - Up to 7th harmonic order will be not any impact to n257 - The IMD problem will be covered in DC_1A-28A_n78A - The IMD problem was not impact to B42 due to synchronous TDD operation |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | 2nd harmonic from B1 into n78 | - | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_28A_n78A | 3rd harmonic from B28 into B1 7th & 8th harmonics from n78 into n257 | 3rd IMD into B1 | - | - 3rd harmonic already covered in CA_1A-28A and Up to 7th harmonic order will be not any impact to n257 - 3rd IMD issue will be covered in DC_1A-28A_n78A |
|  | DC_28A_n257A, DC_28A_n257G, DC_28A_n257H, DC_28A_n257I | - | - | - | No issue |
|  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - | - | - | No issue |
| DC_1-8-11_n77-n257 | DC_1A_n77A | 7th & 8th Harmonics from n77 into n257 | 5th IMD into B8 2nd & 5th IMDs into B11 | Yes | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 5th IMDs issue will be covered in DC_1A-8A_n77A - These IMDs issue will be covered in DC_1A-11A_n77A |
|  | DC_1A_n257A | 2nd harmonic from B1 into n77 | - | - | - Harmonic problem already discussed and will be solved in DC_1A_n77A |
|  | DC_8A_n77A | 7th & 8th Harmonics from n77 into n257 | 3rd IMD into B1 3rd & 4th IMDs into B11 | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - The MSD issue will be covered in 3DL_DC_1A-8A_n77A - The MSD issue will be covered in 3DL_DC_8A-11A_n77A |
|  | DC_8A_n257A | 4th harmonic from B8 into n77 | - | - | - Harmonic problem will be solved in DC_8A_n77A |
|  | DC_11A_n77A | 7th & 8th harmonics from n77 into n257 | 2nd & 5th IMDs into B1 3rd & 4th IMDs into B8 | - | No harmonic problem to n257 - These IMDs issue will be covered in DC_1A-11A_n77A - These IMDs issue will be covered in DC_8A-11A_n77A |
|  | DC_11A_n257A | - | - | - | No issue |

Table 5.3-4 summarizes the EN-DC band combinations with self-interference problems for 6DL/2UL DC operation.

Table 5.3-4: Summary of Self-interference analysis for LTE 4 bands & NR 2 bands DL and 2 bands UL EN-DC operation

| Downlink band configuration | Uplink DC Configuration | Harmonic relation issues |  | intermodulation to own rx band | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- | --- |
| DC_1-3-5-7_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 |  | 2nd IMD into B3 4th IMD into B7 5th IMD into B5 | Yes | - Up to 7th harmonic order will be not any impact to n257 - These IMDs problem will be covered in DC_1A-3A_n78A, DC_1A-7A_n78A and DC_1A-5A_n78A - Proximity also follow LTE_CA_1A-3A |
|  | DC_1A_n257A | 2nd harmonic from B1 into n78 |  | - | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_3A_n78A | 7th & 8th harmonics from n78 into n257 |  | 5th IMD into B1 | - | - Up to 7th harmonic order will be not any impact to n257 - 5th IMD problems will be covered in DC_1A-3A_n78A. |
|  | DC_3A_n257A | 2nd harmonic from B3 into n78 |  | - | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_5A_n78A | 7th & 8th harmonics from n78 into n257 |  | 3rd IMD into B1 3rd IMD into B3 2nd IMD into B7 | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 3rd IMD problems will be covered in DC_1A-5A_n78A, DC_3A-5A_n78A and DC_5A-7A_n78A. |
|  | DC_5A_n257A | 4th harmonic from B5 into n78 |  | - | - | - Harmonic problem will be solved in DC_5A_n78A |
|  | DC_7A_n78A | 7th harmonic from B7 into n257 |  | 3rd & 4th IMDs into B3 4th IMD into B1 2nd & 5th IMDs into B5 | - | - Up to 7th harmonic order will be not any impact to n257 - The IMD problem will be covered in DC_3A-7A_n78A, DC_1A-7A_n78A and DC_5A-7A_n78A |
|  | DC_7A_n257A | - |  | - | - | No issues |
| DC_1-3-7-28_n28-n78 | DC_1A_n28A | 5th harmonic from n28 into n78 |  | 2nd IMD into B7 3rd IMD into n78 | Yes | All desense problems are covered in lower fallback mode. |
|  | DC_3A_n28A DC_3C_n28A | 2nd harmonic from B3 into n78 3rd harmonic from n28 into B1 5th harmonic from n28 into n78 |  | 4th IMD into B1 3rd IMD into B7 5th IMD into n78 | - | All desense problems are covered in lower fallback mode. |
|  | DC_7A_n28A DC_7C_n28A | 3rd harmonic from n28 into B1 5th harmonic from n28 into n78 |  | 2nd IMD into B3 2nd,4th IMDs into n78 | - | All desense problems are covered in lower fallback mode. |
|  | DC_28A_n28A | 3rd harmonic from n28 into B1 |  | 3rd & 5th IMDs into B1 5th IMD into n78 | - | All desense problems are covered in lower fallback mode. |
|  | DC_1A_n78A | - |  | 2nd IMD into B3 4th IMD into B7 5th IMD into B28 or n28 | Yes | All desense problems are covered in lower fallback mode. |
|  | DC_3A_n78A DC_3C_n78A | - |  | 5th IMD into B1 | - | All desense problems are covered in lower fallback mode. |
|  | DC_7A_n78A DC_7C_n78A | - |  | 4th IMD into B1 3rd & 4th IMDs into B3 2nd & 5th IMDs into B28 or n28 | - | All desense problems are covered in lower fallback mode. |
|  | DC_28A_n78A | 3rd harmonic from B28 into B1 |  | 3rd IMD into B1 3rd IMD into B3 2nd IMD into B7 5th IMD into n28 | - | All desense problems are covered in lower fallback mode. |
| DC_1-3-7-28_n5-n78 | DC_1A_n5A | 4th harmonic from n5 into n78 | 3rd IMD into n78 |  | - | - 4th harmonic already covered in Table 7.3B.2.3.1-1 in TS38.101-3 - The 3rd IMD issue will be covered in DC_1_n5A-n78A |
|  | DC_3A_n5A, DC_3C_n5A | 2nd harmonic from B3 into n78 | 2nd & 3rd IMDs into B7 4th IMD into B28 3rd & 5th IMDs into n78 |  | - | - 2nd harmonic problem already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - The IMD issue will be covered in DC_3A-7A_n5A - The IMD issue will be covered in DC_3A-28A_n5A These IMD issue will be covered in DC_3A_n5A-n78A |
|  | DC_7A_n5A, DC_7C_n5A | - | 3rd & 5th IMDs into B28 2nd & 4th IMDs into n78 |  | - | These IMDs problem will be covered in DC_7A-28A_n5A These IMDs problem will be covered in DC_7A_n5A-n78A |
|  | DC_28A_n5A | 3rd harmonic from B28 into B1 4th harmonic from n5 into n78 5th harmonic from B28 into n78 | 5th IMD into B1 4th IMD into B3 5th IMD into B7 |  |  | - These harmonic problems already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - The 5th IMD will be covered in DC_1A-28A_n5A - The 4th IMD will be covered in DC_3A-28A_n5A - The 5th IMD will be covered in DC_7A-28A_n5A |
|  | DC_1A_n78A | - | 2nd IMD into B3 5th IMD into n5 4th IMD into B7 5th IMD into B28 |  | Yes | All desense problems are covered in lower fallback mode. |
|  | DC_3A_n78A, DC_3C_n78A | - | 5th IMD into B1 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_7A_n78A, DC_7C_n78A | - | 4th IMD into B1 3rd & 4th IMDs into B3 2nd & 5th IMDs into n5 2nd & 5th IMDs into B28 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_28A_n78A | 3rd harmonic from B28 into B1 | 3rd IMD into B1 3rd IMD into B3 5th IMD into n5 2nd IMD into B7 |  | - | All desense problems are covered in lower fallback mode. |
| DC_1-3-41-42_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 2nd IMD into B3 4th IMD into B41 4th IMD into B42 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | 2nd harmonic from B1 into n78 | - |  | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_3A_n78A | 2nd harmonic from B3 into B42 7th & 8th harmonics from n78 into n257 | 5th IMD into B1 4th & 5th IMDs into B42 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 | - |  | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_41A_n78A | 7th & 8th harmonics from n78 into n257 | 4th IMD into B1 3rd & 4th IMDs into B3 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_41A_n257A, DC_41A_n257G, DC_41A_n257H, DC_41A_n257I | - | - |  | - | No issue |
|  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - | - |  | - | No issue |
| DC_1-3-18-42_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 2nd IMD into B3 5th IMD into B18 4th IMD into B42 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | 2nd harmonic from B1 into n78 | - |  | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_3A_n78A | 2nd harmonic from B3 into B42 7th & 8th harmonics from n78 into n257 | 5th IMD into B1 4th & 5th IMDs into B42 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 | - |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_18A_n78A | 7th & 8th harmonics from n78 into n257 | 3rd IMD into B1 3rd IMD into B3 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_18A_n257A, DC_18A_n257G, DC_18A_n257H, DC_18A_n257I | - | - |  | - | No issue |
|  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - | - |  | - | No issue |
| DC_1-3-28-42_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 2nd IMD into B3 5th IMD into B28 4th IMD into B42 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | 2nd harmonic from B1 into n78 | - |  | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_3A_n78A | 2nd harmonic from B3 into B42 7th & 8th harmonics from n78 into n257 | 5th IMD into B1 4th & 5th IMDs into B42 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 | - |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_28A_n78A | 3rd harmonic from B28 into B1 7th & 8th harmonics from n78 into n257 | 3rd IMD into B1 3rd IMD into B3 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_28A_n257A, DC_28A_n257G, DC_28A_n257H, DC_28A_n257I | - | - |  | - | No issue |
|  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - | - |  | - | No issue |
| DC_3-28-41-42_n78-n257 | DC_3A_n78A | 2nd harmonic from B3 into B42 7th & 8th harmonics from n78 into n257 | 4th & 5th IMDs into B42 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 | - |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_28A_n78A | 7th & 8th harmonics from n78 into n257 | 3rd IMD into B3 2nd IMD into B41 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_28A_n257A, DC_28A_n257G, DC_28A_n257H, DC_28A_n257I | - | - |  | - | No issue |
|  | DC_41A_n78A | 7th & 8th harmonics from n78 into n257 | 3rd & 4th IMDs into B3 2nd & 5th IMDs into B28 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_41A_n257A, DC_41A_n257G, DC_41A_n257H, DC_41A_n257I | - | - |  | - | No issue |
|  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - | - |  | - | No issue |

Based on these assumptions, we proposed the MSD test configuration and required MSD levels by self-interference as below.

Table 5.3-5: Proposed MSD test configuration for 3DL/2UL DC UE

| DC bands | UL DC | IMD |  | UL Fc (MHz) |  | UL BW (MHz) |  | UL RB # |  | DL Fc (MHz) |  | DL BW (MHz) |  | CF (dB) |  | MSD (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n3A-n78A | 1 | IMD2 | ¦fB1 +fn3¦ | 1950 | 5 |  |  | 25 |  | 2140 |  | 5 |  | 2.1 |  | N/A |  |
|  | n3 |  |  | 1750 |  | 5 |  | 25 |  | 1845 |  | 5 |  |  |  |  |  |
|  | n78 |  |  | 3700 |  | 10 |  | 52 |  | 3700 |  | 10 |  |  |  | 28.4 |  |
|  | 1 | IMD4 | ¦3*fn3 -fB1¦ | 1950 |  | 5 |  | 25 |  | 2140 |  | 5 |  | 1.2 |  | N/A |  |
|  | n3 |  |  | 1770 |  | 5 |  | 25 |  | 1865 |  | 5 |  |  |  |  |  |
|  | n78 |  |  | 3360 |  | 10 |  | 52 |  | 3360 |  | 10 |  |  |  | 11.2 |  |
|  | 1 | IMD2 | ¦fn78 -fB1¦ | 1950 |  | 5 |  | 25 |  | 2140 |  | 5 |  | 1.9 |  | N/A |  |
|  | n78 |  |  | 3780 |  | 10 |  | 52 |  | 3780 |  | 10 |  |  |  |  |  |
|  | n3 |  |  | 1735 |  | 5 |  | 25 |  | 1830 |  | 5 |  |  |  | 27.9 |  |
| DC_1A-7A_n78A | 1 | IMD4 | ¦fB7 -3*fB1¦ | 1970 |  | 5 |  | 25 |  | 2160 |  | 5 |  | 1.2 |  | N/A |  |
|  | 7 |  |  | 2520 |  | 5 |  | 25 |  | 2640 |  | 5 |  |  |  |  |  |
|  | n78 |  |  | 3390 |  | 10 |  | 52 |  | 3390 |  | 10 |  |  |  | 10.1 |  |
|  | 1 | IMD5 | ¦3*fB7 -2*fB1¦ | 1970 |  | 5 |  | 25 |  | 2160 |  | 5 |  | 0.3 |  | N/A |  |
|  | 7 |  |  | 2520 |  | 5 |  | 25 |  | 2640 |  | 5 |  |  |  |  |  |
|  | n78 |  |  | 3620 |  | 10 |  | 52 |  | 3620 |  | 10 |  |  |  | 3.8 |  |
|  | 1 | IMD4 | ¦fn78 -3*fB1¦ | 1977.5 |  | 5 |  | 25 |  | 2167.5 |  | 5 |  | 0.9 |  | N/A |  |
|  | n78 |  |  | 3305 |  | 10 |  | 52 |  | 3305 |  | 10 |  |  |  |  |  |
|  | 7 |  |  | 2507.5 |  | 5 |  | 25 |  | 2627.5 |  | 5 |  |  |  | 9.1 |  |
| DC_3A_n1A-n78A and DC_3A_n1A-n77A | n1 | IMD2 | ¦fn1 +fB3¦ | 1950 |  | 5 |  | 25 |  | 2140 |  | 5 |  | 2.1 |  | N/A |  |
|  | 3 |  |  | 1750 |  | 5 |  | 25 |  | 1845 |  | 5 |  |  |  |  |  |
|  | n78/n77 |  |  | 3700 |  | 10 |  | 52 |  | 3700 |  | 10 |  |  |  | 28.4 |  |
|  | n1 | IMD4 | ¦3*fB3 -fn1¦ | 1950 |  | 5 |  | 25 |  | 2140 |  | 5 |  | 1.2 |  | N/A |  |
|  | 3 |  |  | 1770 |  | 5 |  | 25 |  | 1865 |  | 5 |  |  |  |  |  |
|  | n78/n77 |  |  | 3360 |  | 10 |  | 52 |  | 3360 |  | 10 |  |  |  | 11.2 |  |
|  | 3 | IMD2 | ¦fn77 –fB3¦ | 1775 |  | 5 |  | 25 |  | 1870 |  | 5 |  | 2.4 |  | N/A |  |
|  | n77 |  |  | 3915 |  | 10 |  | 52 |  | 3915 |  | 10 |  |  |  |  |  |
|  | n1 |  |  | 1950 |  | 5 |  | 25 |  | 2140 |  | 5 |  |  |  | 31.0 |  |
|  | 3 | IMD5 | ¦2*fn78 -3*fB3¦ | 1770 |  | 5 |  | 25 |  | 1865 |  | 5 |  | 0.3 |  | N/A |  |
|  | n78/n77 |  |  | 3720 |  | 10 |  | 52 |  | 3720 |  | 10 |  |  |  |  |  |
|  | n1 |  |  | 1940 |  | 5 |  | 25 |  | 2130 |  | 5 |  |  |  | 3.5 |  |
| DC_3A_n7A-n78A | 3 | IMD3 | ¦2*fn7-fB3¦ | 1730 |  | 5 |  | 25 |  | 1825 |  | 5 |  | 1.5 |  | N/A |  |
|  | n7 |  |  | 2560 |  | 5 |  | 25 |  | 2680 |  | 5 |  |  |  |  |  |
|  | n78 |  |  | 3390 |  | 10 |  | 52 |  | 3390 |  | 10 |  |  |  | 16.1 |  |
| DC_3A_n20A-n78A | 3 | IMD3 | ¦fB3+2*fn20¦ | 1730 |  | 5 |  | 25 |  | 1825 |  | 5 |  | 1.5 |  | N/A |  |
|  | n20 |  |  | 845 |  | 5 |  | 25 |  | 804 |  | 5 |  |  |  |  |  |
|  | n78 |  |  | 3420 |  | 10 |  | 52 |  | 3420 |  | 10 |  |  |  | 16.1 |  |
|  | 3 | IMD5 | ¦3*fB3 -2*fn20¦ | 1730 |  | 5 |  | 25 |  | 1825 |  | 5 |  | 0.4 |  | N/A |  |
|  | n20 |  |  | 845 |  | 5 |  | 25 |  | 804 |  | 5 |  |  |  |  |  |
|  | n78 |  |  | 3500 |  | 10 |  | 52 |  | 3500 |  | 10 |  |  |  | 4.5 |  |
| DC_3A_n28A-n78A | 3 | IMD5 | ¦3*fB3 -2*fn28¦ | 1750 |  | 5 |  | 25 |  | 1845 |  | 5 |  | 0.8 |  | N/A |  |
|  | n28 |  |  | 743 |  | 5 |  | 25 |  | 798 |  | 5 |  |  |  |  |  |
|  | n78 |  |  | 3764 |  | 10 |  | 52 |  | 3764 |  | 10 |  |  |  | 4.5 |  |
| DC_7A-n28A-n78A | 7 | IMD2 | ¦fB7 +fB28¦ | 2565 |  | 5 |  | 25 |  | 2685 |  | 5 |  | 2.1 |  | N/A |  |
|  | n28 |  |  | 745 |  | 5 |  | 25 |  | 800 |  | 5 |  |  |  |  |  |
|  | n78 |  |  | 3310 |  | 10 |  | 52 |  | 3310 |  | 10 |  |  |  | 29.7 |  |
|  | 7 | IMD4 | ¦2*fB7 -2*fB28¦ | 2565 |  | 5 |  | 25 |  | 2685 |  | 5 |  | 1.1 |  | N/A |  |
|  | n28 |  |  | 745 |  | 5 |  | 25 |  | 800 |  | 5 |  |  |  |  |  |
|  | n78 |  |  | 3640 |  | 10 |  | 52 |  | 3640 |  | 10 |  |  |  | 10.3 |  |
|  | 7 | IMD2 | ¦fB78 -fB7¦ | 2565 |  | 5 |  | 25 |  | 2685 |  | 5 |  | 2.0 |  | N/A |  |
|  | n78 |  |  | 3365 |  | 10 |  | 52 |  | 3365 |  | 10 |  |  |  |  |  |
|  | n28 |  |  | 745 |  | 5 |  | 25 |  | 800 |  | 5 |  |  |  | 28.8 |  |
|  | 7 | IMD5 | ¦2*fB78 -3*fB7¦ | 2520 |  | 5 |  | 25 |  | 2640 |  | 5 |  | 0.7 |  | N/A |  |
|  | n78 |  |  | 3387 |  | 10 |  | 52 |  | 3387 |  | 10 |  |  |  |  |  |
|  | n28 |  |  | 731 |  | 5 |  | 25 |  | 786 |  | 5 |  |  |  | 4.0 |  |
| DC_28A-n8A-n78A | 28 | IMD4 | ¦3*fB8 +fB28¦ | 728 |  | 5 |  | 25 |  | 783 |  | 5 |  | 1.1 |  | N/A |  |
|  | n8 |  |  | 910 |  | 5 |  | 25 |  | 955 |  | 5 |  |  |  |  |  |
|  | n78 |  |  | 3458 |  | 10 |  | 52 |  | 3458 |  | 10 |  |  |  | 9.1 |  |
|  | 28 | IMD5 | ¦fB8 +4*fB28¦ | 713 |  | 5 |  | 25 |  | 768 |  | 5 |  | 0.7 |  | N/A |  |
|  | n8 |  |  | 910 |  | 5 |  | 25 |  | 955 |  | 5 |  |  |  |  |  |
|  | n78 |  |  | 3762 |  | 10 |  | 52 |  | 3762 |  | 10 |  |  |  | 3.7 |  |
|  | 28 | IMD5 | ¦fB78 -4*fB28¦ | 713 |  | 5 |  | 25 |  | 768 |  | 5 |  | 0.5 |  | N/A |  |
|  | n78 |  |  | 3787 |  | 10 |  | 52 |  | 3787 |  | 10 |  |  |  |  |  |
|  | n8 |  |  | 890 |  | 5 |  | 25 |  | 935 |  | 5 |  |  |  | 4.3 |  |
| DC_7A_n1A-n78A | n1 | IMD4 | ¦fB7 -3*fn1¦ | 1970 |  | 5 |  | 25 |  | 2160 |  | 5 |  | 1.2 |  | N/A |  |
|  | 7 |  |  | 2520 |  | 5 |  | 25 |  | 2640 |  | 5 |  |  |  |  |  |
|  | n78 |  |  | 3390 |  | 10 |  | 52 |  | 3390 |  | 10 |  |  |  | 10.1 |  |
|  | n1 | IMD5 | ¦3*fB7 -2*fn1¦ | 1970 |  | 5 |  | 25 |  | 2160 |  | 5 |  | 0.3 |  | N/A |  |
|  | 7 |  |  | 2520 |  | 5 |  | 25 |  | 2640 |  | 5 |  |  |  |  |  |
|  | n78 |  |  | 3620 |  | 10 |  | 52 |  | 3620 |  | 10 |  |  |  | 3.8 |  |
|  | 7 | IMD4 | ¦2*fn78 -2*fB7¦ | 2530 |  | 5 |  | 25 |  | 2650 |  | 5 |  | 0.9 |  | N/A |  |
|  | n78 |  |  | 3610 |  | 10 |  | 52 |  | 3610 |  | 10 |  |  |  |  |  |
|  | n1 |  |  | 1970 |  | 5 |  | 25 |  | 2160 |  | 5 |  |  |  | 9.0 |  |
| DC_7A_n3A-n78A | 7 | IMD3 | ¦2*fB7-fn3¦ | 2560 |  | 5 |  | 25 |  | 2680 |  | 5 |  | 1.5 |  | N/A |  |
|  | n3 |  |  | 1730 |  | 5 |  | 25 |  | 1825 |  | 5 |  |  |  |  |  |
|  | n78 |  |  | 3390 |  | 10 |  | 52 |  | 3390 |  | 10 |  |  |  | 16.1 |  |
|  | 7 | IMD3 | ¦2*fB7 -fn78¦ | 2565 |  | 5 |  | 25 |  | 2685 |  | 5 |  | 1.4 |  | N/A |  |
|  | n78 |  |  | 3310 |  | 10 |  | 52 |  | 3310 |  | 10 |  |  |  |  |  |
|  | n3 |  |  | 1725 |  | 5 |  | 25 |  | 1820 |  | 5 |  |  |  | 15.6 |  |
|  | 7 | IMD4 | ¦2*fn78 -2*fB7¦ | 2565 |  | 5 |  | 25 |  | 2685 |  | 5 |  | 0.8 |  | N/A |  |
|  | n78 |  |  | 3480 |  | 10 |  | 52 |  | 3480 |  | 10 |  |  |  |  |  |
|  | n3 |  |  | 1735 |  | 5 |  | 25 |  | 1830 |  | 5 |  |  |  | 9.2 |  |
| DC_20A_n1A-n78A | 20 | IMD3 | ¦fn1 +2*fB20¦ | 845 |  | 5 |  | 25 |  | 804 |  | 5 |  | 1.5 |  | N/A |  |
|  | n1 |  |  | 1940 |  | 5 |  | 25 |  | 2130 |  | 5 |  |  |  |  |  |
|  | n78 |  |  | 3630 |  | 10 |  | 52 |  | 3630 |  | 10 |  |  |  | 16.0 |  |
|  | 20 | IMD3 | ¦fn78 -2*fB20¦ | 835 |  | 5 |  | 25 |  | 794 |  | 5 |  | 1.3 |  | N/A |  |
|  | n78 |  |  | 3790 |  | 10 |  | 52 |  | 3790 |  | 10 |  |  |  |  |  |
|  | n1 |  |  | 1930 |  | 5 |  | 25 |  | 2120 |  | 5 |  |  |  | 15.3 |  |
| DC_20A_n3A-n78A | 20 | IMD3 | ¦2*fB20+fn3¦ | 845 |  | 5 |  | 25 |  | 804 |  | 5 |  | 1.5 |  | N/A |  |
|  | n3 |  |  | 1730 |  | 5 |  | 25 |  | 1825 |  | 5 |  |  |  |  |  |
|  | n78 |  |  | 3420 |  | 10 |  | 52 |  | 3420 |  | 10 |  |  |  | 16.1 |  |
|  | 20 | IMD5 | ¦2*fB20 -3*fn3¦ | 845 |  | 5 |  | 25 |  | 804 |  | 5 |  | 0.4 |  | N/A |  |
|  | n3 |  |  | 1730 |  | 5 |  | 25 |  | 1825 |  | 5 |  |  |  |  |  |
|  | n78 |  |  | 3500 |  | 10 |  | 52 |  | 3500 |  | 10 |  |  |  | 4.5 |  |
|  | 20 | IMD3 | ¦2*fB20-fn78¦ | 845 |  | 5 |  | 25 |  | 804 |  | 5 |  | 1.3 |  | N/A |  |
|  | n78 |  |  | 3550 |  | 10 |  | 52 |  | 3550 |  | 10 |  |  |  |  |  |
|  | n3 |  |  | 1765 |  | 5 |  | 25 |  | 1860 |  | 5 |  |  |  | 15.7 |  |
| DC_1A_n40A-n78A | 1 | IMD4 | ¦3*fB1-fn40¦ | 1930 |  |  | 5 |  | 25 |  | 2120 |  | 5 | 1.0 | N/A |  |
|  | n40 |  |  | 2340 |  |  | 5 |  | 25 |  | 2340 |  | 5 |  |  |  |
|  | n78 |  |  | 3450 |  |  | 10 |  | 50 |  | 3450 |  | 10 |  | 9.8 |  |
|  | 1 | IMD5 | ¦2*fB1 -3*fn40¦ | 1925 |  |  | 5 |  | 25 |  | 2115 |  | 5 | 0.4 | N/A |  |
|  | n40 |  |  | 2390 |  |  | 5 |  | 25 |  | 2390 |  | 5 |  |  |  |
|  | n78 |  |  | 3320 |  |  | 10 |  | 50 |  | 3320 |  | 10 |  | 4.2 |  |
|  | 1 | IMD4 | ¦3*fB1 -fn78¦ | 1960 |  |  | 5 |  | 25 |  | 2150 |  | 5 | 0.8 | N/A |  |
|  | n78 |  |  | 3520 |  |  | 10 |  | 50 |  | 3520 |  | 10 |  |  |  |
|  | n40 |  |  | 2360 |  |  | 5 |  | 25 |  | 2360 |  | 5 |  | 10.6 |  |
| DC_3A_n1A-n79A | 3 | IMD5 | ¦4*fB3-fn1¦ | 1720 |  |  | 5 |  | 25 |  | 1815 |  | 5 | 0.5 | N/A |  |
|  | n1 |  |  | 1930 |  |  | 5 |  | 25 |  | 2120 |  | 5 |  |  |  |
|  | n79 |  |  | 4950 |  |  | 40 |  | 216 |  | 4950 |  | 40 |  | 4.7 |  |
|  | 3 | IMD5 | ¦4*fB3 -fn79¦ | 1750 |  |  | 5 |  | 25 |  | 1845 |  | 5 | 0.5 | N/A |  |
|  | n79 |  |  | 4860 |  |  | 40 |  | 216 |  | 4860 |  | 10 |  |  |  |
|  | n1 |  |  | 1950 |  |  | 5 |  | 25 |  | 2140 |  | 5 |  | 3.6 |  |
| DC_3A_n40A-n78A | 3 | IMD5 | ¦2*fB3-3*fn40¦ | 1730 |  |  | 5 |  | 25 |  | 1825 |  | 5 | 0.5 | N/A |  |
|  | n40 |  |  | 2360 |  |  | 5 |  | 25 |  | 2360 |  | 5 |  |  |  |
|  | n78 |  |  | 3620 |  |  | 10 |  | 50 |  | 3620 |  | 10 |  | 4.8 |  |
|  | 3 | IMD5 | ¦3*fB3 -2*fn78¦ | 1720 |  |  | 5 |  | 25 |  | 1815 |  | 5 | 0.5 | N/A |  |
|  | n78 |  |  | 3760 |  |  | 10 |  | 50 |  | 3760 |  | 10 |  |  |  |
|  | n40 |  |  | 2360 |  |  | 5 |  | 25 |  | 2360 |  | 5 |  | 4.4 |  |
| DC_66A_n25A-n41A | 66 | IMD4 | ¦2*fB66 -2*fn41¦ | 1715 |  |  | 5 |  | 25 |  | 2115 |  | 5 | 1.1 | N/A |  |
|  | n41 |  |  | 2685 |  |  | 10 |  | 50 |  | 2685 |  | 10 |  |  |  |
|  | n25 |  |  | 1860 |  |  | 5 |  | 25 |  | 1940 |  | 5 |  | 11.0 |  |

6	EN-DC band combinations of LTE 1 band DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part

6.1	DC_3A_n1A-n77A

6.1.1	Operating bands for DC

Table 6.1.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A_n1A-n77A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.1.2	Channel bandwidths per operating band for DC

Table 6.1.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A_n1A-n77A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 140 |
|  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.1.3	Co-existence studies

The 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products for the UE coexistence analysis of DC_3A_n1A-n77A with dual uplink on DC_3A_n1A and DC_3A_n77A were calculated and listed in Table 6.1.3-1 and 6.1.3-2 respectively.

Table 6.1.3-1: Harmonic and IMD analysis for DC_3_n1-n77 with UL on DC_3_n1

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 5760 | 5940 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 135 | 270 | 3630 | 3765 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1440 | 1650 | 2055 | 2250 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5340 | 5550 | 5550 | 5745 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3150 | 3435 | 3975 | 4230 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7050 | 7335 | 7470 | 7725 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 540 | 270 | 7260 | 7530 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6210 | 5895 | 5220 | 4860 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9390 | 9705 | 8760 | 9120 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2520 | 2190 | 1515 | 1170 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9180 | 9510 | 8970 | 9315 |

Table 6.1.3-2: Harmonic and IMD analysis for DC_3_n1-n77 with UL on DC_3_n77

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 9900 | 12600 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1515 | 2490 | 5010 | 5985 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 780 | 270 | 4815 | 6690 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6720 | 7770 | 8310 | 10185 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 930 | 2055 | 8115 | 10890 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8430 | 9555 | 11610 | 14385 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 4980 | 3030 | 10020 | 11970 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 15090 | 11415 | 3840 | 2640 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14910 | 18585 | 10140 | 11340 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 9180 | 6330 | 1245 | 3270 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13320 | 16170 | 11730 | 13755 |

The coexistence studies of Band 3 + Band n1 and Band 3 + Band n77 will be captured in the constituent fallback modes. The summary of self-interference issues for the 3rd own receive band are listed below.

Based on Table 6.1.3-1 and Table 6.1.3-2:

-	2nd and 4th order IMD generated by dual uplink of Band 3 + Band n1 may also fall into own Rx of band n77

-	2nd and 5th order IMD generated by dual uplink of Band 3 + Band n77 may also fall into own Rx of band n1

6.1.4	∆TIB and ∆RIB values

For DC_3A_n1A-n77, the TIB,c and RIB values are given in the tables below.

Table 6.1.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A_n1A-n77A | 3 | 0.6 |
|  | n1 | 0.6 |
|  | n77 | 0.8 |
| NOTE 1: HTF is assumed for Band 3 and Band n1. |  |  |

Table 6.1.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A_n1A-n77A | 3 | 0.2 |
|  | n1 | 0.2 |
|  | n77 | 0.5 |
| NOTE 1: HTF is assumed for Band 3 and Band n1. |  |  |

6.1.5	MSD

Table 6.1.5-1 lists the MSD required for the dual connectivity configuration.

Table 6.1.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_3A_n1A-n77A | 3 | 1750 | 5 | 25 | 1845 | N/A | FDD | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A |  | N/A |
|  | n77 | 3700 | 10 | 52 | 3700 | 28.4 | TDD | IMD2 |
|  | 3 | 1770 | 5 | 25 | 1865 | N/A | FDD | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A |  | N/A |
|  | n77 | 3360 | 10 | 52 | 3360 | 11.2 | TDD | IMD4 |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A | FDD | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | 31.0 |  | IMD2 |
|  | n77 | 3915 | 10 | 52 | 3915 | N/A | TDD | N/A |
|  | 3 | 1770 | 5 | 25 | 1865 | N/A | FDD | N/A |
|  | n1 | 1940 | 5 | 25 | 2130 | 3.5 |  | IMD5 |
|  | n77 | 3720 | 10 | 52 | 3720 | N/A | TDD | N/A |

6.2	DC_1A-n78A-n257

6.2.1	Operating bands for DC

Table 6.2.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

6.2.2	Channel bandwidths per operating band for DC

Table 6.2.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A_n78A-n257D | DC_1A_n78A DC_1A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 520 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A_n78A-n257E | DC_1A_n78A DC_1A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 720 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A_n78A-n257F | DC_1A_n78A DC_1A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 920 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A_n78A-n257G | DC_1A_n78A DC_1A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 320 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A_n78A-n257H | DC_1A_n78A DC_1A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 420 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A_n78A-n257I | DC_1A_n78A DC_1A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 520 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A_n78A-n257J | DC_1A_n78A DC_1A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 620 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A_n78A-n257K | DC_1A_n78A DC_1A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 720 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A_n78A-n257L | DC_1A_n78A DC_1A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 820 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A_n78A-n257M | DC_1A_n78A DC_1A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 920 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.2.3	Co-existence studies

Based on co-existence studies of DC_1A-n78A and DC_1A_n257A specified in TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

- DC_1A_n78A: No harmonic problems by 7th & 8th order between FR1 and FR2

- DC_1A_n257A: Harmonic problem already specified in TR 37.863-01-01

6.2.4	∆TIB and ∆RIB values

For DC_1A_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 6.2.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A_n78-n257 | 1 | 0.3 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 6.2.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A_n78-n257 | 1 | 0 |
|  | n78 | 0.5 |
|  | n257 | 0 |

6.2.5	MSD

As mentioned in 6.2.3, there is no need to specify additional MSD requirement for this UL DC configuration.

6.3	DC_3-n78-n257

6.3.1	Operating bands for DC

Table 6.3.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n78-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

6.3.2	Channel bandwidths per operating band for DC

Table 6.3.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_3A_n78A-n257D | DC_3A_n78A DC_3A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 520 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A_n78A-n257E | DC_3A_n78A DC_3A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 720 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A_n78A-n257F | DC_3A_n78A DC_3A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 920 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A_n78A-n257G | DC_3A_n78A DC_3A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 320 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A_n78A-n257H | DC_3A_n78A DC_3A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 420 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A_n78A-n257I | DC_3A_n78A DC_3A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 520 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A_n78A-n257J | DC_3A_n78A DC_3A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 620 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A_n78A-n257K | DC_3A_n78A DC_3A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 720 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A_n78A-n257L | DC_3A_n78A DC_3A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 820 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A_n78A-n257M | DC_3A_n78A DC_3A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 920 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.3.2-2: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

|  |  |  | CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_3C_n78A-n257A | DC_3A_n78A DC_3A_n257A |  | 3 |  | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 540 |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | n257 | 60 |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  |  | 120 |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |
| DC_3C_n78A-n257D | DC_3A_n78A DC_3A_n257A |  | 3 |  | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 540 |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | n257 |  | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3C_n78A-n257E | DC_3A_n78A DC_3A_n257A |  | 3 |  |  | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 740 |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | n257 |  | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3C_n78A-n257F | DC_3A_n78A DC_3A_n257A |  | 3 |  | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 940 |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | n257 |  | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3C_n78A-n257G | DC_3A_n78A DC_3A_n257A |  | 3 |  | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 340 |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | n257 |  | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3C_n78A-n257H | DC_3A_n78A DC_3A_n257A |  | 3 |  | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 440 |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | n257 |  | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3C_n78A-n257I | DC_3A_n78A DC_3A_n257A |  | 3 |  | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 540 |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | n257 |  | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3C_n78A-n257J | DC_3A_n78A DC_3A_n257A |  | 3 |  | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 640 |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | n257 |  | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3C_n78A-n257K | DC_3A_n78A DC_3A_n257A |  | 3 |  | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 740 |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | n257 |  | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3C_n78A-n257L | DC_3A_n78A DC_3A_n257A |  | 3 |  | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 840 |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | n257 |  | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3C_n78A-n257M | DC_3A_n78A DC_3A_n257A |  | 3 |  | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 940 |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | n257 |  | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |

6.3.3	Co-existence studies

Based on co-existence studies of DC_3A-n78A and DC_3A_n257A specified in TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

- DC_3A_n78A: No harmonic problems by 7th & 8th order between FR1 and FR2

- DC_3A_n257A: Harmonic problem is already specified in TR 37.863-01-01

6.3.4	∆TIB and ∆RIB values

For DC_3_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 6.3.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n78-n257 | 3 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 6.3.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n78-n257 | 3 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

6.3.5	MSD

As mentioned in 6.3.3, there is no need to specify additional MSD requirement for this UL DC configuration.

6.4	DC_5A-n78A-n257

6.4.1	Operating bands for DC

Table 6.4.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5A_n78-n257 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

6.4.2	Channel bandwidths per operating band for DC

Table 6.4.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_5A_n78A-n257D | DC_5A_n78A DC_5A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 510 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A_n78A-n257E | DC_5A_n78A DC_5A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 710 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A_n78A-n257F | DC_5A_n78A DC_5A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 910 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A_n78A-n257G | DC_5A_n78A DC_5A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 310 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A_n78A-n257H | DC_5A_n78A DC_5A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 410 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A_n78A-n257I | DC_5A_n78A DC_5A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 510 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A_n78A-n257J | DC_5A_n78A DC_5A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 610 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A_n78A-n257K | DC_5A_n78A DC_5A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 710 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A_n78A-n257L | DC_5A_n78A DC_5A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 810 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A_n78A-n257M | DC_5A_n78A DC_5A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 910 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.4.3	Co-existence studies

Based on co-existence studies of DC_5A-n78A and DC_5A_n257A specified in TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

- DC_5A_n78A: No harmonic problems by 7th & 8th order between FR1 and FR2

- DC_5A_n257A: Harmonic problem is already specified in TR 37.863-01-01

6.4.4	∆TIB and ∆RIB values

For DC_5A_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 6.4.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5A_n78-n257 | 5 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 6.4.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5A_n78-n257 | 5 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

6.4.5	MSD

IMD issues are already analysed in TR 37.863-01-01 and there is no IMD issues to 3rd own Rx bands. Harmonics are expected in DC_5A_n78A-n257 configuration. DC_5A_n78A has 7th and 8th harmonics from n78 into n257 but there is no harmonic problems by 7th and 8th order between FR1 and FR2. Furthermore, DC_5A_n257A has 4th harmonic from Band 5 into n78 but this harmonic problem will be solved in DC_5A_n78A

6.5	DC_7A-n78A-n257

6.5.1	Operating bands for DC

Table 6.5.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7A_n78-n257 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

6.5.2	Channel bandwidths per operating band for DC

Table 6.5.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_7A_n78A-n257D | DC_7A_n78A DC_7A_n257A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 520 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_7A_n78A-n257E | DC_7A_n78A DC_7A_n257A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 720 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_7A_n78A-n257F | DC_7A_n78A DC_7A_n257A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 920 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_7A_n78A-n257G | DC_7A_n78A DC_7A_n257A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 320 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_7A_n78A-n257H | DC_7A_n78A DC_7A_n257A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 420 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_7A_n78A-n257I | DC_7A_n78A DC_7A_n257A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 520 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_7A_n78A-n257J | DC_7A_n78A DC_7A_n257A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 620 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_7A_n78A-n257K | DC_7A_n78A DC_7A_n257A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 720 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_7A_n78A-n257L | DC_7A_n78A DC_7A_n257A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 820 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_7A_n78A-n257M | DC_7A_n78A DC_7A_n257A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 920 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.5.3	Co-existence studies

Based on co-existence studies of DC_7A-n78A and DC_7A_n257A specified in TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

- DC_7A_n78A: No harmonic problems by 7th & 8th order between FR1 and FR2

- DC_7A_n257A: There is no issue on this configuration

6.5.4	∆TIB and ∆RIB values

For DC_7A_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 6.5.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A_n78-n257 | 7 | 0.5 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 6.5.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7A_n78-n257 | 7 | 0 |
|  | n78 | 0.5 |
|  | n257 | 0 |

6.5.5	MSD

No IMD issues are expected but harmonics are expected in DC_7A_n78A-n257 DC configuration. DC_7A_n78A has 7th and 8th harmonics from n78 into n257 but there is no harmonic problems by 7th and 8th order between FR1 and FR2.

6.6	DC_7A-7A_n78A-n257

6.6.1	Operating bands for DC

Table 6.6.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7A-7A_n78-n257 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

6.6.2	Channel bandwidths per operating band for DC

Table 6.6.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_7A-7A_n78A-n257D | DC_7A_n78A DC_7A_n257A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 540 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_7A-7A_n78A-n257E | DC_7A_n78A DC_7A_n257A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 740 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_7A-7A_n78A-n257F | DC_7A_n78A DC_7A_n257A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 940 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_7A-7A_n78A-n257G | DC_7A_n78A DC_7A_n257A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 340 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_7A-7A_n78A-n257H | DC_7A_n78A DC_7A_n257A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 440 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_7A-7A_n78A-n257I | DC_7A_n78A DC_7A_n257A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 540 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_7A-7A_n78A-n257J | DC_7A_n78A DC_7A_n257A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 640 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_7A-7A_n78A-n257K | DC_7A_n78A DC_7A_n257A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 740 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_7A-7A_n78A-n257L | DC_7A_n78A DC_7A_n257A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 840 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_7A-7A_n78A-n257M | DC_7A_n78A DC_7A_n257A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 940 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.6.3	Co-existence studies

Based on co-existence studies of DC_7A-n78A and DC_7A_n257A specified in TR 37.863-01-01, 7th harmonic from Band 7 fall into Band n257 when dual uplink is configured with Band 7+n78. There is no additional harmonic and intermodulation impact for the additional band receiver.

6.6.4	∆TIB and ∆RIB values

For DC_7A-7A_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 6.6.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-7A_n78-n257 | 7 | 0.5 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 6.6.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7A-7A_n78-n257 | 7 | 0 |
|  | n78 | 0.5 |
|  | n257 | 0 |

6.6.5	MSD

IMD problems are already analysed in constituent low order combinations between LTE bands and NR bands. 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 repectively and fallback modes are captured in TR 37.864-41-21. Therefore, additional analysis is not needed.
Uplink DC configuration is DC_7A_n78A: Up to 7th harmonic order will be not any impact to n257.

6.7	DC_1A_n3A-n78A

6.7.1	Operating bands for DC

Table 6.7.1-1: DC band combination of one LTE band + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A_n3A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.7.2	Channel bandwidths per operating band for DC

Table 6.7.2-1: Supported bandwidths per DC band combination of one LTE band + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A_n3A-n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 150 |
|  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.7.3	Co-existence studies

Co-existence studies of Band 1 + Band n3 + Band n78 are already captured in Table 5.3-1 of TR 37.716-21-21 [2].

6.7.4	∆TIB and ∆RIB values

For DC_1A_n3A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.7.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n3-n78 | 1 | 0.6 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |

Table 6.7.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1_n3-n78 | 1 | 0.2 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |

6.7.5	MSD

Based on Table 5.3.5 in TR 37.716-21-21 [2] it is needed to have MSD added in table 7.3B.2.3.5.2-1 of TS 38.101-3.

Table 6.7.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n3A-n78A | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A |  | N/A |
|  | n78 | 3700 | 10 | 52 | 3700 | 28.4 | TDD | IMD2 |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | 1770 | 5 | 25 | 1865 | N/A |  | N/A |
|  | n78 | 3360 | 10 | 52 | 3360 | 11.2 | TDD | IMD4 |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | 1735 | 5 | 25 | 1830 | 27.9 |  | IMD2 |
|  | n78 | 3780 | 10 | 52 | 3780 | N/A | TDD | N/A |

6.8	DC_3_n1-n78

6.8.1	Operating bands for DC

Table 6.8.1-1: DC band combination of one LTE band + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n1-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.8.2	Channel bandwidths per operating band for DC

Table 6.8.2-1: Supported bandwidths per DC band combination of one LTE band + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | Uplink Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A_n1A-n78A | DC_3A_n1A DC_3A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 140 |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3C_n1A-n78A | DC_3A_n1A DC_3A_n78A | 3 | 15 | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 160 |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.8.3	Co-existence studies

The 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products for the UE coexistence analysis of DC_3A_n1A-n78A with dual uplink on DC_3A_n1A and DC_3A_n78A were calculated and listed in Table 6.8.3-1 and 6.8.3-2 respectively.

Table 6.8.3-1: Harmonic and IMD analysis for DC_3_n1-n78 with UL on DC_3_n1

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 5760 | 5940 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 135 | 270 | 3630 | 3765 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1440 | 1650 | 2055 | 2250 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5340 | 5550 | 5550 | 5745 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3150 | 3435 | 3975 | 4230 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7050 | 7335 | 7470 | 7725 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 540 | 270 | 7260 | 7530 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6210 | 5895 | 5220 | 4860 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9390 | 9705 | 8760 | 9120 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2520 | 2190 | 1515 | 1170 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9180 | 9510 | 8970 | 9315 |

Table 6.8.3-2: Harmonic and IMD analysis for DC_3_n1-n78 with UL on DC_3_n78

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 9900 | 11400 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1515 | 2090 | 5010 | 5585 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 380 | 270 | 4815 | 5890 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6720 | 7370 | 8310 | 9385 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1330 | 2055 | 8115 | 9690 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8430 | 9155 | 11610 | 13185 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 4180 | 3030 | 10020 | 11170 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13490 | 11415 | 3840 | 3040 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14910 | 16985 | 10140 | 10940 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7980 | 6330 | 1245 | 2470 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13320 | 14970 | 11730 | 12955 |

The coexistence studies of Band 3 + Band n1 and Band 3 + Band n78 will be captured in the constituent fallback modes. The summary of self-interference issues for the 3rd own receive band are listed below.

Based on Table 6.8.3-1 and Table 6.8.3-2:

-	2nd and 4th order IMD generated by dual uplink of Band 3 + Band n1 may also fall into own Rx of band n78

-	5th order IMD generated by dual uplink of Band 3 + Band n78 may also fall into own Rx of band n1

6.8.4	∆TIB and ∆RIB values

For DC_3A_n1A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.8.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n1-n78 | n1 | 0.6 |
|  | 3 | 0.6 |
|  | n78 | 0.8 |
| NOTE 1: HTF is assumed for Band 3 and Band n1. |  |  |

Table 6.8.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n1-n78 | n1 | 0.2 |
|  | 3 | 0.2 |
|  | n78 | 0.5 |
| NOTE 1: HTF is assumed for Band 3 and Band n1. |  |  |

6.8.5	MSD

Based on Table 5.3.5 in TR 37.716-21-21 [2] it is needed to have MSD added in table 7.3B.2.3.5.2-1 of TS 38.101-3.

Table 6.8.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n1A-n78A or DC_3C_n1A-n78A | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | 3 | 1750 | 5 | 25 | 1845 | N/A |  | N/A |
|  | n78 | 3700 | 10 | 52 | 3700 | 28.4 | TDD | IMD2 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | 3 | 1770 | 5 | 25 | 1865 | N/A |  | N/A |
|  | n78 | 3360 | 10 | 52 | 3360 | 11.2 | TDD | IMD4 |
|  | n1 | 1940 | 5 | 25 | 2130 | 3.5 | FDD | IMD5 |
|  | 3 | 1770 | 5 | 25 | 1865 | N/A |  | N/A |
|  | n78 | 3720 | 10 | 52 | 3720 | N/A | TDD | N/A |

6.9	DC_3A_n7A-n78A

6.9.1	Operating bands for DC

Table 6.9.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A_n7A-n78A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.9.2	Channel bandwidths per operating band for DC

Table 6.9.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A_n7A-n78A | DC_3A_n7A DC_3A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 140 |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.9.3	Co-existence studies

Co-existence studies of Band 3 + Band n7 + Band n78 with UL Band 3 +Band n7 are already captured in Table 5.3-1 of TR 37.716-21-21 [2].

-	2nd order harmonic generated by uplink of Band 3 may also fall into own Rx of band n78.

-	4th order IMD generated by dual uplink of Band 3 + Band n7 may also fall into own Rx of band n7.

-	3rd order IMD generated by dual uplink of Band 3 + Band n7 may also fall into own Rx of band n78.

Co-existence studies of Band 3 + Band n7 + Band n78 with UL Band 3 +Band n78 are already captured in Table 5.3-1 of TR 37.716-21-21 [2].

-	2nd order harmonic generated by uplink of Band 3 may also fall into own Rx of band n78.

-	2nd , 4th , 5th order IMD generated by dual uplink of Band 3 + Band n78 may also fall into own Rx of band 3.

-	4th , 5th order IMD generated by dual uplink of Band 3 + Band n78 may also fall into own Rx of band n78. But there is no issue for TDD band.

6.9.4	∆TIB and ∆RIB values

For DC_3A_n7A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.9.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n7-n78 | 3 | 0.6 |
|  | n7 | 0.6 |
|  | n78 | 0.8 |

Table 6.9.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n7-n78 | 3 | 0.2 |
|  | n7 | 0.2 |
|  | n78 | 0.5 |

6.9.5	MSD

2nd order harmonic problem generated by uplink of Band 3 already have been solved in DC_3A_n78A.

2nd , 4th , 5th order IMD generated by dual uplink of Band 3 + Band n78 already have been solved in DC_3A_n78A.

4th order IMD generated by dual uplink of Band 3 + Band n7 already have been solved in DC_3A_n7A.

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD3 product produced by Band 3 and n7 that impact the reference sensitivity of Band n78. Following DC_7A-n3A_n78A requirement in table 5.2-5 of TR 37.864-41-21 [3], reference sensitivity exception requirements are defined in Table 6.9.5-1. It is needed to have MSD added in table 7.3B.2.3.5.2-1 of TS 38.101-3.

Table 6.9.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n7A-n78A | 3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n7 | 2560 | 5 | 25 | 2680 | N/A |  | N/A |
|  | n78 | 3390 | 10 | 52 | 3390 | 16.1 | TDD | IMD3 ¦2*fn7-*fB3¦ |

6.10	DC_3C_n7A-n78A

6.10.1	Operating bands for DC

Table 6.10.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3C_n7A-n78A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.10.2	Channel bandwidths per operating band for DC

Table 6.10.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3C_n7A-n78A | DC_3C_n7A DC_3A_n7A DC_3C_n78A DC_3A_n78A | 3 | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 160 |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.10.3	Co-existence studies

Co-existence studies of Band 3 + Band n7 + Band n78 already have been captured in current specification for DC_3A_n7A-n78A.

6.10.4	∆TIB and ∆RIB values

For DC_3C_n7A-n78A, the TIB,c and RIB values can reuse the same requirement for DC_3A_n7A-n78A.

6.10.5	MSD

MSD requirement for DC_3C_n7A-n78A can reuse the same requirement for DC_3A_n7A-n78A.

6.11	DC_13A_n5A-n260A

6.11.1	Operating bands for DC

Table 6.11.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_13A_n5A-n260A | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

6.11.2	Channel bandwidths per operating band for DC

Table 6.11.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_13A_n5A-n260A | DC_13A_n5A DC_13A_n260A | 13 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 430 |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n260 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes |  |  |

6.11.3	Co-existence studies

Co-existence studies of Band 13 + Band n5 + Band n260 are already captured in Table 5.3-1 of TR 37.716-21-21 [2].

6.11.4	∆TIB and ∆RIB values

For DC_13A_n5A-n260A, the TIB,c and RIB,c values are given in the tables below.

Table 6.11.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13_n5-n260 | 13 | 0.5 |
|  | n5 | 0.5 |
|  | n260 | 0 |

Table 6.11.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_13_n5-n260 | 13 | 0 |
|  | n5 | 0 |
|  | n260 | 0 |

6.11.5	MSD

Based on the co-existence findings in TR [2] MSD does not need to be defined for DC_13A_n5A-n260A.

6.12	DC_66A_n5A-n260A

6.12.1	Operating bands for DC

Table 6.12.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66A_n5A-n260A | 66 | 1710 MHz | – | 1710 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

6.12.2	Channel bandwidths per operating band for DC

Table 6.12.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_66A_n5A-n260A | DC_66A_n5A DC_66A_n260A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 440 |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n260 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes |  |  |

6.12.3	Co-existence studies

Co-existence studies of Band 66 + Band n5 + band n260 are already captured in Table 5.3-1 of TR 37.716-21-21 [2].

6.12.4	∆TIB and ∆RIB values

For DC_66A_n5A-n260A, the TIB,c and RIB,c values are given in the tables below.

Table 6.12.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n5-n260 | 66 | 0.3 |
|  | n5 | 0.3 |
|  | n260 | 0 |

Table 6.12.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n5-n260 | 66 | 0 |
|  | n5 | 0 |
|  | n260 | 0 |

6.12.5	MSD

Based on the co-existence findings in TR [2] MSD does not need to be defined for DC_66A_n5A-n260A.

6.13	DC_3A_n20A-n78A

6.13.1	Operating bands for DC

Table 6.13.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit | BS transmit / UE receive |  |
|  |  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| DC_3A_n20A-n78A | 3 | 1710 MHz -– 1785 MHz | 1805 MHz –1880 MHz | FDD |
|  | n20 | 832 MHz -– 862 MHz | 791 MHz – 821 MHz | FDD |
|  | n78 | 3300 MHz – 3800 MHz | 3300 MHz – 3800 MHz | TDD |

6.13.2	Channel bandwidths per operating band for DC

Table 6.13.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n20A-n78A | DC_3A_n20A DC_3A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 140 |
|  |  | n20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.13.3	Co-existence studies

Co-existence studies of Band 3 + Band n20 + Band n78 are already captured in Table 5.3-1 of TR 37.716-21-21 [2].

6.13.4	∆TIB and ∆RIB values

For DC_ the TIB,c and RIB,c values are derived from CA_3-20-42 and are given in the tables below.

Table 6.13.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n20-n78 | 3 | 0.6 |
|  | n20 | 0.3 |
|  | n78 | 0.8 |

Table 6.13.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n20-n78 | 3 | 0.2 |
|  | n20 | 0 |
|  | n78 | 0.5 |

6.13.5	MSD

Based on the co-existence studies in TR 37.716-21-21 [2] DC_3_n20-n78 need to have MSD added in table 7.3B.2.3.5.2-1 of TS 38.101-3.

Table 6.13.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n20A-n78A | 3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n20 | 845 | 5 | 25 | 804 | N/A | FDD | N/A |
|  | n78 | 3420 | 10 | 52 | 3420 | 16.1 | TDD | IMD3 |
|  | 3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n20 | 845 | 5 | 25 | 804 | N/A | FDD | N/A |
|  | n78 | 3500 | 10 | 52 | 3500 | 4.5 | TDD | IMD5 |

6.14	DC_28A_n8A-n78A

6.14.1	Operating bands for DC

Table 6.14.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28A_n8A-n78A | 28 | 703 MHz |  | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n8 | 880 MHz |  | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz |  | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.14.2	Channel bandwidths per operating band for DC

Table 6.14.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_28A_n8A-n78A | DC_28A_n8A DC_28A_n78A | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 140 |
|  |  | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.14.3	Co-existence studies

Co-existence studies of Band 28 + Band n8 + band n78 are already captured in Table 5.3-1 of TR 37.864-41-21 [2].

6.14.4	∆TIB and ∆RIB values

For DC_28A_n8A-n78A, the TIB,c and RIB,c values are derived from CA_8-28-41 and are given in the tables below.

Table 6.14.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28_n8-n78 | 28 | 0.5 |
|  | n8 | 0.6 |
|  | n78 | 0.3 |

Table 6.14.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_28_n8-n78 | 28 | 0.1 |
|  | n8 | 0.2 |
|  | n78 | 0 |

6.14.5	MSD

Based on the co-existence studies in TR 37.864-41-21 [2], DC_28A_n8A-n78A need to have MSD added in table 7.3B.2.3.5.2-1 of TS 38.101-3.

Table 6.14.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_28A_n8A-n78A | 28 | 728 | 5 | 25 | 783 | N/A | FDD | N/A |
|  | n8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |
|  | n78 | 3458 | 10 | 52 | 3458 | 9.1 | TDD | IMD4 |
|  | 28 | 713 | 5 | 25 | 768 | N/A | FDD | N/A |
|  | n8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |
|  | n78 | 3762 | 10 | 52 | 3762 | 3.7 | TDD | IMD5 |
|  | 28 | 713 | 5 | 25 | 768 | N/A | FDD | N/A |
|  | n8 | 890 | 5 | 25 | 935 | 4.3 | TDD | IMD5 |
|  | n78 | 3787 | 10 | 52 | 3787 | N/A | FDD | N/A |

6.15	DC_28A_n8A-n258A

6.15.1	Operating bands for DC

Table 6.15.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28A_n8A-n258A | 28 | 703 MHz |  | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n8 | 880 MHz |  | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n258 | 24250 MHz |  | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

6.15.2	Channel bandwidths per operating band for DC

Table 6.15.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_28A_n8A-n258A | DC_28A_n8A DC_28A_n258A | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 440 |  |
|  |  | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n258 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes |  |  |

6.15.3	Co-existence studies

Co-existence studies of Band 28 + Band n8 + band n258 are already captured in Table 5.3-1 of TR 37.716-21-21 [2].

6.15.4	∆TIB and ∆RIB values

For DC_28_n8-n258, the TIB,c and RIB,c values are given in the tables below.

Table 6.15.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28_n8-n258 | 28 | 0.5 |
|  | n8 | 0.6 |
|  | n258 | 0 |

Table 6.15.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_28_n8-n258 | 28 | 0.1 |
|  | n8 | 0.2 |
|  | n258 | 0 |

6.15.5	MSD

Based on the co-existence findings in TR [2] MSD does not need to be defined for DC_28_n8-n258.

6.16	DC_3C_n28A-n78A

6.16.1	Operating bands for DC

Table 6.16.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3C_n28A-n78A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.16.2	Channel bandwidths per operating band for DC

Table 6.16.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3C_n28A-n78A | DC_3A_n28A DC_3C_n28A DC_3A_n78A DC_3C_n78A | 3 | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 160 |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.16.3	Co-existence studies

Co-existence studies of Band 3 + Band n28 + Band n78 with UL Band 3 +Band n78 are already captured in Table 5.3-1 of TR 37.716-21-21 [2].

-	2nd order harmonic generated by uplink of Band 3 may also fall into own Rx of band n78.

-	2nd , 4th , 5th order IMD generated by dual uplink of Band 3 + Band n78 may also fall into own Rx of band 3.

-	4th , 5th order IMD generated by dual uplink of Band 3 + Band n78 may also fall into own Rx of band n78. But there is no issue for TDD band.

Co-existence studies of Band 3 + Band n28 + Band n78 with UL Band 3 +Band n28 are already captured in Table 5.3-1 of TR 37.716-21-21 [2].

-	2nd order harmonic generated by uplink of Band 3 may also fall into own Rx of band n78.

-	5th order harmonic generated by uplink of Band n28 may also fall into own Rx of band n78.

-	5th order IMD generated by dual uplink of Band 3 + Band n28 may also fall into own Rx of band n78.

6.16.4	∆TIB and ∆RIB values

For DC_3C_n28A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.16.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n28-n78 | 3 | 0.5 |
|  | n28 | 0.3 |
|  | n78 | 0.8 |

Table 6.16.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n28-n78 | 3 | 0.2 |
|  | n28 | 0 |
|  | n78 | 0.5 |

6.16.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], 5th order IMD generated by dual uplink of Band 3 + Band n28 may also fall into own Rx of band n78. Following DC_3A_n28A-n78A requirement in table 7.3B.2.3.5.2-1 of TS 38.101-3, reference sensitivity exception requirements are defined in Table 6.16.5-1. It is needed to have MSD added in table 7.3B.2.3.5.2-1 of TS 38.101-3.

Table 6.16.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3C_n28A-n78A | 3 | 1750 | 5 | 25 | 1845 | N/A | FDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A |  | N/A |
|  | n78 | 3764 | 10 | 50 | 3764 | 4.5 | TDD | IMD5 ¦2*fn28-3*fB3¦ |

6.17	DC_7C_n28A-n78A

6.17.1	Operating bands for DC

Table 6.17.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7C_n28A-n78A | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.17.2	Channel bandwidths per operating band for DC

Table 6.17.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_7C_n28A-n78A | DC_7C_n28A DC_7C_n78A | 7 | See CA_7C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 160 |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.17.3	Co-existence studies

Co-existence studies of Band 7 + Band n28 + Band n78 with UL Band 7 +Band n78 are already captured in Table 5.3-1 of TR 37.716-21-21 [2].

-	2nd , 5th  order IMD generated by dual uplink of Band 7 + Band n78 may also fall into own Rx of band n28.

-	4th order IMD generated by dual uplink of Band 7 + Band n78 may also fall into own Rx of band n78. But there is no issue for TDD band.

Co-existence studies of Band 7 + Band n28 + Band n78 with UL Band 7 +Band n28 are already captured in Table 5.3-1 of TR 37.716-21-21 [2].

-	5th order harmonic generated by uplink of Band n28 may also fall into own Rx of band n78.

-	2nd , 4th order IMD generated by dual uplink of Band 7 + Band n28 may also fall into own Rx of band n78.

6.17.4	∆TIB and ∆RIB values

For DC_7C_n28A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.17.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n28-n78 | 7 | 0.3 |
|  | n28 | 0.3 |
|  | n78 | 0.8 |

Table 6.17.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7_n28-n78 | 7 | 0 |
|  | n28 | 0 |
|  | n78 | 0.5 |

6.17.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there are IMD2 and IMD5 product produced by Band 7 and n78 that impact the reference sensitivity of Band n28. Following DC_7A-n28A_n78A requirement in table 5.2-5 of TR 37.864-41-21 [3], reference sensitivity exception requirements are defined in Table 6.17.5-1.

Based on Table 5.3-1 in TR 37.716-21-21 [2], there are IMD2 and IMD4 product produced by Band 7 and n28 that impact the reference sensitivity of Band n78. Following DC_7A-n28A_n78A requirement in table 5.2-5 of TR 37.864-41-21 [3], reference sensitivity exception requirements are defined in Table 6.17.5-1.

It is needed to have MSD added in table 7.3B.2.3.5.2-1 of TS 38.101-3.

Table 6.17.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_7C_n28A-n78A | 7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |
|  | n28 | 745 | 5 | 25 | 800 | 28.8 | FDD | IMD2 ¦f n78-f B7 ¦ |
|  | n78 | 3365 | 10 | 50 | 3365 | N/A | TDD | N/A |
|  | 7 | 2520 | 5 | 25 | 2640 | N/A | FDD | N/A |
|  | n28 | 731 | 5 | 25 | 786 | 4.0 | FDD | IMD5 ¦2*f n78-3*f B7 ¦ |
|  | n78 | 3387 | 10 | 50 | 3387 | N/A | TDD | N/A |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |
|  | n28 | 745 | 5 | 25 | 800 | N/A | FDD | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | 29.7 | TDD | IMD2 ¦fn28+fB7¦ |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |
|  | n28 | 745 | 5 | 25 | 800 | N/A | FDD | N/A |
|  | n78 | 3640 | 10 | 50 | 3640 | 10.3 | TDD | IMD4 ¦2*f B7-2*f n28¦ |

6.18	DC_66_n71-n260

6.18.1	Operating bands for DC

Table 6.18.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66A_n71A-n260 | 66 | 1710 MHz | – | 1710 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

6.18.2	Channel bandwidths per operating band for DC

Table 6.18.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_66A_n71A-n260A | DC_66A_n71A DC_66A_n260A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 440 |  |
|  |  | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n260 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes |  |  |
| DC_66A_n71A-n260(2A) | DC_66A_n71A DC_66A_n260A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 840 |  |
|  |  | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n260 | See CA_n260(2A) in Table 5.5A.2-1 of TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.18.3	Co-existence studies

Co-existence studies of DC_66_n71 and DC_66_n260 are already captured in Rel-15 TR 37.863-01-01 [2].

6.18.4	∆TIB and ∆RIB values

For DC_66_n71-n260, the TIB,c and RIB,c values are given in the tables below.

Table 6.18.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n71-n260 | 66 | 0.3 |
|  | n71 | 0.3 |
|  | n260 | 0 |

Table 6.18.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n71-n260 | 66 | 0 |
|  | n71 | 0 |
|  | n260 | 0 |

6.18.5	MSD

No further MSD is needed.

6.19	DC_66A-n71A-n261

6.19.1	Operating bands for DC

Table 6.19.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66A_n71A-n261A DC_66A_n71A-n261(2A) | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n261 | 27500 MHz | – | 28350 MHz | 27500 MHz | – | 28350 MHz | TDD |

6.19.2	Channel bandwidths per operating band for DC

Table 6.19.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | Uplink Configurations | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz |  | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 50 MHz |  |  | 100 MHz | 200 MHz |  | 400 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_66A_n71A-n261A | DC_66A_n71A DC_66A_n261A | 66 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 440 |
|  |  | n71 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n261 | 60 |  |  |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  | Yes | Yes |  | Yes | Yes |  |  |
| DC_66A_n71A-n261(2A) | DC_66A_n71A DC_66A_n261A | 66 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 840 |
|  |  | n71 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n261 | See CA_n261(2A) BCS 0 in Table 5.5A.2-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.19.3	Co-existence studies

Co-existence studies of DC_66A-n71A are captured in TR 37.863-01-01 and for EN-DC between E-UTRA and FR2 DC_66A_n261A there is no requirements specified.

6.19.4	∆TIB and ∆RIB values

For DC_66A_n71A-n261A or DC_66A_n71A-n261(2A), the TIB,c and RIB,c values are given in the tables below.

Table 6.19.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n71-n261 | 66 | 0.3 |
|  | n71 | 0.3 |
|  | n261 | 0 |

Table 6.19.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n71-n261 | 66 | 0 |
|  | n71 | 0 |
|  | n261 | 0 |

6.19.5	MSD

Based on the co-existence study in 37.716-21-21, MSD does not need to be defined.

6.20	Void

6.21	DC_1A_n7A-n78A

6.21.1	Operating bands for DC

Table 6.21.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A_n7A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.21.2	Channel bandwidths per operating band for DC

Table 6.21.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A_n7A-n78A | DC_1A_n7A DC_1A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 140 |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.21.3	Co-existence studies

Co-existence studies of Band 1 + Band n7 + Band n78 are already captured in Table 5.3-1 of TR 37.716-21-21 [2].

-	4th order IMD generated by dual uplink of Band 1 + Band n78 may also fall into own Rx of band n7

-	4th and 5th order IMD generated by dual uplink of Band 1 + Band n7 may also fall into own Rx of band n78

6.21.4	∆TIB and ∆RIB values

For DC_1A_n7A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.21.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n7-n78 | 1 | 0.6 |
|  | n7 | 0.6 |
|  | n78 | 0.8 |

Table 6.21.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1_n7-n78 | 1 | 0.2 |
|  | n7 | 0.2 |
|  | n78 | 0.5 |

6.21.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD4 product produced by Band 1 and n78 that impact the reference sensitivity of Band n7. Following DC_1A-7A_n78A requirement, reference sensitivity exception requirements are defined in Table 6.21.5-1. It is needed to have MSD added in table 7.3B.2.3.5.2-1 of TS 38.101-3.

In addition, there is IMD4 and IMD5 product produced by Band 1 and n7 that impact the reference sensitivity of Band n78. The MSD is shown in the following table.

Table 6.21.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n7A-n78A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | FDD | N/A |
|  | n7 | 2507.5 | 5 | 25 | 2627.5 | 9.1 |  | IMD4 ¦fn78-3*fB1¦ |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | 1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n7 | 2520 | 5 | 25 | 2640 | N/A |  | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 10.1 | TDD | IMD4 ¦fn7-3*fB1¦ |
|  | 1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n7 | 2520 | 5 | 25 | 2640 | N/A |  | N/A |
|  | n78 | 3620 | 10 | 50 | 3620 | 3.8 | TDD | IMD5 ¦3*fn7-2*fB1¦ |

6.22	DC_8_n77-n257

6.22.1	Operating bands for DC

Table 6.22.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n77-n257 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

6.22.2	Channel bandwidths per operating band for DC

Table 6.22.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_8A_n77A-n257A | DC_8A_n77A DC_8A_n257A | 8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 520 |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes |  |
| DC_8A_n77A-n257D | DC_8A_n77A DC_8A_n257A | 8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 520 |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_8A_n77A-n257G | DC_8A_n77A DC_8A_n257A | 8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 320 |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_8A_n77A-n257H | DC_8A_n77A DC_8A_n257A | 8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 420 |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_8A_n77A-n257I | DC_8A_n77A DC_8A_n257A | 8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 520 |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |

6.22.3	Co-existence studies

Based on the co-existence studies of DC_8A-n77A and DC_8A-n257A are captured in TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

6.22.4	∆TIB and ∆RIB values

For DC_8A_n77A-n257A, the TIB,c and RIB,c values are given in the tables below.

Table 6.22.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n77-n257 | 8 | 0.6 |
|  | n77 | 0.8 |
|  | n257 | 0 |

Table 6.22.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n77-n257 | 8 | 0.2 |
|  | n77 | 0.5 |
|  | n257 | 0 |

6.22.5	MSD

As mentioned in 6.22.3, there is no need to specify additional MSD requirement for this UL DC configuration..

6.23	DC_1A_n40A-n78A

6.23.1	Operating bands for DC

Table 6.23.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-n40A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.23.2	Channel bandwidths per operating band for DC

Table 6.23.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | Uplink Configurations | E-UTRA and NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n40A-n78A | DC_1A_n40A DC_1A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 200 |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.23.3	Co-existence studies

The result of co-existence studies for DC_1A-n40A-n78A are provided in TR 37.716-21-21, Table 5.3-2.

6.23.4	∆TIB and ∆RIB values

For DC_1A_n40A-n78A, the TIB,c and RIB,c values are given in the tables below.

Table 6.23.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A_n40A-n78A | 1 | 0.3 |
|  | n40 | 0.5 |
|  | n78 | 0.8 |

Table 6.23.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A_n40A-n78A | 1 | 0 |
|  | n40 | 0 |
|  | n78 | 0.5 |

### 6.23.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD4 & IMD5 products produced by Band 1 and n40 that impact the reference sensitivity of NR n78.

In addition, there is IMD4 product produced by Band 1 and n78 that impact the reference sensitivity of Band n40. The required MSD are shown in the following table.

Table 6.23.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_1A_n40A-n78A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n40A-n78A | 1 | IMD4 | ¦3*fB1-fn40¦ | 1930 | 5 | 25 | 2120 | N/A |
|  | n40 |  |  | 2340 | 5 | 25 | 2340 |  |
|  | n78 |  |  | 3450 | 10 | 50 | 3450 | 9.8 |
|  | 1 | IMD5 | ¦2*fB1 -3*fn40¦ | 1925 | 5 | 25 | 2115 | N/A |
|  | n40 |  |  | 2390 | 5 | 25 | 2390 |  |
|  | n78 |  |  | 3320 | 10 | 50 | 3320 | 4.2 |
|  | 1 | IMD4 | ¦3*fB1 -fn78¦ | 1960 | 5 | 25 | 2150 | N/A |
|  | n78 |  |  | 3520 | 10 | 50 | 3520 |  |
|  | n40 |  |  | 2360 | 5 | 25 | 2360 | 10.6 |

6.24	DC_3_n1-n79

6.24.1	Operating bands for DC

Table 6.24.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A_n1A-n79A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

6.24.2	Channel bandwidths per operating band for DC

Table 6.24.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A_n1A-n79A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 140 |
|  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |
|  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |

6.24.3	Co-existence studies

The 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products for the UE coexistence analysis of DC_3A_n1A-n79A with dual uplink on DC_3A_n1A and DC_3A_n79A were calculated and listed in Table 6.24.3-1 and 6.24.3-2 respectively.

Table 6.24.3-1: Harmonic and IMD analysis for DC_3_n1-n79 with UL on DC_3_n1

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 5760 | 5940 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 135 | 270 | 3630 | 3765 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1440 | 1650 | 2055 | 2250 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5340 | 5550 | 5550 | 5745 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3150 | 3435 | 3975 | 4230 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 540 | 270 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7050 | 7335 | 7470 | 7725 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 7260 | 7530 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6210 | 5895 | 5220 | 4860 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2520 | 2190 | 1515 | 1170 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9390 | 9705 | 8760 | 9120 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9180 | 9510 | 8970 | 9315 |

Table 6.24.3-2: Harmonic and IMD analysis for DC_3_n1-n79 with UL on DC_3_n79

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 4400 | 5000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 8800 | 10000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 13200 | 15000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2615 | 3290 | 6110 | 6785 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1580 | 830 | 7015 | 8290 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7820 | 8570 | 10510 | 11785 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 130 | 955 | 11415 | 13290 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 6580 | 5230 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9530 | 10355 | 14910 | 16785 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 12220 | 13570 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 18290 | 15815 | 2740 | 1840 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 11580 | 9630 | 3445 | 4870 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 19310 | 21785 | 11240 | 12140 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 16620 | 18570 | 13930 | 15355 |

The coexistence studies of Band 3 + Band n1 and Band 3 + Band n79 will be captured in the constituent fallback modes. The summary of self-interference issues for the 3rd own receive band are listed below.

Based on Table 6.24.3-1 and Table 6.24.3-2:

-	5th order IMD generated by dual uplink of Band 3 + Band n1 may also fall into own Rx of band n79

-	5th order IMD generated by dual uplink of Band 3 + Band n79 may also fall into own Rx of band n1

6.24.4	∆TIB and ∆RIB values

For DC_3A_n1A-n79, the TIB,c and RIB values are given in the tables below.

Table 6.24.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A_n1A-n79A | 3 | 0.3 |
|  | n1 | 0.3 |
|  | n79 | 0 |
| NOTE 1: HTF is assumed for Band 3 and Band n1. |  |  |

Table 6.24.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A_n1A-n79A | 3 | 0 |
|  | n1 | 0 |
|  | n79 | 0 |
| NOTE 1: HTF is assumed for Band 3 and Band n1. |  |  |

### 6.24.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD5 products produced by Band 3 and n1 that impact the reference sensitivity of NR n79.

In addition, there is IMD5 product produced by Band 3 and n79 that impact the reference sensitivity of NR Band n1. The required MSD are shown in the following table.

Table 6.24.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_3A_n1A-n79A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n1A-n79A | 3 | IMD5 | ¦4*fB3-fn1¦ | 1720 | 5 | 25 | 1815 | N/A |
|  | n1 |  |  | 1930 | 5 | 25 | 2120 |  |
|  | n79 |  |  | 4950 | 40 | 216 | 4950 | 4.7 |
|  | 3 | IMD5 | ¦4*fB3 -fn79¦ | 1750 | 5 | 25 | 1845 | N/A |
|  | n79 |  |  | 4860 | 40 | 216 | 4860 |  |
|  | n1 |  |  | 1950 | 5 | 25 | 2140 | 3.6 |

6.25	DC_3_n40-n78

6.25.1	Operating bands for DC

Table 6.25.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-n40A-n78A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.25.2	Channel bandwidths per operating band for DC

Table 6.25.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | Uplink Configurations | E-UTRA and NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n40A-n78A | DC_3A_n40A DC_3A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 200 |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.25.3	Co-existence studies

The result of co-existence studies for DC_3A-n40A-n78A are provided in TR 37.716-21-21, Table 5.3-2.

6.25.4	∆TIB and ∆RIB values

For DC_3A_n40A-n78A, the TIB,c and RIB,c values are given in the tables below.

Table 6.25.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A_n40A-n78A | 3 | 0.6 |
|  | n40 | 0.5 |
|  | n78 | 0.8 |

Table 6.25.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A_n40A-n78A | 3 | 0.2 |
|  | n40 | 0 |
|  | n78 | 0.5 |

### 6.25.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD5 products produced by Band 3 and n40 that impact the reference sensitivity of NR band n78.

In addition, there is IMD5 product produced by Band 3 and n78 that impact the reference sensitivity of NR Band n40. The required MSD are shown in the following table.

Table 6.25.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_3A_n40A-n78A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n40A-n78A | 3 | IMD5 | ¦2*fB3-3*fn40¦ | 1730 | 5 | 25 | 1825 | N/A |
|  | n40 |  |  | 2360 | 5 | 25 | 2360 |  |
|  | n78 |  |  | 3620 | 10 | 50 | 3620 | 4.8 |
|  | 3 | IMD5 | ¦3*fB3 -2*fn78¦ | 1720 | 5 | 25 | 1815 | N/A |
|  | n78 |  |  | 3760 | 10 | 50 | 3760 |  |
|  | n40 |  |  | 2360 | 5 | 25 | 2360 | 4.4 |

6.26	DC_66_n25-n41

6.26.1	Operating bands for DC

Table 6.26.1-1: DC band combination of one LTE band + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n25-n41 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

6.26.2	Channel bandwidths per operating band for DC

Table 6.26.2-1: Supported bandwidths per DC band combination of one LTE band + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_66A_n25A-n41A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 140 |
|  | n25 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.26.3	Co-existence studies

Co-existence studies of Band 66 + Band n25 + Band n41 are captured in Table 6.26.3-1 and 6.26.3-2 below.

Table 6.26.3-1 lists up to 7th order harmonics and IMD up to 5th order from dual uplink of B66 and n25 falls into one of these receiving bands.

Table 6.26.3-1: Harmonic and IMD analysis 66_n25

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 | 1850 | 1915 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3560 | 3700 | 3830 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5340 | 5550 | 5745 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7120 | 7400 | 7660 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8900 | 9250 | 9575 |
| 6th harmonics frequency limits | 6* fy_low | 6* fy_high | 6*fx_low | 6*fx_high |
| 6th harmonics frequency limits (MHz) | 10260 | 10680 | 11100 | 11490 |
| 7th harmonics frequency limits | 7* fy_low | 7* fy_high | 7*fx_low | 7*fx_high |
| 7th harmonics frequency limits (MHz) | 11970 | 12460 | 12950 | 13405 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 205 | 70 | 3560 | 3695 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1505 | 1710 | 1920 | 2120 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5270 | 5475 | 5410 | 5610 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 410 | 140 | 7120 | 7390 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3215 | 3490 | 3770 | 4035 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6980 | 7255 | 7260 | 7525 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5950 | 5620 | 5270 | 4925 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9110 | 9440 | 8690 | 9035 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2325 | 1990 | 1640 | 1300 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8970 | 9305 | 8830 | 9170 |

Based on Table 6.26.3-1, it can be seen that

- 2nd order harmonic may fall into Rx frequencies of band 22, 42. 43, 48, 49, n77 and n78.

- 3rd order harmonic may fall into Rx frequencies of band 46.

- 2nd order IMD products may fall into Rx frequencies of bands 22, 42, 43, 48, 49, n77 and n78.

- 3rd order IMD products may fall into Rx frequencies of bands 1, 2, 4, 10, 21, 24, 33, 34, 36, 37, 39, 46, 50, 65, 70, 74 and 75, and also own Rx frequencies of band 66 and n25.

- 4th order IMD products may fall into Rx frequencies of bands 42, 43, n77 and n78.

- 5th order IMD products may fall into Rx frequencies of bands 1, 2, 4, 10, 11, 21, 23, 24, 32, 34, 36, 40, 45, 46, 47, 50, 65, 70, 74, 75, 76 and n79, and also own Rx frequencies of band 66 and n25.

Therefore, there is no harmonic/IMD problem into the own 3rd band B41 frequency range.

Table 6.26.3-2 lists up to 7th order harmonics and IMD up to 5th order from dual uplink of B66 and n41 falls into one of these receiving bands.

Table 6.26.3-2: Harmonic and IMD analysis 66_n41

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3560 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5340 | 7488 | 8070 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7120 | 9984 | 10760 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8900 | 12480 | 13450 |
| 6th harmonics frequency limits | 6* fy_low | 6* fy_high | 6*fx_low | 6*fx_high |
| 6th harmonics frequency limits (MHz) | 10260 | 10680 | 14976 | 16140 |
| 7th harmonics frequency limits | 7* fy_low | 7* fy_high | 7*fx_low | 7*fx_high |
| 7th harmonics frequency limits (MHz) | 11970 | 12460 | 17472 | 18830 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 980 | 716 | 4206 | 4470 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 730 | 1064 | 3212 | 3670 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5916 | 6250 | 6702 | 7160 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1960 | 1432 | 8412 | 8940 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2440 | 2844 | 5708 | 6360 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7626 | 8030 | 9198 | 9850 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9050 | 8204 | 4624 | 4150 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11694 | 12540 | 9336 | 9810 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4650 | 3928 | 348 | 250 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10908 | 11630 | 10122 | 10720 |

Based on Table 6.26.3-2, it can be seen that

- 2nd order harmonic may fall into Rx frequencies of band 22, 42. 46, 48, 49, n77, n78 and n79.

- 3rd order harmonic may fall into Rx frequencies of band 46.

- 2nd order IMD products may fall into Rx frequencies of bands 5, 6, 8, 12, 13, 17, 18, 19, 20, 26, 27, 28, 29, 44, 67, 68 and n79.

- 3rd order IMD products may fall into Rx frequencies of bands 5, 6, 8, 12, 13, 17, 18, 19, 20, 26, 27, 28, 42, 43, 44, 46, 47, 48, 49, 67, 68, n77 and n78.

- 4th order IMD products may fall into Rx frequencies of bands 2, 3, 7, 9, 11, 21, 24, 25, 32, 33, 35, 36, 37, 38, 39, 45, 46, 47, 50, 51, 74, 75 and 76.

- 5th order IMD products may fall into Rx frequencies of bands n77 and n79.

The 4th IMD product by dual uplink of B66 and n41 will be impacted the own 3rd Rx band in Band 25 Rx frequency range.

6.26.4	∆TIB and ∆RIB values

For DC_66_n25-n41, the TIB,c and RIB values are derived as below.

Table 6.26.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n25-n41 | 66 | 0.5 |
|  | n25 | 0.5 |
|  | n41 | 0.81 |
|  |  | 1.32 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

Table 6.26.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n25-n41 | 66 | 0.5 |
|  | n25 | 0.5 |
|  | n41 | 0.51 |
|  |  | 1.02 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

### 6.26.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD4 products produced by Band 66 and n41 that impact the reference sensitivity of NR band n25. The required MSD is shown in the following table.

Table 6.26.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_66A_n25A-n41A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_66A_n25A-n41A | 66 | IMD4 | ¦2*fB66 -2*fn41¦ | 1715 | 5 | 25 | 2115 | N/A |
|  | n41 |  |  | 2685 | 10 | 50 | 2685 |  |
|  | n25 |  |  | 1860 | 5 | 25 | 1940 | 11.0 |

The update needed in 38-101-3 as in following table.

Table 7.3B.2.3.5.2-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | DL LCRB | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_66A_n25A-n41A | 66 | 1715 | 5 | 25 | 2115 | 5 | N/A | FDD | N/A |
|  | n41 | 2685 | 10 | 50 | 2685 | 10 | N/A | TDD |  |
|  | n25 | 1860 | 5 | 25 | 1940 | 5 | 11.0 | FDD | IMD4 |

6.27	DC_3A_n1A-n257A

6.27.1	Operating bands for DC

Table 6.27.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A_n1A-n257A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

6.27.2	Channel bandwidths per operating band for DC

Table 6.27.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n1A-n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 540 |
|  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | n257 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes |  |

6.27.3	Co-existence studies

The 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products for the UE coexistence analysis of DC_3A_n1A-n257A with dual uplink on DC_3A_n1A and DC_3A_n257A were calculated and listed in Table 6.27.3-1 and 6.27.3-2 respectively.

Table 6.27.3-1: Harmonic and IMD analysis for DC_3_n1-n257 with UL on DC_3_n1

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 5760 | 5940 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 135 | 270 | 3630 | 3765 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1440 | 1650 | 2055 | 2250 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5340 | 5550 | 5550 | 5745 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3150 | 3435 | 3975 | 4230 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 540 | 270 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7050 | 7335 | 7470 | 7725 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 7260 | 7530 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6210 | 5895 | 5220 | 4860 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2520 | 2190 | 1515 | 1170 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9390 | 9705 | 8760 | 9120 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9180 | 9510 | 8970 | 9315 |

Table 6.27.3-2: Harmonic and IMD analysis for DC_3_n1-n257 with UL on DC_3_n257

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 79500 | 88500 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 24715 | 27790 | 28210 | 31285 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 26080 | 22930 | 51215 | 57290 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 29920 | 33070 | 54710 | 60785 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | -24370 | -21145 | 77715 | 86790 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 55580 | 49430 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 31630 | 34855 | 81210 | 90285 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 56420 | 62570 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 116290 | 104215 | 19360 | 22660 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 85080 | 75930 | 47645 | 53870 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 107710 | 119785 | 33340 | 36640 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 82920 | 92070 | 58130 | 64355 |

The coexistence studies of Band 3 + Band n1 and Band 3 + Band n257 will be captured in the constituent fallback modes. There are no self-interference issues for the 3rd own receive band of this configuration.

6.27.4	∆TIB and ∆RIB values

For DC_3A_n1A-n257, the TIB,c and RIB values are given in the tables below.

Table 6.27.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A_n1A-n257A | 3 | 0.3 |
|  | n1 | 0.3 |
|  | n257 | 0 |
| NOTE 1: HTF is assumed for Band 3 and Band n1. |  |  |

Table 6.27.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A_n1A-n257A | 3 | 0 |
|  | n1 | 0 |
|  | n257 | 0 |
| NOTE 1: HTF is assumed for Band 3 and Band n1. |  |  |

6.27.5	MSD

There are no additional requirements for this configuration.

6.28	DC_1_n77-n258

6.28.1	Operating bands for DC

Table 6.28.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1_n77-n258 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

6.28.2	Channel bandwidths per operating band for DC

Table 6.28.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| EN-DC configuration | Uplink configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n77A-n258A | DC_1A_n77A DC_1A_n258A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 520 |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n258 | 60 |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |

6.28.3	Co-existence studies

Co-existence studies of DC_1A_n77A are captured in 37.863-01-01 and for EN-DC between E-UTRA and FR2 DC_1A_n258A there is no requirements specified.

6.28.4	∆TIB and ∆RIB values

For DC_1_n77-n258, the TIB,c and RIB values are given in the tables below.

Table 6.28.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n77-n258 | 1 | 0.6 |
|  | n77 | 0.8 |
|  | n258 | 0 |

Table 6.28.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1_n77-n258 | 1 | 0.2 |
|  | n77 | 0.5 |
|  | n258 | 0 |

6.28.5	MSD

There is no additional MSD issue for DC_1_n77-n258.

6.29	DC_1_n78-n258

6.29.1	Operating bands for DC

Table 6.29.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1_n78-n258 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

6.29.2	Channel bandwidths per operating band for DC

Table 6.29.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| EN-DC configuration | Uplink configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n78A-n258A | DC_1A_n78A DC_1A_n258A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 520 |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n258 | 60 |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |

6.29.3	Co-existence studies

Co-existence studies of DC_1A_n78A are captured in 37.863-01-01 and for EN-DC between E-UTRA and FR2 DC_1A_n258A there is no requirements specified.

6.29.4	∆TIB and ∆RIB values

For DC_1_n78-n258, the TIB,c and RIB values are given in the tables below.

Table 6.29.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n78-n258 | 1 | 0.3 |
|  | n78 | 0.8 |
|  | n258 | 0 |

Table 6.29.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1_n78-n258 | 1 | 0 |
|  | n78 | 0.5 |
|  | n258 | 0 |

6.29.5	MSD

There is no additional MSD issue for DC_1_n78-n258.

6.30	DC_1_n79-n258

6.30.1	Operating bands for DC

Table 6.30.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1_n79-n258 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

6.30.2	Channel bandwidths per operating band for DC

Table 6.30.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| EN-DC configuration | Uplink configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n79A-n258A | DC_1A_n79A DC_1A_n258A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 520 |
|  |  | n79 | 15 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | n258 | 60 |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |

6.30.3	Co-existence studies

Co-existence studies of DC_1A_n79A are captured in 37.863-01-01 and for EN-DC between E-UTRA and FR2 DC_1A_n258A there is no requirements specified.

6.30.4	∆TIB and ∆RIB values

For DC_1_n79-n258, the TIB,c and RIB values are given in the tables below.

Table 6.30.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n79-n258 | 1 | 0 |
|  | n79 | 0 |
|  | n258 | 0 |

Table 6.30.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1_n79-n258 | 1 | 0 |
|  | n79 | 0 |
|  | n258 | 0 |

6.30.5	MSD

There is no additional MSD issue for DC_1_n79-n258.

6.31	DC_3_n77-n258

6.31.1	Operating bands for DC

Table 6.31.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n77-n258 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

6.31.2	Channel bandwidths per operating band for DC

Table 6.31.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| EN-DC configuration | Uplink configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n77A-n258A | DC_3A_n77A DC_3A_n258A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 520 |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n258 | 60 |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |

6.31.3	Co-existence studies

Co-existence studies of DC_3A_n77A are captured in 37.863-01-01 and for EN-DC between E-UTRA and FR2 DC_3A_n258A there is no requirements specified.

6.31.4	∆TIB and ∆RIB values

For DC_3_n77-n258, the TIB,c and RIB values are given in the tables below.

Table 6.31.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n77-n258 | 3 | 0.6 |
|  | n77 | 0.8 |
|  | n258 | 0 |

Table 6.31.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n77-n258 | 3 | 0.2 |
|  | n77 | 0.5 |
|  | n258 | 0 |

6.31.5	MSD

There is no additional MSD issue for DC_3_n77-n258.

6.32	DC_3_n78-n258

6.32.1	Operating bands for DC

Table 6.32.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n78-n258 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

6.32.2	Channel bandwidths per operating band for DC

Table 6.32.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| EN-DC configuration | Uplink configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n78A-n258A | DC_3A_n78A DC_3A_n258A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 520 |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n258 | 60 |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |

6.32.3	Co-existence studies

Co-existence studies of DC_3A_n78A are captured in 37.863-01-01 and for EN-DC between E-UTRA and FR2 DC_3A_n258A there is no requirements specified.

6.32.4	∆TIB and ∆RIB values

For DC_3_n78-n258, the TIB,c and RIB values are given in the tables below.

Table 6.32.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n78-n258 | 3 | 0.6 |
|  | n78 | 0.8 |
|  | n258 | 0 |

Table 6.32.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n78-n258 | 3 | 0.2 |
|  | n78 | 0.5 |
|  | n258 | 0 |

6.32.5	MSD

There is no additional MSD issue for DC_3_n78-n258.

6.33	DC_3_n79-n258

6.33.1	Operating bands for DC

Table 6.33.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n79-n258 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

6.33.2	Channel bandwidths per operating band for DC

Table 6.33.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| EN-DC configuration | Uplink configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n79A-n258A | DC_3A_n79A DC_3A_n258A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 520 |
|  |  | n79 | 15 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | n258 | 60 |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |

6.33.3	Co-existence studies

Co-existence studies of DC_3A_n79A are captured in 37.863-01-01 and for EN-DC between E-UTRA and FR2 DC_3A_n258A there is no requirements specified.

6.33.4	∆TIB and ∆RIB values

For DC_3_n79-n258, the TIB,c and RIB values are given in the tables below.

Table 6.33.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n79-n258 | 3 | 0 |
|  | n79 | 0 |
|  | n258 | 0 |

Table 6.33.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n79-n258 | 3 | 0 |
|  | n79 | 0 |
|  | n258 | 0 |

6.33.5	MSD

There is no additional MSD issue for DC_3_n79-n258.

6.34	DC_3-3_n78_n257

6.34.1	Operating bands for DC

Table 6.34.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-3_n78-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

6.34.2	Channel bandwidths per operating band for DC

Table 6.34.2-1: Supported bandwidths per DC one LTE band + inter-band NR 2DL/1UL

| DC configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS [kHz] | 10 |  | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_3A-3A_n78A-n257A | DC_3A_n78A DC_3A_n257A | 3 | 15 | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 540 |  |
|  |  | n78 | 15 | Yes |  | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |  |
| DC_3A-3A_n78A-n257D | DC_3A_n78A DC_3A_n257A | 3 | 15 | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 540 |  |
|  |  | n78 | 15 | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A_n78A-n257E | DC_3A_n78A DC_3A_n257A | 3 | 15 | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 740 |  |
|  |  | n78 | 15 | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A_n78A-n257F | DC_3A_n78A DC_3A_n257A | 3 | 15 | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 940 |  |
|  |  | n78 | 15 | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A_n78A-n257G | DC_3A_n78A DC_3A_n257A | 3 | 15 | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 340 |  |
|  |  | n78 | 15 | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A_n78A-n257H | DC_3A_n78A DC_3A_n257A | 3 | 15 | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 440 |  |
|  |  | n78 | 15 | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A_n78A-n257I | DC_3A_n78A DC_3A_n257A | 3 | 15 | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 540 |  |
|  |  | n78 | 15 | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A_n78A-n257J | DC_3A_n78A DC_3A_n257A | 3 | 15 | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 640 |  |
|  |  | n78 | 15 | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A_n78A-n257K | DC_3A_n78A DC_3A_n257A | 3 | 15 | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 740 |  |
|  |  | n78 | 15 | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A_n78A-n257L | DC_3A_n78A DC_3A_n257A | 3 | 15 | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 840 |  |
|  |  | n78 | 15 | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A_n78A-n257M | DC_3A_n78A DC_3A_n257A | 3 | 15 | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 940 |  |
|  |  | n78 | 15 | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.34.3	Co-existence studies

Based on co-existence studies of DC_3A-n78A and DC_3A_n257A specified in TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

6.34.4	∆TIB and ∆RIB values

For DC_3-3_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 6.34.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-3_n78-n257 | 3 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 6.34.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-3_n78-n257 | 3 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

6.34.5	MSD

No additional MSD requirement is needed.

6.35	DC_20_n3-n78

6.35.1	Operating bands for DC

Table 6.35.1-1: DC band combination of one LTE band + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_20_n3-n78 | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.35.2	Channel bandwidths per operating band for DC

Table 6.35.2-1: Supported bandwidths per DC band combination of one LTE band + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_20_n3-n78 | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 150 |
|  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.35.3	Co-existence studies

Co-existence studies of Band 20 + Band n3 + Band n78 are already captured in Table 5.3-1 of TR 37.716-21-21 [2].

6.35.4	∆TIB and ∆RIB values

For DC_20_n3-n78, the TIB,c and RIB values are same as for DC_3-20_n78 and are given in the tables below.

Table 6.35.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20_n3-n78 | 20 | 0.3 |
|  | n3 | 0.5 |
|  | n78 | 0.8 |

Table 6.35.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_20_n3-n78 | n3 | 0.2 |
|  | n78 | 0.5 |

6.35.5	MSD

Based on the co-existence studies in TR 37.716-21-21 [2] DC_20_n3-n78 need to have MSD added in table 7.3B.2.3.5.2-1 of TS 38.101-3.

Table 6.35.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_20A_n3A-n78A | 20 | 845 | 5 | 25 | 804 | N/A | FDD | N/A |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n78 | 3420 | 10 | 52 | 3420 | 16.1 | TDD | IMD3 |
|  | 20 | 845 | 5 | 25 | 804 | N/A | FDD | N/A |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n78 | 3500 | 10 | 52 | 3500 | 4.5 | TDD | IMD5 |
|  | 20 | 845 | 5 | 25 | 804 | N/A | FDD | N/A |
|  | n78 | 3550 | 10 | 52 | 3550 | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 15.7 | FDD | IMD3 |

6.36	DC_7_n1-n78

6.36.1	Operating bands for DC

Table 6.36.1-1: DC band combination of one LTE band + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n1-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.36.2	Channel bandwidths per operating band for DC

Table 6.36.2-1: Supported bandwidths per DC band combination of one LTE band + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | UL Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_7_n1-n78 |  | DC_7A_n1A DC_7A_n78A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 140 |
|  |  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_7C_n1A-n78A |  | DC_7A_n1A DC_7C_n1A DC_7A_n78A DC_7C_n78A | 7 | 15 | See CA_7C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 160 |
|  |  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.36.3	Co-existence studies

Co-existence studies of Band 7 + Band n1 + Band n78 are already captured in Table 5.3-1 of TR 37.716-21-21 [2].

6.36.4	∆TIB and ∆RIB values

For DC_7_n1-n78, the TIB,c and RIB values are same as for DC_1-7_n78 and are given in the tables below.

Table 6.36.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n1-n78 | 7 | 0.6 |
|  | n1 | 0.6 |
|  | n78 | 0.8 |

Table 6.36.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7_n1-n78 | 7 | 0.2 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |

6.36.5	MSD

Based on the co-existence studies in TR 37.716-21-21 [2] DC_7_n1-n78 need to have MSD added in table 7.3B.2.3.5.2-1 of TS 38.101-3.

Table 6.36.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_7A_n1A-n78A, DC_7C_n1A-n78A | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | 7 | 2520 | 5 | 25 | 2640 | N/A | FDD | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 10.1 | TDD | IMD4 |
|  | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | 7 | 2520 | 5 | 25 | 2640 | N/A | FDD | N/A |
|  | n78 | 3620 | 10 | 50 | 3620 | 3.8 | TDD | IMD5 |
|  | 7 | 2530 | 5 | 25 | 2650 | N/A | FDD | N/A |
|  | n78 | 3610 | 10 | 50 | 3610 | N/A | TDD | N/A |
|  | n1 | 1970 | 5 | 25 | 2160 | 9.0 | FDD | IMD4 |

6.37	DC_20_n1-n78

6.37.1	Operating bands for DC

Table 6.37.1-1: DC band combination of one LTE band + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_20_n1-n78 | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.37.2	Channel bandwidths per operating band for DC

Table 6.37.2-1: Supported bandwidths per DC band combination of one LTE band + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_20_n1-n78 | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 140 |
|  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.37.3	Co-existence studies

Co-existence studies of Band 20 + Band n1 + Band n78 are already captured in Table 5.3-1 of TR 37.716-21-21 [2].

6.37.4	∆TIB and ∆RIB values

For DC_20_n1-n78, the TIB,c and RIB values are same as for DC_1-20_n78 and are given in the tables below.

Table 6.37.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20_n1-n78 | 20 | 0.3 |
|  | n1 | 0.3 |
|  | n78 | 0.8 |

Table 6.37.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_20_n1-n78 | n78 | 0.5 |

6.37.5	MSD

Based on the co-existence studies in TR 37.716-21-21 [2] DC_20_n1-n78 need to have MSD added in table 7.3B.2.3.5.2-1 of TS 38.101-3.

Table 6.37.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_20A_n1A-n78A | 20 | 845 | 5 | 25 | 804 | N/A | FDD | N/A |
|  | n1 | 1940 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | n78 | 3630 | 10 | 52 | 3630 | 16.0 | TDD | IMD3 |
|  | 20 | 835 | 5 | 25 | 794 | N/A | FDD | N/A |
|  | n78 | 3790 | 10 | 52 | 3790 | N/A | TDD | N/A |
|  | n1 | 1930 | 5 | 25 | 2120 | 15.3 | FDD | IMD3 |

6.38	DC_7_n3-n78

6.38.1	Operating bands for DC

Table 6.38.1-1: DC band combination of one LTE band + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n3-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.38.2	Channel bandwidths per operating band for DC

Table 6.38.2-1: Supported bandwidths per DC band combination of one LTE band + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_7_n3-n78 | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 150 |
|  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.38.3	Co-existence studies

Co-existence studies of Band 7 + Band n3 + Band n78 are already captured in Table 5.3-1 of TR 37.716-21-21 [2].

6.38.4	∆TIB and ∆RIB values

For DC_7_n3-n78, the TIB,c and RIB values are same as for DC_3-7_n78 and are given in the tables below.

Table 6.38.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n3-n78 | 7 | 0.6 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |

Table 6.38.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7_n3-n78 | 7 | 0.2 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |

6.38.5	MSD

Based on the co-existence studies in TR 37.716-21-21 [2] DC_7_n3-n78 need to have MSD added in table 7.3B.2.3.5.2-1 of TS 38.101-3.

Table 6.38.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_7A_n3A-n78A | 7 | 2560 | 5 | 25 | 2680 | N/A | FDD | N/A |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n78 | 3390 | 10 | 52 | 3390 | 16.1 | TDD | IMD3 |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |
|  | n78 | 3310 | 10 | 52 | 3310 | N/A | TDD | N/A |
|  | n3 | 1725 | 5 | 25 | 1820 | 15.6 | FDD | IMD3 |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |
|  | n78 | 3480 | 10 | 52 | 3480 | N/A | TDD | N/A |
|  | n3 | 1735 | 5 | 25 | 1830 | 9.2 | FDD | IMD4 |

6.39				DC_40A_n41A-n79A

6.39.1		Operating bands for DC

Table 6.39.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_40-n41-n791,2 | 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |
| NOTE 1: The frequency range below 2506 MHz for Band n41 is not used in this band combination. NOTE 2: Applicable for frequency range above 4800 MHz for Band n79 in this band combination. |  |  |  |  |  |  |  |  |

6.39.2		Channel bandwidths per operating band for DC

Table 6.39.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_40A_n41A-n79A | DC_40A_n41A DC_40A_n79A | 40 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 220 |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes |  | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes |  | Yes |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |

6.39.3		Co-existence studies

Based on co-existence studies of Band 40 + Band n41, Band 40 + Band n79 captured in TR 37.716-11-11, the own Rx impact of the 3rd band is the followings, which is the same as that for CA_n40-n41-n79 captured in TR 38.716-03-02.

-	2nd  order IMD (4796 MHz – 5090 MHz) generated by 2UL of B40 + n41 fall into part of own band n79.

- 	4th order IMD (4210 MHz – 4704 MHz) generated by dual uplink of Band 40 + Band n41 may fall into part of own band n79. However, since this band combination will only be used in China where the frequency range above 4800 MHz for band n79 is allocated to IMT in China, there are no MSD issue.

-	As only inter-band carrier aggregation without simultaneous Rx/Tx between n40 and n41 is supported, there are no MSD issue for n41 due to dual uplink operation in Band 40 + Band n79

6.39.4		∆TIB and ∆RIB values

The same TIB,c and RIB,c values specified for 1 band UL for CA_n40-n41-n79 in TR 38.716-03-02 are used as below.

Table 6.39.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_40_n41-n79 | 40 | 0.51 |
|  | n41 | 0.51 |
|  | n79 | 0.8 |
| NOTE 1: Only applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx among band 40 and n41. |  |  |

Table 6.39.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_40_n41-n79 | 40 | 01 |
|  | n41 | 0.51 |
|  | n79 | 0.5 |
| NOTE 1: Only applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx among band 40 and n41. |  |  |

6.39.5		MSD

Based on the analysis in the UE-coexistence studies, compared to its fall back mode, there is additional MSD issue due to dual uplink DC_40A_n41A operation falling into n79 DL of the third band for this band combination. The required MSD are shown in the following table.

Table 6.39.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_40A_n41A-n79A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_40A_n41A-n79A | 40 | IMD2 | ¦fB40+fn41¦ | 2340 | 5 | 25 | 2340 | N/A |
|  | n41 |  |  | 2600 | 10 | 50 | 2600 |  |
|  | n79 |  |  | 4940 | 40 | 216 | 4940 | 30.5 |

6.40	DC_3-3_n1-n78

6.40.1	Operating bands for DC

Table 6.40.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-3_n1-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.40.2	Channel bandwidths per operating band for DC

Table 6.40.2-1: Supported bandwidths per DC one LTE band + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-3A_n1A-n78A | 3 | 15 | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 160 |
|  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.40.3	Co-existence studies

Based on co-existence studies of the constituent low order combination, DC_3A_n1A-n78A, The summary of self-interference issues for the 3rd own receive band are listed below.

-	2nd and 4th order IMD generated by dual uplink of Band 3 + Band n1 may also fall into own Rx of band n78

-	5th order IMD generated by dual uplink of Band 3 + Band n78 may also fall into own Rx of band n1

6.40.4	∆TIB and ∆RIB values

For DC_3-3_n1-n78, the TIB,c and RIB,c values are given in the tables below.

Table 6.40.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-3_n1-n78 | 3 | 0.6 |
|  | n1 | 0.6 |
|  | n78 | 0.8 |
| NOTE 1: HTF is assumed for Band 3 and Band n1. |  |  |

Table 6.40.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-3_n1-n78 | 3 | 0.2 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |
| NOTE 1: HTF is assumed for Band 3 and Band n1. |  |  |

6.40.5	MSD

The MSD needed can be covered by the constituent low order combination, DC_3_n1-n78. No additional MSD requirement is needed.

6.41	DC_7-7_n1-n78

6.41.1	Operating bands for DC

Table 6.41.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7-7_n1-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.41.2	Channel bandwidths per operating band for DC

Table 6.41.2-1: Supported bandwidths per DC one LTE band + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_7A-7A_n1A-n78A | 7 | 15 | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 160 |
|  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.41.3	Co-existence studies

Based on co-existence studies of the constituent low order combination, DC_7A_n1A-n78A, The summary of self-interference issues for the 3rd own receive band are listed below.

-	4th and 5th order IMD generated by dual uplink of Band 7 + Band n1 may also fall into own Rx of band n78

-	4th order IMD generated by dual uplink of Band 7 + Band n78 may also fall into own Rx of band n1

6.41.4	∆TIB and ∆RIB values

For DC_7-7_n1-n78, the TIB,c and RIB values are same as for DC_7_n1-n78 and are given in the tables below.

Table 6.41.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-7_n1-n78 | 7 | 0.6 |
|  | n1 | 0.6 |
|  | n78 | 0.8 |

Table 6.41.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-7_n1-n78 | 7 | 0.2 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |

6.41.5	MSD

The MSD needed can be covered by the constituent low order combination, DC_7_n1-n78. No additional MSD requirement is needed.

6.42	DC_1_n5-n78

6.42.1	Operating bands for DC

Table 6.42.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1_n5-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.42.2	Channel bandwidths per operating band for DC

Table 6.42.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Max BW [MHz] |
| DC_1A_n5A-n78A | DC_1A_n5A DC_1A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 140 |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |

6.42.3	Co-existence studies

Co-existence analysis for DC_1_n5 shows that there is IMD3 impact from DC_1_n5 UL to Band n78 DL.

Co-existence analysis for DC_1_n78 shows that there is IMD5 impact from DC_1_n78 UL to Band n5 DL.

6.42.4	∆TIB and ∆RIB values

For DC_1_n5-n78 the TIB,c and RIB,c values are given in the tables below.

Table 6.42.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n5-n78 | 1 | 0.3 |
|  | n5 | 0.6 |
|  | n78 | 0.8 |

Table 6.42.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1_n5-n78 | 1 | 0.2 |
|  | n5 | 0.2 |
|  | n78 | 0.5 |

6.42.5	MSD

MSD values by dual uplink are reused from DC_1A-5A_n78A for DC_1_n5-n78 band combination. So the below table is proposed to capture in TS38.101-3.

Table 6.42.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A_n5A-n78A | 1 | 1932 | 5 | 25 | 2122 | 18.1 | IMD3 ¦fn78-2*fB5¦ |
|  | n5 | 829 | 5 | 25 | 874 | N/A | N/A |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | N/A |
|  | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | n5 | 840 | 5 | 25 | 885 | 3.1 | IMD5 ¦2*fn78-3*fB1¦ |
|  | n78 | 3405 | 10 | 50 | 3405 | N/A | N/A |

6.43	DC_3_n5-n78

6.43.1	Operating bands for DC

Table 6.43.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n5-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.43.2	Channel bandwidths per operating band for DC

Table 6.43.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Max BW [MHz] |
| DC_3A_n5A-n78A | DC_3A_n5A DC_3A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 140 |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
| DC_3C_n5A-n78A | DC_3A_n5A DC_3C_n5A DC_3A_n78A DC_3C_n78A | 3 | 15 | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 160 |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |

6.43.3	Co-existence studies

Co-existence analysis for DC_3_n5 shows that there are IMD3 and IMD5 impact from DC_3_n5 UL to Band n78 DL.

Co-existence analysis for DC_3_n78 shows that there is no impact from DC_3_n78 UL to Band n5 DL.

6.43.4	∆TIB and ∆RIB values

For DC_3_n5-n78 the TIB,c and RIB,c values are given in the tables below.

Table 6.43.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n5-n78 | 3 | 0.6 |
|  | n5 | 0.6 |
|  | n78 | 0.8 |

Table 6.43.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n5-n78 | 3 | 0.2 |
|  | n5 | 0.2 |
|  | n78 | 0.5 |

6.43.5	MSD

MSD values by dual uplink are reused from DC_3A_n20A-n78A for DC_3_n5-n78 band combination. So the below table is proposed to capture in TS38.101-3.

Table 6.43.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A_n5A-n78A DC_3C_n5A-n78A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n5 | 845 | 5 | 25 | 890 | N/A | N/A |
|  | n78 | 3420 | 10 | 52 | 3420 | 16.1 | IMD3 ¦fB3+2*fn5¦ |

6.44	DC_7_n5-n78

6.44.1	Operating bands for DC

Table 6.44.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n5-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.44.2	Channel bandwidths per operating band for DC

Table 6.44.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Max BW [MHz] |
| DC_7A_n5A-n78A | DC_7A_n5A DC_7A_n78A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 140 |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
| DC_7C_n5A-n78A | DC_7A_n5A DC_7C_n5ADC_7A_n78A DC_7C_n78A | 7 | 15 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 160 |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |

6.44.3	Co-existence studies

Co-existence analysis for DC_7_n5 shows that there are IMD2 impact from DC_7_n5 UL to Band n78 DL.

Co-existence analysis for DC_7_n78 shows that there IMD2 and IMD5 impact from DC_7_n78 UL to Band n5 DL.

6.44.4	∆TIB and ∆RIB values

For DC_7_n5-n78 the TIB,c and RIB,c values are given in the tables below.

Table 6.44.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n5-n78 | 7 | 0.6 |
|  | n5 | 0.6 |
|  | n78 | 0.8 |

Table 6.44.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7_n5-n78 | 7 | 0.2 |
|  | n5 | 0.2 |
|  | n78 | 0.5 |

6.44.5	MSD

MSD values by dual uplink from UL 7_n78 into DL n5 are reused from DC_5A-7A_n78A for DC_7_n5-n78 band combination.

So the below table is proposed to capture in TS38.101-3.

Table 6.44.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A_n5A-n78A DC_7C_n5A-n78A | n5 | 834 | 5 | 25 | 879 | 30.2 | IMD2 ¦fn78-fB7¦ |
|  | 7 | 2550 | 5 | 25 | 2670 | N/A | N/A |
|  | n78 | 3429 | 10 | 50 | 3429 | N/A | N/A |
|  | n5 | 830 | 5 | 25 | 875 | 3.3 | IMD5 ¦2*fn78-3fB7¦ |
|  | 7 | 2525 | 5 | 25 | 2645 | N/A | N/A |
|  | n78 | 3330 | 10 | 50 | 3350 | N/A | N/A |
|  | 7 | 2540 | 5 | 25 | 2660 | N/A | N/A |
|  | n5 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | n78 | 3375 | 10 | 50 | 3375 | 29.7 | IMD2 ¦fB7 +fn5¦ |

6.45	DC_28_n5-n78

6.45.1	Operating bands for DC

Table 6.45.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28_n5-n78 | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.45.2	Channel bandwidths per operating band for DC

Table 6.45.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Max BW [MHz] |
| DC_28A_n5A-n78A | DC_28A_n5A DC_28A_n78A | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 140 |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |

6.45.3	Co-existence studies

Co-existence analysis for DC_28_n5 shows that there is IMD5 impact from DC_28_n5 UL to Band n78 DL.

Co-existence analysis for DC_28_n78 shows that there is IMD5 impact from DC_28_n78 UL to Band n5 DL.

6.45.4	∆TIB and ∆RIB values

For DC_28_n5-n78 the TIB,c and RIB,c values are reused from DC_18-28_n78 and are given in the tables below.

Table 6.45.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28_n5-n78 | 28 | 0.5 |
|  | n5 | 0.5 |
|  | n78 | 0.8 |

Table 6.45.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_28_n5-n78 | n78 | 0.5 |

6.45.5	MSD

MSD values for impact from UL 28_n78 into DL n5 are reused from DC_18A-28A_n78A. So the below table is proposed to capture in TS38.101-3.

Table 6.45.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_28A_n5A-n78A | 28 | 723 | 5 | 25 | 778 | N/A | N/A |
|  | n5 | 829 | 5 | 25 | 874 | 3.8 | IMD5 ¦fn78 -4*fB28¦ |
|  | n78 | 3721 | 10 | 50 | 3721 | N/A | N/A |
|  | 28 | 723 | 5 | 25 | 778 | N/A | N/A |
|  | n5 | 829 | 5 | 25 | 874 | N/A | N/A |
|  | n78 | 3721 | 10 | 50 | 3721 | 4.7 | IMD5 ¦fn5 +4*fB28¦ |

6.46	DC_2A_n41A-n261A

6.46.1	Operating bands for DC

Table 6.46.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2A_n41A-n261A | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n261 | 27500 MHz | – | 28350 MHz | 27500 MHz | – | 28350 MHz |  |

6.46.2	Channel bandwidths per operating band for DC

Table 6.46.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | Uplink Configurations | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_2A_n41A-n261A | DC_2A_n41A | This part can be omitted unless there is some particular reason. |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A_n41A-n261(2A) | DC_2A_n41A | See CA_ n261(2A) BCS0 in 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |

6.46.3	Co-existence studies

Based on co-existence studies of DC_2A-n41A presented in TR 37.716-01-01, there is no harmonic or intermodulation impact for the own receive band.

6.46.4	∆TIB and ∆RIB values

For DC_2_n41-n261, the TIB,c and RIB,c values are given in the tables below. FR2 does not have relaxation.

Table 6.46.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n41 | 2 | 0.5 |
|  | n41 | 0.41 |
|  |  | 0.92 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

Table 6.46.4-2: ΔRIB

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2_n41 | 2 | 0 |
|  | n41 | 0 |

6.46.5	MSD

As mentioned in 6.46.3, there is no need to specify additional MSD requirement for this UL DC configuration.

6.47	DC_66A_n41A-n261A

6.47.1	Operating bands for DC

Table 6.47.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66A_n41A-n261A | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n261 | 27500 MHz | – | 28350 MHz | 27500 MHz | – | 28350 MHz |  |

6.47.2	Channel bandwidths per operating band for DC

Table 6.47.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | Uplink Configurations | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_66A_n41A-n261A | DC_66A_n41A | This part can be omitted unless there is some particular reason. |  |  |  |  |  |  |  |  |  |  |  |
| DC_66A_n41A-n261(2A) | DC_66A_n41A | See CA_ n261(2A) BCS0 in 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |

6.47.3	Co-existence studies

Co-existence studies have been done for DC_66_n41 and are captured into TR 37.716-11-11.

6.47.4	∆TIB and ∆RIB values

For DC_66_n41-n261, the TIB,c and RIB,c values are same as for DC_66_n41 which is already specified in TS 38.101-3. n261 TIB,c and RIB,c are 0 dB.

Table 6.47.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n41-n261 | 66 | 0.5 |
|  | n41 | [0.81] |
|  |  | [1.32] |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

Table 6.47.4-2: ΔRIB

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_66_n41-261 | 66 | 0.5 |
|  | n41 | [0.51] |
|  |  | [12] |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

6.47.5	MSD

Based on the studies made for DC_66_n41 MSD due to close proximity is needed and is already specified in TS 38.101-3.

6.48	DC_2_n41-n260

6.48.1	Operating bands for DC

Table 6.48.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2_n41-n260 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

6.48.2	Channel bandwidths per operating band for DC

Table 6.48.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Max BW [MHz] |
| DC_2A_n41A-n260A | DC_2A_n41A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 520 |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n260 | 60 |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |
| DC_2A_n41A-n260(2A) | DC_2A_n41A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 920 |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n260 | See CA_n260(2A) in Table 5.5A.2-1 of TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A_n41A-n260(3A) | DC_2A_n41A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 1320 |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n260 | See CA_n260(3A) in Table 5.5A.2-1 of TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A_n41A- n260(4A) | DC_2A_n41A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 1720 |
|  |  | n41 | 15 |  | Yes | § | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n260 | See CA_n260(4A) in Table 5.5A.2-1 of TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.48.3	Co-existence studies

Co-existence analysis for DC_2_n41 shows that there is no impact from DC_2_n41 UL to Band n260 DL.

Co-existence analysis for DC_2_n260 shows that there is no impact from DC_2_n260 UL to Band n41 DL.

6.48.4	∆TIB and ∆RIB values

For DC_2_n41-n260 the TIB,c and RIB,c values are given in the tables below.

Table 6.48.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n41-n260 | 2 | 0.5 |
|  | n41 | 0.41 |
|  |  | 0.92 |
|  | n260 | 0 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545 MHz. |  |  |

Table 6.48.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2_n41-n260 | 2 | 0 |
|  | n41 | 0 |
|  | n260 | 0 |

6.48.5	MSD

No further MSD is needed.

6.49	DC_66_n41-n260

6.49.1	Operating bands for DCTable 6.49.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n41-n260 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

6.49.2	Channel bandwidths per operating band for DC

Table 6.49.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_66A_n41A-n260A | DC_66A_n41A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 520 |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n260 | 60 |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |
| DC_66A_n41A-n260(66A) | DC_66A_n41A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 920 |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n260 | See CA_n260(2A) in Table 5.5A.2-1 of TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_66A_n41A-n260(3A) | DC_66A_n41A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 1320 |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n260 | See CA_n260(3A) in Table 5.5A.2-1 of TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_66A_n41A- n260(4A) | DC_66A_n41A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 1720 |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n260 | See CA_n260(4A) in Table 5.5A.2-1 of TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.49.3	Co-existence studies

Co-existence analysis for DC_66_n41 shows that there is no impact from DC_66_n41 UL to Band n260 DL.

Co-existence analysis for DC_66_n260 shows that there is no impact from DC_66_n260 UL to Band n41 DL.

6.49.4	∆TIB and ∆RIB values

For DC_66_n41-n260 the TIB,c and RIB,c values are given in the tables below.

Table 6.49.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n41-n260 | 66 | 0.5 |
|  | n41 | 0.81 |
|  |  | 1.32 |
|  | n260 | 0 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545 MHz. |  |  |

Table 6.49.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n41-n260 | 66 | 0.5 |
|  | n41 | 0.51 |
|  |  | 12 |
|  | n260 | 0 |

6.49.5	MSD

No further MSD is needed.

6.50	DC_3_n77-n257

6.50.1	Operating bands for DC

Table 6.50.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n77-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

6.50.2	Channel bandwidths per operating band for DC

Table 6.50.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_3A_n77A-n257A | DC_3A_n77A DC_3A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 520 |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes |  |
| DC_3A_n77A-n257D | DC_3A_n77A DC_3A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 520 |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A_n77A-n257G | DC_3A_n77A DC_3A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 320 |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A_n77A-n257H | DC_3A_n77A DC_3A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 420 |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A_n77A-n257I | DC_3A_n77A DC_3A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 520 |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |

6.50.3	Co-existence studies

Based on the co-existence studies of DC_3A-n77A and DC_3A-n257A are captured in TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

6.50.4	∆TIB and ∆RIB values

For DC_3_n77-n257, the TIB,c and RIB,c values are given in the tables below.

Table 6.50.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n77-n257 | 3 | 0.6 |
|  | n77 | 0.8 |
|  | n257 | 0 |

Table 6.50.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n77-n257 | 3 | 0.2 |
|  | n77 | 0.5 |
|  | n257 | 0 |

6.50.5	MSD

As mentioned in 6.50.3, there is no need to specify additional MSD requirement for this UL DC configuration.

6.51	DC_8_n3-n28

6.51.1	Operating bands for DC

Table 6.51.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n3-n28 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

6.51.2	Channel bandwidths per operating band for DC

Table 6.51.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_8_n3-n28 | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 60 |
|  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.51.3	Co-existence studies

Based on the co-existence studies of DC_8A-n3A and DC_8A-n28A captured in TR 37.716-11-11 and 37.863-01-01, 2nd and 5th order IMD generated by dual uplink of Band 8 + Band n3 may also fall into own Rx of band n28.

6.51.4	∆TIB and ∆RIB values

For DC_8A_n3A-n28A, the TIB,c and RIB,c values are given in the tables below.

Table 6.51.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n3-n28 | 8 | 0.6 |
|  | n3 | 0.3 |
|  | n28 | 0.5 |

Table 6.51.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n3-n28 | 8 | 0.2 |
|  | n3 | 0 |
|  | n28 | 0.1 |

### 6.51.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there are IMD2 & IMD5 products produced by Band 8 and n3 that impact the reference sensitivity of NR band n28. The required MSD are shown in the following table.

Table 6.51.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_8A_n3A-n28A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_8A_n3A-n28A | 8 | IMD2 | ¦fB8-fn3¦ | 912.5 | 5 | 25 | 957.5 | N/A |
|  | n3 |  |  | 1712.5 | 5 | 25 | 1807.5 |  |
|  | n28 |  |  | 745 | 5 | 25 | 800 | 30.4 |
|  | 8 | IMD5 | ¦3*fB8 -2*fn3¦ | 910 | 5 | 25 | 955 | N/A |
|  | n3 |  |  | 1750 | 5 | 25 | 1845 |  |
|  | n28 |  |  | 715 | 5 | 25 | 770 | 2.6 |

6.52	DC_11_n77-n257

6.52.1	Operating bands for DC

Table 6.52.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_11_n77-n257 | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

6.52.2	Channel bandwidths per operating band for DC

Table 6.52.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_11A_n77A-n257A | DC_11A_n77A DC_11A_n257A | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 510 |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes |  |
| DC_11A_n77A-n257D | DC_11A_n77A DC_11A_n257A | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 510 |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_11A_n77A-n257G | DC_11A_n77A DC_11A_n257A | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 310 |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_11A_n77A-n257H | DC_11A_n77A DC_11A_n257A | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 410 |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_11A_n77A-n257I | DC_11A_n77A DC_11A_n257A | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 510 |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |

6.52.3	Co-existence studies

Based on the co-existence studies of DC_11A-n77A and DC_11A-n257A captured in TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

6.52.4	∆TIB and ∆RIB values

For DC_11_n77-n257, the TIB,c and RIB,c values are given in the tables below.

Table 6.52.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_11_n77-n257 | 11 | 0.4 |
|  | n77 | 0.8 |
|  | n257 | 0 |

Table 6.52.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_11_n77-n257 | 11 | 0 |
|  | n77 | 0.5 |
|  | n257 | 0 |

6.52.5	MSD

As mentioned in 6.52.3, there is no need to specify additional MSD requirement for this UL DC configuration.

6.53		DC_3A_n40A-n41A

6.53.1	Operating bands for DC

Table 6.53.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A_n40A-n41A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

6.53.2	Channel bandwidths per operating band for DC

Table 6.53.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Configuration | Uplink configuration | E-UTRA and NR Band | SCS [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n40A-n41A | DC_3A_n40A DC_3A_n41A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 210 |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.53.3	Co-existence studies

The co-existence studies of Band 3 + Band n40 and Band 3 + Band n41 have been captured in TR 37.863-01-01 and TR 37.716-11-11, where there are no additional harmonic and IMD impact for the additional band receiver.

    - 6.53.4	∆TIB and ∆RIB values

For DC_3A_n40A-n41A, the TIB,c and RIB values are given in the tables below.

Table 6.53.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A_n40A-n41A | 3 | 0.5 |
|  | n40 | 0.5 |
|  | n41 | 0.51 |
|  |  | 0.82 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. NOTE 3: Only applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx among band 40 and 41. |  |  |

Table 6.53.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A_n40A-n41A | 3 | 0 |
|  | n40 | 0 |
|  | n41 | 01 |
|  |  | 0.52 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. NOTE 3: Only applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx among band 40 and 41. |  |  |

6.53.5	MSD

According the co-existing studies, there is no need to specify additional MSD requirement for this UL DC configuration.

6.54	DC_8A_n1A-n78A

6.54.1	Operating bands for DC

Table 6.54.1-1: DC band combination of one LTE band + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8A_n1A-n78A | 8 | 880 MHz |  | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.54.2	Channel bandwidths per operating band for DC

Table 6.54.2-1: Supported bandwidths per DC band combination of one LTE band + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_8A_n1A-n78A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 130 |
|  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.54.3	Co-existence studies

The 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products for the UE coexistence analysis of DC_8A_n1A-n78A with dual uplink on DC_8A_n1A and DC_8A_n78A were calculated and listed in Table 6.54.3-1 and 6.54.3-2 respectively.

Table 6.54.3-1: Harmonic and IMD analysis for DC_8_n1-n78 with UL on DC_8_n1

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 5760 | 5940 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1005 | 1100 | 2800 | 2895 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 220 | 90 | 2925 | 3080 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3680 | 3810 | 4720 | 4875 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 660 | 825 | 4845 | 5060 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4560 | 4725 | 6640 | 6855 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2200 | 2010 | 5600 | 5790 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7040 | 6765 | 1740 | 1540 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8560 | 8835 | 5440 | 5640 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4180 | 3930 | 1095 | 1320 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7520 | 7770 | 6480 | 6705 |

Table 6.54.3-2: Harmonic and IMD analysis for DC_8_n1-n78 with UL on DC_8_n78

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 9900 | 11400 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2385 | 2920 | 4180 | 4715 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2040 | 1470 | 5685 | 6720 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5060 | 5630 | 7480 | 8515 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1160 | 555 | 8985 | 10520 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5940 | 6545 | 10780 | 12315 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 5840 | 4770 | 8360 | 9430 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14320 | 12285 | 360 | 280 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14080 | 16115 | 6820 | 7460 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 9640 | 8070 | 3855 | 4960 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11660 | 13230 | 9240 | 10345 |

The coexistence studies of Band 8 + Band n1 and Band 8 + Band n78 will be captured in the constituent fallback modes. The summary of self-interference issues for the 3rd own receive band are listed below.

Based on Table 6.54.3-1 and Table 6.54.3-2:

-	3rd order IMD generated by dual uplink of Band 8 + Band n1 may fall into own Rx of band n78

6.54.4	∆TIB and ∆RIB values

For DC_8A_n1A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.54.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n1-n78 | 8 | 0.6 |
|  | n1 | 0.3 |
|  | n78 | 0.8 |

Table 6.54.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n1-n78 | 8 | 0.2 |
|  | n1 | 0 |
|  | n78 | 0.5 |

6.54.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD3 products produced by Band 8 and n1 that impact the reference sensitivity of NR band n78. The required MSD is shown in the following table.

Table 6.54.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_8A_n1A-n78A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_8A_n1A-n78A | 8 | IMD3 | ¦2*fB8+fn1¦ | 900 | 5 | 25 | 945 | N/A |
|  | n1 |  |  | 1945 | 5 | 25 | 2135 |  |
|  | n78 |  |  | 3745 | 10 | 50 | 3745 | 14.9 |

6.55	DC_66_n41-n71

6.55.1	Operating bands for DC

Table 6.55.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n41-n71 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

6.55.2	Channel bandwidths per operating band for DC

Table 6.55.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Max BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_66A_n41A-n71A | DC_66A_n41A DC_66A_n71A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 140 |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n71 | 30 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |

6.55.3	Co-existence studies

Co-existence analysis for DC_66_n41 shows that there is no impact from DC_66_n41 UL to Band n71 DL.

Co-existence analysis for DC_66_n71 shows that there ia no impact from DC_66_n71 UL to Band n41 DL.

6.55.4	∆TIB and ∆RIB values

For DC_66_n41-n71 the TIB,c and RIB,c values are derived from LTE combination CA_4-7-28 and are given in the tables below.

Table 6.55.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n41-n71 | 66 | 0.5 |
|  | n41 | 0.81 |
|  |  | 1.32 |
|  | n71 | 0.6 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

Table 6.55.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n41-n71 | 66 | 0.5 |
|  | n41 | 0.51 |
|  |  | 12 |
|  | n71 | 0.5 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

6.55.5	MSD

Based on the co-existence studies there is no need to define MSD values.

6.56	DC_8_n78-n257

6.56.1	Operating bands for DC

Table 6.56.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n78-n257 | 8 | 880MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

6.56.2	Channel bandwidths per operating band for DC

Table 6.56.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

|  |  |  | CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_8A_n78A-n257A | DC_8A_n78A DC_8A_n257A |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 510 |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | n257 | 60 |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  |  | 120 |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |
| DC_8A_n78A-n257D | DC_8A_n78A DC_8A_n257A |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 510 |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | n257 |  | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_8A_n78A-n257E | DC_8A_n78A DC_8A_n257A |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 710 |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | n257 |  | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_8A_n78A-n257F | DC_8A_n78A DC_8A_n257A |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 910 |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | n257 |  | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_8A_n78A-n257G | DC_8A_n78A DC_8A_n257A |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 310 |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | n257 |  | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_8A_n78A-n257H | DC_8A_n78A DC_8A_n257A |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 410 |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | n257 |  | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_8A_n78A-n257I | DC_8A_n78A DC_8A_n257A |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 510 |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | n257 |  | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_8A_n78A-n257J | DC_8A_n78A DC_8A_n257A |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 610 |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | n257 |  | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_8A_n78A-n257K | DC_8A_n78A DC_8A_n257A |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 710 |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | n257 |  | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_8A_n78A-n257L | DC_8A_n78A DC_8A_n257A |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 810 |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | n257 |  | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_8A_n78A-n257M | DC_8A_n78A DC_8A_n257A |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 910 |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | n257 |  | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |

6.56.3	Co-existence studies

Based on co-existence studies of DC_8A-n78A and DC_8A_n257A specified in TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the third band receiver.

- DC_8A_n78A: Harmonic problem is already specified in TR 37.863-01-01.

- DC_8A_n257A: No harmonic problems by 7th & 8th order between LTE and FR2

6.56.4	∆TIB and ∆RIB values

For DC_8_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 6.56.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n78-n257 | 8 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |
| NOTE 1: HTF is assumed for Band 8. |  |  |

Table 6.56.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_8_n78-n257 | 8 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |
| NOTE 1: HTF is assumed for Band 8. |  |  |

6.56.5	MSD

As mentioned in 6.56.3, there is no need to specify additional MSD requirement for this UL DC configuration.

6.57	DC_2_n41-n71

6.57.1	Operating bands for DC

Table 6.57.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2_n41-n71 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

6.57.2	Channel bandwidths per operating band for DC

Table 6.57.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Max BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A_n41A-n71A | DC_2A_n41A DC_2A_n71A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 140 |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n71 | 30 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |

6.57.3	Co-existence studies

As can be seen in the co-existence analysis for DC_2_n41 in TR [2] shows that there are IMD2 and IMD5 impact from DC_2_n41 UL to Band n71 DL.

As can be seen in the co-existence analysis for DC_2_n71 in TR [3] shows that there IMD2 impact from DC_2_n71 UL to Band n41 DL.

6.57.4	∆TIB and ∆RIB values

For DC_2_n41-n71 the TIB,c and RIB,c values are derived from LTE combinations CA_2-7-12, CA_2-7-28, CA_3-7-28 and DC_3-7_n28 and are given in the tables below.

Table 6.57.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n41-n71 | 2 | 0.5 |
|  | n41 | 0.5 |
|  | n71 | 0.3 |

Table 6.57.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2_n41-n71 | 2 | 0 |
|  | n41 | 0 |
|  | n71 | 0 |

6.57.5	MSD

MSD values for UL 2_n41 into DL n71 and UL 2_n71 into DL n41 are needed. Below table is to be captured in Table 7.3B.2.3.5.2-1 of TS 38.101-3.

Table 6.57.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A_n41A-n71A | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n41 | 2530 | 10 | 50 | 2530 | N/A | N/A |
|  | n71 | 676 | 5 | 50 | 630 | 28.7 | IMD2 |
|  | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n41 | 2530 | 10 | 50 | 2530 | N/A | N/A |
|  | n71 | 686 | 5 | 50 | 640 | 3.8 | IMD5 |
|  | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n41 | 2586 | 10 | 50 | 2586 | 29.2 | IMD2 |
|  | n71 | 686 | 5 | 50 | 640 | N/A | N/A |

6.58	DC_1_n3-n28

6.58.1	Operating bands for DC

Table 6.58.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1_n3-n28 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

6.58.2	Channel bandwidths per operating band for DC

Table 6.58.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A_n3A-n28A | DC_1A_n3A DC_1A_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 70 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.58.3	Co-existence studies

Based on the co-existence studies of DC_1A-n3A and DC_1A-n28A captured in TR 37.716-11-11 and 37.863-01-01, 5th order IMD generated by dual uplink of Band 1 + Band n28 may also fall into own Rx of band n3.

6.58.4	∆TIB and ∆RIB values

For DC_1_n3-n28, the TIB,c and RIB,c values are given in the tables below.

Table 6.58.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n3-n28 | 1 | 0.3 |
|  | n3 | 0.3 |
|  | n28 | 0.6 |

Table 6.58.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1_n3-n28 | 1 | 0 |
|  | n3 | 0 |
|  | n28 | 0.2 |

6.58.5	MSD

As mentioned in 6.58.3, IMD5 of B1 and n28 to Band n3 Rx need to be addressed for REFSENS relaxation. The following values are proposed:

Table 6.58.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order | Single UL allowed |
| DC_1A-n3A_n28A | 1 | 1975 | 5 | 25 | 2165 | N/A | FDD | N/A |  |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | FDD | N/A |  |
|  | n3 | 1723.5 | 5 | 25 | 1818.5 | 4.0 | FDD | IMD5 |  |

6.59 DC_39_n41-n79

[Editor Note]: DC operating bands,DL/UL configuration, coexistence and delta Tib/Rib will be updated.

### 6.59.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there are IMD2 & IMD5 products produced by Band 39 and n41 that impact the reference sensitivity of NR band n79.

In addition, there are IMD2 & IMD5 products produced by Band 39 and n79 that impact the reference sensitivity of NR band n41. The required MSD are shown in the following table.

Table 6.59.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_39A_n41A-n79A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_39A_n41A-n79A | 39 | IMD2 | ¦fB39-fn41¦ | 1900 | 5 | 25 | 1900 | N/A |
|  | n41 |  |  | 2620 | 10 | 50 | 2620 |  |
|  | n79 |  |  | 4520 | 40 | 216 | 4520 | 29.8 |
|  | 39 | IMD5 | ¦4*fB39 -fn41¦ | 1885 | 5 | 25 | 1885 | N/A |
|  | n41 |  |  | 2660 | 10 | 50 | 2660 |  |
|  | n79 |  |  | 4880 | 40 | 216 | 4880 | 4.4 |
|  | 39 | IMD2 | ¦fB39 -fn79¦ | 1900 | 5 | 25 | 1900 | N/A |
|  | n79 |  |  | 4520 | 40 | 216 | 4520 |  |
|  | n41 |  |  | 2620 | 10 | 50 | 2620 | 30.2 |
|  | 39 | IMD5 | ¦4*fB39 -fn79¦ | 1885 | 5 | 25 | 1885 | N/A |
|  | n79 |  |  | 4880 | 40 | 216 | 4880 |  |
|  | n41 |  |  | 2660 | 10 | 50 | 2660 | 4.3 |

6.60	DC_12_n7-n78

[Editor Note]: DC operating bands,DL/UL configuration, coexistence and delta Tib/Rib will be updated.

### 6.60.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD4 products produced by Band 12 and n7 that impact the reference sensitivity of NR band n78.

In addition, there is IMD2 product produced by Band 12 and n78 that impact the reference sensitivity of NR Band n7. The required MSD are shown in the following table.

Table 6.60.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_12A_n7A-n78A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_12A_n7A-n78A | 12 | IMD4 | ¦2*fB12 -2*fn7¦ | 708 | 5 | 25 | 738 | N/A |
|  | n7 |  |  | 2520 | 5 | 25 | 2640 |  |
|  | n78 |  |  | 3624 | 10 | 50 | 3624 | 8.4 |
|  | 12 | IMD2 | ¦fB12 -fn78¦ | 708 | 5 | 25 | 738 | N/A |
|  | n78 |  |  | 3360 | 10 | 50 | 3360 |  |
|  | n7 |  |  | 2542 | 5 | 25 | 2662 | 28.7 |

6.61	DC_2_n66-n78

[Editor Note]: DC operating bands,DL/UL configuration, coexistence and delta Tib/Rib will be updated.

### 6.61.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there are IMD2 & IMD4 products produced by Band 2 and n66 that impact the reference sensitivity of NR band n78. The required MSD are shown in the following table.

Table 6.61.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_2A_n66A-n78A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A_n66A-n78A | 2 | IMD2 | ¦fB2+fn66¦ | 1880 | 5 | 25 | 1960 | N/A |
|  | n66 |  |  | 1740 | 5 | 25 | 2140 |  |
|  | n78 |  |  | 3620 | 10 | 50 | 3620 | 29.4 |
|  | 2 | IMD4 | ¦fB2 -3*fn66¦ | 1880 | 5 | 25 | 1960 | N/A |
|  | n66 |  |  | 1740 | 5 | 25 | 2140 |  |
|  | n78 |  |  | 3340 | 10 | 50 | 3340 | 8.9 |

6.62	DC_12_n66-n78

[Editor Note]: DC operating bands,DL/UL configuration, coexistence and delta Tib/Rib will be updated.

### 6.62.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD5 products produced by Band 12 and n66 that impact the reference sensitivity of NR band n78.

In addition, there is IMD3 product produced by Band 12 and n78 that impact the reference sensitivity of NR Band n66. The required MSD are shown in the following table.

Table 6.62.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_12A_n66A-n78A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_12A_n66A-n78A | 12 | IMD5 | ¦2*fB12-3*fn66¦ | 703 | 5 | 25 | 733 | N/A |
|  | n66 |  |  | 1720 | 5 | 25 | 2120 |  |
|  | n78 |  |  | 3754 | 10 | 50 | 3754 | 4.1 |
|  | 12 | IMD3 | ¦2*fB12 -fn78¦ | 703 | 5 | 25 | 733 | N/A |
|  | n78 |  |  | 3546 | 10 | 50 | 3546 |  |
|  | n66 |  |  | 1740 | 5 | 25 | 2140 | 16.5 |

6.63	DC_7_n66-n78

[Editor Note]: DC operating bands,DL/UL configuration, coexistence and delta Tib/Rib will be updated.

### 6.63.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD3 products produced by Band 7 and n66 that impact the reference sensitivity of NR band n78. The required MSD is shown in the following table.

Table 6.63.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_7A_n66A-n78A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_7A_n66A-n78A | 7 | IMD3 | ¦2*fB7-fn66¦ | 2542 | 5 | 25 | 2662 | N/A |
|  | n66 |  |  | 1740 | 5 | 25 | 2140 |  |
|  | n78 |  |  | 3344 | 10 | 50 | 3344 | 16.0 |

6.64	DC_20_n1-n28

[Editor Note]: DC operating bands,DL/UL configuration, coexistence and delta Tib/Rib will be updated.

### 6.64.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD5 products produced by Band 20 and n28 that impact the reference sensitivity of NR band n1. The required MSD is shown in the following table.

Table 6.64.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_20A_n1A-n28A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_20A_n1A-n28A | 20 | IMD5 | ¦fB20-4*fn28¦ | 837 | 5 | 25 | 796 | N/A |
|  | n28 |  |  | 738 | 5 | 25 | 793 |  |
|  | n1 |  |  | 1925 | 5 | 25 | 2115 | 3.3 |

6.65	DC_20_n7-n28

[Editor Note]: DC operating bands,DL/UL configuration, coexistence and delta Tib/Rib will be updated.

### 6.65.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there are IMD3 & IMD5 products produced by Band 20 and n7 that impact the reference sensitivity of NR band n28. The required MSD are shown in the following table.

Table 6.65.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_20A_n7A-n28A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_20A_n7A-n28A | 20 | IMD3 | ¦2*fB20-fn7¦ | 857 | 5 | 25 | 816 | N/A |
|  | n7 |  |  | 2512 | 5 | 25 | 2632 |  |
|  | n28 |  |  | 743 | 5 | 25 | 798 | 15.7 |
|  | 20 | IMD5 | ¦4*fB20-fn7¦ | 837 | 5 | 25 | 796 | N/A |
|  | n7 |  |  | 2555 | 5 | 25 | 2675 |  |
|  | n28 |  |  | 738 | 5 | 25 | 793 | 2.8 |

6.66	DC_20_n7-n78

[Editor Note]: DC operating bands,DL/UL configuration, coexistence and delta Tib/Rib will be updated.

### 6.66.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there are IMD2 & IMD4 products produced by Band 20 and n7 that impact the reference sensitivity of NR band n78. The required MSD are shown in the following table.

Table 6.66.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_20A_n7A-n78A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_20A_n7A-n78A | 20 | IMD2 | ¦fB20+fn7¦ | 837 | 5 | 25 | 796 | N/A |
|  | n7 |  |  | 2555 | 5 | 25 | 2675 |  |
|  | n78 |  |  | 3392 | 10 | 50 | 3392 | 28.8 |
|  | 20 | IMD4 | ¦2*fB20-2*fn7¦ | 837 | 5 | 25 | 796 | N/A |
|  | n7 |  |  | 2555 | 5 | 25 | 2675 |  |
|  | n78 |  |  | 3436 | 10 | 50 | 3436 | 8.5 |

6.67	DC_3_n7-n28

[Editor Note]: DC operating bands,DL/UL configuration, coexistence and delta Tib/Rib will be updated.

### 6.67.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD2 products produced by Band 3 and n7 that impact the reference sensitivity of NR band n28. The required MSD is shown in the following table.

Table 6.67.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_3A_n7A-n28A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n7A-n28A | 3 | IMD2 | ¦fB3-fn7¦ | 1730 | 5 | 25 | 1825 | N/A |
|  | n7 |  |  | 2518 | 5 | 25 | 2638 |  |
|  | n28 |  |  | 733 | 5 | 25 | 788 | 29.0 |

6.68	DC_18_n3-n78

[Editor Note]: DC operating bands,DL/UL configuration, coexistence and delta Tib/Rib will be updated.

### 6.68.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD3 & IMD5 products produced by Band 18 and n3 that impact the reference sensitivity of NR band n78. The required MSD are shown in the following table.

Table 6.68.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_18A_n3A-n78A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_18A_n3A-n78A | 18 | IMD3 | ¦2*fB18+fn3¦ | 820 | 5 | 25 | 865 | N/A |
|  | n3 |  |  | 1750 | 5 | 25 | 1845 |  |
|  | n78 |  |  | 3390 | 10 | 50 | 3390 | 15.2 |
|  | 18 | IMD5 | ¦2*fB18 -3*fn3¦ | 820 | 5 | 25 | 865 | N/A |
|  | n3 |  |  | 1750 | 5 | 25 | 1845 |  |
|  | n78 |  |  | 3610 | 10 | 50 | 3610 | 4.0 |

6.69	DC_3_n40-n79

[Editor Note]: DC operating bands,DL/UL configuration, coexistence and delta Tib/Rib will be updated.

### 6.69.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD5 products produced by Band 3 and n40 that impact the reference sensitivity of NR band n79.

In addition, there is IMD5 product produced by Band 3 and n79 that impact the reference sensitivity of NR Band n40. The required MSD are shown in the following table.

Table 6.69.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_3A_n40A-n79A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n40A-n79A | 3 | IMD5 | ¦4*fB3-fn40¦ | 1720 | 5 | 25 | 1815 | N/A |
|  | n40 |  |  | 2330 | 5 | 25 | 2330 |  |
|  | n79 |  |  | 4550 | 40 | 216 | 4550 | 4.7 |
|  | 3 | IMD5 | ¦4*fB3-fn79¦ | 1720 | 5 | 25 | 1815 | N/A |
|  | n79 |  |  | 4550 | 40 | 216 | 4550 |  |
|  | n40 |  |  | 2330 | 5 | 25 | 2330 | 3.2 |

6.70	DC_3_n41-n79

[Editor Note]: DC operating bands,DL/UL configuration, coexistence and delta Tib/Rib will be updated.

### 6.70.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there are IMD2 & IMD5 products produced by Band 3 and n41 that impact the reference sensitivity of NR band n79. The required MSD are shown in the following table.

Table 6.70.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_3A_n41A-n79A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n41A-n79A | 3 | IMD2 | ¦fB3-fn41¦ | 1770 | 5 | 25 | 1865 | N/A |
|  | n41 |  |  | 2670 | 10 | 50 | 2670 |  |
|  | n79 |  |  | 4440 | 40 | 216 | 4440 | 30.8 |
|  | 3 | IMD5 | ¦4*fB3-fn41¦ | 1770 | 5 | 25 | 1865 | N/A |
|  | n41 |  |  | 2550 | 10 | 50 | 2550 |  |
|  | n79 |  |  | 4530 | 40 | 216 | 4530 | 5.2 |

6.71	DC_8_n40-n79

[Editor Note]: DC operating bands,DL/UL configuration, coexistence and delta Tib/Rib will be updated.

### 6.71.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD4 products produced by Band 8 and n40 that impact the reference sensitivity of NR band n79.

In addition, there is IMD4 product produced by Band 8 and n79 that impact the reference sensitivity of NR Band n40. The required MSD are shown in the following table.

Table 6.71.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_8A_n40A-n79A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_8A_n40A-n79A | 8 | IMD4 | ¦3*fB8+fn40¦ | 885 | 5 | 25 | 930 | N/A |
|  | n40 |  |  | 2305 | 5 | 25 | 2305 |  |
|  | n79 |  |  | 4960 | 40 | 216 | 4960 | 10.7 |
|  | 8 | IMD4 | ¦3*fB8-fn79¦ | 885 | 5 | 25 | 930 | N/A |
|  | n79 |  |  | 4960 | 40 | 216 | 4960 |  |
|  | n40 |  |  | 2305 | 5 | 25 | 2305 | 9.2 |

6.72	DC_8_n41-n79

[Editor Note]: DC operating bands,DL/UL configuration, coexistence and delta Tib/Rib will be updated.

### 6.72.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD3 products produced by Band 8 and n41 that impact the reference sensitivity of NR band n79.

In addition, there is IMD3 product produced by Band 8 and n79 that impact the reference sensitivity of NR Band n41. The required MSD are shown in the following table.

Table 6.72.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_8A_n41A-n79A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_8A_n41A-n79A | 8 | IMD3 | ¦2*fB8+fn41¦ | 910 | 5 | 25 | 955 | N/A |
|  | n41 |  |  | 2650 | 10 | 50 | 2650 |  |
|  | n79 |  |  | 4470 | 40 | 216 | 4470 | 16.3 |
|  | 8 | IMD3 | ¦2*fB8-fn79¦ | 910 | 5 | 25 | 955 | N/A |
|  | n79 |  |  | 4470 | 40 | 216 | 4470 |  |
|  | n41 |  |  | 2650 | 10 | 50 | 2650 | 15.5 |

6.73	DC_39_n40-n79

[Editor Note]: DC operating bands,DL/UL configuration, coexistence and delta Tib/Rib will be updated.

### 6.73.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD4 products produced by Band 39 and n40 that impact the reference sensitivity of NR band n79. The required MSD is shown in the following table.

Table 6.73.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_39A_n40A-n79A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_39A_n40A-n79A | 39 | IMD4 | ¦fB39-3*fn40¦ | 1917.5 | 5 | 25 | 1917.5 | N/A |
|  | n40 |  |  | 2302.5 | 5 | 25 | 2302.5 |  |
|  | n79 |  |  | 4980 | 40 | 216 | 4980 | 5.8 |

6.74	DC_7-7_n66-n78

[Editor Note]: DC operating bands,DL/UL configuration, coexistence and delta Tib/Rib will be updated.

### 6.74.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD3 products produced by Band 7 and n66 that impact the reference sensitivity of NR band n78. The required MSD is shown in the following table.

Table 6.74.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_7A-7A_n66A-n78A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_7A-7A_n66A-n78A | 7 | IMD3 | ¦2*fB7 -fn66¦ | 2555 | 5 | 25 | 2675 | N/A |
|  | n66 |  |  | 1740 | 5 | 25 | 2140 |  |
|  | n78 |  |  | 3370 | 10 | 50 | 3370 | 15.1 |

7	EN-DC band combinations of LTE 2 bands DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part

7.1	DC_1A-3A_n78A-n257

7.1.1	Operating bands for DC

Table 7.1.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-3A_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

7.1.2	Channel bandwidths per operating band for DC

Table 7.1.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-3A_n78A-n257D | DC_1A_n78A DC_3A_n78A DC_1A_n257A DC_3A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 540 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A_n78A-n257E | DC_1A_n78A DC_3A_n78A DC_1A_n257A DC_3A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 740 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A_n78A-n257F | DC_1A_n78A DC_3A_n78A DC_1A_n257A DC_3A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 940 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A_n78A-n257G | DC_1A_n78A DC_3A_n78A DC_1A_n257A DC_3A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 340 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A_n78A-n257H | DC_1A_n78A DC_3A_n78A DC_1A_n257A DC_3A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 440 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A_n78A-n257I | DC_1A_n78A DC_3A_n78A DC_1A_n257A DC_3A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 540 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A_n78A-n257J | DC_1A_n78A DC_3A_n78A DC_1A_n257A DC_3A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 640 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A_n78A-n257K | DC_1A_n78A DC_3A_n78A DC_1A_n257A DC_3A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 740 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A_n78A-n257L | DC_1A_n78A DC_3A_n78A DC_1A_n257A DC_3A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 840 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A_n78A-n257M | DC_1A_n78A DC_3A_n78A DC_1A_n257A DC_3A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 940 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.1.3	Co-existence studies

Co-existence studies of Band 1 + Band n78, Band 3 + Band n78, Band 1 + n257 and Band 3 + n257 are captured in TR 37.863-01-01, own Rx impact of the 3rd band is the followings

-	7th and 8th harmonics from n78 may fall into n257 and 2nd, 3rd and 5th order IMD may fall into own Rx of Band 3 when dual uplink is configured with Band 1 + Band n78

-	7th and 8th harmonics from n78 may fall into n257 and 5th order IMD may fall into own Rx of Band 1 when dual uplink is configured with Band 3 + Band n78

-	2nd harmonic from Band 1 may fall into n78 and 3rd order IMD may fall into own Rx of Band 3 when dual uplink is configured with Band 1 + Band n257

-	2nd harmonic from Band 3 may fall into n78 and 3rd order IMD may fall into own Rx of Band 1 when dual uplink is configured with Band 3 + Band n257.

7.1.4	∆TIB and ∆RIB values

For DC_1A-3A_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.1.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-3A_n78-n257 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 7.1.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-3A_n78-n257 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

7.1.5	MSD

IMD problems are already analysed in constituent low order combinations between LTE bands and NR bands. 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively and fallback modes are captured in TR 37.864-41-21. Therefore, additional analysis is not needed.

-	Uplink DC configuration is DC_1A_n78A: Up to 7th harmonic order will be not any impact to n257 and IMD issues on 3rd own Rx band will be covered in DC_1A-3A_n78A

-	Uplink DC configuration is DC _3A_n78A: Up to 7th harmonic order will be not any impact to n257 and IMD problems will be covered in DC_1A-3A_n78A

-	Uplink DC configuration is DC_1A_n257A: Harmonic problem already discussed and solved in DC_1A_n78A

-	Uplink DC configuration is DC_3A_n257A: Harmonic problem is already solved in DC_3A_n78A

7.2	DC_1A-5A_n78A-n257

7.2.1	Operating bands for DC

Table 7.2.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-5A_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

7.2.2	Channel bandwidths per operating band for DC

Table 7.2.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-5A_n78A-n257D | DC_1A_n78A DC_5A_n78A DC_1A_n257A DC_5A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 530 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A_n78A-n257E | DC_1A_n78A DC_5A_n78A DC_1A_n257A DC_5A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 730 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A_n78A-n257F | DC_1A_n78A DC_5A_n78A DC_1A_n257A DC_5A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 930 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A_n78A-n257G | DC_1A_n78A DC_5A_n78A DC_1A_n257A DC_5A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 330 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A_n78A-n257H | DC_1A_n78A DC_5A_n78A DC_1A_n257A DC_5A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 430 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A_n78A-n257I | DC_1A_n78A DC_5A_n78A DC_1A_n257A DC_5A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 530 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A_n78A-n257J | DC_1A_n78A DC_5A_n78A DC_1A_n257A DC_5A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 630 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A_n78A-n257K | DC_1A_n78A DC_5A_n78A DC_1A_n257A DC_5A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 730 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A_n78A-n257L | DC_1A_n78A DC_5A_n78A DC_1A_n257A DC_5A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 830 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A_n78A-n257M | DC_1A_n78A DC_5A_n78A DC_1A_n257A DC_5A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 930 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.2.3	Co-existence studies

Co-existence studies of Band 1 + Band n78, Band 5 + Band n78, Band 1 + n257 and Band 5 + n257 are captured in TR 37.863-01-01, own Rx impact of the 3rd band is the followings

-	7th and 8th harmonics from n78 may fall into n257 and 5th order IMD may fall into own Rx of Band 5 when dual uplink is configured with Band 1+ Band n78

-	7th and 8th harmonics from n78 may fall into n257 and 3rd order IMD may fall into own Rx of Band 1 when dual uplink is configured with Band 5 + Band n78

-	2nd harmonic from Band 1 may fall into n78 when dual uplink is configured with Band 1 + Band n257

-	4th harmonic from Band 5 may fall into n78 when dual uplink is configured with Band 5 + Band 257

7.2.4	∆TIB and ∆RIB values

For DC_1A-5A_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.2.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-5A_n78-n257 | 1 | 0.3 |
|  | 5 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 7.2.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-5A_n78-n257 | 1 | 0.2 |
|  | 5 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

7.2.5	MSD

IMD problems are already analysed in constituent low order combinations between LTE bands and NR bands. 2DL/2UL and 3DL/2UL captured in 37.863-01-01 [6] and TR 37.863-02-01 respectively and fallback modes are captured in 37.864-41-21. Therefore, additional analysis is not needed.

-	Uplink DC configuration is DC_1A_n78A: Up to 7th harmonic order will be not any impact to n257 and 5th IMD problems is already covered in DC_1A-5A_n78A

-	Uplink DC configuration is DC_5A_n78A: There is no harmonic problems by 7th & 8th order between FR1 and FR2 and 3rd order IMD problems is already covered in DC_1A-5A_n78A

-	Uplink DC configuration is DC_1A_n257A: Harmonic problem is already discussed and is solved in DC_1A_n78A

-	Uplink DC configuration is DC_5A_n257A: Harmonic problem is already solved in DC_5A_n78A

7.3	DC_1A-7A_n78A-n257

7.3.1	Operating bands for DC

Table 7.3.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-7A_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

7.3.2	Channel bandwidths per operating band for DC

Table 7.3.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-7A_n78A-n257D | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 540 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-7A_n78A-n257E | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 740 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-7A_n78A-n257F | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 940 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-7A_n78A-n257G | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 340 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-7A_n78A-n257H | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 440 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-7A_n78A-n257I | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 540 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-7A_n78A-n257J | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 640 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-7A_n78A-n257K | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 740 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-7A_n78A-n257L | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 840 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-7A_n78A-n257M | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 940 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.3.3	Co-existence studies

Co-existence studies of Band 1 + Band n78, Band 7 + Band n78, Band 1 + n257 and Band 7 + n257 are captured in TR 37.863-01-01, own Rx impact of the 3rd band is the followings

-	7th & 8th harmonics from n78 may fall into n257 and 4th order IMD may fall into own Rx of Band 7 when dual uplink is configured with Band 1 + n78

-	7th harmonic from Band 7 may fall into n257 and 4th order IMD may fall into own Rx of Band 1 when dual uplink is configured with Band 7 + n78

-	2nd harmonic from Band 1 may fall into n78 when dual uplink is configured with Band 1 + n257

7.3.4	∆TIB and ∆RIB values

For DC_1A-7A_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.3.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-7A_n78-n257 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 7.3.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-7A_n78-n257 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

7.3.5	MSD

IMD problems are already analysed in constituent low order combinations between LTE bands and NR bands. 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively and fallback modes are captured in TR 37.864-41-21. Therefore, additional analysis is not needed.

-	Uplink DC configuration is DC_1A_n78A: Up to 7th harmonic order will be not any impact to n257 and 4th IMD problem is already covered in DC_1A-7A_n78A

-	Uplink DC configuration is DC_7A_n78A: Up to 7th harmonic order will be not any impact to n257 and 4th IMD problem is already covered in DC_1A-7A_n78A

-	Uplink DC configuration is DC_1A_n257A: Harmonic problem already discussed and is solved in DC_1A_n78A

7.4	DC_1A-7A-7A_n78A-n257

7.4.1	Operating bands for DC

Table 7.4.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-7A-7A_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

7.4.2	Channel bandwidths per operating band for DC

Table 7.4.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-7A-7A_n78A-n257D | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 560 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-7A-7A_n78A-n257E | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 760 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-7A-7A_n78A-n257F | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 960 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-7A-7A_n78A-n257G | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 360 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-7A-7A_n78A-n257H | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 460 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-7A-7A_n78A-n257I | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 560 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-7A-7A_n78A-n257J | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 660 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-7A-7A_n78A-n257K | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 760 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-7A-7A_n78A-n257L | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 860 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-7A-7A_n78A-n257M | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 960 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.4.3	Co-existence studies

Co-existence studies of Band 1 + Band n78, Band 7 + Band n78, Band 1 + n257 and Band 7 + n257 are captured in TR 37.863-01-01, own Rx impact of the 3rd band is the followings

-	7th & 8th harmonics from n78 may fall into n257 and 4th order IMD may fall into own Rx of Band 7 when dual uplink is configured with Band 1 + n78

-	7th harmonic from Band 7 may fall into n257 and 4th order IMD may fall into own Rx of Band 1 when dual uplink is configured with Band 7 + n78

-	2nd harmonic from Band 1 may fall into n78 when dual uplink is configured with Band 1 + n257

7.4.4	∆TIB and ∆RIB values

For DC_1A-7A-7A_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.4.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-7A-7A_n78-n257 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 7.4.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-7A-7A_n78-n257 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

7.4.5	MSD

IMD problems are already analysed in constituent low order combinations between LTE bands and NR bands. 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively and fallback modes are captured in TR 37.864-41-21. Therefore, additional analysis is not needed.

-	Uplink DC configuration is DC_1A_n78A: Up to 7th harmonic order will be not any impact to n257 and 4th IMD problem is already covered in DC_1A-7A_n78A

-	Uplink DC configuration is DC_7A_n78A: Up to 7th harmonic order will be not any impact to n257 and 4th IMD problem is already covered in DC_1A-7A_n78A

-	Uplink DC configuration is DC_1A_n257A: Harmonic problem already discussed and is solved in DC_1A_n78A

7.5	DC_3A-5A_n78A-n257

7.5.1	Operating bands for DC

Table 7.5.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-5A_n78-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

7.5.2	Channel bandwidths per operating band for DC

Table 7.5.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_3A-5A_n78A-n257D | DC_3A_n78A DC_5A_n78A DC_3A_n257A DC_5A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 530 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A_n78A-n257E | DC_3A_n78A DC_5A_n78A DC_3A_n257A DC_5A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 730 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A_n78A-n257F | DC_3A_n78A DC_5A_n78A DC_3A_n257A DC_5A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 930 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A_n78A-n257G | DC_3A_n78A DC_5A_n78A DC_3A_n257A DC_5A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 330 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A_n78A-n257H | DC_3A_n78A DC_5A_n78A DC_3A_n257A DC_5A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 430 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A_n78A-n257I | DC_3A_n78A DC_5A_n78A DC_3A_n257A DC_5A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 530 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A_n78A-n257J | DC_3A_n78A DC_5A_n78A DC_3A_n257A DC_5A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 630 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A_n78A-n257K | DC_3A_n78A DC_5A_n78A DC_3A_n257A DC_5A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 730 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A_n78A-n257L | DC_3A_n78A DC_5A_n78A DC_3A_n257A DC_5A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 830 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A_n78A-n257M | DC_3A_n78A DC_5A_n78A DC_3A_n257A DC_5A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 930 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.5.3	Co-existence studies

Co-existence studies of Band 3 + Band n78, Band 5 + Band n78, Band 3 + n257 and Band 5 + n257 are captured in TR 37.863-01-01, own Rx impact of the 3rd band is the followings

-	7th & 8th harmonics from n78 may fall into n257 when dual uplink is configured with Band 3 + n78

-	7th & 8th harmonics from n78 may fall into n257 and 3rd order IMD may fall into own Rx of Band 3 when dual uplink is configured with Band 5A + n78

-	2nd harmonic from Band 3 may fall into n78 when dual uplink is configured with Band 3 + n257

-	4th harmonic from Band 5 may fall into n78 when dual uplink is configured with Band 5 + n257

7.5.4	∆TIB and ∆RIB values

For DC_3A-5A_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.5.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-5A_n78-n257 | 3 | 0.6 |
|  | 5 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 7.5.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-5A_n78-n257 | 3 | 0.2 |
|  | 5 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

7.5.5	MSD

IMD problems are already analysed in constituent low order combinations between LTE bands and NR bands. 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively and fallback modes are captured in TR 37.864-41-21. Therefore, additional analysis is not needed.

-	Uplink DC configuration is DC_3A_n78A: Up to 7th harmonic order will be not any impact to n257

-	Uplink DC configuration is DC_5A_n78A: There is no harmonic problems by 7th & 8th order between FR1 and FR2 and 3rd IMD problem is covered in DC_3A-5A_n78A

-	Uplink DC configuration is DC_3A_n257A: Harmonic problem is already solved in DC_3A_n78A

-	Uplink DC configuration is DC_5A_n257A: Harmonic problem is already solved in DC_5A_n78A

7.6	DC_3A-7A_n78A-n257

7.6.1	Operating bands for DC

Table 7.6.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-7A_n78-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

7.6.2	Channel bandwidths per operating band for DC

Table 7.6.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_3A-7A_n78A-n257D | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 540 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-7A_n78A-n257E | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 740 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-7A_n78A-n257F | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 940 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-7A_n78A-n257G | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 340 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-7A_n78A-n257H | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 440 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-7A_n78A-n257I | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 540 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-7A_n78A-n257J | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 640 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-7A_n78A-n257K | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 740 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-7A_n78A-n257L | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 840 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-7A_n78A-n257M | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 940 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.6.3	Co-existence studies

Co-existence studies of Band 3 + Band n78, Band 7 + Band n78, Band 3 + n257 and Band 7 + n257 are captured in TR 37.863-01-01

-	7th & 8th harmonics from n78 may fall into n257 when dual uplink is configured with Band 3 + Band n78

-	7th harmonic from Band 7 may fall into n257 and 3rd & 4th IMDs may fall into Band 3 when dual uplink is configured with Band 7 + Band n78

-	2nd harmonic from Band 3 may fall into n78 when dual uplink is configured with Band 3 + Band n257

7.6.4	∆TIB and ∆RIB values

For DC_3A-7A_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.6.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-7A_n78-n257 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 7.6.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-7A_n78-n257 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

7.6.5	MSD

IMD problems are already analysed in constituent low order combinations between LTE bands and NR bands. 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01  and TR 37.863-02-01  respectively and fallback modes are captured in TR 37.864-41-21. Therefore, additional analysis is not needed.

-	Uplink DC configuration is DC_3A_n78A: Up to 7th harmonic order will be not any impact to n257

-	Uplink DC configuration is DC_7A_n78A: Up to 7th harmonic order will be not impact to n257 and IMD problems are already covered in DC_3A-7A_n78A

-	Uplink DC configuration is DC_3A_n257A: Harmonic problem is already solved in DC_3A_n78A

7.7	DC_3A-7A-7A_n78A-n257

7.7.1	Operating bands for DC

Table 7.7.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-7A-7A_n78-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

7.7.2	Channel bandwidths per operating band for DC

Table 7.7.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_3A-7A-7A_n78A-n257D | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 560 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-7A-7A_n78A-n257E | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 760 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-7A-7A_n78A-n257F | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 960 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-7A-7A_n78A-n257G | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 360 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-7A-7A_n78A-n257H | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 460 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-7A-7A_n78A-n257I | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 560 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-7A-7A_n78A-n257J | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 660 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-7A-7A_n78A-n257K | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 760 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-7A-7A_n78A-n257L | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 860 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-7A-7A_n78A-n257M | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 960 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.7.3	Co-existence studies

Co-existence studies of Band 3 + Band n78, Band 7 + Band n78, Band 3 + n257 and Band 7 + n257 are captured in TR 37.863-01-01

-	7th & 8th harmonics from n78 may fall into n257 when dual uplink is configured with Band 3 + Band n78

-	7th harmonic from Band 7 may fall into n257 and 3rd & 4th IMDs may fall into Band 3 when dual uplink is configured with Band 7 + Band n78

-	2nd harmonic from Band 3 may fall into n78 when dual uplink is configured with Band 3 + Band n257

7.7.4	∆TIB and ∆RIB values

For DC_3A-7A-7A_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.7.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-7A-7A_n78-n257 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 7.7.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-7A-7A_n78-n257 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

7.7.5	MSD

IMD problems are already analysed in constituent low order combinations between LTE bands and NR bands. 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively and fallback modes are captured in TR 37.864-41-21. Therefore, additional analysis is not needed.

-	Uplink DC configuration is DC_3A_n78A: Up to 7th harmonic order will be not any impact to n257

-	Uplink DC configuration is DC_7A_n78A: Up to 7th harmonic order will be not impact to n257 and IMD problems are already covered in DC_3A-7A_n78A

-	Uplink DC configuration is DC_3A_n257A: Harmonic problem is already solved in DC_3A_n78A

7.8	DC_5A-7A_n78A-n257

7.8.1	Operating bands for DC

Table 7.8.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5A-7A_n78-n257 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

7.8.2	Channel bandwidths per operating band for DC

Table 7.8.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_5A-7A_n78A-n257D | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 530 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-7A_n78A-n257E | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 730 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-7A_n78A-n257F | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 930 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-7A_n78A-n257G | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 330 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-7A_n78A-n257H | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 430 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-7A_n78A-n257I | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 530 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-7A_n78A-n257J | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 630 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-7A_n78A-n257K | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 730 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-7A_n78A-n257L | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 830 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-7A_n78A-n257M | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 930 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.8.3	Co-existence studies

Co-existence studies of Band 5 + Band n78, Band 7 + Band n78, Band 5 + n257 and Band 7 + n257 are captured in TR 37.863-01-01

-	7th & 8th harmonics from n78 may fall into n257 and 2nd order IMD may fall into own Rx of Band 7 when dual uplink is configured with Band 5 + Band n78

-	7th harmonic from Band 7 may fall into Band n257 and 2nd order and 5th order IMD may fall into own Rx of Band 5 when dual uplink is configured with Band 7 + Band n78

-	4th harmonic from Band 5 may fall into n78 when dual uplink is configured with Band 5 + Band n257

7.8.4	∆TIB and ∆RIB values

For DC_5A-7A_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.8.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5A-7A_n78-n257 | 5 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 7.8.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5A-7A_n78-n257 | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

7.8.5	MSD

IMD problems are already analysed in constituent low order combinations between LTE bands and NR bands. 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively and fallback modes are captured in TR 37.864-41-21. Therefore, additional analysis is not needed.

-	Uplink DC configuration is DC_5A_n78A: There is no harmonic problem by 7th & 8th order between FR1 and FR2 and 2nd IMD problem is already covered in DC_5A-7A_n78A

-	Uplink DC configuration is DC_7A_n78A: Up to 7th harmonic order will be not any impact to n257 and IMD problems are already covered in DC_5A-7A_n78A

-	Uplink DC configuration is DC_5A_n257A: Harmonic problem is already solved in DC_5A_n78A

7.9	DC_5A-7A-7A_n78A-n257

7.9.1	Operating bands for DC

Table 7.9.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5A-7A-7A_n78-n257 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

7.9.2	Channel bandwidths per operating band for DC

Table 7.9.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_5A-7A-7A_n78A-n257D | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 550 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-7A-7A_n78A-n257E | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 750 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-7A-7A_n78A-n257F | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 950 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-7A-7A_n78A-n257G | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 350 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-7A-7A_n78A-n257H | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 450 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-7A-7A_n78A-n257I | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 550 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-7A-7A_n78A-n257J | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 650 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-7A-7A_n78A-n257K | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 750 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-7A-7A_n78A-n257L | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 850 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-7A-7A_n78A-n257M | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 950 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.9.3	Co-existence studies

Co-existence studies of Band 5 + Band n78, Band 7 + Band n78, Band 5 + n257 and Band 7 + n257 are captured in TR 37.863-01-01

-	7th & 8th harmonics from n78 may fall into n257 and 2nd order IMD may fall into own Rx of Band 7 when dual uplink is configured with Band 5 + Band n78

-	7th harmonic from Band 7 may fall into Band n257 and 2nd order and 5th order IMD may fall into own Rx of Band 5 when dual uplink is configured with Band 7 + Band n78

-	4th harmonic from Band 5 may fall into n78 when dual uplink is configured with Band 5 + Band n257

7.9.4	∆TIB and ∆RIB values

For DC_5A-7A-7A_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.9.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5A-7A-7A_n78-n257 | 5 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 7.9.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5A-7A-7A_n78-n257 | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

7.9.5	MSD

IMD problems are already analysed in constituent low order combinations between LTE bands and NR bands. 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively and fallback modes are captured in TR 37.864-41-21. Therefore, additional analysis is not needed.

-	Uplink DC configuration is DC_5A_n78A: There is no harmonic problem by 7th & 8th order between FR1 and FR2 and 2nd IMD problem is already covered in DC_5A-7A_n78A

-	Uplink DC configuration is DC_7A_n78A: Up to 7th harmonic order will be not any impact to n257 and IMD problems are already covered in DC_5A-7A_n78A

-	Uplink DC configuration is DC_5A_n257A: Harmonic problem is already solved in DC_5A_n78A

7.10	DC_1A-3C_n28A-n78A

7.10.1	Operating bands for DC

Table 7.10.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-3C_n28A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.10.2	Channel bandwidths per operating band for DC

Table 7.10.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A-3C_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_3C_n28A DC_3C_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 180 |
|  |  | 3 | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.10.3	Co-existence studies

The harmonic and IMD products caused by DC_1A-3C_n28A-n78A and protected bands follow from those derived for the constituent fallback modes. The summary of self-interference issues are provided in TR 37.716-21-21, Table 5.3-2.

7.10.4	∆TIB and ∆RIB values

For DC_1A-3C_n28A-n78A, the TIB,c and RIB values are given in the tables below.

Table 7.10.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n28-n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |

Table 7.10.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3_n28-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |

7.10.5	MSD

All MSD requirements for DC_1A-3C_n28A-n78A are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.11	DC_1A-7C_n28A-n78A

7.11.1	Operating bands for DC

Table 7.11.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-7C_n28A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.11.2	Channel bandwidths per operating band for DC

Table 7.11.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A-7C_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_7A_n28A DC_7A_n78A DC_7C_n28A DC_7C_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 180 |
|  |  | 7 | See CA_7C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.11.3	Co-existence studies

The harmonic and IMD products caused by DC_1A-7C_n28A-n78A and protected bands follow from those derived for the constituent fallback modes. The summary of self-interference issues are provided in TR 37.716-21-21, Table 5.3-2.

7.11.4	∆TIB and ∆RIB values

For DC_1A-7C_n28A-n78A, the TIB,c and RIB values are given in the tables below.

Table 7.11.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7_n28-n78 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |

Table 7.11.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-7_n28-n78 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |

7.11.5	MSD

All MSD requirements for DC_1A-7C_n28A-n78A are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.12	DC_1-3_n7-n78

7.12.1	Operating bands for DC

Table 7.12.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3_n7-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.12.2	Channel bandwidths per operating band for DC

Table 7.12.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A_n7A-n78A | DC_1A_n7A DC_1A_n78A DC_3A_n7A DC_3A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 160 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-3C_n7A-n78A | DC_1A_n7A DC_1A_n78A DC_3A_n7A DC_3A_n78A DC_3C_n7A DC_3C_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 180 |
|  |  | 3 | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-3A_n7A-n78(2A) | DC_1A_n7A DC_1A_n78A DC_3A_n7A DC_3A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 260 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78(2A) in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3C_n7A-n78(2A) | DC_1A_n7A DC_1A_n78A DC_3A_n7A DC_3A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 280 |
|  |  | 3 | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78(2A) in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.12.3	Co-existence studies

The harmonic and IMD products caused by DC_1-3_n7-n78 and protected bands follow from those derived for the constituent fallback modes. The summary of self-interference issues are provided in TR 37.716-21-21, Table 5.3-2.

7.12.4	∆TIB and ∆RIB values

For DC_1-3_n7-n78, the TIB,c and RIB values are given in the tables below.

Table 7.12.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n7-n78 | 1 | 0.7 |
|  | 3 | 0.7 |
|  | n7 | 0.7 |
|  | n78 | 0.8 |

Table 7.12.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3_n7-n78 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | n7 | 0.3 |
|  | n78 | 0.5 |

7.12.5	MSD

All MSD requirements for DC_1-3_n7-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.13	DC_3-7_n28-n78

7.13.1	Operating bands for DC

Table 7.13.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7_n28-n78 | 3 | 1710 MHz | – | 1755 MHz | 2110 MHz | – | 2155 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.13.2	Channel bandwidths per operating band for DC

Table 7.13.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A-7C_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_7A_n28A DC_7A_n78A DC_7C_n28A DC_7C_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 180 |
|  |  | 7 | See CA_7C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3C-7A_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_7A_n28A DC_7A_n78A DC_3C_n28A DC_3C_n78A | 3 | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3C-7C_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_7A_n28A DC_7A_n78A DC_3C_n28A DC_3C_n78A DC_7C_n28A DC_7C_n78A | 3 | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 200 |
|  |  | 7 | See CA_7C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.13.3	Co-existence studies

The harmonic and IMD products caused by DC_3-7_n28-n78 and protected bands follow from those derived for the constituent fallback modes. The summary of self-interference issues are provided in TR 37.716-21-21, Table 5.3-2.

7.13.4	∆TIB and ∆RIB values

For DC_3-7_n28-n78, the TIB,c and RIB values are given in the tables below.

Table 7.13.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7_n28-n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |

Table 7.13.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7_n28-n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |

7.13.5	MSD

All MSD requirements for DC_3-7_n28-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.14	DC_1-8_n77-n257

7.14.1	Operating bands for DC

Table 7.14.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-8_n77-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.14.2	Channel bandwidths per operating band for DC

Table 7.14.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_1A-8A_n77A-n257A | DC_1A_n77A DC_1A_n257A DC_8A_n77A DC_8A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 530 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes |  |
| DC_1A-8A_n77A-n257D | DC_1A_n77A DC_1A_n257A DC_8A_n77A DC_8A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 530 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-8A_n77A-n257G | DC_1A_n77A DC_1A_n257A DC_8A_n77A DC_8A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 330 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-8A_n77A-n257H | DC_1A_n77A DC_1A_n257A DC_8A_n77A DC_8A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 430 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-8A_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_8A_n77A DC_8A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 530 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |

7.14.3	Co-existence studies

Based on the co-existence studies of DC_1A_n77A, DC_1A_n257A, DC_8A_n77A and DC_8A_n257A captured in TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.14.4	∆TIB and ∆RIB values

For DC_1A-8A_n77A-n257A, the TIB,c and RIB,c values are given in the tables below.

Table 7.14.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8_n77-n257 | 1 | 0.6 |
|  | 8 | 0.6 |
|  | n77 | 0.8 |
|  | n257 | 0 |

Table 7.14.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-8_n77-n257 | 1 | 0.2 |
|  | 8 | 0.2 |
|  | n77 | 0.5 |
|  | n257 | 0 |

7.14.5	MSD

As mentioned in 7.14.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.15	DC_1-19_n77-n79

7.15.1	Operating bands for DC

Table 7.15.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-19_n77-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.15.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 7.15.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-19A_n77A-n79A | DC_19A_n77A DC_19A_n79A | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 235 |
|  |  | 19 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

7.15.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.15.4	∆TIB and ∆RIB values

For DC_1-19_n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.15.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-19_n77-n79 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | n77 | 0.8 |

Table 7.15.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-19_n77-n79 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | n77 | 0.5 |

7.15.5	MSD

MSD analysis of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.16	DC_1-19_n78-n79

7.16.1	Operating bands for DC

Table 7.16.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-19_n78-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.16.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 7.16.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-19A_n78A-n79A | DC_19A_n78A DC_19A_n79A | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 235 |
|  |  | 19 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

7.16.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.16.4	∆TIB and ∆RIB values

For DC_1-19_n78-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.16.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-19_n78-n79 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | n78 | 0.8 |

Table 7.16.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-19_n78-n79 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | n78 | 0.5 |

7.16.5	MSD

MSD analysis of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.17	DC_1-21_n77-n79

7.17.1	Operating bands for DC

Table 7.17.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-21_n77-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.17.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 7.17.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-21A_n77A-n79A | DC_1A_n77A DC_1A_n79A | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 235 |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

7.17.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.17.4	∆TIB and ∆RIB values

For DC_1-21_n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.17.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-21_n77-n79 | 1 | 0.3 |
|  | 21 | 0.3 |
|  | n77 | 0.8 |

Table 7.17.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-21_n77-n79 | n77 | 0.5 |

7.17.5	MSD

MSD analysis of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.18	DC_1-21_n78-n79

7.18.1	Operating bands for DC

Table 7.18.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-21_n78-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.18.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 7.18.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-21A_n78A-n79A | DC_1A_n78A DC_1A_n79A | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 235 |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

7.18.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.18.4	∆TIB and ∆RIB values

For DC_1-21_n78-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.18.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-21_n78-n79 | 1 | 0.3 |
|  | 21 | 0.3 |
|  | n78 | 0.8 |

Table 7.18.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-21_n78-n79 | n78 | 0.5 |

7.18.5	MSD

MSD analysis of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.19	DC_1-3_n77-n79

7.19.1	Operating bands for DC

Table 7.19.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3_n77-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.19.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 7.19.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-3A_n77A-n79A | DC_1A_n77A DC_3A_n77A DC_1A_n79A DC_3A_n79A | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 240 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

7.19.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.19.4	∆TIB and ∆RIB values

For DC_1-3_n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.19.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n77-n79 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n77 | 0.8 |

Table 7.19.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3_n77-n79 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n77 | 0.5 |

7.19.5	MSD

MSD analysis of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.20	DC_1-3_n78-n79

7.20.1	Operating bands for DC

Table 7.20.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3_n78-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.20.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 7.20.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-3A_n78A-n79A | DC_1A_n78A DC_3A_n78A DC_1A_n79A DC_3A_n79A | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 240 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

7.20.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.20.4	∆TIB and ∆RIB values

For DC_1-3_n78-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.20.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n78-n79 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n78 | 0.8 |

Table 7.20.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3_n78-n79 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n78 | 0.5 |

7.20.5	MSD

MSD analysis of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.21	DC_1-42_n77-n79

7.21.1	Operating bands for DC

Table 7.21.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-42_n77-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.21.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 7.21.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-42A_n77A-n79A DC_1A-42C_n77A-n79A | DC_1A_n77A DC_1A_n79A | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 260 |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

7.21.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.21.4	∆TIB and ∆RIB values

For DC_1-42_n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.21.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-42_n77-n79 | 1 | 0.6 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |

Table 7.21.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-42_n77-n79 | 1 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |

7.21.5	MSD

MSD analysis of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.22	DC_1-42_n78-n79

7.22.1	Operating bands for DC

Table 7.22.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-42_n78-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.22.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 7.22.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-42A_n78A-n79A DC_1A-42C_n78A-n79A | DC_1A_n78A DC_1A_n79A | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 260 |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

7.22.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.22.4	∆TIB and ∆RIB values

For DC_1-42_n78-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.22.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-42_n78-n79 | 1 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |

Table 7.22.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-42_n78-n79 | 1 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |

7.22.5	MSD

MSD analysis of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.23	DC_19-21_n77-n79

7.23.1	Operating bands for DC

Table 7.23.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19-21_n77-n79 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.23.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 7.23.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_19A-21A_n77A-n79A | DC_19A_n77A DC_19A_n79A | 19 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 230 |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

7.23.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.23.4	∆TIB and ∆RIB values

For DC_19-21_n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.23.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19-21_n77-n79 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | n77 | 0.8 |

Table 7.23.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19-21_n77-n79 | n77 | 0.5 |

7.23.5	MSD

MSD analysis of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.24	DC_19-21_n78-n79

7.24.1	Operating bands for DC

Table 7.24.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19-21_n78-n79 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.24.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 7.24.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_19A-21A_n78A-n79A | DC_19A_n78A DC_19A_n79A | 19 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 230 |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

7.24.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.24.4	∆TIB and ∆RIB values

For DC_19-21_n78-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.24.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19-21_n78-n79 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | n78 | 0.8 |

Table 7.24.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19-21_n78-n79 | n78 | 0.5 |

7.24.5	MSD

MSD analysis of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.25	DC_19-42_n77-n79

7.25.1	Operating bands for DC

Table 7.25.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19-42_n77-n79 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.25.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 7.25.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_19A-42A_n77A-n79A DC_19A-42C_n77A-n79A | DC_19A_n77A DC_19A_n79A | 19 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 255 |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

7.25.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.25.4	∆TIB and ∆RIB values

For DC_19-42_n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.25.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19-42_n77-n79 | 1 | 0.3 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |

Table 7.25.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19-42_n77-n79 | 42 | 0.5 |
|  | n77 | 0.5 |

7.25.5	MSD

MSD analysis of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.26	DC_19-42_n78-n79

7.26.1	Operating bands for DC

Table 7.26.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19-42_n78-n79 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.26.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 7.26.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_19A-42A_n78A-n79A DC_19A-42C_n78A-n79A | DC_19A_n78A DC_19A_n79A | 19 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 255 |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

7.26.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.26.4	∆TIB and ∆RIB values

For DC_19-42_n78-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.26.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19-42_n78-n79 | 1 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |

Table 7.26.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19-42_n78-n79 | 42 | 0.5 |
|  | n78 | 0.5 |

7.26.5	MSD

MSD analysis of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.27	DC_21-42_n77-n79

7.27.1	Operating bands for DC

Table 7.27.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_21-42_n77-n79 | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.27.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 7.27.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_21A-42A_n77A-n79A DC_21A-42C_n77A-n79A | DC_21A_n77A DC_21A_n79A | 21 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 255 |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

7.27.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.27.4	∆TIB and ∆RIB values

For DC_21-42_n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.27.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_21-42_n77-n79 | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |

Table 7.27.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_21-42_n77-n79 | 42 | 0.5 |
|  | n77 | 0.5 |

7.27.5	MSD

MSD analysis of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.28	DC_21-42_n78-n79

7.28.1	Operating bands for DC

Table 7.28.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_21-42_n78-n79 | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.28.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 7.28.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_21A-42A_n78A-n79A DC_21A-42C_n78A-n79A | DC_21A_n78A DC_21A_n79A | 21 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 255 |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

7.28.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.28.4	∆TIB and ∆RIB values

For DC_21-42_n78-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.28.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_21-42_n78-n79 | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |

Table 7.28.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_21-42_n78-n79 | 42 | 0.5 |
|  | n78 | 0.5 |

7.28.5	MSD

MSD analysis of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.29	DC_3-19_n77-n79

7.29.1	Operating bands for DC

Table 7.29.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-19_n77-n79 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.29.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 7.29.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-19A_n77A-n79A | DC_19A_n77A DC_19A_n79A | 3 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 235 |
|  |  | 19 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

7.29.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.29.4	∆TIB and ∆RIB values

For DC_3-19_n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.29.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-19_n77-n79 | 3 | 0.6 |
|  | 19 | 0.3 |
|  | n77 | 0.8 |

Table 7.29.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-19_n77-n79 | 3 | 0.2 |
|  | n77 | 0.5 |

7.29.5	MSD

MSD analysis of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.30	DC_3-19_n78-n79

7.30.1	Operating bands for DC

Table 7.30.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-19_n78-n79 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.30.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 7.30.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-19A_n78A-n79A | DC_19A_n78A DC_19A_n79A | 3 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 235 |
|  |  | 19 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

7.30.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.30.4	∆TIB and ∆RIB values

For DC_3-19_n78-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.30.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-19_n78-n79 | 3 | 0.6 |
|  | 19 | 0.3 |
|  | n78 | 0.8 |

Table 7.30.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-19_n78-n79 | 3 | 0.2 |
|  | n78 | 0.5 |

7.30.5	MSD

MSD analysis of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.31	DC_3-21_n77-n79

7.31.1	Operating bands for DC

Table 7.31.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-21_n77-n79 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.31.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 7.31.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-21A_n77A-n79A | DC_3A_n77A DC_21A_n77A DC_3A_n79A DC_21A_n79A | 3 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 235 |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

7.31.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.31.4	∆TIB and ∆RIB values

For DC_3-21_n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.31.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-21_n77-n79 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n77 | 0.8 |

Table 7.31.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-21_n77-n79 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n77 | 0.5 |

7.31.5	MSD

MSD analysis of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.32	DC_3-21_n78-n79

7.32.1	Operating bands for DC

Table 7.32.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-21_n78-n79 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.32.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 7.32.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-21A_n78A-n79A | DC_3A_n78A DC_21A_n78A DC_3A_n79A DC_21A_n79A | 3 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 235 |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

7.32.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.32.4	∆TIB and ∆RIB values

For DC_3-21_n78-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.32.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-21_n78-n79 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n78 | 0.8 |

Table 7.32.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-21_n78-n79 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n78 | 0.5 |

7.32.5	MSD

MSD analysis of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.33	DC_3-42_n77-n79

7.33.1	Operating bands for DC

Table 7.33.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-42_n77-n79 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.33.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 7.33.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-42A_n77A-n79A DC_3A-42C_n77A-n79A | DC_3A_n77A DC_3A_n79A | 3 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 260 |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

7.33.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.33.4	∆TIB and ∆RIB values

For DC_3-42_n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.33.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-42_n77-n79 | 3 | 0.6 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |

Table 7.33.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-42_n77-n79 | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |

7.33.5	MSD

MSD analysis of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.34	DC_3-42_n78-n79

7.34.1	Operating bands for DC

Table 7.34.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-42_n78-n79 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.34.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 7.34.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-42A_n78A-n79A DC_3A-42C_n78A-n79A | DC_3A_n78A DC_3A_n79A | 3 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 260 |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

7.34.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.34.4	∆TIB and ∆RIB values

For DC_3-42_n78-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.34.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-42_n78-n79 | 3 | 0.6 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |

Table 7.34.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-42_n78-n79 | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |

7.34.5	MSD

MSD analysis of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.35	DC_3-3-7_n78_n257, DC_3-3-7-7_n78_n257,

7.35.1	Operating bands for DC

Table 7.35.1-1: DC band combination of two LTE bands + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-3-7_n78-n257, DC_3-3-7-7_n78-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

7.35.2	Channel bandwidths per operating band for DC

Table 7.35.2-1: Supported bandwidths per DC two LTE bands + inter-band NR 2DL/1UL

|  |  |  | DC configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  |  | E-UTRA and NR Band |  | SCS [kHz] |  |  | 5 |  |  |  | 10 |  | 15 |  | 20 |  | 40 |  |  |  | 50 |  |  | 60 |  |  | 80 |  | 90 |  | 100 |  |  | 200 |  | 400 | Maximum aggregated bandwidth [MHz] |  |  |
| DC_3A-3A-7A_n78A-n257A | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A |  |  | 3 |  |  |  |  | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 560 |  |  |
|  |  |  |  | 7 |  | 15 |  |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n78 |  | 15 |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |
|  |  |  |  |  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  |  |  |
| DC_3A-3A-7A-7A_n78A-n257A | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A |  |  | 3 |  |  |  |  | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 580 |  |  |
|  |  |  |  | 7 |  |  |  |  | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n78 |  | 15 |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |
|  |  |  |  |  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  |  |  |
| DC_3A-3A-7A_n78A-n257D |  |  | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A |  | 3 |  | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 560 |  |
|  |  |  |  |  | 7 |  | 15 |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  | 15 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 30 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 60 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  | See CA_n257D in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A-7A_n78A-n257E |  | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A |  |  | 3 |  |  | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 760 |
|  |  |  |  |  | 7 |  |  | 15 |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  |  | 15 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 30 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 60 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  | See CA_n257E in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A-7A_n78A-n257F |  | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A |  |  | 3 |  |  | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 960 |
|  |  |  |  |  | 7 |  |  | 15 |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  |  | 15 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 30 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 60 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  | See CA_n257F in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A-7A_n78A-n257G |  | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A |  |  | 3 |  |  | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 360 |
|  |  |  |  |  | 7 |  |  | 15 |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  |  | 15 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 30 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 60 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  | See CA_n257G in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A-7A_n78A-n257H |  | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A |  |  | 3 |  |  | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 460 |
|  |  |  |  |  | 7 |  |  | 15 |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  |  | 15 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 30 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 60 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  | See CA_n257H in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A-7A_n78A-n257I |  | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A |  |  | 3 |  |  | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 560 |
|  |  |  |  |  | 7 |  |  | 15 |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  |  | 15 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 30 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 60 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  | See CA_n257I in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A-7A_n78A-n257J |  | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A |  |  | 3 |  |  | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 660 |
|  |  |  |  |  | 7 |  |  | 15 |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  |  | 15 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 30 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 60 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  | See CA_n257J in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A-7A_n78A-n257K |  | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A |  |  | 3 |  |  | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 760 |
|  |  |  |  |  | 7 |  |  | 15 |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  |  | 15 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 30 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 60 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  | See CA_n257K in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A-7A_n78A-n257L |  | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A |  |  | 3 |  |  | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 860 |
|  |  |  |  |  | 7 |  |  | 15 |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  |  | 15 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 30 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 60 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  | See CA_n257L in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A-7A_n78A-n257M |  | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A |  |  | 3 |  |  | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 960 |
|  |  |  |  |  | 7 |  |  | 15 |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  |  | 15 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 30 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 60 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  | See CA_n257M in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A-7A-7A_n78A-n257D |  |  | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A |  | 3 |  | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 580 |  |  |
|  |  |  |  |  | 7 |  | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  | 15 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 30 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 60 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  | See CA_n257D in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A-7A-7A_n78A-n257E |  |  |  | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A | 3 |  |  | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 780 |
|  |  |  |  |  | 7 |  |  | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  |  | 15 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 30 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 60 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  | See CA_n257E in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A-7A-7A_n78A-n257F |  |  |  | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A | 3 |  |  | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 980 |
|  |  |  |  |  | 7 |  |  | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  |  | 15 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 30 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 60 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  | See CA_n257F in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A-7A-7A_n78A-n257G |  |  |  | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A | 3 |  |  | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 380 |
|  |  |  |  |  | 7 |  |  | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  |  | 15 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 30 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 60 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  | See CA_n257G in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A-7A-7A_n78A-n257H |  |  |  | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A | 3 |  |  | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 480 |
|  |  |  |  |  | 7 |  |  | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  |  | 15 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 30 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 60 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  | See CA_n257H in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A-7A-7A_n78A-n257I |  |  |  | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A | 3 |  |  | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 580 |
|  |  |  |  |  | 7 |  |  | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  |  | 15 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 30 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 60 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  | See CA_n257I in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A-7A-7A_n78A-n257J |  |  |  | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A | 3 |  |  | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 680 |
|  |  |  |  |  | 7 |  |  | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  |  | 15 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 30 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 60 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  | See CA_n257J in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A-7A-7A_n78A-n257K |  |  |  | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A | 3 |  |  | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 780 |
|  |  |  |  |  | 7 |  |  | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  |  | 15 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 30 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 60 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  | See CA_n257K in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A-7A-7A_n78A-n257L |  |  |  | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A | 3 |  |  | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 880 |
|  |  |  |  |  | 7 |  |  | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  |  | 15 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 30 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 60 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  | See CA_n257L in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A-7A-7A_n78A-n257M |  |  |  | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A | 3 |  |  | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 980 |
|  |  |  |  |  | 7 |  |  | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  |  | 15 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 30 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 60 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  | See CA_n257M in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.35.3	Co-existence studies

Based on co-existence studies of DC_3A-n78A, DC_7A-n78A,  DC_3A_n257A and DC_7A_n257A specified in TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.35.4	∆TIB and ∆RIB values

For DC_3-3-7_n78-n257 and DC_3-3-7-7_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.35.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-3-7_n78-n257, DC_3-3-3-7_n78-n257 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 7.35.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-3-7_n78-n257, DC_3-3-3-7_n78-n257 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

7.35.5	MSD

No additional MSD requirement is needed.

7.36	DC_3-7_n1-n78

7.36.1	Operating bands for DC

Table 7.36.1-1: DC band combination of two LTE bands + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7_n1-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.36.2	Channel bandwidths per operating band for DC

Table 7.36.2-1: Supported bandwidths per DC band combination of two LTE bands + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A-7A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 160 |
|  |  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3C-7A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A |  | 3 | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 180 |
|  |  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-7C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_7C_n1A DC_7C_n78A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 180 |
|  |  |  | 7 | See CA_7C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3C-7C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_7C_n1A DC_7C_n78A |  | 3 | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 200 |
|  |  |  | 7 | See CA_7C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.36.3	Co-existence studies

Co-existence studies of Band 3 + Band n78 and Band 7 + Band n78 are captured in TR 37.863-01-01.

Co-existence studies of Band 3 + Band n1 and Band 7 + Band n1 are captured in TR 37.716-11-11.

-	2nd harmonics from Band 3 may fall into n78 when dual uplink is configured with either Band 3 + Band n1 or Band 3 + Band n78.

7.36.4	∆TIB and ∆RIB values

For DC_3-7_n1-n78, the TIB,c and RIB values are same as for CA_1-3-7-42 and DC_1-3-7_n78, and are given in the tables below.

Table 7.36.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7_n1-n78 | 3 | 0.7 |
|  | 7 | 0.7 |
|  | n1 | 0.7 |
|  | n78 | 0.8 |

Table 7.36.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7_n1-n78 | 3 | 0.3 |
|  | 7 | 0.3 |
|  | n1 | 0.3 |
|  | n78 | 0.5 |

7.36.5	MSD

IMD problems are already analysed in constituent low order combinations between LTE bands and NR bands. The MSD needed are covered in the constituent low order combinations.

7.37	DC_3-3-7_n1-n78, DC_3-7-7_n1-n78, DC_3-3-7-7_n1-n78

7.37.1	Operating bands for DC

Table 7.37.1-1: DC band combination of two LTE bands + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-3-7_n1-n78, DC_3-7-7_n1-n78, DC_3-3-7-7_n1-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.37.2	Channel bandwidths per operating band for DC

Table 7.37.2-1: Supported bandwidths per DC two LTE bands + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A-3A-7A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A | 3 | 15 | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-7A-7A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 180 |
|  |  | 7 | 15 | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-3A-7A-7A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A | 3 | 15 | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  | 200 |
|  |  | 7 | 15 | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.37.3	Co-existence studies

The Co-existence studies of DC_3-3-7_n1-n78, DC_3-7-7_n1-n78, DC_3-3-7-7_n1-n78 can be covered by the fallback combinations DC_3-7_n1-n78 in section 7.36.

7.37.4	∆TIB and ∆RIB values

The TIB,c and RIB values are same as for DC_3-7_n1-n78.

Table 7.37.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-3-7_n1-n78, DC_3-7-7_n1-n78, DC_3-3-7-7_n1-n78 | 3 | 0.7 |
|  | 7 | 0.7 |
|  | n1 | 0.7 |
|  | n78 | 0.8 |

Table 7.37.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-3-7_n1-n78, DC_3-7-7_n1-n78, DC_3-3-7-7_n1-n78 | 3 | 0.3 |
|  | 7 | 0.3 |
|  | n1 | 0.3 |
|  | n78 | 0.5 |

7.37.5	MSD

IMD problems are already analysed in constituent low order combinations between LTE bands and NR bands. The MSD needed are covered in the constituent low order combinations.

7.38	DC_1-3_n5-n78

7.38.1	Operating bands for DC

Table 7.38.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3_n5-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.38.2	Channel bandwidths per operating band for DC

Table 7.38.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Max BW [MHz] |
| DC_1A-3A_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_3A_n5A DC_3A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 160 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
| DC_1A-3C_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_3A_n5A DC_3A_n78A DC_3C_n5A DC_3C_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 180 |
|  |  | 3 | 15 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |

7.38.3	Co-existence studies

The harmonic and IMD products caused by DC_1-3_n5-n78 and protected bands follow from those derived for the constituent fallback modes.

7.38.4	∆TIB and ∆RIB values

For DC_1-3_n5-n78, the TIB,c and RIB values are same as for DC_1-3-5_n78 and are given in the tables below.

Table 7.38.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n5-n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n5 | 0.3 |
|  | n78 | 0.8 |

Table 7.38.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3_n5-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n5 | 0.2 |
|  | n78 | 0.5 |

7.38.5	MSD

All MSD requirements for DC_1-3_n5-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.39	DC_1-7_n5-n78

7.39.1	Operating bands for DC

Table 7.39.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-7_n5-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.39.2	Channel bandwidths per operating band for DC

Table 7.39.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Max BW [MHz] |
| DC_1A-7A_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_7A_n5A DC_7A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 160 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
| DC_1A-7C_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_7A_n5A DC_7A_n78A DC_7C_n5A DC_7C_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 180 |
|  |  | 7 | 15 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |

7.39.3	Co-existence studies

The harmonic and IMD products caused by DC_1-7_n5-n78 and protected bands follow from those derived for the constituent fallback modes.

7.39.4	∆TIB and ∆RIB values

For DC_1-7_n5-n78, the TIB,c and RIB values are same as for DC_1-5-7_n78 and are given in the tables below.

Table 7.39.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7_n5-n78 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | n5 | 0.6 |
|  | n78 | 0.8 |

Table 7.39.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-7_n5-n78 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | n5 | 0.2 |
|  | n78 | 0.5 |

7.39.5	MSD

All MSD requirements for DC_1-7_n5-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.40	DC_1-28_n5-n78

7.40.1	Operating bands for DC

Table 7.40.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-28_n5-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.40.2	Channel bandwidths per operating band for DC

Table 7.40.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Max BW [MHz] |
| DC_1A-28A_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_28A_n5A DC_28A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 160 |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |

7.40.3	Co-existence studies

The harmonic and IMD products caused by DC_1-28_n5-n78 and protected bands follow from those derived for the constituent fallback modes.

7.40.4	∆TIB and ∆RIB values

For DC_1-28_n5-n78, the TIB,c and RIB values are derived from DC_1-20_n28-n78 and are given in the tables below.

Table 7.40.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-28_n5-n78 | 1 | 0.3 |
|  | 28 | 0.6 |
|  | n5 | 0.6 |
|  | n78 | 0.8 |

Table 7.40.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-28_n5-n78 | 1 | 0 |
|  | 28 | 0.2 |
|  | n5 | 0.2 |
|  | n78 | 0.5 |

7.40.5	MSD

All MSD requirements for DC_1-28_n5-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.41	DC_3-7_n5-n78

7.41.1	Operating bands for DC

Table 7.41.3-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7_n5-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.41.2	Channel bandwidths per operating band for DC

Table 7.41.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Max BW [MHz] |
| DC_3A-7A_n5A-n78A | DC_3A_n5A DC_3A_n78A DC_7A_n5A DC_7A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 160 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
| DC_3C-7A_n5A-n78A | DC_3A_n5A DC_3A_n78A DC_3C_n5A DC_3C_n78A DC_7A_n5A DC_7A_n78A | 3 | 15 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
| DC_3A-7C_n5A-n78A | DC_3A_n5A DC_3A_n78A DC_7A_n5A DC_7A_n78A DC_7C_n5A DC_7C_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 180 |
|  |  | 7 | 15 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
| DC_3C-7C_n5A-n78A | DC_3A_n5A DC_3A_n78A DC_3C_n5A DC_3C_n78A DC_7A_n5A DC_7A_n78A DC_7C_n5A DC_7C_n78A | 3 | 15 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 200 |
|  |  | 7 | 15 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |

7.41.3	Co-existence studies

The harmonic and IMD products caused by DC_3-7_n5-n78 and protected bands follow from those derived for the constituent fallback modes.

7.41.4	∆TIB and ∆RIB values

For DC_3-7_n5-n78, the TIB,c and RIB values are same as for DC_3-5-7_n78 and are given in the tables below.

Table 7.41.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7_n5-n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n5 | 0.6 |
|  | n78 | 0.8 |

Table 7.41.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7_n5-n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n5 | 0.2 |
|  | n78 | 0.5 |

7.41.5	MSD

All MSD requirements for DC_3-7_n5-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.42	DC_3-28_n5-n78

7.42.1	Operating bands for DC

Table 7.42.3-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-28_n5-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.42.2	Channel bandwidths per operating band for DC

Table 7.42.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Max BW [MHz] |
| DC_3A-28A_n5A-n78A | DC_3A_n5A DC_3A_n78A DC_28A_n5A DC_28A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 160 |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
| DC_3C-28A_n5A-n78A | DC_3A_n5A DC_3A_n78A DC_3C_n5A DC_3C_n78A DC_28A_n5A DC_28A_n78A | 3 | 15 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |

7.42.3	Co-existence studies

The harmonic and IMD products caused by DC_3-28_n5-n78 and protected bands follow from those derived for the constituent fallback modes.

7.42.4	∆TIB and ∆RIB values

For DC_3-28_n5-n78, the TIB,c and RIB values are derived from DC_3-20_n28-n78 and are given in the tables below.

Table 7.42.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-28_n5-n78 | 3 | 0.6 |
|  | 28 | 0.6 |
|  | n5 | 0.6 |
|  | n78 | 0.8 |

Table 7.42.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-28_n5-n78 | 3 | 0.2 |
|  | 28 | 0.2 |
|  | n5 | 0.2 |
|  | n78 | 0.5 |

7.42.5	MSD

All MSD requirements for DC_3-28_n5-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.43	DC_7-28_n5-n78

7.43.1	Operating bands for DC

Table 7.43.7-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7-28_n5-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.43.2	Channel bandwidths per operating band for DC

Table 7.43.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Max BW [MHz] |
| DC_7A-28A_n5A-n78A | DC_7A_n5A DC_7A_n78A DC_28A_n5A DC_28A_n78A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 160 |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
| DC_7C-28A_n5A-n78A | DC_7A_n5A DC_7A_n78A DC_7C_n5A DC_7C_n78A DC_28A_n5A DC_28A_n78A | 7 | 15 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |

7.43.3	Co-existence studies

The harmonic and IMD products caused by DC_7-28_n5-n78 and protected bands follow from those derived for the constituent fallback modes.

7.43.4	∆TIB and ∆RIB values

For DC_7-28_n5-n78, the TIB,c and RIB values are derived from DC_7-20_n28-n78 and are given in the tables below.

Table 7.43.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-28_n5-n78 | 7 | 0.3 |
|  | 28 | 0.6 |
|  | n5 | 0.6 |
|  | n78 | 0.8 |

Table 7.43.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-28_n5-n78 | 7 | 0 |
|  | 28 | 0.2 |
|  | n5 | 0.2 |
|  | n78 | 0.5 |

7.43.5	MSD

All MSD requirements for DC_7-28_n5-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.44	DC_1A-3A_n77A-n257

7.44.1	Operating bands for DC

Table 7.44.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3_n77-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

7.44.2	Channel bandwidths per operating band for DC

Table 7.44.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-3A_n77A-n257A | DC_1A_n77A DC_3A_n77A DC_1A_n257A DC_3A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 540 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes |  |  |
| DC_1A-3A_n77A-n257D | DC_1A_n77A DC_3A_n77A DC_1A_n257A DC_1A_n257D DC_3A_n257A DC_3A_n257D | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 540 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A_n77A-n257G | DC_1A_n77A DC_3A_n77A DC_1A_n257A DC_1A_n257G DC_3A_n257A DC_3A_n257G | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 340 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A_n77A-n257H | DC_1A_n77A DC_3A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_3A_n257A DC_3A_n257G DC_3A_n257H | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 440 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A_n77A-n257I | DC_1A_n77A DC_3A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 540 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.44.3	Co-existence studies

Co-existence studies of Band 1 + Band n77, Band 3 + Band n77, Band 1 + n257 and Band 3 + n257 are captured in TR 37.863-01-01, own Rx impact of the 3rd band is the followings

-	7th and 8th harmonics from n77 may fall into n257 and 2nd, 3rd and 5th order IMD may fall into own Rx of Band 3 when dual uplink is configured with Band 1 + Band n77

-	7th and 8th harmonics from n77 may fall into n257 and 5th order IMD may fall into own Rx of Band 1 when dual uplink is configured with Band 3 + Band n77

-	2nd harmonic from Band 1 may fall into n77 and 3rd order IMD may fall into own Rx of Band 3 when dual uplink is configured with Band 1 + Band n257

-	2nd harmonic from Band 3 may fall into n77 and 3rd order IMD may fall into own Rx of Band 1 when dual uplink is configured with Band 3 + Band n257.

7.44.4	∆TIB and ∆RIB values

For DC_1A-3A_n77-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.44.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-3A_n77-n257 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n77 | 0.8 |
|  | n257 | 0 |

Table 7.44.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-3A_n77-n257 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n77 | 0.5 |
|  | n257 | 0 |

7.44.5	MSD

IMD problems are already analysed in constituent low order combinations between LTE bands and NR bands. 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively and fallback modes are captured in TR 37.864-41-21. Therefore, additional analysis is not needed.

-	Uplink DC configuration is DC_1A_n77A: Up to 7th harmonic order will be not any impact to n257 and IMD issues on 3rd own Rx band will be covered in DC_1A-3A_n77A

-	Uplink DC configuration is DC _3A_n77A: Up to 7th harmonic order will be not any impact to n257 and IMD problems will be covered in DC_1A-3A_n77A

-	Uplink DC configuration is DC_1A_n257A: Harmonic problem already discussed and solved in DC_1A_n77A

-    Uplink DC configuration is DC_3A_n257A: Harmonic problem is already solved in DC_3A_n77A

7.45	DC_2-66_n41-n71

7.45.1	Operating bands for DC

Table 7.45.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-66_n41-n71 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

7.45.2	Channel bandwidths per operating band for DC

Table 7.45.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Max BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A-66A_n41A-n71A | DC_2A_n41A DC_2A_n71A DC_66A_n41A DC_66A_n71A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 160 |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n71 | 30 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |

7.45.3	Co-existence studies

Co-existence analysis for DC_2_n41 shows that there are IMD2 and IMD5 impacts from DC_2_n41 UL to Band n71 DL, but no impact on 66 DL

Co-existence analysis for DC_2_n71 shows that there is IMD2 impact from DC_2_n71 UL to Band n41 DL, but no impact on 66 DL

Co-existence analysis for DC_66_n41 shows that there is IMD4 impact from DC_66_n41 UL to Band 2 DL, but no impact on n71 DL

Co-existence analysis for DC_66_n71 shows that there is no impact from DC_66_n71 UL to either Band 2 DL or Band n41 DL

7.45.4	∆TIB and ∆RIB values

For DC_2-66_n41-n71 the TIB,c and RIB,c values are derived from LTE combinations CA_2-7-12-66 and are given in the tables below.

Table 7.45.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n41-n71 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n41 | 0.81 |
|  |  | 1.32 |
|  | n71 | 0.8 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

Table 7.45.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-66_n41-n71 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n41 | 0.51 |
|  |  | 12 |
|  | n71 | 0.5 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

7.45.5	MSD

UL 2_n41 into DL n71 create IMD2 and UL 2_n71 into DL n41 create IMD2, and they are both captured in the lower order combination DC_2_n41-n71.

UL 66_n41 into DL n2 create IMD4, and this is captured in the lower order combination DC_2-66_n41.

Based on the co-existence studies there are no further MSD needed to be defined.

7.46	DC_2-66_n41-n260

7.46.1	Operating bands for DC

Table 7.46.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-66_n41-n260 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

7.46.2	Channel bandwidths per operating band for DC

Table 7.46.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Max BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A-66A_n41A-n260A | DC_2A_n41A DC_66A_n41A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 540 |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n260 | 60 |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |
| DC_2A-66A_n41A-n260(2A) | DC_2A_n41A DC_66A_n41A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 940 |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n260 | See CA_n260(2A) BCS 0 in Table 5.5A.2-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A-66A_n41A-n260(3A) | DC_2A_n41A DC_66A_n41A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 1340 |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n260 | See CA_n260(3A) BCS 0 in Table 5.5A.2-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A-66A_n41A-n260(4A) | DC_2A_n41A DC_66A_n41A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 1740 |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n260 | See CA_n260(4A) BCS 0 in Table 5.5A.2-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.46.3	Co-existence studies

Co-existence analysis for DC_2_n41 shows that there is no impact from DC_2_n41 UL to either Band 66 DL or Band n260 DL.

Co-existence analysis for DC_66_n41 shows that there is no impact from DC_66_n41 UL to Band n260 DL, but that there are IMD4 from UL_66_n41 into band 2 DL.

7.46.4	∆TIB and ∆RIB values

For DC_2-66_n41-n260 the TIB,c and RIB,c values are based on DC_2-66_n41 and are given in the tables below.

Table 7.46.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n41-n260 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n41 | 0.81 |
|  |  | 1.32 |
|  | n260 | 0 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545 MHz. |  |  |

Table 7.46.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-66_n41-n260 | 2 | 0.3 |
|  | 66 | 0.5 |
|  | n41 | 0.51 |
|  |  | 12 |
|  | n260 | 0 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 – 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 – 2545 MHz. |  |  |

7.46.5	MSD

UL 66_n41 into band DL creates IMD4 which are captured in lower order combination DC_2-66_n41. Based on the co-existence studies there are no further needs to define additional MSD.

7.47	DC_2A-66A_n41A-n261

7.47.1	Operating bands for DC

Table 7.47.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2A-66A-n41A-n261 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n261 | 27500 MHz | – | 28350 MHz | 27500 MHz | – | 28350 MHz | TDD |

### 7.47.2	Channel bandwidths per operating band for DC

Table 7.47.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Max BW [MHz] |
| DC_2A-66A_n41A-n261A | DC_2A_n41A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 540 |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n261 | 60 |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |
| DC_2A-66A_n41A-n261A | DC_66A_n41A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 540 |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n261 | 60 |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |
| DC_2A-66A_n41A-n261(2A) | DC_2A_n41A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 940 |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n261 | See CA_n261(2A) in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A-66A_n41A-n261(2A) | DC_66A_n41A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 940 |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | § | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n261 | See CA_n261(2A) in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.47.3	Co-existence studies

Co-existence studies of DC_2A-n41A are captured in TR 37.716-01-01, for DC_66_n41 are captured in TR 37.716-11-11 and for EN-DC between E-UTRA and FR2 DC_2A_n261A or DC_66A_n261A there will no requirements specified.

7.47.4	∆TIB and ∆RIB values

For DC_2A-66A_n41A-n261A, the TIB,c and RIB,c values are given in the tables below based on DC_66A_n41A-n261A and DC_2A_n41A-n261A in 37.716-21-21.

Table 7.47.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2A-66A_n41A-n261A | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n41 | 0.81 |
|  |  | 1.32 |
|  | n261 | 0 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

Table 7.47.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2A-66A_n41A-n261A | 2 | 0.3 |
|  | 66 | 0.5 |
|  | n41 | 0.51 |
|  |  | 12 |
|  | n261 | 0 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

7.47.5	MSD

MSD due to close proximity is needed and is already specified in TS 38.101-3 for DC_66_n41.

7.48	DC_3-8_n78-n257

7.48.1	Operating bands for DC

Table 7.48.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-8_n78-n257 | 3 | 1710MHz | – | 1785MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 8 | 880MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

7.48.2	Channel bandwidths per operating band for DC

Table 7.48.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

|  |  | CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | UL Configuration | NR Band | SCS [kHz] | 5 |  | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_3A-8A_n78A-n257A |  | DC_3A_n78A DC_8A_n78A DC_3A_n257A DC_8A_n257A | 3 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 530 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 | 60 |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |  |
|  |  |  |  | 120 |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |  |
| DC_3A-8A_n78A-n257D |  | DC_3A_n78A DC_8A_n78A DC_3A_n257A DC_8A_n257A | 3 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 530 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-8A_n78A-n257E |  | DC_3A_n78A DC_8A_n78A DC_3A_n257A DC_8A_n257A | 3 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 730 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-8A_n78A-n257F |  | DC_3A_n78A DC_8A_n78A DC_3A_n257A DC_8A_n257A | 3 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 930 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-8A_n78A-n257G |  | DC_3A_n78A DC_8A_n78A DC_3A_n257A DC_8A_n257A | 3 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 330 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-8A_n78A-n257H |  | DC_3A_n78A DC_8A_n78A DC_3A_n257A DC_8A_n257A | 3 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 430 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-8A_n78A-n257I |  | DC_3A_n78A DC_8A_n78A DC_3A_n257A DC_8A_n257A | 3 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 530 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-8A_n78A-n257J |  | DC_3A_n78A DC_8A_n78A DC_3A_n257A DC_8A_n257A | 3 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 630 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-8A_n78A-n257K |  | DC_3A_n78A DC_8A_n78A DC_3A_n257A DC_8A_n257A | 3 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 730 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-8A_n78A-n257L |  | DC_3A_n78A DC_8A_n78A DC_3A_n257A DC_8A_n257A | 3 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 830 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-8A_n78A-n257M |  | DC_3A_n78A DC_8A_n78A DC_3A_n257A DC_8A_n257A | 3 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 930 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.48.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01, TR 37.863-02-01 and TR 37.716-21-21 respectively. Therefore, additional analysis is not needed.

7.48.4	∆TIB and ∆RIB values

For DC_3-8_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.48.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-8_n78-n257 | 3 | 0.6 |
|  | 8 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 7.48.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-8_n78-n257 | 3 | 0.2 |
|  | 8 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

7.48.5	MSD

There is no need to specify additional MSD requirement for this UL DC configuration.

7.49	DC_1-8_n78-n257

7.49.1	Operating bands for DC

Table 7.49.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-8_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

7.49.2	Channel bandwidths per operating band for DC

Table 7.49.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

|  |  | CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | UL Configuration | NR Band | SCS [kHz] | 5 |  | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-8A_n78A-n257A |  | DC_1A_n78A DC_8A_n78A DC_1A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 530 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 | 60 |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |  |
|  |  |  |  | 120 |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |  |
| DC_1A-8A_n78A-n257D |  | DC_1A_n78A DC_8A_n78A DC_1A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 530 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-8A_n78A-n257E |  | DC_1A_n78A DC_8A_n78A DC_1A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 730 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-8A_n78A-n257F |  | DC_1A_n78A DC_8A_n78A DC_1A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 930 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-8A_n78A-n257G |  | DC_1A_n78A DC_8A_n78A DC_1A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 330 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-8A_n78A-n257H |  | DC_1A_n78A DC_8A_n78A DC_1A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 430 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-8A_n78A-n257I |  | DC_1A_n78A DC_8A_n78A DC_1A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 530 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-8A_n78A-n257J |  | DC_1A_n78A DC_8A_n78A DC_1A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 630 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-8A_n78A-n257K |  | DC_1A_n78A DC_8A_n78A DC_1A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 730 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-8A_n78A-n257L |  | DC_1A_n78A DC_8A_n78A DC_1A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 830 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-8A_n78A-n257M |  | DC_1A_n78A DC_8A_n78A DC_1A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 930 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.49.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01, TR 37.863-02-01 and TR 37.716-21-21 respectively. Therefore, additional analysis is not needed.

7.49.4	∆TIB and ∆RIB values

For DC_1-8_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.49.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8_n78-n257 | 1 | 0.3 |
|  | 8 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 7.49.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-8_n78-n257 | 1 | 0 |
|  | 8 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

7.49.5	MSD

There is no need to specify additional MSD requirement for this UL DC configuration.

7.50	DC_2A-7A-7A_n38A-n78A

7.50.1	Operating bands for DC

Table 7.50.1-1: DC band combination of two LTE band + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2A-7A-7A_n38A-n78A | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.50.2	Channel bandwidths per operating band for DC

Table 7.50.2-1: Supported bandwidths per DC band combination of two LTE band + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_2A-7A-7A_n38A-n78A | DC_2A-n78A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 180 |
|  |  | 7 | 15 | See the CA_7A-7A configuration in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n38 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.50.3	Co-existence studies

The co-existence studies for the lower order DC configuration (DC_2-7_n78) is already captured in TR 37.716-21-11. For additional receiver band (band n38), there is no MSD issue for this DC configuration.

7.50.4	∆TIB and ∆RIB values

For DC_2A-7A-7A_n38A-n78A, the TIB,c and RIB values are given in the tables below.

Table 7.50.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2A-7A-7A_n38A-n78A | 2 | 0.6 |
|  | n78 | 0.8 |

Table 7.50.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2A-7A-7A_n38A-n78A | 2 | 0.2 |
|  | 7 | 0 |
|  | n38 | 0 |
|  | n78 | 0.5 |

7.50.5	MSD

No MSD is required for this DC configuration.

7.51	DC_7A-7A-66A_n38A-n78A

7.51.1	Operating bands for DC

Table 7.51.1-1: DC band combination of two LTE band + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7A-7A-66A_n38A-n78A | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.51.2	Channel bandwidths per operating band for DC

Table 7.51.2-1: Supported bandwidths per DC band combination of two LTE band + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | UL Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_7A-7A-66A_n38A-n78A |  | DC_66A_ n78A | 7 | 15 | See the CA_7A-7A configuration in TS36.101 |  |  |  |  |  |  |  |  |  |  |  | 180 |
|  |  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | n38 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.51.3	Co-existence studies

The co-existence studies for the lower order DC configuration (DC_7-66_n78) is already captured in TR 37.716-21-11. For additional receiver band (band n38), there is no MSD issue for this DC configuration.

7.51.4	∆TIB and ∆RIB values

For DC_7A-7A-66A_n38A-n78A, the TIB,c and RIB values are given in the tables below.

Table 7.51.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-7A-66A_n38A-n78A | 66 | TBD |
|  | n78 | 0.8 |

Table 7.51.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7A-7A-66A_n38A-n78A | 7 | 0 |
|  | 66 | TBD |
|  | n38 | TBD |
|  | n78 | 0.5 |

7.51.5	MSD

No MSD is required for this DC configuration.

8	EN-DC band combinations of LTE 3 bands DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part

8.1	DC_1A-3A-5A_n78A-n257

8.1.1	Operating bands for DC

Table 8.1.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-3A-5A_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

8.1.2	Channel bandwidths per operating band for DC

Table 8.1.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-3A-5A_n78A-n257D | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 550 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A_n78A-n257E | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 750 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A_n78A-n257F | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 950 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A_n78A-n257G | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 350 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A_n78A-n257H | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 450 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A_n78A-n257I | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 550 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A_n78A-n257J | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 650 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A_n78A-n257K | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 750 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A_n78A-n257L | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 850 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A_n78A-n257M | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 950 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.1.3	Co-existence studies

Co-existence studies of this 5DL/2UL DC configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively. Furthermore, harmonics and IMD issues are covered by constituent fallback modes in TR 37.864-41-21. Therefore, additional analysis is not needed.

8.1.4	∆TIB and ∆RIB values

For DC_1A-3A-5A_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 8.1.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-3A-5A_n78-n257 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 5 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 8.1.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-3A-5A_n78-n257 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 5 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

8.1.5	MSD

MSD of this 5DL/2UL DC configuration are already covered by those of 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively. And constituent fallback modes will be covered with TR 37.864-41-21. Therefore, additional analysis is not needed.

8.2	DC_1A-3A-7A_n78A-n257

8.2.1	Operating bands for DC

Table 8.2.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-3A-7A_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

8.2.2	Channel bandwidths per operating band for DC

Table 8.2.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-3A-7A_n78A-n257D | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 560 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-7A_n78A-n257E | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 760 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-7A_n78A-n257F | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 960 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-7A_n78A-n257G | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 360 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-7A_n78A-n257H | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 460 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-7A_n78A-n257I | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 560 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-7A_n78A-n257J | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 660 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-7A_n78A-n257K | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 760 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-7A_n78A-n257L | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 860 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-7A_n78A-n257M | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 960 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.2.3	Co-existence studies

Co-existence studies of this 5DL/2UL DC configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively. Furthermore, harmonics and IMD issues are covered by constituent fallback modes in TR 37.864-41-21. Therefore, additional analysis is not needed.

8.2.4	∆TIB and ∆RIB values

For DC_1A-3A-7A_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 8.2.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-3A-7A_n78-n257 | 1 | 0.7 |
|  | 3 | 0.7 |
|  | 7 | 0.7 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 8.2.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-3A-7A_n78-n257 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 7 | 0.3 |
|  | n78 | 0.5 |
|  | n257 | 0 |

8.2.5	MSD

MSD of this 5DL/2UL DC configuration are already covered by those of 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively. And constituent fallback modes will be covered with TR 37.864-41-21. Therefore, additional analysis is not needed.

8.3	DC_1A-3A-7A-7A_n78A-n257

8.3.1	Operating bands for DC

Table 8.3.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-3A-7A-7A_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

8.3.2	Channel bandwidths per operating band for DC

Table 8.3.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-3A-7A-7A_n78A-n257D | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 580 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-7A-7A_n78A-n257E | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 780 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-7A-7A_n78A-n257F | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 980 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-7A-7A_n78A-n257G | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 380 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-7A-7A_n78A-n257H | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 480 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-7A-7A_n78A-n257I | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 580 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-7A-7A_n78A-n257J | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 680 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-7A-7A_n78A-n257K | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 780 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-7A-7A_n78A-n257L | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 880 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-7A-7A_n78A-n257M | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 980 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.3.3	Co-existence studies

Co-existence studies of this 5DL/2UL DC configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively. Furthermore, harmonics and IMD issues are covered by constituent fallback modes in TR 37.864-41-21. Therefore, additional analysis is not needed.

8.3.4	∆TIB and ∆RIB values

For DC_1A-3A-7A-7A_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 8.3.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-3A-7A-7A_n78-n257 | 1 | 0.7 |
|  | 3 | 0.7 |
|  | 7 | 0.7 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 8.3.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-3A-7A-7A_n78-n257 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 7 | 0.3 |
|  | n78 | 0.5 |
|  | n257 | 0 |

8.3.5	MSD

MSD of this 5DL/2UL DC configuration are already covered by those of 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively. And constituent fallback modes will be covered with TR 37.864-41-21. Therefore, additional analysis is not needed.

8.4	DC_1A-5A-7A_n78A-n257

8.4.1	Operating bands for DC

Table 8.4.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-5A-7A_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

8.4.2	Channel bandwidths per operating band for DC

Table 8.4.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-5A-7A_n78A-n257D | DC_1A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 550 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A-7A_n78A-n257E | DC_1A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 750 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A-7A_n78A-n257F | DC_1A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 950 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A-7A_n78A-n257G | DC_1A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 350 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A-7A_n78A-n257H | DC_1A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 450 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A-7A_n78A-n257I | DC_1A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 550 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A-7A_n78A-n257J | DC_1A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 650 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A-7A_n78A-n257K | DC_1A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 750 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A-7A_n78A-n257L | DC_1A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 850 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A-7A_n78A-n257M | DC_1A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 950 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.4.3	Co-existence studies

Co-existence studies of this 5DL/2UL DC configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively. Furthermore, harmonics and IMD issues are covered by constituent fallback modes in TR 37.864-41-21. Therefore, additional analysis is not needed.

8.4.4	∆TIB and ∆RIB values

For DC_1A-5A-7A_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 8.4.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-5A-7A_n78-n257 | 1 | 0.6 |
|  | 5 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 8.4.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-5A-7A_n78-n257 | 1 | 0.2 |
|  | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

8.4.5	MSD

MSD of this 5DL/2UL DC configuration are already covered by those of 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively. And constituent fallback modes will be covered with TR 37.864-41-21. Therefore, additional analysis is not needed.

8.5	DC_1A-5A-7A-7A_n78A-n257

8.5.1	Operating bands for DC

Table 8.5.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-5A-7A7A_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

8.5.2	Channel bandwidths per operating band for DC

Table 8.5.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-5A-7A-7A_n78A-n257D | DC_1A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 570 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A-7A-7A_n78A-n257E | DC_1A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 770 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A-7A-7A_n78A-n257F | DC_1A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 970 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A-7A-7A_n78A-n257G | DC_1A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 370 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A-7A-7A_n78A-n257H | DC_1A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 470 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A-7A-7A_n78A-n257I | DC_1A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 570 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A-7A-7A_n78A-n257J | DC_1A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 670 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A-7A-7A_n78A-n257K | DC_1A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 770 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A-7A-7A_n78A-n257L | DC_1A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 870 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A-7A-7A_n78A-n257M | DC_1A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 970 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.5.3	Co-existence studies

Co-existence studies of this 5DL/2UL DC configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively. Furthermore, harmonics and IMD issues are covered by constituent fallback modes in TR 37.864-41-21. Therefore, additional analysis is not needed.

8.5.4	∆TIB and ∆RIB values

For DC_1A-5A-7A-7A_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 8.5.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-5A-7A-7A_n78-n257 | 1 | 0.6 |
|  | 5 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 8.5.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-5A-7A-7A_n78-n257 | 1 | 0.2 |
|  | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

8.5.5	MSD

MSD of this 5DL/2UL DC configuration are already covered by those of 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively. And constituent fallback modes will be covered with TR 37.864-41-21. Therefore, additional analysis is not needed.

8.6	DC_3A-5A-7A_n78A-n257

8.6.1	Operating bands for DC

Table 8.6.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-5A-7A_n78-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

8.6.2	Channel bandwidths per operating band for DC

Table 8.6.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_3A-5A-7A_n78A-n257D | DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 550 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A-7A_n78A-n257E | DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 750 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A-7A_n78A-n257F | DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 950 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A-7A_n78A-n257G | DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 350 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A-7A_n78A-n257H | DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 450 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A-7A_n78A-n257I | DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 550 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A-7A_n78A-n257J | DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 650 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A-7A_n78A-n257K | DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 750 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A-7A_n78A-n257L | DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 850 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A-7A_n78A-n257M | DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 950 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.6.3	Co-existence studies

Co-existence studies of this 5DL/2UL DC configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively. Furthermore, harmonics and IMD issues are covered by constituent fallback modes in TR 37.864-41-21. Therefore, additional analysis is not needed.

8.6.4	∆TIB and ∆RIB values

For DC_3A-5A-7A_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 8.6.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-5A-7A_n78-n257 | 3 | 0.6 |
|  | 5 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 8.6.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-5A-7A_n78-n257 | 3 | 0.2 |
|  | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

8.6.5	MSD

MSD of this 5DL/2UL DC configuration are already covered by those of 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively. And constituent fallback modes will be covered with TR 37.864-41-21. Therefore, additional analysis is not needed.

8.7	DC_3A-5A-7A-7A_n78A-n257

8.7.1	Operating bands for DC

Table 8.7.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-5A-7A-7A_n78-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

8.7.2	Channel bandwidths per operating band for DC

Table 8.7.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_3A-5A-7A-7A_n78A-n257D | DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 570 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A-7A-7A_n78A-n257E | DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 770 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A-7A-7A_n78A-n257F | DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 970 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A-7A-7A_n78A-n257G | DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 370 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A-7A-7A_n78A-n257H | DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 470 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A-7A-7A_n78A-n257I | DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 570 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A-7A-7A_n78A-n257J | DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 670 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A-7A-7A_n78A-n257K | DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 770 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A-7A-7A_n78A-n257L | DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 870 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A-7A-7A_n78A-n257M | DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 970 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.7.3	Co-existence studies

Co-existence studies of this 5DL/2UL DC configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively. Furthermore, harmonics and IMD issues are covered by constituent fallback modes in TR 37.864-41-21. Therefore, additional analysis is not needed.

8.7.4	∆TIB and ∆RIB values

For DC_3A-5A-7A-7A_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 8.7.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-5A-7A-7A_n78-n257 | 3 | 0.6 |
|  | 5 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 8.7.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-5A-7A-7A_n78-n257 | 3 | 0.2 |
|  | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

8.7.5	MSD

MSD of this 5DL/2UL DC configuration are already covered by those of 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively. And constituent fallback modes will be covered with TR 37.864-41-21. Therefore, additional analysis is not needed.

8.8	DC_1-3-7_n28-n78

8.8.1	Operating bands for DC

Table 8.8.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-7_n28-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

8.8.2	Channel bandwidths per operating band for DC

Table 8.8.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A-7C_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_7C_n28A DC_7C_n78A DC_7A_n28A DC_7A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 200 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | See CA_7C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-3C-7A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_3C_n28A DC_3C_n78A DC_7A_n28A DC_7A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 200 |
|  |  | 3 | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-3C-7C_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_3C_n28A DC_3C_n78A DC_7A_n28A DC_7A_n78A DC_7C_n28A DC_7C_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 220 |
|  |  | 3 | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | See CA_7C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

8.8.3	Co-existence studies

The harmonic and IMD products caused by DC_1-3-7_n28-n78 and protected bands follow from those derived for the constituent fallback modes. The summary of self-interference issues are provided in TR 37.716-21-21, Table 5.3-2.

8.8.4	∆TIB and ∆RIB values

For DC_1-3-7_n28-n78, the TIB,c and RIB values are given in the tables below.

Table 8.8.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-7_n28-n78 | 1 | 0.7 |
|  | 3 | 0.7 |
|  | 7 | 0.7 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |

Table 8.8.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-7_n28-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |

8.8.5	MSD

All MSD requirements for DC_1-3-7_n28-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

8.9	DC_1-19-42_n77-n79

8.9.1	Operating bands for DC

Table 8.9.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-19-42_n77-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

8.9.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 8.9.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-19A-42A_n77A-n79A DC_1A-19A-42C_n77A-n79A | DC_19A_n77A DC_19A_n79A | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 275 |
|  |  | 19 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

8.9.3	Co-existence studies

Co-existence studies of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

8.9.4	∆TIB and ∆RIB values

For DC_1-19-42_n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 8.9.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-19-42_n77-n79 | 1 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |

Table 8.9.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-19-42_n77-n79 | 1 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |

8.9.5	MSD

MSD analysis of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed

8.10	DC_1-19-42_n78-n79

8.10.1	Operating bands for DC

Table 8.10.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-19-42_n78-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

8.10.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 8.10.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-19A-42A_n78A-n79A DC_1A-19A-42C_n78A-n79A | DC_19A_n78A DC_19A_n79A | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 275 |
|  |  | 19 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

8.10.3	Co-existence studies

Co-existence studies of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

8.10.4	∆TIB and ∆RIB values

For DC_1-19-42_n78-n79, the TIB,c and RIB,c values are given in the tables below.

Table 8.10.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-19-42_n78-n79 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |

Table 8.10.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-19-42_n78-n79 | 42 | 0.5 |
|  | n78 | 0.5 |

8.10.5	MSD

MSD analysis of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

8.11	DC_1-21-42_n77-n79

8.11.1	Operating bands for DC

Table 8.11.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-21-42_n77-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

8.11.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 8.11.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-21A-42A_n77A-n79A DC_1A-21A-42C_n77A-n79A | DC_1A_n77A DC_1A_n79A | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 275 |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

8.11.3	Co-existence studies

Co-existence studies of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

8.11.4	∆TIB and ∆RIB values

For DC_1-21-42_n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 8.11.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-21-42_n77-n79 | 1 | 0.6 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |

Table 8.11.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-21-42_n77-n79 | 1 | 0.2 |
|  | 21 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |

8.11.5	MSD

MSD analysis of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

8.12	DC_1-21-42_n78-n79

8.12.1	Operating bands for DC

Table 8.12.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-21-42_n78-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

8.12.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 8.12.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-21A-42A_n78A-n79A DC_1A-21A-42C_n78A-n79A | DC_1A_n78A DC_1A_n79A | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 275 |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

8.12.3	Co-existence studies

Co-existence studies of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

8.12.4	∆TIB and ∆RIB values

For DC_1-21-42_n78-n79, the TIB,c and RIB,c values are given in the tables below.

Table 8.12.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-21-42_n78-n79 | 1 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |

Table 8.12.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-21-42_n78-n79 | 21 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |

8.12.5	MSD

MSD analysis of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

8.13	DC_1-3-21_n77-n79

8.13.1	Operating bands for DC

Table 8.13.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-21_n77-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

8.13.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 8.13.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-3A-21A_n77A-n79A | DC_3A_n77A DC_3A_n79A | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 255 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

8.13.3	Co-existence studies

Co-existence studies of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

8.13.4	∆TIB and ∆RIB values

For DC_1-3-21_n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 8.13.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-21_n77-n79 | 1 | 0.6 |
|  | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n77 | 0.8 |

Table 8.13.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-21_n77-n79 | 1 | 0.2 |
|  | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n77 | 0.5 |

8.13.5	MSD

MSD analysis of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

8.14	DC_1-3-21_n78-n79

8.14.1	Operating bands for DC

Table 8.14.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-21_n78-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

8.14.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 8.14.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-3A-21A_n78A-n79A | DC_3A_n78A DC_3A_n79A | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 255 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

8.14.3	Co-existence studies

Co-existence studies of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

8.14.4	∆TIB and ∆RIB values

For DC_1-3-21_n78-n79, the TIB,c and RIB,c values are given in the tables below.

Table 8.14.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-21_n78-n79 | 1 | 0.6 |
|  | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n78 | 0.8 |

Table 8.14.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-21_n78-n79 | 1 | 0.2 |
|  | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n78 | 0.5 |

8.14.5	MSD

MSD analysis of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

8.15	DC_19-21-42_n77-n79

8.15.1	Operating bands for DC

Table 8.15.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19-21-42_n77-n79 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

8.15.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 8.15.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_19A-21A-42A_n77A-n79A DC_19A-21A-42C_n77A-n79A | DC_19A_n77A DC_19A_n79A | 19 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 270 |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

8.15.3	Co-existence studies

Co-existence studies of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

8.15.4	∆TIB and ∆RIB values

For DC_19-21-42_n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 8.15.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19-21-42_n77-n79 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |

Table 8.15.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19-21-42_n77-n79 | 42 | 0.5 |
|  | n77 | 0.5 |

8.15.5	MSD

MSD analysis of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

8.16	DC_19-21-42_n78-n79

8.16.1	Operating bands for DC

Table 8.16.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19-21-42_n78-n79 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

8.16.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 8.16.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_19A-21A-42A_n78A-n79A DC_19A-21A-42C_n78A-n79A | DC_19A_n78A DC_19A_n79A | 19 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 270 |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

8.16.3	Co-existence studies

Co-existence studies of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

8.16.4	∆TIB and ∆RIB values

For DC_19-21-42_n78-n79, the TIB,c and RIB,c values are given in the tables below.

Table 8.16.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19-21-42_n78-n79 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |

Table 8.16.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19-21-42_n78-n79 | 42 | 0.5 |
|  | n78 | 0.5 |

8.16.5	MSD

MSD analysis of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

8.17	DC_1-7-28_n5-n78

8.17.1	Operating bands for DC

Table 8.17.1-1: DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-7-28_n5-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

8.17.2	Channel bandwidths per operating band for DC

Table 8.17.2-1: Supported bandwidths per DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Max BW [MHz] |
| DC_1A-7A-28A_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_7A_n5A DC_7A_n78A DC_28A_n5A DC_28A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 180 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
| DC_1A-7C-28A_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_7A_n5A DC_7C_n5A DC_7A_n78A DC_7C_n78A DC_28A_n5A DC_28A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 200 |
|  |  | 7 | 15 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |

8.17.3	Co-existence studies

The harmonic and IMD products caused by DC_1-7-28_n5-n78 and protected bands follow from those derived for the constituent fallback modes.

8.17.4	∆TIB and ∆RIB values

For DC_1-7-28_n5-n78, the TIB,c and RIB values are derived from DC_1-7-20_n28-n78 and are given in the tables below.

Table 8.17.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7-28_n5-n78 | 1 | 0.6 |
|  | 7 | 0.7 |
|  | 28 | 0.6 |
|  | n5 | 0.6 |
|  | n78 | 0.8 |

Table 8.17.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-7-28_n5-n78 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | 28 | 0.2 |
|  | n5 | 0.2 |
|  | n78 | 0.5 |

8.17.5	MSD

All MSD requirements for DC_1-7-28_n5-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

8.18	DC_3-7-28_n5-n78

8.18.1	Operating bands for DC

Table 8.18.1-1: DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7-28_n5-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

8.18.2	Channel bandwidths per operating band for DC

Table 8.18.2-1: Supported bandwidths per DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Max BW [MHz] |
| DC_3A-7A-28A_n5A-n78A | DC_3A_n5A DC_3A_n78A DC_7A_n5A DC_7A_n78A DC_28A_n5A DC_28A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 180 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
| DC_3C-7A-28A_n5A-n78A | DC_3A_n5A DC_3C_n5A DC_3A_n78A DC_3C_n78A DC_7A_n5A DC_7A_n78A DC_28A_n5A DC_28A_n78A | 3 | 15 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 200 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
| DC_3A-7C-28A_n5A-n78A | DC_3A_n5A DC_3A_n78A DC_7A_n5A DC_7C_n5A DC_7A_n78A DC_7C_n78A DC_28A_n5A DC_28A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 200 |
|  |  | 7 | 15 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
| DC_3C-7C-28A_n5A-n78A | DC_3A_n5A DC_3C_n5A DC_3A_n78A DC_3C_n78A DC_7A_n5A DC_7C_n5A DC_7A_n78A DC_7C_n78A DC_28A_n5A DC_28A_n78A | 3 | 15 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 220 |
|  |  | 7 | 15 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |

8.18.3	Co-existence studies

The harmonic and IMD products caused by DC_3-7-28_n5-n78 and protected bands follow from those derived for the constituent fallback modes.

8.18.4	∆TIB and ∆RIB values

For DC_3-7-28_n5-n78, the TIB,c and RIB values are derived from DC_3-7-20_n28-n78 and are given in the tables below.

Table 8.18.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-28_n5-n78 | 3 | 0.6 |
|  | 7 | 0.7 |
|  | 28 | 0.6 |
|  | n5 | 0.6 |
|  | n78 | 0.8 |

Table 8.18.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7-28_n5-n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 28 | 0.2 |
|  | n5 | 0.2 |
|  | n78 | 0.5 |

8.18.5	MSD

All MSD requirements for DC_3-7-28_n5-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

8.19	DC_1-3-7_n5-n78

8.19.1	Operating bands for DC

Table 8.19.1-1: DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-7_n5-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

8.19.2	Channel bandwidths per operating band for DC

Table 8.19.2-1: Supported bandwidths per DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Max BW [MHz] |
| DC_1A-3A-7A_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_3A_n5A DC_3A_n78A DC_7A_n5A DC_7A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 180 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
| DC_1A-3C-7A_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_3C_n5A DC_3C_n78A DC_7A_n5A DC_7A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 200 |
|  |  | 3 | 15 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
| DC_1A-3A-7C_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_3A_n5A DC_3A_n78A DC_7C_n5A DC_7C_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 200 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
| DC_1A-3C-7C_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_3C_n5A DC_3C_n78A DC_7C_n5A DC_7C_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 220 |
|  |  | 3 | 15 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |

8.19.3	Co-existence studies

The harmonic and IMD products caused by DC_1-3-7_n5-n78 and protected bands follow from those derived for the constituent fallback modes.

8.19.4	∆TIB and ∆RIB values

For DC_1-3-7_n5-n78, the TIB,c and RIB values are same as for DC_1-3-5-7_n78 and are given in the tables below.

Table 8.19.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-7_n5-n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n5 | 0.6 |
|  | n78 | 0.8 |

Table 8.19.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-7_n5-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n5 | 0.2 |
|  | n78 | 0.5 |

8.19.5	MSD

All MSD requirements for DC_1-3-7_n5-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

8.20	DC_1-3-28_n5-n78

8.20.1	Operating bands for DC

Table 8.20.1-1: DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-28_n5-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

8.20.2	Channel bandwidths per operating band for DC

Table 8.20.2-1: Supported bandwidths per DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Max BW [MHz] |
| DC_1A-3A-28A_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_3A_n5A DC_3A_n78A DC_28A_n5A DC_28A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 180 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
| DC_1A-3C-28A_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_3C_n5A DC_3C_n78A DC_28A_n5A DC_28A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 200 |
|  |  | 3 | 15 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |

8.20.3	Co-existence studies

The harmonic and IMD products caused by DC_1-3-28_n5-n78 and protected bands follow from those derived for the constituent fallback modes.

8.20.4	∆TIB and ∆RIB values

For DC_1-3-28_n5-n78, the TIB,c and RIB values are derived from DC_1-3-20_n28-n78 and are given in the tables below.

Table 8.20.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-28_n5-n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 28 | 0.6 |
|  | n5 | 0.6 |
|  | n78 | 0.8 |

Table 8.20.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-28_n5-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 28 | 0.2 |
|  | n5 | 0.2 |
|  | n78 | 0.5 |

8.20.5	MSD

All MSD requirements for DC_1-3-28_n5-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

8.21	DC_1-3-8-n78-n257

8.21.1	Operating bands for DC

Table 8.21.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-8_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 8 | 880MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

8.21.2	Channel bandwidths per operating band for DC

Table 8.21.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

|  |  | CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | UL Configuration | NR Band | SCS [kHz] | 5 |  | 10 |  | 15 |  | 20 |  | 40 |  | 50 | 60 |  | 80 |  | 90 | 100 |  | 200 |  | 400 |  | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-3A-8A_n78A-n257A |  | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_1A_n257A DC_3A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 550 |  |
|  |  |  | 3 | 15 | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | n257 | 60 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  | Yes |  |  |  |  |  |
|  |  |  |  | 120 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  |
| DC_1A-3A-8A_n78A-n257D |  | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_1A_n257A DC_3A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 550 |  |
|  |  |  | 3 | 15 | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-8A_n78A-n257E |  | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_1A_n257A DC_3A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 750 |  |
|  |  |  | 3 | 15 | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-8A_n78A-n257F |  | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_1A_n257A DC_3A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 950 |  |
|  |  |  | 3 | 15 | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-8A_n78A-n257G |  | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_1A_n257A DC_3A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 350 |  |
|  |  |  | 3 | 15 | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-8A_n78A-n257H |  | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_1A_n257A DC_3A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 450 |  |
|  |  |  | 3 | 15 | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-8A_n78A-n257I |  | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_1A_n257A DC_3A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 550 |  |
|  |  |  | 3 | 15 | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-8A_n78A-n257J |  | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_1A_n257A DC_3A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 650 |  |
|  |  |  | 3 | 15 | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-8A_n78A-n257K |  | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_1A_n257A DC_3A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 750 |  |
|  |  |  | 3 | 15 | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-8A_n78A-n257L |  | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_1A_n257A DC_3A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 850 |  |
|  |  |  | 3 | 15 | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-8A_n78A-n257M |  | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_1A_n257A DC_3A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 950 |  |
|  |  |  | 3 | 15 | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3C-8A_n78A-n257A |  | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_1A_n257A DC_3A_n257A DC_8A_n257A | 1 | 15 |  | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 570 |  |
|  |  |  | 3 |  |  | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8 | 15 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | n257 | 60 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  | Yes |  | Yes |  |  |  |  |
|  |  |  |  | 120 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |
| DC_1A-3C-8A_n78A-n257D |  | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_1A_n257A DC_3A_n257A DC_8A_n257A | 1 | 15 |  | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 570 |  |
|  |  |  | 3 |  |  | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8 | 15 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3C-8A_n78A-n257E |  | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_1A_n257A DC_3A_n257A DC_8A_n257A | 1 | 15 |  | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 770 |  |
|  |  |  | 3 |  |  | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8 | 15 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3C-8A_n78A-n257F |  | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_1A_n257A DC_3A_n257A DC_8A_n257A | 1 | 15 |  | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 970 |  |
|  |  |  | 3 |  |  | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8 | 15 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3C-8A_n78A-n257G |  | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_1A_n257A DC_3A_n257A DC_8A_n257A | 1 | 15 |  | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 370 |  |
|  |  |  | 3 |  |  | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8 | 15 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3C-8A_n78A-n257H |  | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_1A_n257A DC_3A_n257A DC_8A_n257A | 1 | 15 |  | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 470 |  |
|  |  |  | 3 |  |  | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8 | 15 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3C-8A_n78A-n257I |  | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_1A_n257A DC_3A_n257A DC_8A_n257A | 1 | 15 |  | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 570 |  |
|  |  |  | 3 |  |  | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8 | 15 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3C-8A_n78A-n257J |  | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_1A_n257A DC_3A_n257A DC_8A_n257A | 1 | 15 |  | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 670 |  |
|  |  |  | 3 |  |  | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8 | 15 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3C-8A_n78A-n257K |  | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_1A_n257A DC_3A_n257A DC_8A_n257A | 1 | 15 |  | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 770 |  |
|  |  |  | 3 |  |  | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8 | 15 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3C-8A_n78A-n257L |  | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_1A_n257A DC_3A_n257A DC_8A_n257A | 1 | 15 |  | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 870 |  |
|  |  |  | 3 |  |  | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8 | 15 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3C-8A_n78A-n257M |  | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_1A_n257A DC_3A_n257A DC_8A_n257A | 1 | 15 |  | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 970 |  |
|  |  |  | 3 |  |  | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8 | 15 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.21.3	Co-existence studies

Co-existence studies of this 5DL/2UL DC configuration are already covered by those of the 2DL/2UL, 3DL/2UL and 4UL/2UL captured in TR 37.863-01-01, TR 37.863-02-01, TR 37.863-03-01 and TR 37.716-21-21 respectively. Therefore, additional analysis is not needed.

8.21.4	∆TIB and ∆RIB values

For DC_1-3-8_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 8.21.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-8_n78-n257 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 8 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 8.21.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3-8_n78-n257 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 8 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

8.21.5	MSD

There is no need to specify additional MSD requirement for this UL DC configuration.

9	EN-DC band combinations of LTE 4 band DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part

9.1	DC_1A-3A-5A-7A_n78A-n257

9.1.1	Operating bands for DC

Table 9.1.1-1: DC band combination of LTE 4DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-3A-5A-7A_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

9.1.2	Channel bandwidths per operating band for DC

Table 9.1.2-1: Supported bandwidths per DC LTE 4DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-3A-5A-7A_n78A-n257D | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 570 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A-7A_n78A-n257E | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 770 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A-7A_n78A-n257F | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 970 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A-7A_n78A-n257G | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 370 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A-7A_n78A-n257H | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 470 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A-7A_n78A-n257I | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 570 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A-7A_n78A-n257J | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 670 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A-7A_n78A-n257K | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 770 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A-7A_n78A-n257L | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 870 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A-7A_n78A-n257M | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 970 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

9.1.3	Co-existence studies

Co-existence studies of this 5DL/2UL DC configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively. Furthermore, harmonics and IMD issues are covered by constituent fallback modes in TR 37.864-41-21. Therefore, additional analysis is not needed.

9.1.4	∆TIB and ∆RIB values

For DC_1A-3A-5A-7A_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 9.1.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-3A-5A-7A_n78-n257 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 5 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 9.1.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-3A-5A-7A_n78-n257 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

9.1.5	MSD

MSD of this 5DL/2UL DC configuration are already covered by those of 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively. And constituent fallback modes will be covered with TR 37.864-41-21. Therefore, additional analysis is not needed.

9.2	DC_1A-3A-5A-7A-7A_n78A-n257

9.2.1	Operating bands for DC

Table 9.2.1-1: DC band combination of LTE 4DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-3A-5A-7A-7A_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

9.2.2	Channel bandwidths per operating band for DC

Table 9.2.2-1: Supported bandwidths per DC LTE 4DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-3A-5A-7A-7A_n78A-n257D | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 590 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A-7A-7A_n78A-n257E | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 790 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A-7A-7A_n78A-n257F | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 990 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A-7A-7A_n78A-n257G | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 390 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A-7A-7A_n78A-n257H | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 490 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A-7A-7A_n78A-n257I | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 590 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A-7A-7A_n78A-n257J | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 690 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A-7A-7A_n78A-n257K | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 790 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A-7A-7A_n78A-n257L | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 890 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A-7A-7A_n78A-n257M | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 990 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

9.2.3	Co-existence studies

Co-existence studies of this 5DL/2UL DC configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively. Furthermore, harmonics and IMD issues are covered by constituent fallback modes in TR 37.864-41-21. Therefore, additional analysis is not needed.

9.2.4	∆TIB and ∆RIB values

For DC_1A-3A-5A-7A-7A_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 9.2.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-3A-5A-7A-7A_n78-n257 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 5 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 9.2.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-3A-5A-7A-7A_n78-n257 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

9.2.5	MSD

MSD of this 5DL/2UL DC configuration are already covered by those of 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively. And constituent fallback modes will be covered with TR 37.864-41-21. Therefore, additional analysis is not needed.

9.3	DC_1-3-7-28_n5-n78

9.3.1	Operating bands for DC

Table 9.3.1-1: DC band combination of LTE 4DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-7-28_n5-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

9.3.2	Channel bandwidths per operating band for DC

Table 9.3.2-1: Supported bandwidths per DC band combination of LTE 4DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Max BW [MHz] |
| DC_1A-3A-7A-28A_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_3A_n5A DC_3A_n78A DC_7A_n5A DC_7A_n78A DC_28A_n5A DC_28A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 200 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
| DC_1A-3C-7A-28A_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_3A_n5A DC_3C_n5A DC_3A_n78A DC_3C_n78A DC_7A_n5A DC_7A_n78A DC_28A_n5A DC_28A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 220 |
|  |  | 3 | 15 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
| DC_1A-3A-7C-28A_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_3A_n5A DC_3A_n78A DC_7A_n5A DC_7C_n5A DC_7A_n78A DC_7C_n78A DC_28A_n5A DC_28A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 220 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
| DC_1A-3C-7C-28A_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_3A_n5A DC_3C_n5A DC_3A_n78A DC_3C_n78A DC_7A_n5A DC_7C_n5A DC_7A_n78A DC_7C_n78A DC_28A_n5A DC_28A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 240 |
|  |  | 3 | 15 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |

9.3.3	Co-existence studies

The harmonic and IMD products caused by DC_1-3-7-28_n5-n78 and protected bands follow from those derived for the constituent fallback modes.

9.3.4	∆TIB and ∆RIB values

For DC_1-3-7-28_n5-n78, the TIB,c and RIB values are derived from DC_1-3-7-20_n28-n78 and are given in the tables below.

Table 9.3.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-7-28_n5-n78 | 1 | 0.7 |
|  | 3 | 0.7 |
|  | 7 | 0.7 |
|  | 28 | 0.6 |
|  | n5 | 0.6 |
|  | n78 | 0.8 |

Table 9.3.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-7-28_n5-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 28 | 0.2 |
|  | n5 | 0.2 |
|  | n78 | 0.5 |

9.3.5	MSD

All MSD requirements for DC_1-3-7-28_n5-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

Annex A: Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Subject/Comment | Old | New |
| 2018-08 | RAN4 #88 | R4-1810526 |  |  | TR37.716-21-21 Skeleton for EN-DC band combinations of LTE xDL/1UL (x=1,2,3,4) + inter-band NR 2DL/1UL | N/A | 0.0.1 |
|  |  |  |  |  | Approved TPs and revised WID in RAN4 #88 - R4-1810307: TP on general part for EN-DC LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16 - R4-1810319: TP to TR 37.864-xx-21: channel bandwidth and co-existence studies for DC_3_n1-n77 - R4-1811502: Revised WID for LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16 |  |  |
| 2018-10 | RAN4 #88BIS | R4-1813794 |  |  | Updated TR37.716-21-21 for EN-DC band combinations of LTE xDL/1UL (x=1,2,3,4) + inter-band NR 2DL/1UL | 0.0.1 | 0.1.0 |
|  |  |  |  |  | Approved TPs and revised WID in RAN4 #88BIS - R4-1812242: TP on summary of self-interference analysis for EN-DC LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16 - R4-1813779: TP for TR 37.716-21-21 DC_LTE 1band_NR 2bands - R4-1813780: TP for TR 37.716-21-21 DC_LTE 2band_NR 2bands - R4-1813781: TP for TR 37.716-21-21 DC_LTE 3band_NR 2bands - R4-1813782: TP for TR 37.716-21-21 DC_LTE 4band_NR 2bands R4-1812240: Revised WID for LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16 |  |  |
| 2018-11 | RAN4 #89 | R4-1814769 |  |  | Updated TR37.716-21-21 for EN-DC band combinations of LTE xDL/1UL (x=1,2,3,4) + inter-band NR 2DL/1UL | 0.1.0 | 0.2.0 |
|  |  |  |  |  | Approved TPs and revised WID in RAN4 #89 - R4-1814940: TP to TR 37.716-21-21: finalize UE requirements for DC_3_n1-n77 - R4-1816175: TP for TR 37.716-21-21 for DC_1A_n3A-n78A - R4-1816176: TP for TR 37.716-21-21 for DC_3A_n1A-n78A - RP-182302: Revised WID for LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16 |  |  |
| 2019-02 | RAN4 #90 | R4-1900246 |  |  | Updated TR37.716-21-21 v0.3.0 for EN-DC band combinations of LTE xDL/1UL (x=1,2,3,4) + inter-band NR 2DL/1UL | 0.2.0 | 0.3.0 |
|  |  |  |  |  | Approved TPs and revised WID in RAN4 #90 - R4-1900247, "Revised WID on LTE (xDL/UL x=1.2,3,4) with NR 2 bands (2DL/1UL) EN DC in rel-16," - R4-1900249, "TP on summary of self-interference analysis for new EN-DC LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16," - R4-1900781, "TP for 37.716-21-21 for DC_3A_n7A-n78A," - R4-1900782, "TP for 37.716-21-21 for DC_3C_n7A-n78A," - R4-1901436, "TP for TR 37.716-21-21 for DC_13A_n5A-n260A," - R4-1901438, "TP for TR 37.716-21-21 for DC_66A_n5A-n260A," - R4-1901439, "TP for TR 37.716-21-21 for DC_3A_n20A-n78A," - R4-1901455, "TP for TR 37.716-21-21 for DC_28A_n8A-n78A," - R4-1901456, "TP for TR 37.716-21-21 for DC_28A_n8A-n258A," - R4-1902112, "TP for 37.716-21-21 for DC_3C_n28A-n78A," - R4-1902113, "TP for 37.716-21-21 for DC_7C_n28A-n78A," - R4-1902115, "TP for TR 37.716-21-21 to include DC_66A_n71A-n260A, DC_66A_n71A-n260(2A)," - R4-1902120, "TP to TR 37.716-21-21: DC_66A_n71A-n261A," - R4-1902121, "TP to TR 37.716-21-21 DC_66A_n71A-n261(2A)," - R4-1902466, "TP for 37.716-21-21 for DC_1A_n7A-n78A," |  |  |
| 2019-04 | RAN4 #90BIS | R4-1903007 |  |  | Updated TR37.716-21-21 v0.4.0 for EN-DC band combinations of LTE xDL/1UL (x=1,2,3,4) + inter-band NR 2DL/1UL | 0.3.0 | 0.4.0 |
|  |  |  |  |  | Approved TPs and revised WID in RAN4 #90BIS - R4-1904438, "TP for TR 37.716-21-21 to include DC_66_n25-n41," Ericsson, T-Mobile US - R4-1903993, "TP for TR 37.716-21-21: DC_3A_n1A-n257A," KDDI - R4-1903992, "TP for TR 37.716-21-21: DC_3A_n1A-n79A," KDDI - R4-1904885, "TP for TR 37.716-21-21 for DC_1-3-7_n28-n78," Huawei, HiSilicon - R4-1904884, "TP for TR 37.716-21-21 for DC_3-7_n28-n78," Huawei, HiSilicon - R4-1903182, "TP for TR 37.716-21-21 for DC_1-7_n28-n78," Huawei, HiSilicon - R4-1903181, "TP for TR 37.716-21-21 for DC_1-3_n28-n78," Huawei, HiSilicon - R4-1904883, "TP for TR 37.716-21-21 for DC_1-3_n7-n78," Huawei, HiSilicon - R4-1904900, "TP for TR 37.716-21-21 DC_3A_n40A-n78A," Nokia, Nokia Shanghai Bell - R4-1904899, "TP for TR 37.716-21-21 DC_1A_n40A-n78A," Nokia, Nokia Shanghai Bell - R4-1903011, "MSD results for new LTE xbands DL_1band UL (x=1,2,3,4) + NR 2bands DL_1band UL DC with self-interference problems," LG Electronics France - R4-1903010, "TP on summary of self-interference analysis for new EN-DC LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16," LG Electronics France - R4-1903008, "Revised WID on LTE (xDL/UL x=1.2,3,4) with NR 2 bands (2DL/1UL) EN DC in rel-16," LG Electronics France - R4-1902993, "Draft CR for TS 38.101-3: Support of DL_n257G-I in DC_1_n77-n257," SoftBank Corp.- R4-1902973, "TP for TR 37.716-21-21: EN-DC_8_n77-n257," SoftBank Corp. |  |  |
| 2019-05 | RAN4 #91 | R4-1906054 |  |  | Updated TR 37.716-21-21 v0.5.0: LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16," | 0.4.0 | 0.5.0 |
|  |  |  |  |  | Approved TPs and revised WID - R4-1905513, "TP for TR 37.716-21-21 for DC_1_n77-n258," Huawei, HiSilicon - R4-1905514, "TP for TR 37.716-21-21 for DC_1_n78-n258," Huawei, HiSilicon - R4-1905515, "TP for TR 37.716-21-21 for DC_1_n79-n258," Huawei, HiSilicon - R4-1905516, "TP for TR 37.716-21-21 for DC_3_n77-n258," Huawei, HiSilicon - R4-1905517, "TP for TR 37.716-21-21 for DC_3_n78-n258," Huawei, HiSilicon - R4-1905518, "TP for TR 37.716-21-21 for DC_3_n79-n258," Huawei, HiSilicon - R4-1905787, "TP for TR 37.716-21-21: EN-DC_1-8_n77-n257," SoftBank Corp. - R4-1906055, "Revised WID on LTE (xDL/UL x=1.2,3,4) with NR 2 bands (2DL/1UL) EN DC in rel-16," LG Electronics France - R4-1906057, "TP on summary of self-interference analysis for new EN-DC LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16," LG Electronics France - R4-1906191, "TP for DC_1-19-42_n77-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906192, "TP for DC_1-19-42_n78-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906193, "TP for DC_1-19_n77-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906194, "TP for DC_1-19_n78-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906195, "TP for DC_1-21-42_n77-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906196, "TP for DC_1-21-42_n78-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906197, "TP for DC_1-21_n77-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906198, "TP for DC_1-21_n78-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906199, "TP for DC_1-3-21_n77-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906200, "TP for DC_1-3-21_n78-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906202, "TP for DC_1-3_n77-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906203, "TP for DC_1-3_n78-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906204, "TP for DC_1-42_n77-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906205, "TP for DC_1-42_n78-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906206, "TP for DC_19-21-42_n77-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906207, "TP for DC_19-21-42_n78-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906208, "TP for DC_19-21_n77-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906209, "TP for DC_19-21_n78-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906210, "TP for DC_19-42_n77-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906211, "TP for DC_19-42_n78-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906212, "TP for DC_21-42_n77-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906213, "TP for DC_21-42_n78-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906214, "TP for DC_3-19_n77-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906215, "TP for DC_3-19_n78-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906216, "TP for DC_3-21_n77-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906217, "TP for DC_3-21_n78-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906218, "TP for DC_3-42_n77-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906219, "TP for DC_3-42_n78-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906329, "TP for TR 37.716-21-21: UE requirements for DC_3-3_n78-n257, DC_3-3-7_n78-n257, DC_3-3-7-7_n78-n257," CHTTL - R4-1906750, “TP for TR 37.716-21-11 to include DC_20_n3-n78," Ericsson, Swisscom - R4-1906751, “TP for TR 37.716-21-11 to include DC_7_n1-n78," Ericsson, CHTTL, Swisscom - R4-1906752, “TP for TR 37.716-21-11 to include DC_20_n1-n78," Ericsson, Swisscom - R4-1906753, "TP for TR 37.716-21-11 to include DC_3-7_n1-n78," Ericsson, CHTTL, Swisscom - R4-1907393, "TP for TR 37.716-21-11 to include DC_7_n3-n78," Ericsson, Swisscom |  |  |
| 2019-08 | RAN4 #92 | R4-1908693 |  |  | Updated TR 37.716-21-21 v0.6.0: LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16," | 0.5.0 | 0.6.0 |
|  |  |  |  |  | Approved TPs and revised WID - R4-1908278, "TP for TR 37.716-21-21: DC_1A-3A_n77-n257," KDDI - R4-1908289, "TP to TR 37.716-21-21: DC_2_n41-n261," Nokia - R4-1908291, "TP to TR 37.716-21-21: DC_66_n41-n261," Nokia - R4-1908566, "TP for TR37.716-21-21 for DC_40_n41-n79," ZTE Corporation, - R4-1908581, "TP for TR 37.716-21-21: UE requirements for DC_3-3_n1-n78, DC_7-7_n1-n78," CHTTL - R4-1908582, "TP for TR 37.716-21-21: UE requirements for DC_3-3-7_n1-n78, DC_3-7-7_n1-n78, DC_3-3-7-7_n1-n78," CHTTL - R4-1908693, "TR 37.716-21-21 v0.6.0 update: LTE(xDL/1UL)+ NR(2DL/1UL) DC in Rel-16," LG Electronics France - R4-1909802, "TP for TR 37.716-21-21 to conclude DC_66A_n25A-n41A," Ericsson, T-Mobile US - R4-1909804, "TP for TR 37.716-21-21 to include DC_2_n41-n260," Ericsson, T-Mobile US - R4-1909805, "TP for TR 37.716-21-21 to include DC_66_n41-n260," Ericsson, T-Mobile US - R4-1909826, "TP for TR 37.716-21-21 to include DC_1_n5-n78," Ericsson, Telstra - R4-1909828, "TP for TR 37.716-21-21 to include DC_7_n5-n78," Ericsson, Telstra - R4-1909827, "TP for TR 37.716-21-21 to include DC_3_n5-n78," Ericsson, Telstra - R4-1909829, "TP for TR 37.716-21-21 to include DC_28_n5-n78," Ericsson, Telstra - R4-1909830, "TP for TR 37.716-21-21 to include DC_1-3_n5-n78," Ericsson, Telstra - R4-1909831, "TP for TR 37.716-21-21 to include DC_1-7_n5-n78," Ericsson, Telstra - R4-1909832, "TP for TR 37.716-21-21 to include DC_1-28_n5-n78," Ericsson, Telstra - R4-1909833, "TP for TR 37.716-21-21 to include DC_3-7_n5-n78," Ericsson, Telstra - R4-1909834, "TP for TR 37.716-21-21 to include DC_3-28_n5-n78," Ericsson, Telstra - R4-1909835, "TP for TR 37.716-21-21 to include DC_7-28_n5-n78," Ericsson, Telstra - R4-1909836, "TP for TR 37.716-21-21 to include DC_1-3-7_n5-n78," Ericsson, Telstra - R4-1910191, "TP for TR 37.716-21-21 to include DC_3-7-28_n5-n78," Ericsson, Telstra - R4-1910192, "TP for TR 37.716-21-21 to include DC_1-3-7-28_n5-n78," Ericsson, Telstra - R4-1910193, "TP for TR 37.716-21-21 to include DC_1-7-28_n5-n78," Ericsson, Telstra - R4-1910307, "Revised WID on LTE (xDL/UL x=1.2,3,4) with NR 2 bands (2DL/1UL) EN DC in Rel-16," LG Electronics France - R4-1910363, "CR on introduction of completed EN-DC of x bands LTE and 2 band NR from RAN4#92 into TS 38.101-3," LG Electronics France |  |  |
| 2019-10 | RAN4 #92BIS | R4-1911443 |  |  | Updated TR 37.716-21-21 v0.7.0: LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16," | 0.6.0 | 0.7.0 |
|  |  |  |  |  | Approved TPs and revised WID at #92BIS - R4-1911153, "TP for TR 37.716-21-21: EN-DC_3_n77-n257," SoftBank Corp. - R4-1911155, "TP for TR 37.716-21-21: EN-DC_8_n3-n28," SoftBank Corp. - R4-1911157, "TP for TR 37.716-21-21: EN-DC_11_n77-n257," SoftBank Corp. - R4-1911246, "TP for TR37.716-21-21_ DC_3A_n40A-n41A," ZTE Corporation - R4-1911266, "TP for TR 37.716-21-21: adding support of DC_3-3_n78_n257, DC_3-3-7_n78_n257, DC_3-3-7-7_n78_n257 with n257D to n257M bandwidth classes," CHTTL - R4-1911271, "TP for TR 37.716-21-21: UE requirements for DC_8_n1-n78," CHTTL - R4-1911443, "TR 37.716-21-21 v0.7.0 update: LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16," LG Electronics France - R4-1911447, "TP on summary of self-interference analysis for new EN-DC LTE(xDL/1UL)+ NR(2DL/1UL) DC in Rel-16," LG Electronics France - R4-1911448, "MSD results for new EN-DC LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16," LG Electronics France - R4-1911444, "vised WID on LTE (xDL/UL x=1.2,3,4) with NR 2 bands (2DL/1UL) EN DC in Rel-16," LG Electronics France - R4-1911535, "TP to TR 37.716-21-21: Addition of DL_7C_n1A-n78A," Nokia, BT - R4-1911606, "TP for 37.716-21-21 to introduce DC_2A-66A_n41A-n261," Nokia, Nokia Shanghai Bell, T-Mobile USA - R4-1912272, "TP for TR 37.716-21-21 to include DC_66_n41-n71," Ericsson, T-Mobile - R4-1912273, "TP for TR 37.716-21-21 to include DC_2-66_n41-n71," Ericsson, T-Mobile - R4-1912274, "TP for TR 37.716-21-21 to include DC_2-66_n41-n260," Ericsson, T-Mobile - R4-1912568, "updated TP for TR 37.716-21-21: DC_1-3_n7-n78 to add configurations," Huawei, HiSilicon - R4-1912569, "updated TP for TR 37.716-21-21: DC_3-7_n1-n78 to add configurations," Huawei, HiSilicon - R4-1912574, "TP to TR 37.716-21-21: Addition of DL_3A-7C_n1A-n78A, DL_3C-7A_n1A-n78A and DL_3C-7C_n1A-n78A," Nokia, BT - R4-1912576, "TP to TR 37.716-21-21: Addition of DL_3C_n1A-n78A," Nokia, BT - R4-1912594, "TP for TR 37.716-21-21: DC_8A_n78A-n257," Samsung, KT - R4-1912595, "TP for TR 37.716-21-21: DC_3C_n78A-n257," Samsung, KT - R4-1912596, "TP for TR 37.716-21-21: DC_3A-8A_n78A-n257," Samsung, KT - R4-1912597, "TP for TR 37.716-21-21: DC_1A-8A_n78A-n257A," Samsung, KT - R4-1912598, "TP for TR 37.716-21-21: DC_1A-3-8A_n78A-n257A," Samsung, KT - R4-1912599, "TP to TR 37.716-21-21: DC_2A-7A-7A_n38A-n78A," Intel Corporation - R4-1912600, "TP for TR 37.716-21-21 to include DC_2_n41-n71," Ericsson, T-Mobile - R4-1912603, "TP for TR 37.716-21-21: EN-DC_1_n3-n28," SoftBank Corp. - R4-1912613, "TP to TR 37.716-21-21: DC_7A-7A-66A_n38A-n78A," Intel Corporation |  |  |
| 2019-11 | RAN4 #3 | R4-191xxxx |  |  | Updated TR 37.716-21-21 v0.8.0: LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16," | 0.7.0 | 0.8.0 |