3GPP TSG-RAN WG4 Meeting #93	R4-1913982

Reno, US 18th – 22nd Nov 2019

Source: 	Nokia, Nokia Shanghai Bell

Title: 					FR2 MPR enhancement CIM-3 Study

Agenda Item: 			8.14.1

Document for:	Approval

# Introduction

In previous RAN4 meeting a WF [1] was approved for FR2 0 dB MPR region extension.

This WF stated:

- MPR improvement assumption: Range 1 is extended as in [4] except for the allocations mentioned in slide 4. More simulation or measurement results on range 1 extension are encouraged to provide for RAN4#93.

- Companies will provide MPR simulation results/measurements for allocations mentioned in slide 4 for RAN4#93 to address potential CIM-3 issue.

Where the slide 4 is as follows:

C-IM3 fall into the spurious emission range and may exceed the -13dBm/MHz for, the MPR value for CLRB<=4(120kHz SCS) shall evaluated further.

Affected allocations are depicted in Figure with brown color.

Figure 1: CIM3-region under study (brown area)

# Discussion

We have earlier presented MPR simulations [2] that showed that FR2 0 dB MPR region extension is feasible as proposed in [3]. In these studies, we used CIM3 = 60 dBc but had done studies with CIM3=40 dBc which showed no impact to results.

Now to stress the CIM-3 behaviour even more we present simulation results for CIM3= 25 dBc and 30 dBc.

Figure 2: CIM3 study results CIM3=25 dBc (left) 30 dBc (right) PI/2-PBSK above, QPSK below

We did not just simulate the specific region under study instead all allocations were simulated. Figure 2 presents the delta between required backoff and allowed MPR (extended MPR proposal) and the green colour indicates that the MPR is sufficient. As can be seen from Figure 2 CIM3 needs to be as low as 25 dBc to create issues with – 13 dBm/MHz limit.

Thus we can confirm the proposal from [2] [4] [5].

Proposal: REGION 1 MPR for FR2 is extended as described in [3] for channel bandwidth less or equal to 200MHz

# 3 	Conclusion

I this contribution we have shown that CIM3 is not an issue for extension of 0 dB FR2 MPR region and made following proposal.

Proposal: REGION 1 MPR for FR2 is extended as described in [3] for channel bandwidth less or equal to 200MHz

# 4	References

[1] R4-1912976, WF on Region 1 extension, RAN4#92bis, Nokia

[2] R4-1911523, Region 1 Extension for Power Class 3 Operation in FR2 and output power boosting, RAN4#92bis, Nokia

[3] R4-1910280, WF on Region 1 Extension for Power Class 3 Operation in FR2, Skyworks, Nokia, Qualcomm, RAN4#92

[4] R4-1912464, Input to WF on Region 1 Extension for Power Class 3 Operation in FR2 and Output Power Boosting, RAN4#92bis, Skyworks Solutions, Inc.

[5] R4-1911031, FR2 PC3 MPR enhancement for Rel-16, RAN4#92bis, Intel Corporation