3GPP TSG-RAN WG4 Meeting #93	R4-1914092

Reno, US, 18th– 22nd November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.141 | CR | 1242 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 36.141 Introduction of PRACH performance requirements for enhanced HST senario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_high_speed_enh2-Perf |  |  |  |  |  | Date: |  |  | 2019-10-4 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR is to introduce new requirements for PRACH restricted set type B under high speed train conditions assuming a UE velocity of up to 500km/h. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR is based on a draft CR (R4-1912446) endorsed in RAN4 #92bis, and additional changes in RAN4 #93 are highlighted in yellow. Add new test cases for PRACH restricted set type B performance with 1944 Hz frequency offset |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PRACH performance under high speed train conditions assuming a UE velocity of up to 500km/h is not guaranteed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS36.104 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------Start of text proposal-------------

8.4.1.5	Test Requirement

Pfa shall not exceed 0.1%. Pd shall not be below 99% for the SNRs in Tables 8.4.1.5-1 to 8.4.1.5-5.

Table 8.4.1.5-1: PRACH missed detection test requirements for Normal Mode

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR [dB] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 | Burst format 1 | Burst format 2 | Burst format 3 | Burst format 4 |
| 1 | 2 | AWGN | 0 | -13.9 | -13.9 | -16.1 | -16.2 | -6.9 |
|  |  | ETU 70 Low* | 270 Hz | -7.4 | -7.2 | -9.4 | -9.5 | 0.5 |
|  | 4 | AWGN | 0 | -16.6 | -16.4 | -18.7 | -18.5 | -9.5 |
|  |  | ETU 70 Low* | 270 Hz | -11.5 | -11.1 | -13.5 | -13.3 | -4.5 |
|  | 8 | AWGN | 0 | -19.5 | -19.1 | -21.2 | -21 | -11.5 |
|  |  | ETU 70 Low* | 270 Hz | -15.7 | -15.3 | -17.2 | -16.9 | -8.0 |
| Note*: Not applicable for Local Area BS and Home BS. |  |  |  |  |  |  |  |  |

The requirements in Table 8.4.1.5-2 shall not be applied to Local Area BS and Home BS

Table 8.4.1.5-2: PRACH missed detection test requirements for High speed Mode restricted set type A

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 | Burst format 1 | Burst format 2 | Burst format 3 |
| 1 | 2 | AWGN | 0 | -13.8 | -13.9 | -16.0 | -16.3 |
|  |  | ETU 70 Low | 270 Hz | -6.8 | -6.7 | -8.7 | -8.9 |
|  |  | AWGN | 625 Hz | -12.1 | -12.0 | -14.1 | -14.1 |
|  |  | AWGN | 1340 Hz | -13.1 | -13.2 | -15.2 | -15.4 |
|  | 4 | AWGN | 0 | -16.6 | -16.3 | -18.6 | -18.5 |
|  |  | ETU 70 Low | 270 Hz | -11.2 | -10.8 | -13.1 | -13.1 |
|  |  | AWGN | 625 Hz | -14.6 | -14.3 | -16.5 | -16.5 |
|  |  | AWGN | 1340 Hz | -15.6 | -15.2 | -17.5 | -17.5 |
|  | 8 | AWGN | 0 | -19.0 | -18.8 | -20.6 | -20.7 |
|  |  | ETU 70 Low | 270 Hz | -15.0 | -14.5 | -16.4 | -16.4 |
|  |  | AWGN | 625 Hz | -17.4 | -17.1 | -19.0 | -19.1 |
|  |  | AWGN | 1340 Hz | -18.4 | -18.1 | -20.2 | -20.2 |

Table 8.4.1.5-3: PRACH missed detection requirements for coverage enhancement (PRACH frequency hopping OFF)

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Frequency offset | Number of Repetitions | SNR [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Burst format 0 | Burst format 1 | Burst format 2 | Burst format 3 |
| 1 | 2 | AWGN | 0 | 4 | - | - | -21.0 | -20.8 |
|  |  |  |  | 8 | -21.4 | -21.0 | - | - |
|  |  |  |  | 16 | - | - | -24.8 | -24.7 |
|  |  |  |  | 32 | -25.3 | -25.0 | - | - |
|  |  | EPA1 Low | 270 Hz | 4 | - | - | -11.5 | -11.1 |
|  |  |  |  | 8 | -12.4 | -11.7 | - | - |
|  |  |  |  | 16 | - | - | -16.6 | -16.6 |
|  |  |  |  | 32 | -18.4 | -18.0 | - | - |
| Note 1: Under fading channels, the PRACH detection performance may be significantly different with different PRACH Configuration Indexes. The requirements in this table are defined based on the simulation results with PRACH Configuration Indexes (3, 19, 35, 51) for Format 0, Format 1, Format 2, and Format 3 respectively. |  |  |  |  |  |  |  |  |

Table 8.4.1.5-4: PRACH missed detection requirements for coverage enhancement (PRACH frequency hopping ON)

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Frequency offset | Number of Repetitions | SNR [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Burst format 0 | Burst format 1 | Burst format 2 | Burst format 3 |
| 1 | 2 | EPA1 Low | 270 Hz | 4 | - | - | -14.9 | -14.7 |
|  |  |  |  | 8 | -15.6 | -15.2 | - | - |
|  |  |  |  | 16 | - | - | -19.5 | -19.6 |
|  |  |  |  | 32 | -20.7 | -20.5 | - | - |
| Note 1: Under fading channels, the PRACH detection performance may be significantly different with different PRACH Configuration Indexes. The requirements in this table are defined based on the simulation results with PRACH Configuration Indexes (3, 19, 35, 51) for Format 0, Format 1, Format 2, and Format 3 respectively. Note 2: The requirements in this table are defined under the assumption that UE RF tuning during PRACH frequency hopping has no impact on the symbols in PRACH subframes and thus all symbols in PRACH subframes are available for the transmission of PRACH preambles. Note 3: The requirements in this table are defined under the assumption that the PRACH frequency offset (prach-FreqOffset-r13) is 0 and frequency hopping offset is -6, where is defined in TS36.211 [12]. Note 4: The requirements in this table apply for channel bandwidth of 5MHz, 10MHz, 15MHz or 20MHz. For channel bandwidth of 3MHz, the requirements in Table 8.4.1.5-3 apply. |  |  |  |  |  |  |  |  |

Table 8.4.2.1-5 PRACH missed detection requirements for High speed Mode restricted set type B

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 | Burst format 1 | Burst format 2 | Burst format 3 |
| 1 | 2 | AWGN | 0 | -14.2 | -13.8 | -16.4 | -16.5 |
|  |  | AWGN | 625 Hz | -11.7 | -11.4 | -13.6 | -13.6 |
|  |  | ETU 70 Low | 270 Hz | -6.7 | -6.3 | -8.5 | -8.6 |
|  |  | AWGN | 1875 Hz | -11.5 | -11.1 | -13.5 | -13.7 |
|  |  | AWGN | 1944 Hz | TBD | TBD | TBD | TBD |
|  | 4 | AWGN | 0 | -16.8 | -16.3 | -18.8 | -18.8 |
|  |  | AWGN | 625 Hz | -14.1 | -13.8 | -15.8 | -15.9 |
|  |  | ETU 70 Low | 270 Hz | -11.2 | -10.7 | -12.9 | -12.8 |
|  |  | AWGN | 1875 Hz | -13.9 | -13.5 | -15.6 | -16.0 |
|  |  | AWGN | 1944 Hz | TBD | TBD | TBD | TBD |
|  | 8 | AWGN | 0 | -19.3 | -18.8 | -20.9 | -20.9 |
|  |  | AWGN | 625 Hz | -16.1 | -16.0 | -17.8 | -17.9 |
|  |  | ETU 70 Low | 270 Hz | -14.7 | -14.5 | -16.5 | -16.9 |
|  |  | AWGN | 1875 Hz | -16.0 | -15.7 | -17.7 | -18.1 |
|  |  | AWGN | 1944 Hz | TBD | TBD | TBD | TBD |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in Annex G.

--------------End of text proposal-------------