3GPP TSG-RAN WG4 Meeting #93	R4-1914116

Reno, USA, 18th – 22th Nov., 2019

Agenda item:		9.7.2

Source:	MediaTek Inc.

Title:	Discussion on UE dynamic adaptation to the maximum number of MIMO layers

Document for:		Discussion

# 1	Introduction

In RAN4 #92 meeting, companies agreed a WF [1] and separated the discussion on switching delay and interruption times for UE maximal MIMO layers adaptation into 2 cases. For case 1, current BWP switch delay and interruption requirements for Type 1 and Type 2 UE are agreed to reuse. For case 2, in RAN4 #92Bis meeting, companies agreed to evaluate the performance of Type1 and Type2 switching delay based on the traffic model and RF architecture:

- Agreement: - Based on the traffic model and RF architecture - No new switching delay requirements will be introduced for MIMO layer adaption except for type 1 and type 2 switching delay - Power saving gain shall be considered

In TR38.840 [2] Section 8.2, it is specified that the following performance matrices are taken into accounts:

Performance metrics - UE power saving gain - percentage of power consumption reduction of the proposed power saving scheme from the baseline scheme - FFS: For the case multiple applications are evaluated, whether power consumption is the overall DoU power across the applications - System performance - Latency - Scheduling delay - User throughput - System throughput and/or resource utilization/overhead (if applicable) should be reported as the result of the evaluation, in addition to power saving gain.

In this paper, based on the agreed traffic model FTP3 in [2], we provide our evaluation results on the power saving gain and latency with different SCSs and DRX cycles in Section 2. Besides, the evaluation results based on RF architecture are shown in Section 3. A summary is provided in Section 4. The simulation assumptions are listed in Appendix.

# 2	Evaluation results based on traffic model

In this section, we compare the power saving gain and latency between Type 1 and Type 2 UE, using the power consumption and latency of no MIMO layer adaptation UE as the baseline. Here, the latency is defined as the time for gNB’s higher layer to collect all ACK feedback of one packets from UE. For each cases, the resource utilization (RU) rates are also shown to indicate the average number of scheduled PRB over the number of total PRB in each slot. In our evaluations, the packet size and arrival rate are varied with the DRX cycle length. Besides, the fixed data rate 20Mbps is assumed. The notations and units of DRX configurations and packet size are shown as follows:

- DRX (cycle length, on-duration length, inactivity timer length) [ms]

- Pkt [MBytes]

In FR1, assuming that only 1 CC is in use, the evaluation results of 15kHz and 30kHz SCSs are shown in Table 1 and Table 2, respectively.

Table 1: Power saving gain and latency for Type 1 and Type 2 UEs in SCS 15kHz

|  |  | RU(%) | RU Increment (w.r.t baseline) | Latency (ms) | Latency Increment (w.r.t baseline) | Power Consumption (units) | PS gain (w.r.t baseline) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DRX(160, 5, 80) Pkt =0.5 | Baseline | 27.78 |  | 67.76 |  | 44.92 |  |
|  | Type 1 | 27.82 | 0.14% | 67.99 | 0.34% | 36.53 | 18.68% |
|  | Type 2 | 27.49 | -1.04% | 70.64 | 4.25% | 36.93 | 17.79% |
| DRX(80, 5, 40) Pkt =0.25 | Baseline | 27.72 |  | 33.12 |  | 49.68 |  |
|  | Type 1 | 27.91 | 0.69% | 33.81 | 2.08% | 41.60 | 16.26% |
|  | Type 2 | 27.74 | 0.07% | 35.60 | 7.49% | 42.47 | 14.51% |
| DRX(40, 5, 20) Pkt =0.125 | Baseline | 27.57 |  | 15.84 |  | 58.58 |  |
|  | Type 1 | 27.64 | 0.25% | 16.23 | 2.46% | 50.46 | 13.86% |
|  | Type 2 | 27.66 | 0.33% | 17.79 | 12.31% | 51.84 | 11.51% |
| DRX(20, 5, 10) Pkt =0.0625 | Baseline | 27.08 |  | 8.30 |  | 73.11 |  |
|  | Type 1 | 27.06 | -0.07% | 8.55 | 3.01% | 64.83 | 11.33% |
|  | Type 2 | 27.22 | 0.52% | 10.11 | 21.81% | 66.04 | 9.67% |

Table 2: Power saving gain and latency for Type 1 and Type 2 UEs in SCS 30kHz

|  |  | RU(%) | RU Increment (w.r.t baseline) | Latency (ms) | Latency Increment (w.r.t baseline) | Power Consumption (units) | PS gain (w.r.t baseline) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DRX(160, 5, 80) Pkt =0.5 | Baseline | 29.47 |  | 63.59 |  | 45.29 |  |
|  | Type 1 | 29.42 | -0.17% | 63.71 | 0.19% | 37.21 | 17.84% |
|  | Type 2 | 29.34 | -0.44% | 64.92 | 2.09% | 37.57 | 17.05% |
| DRX(80, 5, 40) Pkt =0.25 | Baseline | 30.14 |  | 30.28 |  | 50.11 |  |
|  | Type 1 | 30.19 | 0.17% | 31.08 | 2.64% | 42.03 | 16.12% |
|  | Type 2 | 30.17 | 0.10% | 32.43 | 7.10% | 42.73 | 14.73% |
| DRX(40, 5, 20) Pkt =0.125 | Baseline | 30.40 |  | 14.48 |  | 58.50 |  |
|  | Type 1 | 30.41 | 0.03% | 15.25 | 5.32% | 50.40 | 13.85% |
|  | Type 2 | 30.50 | 0.33% | 16.57 | 14.43% | 51.63 | 11.74% |
| DRX(20, 5, 10) Pkt =0.0625 | Baseline | 30.24 |  | 6.60 |  | 71.31 |  |
|  | Type 1 | 30.24 | 0% | 7.07 | 7.12% | 63.00 | 11.65% |
|  | Type 2 | 30.25 | 0.03% | 8.24 | 24.85% | 64.71 | 9.21% |

In FR2, assuming that 4 CCs are in use, the evaluation results of 120kHz are shown in Table 3.

Table 3: Power saving gain and latency for Type 1 and Type 2 UEs in SCS 120kHz

|  |  | RU(%) | RU Increment (w.r.t baseline) | Latency (ms) | Latency Increment (w.r.t baseline) | Power Consumption (units) | PS gain (w.r.t baseline) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DRX(160, 5, 40) Pkt =0.5 | Baseline | 8.34 |  | 64.36 |  | 120.56 |  |
|  | Type 1 | 8.35 | 0.12% | 64.97 | 0.95% | 91.52 | 24.09% |
|  | Type 2 | 8.34 | 0% | 65.70 | 2.08% | 94.62 | 21.52% |
| DRX(80, 5, 20) Pkt =0.25 | Baseline | 8.45 |  | 31.19 |  | 131.55 |  |
|  | Type 1 | 8.42 | -0.36% | 31.54 | 1.12% | 102.43 | 22.14% |
|  | Type 2 | 8.44 | -0.12% | 32.47 | 4.10% | 108.60 | 17.45% |
| DRX(40, 5, 10) Pkt =0.125 | Baseline | 8.30 |  | 14.51 |  | 152.77 |  |
|  | Type 1 | 8.31 | 0.12% | 14.96 | 3.10% | 123.61 | 19.09% |
|  | Type 2 | 8.26 | -0.48% | 15.92 | 9.72% | 135.58 | 11.25% |
| DRX(20, 5, 5) Pkt =0.0625 | Baseline | 8.42 |  | 6.46 |  | 192.82 |  |
|  | Type 1 | 8.34 | -0.95% | 6.85 | 6.04% | 163.55 | 15.18% |
|  | Type 2 | 8.34 | -0.95% | 7.83 | 21.21% | 187.48 | 2.77% |

According to above evaluation results, we can see that the power saving gain difference for UEs adopting Type 1 and Type 2 switching delay are approximately ranged from 1% to 2% in FR1 and from 2% to 12.41% in FR2. However, the latency difference for UE adopting Type 1 and Type 2 switching delay can be up to 26.3% in FR1 and 48% in FR2.

Observation 1: The power saving gain difference for UE adopting Type 1 and Type 2 switching delay are up to 2% in FR1 and up to 12.41% in FR2.

Observation 2: The latency difference for UE adopting Type 1 and Type 2 switching delay can be up to 26.3% in FR1 and 48% in FR2.

As we know, the long latency may impact the performance of some real time services, e.g., gaming or live video services. So we suggest that

Proposal 1: In Rel-16, Type 1 switching delay should be supported.

# 3	Evaluation results based on RF architecture

For case 2, our understanding is that if only the number of maximum MIMO layer is changed, the switching delay and interruption time could be shorter than those of the current type 2 BWP switch. In Figure 1, we illustrate the time components to be considered, and in Table 2, we assume 15kHz SCS to summarize our suggested requirements for each of the components.

Figure 1: Reference time components for a physical layer setting change

Table 4: Requirements for the time components in adapting the maximum number of MIMO layers

| Time component | Suggested requirement | Comment |
| --- | --- | --- |
| Control information monitoring (from PDCCH symbol reception to DCI parsing) | min(500 us, slot duration) | For 15k SCS, total reception and processing time can be finished within 6 to 7 symbols [3]. For higher SCS, UE should at least finish control information monitoring within a slot duration. |
| RF/BB parameter calculating and loading | 210 us | There is no need to re-calculate the parameters. We need to consider the required time for parameter selection and loading, where the margins for reading/writing parameters from/into the temporary memory are included. |
| Applying the new parameters | 250 us | If there is no carrier frequency, bandwidth or SCS change involved in adapting the maximum number of MIMO layers, the required time can be much less than the proposed value in [3]. |

Similar to above analysis, the corresponding delays with other SCSs are evaluated and the results are shown in Table 5.

Table 5: Requirements for the time components in adapting the maximum number of MIMO layers

| μ (SCS = 15 kHz⋅2μ) | NR Slot length (ms) | MIMO switch delay TMIMOswitchDelay (slots)* |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 2 |
| 2 | 0.25 | 4 |
| 3 | 0.125 | 6 |

The evaluation results are actually very close to the existing type 1 BWP switching delay.

Observation 3: For case 2, current BWP switch delay for Type 1 UE in 38.133 is achievable: For DCI-triggered switch, when UE receives the request at slot n, UE shall be ready to receive PDSCH with new settings at slot n+ TMIMOswitchDelay

|  | NR Slot length (ms) | MIMO switch delay TMIMOswitchDelay (slots) |
| --- | --- | --- |
|  |  | Type 1Note 1 |
| 0 | 1 | 1 |
| 1 | 0.5 | 2 |
| 2 | 0.25 | 3 |
| 3 | 0.125 | 6 |
| Note 1: Depends on UE capability. Note 2: Rel-15 per-CC maximum number of MIMO layers is not changed by the dynamic adaptation |  |  |

Interruption time is applied to the active serving cells other than the switching cell. Whether there is impact to the other serving cells depends on UE implementation in RX/TX antenna switching as well as UE intelligence in deciding RX/TX antenna switching.

It is a common understanding that when UE needs to switch its RX/TX antenna, synthesizer will introduce an absolute phase jump and cause interruption to the other active serving cells. This interruption time might be very small and much less than 1 slot. Considering that the starting point of this interruption time depends on the UE implementation, it is very hard to predict which symbols of other serving cells will be interrupted by the antenna switching. Besides, in current RAN4 spec, the smallest unit to address the interruption time is 1 slot. So we suggest to use 1 slot interruption time for MIMO layers adaptation in all SCSs.

Observation 4: When UE needs to switch the RX/TX antenna, synthesizer will introduce an absolute phase jump and cause interruption to the other active serving cells.

Observation 5: For case 2, 1 slot interruption time is sufficient for all SCSs

|  | NR Slot length (ms) | Interruption length X (slots) |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 1 |
| 2 | 0.25 | 1 |
| 3 | 0.125 | 1 |

# 4	Summary

In this contribution, we have the following observations:

Observation 1: The power saving gain difference for UE adopting Type 1 and Type 2 switching delay are up to 2% in FR1 and up to 12.41% in FR2.

Observation 2: The latency difference for UE adopting Type 1 and Type 2 switching delay can be up to 26.3% in FR1 and 48% in FR2.

Observation 3: For case 2, current BWP switch delay for Type 1 UE in 38.133 is achievable: For DCI-triggered switch, when UE receives the request at slot n, UE shall be ready to receive PDSCH with new settings at slot n+ TMIMOswitchDelay

|  | NR Slot length (ms) | MIMO switch delay TMIMOswitchDelay (slots) |
| --- | --- | --- |
|  |  | Type 1Note 1 |
| 0 | 1 | 1 |
| 1 | 0.5 | 2 |
| 2 | 0.25 | 3 |
| 3 | 0.125 | 6 |
| Note 1: Depends on UE capability. Note 2: Rel-15 per-CC maximum number of MIMO layers is not changed by the dynamic adaptation |  |  |

Observation 4: When UE needs to switch the RX/TX antenna, synthesizer will introduce an absolute phase jump and cause interruption to the other active serving cells.

Observation 5: For case 2, 1 slot interruption time is sufficient for all SCSs

|  | NR Slot length (ms) | Interruption length X (slots) |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 1 |
| 2 | 0.25 | 1 |
| 3 | 0.125 | 1 |

And we propose

Proposal 1: In Rel-16, Type 1 switching delay should be supported.

# Reference

- R4-1910590, WF on switching time and interruption for MIMO layer adaption, Aug., 2019.

- TR 38.84 v16.0.0, 3GPP, Jun., 2019.

- R4-1810021, BWP Switching Delay, Aug., 2019.

# Appendix

The simulation settings used in our system level simulation assumptions are listed as follows:

- General Parameters:

|  | FTP/Video |
| --- | --- |
| Model | FTP model 3 |
| Packet size (Mbytes) | 0.5/0.25/0.125/0.0625/0.0313 |
| Mean inter-arrival time (ms) | 200/100/50/25/12.5 |

- Settings in different frequency ranges:

| Simulation parameters | FR1 | FR2 |
| --- | --- | --- |
| Carrier center frequency | 4 GHz | 30 GHz |
| Subcarrier spacing | 15, 30 kHz | 120 kHz |
| Bandwidth (per CC) | 100MHz | 100MHz |
| # of CC | 1 | 4 |
| Channel model | IMT2020 3D UMa | IMT2020 3D UMa |
| Deployment | Dense Urban | Dense Urban |
| ISD | 200 m | 200 m |
| # of BS Tx antennas | 32 | 2 |
| # of UE Rx antennas | 4 | 2 |
| MIMO scheme | SU-MIMO | SU-MIMO |
| Max. # of HARQ RETX | 4 | 4 |
| # of UEs/Cell | 10 (FTP/Video) | 10 (FTP/Video) |