3GPP TSG-RAN WG4 Meeting #93	 R4-1914154

Reno, US, 18-22 Nov, 2019

Title:	Draft LS on FR2 MPE enhancements

Response to:	-

Release:	Release 16

Work Item:	NR_RF_FR2_req_enh

Source:	OPPO (TSG RAN WG4)

To:	TSG RAN WG2

Cc:	TSG RAN WG1, TSG RAN

Contact Person:

Name:	Jinqiang Xing

E-mail Address:	xingjinqiang@oppo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

In Rel-15, maxUplinkDutyCycle-FR2 capability was defined to help UEs meet MPE requirements, when the scheduled UL duty cycle exceeds this capability then PMPR will be applied. Radio link failure might happen when PMPR is large. RAN4 has discussed Rel-16 enhancements to solve this potential radio link failure issue and got following agreed solution.

- A new network configured UL duty cycle threshold below maxUplinkDutyCycle-FR2 is introduced, and the values could be {10%, 20% … 90%} corresponding to the maxUplinkDutyCycle-FR2 capability with default value 10%.

- PMPR value sends by UE to the network when the scheduled UL duty cycle within the evaluation window achieves this threshold. The PMPR value could be {0, 2, 4, 6, 8, 10, 12, 14, 16, 18} and default value is 0.

RAN4 would like to ask RAN2 to take above into consideration when design necessary signalling.

2. Actions:

ACTION: 	RAN4 respectfully ask RAN2 to design signalling for network configured UL duty cycle threshold and UE reported PMPR in Rel-16.

3. Date of Next TSG-RAN WG4 Meetings:

3GPP RAN4#94  	24 – 28 Feb 2020      Athens, Greece