3GPP TSG-RAN WG4 Meeting #93	R4-1914339

Reno, NV, USA, 18 – 22 November 2019

Agenda Item:	8.13.4.2

Source:	Ericsson

Title:	Simulation results of PRACH for LTE HST

Document for:	Discussion

# 1	Introduction

RAN4#92bis agreed with the WF on BS PRACH demodulation requirements for LTE HST [1].

- Agreement - Introduce new test cases with frequency offset = 1944Hz for PRACH restricted set type B. Define test cases for PRACH formats 0/1/2/3. - PRACH format: 0/1/2/3 - Channel model: AWGN - Frequency offset: 1944 Hz - Antenna configuration: 1x2 / 1x4 / 1x8 - Other assumptions are same as existing PRACH restricted set type B requirement

We provide our alignment simulation results according to WF.

# 2	Simulation results

Table 1 lists SNR values to achieve 1% of PRACH preamble missed detection rate with AWGN with frequency offset of 1944Hz.

Table 1	Alignment simulation results of PRACH missed detection requirements for restriction set type B.

| Number of Rx antennas | Format 0 | Format 1 | Format 2 | Format 3 |
| --- | --- | --- | --- | --- |
| 2 | -14.76 | -14.5 | -16.91 | -17.15 |
| 4 | -17.22 | -16.97 | -19.22 | -19.42 |
| 8 | -19.5 | -19.24 | -21.2 | -21.54 |

# References

- R4-1912777, “Way forward on LTE HST BS demodulation requirements”, NTT DOCOMO.

# Appendix

Table A.6-4 Test preambles for High speed Mode restricted set type B

| Burst format | Ncs | Logical sequence index | v |
| --- | --- | --- | --- |
| 0 | 15 | 30 | 30 |
| 1 | 100 | 168 | 20 |
| 2 | 118 | 204 | 10 |
| 3 | 137 | 264 | 0 |