3GPP TSG-RAN WG4 Meeting #93	R4-1914347

Reno, NV, USA, 18 – 22 November 2019

Agenda Item:	9.18.1.1

Source:	Ericsson

Title:	Simulation results of NR PDSCH demodulation requirements with CA

Document for:	Discussion

# 1	Introduction

RAN4#92bis agreed with the way forward on NR PDSCH demodulation requirement for CA [1] and agreed to start with the single carrier channel bandwidth FR1 FDD 15kHz, FR1 TDD 30kHz and FR2 120kHz SCS.

We provide our initial simulation based on the simulation assumption [2] and also discuss MCS/rank for FR2.

# 2	Simulation results

## 2.1	FR1 FDD

Table 1 shows the SNRs to achieve 70% of the maximum throughput for FR1 FDD SCS=15kHz with MCS13.

Table 1	SNRs to achieve 70% of the maximum throughput for FR1 FDD SCS=15kHz.

|  | 5MHz | 10MHz | 15MHz | 20MHz | 25MHz | 30MHz | 40MHz | 50MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2Rx | 10.1 | 10.1 | 10.3 | 10.5 | 10.6 | 10.7 | 10.9 | 11.3 |
| 4Rx | 6.6 | 6.5 | 6.6 | 6.7 | 6.8 | 6.8 | 6.9 | 7.2 |

## 2.2	FR1 TDD

Table 2 shows the SNRs to achieve 70% of the maximum throughput for FR1 TDD SCS=30kHz with MCS13.

Table 2	SNRs to achieve 70% of the maximum throughput for FR1 TDD SCS=30kHz.

|  | 5MHz | 10MHz | 15MHz | 20MHz | 25MHz | 30MHz | 40MHz | 50MHz | 60MHz | 80MHz | 90MHz | 100MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2Rx | 10.2 | 10.4 | 10.3 | 10.5 | 10.6 | 10.7 | 11.0 | 11.0 | 11.3 | 11.4 | 11.3 | 11.7 |
| 4Rx | 6.5 | 6.6 | 6.6 | 6.7 | 6.9 | 6.8 | 6.9 | 7.2 | 7.2 | 7.4 | 7.3 | 7.7 |

## 2.3	FR2 TDD

For FR2 TDD SCS=120kHz, there are several options.

Rank:

- Option 1: rank 2

- Option 2: rank 1

MCS

- Option 1: MCS 13 (16QAM 1/2)

- Option 2: MCS 4 (QPSK 1/3)

- Option 3-1: MCS10 (16QAM)

- Option 3-2: MCS11 (16QAM)

When we decide the test parameter, it is important to consider the achievable SNR levels on OTA as shown in Appendix.

Table 3 shows the SNRs to achieve 70% of the maximum throughput for possible FR2 parameter options. According to the TE vendor input, the smallest achievable SNR level is 10dB in the case of n260 with 800MHz CBW. If we consider the impairment margins etc, it is safer to choose MCS/rank so that SNR < 8.0dB. According to our simulation results the possible options are 1) MCS13 with rank 1 or 2) MCS10 with rank 2.

Table 3	Required SNRs to achieve 70% of the maximum throughput.

| Rank | MCS | 50MHz | 100MHz | 200MHz | 400MHz |
| --- | --- | --- | --- | --- | --- |
| 1 | 13 (16QAM 1/2) | 5.9 | 5.4 | 5.1 | 5.1 |
|  | 4 (QPSK 1/3) | -1.8 | -2.4 | -2.7 | -2.8 |
|  | 10 (16QAM) | 3.1 | 2.7 | 2.2 | 2.1 |
|  | 11 (16QAM) | 3.8 | 3.5 | 3.0 | 3.0 |
| 2 | 13 (16QAM 1/2) | 11.8 | 11.9 | 11.7 | 11.8 |
|  | 4 (QPSK 1/3) | 1.6 | 1.4 | 1.3 | 1.4 |
|  | 10 (16QAM) | 8.1 | 7.9 | 7.7 | 7.8 |
|  | 11 (16QAM) | 9.0 | 9.0 | 8.9 | 9.0 |

Proposal: RAN4 will choose MCS and Rank for FR2 CA scenario from the following options:

Option 1: MCS13 with rank 1

Option 2: MCS10 with rank 2

# 3	Summary

Proposal: RAN4 will choose MCS and Rank for FR2 CA scenario from the following options:

Option 1: MCS13 with rank 1

Option 2: MCS10 with rank 2

# References

- R4-1912832, “Way forward on PDSCH CA normal demodulation requirements”, Intel.

- R4-1912810, “Simulation assumptions for NR FR1 normal CA UE performance requirements”, Intel.

# Appendix	Achievable SNR levels on OTA

| Operating band | SNR (dB) / Aggregated Channel bandwidth |  |  |
| --- | --- | --- | --- |
|  | 200 MHz | 400 MHz | 800 MHz |
| n257 | 21.8 | 18.8 | 15.8 |
| n258 | 21.8 | 18.8 | 15.8 |
| n260 | 16 | 13 | 10 |
| n261 | 21.8 | 18.8 | 15.8 |