3GPP TSG-RAN WG4 Meeting #93	R4-1914349

Reno, NV, USA, 18 – 22 November 2019

Agenda Item:	9.18.1

Source:	Ericsson

Title:	Release independent UE demodulation and CSI reporting requirement for Rel-16

Document for:	Discussion

# 1	Introduction

3GPP approved the initial release of the release independent specification for NR UE in Release 16 [1], and this specification contains the clause corresponds to UE demodulation requirements. In this contribution, we initiate the discussion of the release independent UE demodulation requirements.

# 2	Release independent features

According to [1], release independent features are defined as follows:

release independent: applicable to some frozen releases, starting from a certain release Rel-M NOTE 1: Normally, a feature is introduced only in the latest open release Rel-N and future releases are based on the previous one so that future releases inherit the requirements of this feature. Introducing a feature "in a release independent way from Rel-M onwards" (M<N) means it was decided by TSG RAN that this feature would be also beneficial in previous, already frozen releases starting with Rel-M until Rel-(N-1). In order to avoid touching TS 38.101 [2-5] or TS 38.133 [6] of these frozen releases, the corresponding requirements are captured in TS 38.307 via pointers to [2-5] or [6] of the release in which the feature was introduced. NOTE 2: Release independent does not mean applicable to all releases.

It also specifies that:

TSG-RAN has agreed for certain features (see the following clauses) to introduce them in a "release independent way". This means for each feature: - it is "introduced" in a release N, i.e. TS 38.101 [2-5] and TS 38.133 [6] of release N define certain UE requirements for this feature; the feature is indicated in the tables of the following clauses; - it is "release independent" starting from a release M (M<N); M for the given feature is provided in the tables of the following clauses; - UEs supporting this feature have to fulfil additional requirements in release M or higher which are specified in one or more Annexes of TS 38.307 of release N; the applicable Annexes for a given feature are provided in the tables of the following clauses. The applicable UE Categories are specified in TS 38.306 [7] according to the release to which the UE conforms.

Figure 1 illustrates an example of release independent features.

Suppose the open release N introduces a new UE demodulation requirement X as a part of work item in RAN4. If 3GPP agrees that the requirement X is release independent from release M (M<N), then 3GPP introduces the information in TS 38.307 of Release N:

- feature name

- release independent from, and

- corresponding sections in TS38.101-4

Note for LTE, 3GPP has applied the release independent features for CA (from Rel-10), 4Rx UE (from Rel-10), 8Rx UE (from Rel-13), and High speed UE (from Rel-13).

Figure 1	Example of release independent feature of ‘Requirement X’ from Release M.

# 3	Way forward

In Rel-16, 3GPP approved several UE performance improvement work items [2][3] that are based on Rel-15 core specification, such as NR CA, PMI test with 16/32 Tx ports, LTE-NR co-existence and high speed train scenario. We think these features are the candidate of release independent features from Rel-15.

Observation: The following UE demodulation and CSI reporting requirements are the candidate of release independent features from Release 15:

- UE demodulation and CSI reporting test for NR CA

- PMI reporting test with Tx ports more than 8

- LTE-NR co-existence scenario

- High speed train

Based on the observation, we propose to discuss which requirements are specified as the release independent from release 15 onwards.

Proposal: RAN4 discuss what NR UE demodulation and CSI reporting requirements are release independent from Release 15 onward.

# References

- 3GPP TS 38.307, “NR; Requirements on User Equipments (UEs) supporting a release-independent frequency band”, V16.0.0.

- RP-191512, “New WID on NR support for high speed train scenario”, CMCC.

- RP-191585, “New WID on NR performance requirement enhancement”, China Telecom.