3GPP TSG-RAN4 Meeting #93	R4-1914455

# Reno, US, 18th - 22th Nov, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 6697 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on the conditional handover in TS 36.133 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_feMob-Core |  |  |  |  |  | Date: |  |  | 2019-10-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The conditional handover requirements shall be specifed in the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The conditional handover requirements are specified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

5.2	E-UTRAN Conditional Handover

5.2.1	Introduction

The purpose of conditional handover is to change the PCell to another cell which meets the CHO execution condition.

5.2.2	Requirements

5.2.2.1	E-UTRAN FDD – FDD

The requirements in this clause are applicable to both intra-frequency and inter-frequency handovers.

5.2.2.1.1	Conditional Handover delay

Procedure delays for the procedure that can command a conditional handover are specified in TS 36.331 [2].

When the UE receives a RRC message containing conditional handover execution conditions, the UE shall be ready to start the transmission of the new uplink PRACH channel within DCHO seconds from the end of the last TTI containing the RRC command.

DCHO = TRRC_1 + Tevaluation + Tinterrupt ms

Where:

TRRC_1 is the RRC process time to decode a RRC message containing the conditional handover execution conditions for the selected target cell.

Tevaluation is the time from the end of UE successfully decoding RRC message containing conditional handover execution conditions until the UE starts to execute a conditional handover upon UE realizes the CHO execution condition is met.

Tinterrupt is the interruption time stated in clause 5.1.2.1.2.

During the time period Tevaluation, the UE shall perform the relevant intra-frequency or inter-frequency meansurements on the target cell, according to the measurements requirements described in Clause 8.1.2.2 for intra-frequency handover and Clause 8.1.2.3 for inter-frequency handover and evaluate the conditional handover execution contition defined in [2].

5.2.2.1.2	Interruption time

The interruption time is the time between end of the last TTI containing the RRC command on the old PDSCH and the time the UE starts transmission of the new PRACH.

When intra-frequency or inter-frequency handover is commanded, the interruption time shall be less than Tinterrupt

Tinterrupt = TRRC_2 + TIU + 20 ms

Where:

TRRC_2 is the RRC process time to apply the stored RRC reconfiguration prepared for the selected target cell.

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to 30 ms.

NOTE: The actual value of TIU shall depend upon the PRACH configuration used in the target cell.

5.2.2.2	E-UTRAN FDD – TDD

The requirements in this clause are applicable to conditional handover from FDD to TDD. The requirements in this clause shall apply to UE supporting FDD and TDD.

The requirements in clause 5.2.2.1 apply for this section.

5.2.2.3	E-UTRAN TDD – FDD

The requirements in this clause are applicable to conditional handover from TDD to FDD. The requirements in this clause shall apply to UE supporting FDD and TDD.

The requirements in clause 5.2.2.1 apply for this section.

5.2.2.4	E-UTRAN TDD – TDD

The requirements in this clause are applicable to conditional handover from TDD to TDD. The requirements in this clause shall apply to UE supporting TDD.

The requirements in clause 5.2.2.1 apply for this section.

<End of Change 1>