3GPP TSG-RAN WG4 Meeting # 93	R4-1914482

# Reno, USA, November 18th - 22nd 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 6713 | rev |  | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of NR Inter-RAT measurement for Idle Mode CA measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core |  |  |  |  |  | Date: |  |  | 2019-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of NR Inter-RAT measurement for Idle Mode CA measurements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of NR inter-RAT measurement for idle mode CA measurement for a UE supporting EN-DC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.9. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Next Modified Subclause

## 4.9	Idle Mode CA Measurement

### 4.9.1	Introduction

A UE supporting ca-IdleModeMeasurements shall perform the idle mode measurement on the inter-frequency CA candidate frequencies/cells indicated by higher layers and meet the requirement specified in this section.

A UE supporting ca-IdleModeMeasurements and EN-DC shall perform the idle mode measurement on the inter-frequency CA candidate and on inter-RAT DC candidate frequencies/cells indicated by higher layers and meet the requirement specified in this section.

Additionally, a UE supporting ca-IdleModeMeasurements or a UE supporting ca-IdleModeMeasurements and EN-DC shall perform the idle mode measurement on serving cell and meet the requirement specified in this section.

### 4.9.2	Requirement

For a UE which supports ca-IdleModeMeasurements the UE shall support the idle mode CA measurements on the serving cell, overlapping and non-overlapping carriers. An overlapping carrier is defined as a carrier configured by higher layer for early measurement reporting and inter-frequency mobility measurements. A non-overlapping carrier is defined as a carrier configured by higher layer for early measurement reporting while not configured for inter-frequency mobility measurements.

For a UE which supports ca-IdleModeMeasurements and EN-DC the UE shall support the idle mode CA measurements on the serving cell, overlapping and non-overlapping inter-frequency carriers. Additionally, the UE shall support idle mode DC measurements on inter-RAT NR carriers. An overlapping inter-frequncy carrier is defined as a carrier configured by higher layer for early measurement reporting and inter-frequency mobility measurements. A non-overlapping inter-frequency carrier is defined as a carrier configured by higher layer for early measurement reporting while not configured for inter-frequency mobility measurements.

#### 4.9.2.1	Detected cell requirement during state transition and Idle mode

This subclause defines the requirements for the detected cell status for the idle mode CA and DC measurement when UE transitions from RRC Connected mode to Idle mode and after UE has entered Idle mode. The requirements are applicable to an E-UTRAN carrier aggregation capable UE which has been configured with one or more downlink SCells during the Connected mode and which supports ca-IdleModeMeasurements. The requirements are applicable to an E-UTRAN carrier aggregation capable UE also capable of EN-DC, which has been configured with one or more downlink SCells and an NR PSCell during the Connected mode and which supports ca-IdleModeMeasurements and EN-DC. The requirements are applicable for E-UTRAN FDD and TDD SCell(s) and FDD and TDD NR PSCell.

Upon releasing the connection and if the UE has been configured with idle mode CA or DC measurement reporting, following requirements apply concerning the detected cells in Connected mode upon state transitioning to Idle mode and during Idle mode:

-	A cell which is detected cell in Connected mode prior to connection release, shall remain detected after UE has entered Idle mode and during Idle mode, provided that the following conditions are met:

-	The UE has been provided with a list of cells and/or carrier frequencies for early measurement reporting by dedicated RRC signaling and

-	The detected cell is among the list of cells or on a carrier frequency provided for early measurement reporting, and

-	The UE is provided with a valid timer T331 by dedicated RRC signaling, and

-	The detected cell remains detectable until UE reconnects to the network and transmits the early measurement report.

-	The carrier frequency of the detected cell and the carrier frequency of the serving cell are among the supported band combination of the UE.

An inter-frequency cell is considered detectable according to RSRP, RSRP Ês/Iot, SCH_RP and SCH Ês/Iot defined in Annex B.1.1 and Annex B.1.2 for a corresponding Band.

An inter-RAT NR cell is considered to be detectable according to the conditions defined in Annex B.1.3 [38.133] for a corresponding Band.

In the absence or expiration of T331, it is up to UE implementation to apply the requirements on the detected cell status in this subclause.

#### 4.9.2.2	Measurements of inter-frequency CA candidate cells

While T331 is running, the UE shall perform measurement on the configured overlapping and non-overlapping inter-frequency carriers for idle mode measurement reporting.

A UE which supports ca-IdleModeMeasurements shall support the idle mode CA measurements of at least 3 inter-frequency carriers of which zero or one inter-frequency carrier may be a non-overlapping carrier when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ.

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ the UE shall support the idle mode CA measurements of at least 1 inter-frequency carrier of which zero or one inter-frequency carrier may be a non-overlapping carrier.

For overlapping carriers, the inter-frequency measurement requirements in section 4.2.2.4 apply when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ. If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, at least prior to transmission of the idle mode measurement report, the UE shall perform at least a single measurement on detected cells on the overlapping inter-frequency carrier(s) configured to be measured for early measurement reporting.

For non-overlapping carriers, at least prior to transmission of the idle mode measurement report, the UE shall perform at least a single measurement on detected cells on the non-overlapping inter-frequency carrier(s) configured to be measured for early measurement reporting.

In the absence or expiration of T331, it is up to UE implementation to perform the idle mode CA measurement.

For overlapping carriers, when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, the UE shall be capable of performing RSRP and RSRQ measurements of the overlapping carriers, and the UE physical layer shall be capable of reporting RSRP and RSRQ measurements of the overlapping carriers to higher layers, with measurement accuracy as specified in sub-clauses 9.1.3B.2 and 9.1.6B.2, respectively. The UE shall be able to report idle mode CA measurements when idle mode CA measurement reporting is requested by the network.

For overlapping carriers, when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, and non-overlapping carriers, the UE shall be capable of performing RSRP and RSRQ measurements of the overlapping and non-overlapping carriers, and the UE physical layer shall be capable of reporting RSRP and RSRQ measurements to higher layers, with measurement accuracy as specified in sub-clauses 9.1.3B.3 and 9.1.6B.3, respectively. The UE shall be able to report idle mode CA measurements when idle mode CA measurement reporting is requested by the network.

#### 4.9.2.3	Measurements on serving cell

The UE shall measure the RSRP and RSRQ level of the serving cell and evaluate the cell selection criterion S defined in section 4.2.2.1 and the UE physical layer shall be capable of reporting RSRP and RSRQ measurements of the serving cell to higher layers, with measurement accuracy as specified in sub-clauses 9.1.2B.2 and 9.1.5B.2, respectively.

#### 4.9.2.4	Measurements of inter-RAT DC candidate cell

While T331 is running, the UE shall perform measurement on the configured NR inter-RAT carriers for idle mode measurement reporting.

A UE which supports ca-IdleModeMeasurements and EN-DC shall support the idle mode DC NR inter-RAT measurements of at least [3] inter-RAT carriers in addition to the requirements in section 4.9.2.2 and 4.9.2.3.

The NR inter-RAT measurement requirements in section 4.2.2.5.6 apply assuming no s-measure threshold configured.

In the absence or expiration of T331, it is up to UE implementation to perform the idle mode CA measurement.

For configured inter-RAT carriers, the UE shall be capable of performing SS-RSRP and SS-RSRQ measurements, and the UE physical layer shall be capable of reporting SS-RSRP and SS-RSRQ measurements of the carriers to higher layers, with measurement accuracy as specified in sub-clauses [TBD] and [TBD], respectively. The UE shall be able to report idle mode DC measurements when idle mode DC measurement reporting is requested by the network.

Next Modified Subclause

## 4.10	Idle Mode EN-DC Measurement

### 4.10.1	Introduction

A UE supporting EN-DC shall perform the idle mode measurement on the inter-RAT DC candidate frequencies/cells as indicated by higher layers and meet the requirement specified in this section.

Additionally, a UE supporting EN-DC shall perform the idle mode measurement on serving cell and meet the requirement specified in this section.

### 4.10.2	Requirement

For a UE which supports EN-DC, the UE shall support the idle mode EN-DC measurements on inter-RAT NR carriers configured for idle mode EN-DC measurements.

#### 4.10.2.1	Detected cell requirement during state transition and Idle mode

This subclause defines the requirements for the detected cell status for the EN-DC measurement when UE transitions from RRC Connected mode to Idle or Inactive mode and after UE has entered Idle or Inactive mode. The requirements are applicable to an E-UTRAN EN-DC capable UE which has been configured with an NR PSCells during the Connected mode. The requirements are applicable for NR FDD and TDD PSCell.

Upon releasing the connection and if the UE has been configured with EN-DC measurement reporting, following requirements apply concerning the detected cells in Connected mode upon state transitioning to Idle or Inactive mode and during Idle and Inactive mode:

-	A cell which is detected cell in Connected mode prior to connection release, shall remain detected after UE has entered Idle mode and during Idle mode, provided that the following conditions are met:

-	The UE has been provided with a list of cells and/or carrier frequencies for early measurement reporting by dedicated RRC signaling and

-	The detected cell is among the list of cells or on a carrier frequency provided for early measurement reporting, and

-	The UE is provided with a valid timer T331 by dedicated RRC signaling, and

-	The detected cell remains detectable until UE reconnects to the network and transmits the early measurement report.

-	The carrier frequency of the detected cell and the carrier frequency of the serving cell are among the supported band combination of the UE.

An inter-RAT NR cell is considered to be detectable according to the conditions defined in Annex B.1.3 [38.133] for a corresponding Band.

In the absence or expiration of T331, it is up to UE implementation to apply the requirements on the detected cell status in this subclause.

#### 4.10.2.3	Measurements on serving cell

The UE shall measure the RSRP and RSRQ level of the serving cell and evaluate the cell selection criterion S defined in section 4.2.2.1 and the UE physical layer shall be capable of reporting RSRP and RSRQ measurements of the serving cell to higher layers, with measurement accuracy as specified in sub-clauses 9.1.2B.2 and 9.1.5B.2, respectively.

#### 4.10.2.4	Measurements of inter-RAT DC candidate cell

While T331 is running, the UE shall perform measurement on the configured NR inter-RAT carriers for idle mode measurement reporting.

A UE which supports EN-DC shall support the idle mode EN-DC NR inter-RAT measurements of at least [3] NR inter-RAT carriers in addition to the requirements in section 4.9.2.2 and 4.9.2.3.

The NR inter-RAT measurement requirements in section 4.2.2.5.6 apply.

In the absence or expiration of T331, it is up to UE implementation to perform the EN-DC measurements.

For configured NR inter-RAT carriers, the UE shall be capable of performing SS-RSRP and SS-RSRQ measurements, and the UE physical layer shall be capable of reporting SS-RSRP and SS-RSRQ measurements of the carriers to higher layers, with measurement accuracy as specified in sub-clauses [TBD] and [TBD], respectively. The UE shall be able to report idle mode EN-DC measurements when idle mode EN-DC measurement reporting is requested by the network.

Next Modified Subclause