3GPP TSG-RAN WG4 Meeting #93	R4-1914484

# Reno, USA, November 18th – 22nd 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0211 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR Introduction of handover delay requirements for conditional handover (section 6.1) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh-Core |  |  |  |  |  | Date: |  |  | 2019-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | CR capturing handover delay requirements for conditional handover |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR captures the handover delay requirements for conditional handover in section 6.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete requirements for handover delay requirements for conditional handover. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Next Modified Subclause

### 6.1	Handover

### 6.1.1	NR Handover

#### 6.1.1.1	Introduction

The purpose of NR handover is to change the NR PCell to another NR cell. The requirements in this clause are applicable to SA NR, NE-DC and NR-DC. The requirements in this clause are applicable to handover and conditional handover.

Next Modified Subclause

##### 6.1.1.6	NR FR1 – NR FR1 conditional handover

The requirements in this clause are applicable to both intra-frequency and inter-frequency conditional handover from NR FR1 cell to NR FR1 cell.

##### 6.1.1.6.1	Handover delay

Procedure delays for all procedures that can command a conditional handover are specified in TS 38.331 [2].

When the UE receives a RRC message implying conditional handover the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover seconds from the end of the last TTI containing the RRC command.

Where:

Dhandover equals the maximum RRC procedure delay to be defined in clause12 in TS 38.331 [2] plus the measurements time stated in clause 6.1.1.6.2 and plus the interruption time stated in clause 6.1.1.6.3.

6.1.1.6.2	Measurement time

The measurement time is the time between end of the last TTI containing the RRC command on the old PDSCH and the time when the UE executes the conditional handover to the target cell.

For intra-frequency or inter-frequency conditional conditional handover, the measurment time shall be less than

Tmeasure = Tidentify + Tuncertainty + TTTT + TL3-filter

Where:

Tidentify is the time needed for cell detection if the target cell included in the CHO has not been detected by the UE when the CHO command is received from the network. Tidentify depend on the target cell detection condition and equal one of: Tidentify_intra_without_index or Tidentify_intra_with_index or Tidentify_inter_without_index or Tidentify_inter_with_index.

Tuncertainty is the time period from when UE has received and decoded the RRC command including the CHO command and until the initial condition for a configured target cell is fulfilled excluding Tidentify.

TTTT is the TTT as configured in the CHO by the network.

TL3 Filter is the L3 filter to be applied as defined by the network in the CHO command.

6.1.1.6.3	Interruption time

The interruption time is the time between when the conditional handover execution is triggered in the UE and the time when the UE executes the conditional handover to the target cell.

For intra-frequency or inter-frequency conditional conditional handover, the measurment time shall be less than

Tinterrupt = Tprocessing + TIU + T∆ ms

Where:

Tprocessing is time for UE processing. Tprocessing can be up to 20ms.

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3]

T∆ is time for fine time tracking and acquiring full timing information of the target cell. TΔ = Trs.

NOTE 1:	The actual value of TIU shall depend upon the PRACH configuration used in the target cell.

##### 6.1.1.7	NR FR2 – NR FR1 conditional handover

The requirements in this clause are applicable to both intra-frequency and inter-frequency conditional handover from NR FR2 cell to NR FR1 cell.

##### 6.1.1.7.1	Handover delay

Procedure delays for all procedures that can command a conditional handover are specified in TS 38.331 [2].

When the UE receives a RRC message implying conditional handover the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover seconds from the end of the last TTI containing the RRC command.

Where:

Dhandover equals the maximum RRC procedure delay to be defined in clause12 in TS 38.331 [2] plus the measurements time stated in clause 6.1.1.7.2 and plus the interruption time stated in clause 6.1.1.7.3.

6.1.1.7.2	Measurement time

The measurement time is the time between end of the last TTI containing the RRC command on the old PDSCH and the time when the UE executes the conditional handover to the target cell.

For intra-frequency or inter-frequency conditional conditional handover, the measurment time shall be less than

Tmeasure = Tidentify + Tuncertainty + TTTT + TL3-filter

Where:

Tidentify is the time needed for cell detection if the target cell included in the CHO has not been detected by the UE when the CHO command is received from the network. Tidentify depend on the target cell detection condition and equal one of: Tidentify_intra_without_index or Tidentify_intra_with_index or Tidentify_inter_without_index or Tidentify_inter_with_index.

Tuncertainty is the time period from when UE has received and decoded the RRC command including the CHO command and until the initial condition for a configured target cell is fulfilled excluding Tidentify.

TTTT is the TTT as configured in the CHO by the network.

TL3 Filter is the L3 filter to be applied as defined by the network in the CHO command.

6.1.1.7.3	Interruption time

The interruption time is the time between when the conditional handover execution is triggered in the UE and the time when the UE executes the conditional handover to the target cell.

For intra-frequency or inter-frequency conditional conditional handover, the measurment time shall be less than

Tinterrupt = Tprocessing + TIU + T∆ ms

Where:

Tprocessing is time for UE processing. Tprocessing can be up to 40ms.

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3]

T∆ is time for fine time tracking and acquiring full timing information of the target cell. TΔ = Trs.

NOTE 1:	The actual value of TIU shall depend upon the PRACH configuration used in the target cell.

##### 6.1.1.8	NR FR2 – NR FR2 conditional handover

The requirements in this clause are applicable to both intra-frequency and inter-frequency conditional handover from NR FR2 cell to NR FR2 cell.

##### 6.1.1.8.1	Handover delay

Procedure delays for all procedures that can command a conditional handover are specified in TS 38.331 [2].

When the UE receives a RRC message implying conditional handover the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover seconds from the end of the last TTI containing the RRC command.

Where:

Dhandover equals the maximum RRC procedure delay to be defined in clause12 in TS 38.331 [2] plus the measurements time stated in clause 6.1.1.8.2 and plus the interruption time stated in clause 6.1.1.8.3.

6.1.1.8.2	Measurement time

The measurement time is the time between end of the last TTI containing the RRC command on the old PDSCH and the time when the UE executes the conditional handover to the target cell.

For intra-frequency or inter-frequency conditional conditional handover, the measurment time shall be less than

Tmeasure = Tidentify + Tuncertainty + TTTT + TL3-filter

Where:

Tidentify is the time needed for cell detection if the target cell included in the CHO has not been detected by the UE when the CHO command is received from the network. Tidentify depend on the target cell detection condition and equal one of: Tidentify_intra_without_index or Tidentify_intra_with_index or Tidentify_inter_without_index or Tidentify_inter_with_index.

Tuncertainty is the time period from when UE has received and decoded the RRC command including the CHO command and until the initial condition for a configured target cell is fulfilled excluding Tidentify.

TTTT is the TTT as configured in the CHO by the network.

TL3 Filter is the L3 filter to be applied as defined by the network in the CHO command.

6.1.1.8.3	Interruption time

The interruption time is the time between when the conditional handover execution is triggered in the UE and the time when the UE executes the conditional handover to the target cell.

For intra-frequency or inter-frequency conditional conditional handover, the measurment time shall be less than

Tinterrupt = Tprocessing + TIU + T∆ ms

Where:

Tprocessing is time for UE processing. Tprocessing can be up to 20ms.

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3]

T∆ is time for fine time tracking and acquiring full timing information of the target cell. TΔ = Trs.

NOTE 1:	The actual value of TIU shall depend upon the PRACH configuration used in the target cell.

##### 6.1.1.9	NR FR1 – NR FR2 conditional handover

The requirements in this clause are applicable to both intra-frequency and inter-frequency conditional handover from NR FR1 cell to NR FR2 cell.

##### 6.1.1.9.1	Handover delay

Procedure delays for all procedures that can command a conditional handover are specified in TS 38.331 [2].

When the UE receives a RRC message implying conditional handover the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover seconds from the end of the last TTI containing the RRC command.

Where:

Dhandover equals the maximum RRC procedure delay to be defined in clause12 in TS 38.331 [2] plus the measurements time stated in clause 6.1.1.9.2 and plus the interruption time stated in clause 6.1.1.9.3.

6.1.1.9.2	Measurement time

The measurement time is the time between end of the last TTI containing the RRC command on the old PDSCH and the time when the UE executes the conditional handover to the target cell.

For intra-frequency or inter-frequency conditional conditional handover, the measurment time shall be less than

Tmeasure = Tidentify + Tuncertainty + TTTT + TL3-filter

Where:

Tidentify is the time needed for cell detection if the target cell included in the CHO has not been detected by the UE when the CHO command is received from the network. Tidentify depend on the target cell detection condition and equal one of: Tidentify_intra_without_index or Tidentify_intra_with_index or Tidentify_inter_without_index or Tidentify_inter_with_index.

Tuncertainty is the time period from when UE has received and decoded the RRC command including the CHO command and until the initial condition for a configured target cell is fulfilled excluding Tidentify.

TTTT is the TTT as configured in the CHO by the network.

TL3 Filter is the L3 filter to be applied as defined by the network in the CHO command.

6.1.1.9.3	Interruption time

The interruption time is the time between when the conditional handover execution is triggered in the UE and the time when the UE executes the conditional handover to the target cell.

For intra-frequency or inter-frequency conditional conditional handover, the measurment time shall be less than

Tinterrupt = Tprocessing + TIU + T∆ ms

Where:

Tprocessing is time for UE processing. Tprocessing can be up to 40ms.

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3]

T∆ is time for fine time tracking and acquiring full timing information of the target cell. TΔ = Trs.

NOTE 1:	The actual value of TIU shall depend upon the PRACH configuration used in the target cell.

Next Modified Subclause