3GPP TSG-RAN WG4 Meeting #93	R4-1914725

# Reno, USA, 14 November, 2019 – 18 November, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR: NR-U requirements for IDLE/INACTIVE states |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic |  |  |  |  |  | Date: |  |  | 2019-11-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | IDLE/INACTIVE state requirements needs to be specified for NR-U |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR contains changes for the IDLE/INACTIVE states for NR-U based collected agreements from R4-1912846, R4-1910551 and R4-1907331. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No requirements will apply for NR-U UEs operating in IDLE/INACTIVE states |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section: 4.2A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Change 1

## 4.2A	Cell Re-selection with CCA

### 4.2A.1	Introduction

The cell reselection procedure allows the UE to select a more suitable cell that is subject to CCA and camp on it.

When the UE is in either Camped Normally state or Camped on Any Cell state on a cell, the UE shall attempt to detect, synchronise, and monitor intra-frequency, inter-frequency and inter-RAT cells indicated by the serving cell. For intra-frequency and inter-frequency cells the serving cell may not provide explicit neighbour list but carrier frequency information and bandwidth information only. UE measurement activity is also controlled by measurement rules defined in TS 38.304, allowing the UE to limit its measurement activity.

In the requirements of clause 4.2A, the exceptions for side conditions apply as follows:

-	for the UE capable of CA, the applicable exceptions for side conditions are specified in Annex B, clause B.x.y.z for UE supporting CA in FR1;

-	for the UE capable of SUL, the applicable exceptions for side conditions are specified in Annex B, clause B.x.y.z for UE supporting SUL in FR1.

### 4.2A.2	Requirements

#### 4.2A.2.1	UE measurement capability

For idle mode cell re-selection purposes, the UE shall be capable of monitoring at least:

-	Intra-frequency carrier, and

-	Depending on UE capability, 7 NR inter-frequency carriers, and

-	Depending on UE capability, 7 FDD E-UTRA inter-RAT carriers, and

-	Depending on UE capability, 7 TDD E-UTRA inter-RAT carriers.

In addition to the requirements defined above, a UE supporting E-UTRA measurements in RRC_IDLE state shall be capable of monitoring a total of at least 14 carrier frequency layers, which includes serving layer, comprising of any above defined combination of E-UTRA FDD, E-UTRA TDD and NR layers.

#### 4.2A.2.2	Measurement and evaluation of serving cell subject to CCA

The UE shall measure the SS-RSRP and SS-RSRQ level of the serving cell and evaluate the cell selection criterion S defined in TS 38.304 [1] for the serving cell at least once every (1+Mn)*M1 DRX cycle; where:

M1=2 if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤  0.64 second,

otherwise M1=1,

Mn = the maximum separation in time between two measurements that are used for filtering of SS-RSRP/SS-RSRQ

The UE shall filter the SS-RSRP and SS-RSRQ measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by, at least DRX cycle/2 but not spaced by more than Mn where Mn=[2].

If the UE has evaluated according to Table 4.2A.2.2-1 in Nserv_CCA consecutive DRX cycles that the serving cell does not fulfil the cell selection criterion S, the UE shall initiate the measurements of all neighbour cells indicated by the serving cell, regardless of the measurement rules currently limiting UE measurement activities.

If the UE failed to measure on the serving cell for at least Mp consecutive number of DRX cycles, the UE shall initiate the measurements of all neighbour cells indicated by the serving cell, regardless of the measurement rules currently limiting UE measurement activities where:

Mp = [4] if DRX cycle < 1.28 seconds and Mp=[2] if DRX cycle ≥ 1.28 seconds

If the UE failed to measure on the serving cell for at least Mq consecutive number of DRX cycles, the UE shall initiate the measurements on any intra-frequency or inter-frequency neighbour cells, regardless of any condition on SnonIntraSearchP_CCA  and SnonIntraSearchQ_CCA.

If the UE in RRC_IDLE has not found any new suitable cell based on searches and measurements using the intra-frequency, inter-frequency and inter-RAT information indicated in the system information for 10 s, the UE shall initiate cell selection procedures for the selected PLMN as defined in TS 38.304 [1].

Table 4.2A.2.2-1: Nserv_CCA

| DRX cycle length [s] | Nserv_CCA [number of DRX cycles] |
| --- | --- |
| 0.32 | M1*4 + M1*Ms |
| 0.64 | M1*4 + M1*Ms |
| 1.28 | 2+Ms |
| 2.56 | 2+Ms |
| Note 1: Ms = the number of DRX cycles with at least one SMTC where there are no SSBs available at the UE during Nserv_CCA and Ms≤Ms,max where Ms,max = [8] for DRX cycle length < 1.28 seconds, Ms,max = [4] for DRX cycle length ≥ 1.28 seconds. |  |

#### 4.2A.2.3 	Measurements of intra-frequency NR cells subject to CCA

The UE shall be able to identify new intra-frequency cells subject to CCA and perform SS-RSRP and SS-RSRQ measurements of the identified intra-frequency cells without an explicit intra-frequency neighbour list containing physical layer cell identities.

The UE shall be able to evaluate whether a newly detectable intra-frequency cell meets the reselection criteria defined in TS38.304 within Tdetect,NR_Intra_CCA when that Treselection= 0  and provided that Md ≤ Md,max.  Otherwise, the UE shall restart the evaluation. An intra frequency cell is considered to be detectable according to the conditions defined in Annex B.x.y for a corresponding Band.

The UE shall measure SS-RSRP and SS-RSRQ at least every Tmeasure,NR_Intra_CCA (see table 4.2A.2.3-1) for intra-frequency cells that are identified and measured according to the measurement rules provided Mm ≤ Mm,max. Otherwise, the UE shall restart the measurement.

The UE shall filter SS-RSRP and SS-RSRQ measurements of each measured intra-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,NR_Intra_CCA/2.

The UE shall not consider a NR neighbour cell subject to CCA in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell. The UE shall instead evaluate using two cells that at least one of them meets the reselection criteria defined in TS38.304 provided that it is not more than TBD dB weaker than the serving cell.

For an intra-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the intra-frequency cell has met reselection criterion defined [1] within Tevaluate,NR_Intra_CCA when Treselection = 0 as specified in table 4.2A.2.3-1 provided that:

when rangeToBestCell is not configured:

-	the cell is at least 3dB better ranked in FR1.

when rangeToBestCell is configured:

-	the cell has the highest number of beams above the threshold absThreshSS-BlocksConsolidation among all detected cells whose cell-ranking criterion R value [1] is within rangeToBestCell of the cell-ranking criterion R value of the highest ranked cell.

-	if there are multiple such cells, the cell has the highest rank among them.

- the cell is at least 3dB better ranked in FR1 if the current serving cell is among them.

when Me ≤ Me,max. Otherwise, the UE shall restart the evaluation.

When evaluating cells for reselection, the SSB side conditions apply to both serving and non-serving intra-frequency cells.

If Treselection timer has a non zero value and the intra-frequency cell is satisfied with the reselection criteria which are defined in TS38.304 [1], the UE shall evaluate this intra-frequency cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

Table 4.2A.2.3-1: Tdetect,NR_Intra_CCA, Tmeasure,NR_Intra_CCA and Tevaluate,NR_Intra_CCA

| DRX cycle length [s] | Tdetect,NR_Intra_CCA [s] (number of DRX cycles) | Tmeasure,NR_Intra_CCA [s] (number of DRX cycles) | Tevaluate,NR_Intra_CCA [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 0.32x([36]+Md)xM2 {([36]+ Md)xM2} | 0.32x([4]+Mm) xM2 {([4]+ Mm)xM2} | 0.32x([16]+Me) x M2 {([16]+ Me)xM2} |
| 0.64 | 0.64x([28]+ Md) {[28]+ Md } | 0.64x([2]+ Mm) {[2]+ Mm } | 0.64x([8]+ Me) {[8]+ Me } |
| 1.28 | 1.28x([25]+ Md) {[25]+ Md } | 1.28x([1]+ Mm) {[1]+ Mm } | 1.28x([5]+ Me) {[5]+ Me } |
| 2.56 | 2.56x([23]+ Md) {[23]+ Md } | 2.56x([1]+ Mm) {[1]+ Mm } | 2.56x([3]+ Me) {[3]+ Me } |
| Note 1: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. Note 2: Mm ≤ Mm,max, where: Mm,max = TBD for DRX cycle = 0.32 seconds, Mm,max = [8] for DRX cycle = 0.64 seconds, Mm,max = [4] for DRX cycle = 1.28 seconds, Mm,max = TBD for DRX cycle = 2.56 seconds, Note 3: Md ≤ Md,max, where: Md,max = [4] * Mm,max, Note 4: Me ≤ Me,max, where: Me,max = [2] * Mm,max, |  |  |  |

#### 4.2A.2.4	Measurements of inter-frequency NR cells subject to CCA

The UE shall be able to identify new inter-frequency cells subject to CCA and perform SS-RSRP or SS-RSRQ measurements of identified inter-frequency cells if carrier frequency information is provided by the serving cell, even if no explicit neighbour list with physical layer cell identities is provided.

If Srxlev > SnonIntraSearchP_CCA and Squal > SnonIntraSearchQ_CCA then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2A.2.7.

If Srxlev ≤ SnonIntraSearchP_CCA or Squal ≤ SnonIntraSearchQ_CCA then the UE shall search for and measure inter-frequency layers of higher, equal or lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority layers shall be the same as that defined below in this subclause.

The UE shall be able to evaluate whether a newly detectable inter-frequency cell meets the reselection criteria defined in TS38.304 within Kcarrier_CCA * Tdetect,NR_Inter_CCA  if at least carrier frequency information is provided for inter-frequency neighbour cells by the serving cells when Treselection = 0 provided that the reselection criteria is met by a margin of at least 5 dB in FR1 for reselections based on ranking or 6dB in FR1 for SS-RSRP reselections based on absolute priorities or 4dB in FR1 for SS-RSRQ reselections based on absolute priorities and provided that Md ≤ Md,max. If the condition on Md ≤ Md,max is not fulfilled, the UE shall restart the evaluation. The parameter Kcarrier_CCA is the number of NR inter-frequency carriers indicated by the serving cell. An inter-frequency cell is considered to be detectable according to the conditions defined in Annex B.x.y for a corresponding Band.

When higher priority cells are found by the higher priority search, they shall be measured at least every Tmeasure,NR_Inter_CCA. If, after detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of reselection. However, the minimum measurement filtering requirements specified later in this section shall still be met by the UE before it makes any determination that it may stop measuring the cell. If the UE detects on a NR carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall measure SS-RSRP or SS-RSRQ at least every Kcarrier_CCA * Tmeasure,NR_Inter_CCA (see table 4.2A.2.4-1) for identified lower or equal priority inter-frequency cells provided Mm ≤ Mm,max. Otherwise, the UE shall restart the measurement. If the UE detects on a NR carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall filter SS-RSRP or SS-RSRQ measurements of each measured higher, lower and equal priority inter-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,NR_Inter_CCA/2.

The UE shall not consider a NR neighbour cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell. The UE shall instead evaluate using two cells that at least one of them meets the reselection criteria defined in TS38.304 provided that it is not more than TBD dB weaker than the serving cell.

For an inter-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the inter-frequency cell has met reselection criterion defined TS 38.304 within Kcarrier_CCA * Tevaluate,NR_Inter_CCA when Treselection = 0 as specified in table 4.2A.2.4-1 provided that the reselection criteria is met by

-	the condition when performing equal priority reselection and

when rangeToBestCell is not configured:

-	the cell is at least 5dB better ranked in FR1 or.

when rangeToBestCell is configured:

-	the cell has the highest number of beams above the threshold absThreshSS-BlocksConsolidation among all detected cells whose cell-ranking criterion R value [1] is within rangeToBestCell of the cell-ranking criterion R value of the highest ranked cell.

-	if there are multiple such cells, the cell has the highest rank among them

-	the cell is at least 5dB better ranked in FR1 if the current serving cell is among them. or

-	6dB in FR1 for SS-RSRP reselections based on absolute priorities or

-	4dB in FR1 for SS-RSRQ reselections based on absolute priorities.

- when Me ≤ Me,max. Otherwise, the UE shall restart the evaluation.

When evaluating cells for reselection, the SSB side conditions apply to both serving and inter-frequency cells.

If Treselection timer has a non zero value and the inter-frequency cell is satisfied with the reselection criteria, the UE shall evaluate this inter-frequency cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

The UE is not expected to meet the measurement requirements for an inter-frequency carrier under DRX cycle=320 ms defined in Table 4.2A.2.4-1 under the following conditions:

-	TSMTC_intra = TSMTC_inter = 160 ms; where TSMTC_intra and TSMTC_inter are periodicities of the SMTC occasions configured for the intra-frequency carrier and the inter-frequency carrier respectively, and

-	SMTC occasions configured for the inter-frequency carrier occur up to 1 ms before the start or up to 1 ms after the end of the SMTC occasions configured for the intra-frequency carrier, and

-	SMTC occasions configured for the intra-frequency carrier and for the inter-frequency carrier occur up to 1 ms before the start or up to 1 ms after the end of the paging occasion [1].

Table 4.2A.2.4-1: Tdetect,NR_Inter_CCA, Tmeasure,NR_Inter_CCA and Tevaluate,NR_Inter_CCA

| DRX cycle length [s] | Tdetect,NR_Inter_CCA [s] (number of DRX cycles) | Tmeasure,NR_Inter_CCA [s] (number of DRX cycles) | Tevaluate,NR_Inter_CCA [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 0.32x([36]+Md) {([36]+ Md) } | 0.32x([4]+Mm) {([4]+ Mm) } | 0.32x([16]+Me) {([16]+ Me) } |
| 0.64 | 0.64x([28]+ Md) {[28]+ Md } | 0.64x([2]+ Mm) {[2]+ Mm } | 0.64x([8]+ Me) {[8]+ Me } |
| 1.28 | 1.28x([25]+ Md) {[25]+ Md } | 1.28x([1]+ Mm) {[1]+ Mm } | 1.28x([5]+ Me) {[5]+ Me } |
| 2.56 | 2.56x([23]+ Md) {[23]+ Md } | 2.56x([1]+ Mm) {[1]+ Mm } | 2.56x([3]+ Me) {[3]+ Me } |
| Note 1: Mm ≤ Mm,max, where: Mm,max = TBD for DRX cycle = 0.32 seconds, Mm,max = [8] for DRX cycle = 0.64 seconds, Mm,max = [4] for DRX cycle = 1.28 seconds, Mm,max = TBD for DRX cycle = 2.56 seconds, Note 2: Md ≤ Md,max, where: Md,max = [4] * Mm,max, Note 3: Me ≤ Me,max, where: Me,max = [2] * Mm,max, |  |  |  |

#### 4.2A.2.5	Measurements of inter-RAT E-UTRAN cells

If Srxlev > SnonIntraSearchP_CCA and Squal > SnonIntraSearchQ_CCA then the UE shall search for inter-RAT E-UTRAN layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2A.2

If Srxlev ≤ SnonIntraSearchP_CCA or Squal ≤ SnonIntraSearchQ_CCA then the UE shall search for and measure inter-RAT E-UTRAN layers of higher, lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority inter-RAT E-UTRAN layers shall be the same as that defined below for lower priority RATs.

The requirements in this section apply for inter-RAT E-UTRAN FDD measurements and E-UTRA TDD measurements. When the measurement rules indicate that inter-RAT E-UTRAN cells are to be measured, the UE shall measure RSRP and RSRQ of detected E-UTRA cells in the neighbour frequency list at the minimum measurement rate specified in this section. The parameter NEUTRA_carrier is the total number of configured E-UTRA carriers in the neighbour frequency list. The UE shall filter RSRP and RSRQ measurements of each measured E-UTRA cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,EUTRAN/2.

An inter-RAT E-UTRA cell is considered to be detectable provided the following conditions are fulfilled:

-	the same conditions as for inter-frequency RSRP measurements specified in TS 36.133 [15, Annex B.x.y] are fulfilled for a corresponding Band, and

-	the same conditions as for inter-frequency RSRQ measurements specified in TS 36.133 [15, Annex B.x.y] are fulfilled for a corresponding Band.

-	SCH conditions specified in TS 36.133 [15, Annex B.x.y] are fulfilled for a corresponding Band

The UE shall be able to evaluate whether a newly detectable inter-RAT E-UTRAN cell meets the reselection criteria defined in TS38.304 within (NEUTRA_carrier) * Tdetect,EUTRAN when Srxlev ≤ SnonIntraSearchP_CCA or Squal ≤ SnonIntraSearchQ_CCA when Treselection = 0 provided that the reselection criteria is met by a margin of at least 6dB for RSRP reselections based on absolute priorities or 4dB for RSRQ reselections based on absolute priorities.

Cells which have been detected shall be measured at least every (NEUTRA_carrier) * Tmeasure,EUTRAN when Srxlev ≤ SnonIntraSearchP_CCA or Squal ≤ SnonIntraSearchQ_CCA.

When higher priority cells are found by the higher priority search, they shall be measured at least every Tmeasure,EUTRAN. If, after detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of reselection. However, the minimum measurement filtering requirements specified later in this section shall still be met by the UE before it makes any determination that it may stop measuring the cell.

If the UE detects on an inter-RAT E-UTRAN carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall not consider an inter-RAT E-UTRA cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell. The UE shall instead evaluate using two cells that at least one of them meets the reselection criteria defined in TS38.304 provided that it is not more than TBD dB weaker than the serving cell.

For a cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that an already identified inter-RAT E-UTRA cell has met reselection criterion defined in TS 38.304 [1] within (NEUTRA_carrier) * Tevaluate,EUTRAN when Treselection = 0 as speficied in table 4.2A.2.5-1 provided that the reselection criteria is met by a margin of at least 6dB for RSRP reselections based on absolute priorities or 4dB for RSRQ reselections based on absolute priorities.

If Treselection timer has a non zero value and the inter-RAT E-UTRA cell is satisfied with the reselection criteria which are defined in TS 38.304 [1], the UE shall evaluate this E-UTRA cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

Table 4.2A.2.5-1: Tdetect,EUTRAN, Tmeasure,EUTRAN, and Tevaluate,EUTRAN

| DRX cycle length [s] | Tdetect,EUTRAN [s] (number of DRX cycles) | Tmeasure,EUTRAN [s] (number of DRX cycles) | Tevaluate,EUTRAN [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 11.52 (36) | 1.28 (4) | 5.12 (16) |
| 0.64 | 17.92 (28) | 1.28 (2) | 5.12 (8) |
| 1.28 | 32(25) | 1.28 (1) | 6.4 (5) |
| 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |

#### 4.2A.2.7	General requirements

The UE shall search every layer of higher priority at least every Thigher_priority_search = (60 * Nlayers) seconds, where Nlayers is the total number of higher priority NR and E-UTRA carrier frequencies broadcasted in system information.

End of change 1