3GPP TSG-RAN WG4 Meeting #93	R4-1914730

Reno, USA, 18 – 22 October 2019

Agenda Item:	8.11.3.2

Source:	Ericsson

Title:	Remaining discussions on RRM requirements for NB-IoT PUR

Document for:	Discussion

# 1	Introduction

At previous meeting, RAN4 reached following agreement about time range for measurements used for validating the TA for PUR transmissions [1]:

- The first and the second measurement conditions agreed at RAN4#92 meeting (R4-1910576) include parameters M and N, whose values are defiend as follows: - When TA is obtained in RRC_CONNECTED mode: M=[TEvaluate_Qin_ NB-IoT /2] - When TA is obtained in RRC_IDLE mode: - Option 1: - M = min (Tmeasure_intra_UE NB-IoT in nonDRX, DRX_cycle length) - N = min (Tmeasure_intra_UE NB-IoT in nonDRX, DRX_cycle length) - Other options are not precluded.

As can be seen, there is one option that under consideration. In this contribution we discuss and provide our view on this option.

# 2	Discussion

NB-IOT operations support following DRX cycle configurations:

Tabell 1 DRX cycles support for NB-IoT

DRX cycle length [s]

1.28

2.56

5.12

10.24

The formula in option 1 for deriving the value of M and N are as follows:

M = min (Tmeasure_intra_UE NB-IoT in nonDRX, DRX_cycle length)            (1)

N = min (Tmeasure_intra_UE NB-IoT in nonDRX, DRX_cycle length)            (2)

Tmeasure_intra_UE NB-IoT in nonDRX corresponds to the L1 measurement period in RRC_CONNECTED mode, which is s follows [2]:

Table 1 L1 measurement period for NRSRP measurement based on NRS

| Coverage level | L1 measurement period [ms] |
| --- | --- |
| Normal coverage | 800 |
| Enhanced coverage | 1600 |

Table 2 L1 measurement period for NRSRP measurement based on NSSS

| Coverage level | L1 measurement period [ms] |
| --- | --- |
| Normal coverage | 1600 |
| Enhanced coverage | 1600 |

It is observed that the measurement delay depends on the coverage level and type of reference signal used for measurements. Having the delay expressed in absolute numbers is desired as it simplifies the formula and enhances the readability. We propose to take the measurement period corresponding to the NSSS which is longer than corresponding NRS based measurement, and this can work regardless of type of signals used for measurements, i.e. 1600 ms in normal coverage and enhanced coverage. This will lead to improved UE power consumption as UE can reuse the old measurements for a longer time and the PUR transmissions can be received timely by the base station. Following this formula, the values of M and N are as shown in Table 4.

We therefore would like to confirm using option 1 in the agreed way forward, and the parameter Tmeasure_intra_UE NB-IoT in nonDRX, is replaced by absolute numbers.

- Proposal: The value of M and N are agreed as in Table 4.

Tabell 4 The validity time of M and N

| DRX cycle [2] | min (Tmeasure_intra_UE cat M1 in nonDRX, DRX_cycle length) | Value of M and N [ms] |
| --- | --- | --- |
| 1.28 | min(1600, DRX cycle) | 1280 |
| 2.56 | min(1600, DRX cycle) | 1600 |
| 5.12 | min(1600, DRX cycle) | 1600 |
| 10.24 | min(1600, DRX cycle) | 1600 |

# Summary

In this contribution we have discussed the option identified for deriving the value of M and N from the way forward in [1]. Based on the discussion, we see the need to express the formula in absolute numbers instead of variables which are CONNECTED mode specific, and following proposal is made:

- Proposal: The value of M and N are agreed as in Table 4.

Tabell 4 The validity time of M and N

| DRX cycle [2] | min (Tmeasure_intra_UE NB-IoT in nonDRX, DRX_cycle length) | Value of M and N [ms] |
| --- | --- | --- |
| 1.28 | min(1600, DRX cycle) | 1280 |
| 2.56 | min(1600, DRX cycle) | 1600 |
| 5.12 | min(1600, DRX cycle) | 1600 |
| 10.24 | min(1600, DRX cycle) | 1600 |

# References

- R4-1912783 , “Way forward on RRM requirements of R16 enhancement for NB-IoT”, Ericsson, Huawei, HiSilicon, October 2019

- TS 36.133, V15.7.0