3GPP TSG-RAN WG4 Meeting #93	R4-1914876

RENO, USA, October 18 – 22, 2019

Agenda item:	9.1.4.9

Source: 	Huawei, HiSilicon

Title: 	Discussion on measurement requirements in NR-U

Document for:	Discussion

- Introduction

In last meeting RAN4 92bis, the RRM requirements and impact of NR-U are discussed, and the potential agreements and the remaining open issues about measurement requirements are summarized in the WF [1].

In this paper, we discuss the measurement requirements in NR-U based on the discussion in the last meeting.

- Discussion

In the existing requirements [2], the UE shall conduct intra-frequency and inter-frequency measurements as required in terms of TPSS/SSS_sync_intra/inter, T SSB_measurement_period_intra/inter and TSSB_time_index_intra/inter . TSSB_time_index_intra/inter is included when the UE is indicated to report SSB based RRM measurement result with the associated SSB index. As discussed in the last RAN4 meeting, the corresponding requirements shall be extended to capture the unavailable DRS occasions due to LBT in NR-U scenarios.

The existing SSB index identification procedure is described in TS 38.213 [3] as follows:

“

The candidate SS/PBCH blocks in a half frame are indexed in an ascending order in time from 0 to . A UE determines the 2 LSB bits, for , or the 3 LSB bits, for , of a SS/PBCH block index per half frame from a one-to-one mapping with an index of the DM-RS sequence transmitted in the PBCH. For , the UE determines the 3 MSB bits of the SS/PBCH block index per half frame from PBCH payload bits  as described in TS 38.212 [4].

”

As defined in TS 38.133 [2], the time requirements for SSB index identification for FR2 inter-frequency measurement is extended considering the delay of PBCH payload acquiring. When it comes to NR-U scenarios, the SSB index identification procedure is defined as follows from RAN 1 agreements:

RAN1 97 Agreement: The mechanism to determine serving cell timing is as follows: - The SS/PBCH block position index within a DRS transmission window is detected using a combination of PBCH DMRS sequence index and 1 bit/2 bits for 15 kHz SCS/30 kHz SCS of the 3 available bits in the PBCH payload (not in MIB) originally used in Rel-15 FR2 for MSB SSB index - 10-bits SFN and half-frame indicator are indicated as in Rel-15 - PBCH payload size is not increased compared to Rel-15 - FFS: Whether reuse of other available bits in PBCH payload is also required for timing determination - If the UE is required to perform PBCH decoding of neighbor cell(s) (e.g., in asynchronous deployments), an explicit time allowance for acquisition of SSB index is provided to the UE RAN1 98 Agreement: The 3 LSB bits of the SSB candidate position index are represented by the PBCH DMRS sequence index.

Based on the design in RAN1 of SSB index indication in NR-U, it can be observed that the PBCH payload shall always be acquired when identifying the SSB index in NR-U scenarios.

Observation 1: In NR-U scenarios, the PBCH payload shall always be acquired when identifying the SSB index.

The requirements for SSB index acquisition in FR1 in TS 38.133 [2] only considered the time for PBCH DMRS sequence reading. Therefore, in NR-U scenarios, the time requirements for SSB index identification in FR1 should be extended to capture the change of SSB index identification procedure.

Proposal 1: In NR-U scenarios, the time requirements for SSB index identification in FR1 should be extended to capture the change of SSB index identification procedure.

- Conclusions

Observation 1: In NR-U scenarios, the PBCH payload shall always be acquired when identifying the SSB index.

Proposal 1: In NR-U scenarios, the time requirements for SSB index identification in FR1 should be extended to capture the change of SSB index identification procedure.

References

[1] R4-1912851 “WF on RRM Requirements for NR-U”

[2] TS 38.133