3GPP TSG-RAN4 Meeting #93	R4-1914965

# Reno, US, 18th – 22nd November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0353 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on interruption due to autonomous gaps in LTE CGI reading in 38.133 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_Enh_Core |  |  |  |  |  | Date: |  |  | 2019-10-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The interruption requirements need to be specified for CGI reading on LTE cells, when configured by NR PCell when UE is in NR SA and NE-DC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define interruption requirements for CGI reading on LTE cells, when configured by NR PCell when UE is in NR SA and NE-DC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | interruption requirements for CGI reading on LTE cells are missing in 38.133 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 8.2.1, 8.2.2, 8.2.3, 8.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

8.2.1.2.8	Interruption at E-UTRA measurement with autonomous gaps

When UE is configured by E-UTRA PCell to identify CGI of an E-UTRA FDD or E-UTRA TDD cell, UE may cause interruptions on PSCell, and SCell(s) in SCG if configured.

During the time period of  and  as specified in clause 8.17.15 and 8.17.16 of TS 36.133 [16], over which the UE identifies the new CGI of the E-UTRA cell, the UE shall transmit at least NACK/NACK ACK/NACKs on PSCell and each of activated SCell(s) in the frequency range where the autonomous gaps are created, where NACK/NACK is specified in Table 8.2.1.2.8-1, provided that

-	there is continuous DL data allocation,

-	no DRX cycle is used,

-	no measurement gaps are configured,

-	only one code word is transmitted in each slot,

-	2 slot ACK/NACK feedback is configured,

-	20 ms SMTC period is configured,

-	SSBs are transmitted in one slot within SMTC window.

Table 8.2.1.2.8-1: Number of ACK/NACKs transmitted by the UE during the time for identification of CGI of an E-UTRA cell

| NACK/NACK | Configuration of the serving cell in which the transmitted ACK/NACKs are counted |  |
| --- | --- | --- |
|  | Duplex mode configuration | SCS |
| 84 | FDD | 15 kHz |
| 193 | FDD | 30 kHz |
| 402 | FDD | 60 kHz |
| 28 | TDD Note 1 | 15 kHz |
| 81 | TDD Note 1 | 30 kHz |
| 159 | TDD Note 1 | 60 kHz |
| 233 | TDD Note 2 | 60 kHz |
| 491 | TDD Note 2 | 120 kHz |
| NOTE 1: TDD UL-DL configuration is as specified in Table A.3.3.1-1 of TS 38.101-1 [18]. NOTE 2: TDD UL-DL configuration is as specified in Table A.3.3.1-1 of TS 38.101-2 [19]. |  |  |

<End of Change 1>

<Start of Change 2>

##### 8.2.2.2.7	Interruptions at E-UTRA measurement with autonomous gaps

When UE is configured by PCell to identify CGI of an E-UTRA FDD or E-UTRA TDD cell, UE may cause interruptions on PCell, and SCell(s) if configured.

During the time period of  as specified in clause 9.4.6, over which the UE identifies the new CGI of the E-UTRA cell, the UE shall transmit at least NACK/NACK ACK/NACKs on PCell and each of activated SCell(s) in the frequency range where the autonomous gaps are created as specified in clause 8.2.1.2.8.

<End of Change 2>

<Start of Change 3>

8.2.3.2.8	Interruption at E-UTRA measurement with autonomous gaps

When UE is configured by PCell or E-UTRA PSCell to identify CGI of an E-UTRA FDD or E-UTRA TDD cell, UE may cause interruptions on PCell, and SCell(s) in MCG if configured.

During the time period of  as specified in clause 9.4.6 when identification of CGI of E-UTRA cell is configured by PCell, or during the time period of  and  as specified in clause 8.19.6 and 8.9.7 of TS 36.133 [16] when identification of CGI of E-UTRA cell is configured by E-UTRA PSCell, over which the UE identifies the new CGI of the E-UTRA cell, the UE shall transmit at least NACK/NACK ACK/NACKs on PCell and each of activated SCell(s) in the frequency range where the autonomous gaps are created as specified in clause 8.2.1.2.8.

<End of Change 3>

<Start of Change 4>

8.2.4.2.8	Interruption at E-UTRA measurement with autonomous gaps

When UE is configured by PCell or PSCell to identify CGI of an E-UTRA FDD or E-UTRA TDD cell, UE may cause interruptions on PCell, PSCell and SCell(s) in MCG and SCG if configured.

During the time period of  as specified in clause 9.4.6, over which the UE identifies the new CGI of the E-UTRA cell, the UE shall transmit at least NACK/NACK ACK/NACKs on PCell and each of activated SCell(s) in the frequency range where the autonomous gaps are created as specified in clause 8.2.1.2.8.

<End of Change 4>