3GPP TSG-RAN WG4 Meeting 93	R4-195047

# Reno, USA, 18th – 22nd Nov, 2019

Source: 	Skyworks Solutions, Inc.

Title: 	A-MPR&MPR Improvements for DC_(n)41 DC_41_n41 and DC_3_n3

Agenda Item:	10.14.2 Improvements to A-MPR/MPR for 26 dBm n41 and B41/n41 EN-DC   [LTE_NR_B41_Bn41_PC29dBm]

Document for:	Approval

# Introduction

In RAN4#92b meeting, three contributions [1, 2, 3] provided measurements and analysis for Band 41/n41 NS01 and NS04 performance for 10 and 13dB antenna isolation. Although a formal agreement could not be achieved on values, the conclusions were very similar. In this contribution, we try to summarize the different company’s input and make proposals. Also proposal for DC_3_n3 is provided.

# Discussion

## Inputs from the Different Contributions

Contribution [2, 3] provided measurements for equal power sharing while [1] also provided data across different LTE and NR power pairs. Also, [1] includes analysis of the data form earlier contributions in [4] to cover ET solutions.

Although proposals are different between the different contributions, in fact they all point at the same cases for potential improvement and to the more challenging cases. Also, they are fairly aligned in terms of improvement related to 3 dB higher antenna isolation. The main difference is mostly related to how the curves decay for larger allocation and use of different inflection  points.

In an effort to consolidate a common understanding and hopefully a find a consensus, the different companies input are summarized in Table 1 and 2:

- Table 1 provides values for NSO4 -13 dBm/MHz IM3 and NS01 contiguous case

- Table 1 provides values for NSO4 -13 dBm/MHz IM3 and NS01 non-contiguous case

Table 1: NSO4 -13 dBm/MHz IM3 and NS01 DC_(n)X

| cases | 38.101-3 | company A | company B | company C |
| --- | --- | --- | --- | --- |
| power | valid across Pshare with AMPRtot | equal power only | equal power only | includes margin for different LTE/NR powers and ET |
| PC2 NS04 10dB | 15dB B≤0.5 | 10dB 0≤B<1 | 9dB 0≤B<1 | 12dB B≤0.54 |
|  | 10dB 0.5≤B<1 | 8dB 1≤B<2 | 8dB 1≤B<5.4 | 10dB 0.54≤B<1.08 |
|  | 8dB 1≤B<2 | 6dB 2≤B<8 | 6dB 5.4≤B<8.1 | 8dB 1.08≤B<2 |
|  | 6dB 2≤B | 0dB 8≤B | 5dB 8.1≤B<13.5 | 6dB 2≤B for ACLR ! Not needed for NS04 ( ACLR term) |
|  |  |  | 4dB 13.5≤B |  |
| PC2 NS04 13dB | na | 10dB 0 ≤B<1 | Reduction of 1dB | Reduction of 1.5dB |
|  |  | 7dB 1≤B<2 |  |  |
|  |  | 4dB 2≤B<5 |  |  |
|  |  | 0dB 5≤B |  |  |
| PC2 NS01 10dB | same as NS04 but no separate term for ACLR | same as NS04 | same as NS04 + with -30dBm check but less than -13dBm case | same as NS04 |
|  |  |  |  | ! For NS01 6dB needed for ACLR cases |
| PC2 NS01 13dB | na | same as NS04 | Reduction of 1dB | same as NS04 |
|  |  |  |  | ! For NS01 6dB needed for ACLR cases |
| PC3 NS01 | same as PC2 | na | na | use 13dB isolation curve |

For the baseline 10 dB isolation case, all companies point at improvement for small allocation which was obvious as it had the same MA value than for a -25 dBm/MHz IMD3 requirement.

The current specification and company A uses inflection points which are not multiples of 15 kHz SCS RB bandwidth while company B and C have modified some inflection points to do so. It is important to modify inflection point to be multiple of 180 kHz to avoid corner cases at the limit as has been discovered by company B in their measurements.

Initial spec used a first inflection point for B=0.5 and company C uses and inflection point at 0.54. This is important to allow higher back-off for 1RB+1RB and 1RB+2RB cases at 15 kHz SCS which are the most critical and need margin in particular for ET implementation.

From B>2 the main difference is related to how fast the MPR decays. Here it is important to say that solutions without DPD will perform worse than ET using DPD. Note that in NS04 the ACLR AMPR term anyhow limits to 6 dB.

Company C observed that 6 dB floor was needed to account for ACLR when there are large power differences between LTE and NR. In fact, in NS04 there is a separate AMPR term that will take of this so it is acceptable to further decay AMPR for IM3. However, for NS01, such term does not exist as a single curve covers all cases and provision for ACLR is needed.

For improvement versus 3 dB higher isolation, the measured improvement ranges from 1 to 2 dB.

Based on the above examinations we make the following proposals:

Proposal 1 for NS04 -13 dBm/MHz curve MA=:

- 12 dB for B≤0.54

- 10 dB for 0.54≤B<1.08

- 8 dB for 1.08≤B<[5.4]

- 6 dB for [5.4]≤B<8.1

- [4] dB for B<8.1

- MA is improved by [1.5]dB when optional improved performance is signaled

- Valid for PC2 ENDC with two PC2 PAs

Proposal 2 for NS01 contiguous intra-band curve MA=:

- 12 dB for B≤0.54

- 10 dB for 0.54≤B<1.08

- 8 dB for 1.08≤B<[5.4]

- 6 dB for [5.4]≤B (alternatively ACLR issue is handled with another term as for NS04 of with single RAT MPR in Pshare equations)

- Optional improved performance is not possible for bands <2.3GHz

- Valid for PC2 ENDC with two PC2 PAs

- MA is reduced by 1 dB for PC3 ENDC two PC3 PAs

Table 2: NSO4 -25 dBm/MHz IM3 and NS01 DC_X_nX

| cases | 38.101-3 | company A | company B | company C |
| --- | --- | --- | --- | --- |
| power | valid across Pshare with AMPR tot | equal power only | equal power only | includes margin for different LTE/NR powers and ET |
| PC2 NS04 10dB | 15dB B≤1 | 15dB 0≤B<1 | 15dB 0≤B< 1 | No improvement for A<5 |
|  | 14dB 1≤B<2 | 14dB 1≤B<2 | 14dB 1≤B<5 | possible improvement for ACLR region |
|  | 13dB 2≤B<5 | 13dB 2 ≤ B<5 | 13dB 5≤B<8.1 |  |
|  | 12dB 5≤B | 12dB 5≤B<10 | 12dB 8.1≤B<10.8 |  |
|  |  | 10dB 10≤B | 11dB 10.8≤B<13.5 |  |
|  |  |  | 10dB 13.5≤B<21.6 |  |
|  |  |  | 9dB 21.6≤B |  |
| PC2 NS04 13dB | na | 14dB 0≤B<2 | Reduction of 1dB | Reduction of 1dB |
|  |  | 11dB 2≤ B<5 |  | possible improvement for ACLR region |
|  |  | 10dB 5≤B<10 |  |  |
|  |  | 9dB 10≤B |  |  |
| PC2 NS01 10dB | 18dB B≤1 | 18dB 0≤B<1 | 16dB 0≤B< 2.7 | No improvement for A<5 |
|  | 17dB 1≤B<2 | 17dB 1≤B<2 | 15dB 1≤B<5.4 | possible improvement for ACLR region |
|  | 16dB 2≤B<5 | 15dB 2≤B<5 | 13dB 5.4≤B<10.8 |  |
|  | 15dB 5≤B | 14dB 5≤B<10 | 12dB 10.8≤B<16.2 |  |
|  |  | 13dB 10≤B | 11dB 16.2≤B |  |
| PC2 NS01 13dB | na | 17dB 0≤B<2 | Reduction of 1dB | Reduction of 1dB |
|  |  | 13dB 2≤B<10 |  | possible improvement for ACLR region |
|  |  | 9dB 10≤B |  |  |
| PC3 NS01 | na | na | na | use 13dB isolation curve |

For the baseline NS04 10 dB isolation case, all companies point that improvements are only feasible for larger allocations and that the current spec should be maintained up to B=10.8 then decaying is slightly different and some inflection points are different.

For the baseline NS01 10 dB isolation case, some improvements are shown as all data show that the back-off difference between -25 dBm/MHz and -30 dBm/MHz is less than 3 dB. The main difference is in the appreciation of margin needed for implementation and notably for ET and power variations which are becoming more and more critical at large back-off as already explained in [1]. Improvements should be carefully weighed.

In most cases the proposed improvement for 13 dB antenna isolation is 1 dB.

Based on the above examinations we make the following proposals:

Proposal 3 for NS04 -25 dBm/MHz curve MA=:

- 15 dB for 0.54≤B<1.08

- 14 dB for 1.08≤B<2.16

- 13 dB for 2.16≤B<5.4

- 12 dB for 5.4≤B<10.8

- [10] dB for B<10.8

- MA is improved by [1] dB when optional improved performance is signaled

- Valid for PC2 ENDC with two PC2 PAs

Proposal 4 for PC2 NS01 non-contiguous intra-band curve MA=:

- 18 dB for 0.54≤B<1.08

- 17 dB for 1.08≤B<2.16

- 15 dB for 2.16≤B<5.4

- 14 dB for 5.4≤B<10.8

- [12] dB for B<10.8

- Optional improved performance is not possible for bands <2.3 GHz

- Valid for PC2 ENDC with two PC2 PAs

- MA is reduced by 1 dB for PC3 ENDC two PC3 PAs

## Applicability to DC_3_n3

In last meeting, it was explained in [1] that the NS01 non-contiguous EN-DC curve was valid for DC_3_n3. It was argued that it should rather wait for whatever improvements are agreed for Band 41 non-contiguous EN-DC. In this contribution, we have proposed improvements for PC2 NS01 non-contiguous intra-band curve and propose that 13 dB isolation related improvement is not possible in Band 3, but that PC3 can have an improved MPR by 1 dB. Since the other improvements are related to power sharing or allocation types that are not valid for non-contiguous cases, we can thus conclude on DC_3_n3 numbers.

Proposal 5 for DC_3_n3: MPR numbers from proposal 4 are used with the additional 1 dB improvement for PC3 operation.

## Further Optimizations

As already shown in [1] there are two types of further optimizations:

- Related to power sharing which allows to reach higher NR power (without dropping NR) when LTE power is low, these are band independent and valid for both contiguous and non-contiguous EN-DC. The MPR/AMPR total approach needs to be revisited for this and data is provided in [5] to support such optimization.

- Related to allocation types that is only suggested for contiguous intra-band ENDC and follow principles that will be elaborated for NR UL CA (based on chairman’s input) but need to also accommodate for power sharing cases different to equal PSD. This contiguous/non-contiguous and inner/outer allocation approach is described in [5] for NR UL CA and data is provided in [6] to support such optimization for contiguous intra-band EN-DC. Notably inner allocation for intra-band EN-DC are key to enable a 29 dBm power class.

# Conclusions

In this contribution, we discuss the input provided by three companies [1, 2, 3] including the analysis of earlier contributions from [4]. In an effort to finally achieve progress now that multiple companies have provided input based on measured data, the flowing proposals consolidating all the data are made.

Proposal 1 for NS04 -13 dBm/MHz curve MA=:

- 12 dB for B≤0.54

- 10 dB for 0.54≤B<1.08

- 8 dB for 1.08≤B<[5.4]

- 6 dB for [5.4]≤B<8.1

- [4] dB for B<8.1

- MA is improved by [1.5]dB when optional improved performance is signaled

- Valid for PC2 ENDC with two PC2 PAs

Proposal 2 for NS01 contiguous intra-band curve MA=:

- 12 dB for B≤0.54

- 10 dB for 0.54≤B<1.08

- 8 dB for 1.08≤B<[5.4]

- 6 dB for [5.4]≤B (alternatively ACLR issue is handled with another term as for NS04 of with single RAT MPR in Pshare equations)

- Optional improved performance is not possible for bands <2.3GHz

- Valid for PC2 ENDC with two PC2 PAs

- MA is reduced by 1 dB for PC3 ENDC two PC3 PAs

Proposal 3 for NS04 -25 dBm/MHz curve MA=:

- 15 dB for 0.54≤B<1.08

- 14 dB for 1.08≤B<2.16

- 13 dB for 2.16≤B<5.4

- 12 dB for 5.4≤B<10.8

- [10] dB for B<10.8

- MA is improved by [1] dB when optional improved performance is signaled

- Valid for PC2 ENDC with two PC2 PAs

Proposal 4 for PC2 NS01 non-contiguous intra-band curve MA=:

- 18 dB for 0.54≤B<1.08

- 17 dB for 1.08≤B<2.16

- 15 dB for 2.16≤B<5.4

- 14 dB for 5.4≤B<10.8

- [12] dB for B<10.8

- Optional improved performance is not possible for bands <2.3 GHz

- Valid for PC2 ENDC with two PC2 PAs

- MA is reduced by 1 dB for PC3 ENDC two PC3 PAs

Also we propose to finalize the pending DC_3_n3 case with following proposal:

Proposal 5 for DC_3_n3: MPR numbers from proposal 4 are used with the additional 1 dB improvement for PC3 operation.

Other optimization related to power sharing and allocation types are covered in other documents in this meeting.

# References

[1]	R4-1911647	Potential Improvements for Band 41 Intra-band ENDC and Applicability to Other Cases, Skyworks Solutions Inc., RAN4#92b

[2] R4-1911029	A-MPR/MPR improvement for B41/n41 EN-DC	Intel Corporation, RAN4#92

[3] R4-1911306	A-MPR measurement results for 29dBm HPUE B41/n41 EN-DC 	LG Electronics Polska, RAN4#92b

[4] R4-1910306	WF on B41/n41 29 dBm Power Class and EN-DC A-MPR Improvement	Sprint Corporation, Qorvo, Skyworks, RAN4#92

[5] R4-1914273 [NR ULCA] types of contiguous and non-contiguous allocation for class B and C UL CA, Skyworks Solutions Inc., RAN4#93

[6] R4-1915398, DC_(n)41 and DC_41_n41 Data in support for MPR&AMPR Improvements, Skyworks Solutions Inc., RAN4#93