3GPP TSG-RAN WG4 Meeting #93	R4-1915187

Reno, NV, USA, 18 – 22 November, 2019

Agenda item:	8.10.3.4

Source: 	Qualcomm Incorporated

Title: 	On remaining issues for MTC PUR

Document for:	Discussion

# Introduction

In RAN4#92-Bis meeting, the following agreements were reached for PUR TA validation in MTC [1]:

- The first and the second measurement conditions agreed at RAN4#92 meeting  (R4-1910107) include parameters M and N, whose values are defiend as follows:

  - When TA is obtained in RRC_CONNECTED mode: M=[TEvaluate_Qin_CatM1/2]

  - When TA is obtained in RRC_IDLE mode:

    - Option 1:

      - M = min (Tmeasure_intra_UE cat M1 in nonDRX, DRX_cycle length)

      - N = min (Tmeasure_intra_UE cat M1 in nonDRX, DRX_cycle length)

    - Other options are not precluded.

In this paper, we discuss the remaining issues related to measurements in RRC idle.

# Discussion

In the last RAN1 meetings, it was agreed that UE can receive a TA update through a dedicated PUR ACK DCI [2]:

Agreement [36.212]

The dedicated PUR ACK DCI at least includes the Timing Advance adjustment (including TA adjustment of 0). The TA adjustment field is [6] bits as legacy.

If UE receives a TA update through PUR ACK DCI in RRC IDLE, then the first RSRP measurement that was performed in RRC CONNECTED to obtain reference RSRP measurement should be discarded and replaced with the most recent version of the second RSRP measurement.

Proposal 1. When UE receives a TA update through PUR ACK DCI in RRC IDLE, then the first RSRP measurement that was performed in RRC CONNECTED to obtain reference RSRP measurement should be discarded and replaced with the most recent version of the second RSRP measurement, i.e., RSRP1 := RSRP2.

We believe there is a typo error in the WF [1] shown in Section 1 above. In RRC CONNECTED mode, the first measurement is relevant and per previous WF [3], the first measurement shall be performed within the following time range: T1-N ≤T1'≤T1+N. Hence, we believe the agreement should be corrected to the following:

Proposal 2. When TA is obtained in RRC_CONNECTED mode: M N=[TEvaluate_Qin_CatM1/2] for the first measurement (RSRP1).

When UE is in RRC IDLE, it continues to perform measurements every DRX cycle at least for serving cell evaluation unless it is in relaxed serving cell monitoring state in which case the measurement is done every Nrelax DRX cycle (we use Nrelax notation to avoid confusion with the N parameter discussed above).

Observation 1. When UE is in RRC IDLE, it continues to perform measurements every DRX cycle at least for serving cell evaluation unless it is in relaxed serving cell monitoring state in which case the measurement is done every Nrelax DRX cycle.

If DRX cycles are short enough, then UE should be allowed to re-use the filtered measurements from DRX cycles used in serving cell evaluations for TA validation of PUR. This refers to the top portion of Figure 1 below. However, if DRX cycles are long or if UE is in relaxed serving cell monitoring with relaxation factor Nrelax that effectively makes serving cell evaluation periods long, then UE should wake up no earlier than Tmeasure_intra_UE cat M1 in nonDRX to make a measurement for PUR validation. It is noted that that a UE that requires less number of subframes than Tmeasure_intra_UE cat M1 in nonDRX to make a valid measurement should be allowed to wake up later than point (A) in bottom portion of Figure 1.

Figure 1 DRX cycles and PUR occasions

Proposal 3. When TA is obtained in RRC_IDLE mode: M = min (Tmeasure_intra_UE cat M1 in nonDRX, Nrelax .DRX_cycle length) for the second measurement (RSRP2); i.e., UE shall not wake up earlier than M to make measurements.

# Conclusions

Proposal 1. When UE receives a TA update through PUR ACK DCI in RRC IDLE, then the first RSRP measurement that was performed in RRC CONNECTED to obtain reference RSRP measurement should be discarded and replaced with the most recent version of the second RSRP measurement, i.e., RSRP1 := RSRP2.

Proposal 2. When TA is obtained in RRC_CONNECTED mode: M N=[TEvaluate_Qin_CatM1/2] for the first measurement (RSRP1).

Observation 1. When UE is in RRC IDLE, it continues to perform measurements every DRX cycle at least for serving cell evaluation unless it is in relaxed serving cell monitoring state in which case the measurement is done every Nrelax DRX cycle.

Proposal 3. When TA is obtained in RRC_IDLE mode: M = min (Tmeasure_intra_UE cat M1 in nonDRX, Nrelax .DRX_cycle length) for the second measurement (RSRP2), i.e., UE shall not wake up earlier than M to make measurements.

# References

[1] R4-1912735

[2] R1-1911378

[3] R4-1910107