3GPP TSG-RAN WG4 Meeting #93	R4-1915418

Reno, US 18th – 22nd Nov 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR |  | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Frequency Awareness in MPR for intra-band EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sprint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CA_R16_intra-Core |  |  |  |  |  | Date: |  |  | 2019-11-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Intra-band non-contiguous MPR definition assumes all IM3s fall into -30 dBm/MHz spurious region, requiring large backoff allowances. Many allocation combinations result in IM3 falling into -25 or -13 dBm/MHz regions and require less backoff. This CR optimizes MPR, adapting the approach used by A-MPR B41/n41 A-MPR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds -25 dBm/MHz MPR table. Adds frequency domain calculation to determine applicable SEM region, and defines MPR based on SEM region. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Excessive MPR is allowed for non-contiguous intra-band EN-DC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.2.1.1, 6.2B.2.2.2, 7.3B.2.3.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************************************ Start of changes *****************************************

### 6.2B.2	UE maximum output power reduction for DC

#### 6.2B.2.0	General

The UE maximum output power reduction (MPR) specified in this subclause is applicable for UEs configured with EN-DC when NS_01 is indicated in the MCG and the SCG. The MPR applies subject to indication in the field modifiedMPRbehavior for the SCG [2].

#### 6.2B.2.1	Intra-band contiguous EN-DC

##### 6.2B.2.1.1	General

When the UE is configured for intra-band contiguous EN-DC, the UE determines the total allowed maximum output power reduction as specified in this subclause.

For UE supporting dynamic power sharing the following:

-	for the MCG, MPRc in accordance with TS 36.101 [4]

-	for the SCG,

MPR'c = MPRNR = MAX( MPRsingle,NR, MPREN-DC)

-	for the total configured transmission power,

MPRtot = PPowerClass,EN-DC – min(PPowerClass,EN-DC ,10*log10(10^((PPowerClass,E-UTRA - MPRE-UTRA)/10) + 10^((PPowerClass,NR - MPRNR)/10))

where

MPRE-UTRA = MAX(MPRsingle,E-UTRA, MPREN-DC )

with

-	MPRsingle, E-UTRAis the MPR defined for the E-UTRA transmission in TS 36.101 [4]

-	MPRsingle,NR is the MPR defined for the NR transmission in TS 38.101-1 [2]

-	MPREN-DC = MPRIM3 defined in Subclause 6.2B.2.1.2

For UEs not supporting dynamic power sharing the following

-	for the MCG,

MPRc = MAX(MPRsingle,E-UTRA, MPREN-DC )

-	for the SCG,

MPR'c = MAX( MPRsingle,NR, MPREN-DC )

where

-	MPRsingle,NR is the MPR defined for the NR transmission in TS 38.101-1 [2]

-	MPRsingle,E-UTRA is the MPR defined for the E-UTRA transmission in TS 36.101 [4]

-	MPREN-DC = MPRIM3 defined in Subclause 6.2B.2.1.2

##### 6.2B.2.1.2	MPR for power class 3 and power class 2 to meet -13 dBm / 1MHz for 26dBm UE power

MPR in this subclause is applicable for power class 3 and power class 2 UEs indicating IE dualPA-Architecture supported with ENDC power class being the same as the E-UTRA and NR power class. For UEs not indicating dualPA-Architecture supported, MPR in subclause 6.2.4 of TS 36.101 [4] and 6.2.3 of TS 38.101-1 [2] apply when the UE is scheduled with single uplink transmission, otherwise the UE can use as much MPR as needed to fulfil emissions requirements. For a UE supporting dynamic power sharing for DC_(n)71AA for which dual simultaneous uplink transmissions are mandatory and A-MPR defined in subclause 6.2B.3.1.1 is applied as MPR. The allowed maximum output power reduction applied to transmission on the MCG and the SCG is defined as follows:

MPRIM3 = MA

Where MA is defined as follows

MA = 	[15]	;	 0 ≤ B < 0.5

[10]	;	 0.5 ≤ B < 1.0

[8]		; 	1.0 ≤ B < 2.0

[6]		; 	2.0 ≤ B

Where:

For UEs supporting dynamic power sharing,

B = (LCRB_alloc, E-UTRA * 12* SCSE-UTRA + LCRB_alloc,NR * 12 * SCSNR)/1,000,000

For UEs not supporting dynamic power sharing,

For E-UTRA

B = (LCRB_alloc, E-UTRA * 12* SCSE-UTRA + 12 * SCSNR)/1,000,000

Where SCSNR = 15 kHz is assumed in calculation of B.

For NR

B = (12* SCSE-UTRA + LCRB_alloc,NR * 12 * SCSNR)/1,000,000

Where SCSE-UTRA = 15 kHz is assumed in calculation of B.

and MA is reduced by 1 dB for B < 2.

#### 6.2B.2.2	Intra-band non-contiguous EN-DC

##### 6.2B.2.2.1	General

When the UE is configured for intra-band non-contiguous EN-DC, the UE determines the total allowed maximum output power reduction as specified in this subclause.

For UE supporting dynamic power sharing the following:

-	for the MCG, MPRc in accordance with TS 36.101 [4]

-	for the SCG,

MPR'c = MPRNR = MAX( MPRsingle,NR, MPREN-DC)

-	for the total configured transmission power,

MPRtot = PPowerClass,EN-DC – min(PPowerClass,EN-DC ,10*log10(10^((PPowerClass,E-UTRA - MPRE-UTRA)/10) + 10^((PPowerClass,NR - MPRNR)/10))

where

MPRE-UTRA = MAX(MPRsingle,E-UTRA, MPREN-DC )

MPREN-DC = MAX(MPRIM3, MPRACLRoverlap )

with

-	MPRsingle, E-UTRAis the MPR defined for the E-UTRA transmission in TS 36.101 [4]

-	MPRsingle,NR is the MPR defined for the NR transmission in TS 38.101-1 [2]

For UEs not supporting dynamic power sharing the following

-	for the MCG,

MPRc = MAX(MPRsingle,E-UTRA, MPRIM3, MPRACLRoverlap )

-	for the SCG,

MPR'c = MAX( MPRsingle,NR, MPRIM3, MPRACLRoverlap )

where

-	MPRsingle,NR is the MPR defined for the NR transmission in TS 38.101-1 [2]

-	MPRsingle,E-UTRA is the MPR defined for the E-UTRA transmission in TS 36.101 [4]

The UE determines the value of MPREN-DC as follows:

If AND( FIM3,low_block,low > SEM-13,low ,  FIM3,high_block,high < SEM-13,high )

MPRIM3 defined in Subclause 6.2B.2.1.2

Else If AND( FIM3,low_block,low > SEM-25,low ,  FIM3,high_block,high < SEM-25,high )

MPRIM3 defined in Subclause 6.2B.2.2.2

Else

MPRIM3 defined in Subclause 6.2B.2.2.3

where

-	FIM3,low_block,high = (2 * Flow_alloc,high_edge ) – Fhigh_alloc,low_edge

-	FIM3,high_block,low = (2 * Fhigh_alloc,low_edge) – Flow_alloc,high_edge

-	Flow_alloc,low_edge is the lowermost frequency of the lower transmission bandwidth allocation.

-	Flow_alloc,high_edge is the uppermost frequency of the lower transmission bandwidth allocation.

-	Fhigh_alloc,low_edge is the lowermost frequency of the upper transmission bandwidth allocation.

-	Fhigh_alloc,high_edge is the uppermost frequency of the upper transmission bandwidth allocation.

-	SEM-13,low = Threshold frequency where lower spectral emission mask below the lower channel drops from -13 dBm / 1MHz to -25 dBm / 1MHz, as specified in Subclause 6.5B.2.2.1.

-	SEM-13,high = Threshold frequency where upper spectral emission mask above the upper channel drops from -13 dBm / 1MHz to -25 dBm / 1MHz, as specified in Subclause 6.5B.2.2.1.

-	SEM-25,low = Threshold frequency where lower spectral emission mask below the lower channel drops from -25 dBm / 1MHz to -30 dBm / 1MHz, as specified in Subclause 6.5B.2.2.1.

-	SEM-25,high = Threshold frequency where upper spectral emission mask above the upper channel drops from -25 dBm / 1MHz to -30 dBm / 1MHz, as specified in Subclause 6.5B.2.2.1.

NOTE:	For non-dynamic power sharing capable UEs, since the allocation is unknown for one RAT, the edges of the channel transmission bandwidth are used instead of the edges of the RB allocations for that RAT.

The UE determines the value of MPRACLRoverlap as specified in Table 6.2B.2.2-1:

Table 6.2B.2.2-1: MPRACLRoverlap

| Wgap | MPRACLRoverlap |
| --- | --- |
| < BWchannel,E-UTRA + BWchannel,NR | 4 dB |
| ≥ BWchannel,E-UTRA + BWchannel,NR | 0 dB |
| NOTE 1: Wgap = Fhigh_channel,low_edge - Flow_channel,high_edge |  |

##### 6.2B.2.2.2	MPR for power class 3 and power class 2 to meet -25 dBm / 1MHz for 26dBm UE power

MPR in this subclause is applicable for power class 3 and power class 2 UEs indicating IE dualPA-Architecture supported with ENDC power class being the same as the E-UTRA and NR power class. For UEs not indicating dualPA-Architecture supported, MPR in subclause 6.2.4 of TS 36.101 [4] and 6.2.3 of TS 38.101-1 [2] apply when the UE is scheduled with single uplink transmission, otherwise the UE can use as much MPR as needed to fulfil emissions requirements. The allowed maximum output power reduction for IM3 related emissions applied to transmission on the MCG and the SCG is defined as follows:

MPRIM3 = MA

Where MA is defined as follows

MA = 	[15]	; 	0 ≤ B < 1.0

[14]	; 	1.0 ≤ B < 2.0

[13]	; 	2.0 ≤ B < 5.0

[12]	; 	5.0 ≤ B

Where:

For UEs supporting dynamic power sharing,

B = (LCRB_alloc, E-UTRA * 12* SCSE-UTRA + LCRB_alloc,NR * 12 * SCSNR)/1,000.000

For UEs not supporting dynamic power sharing,

For E-UTRA

B= (LCRB_alloc, E-UTRA * 12* SCSE-UTRA + 12 * SCSNR)/1,000.000

Where SCSNR = 15 kHz is assumed in calculation of B.

For NR

B = (12 * SCSE-UTRA + LCRB_alloc,NR * 12 * SCSNR)/1,000.000

Where SCSE-UTRA = 15 kHz is assumed in calculation of B.

and MA is reduced by 1 dB for B < 2.

##### 6.2B.2.2.3	MPR for power class 3 and power class 2 to meet -30 dBm / 1MHz for 26dBm UE power

MPR in this subclause is applicable for power class 3 and power class 2 UEs indicating IE dualPA-Architecture supported with ENDC power class being the same as the E-UTRA and NR power class. For UEs not indicating dualPA-Architecture supported, MPR in subclause 6.2.4 of TS 36.101 [4] and 6.2.3 of TS 38.101-1 [2] apply when the UE is scheduled with single uplink transmission, otherwise the UE can use as much MPR as needed to fulfil emissions requirements. The allowed maximum output power reduction for IM3 related emissions applied to transmission on the MCG and the SCG is defined as follows:

MPRIM3 = MA

Where MA is defined as follows

MA = 	[18]	; 	0 ≤ B < 1.0

[17]	; 	1.0 ≤ B < 2.0

[16]	; 	2.0 ≤ B < 5.0

[15]	; 	5.0 ≤ B

Where:

For UEs supporting dynamic power sharing,

B = (LCRB_alloc, E-UTRA * 12* SCSE-UTRA + LCRB_alloc,NR * 12 * SCSNR)/1,000.000

For UEs not supporting dynamic power sharing,

For E-UTRA

B= (LCRB_alloc, E-UTRA * 12* SCSE-UTRA + 12 * SCSNR)/1,000.000

Where SCSNR = 15 kHz is assumed in calculation of B.

For NR

B = (12 * SCSE-UTRA + LCRB_alloc,NR * 12 * SCSNR)/1,000.000

Where SCSE-UTRA = 15 kHz is assumed in calculation of B.

and MA is reduced by 1 dB for B < 2.

#### 6.2B.2.3	Inter-band EN-DC within FR1

For inter-band EN-DC between E-UTRA and FR1 NR, UE maximum output power reduction specified in TS 36.101 [4] and TS 38.101-1 [2] apply for E-UTRA and NR respectively.

#### 6.2B.2.3a	Inter-band NE-DC within FR1

For inter-band NE-DC between E-UTRA and FR1 NR, UE maximum output power reduction specified in TS 36.101 [4] and TS 38.101-1 [2] apply for E-UTRA and NR respectively.

#### 6.2B.2.4	Inter-band EN-DC including FR2

UE maximum output power reduction requirement for E-UTRA single carrier and CA operation specified in subclauses 6.2.3 and 6.2.3A of TS 36.101 [4] and for NR single carrier and CA operation specified in subclause 6.2.2, 6.2A.2 , and 6.2D.2 of TS 38.101-2 [3] apply.

#### 6.2B.2.5	Inter-band EN-DC including both FR1 and FR2

UE maximum output power reduction requirement for E-UTRA single carrier and CA operation specified in subclauses 6.2.3 and 6.2.3A of TS 36.101 [4] and for NR single carrier and CA operation specified in subclause 6.2.2 of TS 38.101-1 [2] and subclause 6.2.2, 6.2A.2 , and 6.2D.2 of TS 38.101-2 [3] apply.

************************************ End of changes *****************************************